********************************************************************************

*                                                                              *

*                            THE MOTOROLA BIBLE (c)                            *

*                       For all Cellular AND Pager Info                        *

*                                                                              *

*                    MOTOROLA USERS AND PROGRAMMING GUIDES                     *

*                                By Mike Larsen                                *

*                                                     Ver. 3.0      ************

*                                                                   *  1/14/97 *

********************************************************************************

Table of Contents:

Section 1  Introduction             8   Phone Pin Outs

2  General User Info        9   Cable Specs

3  Programming Info         10  Channel Number vs. Frequency

4  Test Mode                11  Trik Clip

5  GSM Info and Test Mode   12  Pager Info

6  Hacking the FOVC         13  Disclaimer

7  Reading the SID

I*N*T*R*O*D*U*C*T*I*O*N****************************************S*E*C*T*I*O*N***1

GENERAL DISCLAIMER:

I, Mike Larsen, am in no way affiliated with Motorola or the Motorola

Cellular Subscriber Group or any other subsidiaries.  I offer this guide free to

the public not to encourage fraudulent use, but to give others the same

information I obtained for free (and sometimes for money).  Use of any

information from this text (in part or whole) is encouraged provided proper

credit is given to the author (Mike Larsen) and the work (The Motorola Bible).

Sale of this work (in whole or in part) is not discouraged provided proper

credit is given, names are left intact, and Email notice is given.  After all,

if someone is stupid enough to give money for something that is available on

the web for free, well...

This manual is not intended to be an aid in cellular fraud.  Cell fraud

is both illegal and immoral.  Would you like someone to make charges on your

phone?  If you want free calls, you want to check elsewhere for that information

because that type of information is NOT mentioned in the Motorola Bible.

This manual is not intended for use by people with little electronics

experience.  This is not a tutorial and not intended to be used except by

people with previous cellular experience and are familiar with programming

cellular phones.

If you have any additions or corrections about this manual, please

email me at:

mjlarsen@hotmail.com

Additional Info:

It has come to my attention that this has been sold at ham fests and

my name removed throughout the text.  This will no longer happen.  As of July

10, 1996, this is copyrighted work.  All rights are reserved and I will

prosecute.  If you want to sell it, fine.  I don't ask for any of the profits.

All I ask is you leave the entire text intact with my name as it appears.

Also, Dr. Who has been crying about the fact I removed a little

anecdotal story from the beginning of his portion in the bible.  He released

version 2.3 and put the story back in.  Since it was not pertinent to cellular,

it will not be included.  I have given him credit for what I have used and I

am within my legal right to publish his little section provided I give him

credit (which I have).  I do find it very amusing that he failed to give me

any credit on his web page, considering his Motorola section is just my bible

in HTML format...and I didn't cry.  (Hah!)

G*E*N*E*R*A*L***U*S*E*R***I*N*F*O******************************S*E*C*T*I*O*N***2

Before going in to the programming of the cellular phone, it is

important for the user to know the normal things necessary for day to day

operation.  While the majority of the stuff in the users manual is intended

for people that have problems programming their VCR, their are a few things

that are very important and are only mentioned in the users manual.

Turn On or Off: [Pwr]

Place Call: Enter number, [Snd]

Receive Call: [Snd] or open flip fone

End Call: [End] or close flip fone

Automatic Redial: To retry call, press [Snd] before ending the

unsuccessful call attempt.

Store Number: Phone number, [Sto], 2-digit location number

or press [*] to autoload.

If phone is Alphanumeric, display will flash

"Alpha_".  Press [Sto] to bypass or use

keypad to enter letter (up to 14).  [#]

advances to next position, [*] backspaces.

When finished, press [Sto].

Recall Number: [Rcl], 2-digit location number.  Entering

location 01, then hitting [#] will allow

scrolling through all stored locations.

Recall Name: Press [Name/Menu], [#] or [*] to scroll.

[Rcl] will display location number then name,

[Rcl] again will display corresponding phone

number and [Snd] to dial.

Recalling Last 10 Numbers Called: [Rcl], [0], [0].  Display will show "L0'"

followed by the name or number of the most

recently placed call.  Use [#] or [*] to

scroll and [Snd] to call displayed number.

Super Speed Dialing: Directory location number, [Snd]

Changing Entries: Press [Rcl] and the 2-digit location number

so that the number to be changed is

displayed.  Press and release [Clr] to back

out each of the digits. Enter a new number

and press [Sto].

Call Number Displayed: [Snd]

Microphone Muting: Press [Fcn], [6].

To unmute, press [Fcn], [6]

Lock Unit: [Fcn], [5] or [LOCK]

Unlock: Three digit unlock code.  If you make an

error, [Clr] and enter again.

Automatic Lock: [FCN], [6] (not all phones)

"EnAbLE" will appear if compatible.

Display Unlock Code: Press [Fcn], [0], your six-digit security

code, [Rcl].

Changing Your Unlock Code: Press [Fcn], [0], your six-digit security

code, your NEW 3-digit unlock code, [Sto].

Review Battery Meter: Press [Fcn], [4] and release.

Adjust Volume: Earpiece - Press and hold [Vol] to increase.

Release, press again to decrease.

Ringer - [Fcn], then Vol as above.

Recall Own Phone Number: [Rcl], [#]

Individual Call Timer: [Rcl], [#], [#]

Resettable Call Timer: [Rcl], [#], [#], [#]

Reset Resettable Call Timer: [Fcn], [0], [7], [Clr]

Cumulative Call Timer: [Rcl], [#], [#], [#], [#]

Access Features: Press [Fcn], [1].  To change features, press

[*] and [#] to scroll and [Clr] to change.

To exit feature menu, press [END].

Review/Scroll Menu Features: Press [*] or [#]

Status Review: [Fcn], [0], [9], [Rcl], [#] or [*] scrolls

messages.  To end press [END].

Changing System Type: Press [Rcl], [*].  Repeatedly press [*]

until the desired system type appears.  To

select press [Sto].

Outgoing Call Restrictions: Press [Fcn], [0], 6-digit security code,

[1], [Sto].  Phone will place calls only

from memory locations 1-10.

To change back to unrestricted dialing

press [Fcn], [0], 6-digit security code,

[4], [Sto].

Switch to Second Phone Number: [Rcl], [#], [Sto]  (Press [Clr] to finish

viewing newly activated number).

Key     1st Press   2nd Press   3rd Press   4th Press

2          A           B           C

3          D           E           F

Alpha             4          G           H           I

Entry             5          J           K           L

Mode              6          M           N           O

7          P           Q           R           S

8          T           U           V

9          W           X           Y           Z

Menu Features:  For a more detailed description or instructions on use, consult

the user manual.  Not all features listed are available on all

models.

Message             Description

1 Min Beep     One Minute Beep

Answer M Key   Multiple Key Answer

Auto Answer    Automatic Answer

C Scan         System Type Selection

Call Screen    Incoming Call Screening

Emrcy Off      Emergency Dialing

Home           System Type Selection

Level 1-6      Call Restrictions (Service Levels)

Lock Auto      Automatic Lock

Name Pref'd    Memory Recall Preference

Pin Active     PIN Code Activation

Pref'd SIDs    System Type Selection

Remind Beeps   Reminder Beep Off

Ringer Silent  Alert Off (Silent Ringer)

Scan A         System Type Selection

Scan AB        System Type Selection

Scan B         System Type Selection

Scan BA        System Type Selection

Scrpd Silent   Silent Scratch Pad

Signal Off     Signal Strength Meter

Silent Keypad  Feedback Tones Off

Std            Features Set as Standard

Theft Alarm    Theft Alarm

Timer Auto     Automatic Display Timer

Vox Mode       Voice Operated Transmission

Vibrat Mode    VibraCall

I would like to add that while I have extensively worked on finding

additional test mode commands, I (nor anyone else) have ever worked with the

normal operation commands as listed above.  For example, above you will

notice sequences with [Fcn], [1] or [Fcn], [0], [7].  This is totally

unexplored territory.  Happy hacking :)  See entering test mode on the new

95xx phones.

--------------------------------------------------------------------------------

Activating the PIN in PIN READY cellfones      Thanks to fringe@rndhse.chi.il.us

(You cannot use their fone without the PIN activated)

Activating the PIN Ready feature in Motorola fones:

1. Enter user menu - press FNC,1 or FNC,Menu

2. Select Pin Active in menu and press * once or until "Pin Active"

appears in the fone display.

3. Enable 'Pin Active' feature - press CLR.

A small square will appear before 'Pin Active' sayings its enabled.

4. Exit user menu - press END

To store the PIN into memory:

1. Enter the specific PIN code - press XXXX

2. Store four digit PIN into memory location 07 - press STO,0,7

3. Return to normal operation - press CLR

To initiate a call using the PIN Ready feature:

1. Enter the phone number you wish to call:

press (XXX)XXX-XXXX, SND

2. You will hear two short rings, then press SND again.  The PIN

Ready feature will automatically send the PIN code you previously

stored into memory location 07 and initiate your call.

---------

What is EE3??

EE3 is the software that Motorola has added to the cellular

product line which provides feature enhancements and increased

security by restricting ESN transfer...

---------

Why did the changes take place...

Due to the FCC Rule change, all new cellular telephones

that were introduced after Jan. 1, 1995 with new FCC IDs must restrict

ESN transfer. Phones introduced prior to this date are "grandfathered"

or not required to be compliant with this rule.

---------

How ya can tell the difference between an EE3 and a non-EE3 fone.

These fones will be identified with the marking of EE3 on the

FCC label (look on da back of da fone)

---------

How does the fone change?!

Some of the changes EE3 phones have are feature differences,

accessory compatibility, and service differences.

---------

Which fones have the PIN CODE feature?!

If the phones has EE3 on the back label, then the fone has the

PIN CODE feature; with the exception of the EE3 TeleTacs and the

pre-AC-P300 boxed contours.

---------

Can a 3-watt VA be used with EE3 fones?!

No, the existing 3-watt VA is not compatible with EE3 fones.

The following part numbers are for the new 3-watt booster for EE3 fones

and for conversion kits for existing 3-watt kits.  The 3-watt booster

has its own handset that comes with it.

Deluxe Booster w/Micro Car Handsfree Kit (EE3 pocket and flip)

# S-5415

Deluxe Booster w/Micro Car Handsfree Kit (EE3 Elite) <no, eleet!>

# S-5093

Deluxe Booster Conversion Kit (EE3 pocket and flip)

# S-5094

Deluxe Booster Conversion Kit (EE3 Elite)

# TBA

-----------

Which Motorola fones have the new Lock feature?!

All EE3 fones have the NEW lock feature - FCN,5 and STO to lock it

--------------------------------------------------------------------------------

Well, it seems a lot of people have been trying to figure out the date of

manufacture of Motorola cellular phones.  This is easily accomplished by

locating the MSN (Mechanical Serial Number) somewhere on the telephone.  It can

be 10 or 11 digits.  If there is no 11th digit, warranty period is one year.  If

there is an 11th digit, the warranty period can be determined from the following

charts.  Without a proof of purchase, warranty date is determined by adding 3

months to the date of manufacture.

MSN Example: 194GSTxxxxW

194 is the Accounting Product Code (APC) and has little use.

G is the location of manufacture

S is the Year Code of manufacture

T is the Month Code of manufacture

xxxx is the actual serial number (hex) of that telephone

W is the Warranty Period

Year Code Year   Month Code Month                  Warranty Period Definition

H     1983     A or B   January          A   1 yr w/ possible MCSI coverage

J     1984     C or D   February         B   3 yrs w/ possible MCSI coverage

K     1985     E or F   March            C   5 yrs w/ possible MCSI coverage

L     1986     G or H   April            D   2 yrs w/ possible MCSI coverage

M     1987     J or K   May              E   No Warranty

N     1988     L or M   June             F   90 Days OEM only

P     1989     N or P   July             H   3 Years, Canada Only

Q     1990     Q or R   August           L   3 Years, OEM Only

R     1991     S or T   September        M   1 Year, OEM Only

S     1992     U or V   October          N, P, X   5 Yrs

T     1993     W or X   November         Q, R, S, Y, Z   3 Yrs

U     1994     Y or Z   December         T   OEM Telephone

V     1995                               U   90 Days (Reconditioned Units)

W     1996                               W   4 Yrs.

X     1997

Y     1998     Location of Manufacture Code     Location

Z     1999                   G                  Libertyville, IL.

P*R*O*G*R*A*M*M*I*N*G***I*N*F*O********************************S*E*C*T*I*O*N***3

NOTES:  Some units have dual NAM's.

The ESN prefix is 130 decimal, 82 hex.

Motorola: 1-800-331-6456

There are MANY different models of Motorola phones sold under various

brand names, if you think it's a Motorola, it probably is.

Determine which access sequence to use:

HAND HELD PORTABLE MODELS

If the phone has a FCN button and no MENU button use sequence 1.

If the phone has no FCN button use sequence 2.

If the phone has a MENU button and a FCN button use sequence 4.

INSTALLED MOBILE PHONES AND TRANSPORTABLE MODELS

If the phone has no FCN button and no RCL button use sequence 3.

If the phone has a FCN button use sequence 4.

If the phone has a MEM button use sequence 5.

If the phone has a RCL button and no FCN button use sequence 6.

SEQUENCE#     ACCESS CODE

1             FCN   (SECURITY CODE TWICE) RCL

2             STO # (SECURITY CODE TWICE) RCL

3             CTL 0 (SECURITY CODE TWICE) *

4             FCN 0 (SECURITY CODE TWICE) RCL

5             FCN 0 (SECURITY CODE TWICE) MEM

6             CTL 0 (SECURITY CODE TWICE) RCL

The default security code is 000000.  The CTL (control) button is the

single black button on the side of the handset.

NAM programming:

1.  Turn the power on.

2.  Within ten seconds enter the access sequence as determined above.

3.  The phone should now show "01" in the left of the display, this is the

first programming entry step number.  If it does not the security code

is incorrect, or the programming lock-out counter has been exceeded.  In

either case you can still program the unit by following the steps under

TEST MODE PROGRAMING below.

4.  The * key is used to increment each step:

Each time you press * the display will increment from the step number,

displayed on the left, to the data stored in that step, displayed on

the right.  When the data is displayed make any necessary changes and

press * to increment to the next step number.

5.  The SND key is used to complete and exit programming when any STEP

NUMBER is displayed.

If you have enabled the second phone number bit in step 10 below then

pressing SND will switch to NAM 2.  Steps 01 thru 06, 09 and 10 will

repeat for NAM 2, the step number will be followed by a "2" to indicate

NAM two.

5.  The CLR key will revert the display to the previously stored data.

6.  The # key will abort programming at any time.

PROGRAMING DATA:

STEP#    #OF DIGITS/RANGE    DESCRIPTION

01       00000 - 32767       SYSTEM ID

02       3 DIGITS            AREA CODE

03       7 DIGITS            TEL NUMBER

04       2 DIGITS            STATION CLASS MARK

05       2 DIGITS            ACCESS OVERLOAD CLASS

06       2 DIGITS            GROUP ID (10 IN USA)

07       6 DIGITS            SECURITY CODE

08       3 DIGITS            LOCK CODE

09       0333 OR 0334        INITIAL PAGING CHANNEL

10       6 DIGIT BINARY      OPTION PROGRAMING (SEE NOTE 1)

11       3 DIGIT BINARY      OPTION PROGRAMING (SEE NOTE 2)

NOTES:

Take care with Motorola's use of "0" and "1".  Some options use "0" to

enable, some use "1".

1.  This is a 6 digit binary field used to select the following options:

Digit 1:  Internal handset speaker, 0 to enable.

Digit 2:  Local Use Mark, 0 or 1.

Digit 3:  MIN Mark, 0 or 1.

Digit 4:  Auto Recall, always set to 1 (enabled).

Digit 5:  Second phone number (not all phones), 1 to enable.

Digit 6:  Diversity (Two antennas, not all phones), 1 to enable.

2.  This is a 3 digit binary field used to select the following options:

Digit 1:  Continuous DTMF, 1 to enable.

Digit 2:  Transportable Ringer/Speaker, 0=Transducer, 1=Handset.

Digit 3:  8 hour time out in transportable mode, 0 to enable.

On newer models, they have added and changed some numbers.  The numbers

as of the 3/27/92 manual are as follows:

1.  The 6 digit binary field is still the same.

2.  The 3 digit binary field has become a 5 digit binary field.

Digit 1:  Failed Page Indicator                        1=Disabled;0=Enabled

Digit 2:  Motorola Enhanced Scan                       1=Enabled; 0=Disabled

Digit 3:  Long Tone DTMF                               1=Enabled; 0=Disabled

Digit 4:  Transportable Internal Ringer Speaker        1=Handset; 0=Transdcr

Digit 5:  Eight Hour Timeout                           1=Disabled;0=Enabled

T*E*S*T***M*O*D*E**********************************************S*E*C*T*I*O*N***4

TEST MODE ACCESS:

NEWER 95xx PHONES (Thank you Motorola!!!)

Many newer phones don't require grounding.  If your software version number

is 9526 (I think) or newer, enter this:

[FCN] [0] [0] [*] [*] [8] [3] [7] [8] [6] [6] [3] [3] [STO]

In case you have trouble remembering the number sequence, it spells out

"TESTMODE."  Leave it to Motorola to make this easier and easier all the time.

I have used this and it does work.  This command just backs up my claim even

further that esn changing via handset is a reality.  It's a matter of finding

the correct combination of keys.

Normal test mode commands work like usual from then on.

For some odd reason, this hasn't been included in all the 95xx phones.  I

believe they started it in Software 9526.  This is only an estimate, so if

you have a 95xx flip, let me know what software version you have and whether

it works or not so this date can be isolated.  Mine is a 9562 that worked.

INSTALLED MOBILE PHONES AND TRANSPORTABLE MODELS

To enter test mode on units with software version 85 and higher you must

short pins 20 and 21 of the transceiver data connector.  An RS232 break out

box is useful for this, or construct a test mode adapter from standard

Radio Shack parts.

For MINI TR or Silver Mini Tac transceivers (smaller data connector) you

can either short pins 9 and 14 or simply use a paper clip to short the

hands free microphone connector.

HAND HELD PORTABLE MODELS:

There are two basic types of Motorola portable phones, the Micro-Tac series

"Flip" phones, and the larger 8000 and Ultra Classic phones.  Certain newer

Motorola and Pioneer badged Micro-Tac phones do not have a "flip", but

follow the same procedure as the Micro-Tac.

8000 & ULTRA CLASSIC SERIES:

If you have an 8000 series phone determine the "type" before trying to

enter test mode.  On the back of the phone, or on the bottom in certain

older models, locate the F09... number this is the series number.  If the

FOURTH digit of this number is a "D" you CAN NOT program the unit through

test mode, a Motorola RTL4154/RTL4153 programmer is required to make any

changes to this unit.

Having determined that you do not have a "D" series phone the following

procedure is used to access test mode:

Remove the battery from the phone and locate the 12 contacts at the top

near the antenna connector.  These contacts are numbered 1 through 12 from

top left through bottom right.  Pin 6, top right, is the Manual Test Mode

Pin. You must ground this pin while powering up the phone.  Pin 7 (lower

left) or the antenna connector should be used for ground.  Follow one of

these procedures to gain access to pin 6:

1.  The top section of the battery that covers the contacts contains

nothing but air.  By careful measuring you can drill a small hole in the

battery to gain access to pin 6, alternately simply cut the top off the

battery with a hack saw.  Having gained access use a paper clip to short

pin six to the antenna connector ground while powering up the phone.

2.  If you do not want to "destroy" a battery you can apply an external 7.5

volts to the + and - connectors at the bottom of the phone, ground pin 6

while powering up the phone as above.

3.  You can also try soldering or jamming a small jumper between pins 6 and

7 (top right to lower left), or between pin 6 and the antenna connector

housing ground.  Carefully replace the battery and power up the phone.  Use

caution with this method not to short out any other pin.

4.  A cigarette lighter adapter, if you have one, also makes a great test

mode adapter as it can be disassembled to give you easier access to pin 6.

Many are pre marked, or even have holes in the right location.  This is

because they are often stamped from the same mold that the manufacturer

uses for making hands free adapter kits and these kits require access to

the phone's connectors.

ULTRA CLASSIC II SERIES:

Ground Pin 2 to pin 4.

MICRO-TAC "FLIP" SERIES:

This phone follows similar methods as outlined for the 8000 series above.

Remove the battery and locate the three contacts at the bottom of the

phone, the two outer contacts are raised and connect with the battery.  The

center contact is recessed, this is the Manual Test Mode connector.

Now look at the battery contacts, the two outer ones supply power to the

phone, the center contact is an "extra" ground.  This ground needs to be

shorted to the test mode connector on the phone.  The easiest way to do

this is to put a small piece of solder wick, wire, aluminum foil or any

other conductive material into the recess on the phone.  Having done this

carefully replace the battery and turn on the power, if you have been

successful the phone will wake up in test mode.

GENERAL NOTES:

HANDSETS:  Most Motorola handsets are interchangeable, when a handset is

used with a transceiver other than the one it was designed for the display

will show "LOANER".  Some features and buttons may not work, for instance

if the original handset did not have a RCL or STO button, and the

replacement does, you will have to use the control * or control # sequence

to access memory and A/B system select procedures.

LOCK/UNLOCK PROCEDURES:

Phones with "LOCK" buttons:  Press lock for at least 1/2 a second.

Phones with a "FCN" button:  Press FCN 5, note that 5 has the letter's

"J,K, and L" for lock.

Phones with no FCN or LOCK button:  Press Control 5, control is the black

volume button on the side of the

handset.

SYSTEM SELECT PROCEDURES:

Phones with a RCL button:  Press RCL *, then * to select, STO to store.

Phones with no RCL button: Press Control * then * to select, # to store.

Options are:    CSCAn:    Preferred/Non preferred with system lockout.

Std A/b, or Std b/A:  Preferred/Non preferred.

SCAn Ab, or SCAn bA:  Non preferred/Preferred

SCAn A:  "A" ONLY

SCAn b:  "B" ONLY

HOME:     Home only

Pref'd SIDS:  Service within the entered SIDS (up to 15)

(these are typical options, some phone's vary.  C-Scan is only available

on newer models and does not appear unless programmed, see below.)

--------------------------------------------------------------------------------

TEST MODE

NOTE: Not all commands work on all telephones.  If a command is not valid the

display will show "ErrOr."  Not all numbers have been assigned.  Not all

numbers have been listed here.  Some commands were intended only for

Motorola factory applications.   (This is the disclaimer in the

technical training manual.  I have included all of the other commands I

have discovered one way or another.  Some that say no function do have

a function but it is unknown until it is figured out.)

Three test commands are significant for programming and registering the

the telephone for service:  see full descriptions under TEST MODE COMMANDS.

32#  Clears the telephone.  (Older Motorola allowed either three or fifteen

changes in the MIN.  After that, the phone had to be sent to Motorola to reset

the counter.  This is the command they use.)

38#  Displays the ESN

55#  This is the TEST MODE PROGRAMMING (as described below).

TEST MODE DISPLAY:

Test mode consists of two separate levels.  When the telephone is first placed

in Test Mode, it is in the STATUS DISPLAY LEVEL.  The display will be scrolling

(or flashing), or it will be locked.  If locked, enter the unlock code and the

display will begin scrolling.  If the unlock code is not known, press #.  By

pushing the # key, the technician causes the cellular telephone to change to

its SERVICING LEVEL.  The display will be   US'  .  There are five types of

display, depending on the model of the telephone: a 16 character display, a 14

character display, a 10 digit display (with two versions), an 8 character

display, and a 7 character display.  The status display is different in analog

operation than in a TDMA call.

NOTE: Use of a loaner handset is allowed in servicing level, but may not be

allowed in the status display level.  A locked telephone will not show the

status display, but will enter the servicing level.

14 Character Analog Call Display                  14 Character TDMA Call Display

+---------------+                                 +---------------+

| A B C D E F G |                                 | A B C D E F G |

| H I J K L M N |                                 | H I J K L M N |

+---------------+                                 +---------------+

ABC = Channel                                     ABC = Channel ("A" in the

D   = *Call Processing Mode                             position indicates a

EFG = RSSI                                              channel above 1000)

H   = **(D)SAT                                    D   = *Call Processing Mode

I   = 1=TX on                                     EFG = RSSI

J   = 1=Signalling Tone On                        HIJ = Digital Verification

K   = Power Level (0-7)                                 Color Code (1-255)

L   = 1=Control Channel                           K   = Power Level (0-7)

M   = 1=RX Audio off                              L   = 1=TX on

N   = 1=TX Audio off                              M   = 1=Bit Error Rate (0-7)

N   = 1=Audio Muted

*Call Processing Mode:                            *Call Processing Mode

BLANK = AMPS                                      BLANK = AMPS

A = NAMPS High Sub-Channel                        1 = Slot 1, half rate

B = NAMPS Center Sub-Channel                      2 = Slot 2, half rate

C = NAMPS Low Sub-Channel                         3 = Slot 3, half rate

4 = Slot 4, half rate

**(D)SAT:                                         5 = Slot 5, half rate

0 = 5970 Hz                                       6 = Slot 6, half rate

1 = 6000 Hz                                       7 = Slot 1, full rate

2 = 6030 Hz                                       8 = Slot 2, full rate

3 = No SAT                                        9 = Slot 3, full rate

------------

0-6 = DSAT Vector                                 NOTE:  The Analog Call

7 = No DSAT                                              Display will always

show when on a control

channel.

--------------------------------------------------------------------------------

#    Enter SERVICING LEVEL from Test Command Mode.

00#  no function

01#  Restart (Re-enter DC power start-up routine.)  On TDMA telephones, this

command has the same effect as 13#.

02#  Display Current Telephone Status (This is a non-altering version of the

STATUS DISPLAY.  On a 14 character display, all the information is shown.

On a 7 character display only the information on the second line of a 14

character display is shown.  On a 10 character display, all the

information on the second line of a 14 character display plus the last

three characters of the first line are shown.)

STATUS DISPLAY, ALTERNATES BETWEEN:

AAA BBB   AAA = Channel Number (decimal)    BBB = RSSI reading for channel

CDEFGHI   are as follows:

C = SAT frequency (0=5970, 1=6000, 2=6030, 3=no channel lock)

D = Carrier (0=off, 1=on)

E = Signalling tone (0=off, 1=on)

F = Power attenuation level (0 through 7)

G = Channel mode (0=voice channel, 1=control channel)

H = Receive audio mute (0=unmuted, 1=muted)

I = Transmit audio mute (0=unmuted, 1=muted)

Press * to hold display and # to end.

03#  Reset Autonomous Timer.   This command results in the reset of the

autonomous timer but does not provide any test function on these models.

04#  Initializes Telephone to Standard Default Conditions:

Carrier Off, Power Level 0, Receiver Audio Muted, Transmit Audio Muted,

Signalling Tone Off, SAT Off, Resetting of Watch-Dog Timer Enabled,

DTMF and Audio Tones Off, Audio Path Set to Speaker

05#  TX Carrier On (Key Transmitter)

06#  TX Carrier Off

07#  RX Audio Off (Mute Receiver Audio)

08#  RX Audio On (Unmute Receiver Audio)

NOTE:  It seems Motorola finally discovered that people were using

this command to eaves-drop on cellular converstions.  On MOST

of the newer phones (95xx and up), this command when used

with 11xxxx# will only work with the following channels:

11xxxx#     Tower Freq.  Mobile Freq. Channel

#300      Tx  879.00   Rx  834.00    0300

#333      Tx  879.99   Rx  834.99    0333

#385      Tx  881.55   Rx  836.55    0385

#799      Tx  893.97   Rx  848.97    0799

#991      Tx  869.04   Rx  824.04    0800

09#  TX Audio Off

10#  TX Audio On

11(Ch.No.)# Set Transceiver to Channel xxxx (Receive and Transmit in Decimal;

accepts 1, 2, 3, or 4 digits)

see Section 9 for detailed information on this command

12x# Set Power Step to x; (0,1-7) 0=Maximum Power (3 Watts) 7=Minimum Power Out

13#  Power Off (Shuts off the radio)

14#  10 kHz Signalling Tone On

15#  10 kHz Signalling Tone Off

16#  Setup  (Transmits a five word RECC message; each of the five words will

be "FF00AA55CC33."  Transmitter de-keys at the end of the message.)

17#  Voice  (Transmits a two word REVC message; each of the two words will be

"FF00AA55CC33."  Transmitter de-keys at the end of the message.)

18#  C-Scan  (Allows for entry of as many as 5 negative SID's for each NAM.)

Newer Motorola phones are equipped with a feature called C-Scan, this is

an option along with the standard A/B system selections.  C-Scan allows

the phone to be programmed with up to five inhibited system ID's per NAM.

This is designed to prevent the phone from roaming onto specified non-home

systems and therefore reduce "accidental" roaming fees.

1.  C-Scan can only be programmed from test mode, power phone up with the

relevant test mode contact grounded (see above).

2.  Press # to access test mode.

3.  Press 18#, the phone will display "0    40000".

4.  Enter the first inhibited system ID and press *.

Continue to enter additional system ID's if required.  After the 5th entry

the phone will display "N2". Press * to continue and add system ID's for

NAM 2 as required.

5.  If an incorrect entry is made (outside the range of 00000-32767) the

display will not advance, press CLR and re-enter.  Use a setting of

40000 for any un-needed locations.

6.  When the last entry has been made press * to store and press # to exit,

turn off power.

or

[**Phones without the C-Scan option used this command to SEND NAM.**]

18#  SEND NAM.  Display shows AA BB.  Where AA=Address and BB=Data.  Displays

the contents of the NAM, one address at a time, advanced by pressing the

* key.  The following data is contained in NAM.  The test is exited by

depressing the # key.

SIDH                 Sec. Code

OPT. (1,2,&3)        MIN

MIN1, MIN2           FCHNA

SCM                  FCHNB

IPCH                 NDED

ACCOLC               CHKSUM       GIM

19#  Display Software Version Number  (4 digits displayed as year and week)

NOTE:  Entering commands 20# through 23# or 27# causes the transceiver to begin

a counting sequence or continuos transmission as described below.  In

order to exit from the commands to enter another test command, the #

key must be depressed; all other key depressions are ignored.

20#  Receive control channel messages counting correctable and uncorrectable

errors.  When the command starts, the number of the command will be

displayed in the upper-right corner of the display.  Entering a # key

will terminate the command and display two three-digit numbers in the

display.  The first number is the number of correctable errors and the

second is the uncorrectable errors.

21#  Received voice channel messages counting correctable and uncorrectable

errors.  When the command starts, the number of the command will be

displayed in the upper right-hand corner of the display.  Entering a #

key terminates the command and will display two three-digit numbers in

display.  The first is the number of correctable errors and the second

is the uncorrectable errors.

22#  Receive control channel messages counting word sync sequence.  When the

command starts, the number of the command will be displayed in the upper

right-hand corner of the display.  Entering a # key will terminate the

command and display the number of word sync sequences in the display.

23#  Receive voice channel messages counting word sync sequences.  When the

command starts, the number of the command will be displayed in the upper

right-hand corner of the display.  Entering a # key will terminate the

command and display the number of word sync sequences in the display.

24#  Receive control channel data and display the majority voted busy/idle

bit.  0=idle  1=busy

25x# SAT On         When                        x=0, SAT=5970HZ

x=1, SAT=6000HZ

x=2, SAT=6030HZ

26#  SAT Off

27#  Transmit Data  (Transmits continuous control channel data.  All words

will be "FF00AA55CC33."  When the command starts, '27' will be displayed

in the right side of the display.  Entering a # key will terminate the

command.  The transmitter de-keys when finished.)

28#  Activate the high tone (1150 Hz +/- 55 Hz)

29#  De-activate the high tone

30#  Activate the low tone (770 Hz +/- 40 Hz)

31#  De-activate the low tone

32#  Clear (Sets non-volatile memory to zeroes or factory default.  This

command will affect all counters, all repertory memory including the last

number called stack, and all user programmable features including the

setting of System Registration.  It does not affect the ESN, NAM, phasing

data, or lock code.  This takes a minute or so.  DO NOT TURN OFF THE

TELEPHONE WHILE THIS IS SHOWING '32' ON THE DISPLAY.  WAIT UNTIL THE

NORMAL SERVICE LEVEL DISPLAY RESUMES! [maybe a minute or so])

33x# Turn on DTMF for x  (1-9, *, 0, #, plus the single tones)

Where x=1    697 Hz + 1209 Hz       14  1150 HZ (not used in cellular)

2    697 Hz + 1336 Hz       15  1209 Hz

3    697 Hz + 1477 Hz       16  1336 Hz

4    770 Hz + 1209 Hz       17  1477 Hz

5    770 Hz + 1336 Hz       18  1633 Hz (not used in cellular)

6    770 Hz + 1477 Hz       19  Turn DTMF off

7    852 Hz + 1209 Hz       20  2087 Hz

8    852 Hz + 1336 Hz       21  2308 Hz

9    852 Hz + 1477 Hz       22  2553 Hz (not used in cellular)

*    941 Hz + 1209 Hz       23  Turn DTMF off

0    941 Hz + 1336 Hz       24  3428 Hz (not used in cellular)

#    941 Hz + 1477 Hz       25  3636 Hz (not used in cellular)

10    697 Hz                 26  4000 Hz (not used in cellular)

11    770 Hz                 27  3555 Hz (not used in cellular)

12    852 Hz                 28  4571 Hz (not used in cellular)

13    941 Hz                 29  Turn DTMF off

Someone Please Check Out 24 thru 28 for accuracy.  I had weak equipment.

34#  Turn DTMF Off

35#  Display RSSI ("D" Series Portable Only)

or

35x# Set Audio Path to x    x=0, V.S.P Microphone (Applies to mobiles only.)

x=1, Speaker

x=2, Alert

x=3, Handset

x=4, Mute

x=5, External Telephone (Applies to Portables Only)

x=6, External Handset (Applies to NEWER Portables)

36nnn# Scan (TDMA Telephones only.  Scans the primary control channels and

attempts to decipher the forward data stream.  The display will show PASS1

if the strongest control channel was accessed, PASS2 if the second

strongest was accessed, and FAIL if no control channel could be accessed.)

(nnn=Scan speed in milliseconds).  Tunes from channel 1 to 666 in order.

Entering a * pauses the scan and displays current Channel Number and

RSSI reading (AAA=Channel Number and BBB=RSSI Reading).  When scan speed

is 300 milliseconds or greater, the current status is displayed during the

scan; when less than 300 milliseconds the status is displayed only during

pause.  Entering * during a pause causes the scan to resume.  Entering #

aborts the scan and leaves the mobile tuned to the current channel.  During

this command only the * and # keys are recognized.

NOTE:  While I haven't heard from ONE single person that this has worked,

Motorola has continued to print this command in all the Technical

Training Books (including the January 96 edition).

37#  Sets Low Battery Threshold.  Usage: #37#x# where x is any number

from 1 to 255.  If set to 1, the Low Battery indicator will come up

when the phone is powered on.  If set to 255, it may never come up.

38#  Display ESN (Displays ESN in four steps, two hexadecimal digits at a time

in a four digit display.  The decimal shows the address, 00 through 03 as

the first two digits, and two digits of the ESN as the last two digits.

Use the '*' to step through the entire hexadecimal ESN.)

Compander OFF ("D" Series Portables)

or

38#  SND-SNM.  Display shows AA BB.  Where AA=Address;BB=Data.  Send the SNM

to the display.  All 32 bytes of the SNM will be displayed, one byte at

a time.  The byte address will be displayed in the upper right-hand

corner and the contents of that address will be displayed in the hex.

The * key is used to step through the address similar to the SEND-NAM

(18#) command.

39#  Compander ON ("D" Series Portables)

or

39#  RCVSU.  Receive one control channel word.  When the word is received it

is displayed in hex.  This command will be complete when a control channel

word is received or when the # key is entered to abort the command.

40#  RCVVC.  Receive one voice channel word.  When the word is received it is

displayed in hex.  This command will be complete when a voice channel

word is received or when the # key is entered to abort the command.

41#  Enables Diversity (On F19CTA... Series only.)

42#  Disables Diversity (On F19CTA... Series only.)

43#  Disable Diversity

USE T/R ANTENNA (On F19CTA... Series only.)

USE R ANTENNA (On D.M.T./ Mini TAC)

44#  Disable Diversity

USE R ANTENNA (On F19CTA... Series only.)

USE T/R ANTENNA (On D.M.T./ Mini TAC)

45#  Display Current Receive Signal Strength Indicator (Displayed as a 3 digit

decimal number) The strongest signal I have ever received was 179 and I

was sitting directly below the tower WITHOUT an external antenna.

46#  Display Cumulative Call Timer

47x# Set RX Audio level to X

(For F19CTA ...Series Transceivers)

X=0, Lowest Volume

X=6, Highest Volume

X=7, mute

Normal setting is 4.

(For D.M.T./ Mini TAC Transceivers)

X=0, Lowest Volume

X=7, Highest Volume

Normal setting is 4.

(For TDMA Transceivers and F09F... Series and Higher Portables)

X=0, Lowest Volume

X=15, Highest Volume

Normal setting is 2 to 4.  (On TDMA

Transceivers and Micro TAC portables,

settings 8 through 15 are for DTMF

applications only.)

48#  Side Tone On.  Use this command in conjunction with 350# to test the

entire audio path in hands-free applications.

49#  Side Tone Off

50#  Maintenance data is transmitted and test results displayed:

PASS=received data is correct

FAIL 1=2second timeout, no data rec.

FAIL 2=received data is incorrect

51#  Test of mobile where maintenance data is transmitted and looped back.

Display is as follows:

PASS=looped-back data is correct

FAIL 1=2 second timeout, no looped-back data

FAIL 2=looped-back data is incorrect

52x# SAT Phase Adjustment.  A decimal value that corresponds to phase shift

compensation in 4.5 degree increments.  Compensation added to inherent

phase shift in transceiver to achieve a total of 0 degrees phase shift.

Do NOT enter any values except those shown below.

0 degrees = 0          121.5 degrees = 59           243.0 degrees = 86

4.5       = 1          126.0         = 60           247.5         = 87

9.0       = 2          130.5         = 61           252.0         = 112

13.5      = 3          135.0         = 62           256.5         = 113

18.0      = 4          139.5         = 63           261.0         = 114

22.5      = 5          144.0         = 40           265.5         = 115

27.0      = 6          148.5         = 41           270.0         = 116

31.5      = 7          153.0         = 42           274.5         = 117

36.0      = 16         157.5         = 43           279.0         = 118

40.5      = 17         162.0         = 44           283.5         = 119

45.0      = 18         166.5         = 45           288.0         = 120

49.5      = 19         171.0         = 46           292.5         = 121

54.0      = 20         175.5         = 47           297.0         = 122

58.5      = 21         180.0         = 64           301.5         = 123

63.0      = 22         184.5         = 65           306.0         = 124

67.5      = 23         189.0         = 66           310.5         = 125

72.0      = 48         193.5         = 67           315.0         = 126

76.5      = 49         198.0         = 68           319.5         = 127

81.0      = 50         202.5         = 69           324.0         = 104

85.5      = 51         207.0         = 70           328.5         = 105

90.0      = 52         211.5         = 71           333.0         = 106

94.5      = 53         216.0         = 80           337.5         = 107

99.0      = 54         220.5         = 81           342.0         = 108

103.5     = 55         225.0         = 82           346.5         = 109

108.0     = 56         229.5         = 83           351.0         = 110

112.5     = 57         234.0         = 84           355.5         = 111

117.0     = 58         238.5         = 85           360.0         = 70

53#  Enable scrambler option, when equipped.

54#  Disable scrambler option, when equipped.

55#  Display/Program N.A.M. (Test Mode Programming)

TEST MODE PROGRAMING:

Assuming you have completed one of the above steps correctly the phone

will wake up in test mode when you turn the power on.  When you first

access test mode the phone's display will alternate between various status

information that includes the received signal strength and channel number.

The phone will operate normally in this mode.  You can now access Service

Mode by pressing the # key, the display will clear and a ' will appear.

Use the following procedure to program the phone:

1.  Enter 55# to access programming mode.

2.  The * key advances to the next step.  (NOTE that test mode programming

does NOT have step numbers, each time you press the * key the phone

will display the next data entry).

3.  The CLR key will revert the display to the previously stored data.

4.  The # key aborts programming at any time.

5.  To complete programming you must scroll through ALL entries until a '

appears in the display.

6.  Note that some entries contain more digits than can be displayed by the

phone, in this case only the last part of the data can be seen.

TEST MODE PROGRAMING DATA: For AMPS and NAMPS Cellular Telephones

STEP#    #OF DIGITS/RANGE    DESCRIPTION

01       00000 - 32767       SYSTEM ID

02       8 DIGIT BINARY      OPTION PROGRAMING, SEE NOTE 1 BELOW

03       10 DIGITS           MIN (AREA CODE & TEL#)

04       2 DIGITS            STATION CLASS MARK, SEE NOTE 2 BELOW

05       2 DIGITS            ACCESS OVERLOAD CLASS

06       2 DIGITS            GROUP ID (10 IN USA)

07       6 DIGITS            SECURITY CODE

08       3 DIGITS            UNLOCK CODE

09       3 DIGITS            SERVICE LEVEL, SEE NOTE 3 BELOW

10       8 DIGIT BINARY      OPTION PROGRAMING, SEE NOTE 4 BELOW

11       8 DIGIT BINARY      OPTION PROGRAMING, SEE NOTE 5 BELOW

12       0333 OR 0334        INITIAL PAGING CHANNEL

13       0333                "A" SYSTEM IPCH

14       0334                "B" SYSTEM IPCH

15       3 DIGIT             NUMBER PAGING CHANNEL (021 IN USA)

16       8 DIGIT BINARY      OPTION PROGRAMING, SEE NOTE 6 BELOW

Steps 01 through 06 and 12 will repeat for NAM 2 if the second phone

number bit has been enabled in step 11.

TEST MODE PROGRAMING DATA: For TDMA Cellular Telephones

STEP#    #OF DIGITS/RANGE    DESCRIPTION

01       00000 - 32767       SYSTEM ID

02       8 DIGIT BINARY      OPTION PROGRAMING, SEE NOTE 1 BELOW

03       10 DIGITS           MIN (AREA CODE & TEL#)

04       2 DIGITS            STATION CLASS MARK, SEE NOTE 2 BELOW

05       2 DIGITS            ACCESS OVERLOAD CLASS

06       2 DIGITS            GROUP ID (10 IN USA)

07       6 DIGITS            SECURITY CODE

08       3 DIGITS            LOCK CODE

09       3 DIGITS            SERVICE LEVEL, SEE NOTE 3 BELOW

10       8 DIGIT BINARY      OPTION PROGRAMING, SEE NOTE 4 BELOW

11       8 DIGIT BINARY      OPTION PROGRAMING, SEE NOTE 5 BELOW

12       0333 OR 0334        INITIAL PAGING CHANNEL

13       0333                "A" SYSTEM IPCH

14       0334                "B" SYSTEM IPCH

15       3 DIGITS            DEDICATED PAGING CHANNELS (021 IN USA)

16       3 DIGITS            SECONDARY INITIAL PAGING CHANNEL.  708 for

system A, 737 for system B.  Allows the TDMA

telephone to be assigned to a TDMA channel in

a call

17       708                 SECONDARY INITIAL PAGING CHANNEL FOR SYSTEM A

18       737                 SECONDARY INITIAL PAGING CHANNEL FOR SYSTEM B

19       8 DIGITS            OPTION PROGRAMMING, SEE NOTE 6 BELOW

NOTES:

Take care with Motorola's use of "0" and "1".  Some options use "0" to

enable, some use "1".

These are eight digit binary fields used to select the following options:

1.  (step 02 above, suggested entry is: 11101001 for "A" system, 10101001

for "B" sys)

Digit 1: Local use mark, 0 or 1.

Digit 2: Preferred system, 1=system A, 0=system B.

Digit 3: End to end (DTMF) dialing, 1 to enable.

Digit 4: Not used, enter 0. Formerly used for test mobile.

Digit 5: Repertory (speed) dialing, 1 to enable. (Not used in TDMA)

Digit 6: Auxiliary (horn) alert, 1 to enable.

Digit 7: Hands free (VSP) auto mute, 1 to enable (mutes outgoing hands

free audio until the MUTE key is pressed). (Not used in TDMA)

Digit 8: Min mark, 1 = Enabled.  NOT CHANGEABLE on series II or III.

2.  Station Class Mark

SCM | 666 or 832 Ch. | VOX | Max Power

-----+----------------+-----+-----------

00 |      666       |  N  |   3.0 W

01 |      666       |  N  |   1.2 W

02 |      666       |  N  |   0.6 W

03 |                |     |

04 |      666       |  Y  |   3.0 W

05 |      666       |  Y  |   1.2 W

06 |      666       |  Y  |   0.6 W

07 |                |     |

08 |      832       |  N  |   3.0 W

09 |      832       |  N  |   1.2 W

10 |      832       |  N  |   0.6 W

11 |                |     |

12 |      832       |  Y  |   3.0 W

13 |      832       |  Y  |   1.2 W

14 |      832       |  Y  |   0.6 W

15 |                |     |

3.  Service Level Codes:

001   The telephone will only dial numbers in memory locations 01, 02

and 03.  No keypad entries or memory storage is possible.

Restrict ALL outgoing calls by clearing locations 01, 02, and 03

and place the phone in servicing level 001.  In some phones this

applies to memory locations 01 - 10.

002   The telephone will dial only numbers from memory locations.  The

keypad is disabled and super speed dialing is not enabled.

003   Keypad dial only; no memory recall allowed.

004   Unlimited keypad and memory dialing.  (DEFAULT)

005   Seven-digit dialing only

006   Full keypad and memory dialing, but memory locations 1 through

10 cannot be changed.

007   The phone will dial only from as many as 50 programmable memory

locations

4.  (step 10 above, suggested entry is: 00000100)

Digits 1 - 3: Not used in USA, enter 0.

Digit 4: Extended Field.  When enabled, the telephone will scan

more than 32 paging channels.  Not used in USA, 0 to disable

Digit 5: Single system scan, 1 to enable (scan A or B system only,

determined by bit 2 of step 02. Set to "0" to allow user the

option).

Digit 6: Super speed dial, 1 to enable (pressing N, or NN SND will

dial the number stored in memory location NN).

Digit 7: User selectable service level, 0 to enable (allows user to

set long distance/memory access dialing restrictions).

Digit 8: Lock function, 0 to enable (allows user to lock/un-lock the

phone, if this is set to 1 the phone can not be locked).

5.  (step 11 above, suggested entry is: 00000000)

Digit 1: Handset programming, 0 to enable (allows access to programming

mode without having to enter test mode).

Digit 2: Second phone number (not all phones), 1 to enable.

Digit 3: Call timer access, 0 to enable. (Not used in TDMA)

Digit 4: Auto system busy redial, 0 to enable.

Digit 5: Internal Speaker disable, 1 to enable (use with select VSP

units only, do not use with 2000 series mobiles).

Digit 6: IMTS/Cellular, 1 to enable (rarely used).

Digit 7: User selectable system registration, 0 to enable.

Digit 8: Dual antenna (diversity), 1 to enable.

6.  (step 16 and 19 above, suggested entry is: 0011010 for portable and 0011011

for mobile units)

Digit 1: Enhanced Scan, when enabled, four strongest signalling

channels are scanned instead of two. 1=enabled, 0-disabled.

Digit 2: Cellular Connection, used only in series II phones if a

series I cellular connection is used with a series II.

0=series II, 1=series I, 0 for ALL TDMA PHONES

Digit 3: Continuous DTMF, 1 to enable (software version 8735 and later)

Digit 4: Transportable Internal Ringer/Speaker.  When set to 0, audio

is routed to the external speaker of the transportable; 1

routes it to the handset.

Digit 5: 8 hour time-out, 0 to enable (software version 8735 and later)

Digit 6: Not used, 0 only.

Digit 7: Failed page indicator, 0 to enable (phone beeps when an

incoming call is detected but signal conditions prevent

completion of the call).

Digit 8: Portable scan, 0 for portable, 1 for mobile units.

56#  Illumination Diagnostic.  Lights up all lights (except the green in use

light) and displays all "8"'s.  The phone is also muted until repowered.

57x# Call Processing Mode

x=0, AMPS

x=1, NAMPS

x=2-4, RESERVED

x=5, TDMA signalling

x=6, TDMA signalling with loopback before decoding

x=7, TDMA signalling with loopback voice after decoding

x=8, TDMA signalling with loopback FACCH after decoding

x=9, TDMA forced synchronization

58#  Compander On (Audio compressor and expander) (See 39#)

59#  Compander Off (Audio compressor and expander) (See 38#)

60#  no function

61#  ESN Transfer (For Series I D.M.T./Mini TAC only)

62#  Turn On Ringer Audio Path

63#  Turn Off Ringer Audio Path

64#  ? Does something, doesn't display anything

65#  ? Does something, doesn't display anything

66#  Identity Transfer (Series II Trancvrs and later mobiles, F09HG...,

F09HL..., F09HY..., F09HR..., F09LF..., F09NF..., F09PY..., F09QY...,

F09RY..., and most retail portables shipping prior to April 1, 1995.)

Does the actual transfer of the ESN and NAM info.  See the 80x# command.

67#  Displays two 3 digit numbers.  If you keep entering this command

repeatedly, the first number will constantly change, the second won't

(as far as I have seen).

68#  Display FLEX and Model Information

69#  Used with 66# for Identity Transfer.  In models shipped without the 66#

command, this is used with 80x# instead.  Reads NAM information, repertory

memory, and C-Scan ID SID's from old phone.

70#  Abbreviated field transmitter audio deviation command, for transceivers

with FCC ID ABZ89FT5668.

71#  Abbreviated field power adjustment command, for transceivers with FCC ID

ABZ89FT5668.

72#  Field audio phasing commands.  The left side of the display should read

"00" followed by a two digit number.  The "00" indicates the first

programming step.  If you press the *, the 00 changes to 01 and so on until

08.  The "06" and "0A" are used to change the audio level (to change:

press the volume up or down keys).  Other registers...don't know.

73#  Field power adjustment command.

75#  Display ESN (Displays ESN in four steps, two decimal digits at a time

in a three digit display.  The decimal shows the address, 0 through 3 as

the first digit, and two digits of the ESN as the last two digits.

Use the '*' to step through the entire hexadecimal ESN.)

38#                  75#

00-D4              0       30

01-25              1       16

02-D4              2       13

03-1E              3       54309

80x# Current Identity Transfer Procedure.  (Available in telephones shipping

after April 1, 1995.)  This does NOT transfer the ESN.

x=0, Transfers NAM information (On TDMA telephones, this

command also transfers C-Scan SID's.)

x=1, Transfers repertory memory (names & telephone numbers

in memory.)

x=2, Transfers C-Scan SID's on analog telephones (Not

available on TDMA telephones.)

NOTES:  As new fones come out, more commands are added/deleted as needed.

The majority of these commands were figured using VERY old software

versions.  Some commands won't work on some phones.  If you find a

command that does something, please inform me as well as the software

version number of the phone it was discovered on.

--------------------------------------------------------------------------------

* NEW SECTION *

COMMANDS THAT DO SOMETHING BUT I DON'T KNOW WHAT!!!

74#

76#

77#

78#

99#

If you have any insight to these commands or if you have any more to add to the

list, please email me promptly.  Thank you.

G*S*M***I*N*F*O***A*N*D***T*E*S*T***M*O*D*E********************S*E*C*T*I*O*N***5

Thanks to Janus Krarup (jckrarup@post1.com    http://www.ibt.dk/morten/friends/)

for all the information in this section.  His web page has all of this

information (and more) and is continually updated.  It's also offered in Italian

and will be available soon in Czech.

TESTMODE for GSM PHONES:

Start with the sequence:

Testmode

Command#

|

|

[pause] [pause] [pause] [1] [1] [3] [pause] [1] [pause] [ok] [MENU]    (pause is

accomplished by holding down the [*] key until box appears).

This accomplishes the editing/viewing of the Engineer Field Options.  By

substituting different numbers for the [1] [1] [3], many more test mode

commands and functions are available.  (see below for an almost complete list)

After pressing [MENU], scroll until the new option appears.  Pressing [OK] will

then reveal 3 new options:

ACTIVE CELL: Displays Active Channel and lets you scroll through: RxLev, RxLevAM,

NCC, BCC, MSTxPwr, and C1.

During a call you can see: RxLev, RxLevFull, RxLevSub, RxQualFul,

RxQualSub, Timeslot, TimeAdv, and PwrLev.

The active channel may read "Hopping" during a call.

When establishing a network connection you can see what SDCCH

(Standalone Dedicated Control CHannel) the negotiation takes place

on.

ADJACENT CELLS: Lets you scroll through the adjacent cells (1-6) and view their

channel allocations.

SYSTEM PARAMETERS: Displays the following: Combined, AcsClas, MCC, MNC, LAC,

CellID, T3212, BS-PA-MFRM, and XZQTY.

During a call you can see: Combined, DTX, MCC, MNC, LAC, and

CellID.

--------------------------------------------------------------------------------

Key To The Readouts:

Combined: Haven't got a clue.  The two Danish operators has OFF and Swedish

Comviq has ON.

RxLev: The strength of the received signal (in dBm), normally between -55 and

-90  -  the signal drops at RxLevAm

When comparing RxLev's, remember the logarithmic nature of the dB scale and

that the signal intensity decays by a factor of 4 when the distance from

the BST is doubled (assuming line of sight to the BST, the signal will

drop 6 db when the distance is doubled.)

RxLevAm: Receive signal strength cut off point (around -100 dBm and -110 dBm).

BCC: Broadcast Color Code (0 through 7)

NCC: Network Color Code (0 through 7)

MSTxPwr: BTS transmit power in dBm (see notes on power control)

C1: Measurement of the signal quality (bit failure).  A forced handoff will be

made if C1<0.  ( I have seen C1 as high as 039).  This value is reported

from MS back to the BSC which decides when to make a handoff.

RxLevFull: C1 value with continuous transmission from tower.

RxLevSub: C1 value with discontinuous transmission from tower.

RxQualFull: Bit error rate with continuous transmission from tower.

RxQualSub: Bit error rate with discontinuous transmission from tower.

Timeslot: The current timeslot (0 through 7).  TDMA allows eight channels to be

accommodated on a single RF carrier.

TimeAdv: Timing advance (0 through 63).  TA can be multiplied with 547 meters

(35 km/64 TA steps) to get the distance to the BTS.

PwrLev: Shows which power step/level the phone is transmitting at (see section

below on power control)

DTX: Discontinuous transmission, a feature used to save battery and reduce

network traffic.

MCC: Mobile Country Code.  238=Denmark

MNC: Mobile Network Code.  1=Tele Denmark, 2=Sonofon (these are the ones for

the MCC 238).

LAC: Local Area Code.  (not sure, but I could imagine it being the BSC

identity).

CellID: A unique number that identifies the cell.  (BTS identity)

T3212: Time between periodic network updates.  Appears to be fixed (010 for

Tele Denmark and 050 for Sonofon).

--------------------------------------------------------------------------------

POWER CONTROL:

To minimize co-channel interference and to conserve power, both the mobiles and

the Base Transceiver Stations operate at the lowest power level that will

maintain an acceptable signal quality.  Power levels can be stepped up or down

in steps of 2 db from the peak power for the class down to a minimum of 13 dBm

(20 milliwatts).

Use this table to translate between dBm, PwrLev, and Power:

+--+----+--+---+---+--+---+---+---+----+---+----+----+----+----+----+

Power Level| 0|   1| 2|  3|  4| 5|  6|  7|  8|  9 | 10|  11|  12|  13|  14|  15|

+--+----+--+---+---+--+---+---+---+----+---+----+----+----+----+----+

dBm|43|  41|39| 37| 35|33| 31| 29| 27|  25| 23|  21|  19|  17|  15|  13|

+--+----+--+---+---+--+---+---+---+----+---+----+----+----+----+----+

Watts|16|12.8| 8|5.2|3.2| 2|1.3|0.8|0.5|0.32|0.2|0.13|0.08|0.05|0.03|0.02|

+--+----+--+---+---+--+---+---+---+----+---+----+----+----+----+----+

If you can help out with deciphering the readouts, additional sequences or

corrections please feel free to contact Janus Krarup at jckrarup@post1.com.

--------------------------------------------------------------------------------

Security Codes on the GSM phones:

The phones are protected by two codes:

The UNLOCK code which consists of 4 digits (default 1234)

This is changed by typing MENU 51 and providing the security code.  You can type

in wrong codes until you hit the right one.

The SECURITY code which consists of 6 digits (default 000000)

This is changed by typing MENU 54 and providing the old code.  Again, you can

keep hitting bogus codes all day until you get the right one (hope you got

some time to kill).

The SIM card is protected by 2 codes:

The PIN code (Personal Identification Number) which consists of 4 to 8 digits.

The PIN code can only be typed in wrong 3 times (you heard right...3).  After

this the SIM card will be blocked until the PUK code is used.  The PIN code is

user definable.  It can be changed by providing the old PIN code or the PUK

code.  There is a menu for changing the PIN code by providing the old one, but

you can also type **04* in order to do so.  To enter a whole new PIN on a

blocked card or without knowing the old PIN, you need to use **05* PUK (OK)

NEWPIN (OK) NEWPIN (OK)

The PUK code (Personal Unblocking Key) consists of 8 digits.

This is set by the network provider and can not be changed.  If entered wrongly

10 times, the card will become permanently blocked and will need to be replaced.

--------------------------------------------------------------------------------

Undocumented Menu shortcuts (7500, 8200, and maybe the 5200 and 7200)

Menu + XX

11  Status Review

13  Available networks

14  Preferred networks

21  Incoming call alert

22  Select keypad tones

25  Require SIM card PIN

26  Language selection

32  Repetitive timer

33  Single alert timer

34  Show meter during calls

35  Resettable call meters

36  Resettable call meters

37  Set call cost rates

38  Reset all meters

41  Keypad dialing (call restrictions)

43  Restrict incoming calls

44  Phone number length

51  Change unlock code

52  Master reset

53  Master clear

54  New security code

55  Automatic lock

63  Battery saving mode

--------------------------------------------------------------------------------

Test Mode Functions:

3   MENU-phone book

4   MENU-messages

5   mute on/off (menu during call)

6   MENU-Call related features

7   MENU-"arrows"

8   select phone line

9   last ten calls

10  restrict my phone number

11  call diverting

12  divert when available

13  Detailed diverting

14  Divert voice calls

15  Divert ALL voice calls

16  If Busy

17  If no answer

18  If not reachable

19  Divert fax calls

20  Divert Data calls

21  Cancel all diverting

22  Call waiting

23  Call barring

24  Bar outgoing calls

25  Bar incoming calls

26  Change Barr password

27  Call restrictions

28  Restrict incoming calls

29  Phone number length

30  Keypad dialing

31  Call voicemail

32  Received messages

33  Cell broadcast

34  Message settings

35  Message alert tone

36  Voicemail number

37  Incoming call alert

38  No alert

39  Vibrate only

40  Adjust ring volume

41  Lock now

42  Automatic lock

43  Change unlock code

44  Require SIM card PIN

45  Change SIM PIN code

46  New security code

47  Extended menus

48  Language selection

49  Automatic (Language Selection)_

50  Change greeting

51  Battery saving mode

52  Select keypad tones

53  Phone status

54  Status review

55  Master reset

56  Master clear

57  Available networks

58  Network search

59  Registration preferences

60  Frequency of search

61  Preferred networks

62  Add network to list

63  Show list of networks

64  Find new network

65  Show last call

66  Resettable call timers

67  Set audible call timers

68  Single alert timers

69  Repetitive timer

70  **1 = register write lock!!!

71  Show meter during calls

72  Set meter format

73  Set call cost rates

74  Lifetime meter

81  Find entry by name

82  Find entry by location

83  Show services

84  Enter my phone number

85  Show my phone number

86  Add entry

87  Check phone capacity

88  Check SIM capacity

89  Prevent access

90  Edit name (my phone number)

91  Edit number (my phone number)

92  MENU - Call related features

93  MENU - Phone setup

94  MENU - Network selection

95  MENU - Call meters

99  Periodic search

100 Continuous search

101 Slow search

102 Medium search

103 Fast search

104 MENU - Phonebook

105 MENU - Phonebook

106 MENU - Messages

107 MENU - Messages

108 Copy SIM memory

110 Edit number (direct dial key)

111 Edit name (direct dial key)

112 Direct dial key

113 MENU - Eng Field Options

126 erase all numbers (last 10)

ATTENTION:  These are but a few of the possible commands.  There are 1000

possibilities!  Please attempt to find more.  And if you do, Email

me to keep this list as accurate as possible.

I know that an add-on setup menu exists.  This is used when the phone is

connected to a hands-free car kit.

The HF-setup menu should contain 5 choices: turn off radio/aut. HF answer/aut.

HF/security timer/external alert.

If you have an HF kit, please help me to complete the list (I suspect the menus

to have the numbers from 75 to 80).

Again all correspondence about this entire GSM section should be addressed to

jckrarup@post1.com.  He's the GSM guy.  I've never used a GSM phone.  -MikEY

H*A*C*K*I*N*G***T*H*E***F*O*V*C********************************S*E*C*T*I*O*N***6

Note:  This is NOT my hack.  Thanks to Patrk@delphi.com for this addition.

HACKING THE FOVC

Problem:  When listening to something interesting (a conversation),

just when that sexy sounding horny broad begins to give her

phone number to some lucky guy, HANDOFF!!! then static... DAMN!

Trick:    Hack the FOVC.

a quick definition:  FOVC  = FOward Voice Channel

FOCC  = FOward Control Channel

REVC  = REverse Voice Channel

RECC  = REverse Control Channel

As the phone travels through cells, the FOVC is where the tower tells

the phone to adjust power levels for the current cell or to change to

a new channel for use in the new cell. This info can be hacked apart.

So. When you've found a good conversation, don't be lazy! Enter 40#!

This makes the phone listen for commands on the voice channel

(embedded in the audio portion- you can hear it as a "bump" sound). It

will just sit there and the display will read '40' , but the

conversation will still be audible.  Now when the phone receives a

FOVC command (a 40 bit sequence) data will flow across the display, in

hex format, and stop. Listen to the phone, if the conversation is

still there, then the command was only to adjust power levels. If the

conversation is gone, then its a handoff. If you only got a power

adjustment command just press # or clr,  which ever gets you back to

the ' prompt. Enter 40# and keep listening. You can also use the # key

to cancel the 40# command, if you want to change channels or something.

If it was a handoff, its time for some quick math. You have to convert

some of the numbers to binary, and then to decimal. I don't know how

many characters your phone's display will show. Mine only shows the

last seven of the ten hex digits. Count left from the end 6 digits.

Write down that digit and the next two on a piece of paper, i.e.:

???j16djjj    j=junk numbers  (hex numbers range from 0-9,a-f)

/ \

these are lost due to scrolling

write down  16d then convert it to a binary string:

1 = 0001

6 = 0110

d = 1101    (d=13)

now you have a binary string like this:  000101101101

throw away the first 2 bits and get:       0101101101

convert this to decimal and get:                  365

365 is the new channel the conversation has moved to! Enter 110365#

and voila! You too, can hear the horny babe's phone number!

Don't forget to enter 40# again, as the call may be moving quickly

through cells ( small cells or freeway driving ) or the call can get

bounced around by the tower for cell traffic purposes.

Here's one more example of the hex>binary>decimal conversion.

???j5aejjj

5  =  0101

a  =  1010

e  =  1110

full string        = 010110101110

truncate 2 msb     =   0110101110

convert to decimal =          430

R*E*A*D*I*N*G***T*H*E***S*I*D**********************************S*E*C*T*I*O*N***7

READING THE SID by Doctor Who (drwho@l0pht.com)

The SID (System IDentification) of a control channel can be determined using

the test mode of the Motorola cellular phone.  This document assumes the

reader understands cellular technology in general, and how to access

Motorola's test mode in specific.

Tune the phone to the desired control channel with 11xxxx# where xxxx is the

channel number.  Hit 39# to receive one control channel word.  One should

appear in less than two seconds, filling up all ten digits on the display with

hexadecimal digits.  Do this repeatedly until one is found with the correct

pattern.  Digit places start at the left hand side and go to the right.

The first digit should be C,D,E, or F.  This letter can be used to determine

the DCC/SAT of the cell.  A "C" is SAT 0, D is 1, E is 2, and F is 3.  Ignore

digits 8,9, and 10.  They are parity bytes.  Digit 7 should be "6" or "E",

though I have never found it to be other than "E".  The hexadecimal value of

represented by digits 2 through 5 is then divided by two, and then 1 added if

the carrier as an "A" side, "non-wireline" carrier.  The result is the system

ID.

for example:

E00388EA08

E means this cell has an SAT/DCC of 3.  The A08 is ignored.  The E to the left

of it is proper and normal, so this is the right kind of message.  Ignore the

8 in position 6, that is just to the left E.  0038 in hexadecimal translates

((3*16=48)+8) to 56.  56/2=28.  Looking up System ID 28 on my chart indicates

Nynex in Boston.  This is correct.

Please be aware that the two SID charts I have seen around the net are very

outdated.  I have a more recent version on paper which I may eventually type

in, when I have the time and energy.

The methods used above are only a very crude way to do what could be done much

more efficiently by computer.  I am sure that programs will be written to do

exactly this, but I am holding off until I have thoroughly hacked the meaning

of all these types of message before writing such a program.  I am also

contemplating the design of a cable to replace the handset, running from the

25 pin connector on the side of my bag phone to a computer.

---------=?> Doctor Who <?=--------

P*H*O*N*E***P*I*N*-*O*U*T*S************************************S*E*C*T*I*O*N***8

Before going into the cable specs, here are the pin-outs to all phones as of

now (in the US).  A very special thanks go to Motorola for faxing me the new

Ultra Classic II pin-outs!

--------------------------------------------------------------------------------

15 pin cable pinouts

PIN    DESIGNATION/FUNCTION

1     Battery A+, red wire

2     Transmit Audio / ON-OFF Function, a shared line between Audio (AC) and

ground.  This line will toggle the ON/OFF status of the telephone.

3     Ground (A+ return), black wire

4     Ignition Sense Lead, green with a black tracer

5     Receiver Audio (RX High), to handset connector pin 8

6     Regulated +9.5 volts, to handset connector pin 2

7     T-Data, one of the 3-wire bus lines, to handset connector pin 3

8     C-Data, one of the 3-wire bus lines, to handset connector pin 4

9     Digital Hands-Free Microphone / Manual Test.  When the pin is grounded,

which can be done by shorting the two connectors of the Hands-Free

microphone, the unit is enabled to work in TEST MODE.

10    R-Data, one of the 3-wire bus lines, to handset connector pin 5

11    Handset Logic Ground, to handset connector pin 1

12    Speaker High \

| -> Only on SKN4279A and SKN4277A

13    Speaker Low  /

14    Handset Audio Ground, to handset connector pin 6

15    Auxiliary Alert, yellow lead with a black tracer, used to blow the horn

or flash the headlights.  Provides a ground for the relay; maximum

current is 1/2 amp.  It is  *N O T*  recommended that this circuit be

used to drive the horn or headlights directly.

--------------------------------------------------------------------------------

25 pin cable pinouts (series 2 and 3 transceivers)

PIN    DESIGNATION/FUNCTION

1     Transmit Audio/ON - OFF Function

2     Mobile/Transportable Select Line

3     Ground (A + return), one of 2 black wires.  Both are required for proper

operation

4     Battery A +, one of 2 red wires. Both are required for proper operation.

5     Ignition Sense Lead, green with red tracer

6     Receiver audio to handset (RX High), pin 8 on the handset connector

7     Ground

8     Regulated +9.5 volts to handset, pin 2 on handset connector

9     Ground

10    Auxiliary Alert, yellow with black tracer, used to blow the horn or

flash the headlights.  Provides a ground function. NOTE: 1/2 amps

maximum current.  The recommended method is to drive a relay

(e.g. MOT 59K813674).  Ignition Sense, pin 5, must be low for this

function to work.

11    T-Data, one of the 3-wire bus lines, to pin 3 of the handset connector

12    C-Data, one of the 3-wire bus lines, to pin 4 of the handset connector

13    Ground

14    Transmit Audio Shield

15    Transmit Audio

16    Battery A+, one of two red wires. Both are required for proper operation

17    Ground, one of two black wires. Both are required for proper operation

18    R-Data, one of the 3-wire bus lines, to pin 5 of the handset connector

19    Receiver audio to external speaker

20    Ground for receiver audio (shield) to external speaker

21    Manual test line.  When connected to ground, puts phone in test mode

22    Ground

23    Handset logic ground, to handset connector pin 1

24    Handset audio ground, to handset connector pin 6

25    Accessory ground, to external speaker

--------------------------------------------------------------------------------

OEM 32 pin cable pinouts        16 15 14 13 12 11 10  9  8  7  6  5  4  3  2  1

+----------------------------------------------+

Note: Looking into connector  C |*  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *|

on the transceiver with the  D |*  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *|

antenna port down.            +----------------------------------------------+

Pin#  Designation/Function Row C         Pin#  Designation/Function Row D

1  Battery A+                            1  Not Used

2  Ignition Sense                        2  Antenna Drive (for power antenna)

3  Status Display Control                3  Not Used

4  Not Used                              4  Auxiliary Alert, used to blow the

horn or flash the headlights

through a relay

5  Ground (A+ Return)                    5  Not Used

6  GM Proprietary Serial Data Bus         6  GM proprietary serial bus

7  Audio Ground, from GM Audio bus to    7  Audio Ground (not connected)

telephone

8  Low level audio, from telephone to GM 8  Low Level Audio (not connected)

Audio bus

9  Not used                              9  Manual Test Line; when grounded

this line enables TEST MODE

10 Not used                              10 RX High

11 Mobile / Transportable select line    11 Ground (TX Shield)

12 Handset Logic Ground                  12 Regulated +9.5 volts

13 T-Data, one of the 3-wire bus lines   13 C-Data, one of the 3-wire bus lines

14 R-Data, one of the 3-wire bus lines   14 Audio Ground

15 TX High / ON-OFF                      15 Ground (Rx Shield)

16 VSP Microphone High                   16 VSP Microphone Ground

--------------------------------------------------------------------------------

Pinouts for the Motorola 8000 brick phone - "N" series

numbering starts on top left         1  2  3  *  4  5  6

7  8  9  *  10 11 12

PIN    SIGNAL

*     GROUND

1     logic ground

2     not used

3     audio in to phone

4     audio out (and on/off toggle)

5     4.75 Bias

6     Manual test line

7     Ground for audio signals (common)

8     TRU data line

9     not used

10    CMP data line

11    RTN data line

12    ignition sense

--------------------------------------------------------------------------------

CVC BLOCK

[ 1] [ 2] [ 3]   [ 4] [ 5] [ 6]

[ 7] [ 8] [ 9]   [10] [11] [12]

1   VSP Enable        7   GROUND

2   SPKR Enable       8   S TRU

3   TX HI on/off      9   AUX Alert

4   RX HI             10  S CMP

5   RX HI OPT         11  S RTN

6   MAN TEST          12  IGN

--------------------------------------------------------------------------------

Flip Fones and all fones using the dpc/pt flip cable

J3 Pin  Function

1   2   3   4   5   6   7   8           + G -       1     Logic Ground

|   |   |   |   |   |   |   |           | | |       2     Ext. 7.5V

3     TRU

4     CMP

(looking at back of phone with battery removed)    5     RTN

6     Audio Ground

7     RX Audio OUT (spkr)

8     TX Audio IN (mic)

--------------------------------------------------------------------------------

New External Connector for the Ultra Classic II CVC Pinblocks

\

\

\

\

[]            []    \      Pin   Designation/Function

\                          \

+-------------------------+     1    Regulated 8 volts

|     12 10  8  6  4  2   |     3    ext. switched A+ enable

|      -  -  -  -  -  -   |     5    ext. spkr/mic enable

| O                       |     7    TX hi - on/off

|      -  -  -  -  -  -   |     9    T Data

\ |     11  9  7  5  3  1   |    11    R Data

+-------------------------+

|              2    Audio Ground

PHONE /               4    manual test

6    ignition / charger B+

8    RX hi

10    C Data

12    logic ground

C*A*B*L*E***S*P*E*C*S******************************************S*E*C*T*I*O*N***9

OK OK OK.  Here are the cable specs.  They are 100% correct.  Of course I

wouldn't know because these are for information purposes only.  I have been

told however by VERY reliable sources that they are guaranteed, 100%, GRADE-A,

correct.  If they don't work for you, you did something wrong.

--------------------------------------------------------------------------------

Cable Instructions for the bag phones       (thanks Jakey)

phone (female 25 pin)      computer (male 25 pin) (parallel port)

18 ----------------------- 1

21 ----------------------- 2

1 ------------------------ 4   Below are 10K ohm resistors

12 ----------------------- 12 ----/\/\/----.

11 ----------------------- 13 ----/\/\/----+

4,5,8 -------------------------------------'

2,3,17,20----------------- 18 --- Ground/black wire (-12 Volts)

16 ------------------------------ Positive/yellow wire (+12 volts)

By the way, Jakey told me to remind you that the handset can NOT be plugged in

while this cable is hooked up.

--------------------------------------------------------------------------------

Motorola Cellphone cable construction  for flips

-------------------------------------

DB25        FLIP                                  Battery Eliminator cable

----        ----                                  attachment pins up:

1(--------)4

--------=

2(--|<----)Jump this line to the Center                1  =

Pin on the back of phone.              --------=

=

4(--|<----)1   "|<" is the IN4001 diode.          --------=

=

12(--------)5                                      --------=

4  =  -->To phone

13(--------)6                                      --------=

5  =

18-25(-+------)8                                      --------=

|                                                   6  =

|    +-)7                                      --------=

|    |                                              7* =

|    |                                         --------=

|    |                                              8* =

NeG  PoS ---Cig adapter                         --------=

DB25 Male                Phone                     Power Connector

(see Note 1)

1-To phone pin 4         1-DB25 pin 4(see note 2)  Gnd-To Db25 Pins 18-25 and

2-To Phone test lead     2-NC                          Phone pin 8

(see note 2)

3-NC                     3-NC                      Tip-To phone pin 7

4-To phone pin 1         4-To DB25 pin 1

(see note 2)

5-NC                     5-To DB25 pin 12

6-NC                     6-To DB25 pin 13

7-NC                     7-To tip on power connector

8-NC                     8-GND

9-NC                     Test Lead-To DB25 pin 2 (See note 2)

10-NC

11-NC

12-To Phone pin 5

13-To Phone pin 6

14-NC

15-NC

16-NC

17-NC

18-GND \

19-GND |

20-GND |

21-GND |--Conn together to GND on 12v conn

22-GND |  And pin 8 on phone plug

23-GND |

24-GND |

25-GND /

NOTE 1:

The power adapter on the cable is 12 volt input but is a regulated

7.95 volts out. DO NOT connect 12 volts between pins 7 and 8 on the

phone connector.

NOTE 2:

|  /|

DB25 Pin 4-----| < |------Phone pin 1

|  \|

|  /|

DB25 Pin 2-----| < |------Phone test lead

|  \|

--------------------------------------------------------------------------------

Motorola Transceivers

_____________________

4500x,4800x,6800x,Etc.

______________________

Female 25 Pin                                Male 25 Pin

D-Connector                                  D-connector

To transceiver                               To P.C.Parallel Port

Pin                                     Pin

1._____________________________________  4.

2._____________________________________ 18.

___10k______ 13.

/

______4+5._______________________/____10K______ 12.

|       12.____________________________________/

|       11._____________________________________ 13.

|       18._____________________________________  1.

|       21._____________________________________  2.

|    ___14+17+20+23.____________________________ 18.

|    |

|    |___ -ve

|

|________ +ve 9 Volt

--------------------------------------------------------------------------------

Motorola 8500x 8800x(early type)

________________________________

25 Pin D-plug(P.C.lpt1)           Phone Back(battery removed)

Pin          Diodes         []    []    []      []    []    []

3.--orange-|<1n4001---------------------/       /           /

4.--blue---|<1n4001----------------------------/           /

2.--red----|<1n4001---------------------------------------/

19.-------------\

20.-----------\ |            []    []    []     []     []    []

18.--black----+-+------------/     /            /      /

13.--yellow-----------------------/            /      /

12.--brown------------------------------------/      /

1.--gray-------------------------------------------/

[]                              []

-8 to -12V.                     +8 to =12V.

NOTE Diode protocol: Cathode---|<diode---Anode

C*H*A*N*N*E*L***N*U*M*B**E*R***V*S*.***F*R*E*Q*U*E*N*C*Y*****S*E*C*T*I*O*N***1*0

Frequency Range for 666 Channels:      Reverse (Mobile TX) 825.020 - 844.990 MHz

Forward (Mobile RX) 870.020 - 889.990 MHz

Frequency Range for 832 and 2412 Channels:

Reverse (Mobile TX) 824.030 - 848.980 MHz

Forward (Mobile RX) 869.030 - 893.980 MHz

Frequency Calculation for Channels 1-799:

Reverse (Mobile TX) Frequency = 825.00 MHz + (Ch.# X .030 MHz)

Forward (Mobile RX) Frequency = 870.00 MHz + (Ch.# X .030 MHz)

Frequency Calculation for Channels 991-1023:

Reverse (Mobile TX) Frequency = 825.00 MHz - [.030 MHz X (1023 - Ch.#)]

Forward (Mobile RX) Frequency = 870.00 MHz - [.030 MHz X (1023 - Ch.#)]

To determine the center frequency of an associated NAMPS sub-channel in these

formulas, subtract 10 kHz from the result for the low sub-channel, leave the

result as is for the center sub-channel, and add 10 kHz to the result for the

high sub-channel.

--------------------------------------------------------------------------------

The following text I took from the Poisoned Pen BBS (Hi guys).  Thanks Jakey

for taking the time to decipher all of this shit.  As far as I know, with

the exception of a post on #cellular and the upload to Poisoned Pen, there is

nothing in print with this compilation.  Again, special thanks go to Jakey

(jbs@mcs.net) for the long, seemingly endless work.

CELLULAR PHONE FREQUENCIES AND MOTOROLA

TEST MODE NUMERIC CODES.

( Motorola test mode channel numbers )

( are for use in Motorola test mode  )

( with function 11xxxx#              )

( All frequencies in Megahertz FM    )

Lower Set (1-666)

Tower Freq.  Mobile Freq.    11xxxx# Channel

Tx  870.03   Rx  825.03  Chan  0001  #1

Tx  870.06   Rx  825.06  Chan  0002  #2

Tx  870.09   Rx  825.09  Chan  0003  #3

Tx  870.12   Rx  825.12  Chan  0004  #4

Tx  870.15   Rx  825.15  Chan  0005  #5

Tx  870.18   Rx  825.18  Chan  0006  #6

Tx  870.21   Rx  825.21  Chan  0007  #7

Tx  870.24   Rx  825.24  Chan  0008  #8

Tx  870.27   Rx  825.27  Chan  0009  #9

Tx  870.30   Rx  825.30  Chan  0010  #10

Tx  870.33   Rx  825.33  Chan  0011  #11

Tx  870.36   Rx  825.36  Chan  0012  #12

Tx  870.39   Rx  825.39  Chan  0013  #13

Tx  870.42   Rx  825.42  Chan  0014  #14

Tx  870.45   Rx  825.45  Chan  0015  #15

Tx  870.48   Rx  825.48  Chan  0016  #16

Tx  870.51   Rx  825.51  Chan  0017  #17

Tx  870.54   Rx  825.54  Chan  0018  #18

Tx  870.57   Rx  825.57  Chan  0019  #19

Tx  870.60   Rx  825.60  Chan  0020  #20

Tx  870.63   Rx  825.63  Chan  0021  #21

Tx  870.66   Rx  825.66  Chan  0022  #22

Tx  870.69   Rx  825.69  Chan  0023  #23

Tx  870.72   Rx  825.72  Chan  0024  #24

Tx  870.75   Rx  825.75  Chan  0025  #25

Tx  870.78   Rx  825.78  Chan  0026  #26

Tx  870.81   Rx  825.81  Chan  0027  #27

Tx  870.84   Rx  825.84  Chan  0028  #28

Tx  870.87   Rx  825.87  Chan  0029  #29

Tx  870.90   Rx  825.90  Chan  0030  #30

Tx  870.93   Rx  825.93  Chan  0031  #31

Tx  870.96   Rx  825.96  Chan  0032  #32

Tx  870.99   Rx  825.99  Chan  0033  #33

Tx  871.02   Rx  826.02  Chan  0034  #34

Tx  871.05   Rx  826.05  Chan  0035  #35

Tx  871.08   Rx  826.08  Chan  0036  #36

Tx  871.11   Rx  826.11  Chan  0037  #37

Tx  871.14   Rx  826.14  Chan  0038  #38

Tx  871.17   Rx  826.17  Chan  0039  #39

Tx  871.20   Rx  826.20  Chan  0040  #40

Tx  871.23   Rx  826.23  Chan  0041  #41

Tx  871.26   Rx  826.26  Chan  0042  #42

Tx  871.29   Rx  826.29  Chan  0043  #43

Tx  871.32   Rx  826.32  Chan  0044  #44

Tx  871.35   Rx  826.35  Chan  0045  #45

Tx  871.38   Rx  826.38  Chan  0046  #46

Tx  871.41   Rx  826.41  Chan  0047  #47

Tx  871.44   Rx  826.44  Chan  0048  #48

Tx  871.47   Rx  826.47  Chan  0049  #49

Tx  871.50   Rx  826.50  Chan  0050  #50

Tx  871.53   Rx  826.53  Chan  0051  #51

Tx  871.56   Rx  826.56  Chan  0052  #52

Tx  871.59   Rx  826.59  Chan  0053  #53

Tx  871.62   Rx  826.62  Chan  0054  #54

Tx  871.65   Rx  826.65  Chan  0055  #55

Tx  871.68   Rx  826.68  Chan  0056  #56

Tx  871.71   Rx  826.71  Chan  0057  #57

Tx  871.74   Rx  826.74  Chan  0058  #58

Tx  871.77   Rx  826.77  Chan  0059  #59

Tx  871.80   Rx  826.80  Chan  0060  #60

Tx  871.83   Rx  826.83  Chan  0061  #61

Tx  871.86   Rx  826.86  Chan  0062  #62

Tx  871.89   Rx  826.89  Chan  0063  #63

Tx  871.92   Rx  826.92  Chan  0064  #64

Tx  871.95   Rx  826.95  Chan  0065  #65

Tx  871.98   Rx  826.98  Chan  0066  #66

Tx  872.01   Rx  827.01  Chan  0067  #67

Tx  872.04   Rx  827.04  Chan  0068  #68

Tx  872.07   Rx  827.07  Chan  0069  #69

Tx  872.10   Rx  827.10  Chan  0070  #70

Tx  872.13   Rx  827.13  Chan  0071  #71

Tx  872.16   Rx  827.16  Chan  0072  #72

Tx  872.19   Rx  827.19  Chan  0073  #73

Tx  872.22   Rx  827.22  Chan  0074  #74

Tx  872.25   Rx  827.25  Chan  0075  #75

Tx  872.28   Rx  827.28  Chan  0076  #76

Tx  872.31   Rx  827.31  Chan  0077  #77

Tx  872.34   Rx  827.34  Chan  0078  #78

Tx  872.37   Rx  827.37  Chan  0079  #79

Tx  872.40   Rx  827.40  Chan  0080  #80

Tx  872.43   Rx  827.43  Chan  0081  #81

Tx  872.46   Rx  827.46  Chan  0082  #82

Tx  872.49   Rx  827.49  Chan  0083  #83

Tx  872.52   Rx  827.52  Chan  0084  #84

Tx  872.55   Rx  827.55  Chan  0085  #85

Tx  872.58   Rx  827.58  Chan  0086  #86

Tx  872.61   Rx  827.61  Chan  0087  #87

Tx  872.64   Rx  827.64  Chan  0088  #88

Tx  872.67   Rx  827.67  Chan  0089  #89

Tx  872.70   Rx  827.70  Chan  0090  #90

Tx  872.73   Rx  827.73  Chan  0091  #91

Tx  872.76   Rx  827.76  Chan  0092  #92

Tx  872.79   Rx  827.79  Chan  0093  #93

Tx  872.82   Rx  827.82  Chan  0094  #94

Tx  872.85   Rx  827.85  Chan  0095  #95

Tx  872.88   Rx  827.88  Chan  0096  #96

Tx  872.91   Rx  827.91  Chan  0097  #97

Tx  872.94   Rx  827.94  Chan  0098  #98

Tx  872.97   Rx  827.97  Chan  0099  #99

Tx  873.00   Rx  828.00  Chan  0100  #100

Tx  873.03   Rx  828.03  Chan  0101  #101

Tx  873.06   Rx  828.06  Chan  0102  #102

Tx  873.09   Rx  828.09  Chan  0103  #103

Tx  873.12   Rx  828.12  Chan  0104  #104

Tx  873.15   Rx  828.15  Chan  0105  #105

Tx  873.18   Rx  828.18  Chan  0106  #106

Tx  873.21   Rx  828.21  Chan  0107  #107

Tx  873.24   Rx  828.24  Chan  0108  #108

Tx  873.27   Rx  828.27  Chan  0109  #109

Tx  873.30   Rx  828.30  Chan  0110  #110

Tx  873.33   Rx  828.33  Chan  0111  #111

Tx  873.36   Rx  828.36  Chan  0112  #112

Tx  873.39   Rx  828.39  Chan  0113  #113

Tx  873.42   Rx  828.42  Chan  0114  #114

Tx  873.45   Rx  828.45  Chan  0115  #115

Tx  873.48   Rx  828.48  Chan  0116  #116

Tx  873.51   Rx  828.51  Chan  0117  #117

Tx  873.54   Rx  828.54  Chan  0118  #118

Tx  873.57   Rx  828.57  Chan  0119  #119

Tx  873.60   Rx  828.60  Chan  0120  #120

Tx  873.63   Rx  828.63  Chan  0121  #121

Tx  873.66   Rx  828.66  Chan  0122  #122

Tx  873.69   Rx  828.69  Chan  0123  #123

Tx  873.72   Rx  828.72  Chan  0124  #124

Tx  873.75   Rx  828.75  Chan  0125  #125

Tx  873.78   Rx  828.78  Chan  0126  #126

Tx  873.81   Rx  828.81  Chan  0127  #127

Tx  873.84   Rx  828.84  Chan  0128  #128

Tx  873.87   Rx  828.87  Chan  0129  #129

Tx  873.90   Rx  828.90  Chan  0130  #130

Tx  873.93   Rx  828.93  Chan  0131  #131

Tx  873.96   Rx  828.96  Chan  0132  #132

Tx  873.99   Rx  828.99  Chan  0133  #133

Tx  874.02   Rx  829.02  Chan  0134  #134

Tx  874.05   Rx  829.05  Chan  0135  #135

Tx  874.08   Rx  829.08  Chan  0136  #136

Tx  874.11   Rx  829.11  Chan  0137  #137

Tx  874.14   Rx  829.14  Chan  0138  #138

Tx  874.17   Rx  829.17  Chan  0139  #139

Tx  874.20   Rx  829.20  Chan  0140  #140

Tx  874.23   Rx  829.23  Chan  0141  #141

Tx  874.26   Rx  829.26  Chan  0142  #142

Tx  874.29   Rx  829.29  Chan  0143  #143

Tx  874.32   Rx  829.32  Chan  0144  #144

Tx  874.35   Rx  829.35  Chan  0145  #145

Tx  874.38   Rx  829.38  Chan  0146  #146

Tx  874.41   Rx  829.41  Chan  0147  #147

Tx  874.44   Rx  829.44  Chan  0148  #148

Tx  874.47   Rx  829.47  Chan  0149  #149

Tx  874.50   Rx  829.50  Chan  0150  #150

Tx  874.53   Rx  829.53  Chan  0151  #151

Tx  874.56   Rx  829.56  Chan  0152  #152

Tx  874.59   Rx  829.59  Chan  0153  #153

Tx  874.62   Rx  829.62  Chan  0154  #154

Tx  874.65   Rx  829.65  Chan  0155  #155

Tx  874.68   Rx  829.68  Chan  0156  #156

Tx  874.71   Rx  829.71  Chan  0157  #157

Tx  874.74   Rx  829.74  Chan  0158  #158

Tx  874.77   Rx  829.77  Chan  0159  #159

Tx  874.80   Rx  829.80  Chan  0160  #160

Tx  874.83   Rx  829.83  Chan  0161  #161

Tx  874.86   Rx  829.86  Chan  0162  #162

Tx  874.89   Rx  829.89  Chan  0163  #163

Tx  874.92   Rx  829.92  Chan  0164  #164

Tx  874.95   Rx  829.95  Chan  0165  #165

Tx  874.98   Rx  829.98  Chan  0166  #166

Tx  875.01   Rx  830.01  Chan  0167  #167

Tx  875.04   Rx  830.04  Chan  0168  #168

Tx  875.07   Rx  830.07  Chan  0169  #169

Tx  875.10   Rx  830.10  Chan  0170  #170

Tx  875.13   Rx  830.13  Chan  0171  #171

Tx  875.16   Rx  830.16  Chan  0172  #172

Tx  875.19   Rx  830.19  Chan  0173  #173

Tx  875.22   Rx  830.22  Chan  0174  #174

Tx  875.25   Rx  830.25  Chan  0175  #175

Tx  875.28   Rx  830.28  Chan  0176  #176

Tx  875.31   Rx  830.31  Chan  0177  #177

Tx  875.34   Rx  830.34  Chan  0178  #178

Tx  875.37   Rx  830.37  Chan  0179  #179

Tx  875.40   Rx  830.40  Chan  0180  #180

Tx  875.43   Rx  830.43  Chan  0181  #181

Tx  875.46   Rx  830.46  Chan  0182  #182

Tx  875.49   Rx  830.49  Chan  0183  #183

Tx  875.52   Rx  830.52  Chan  0184  #184

Tx  875.55   Rx  830.55  Chan  0185  #185

Tx  875.58   Rx  830.58  Chan  0186  #186

Tx  875.61   Rx  830.61  Chan  0187  #187

Tx  875.64   Rx  830.64  Chan  0188  #188

Tx  875.67   Rx  830.67  Chan  0189  #189

Tx  875.70   Rx  830.70  Chan  0190  #190

Tx  875.73   Rx  830.73  Chan  0191  #191

Tx  875.76   Rx  830.76  Chan  0192  #192

Tx  875.79   Rx  830.79  Chan  0193  #193

Tx  875.82   Rx  830.82  Chan  0194  #194

Tx  875.85   Rx  830.85  Chan  0195  #195

Tx  875.88   Rx  830.88  Chan  0196  #196

Tx  875.91   Rx  830.91  Chan  0197  #197

Tx  875.94   Rx  830.94  Chan  0198  #198

Tx  875.97   Rx  830.97  Chan  0199  #199

Tx  876.00   Rx  831.00  Chan  0200  #200

Tx  876.03   Rx  831.03  Chan  0201  #201

Tx  876.06   Rx  831.06  Chan  0202  #202

Tx  876.09   Rx  831.09  Chan  0203  #203

Tx  876.12   Rx  831.12  Chan  0204  #204

Tx  876.15   Rx  831.15  Chan  0205  #205

Tx  876.18   Rx  831.18  Chan  0206  #206

Tx  876.21   Rx  831.21  Chan  0207  #207

Tx  876.24   Rx  831.24  Chan  0208  #208

Tx  876.27   Rx  831.27  Chan  0209  #209

Tx  876.30   Rx  831.30  Chan  0210  #210

Tx  876.33   Rx  831.33  Chan  0211  #211

Tx  876.36   Rx  831.36  Chan  0212  #212

Tx  876.39   Rx  831.39  Chan  0213  #213

Tx  876.42   Rx  831.42  Chan  0214  #214

Tx  876.45   Rx  831.45  Chan  0215  #215

Tx  876.48   Rx  831.48  Chan  0216  #216

Tx  876.51   Rx  831.51  Chan  0217  #217

Tx  876.54   Rx  831.54  Chan  0218  #218

Tx  876.57   Rx  831.57  Chan  0219  #219

Tx  876.60   Rx  831.60  Chan  0220  #220

Tx  876.63   Rx  831.63  Chan  0221  #221

Tx  876.66   Rx  831.66  Chan  0222  #222

Tx  876.69   Rx  831.69  Chan  0223  #223

Tx  876.72   Rx  831.72  Chan  0224  #224

Tx  876.75   Rx  831.75  Chan  0225  #225

Tx  876.78   Rx  831.78  Chan  0226  #226

Tx  876.81   Rx  831.81  Chan  0227  #227

Tx  876.84   Rx  831.84  Chan  0228  #228

Tx  876.87   Rx  831.87  Chan  0229  #229

Tx  876.90   Rx  831.90  Chan  0230  #230

Tx  876.93   Rx  831.93  Chan  0231  #231

Tx  876.96   Rx  831.96  Chan  0232  #232

Tx  876.99   Rx  831.99  Chan  0233  #233

Tx  877.02   Rx  832.02  Chan  0234  #234

Tx  877.05   Rx  832.05  Chan  0235  #235

Tx  877.08   Rx  832.08  Chan  0236  #236

Tx  877.11   Rx  832.11  Chan  0237  #237

Tx  877.14   Rx  832.14  Chan  0238  #238

Tx  877.17   Rx  832.17  Chan  0239  #239

Tx  877.20   Rx  832.20  Chan  0240  #240

Tx  877.23   Rx  832.23  Chan  0241  #241

Tx  877.26   Rx  832.26  Chan  0242  #242

Tx  877.29   Rx  832.29  Chan  0243  #243

Tx  877.32   Rx  832.32  Chan  0244  #244

Tx  877.35   Rx  832.35  Chan  0245  #245

Tx  877.38   Rx  832.38  Chan  0246  #246

Tx  877.41   Rx  832.41  Chan  0247  #247

Tx  877.44   Rx  832.44  Chan  0248  #248

Tx  877.47   Rx  832.47  Chan  0249  #249

Tx  877.50   Rx  832.50  Chan  0250  #250

Tx  877.53   Rx  832.53  Chan  0251  #251

Tx  877.56   Rx  832.56  Chan  0252  #252

Tx  877.59   Rx  832.59  Chan  0253  #253

Tx  877.62   Rx  832.62  Chan  0254  #254

Tx  877.65   Rx  832.65  Chan  0255  #255

Tx  877.68   Rx  832.68  Chan  0256  #256

Tx  877.71   Rx  832.71  Chan  0257  #257

Tx  877.74   Rx  832.74  Chan  0258  #258

Tx  877.77   Rx  832.77  Chan  0259  #259

Tx  877.80   Rx  832.80  Chan  0260  #260

Tx  877.83   Rx  832.83  Chan  0261  #261

Tx  877.86   Rx  832.86  Chan  0262  #262

Tx  877.89   Rx  832.89  Chan  0263  #263

Tx  877.92   Rx  832.92  Chan  0264  #264

Tx  877.95   Rx  832.95  Chan  0265  #265

Tx  877.98   Rx  832.98  Chan  0266  #266

Tx  878.01   Rx  833.01  Chan  0267  #267

Tx  878.04   Rx  833.04  Chan  0268  #268

Tx  878.07   Rx  833.07  Chan  0269  #269

Tx  878.10   Rx  833.10  Chan  0270  #270

Tx  878.13   Rx  833.13  Chan  0271  #271

Tx  878.16   Rx  833.16  Chan  0272  #272

Tx  878.19   Rx  833.19  Chan  0273  #273

Tx  878.22   Rx  833.22  Chan  0274  #274

Tx  878.25   Rx  833.25  Chan  0275  #275

Tx  878.28   Rx  833.28  Chan  0276  #276

Tx  878.31   Rx  833.31  Chan  0277  #277

Tx  878.34   Rx  833.34  Chan  0278  #278

Tx  878.37   Rx  833.37  Chan  0279  #279

Tx  878.40   Rx  833.40  Chan  0280  #280

Tx  878.43   Rx  833.43  Chan  0281  #281

Tx  878.46   Rx  833.46  Chan  0282  #282

Tx  878.49   Rx  833.49  Chan  0283  #283

Tx  878.52   Rx  833.52  Chan  0284  #284

Tx  878.55   Rx  833.55  Chan  0285  #285

Tx  878.58   Rx  833.58  Chan  0286  #286

Tx  878.61   Rx  833.61  Chan  0287  #287

Tx  878.64   Rx  833.64  Chan  0288  #288

Tx  878.67   Rx  833.67  Chan  0289  #289

Tx  878.70   Rx  833.70  Chan  0290  #290

Tx  878.73   Rx  833.73  Chan  0291  #291

Tx  878.76   Rx  833.76  Chan  0292  #292

Tx  878.79   Rx  833.79  Chan  0293  #293

Tx  878.82   Rx  833.82  Chan  0294  #294

Tx  878.85   Rx  833.85  Chan  0295  #295

Tx  878.88   Rx  833.88  Chan  0296  #296

Tx  878.91   Rx  833.91  Chan  0297  #297

Tx  878.94   Rx  833.94  Chan  0298  #298

Tx  878.97   Rx  833.97  Chan  0299  #299

Tx  879.00   Rx  834.00  Chan  0300  #300

Tx  879.03   Rx  834.03  Chan  0301  #301

Tx  879.06   Rx  834.06  Chan  0302  #302

Tx  879.09   Rx  834.09  Chan  0303  #303

Tx  879.12   Rx  834.12  Chan  0304  #304

Tx  879.15   Rx  834.15  Chan  0305  #305

Tx  879.18   Rx  834.18  Chan  0306  #306

Tx  879.21   Rx  834.21  Chan  0307  #307

Tx  879.24   Rx  834.24  Chan  0308  #308

Tx  879.27   Rx  834.27  Chan  0309  #309

Tx  879.30   Rx  834.30  Chan  0310  #310

Tx  879.33   Rx  834.33  Chan  0311  #311

Tx  879.36   Rx  834.36  Chan  0312  #312

Tx  879.39   Rx  834.39  Chan  0313  #313

Tx  879.42   Rx  834.42  Chan  0314  #314

Tx  879.45   Rx  834.45  Chan  0315  #315

Tx  879.48   Rx  834.48  Chan  0316  #316

Tx  879.51   Rx  834.51  Chan  0317  #317

Tx  879.54   Rx  834.54  Chan  0318  #318

Tx  879.57   Rx  834.57  Chan  0319  #319

Tx  879.60   Rx  834.60  Chan  0320  #320

Tx  879.63   Rx  834.63  Chan  0321  #321

Tx  879.66   Rx  834.66  Chan  0322  #322

Tx  879.69   Rx  834.69  Chan  0323  #323

Tx  879.72   Rx  834.72  Chan  0324  #324

Tx  879.75   Rx  834.75  Chan  0325  #325

Tx  879.78   Rx  834.78  Chan  0326  #326

Tx  879.81   Rx  834.81  Chan  0327  #327

Tx  879.84   Rx  834.84  Chan  0328  #328

Tx  879.87   Rx  834.87  Chan  0329  #329

Tx  879.90   Rx  834.90  Chan  0330  #330

Tx  879.93   Rx  834.93  Chan  0331  #331

Tx  879.96   Rx  834.96  Chan  0332  #332

Tx  879.99   Rx  834.99  Chan  0333  #333

Tx  880.02   Rx  835.02  Chan  0334  #334

Tx  880.05   Rx  835.05  Chan  0335  #335

Tx  880.08   Rx  835.08  Chan  0336  #336

Tx  880.11   Rx  835.11  Chan  0337  #337

Tx  880.14   Rx  835.14  Chan  0338  #338

Tx  880.17   Rx  835.17  Chan  0339  #339

Tx  880.20   Rx  835.20  Chan  0340  #340

Tx  880.23   Rx  835.23  Chan  0341  #341

Tx  880.26   Rx  835.26  Chan  0342  #342

Tx  880.29   Rx  835.29  Chan  0343  #343

Tx  880.32   Rx  835.32  Chan  0344  #344

Tx  880.35   Rx  835.35  Chan  0345  #345

Tx  880.38   Rx  835.38  Chan  0346  #346

Tx  880.41   Rx  835.41  Chan  0347  #347

Tx  880.44   Rx  835.44  Chan  0348  #348

Tx  880.47   Rx  835.47  Chan  0349  #349

Tx  880.50   Rx  835.50  Chan  0350  #350

Tx  880.53   Rx  835.53  Chan  0351  #351

Tx  880.56   Rx  835.56  Chan  0352  #352

Tx  880.59   Rx  835.59  Chan  0353  #353

Tx  880.62   Rx  835.62  Chan  0354  #354

Tx  880.65   Rx  835.65  Chan  0355  #355

Tx  880.68   Rx  835.68  Chan  0356  #356

Tx  880.71   Rx  835.71  Chan  0357  #357

Tx  880.74   Rx  835.74  Chan  0358  #358

Tx  880.77   Rx  835.77  Chan  0359  #359

Tx  880.80   Rx  835.80  Chan  0360  #360

Tx  880.83   Rx  835.83  Chan  0361  #361

Tx  880.86   Rx  835.86  Chan  0362  #362

Tx  880.89   Rx  835.89  Chan  0363  #363

Tx  880.92   Rx  835.92  Chan  0364  #364

Tx  880.95   Rx  835.95  Chan  0365  #365

Tx  880.98   Rx  835.98  Chan  0366  #366

Tx  881.01   Rx  836.01  Chan  0367  #367

Tx  881.04   Rx  836.04  Chan  0368  #368

Tx  881.07   Rx  836.07  Chan  0369  #369

Tx  881.10   Rx  836.10  Chan  0370  #370

Tx  881.13   Rx  836.13  Chan  0371  #371

Tx  881.16   Rx  836.16  Chan  0372  #372

Tx  881.19   Rx  836.19  Chan  0373  #373

Tx  881.22   Rx  836.22  Chan  0374  #374

Tx  881.25   Rx  836.25  Chan  0375  #375

Tx  881.28   Rx  836.28  Chan  0376  #376

Tx  881.31   Rx  836.31  Chan  0377  #377

Tx  881.34   Rx  836.34  Chan  0378  #378

Tx  881.37   Rx  836.37  Chan  0379  #379

Tx  881.40   Rx  836.40  Chan  0380  #380

Tx  881.43   Rx  836.43  Chan  0381  #381

Tx  881.46   Rx  836.46  Chan  0382  #382

Tx  881.49   Rx  836.49  Chan  0383  #383

Tx  881.52   Rx  836.52  Chan  0384  #384

Tx  881.55   Rx  836.55  Chan  0385  #385

Tx  881.58   Rx  836.58  Chan  0386  #386

Tx  881.61   Rx  836.61  Chan  0387  #387

Tx  881.64   Rx  836.64  Chan  0388  #388

Tx  881.67   Rx  836.67  Chan  0389  #389

Tx  881.70   Rx  836.70  Chan  0390  #390

Tx  881.73   Rx  836.73  Chan  0391  #391

Tx  881.76   Rx  836.76  Chan  0392  #392

Tx  881.79   Rx  836.79  Chan  0393  #393

Tx  881.82   Rx  836.82  Chan  0394  #394

Tx  881.85   Rx  836.85  Chan  0395  #395

Tx  881.88   Rx  836.88  Chan  0396  #396

Tx  881.91   Rx  836.91  Chan  0397  #397

Tx  881.94   Rx  836.94  Chan  0398  #398

Tx  881.97   Rx  836.97  Chan  0399  #399

Tx  882.00   Rx  837.00  Chan  0400  #400

Tx  882.03   Rx  837.03  Chan  0401  #401

Tx  882.06   Rx  837.06  Chan  0402  #402

Tx  882.09   Rx  837.09  Chan  0403  #403

Tx  882.12   Rx  837.12  Chan  0404  #404

Tx  882.15   Rx  837.15  Chan  0405  #405

Tx  882.18   Rx  837.18  Chan  0406  #406

Tx  882.21   Rx  837.21  Chan  0407  #407

Tx  882.24   Rx  837.24  Chan  0408  #408

Tx  882.27   Rx  837.27  Chan  0409  #409

Tx  882.30   Rx  837.30  Chan  0410  #410

Tx  882.33   Rx  837.33  Chan  0411  #411

Tx  882.36   Rx  837.36  Chan  0412  #412

Tx  882.39   Rx  837.39  Chan  0413  #413

Tx  882.42   Rx  837.42  Chan  0414  #414

Tx  882.45   Rx  837.45  Chan  0415  #415

Tx  882.48   Rx  837.48  Chan  0416  #416

Tx  882.51   Rx  837.51  Chan  0417  #417

Tx  882.54   Rx  837.54  Chan  0418  #418

Tx  882.57   Rx  837.57  Chan  0419  #419

Tx  882.60   Rx  837.60  Chan  0420  #420

Tx  882.63   Rx  837.63  Chan  0421  #421

Tx  882.66   Rx  837.66  Chan  0422  #422

Tx  882.69   Rx  837.69  Chan  0423  #423

Tx  882.72   Rx  837.72  Chan  0424  #424

Tx  882.75   Rx  837.75  Chan  0425  #425

Tx  882.78   Rx  837.78  Chan  0426  #426

Tx  882.81   Rx  837.81  Chan  0427  #427

Tx  882.84   Rx  837.84  Chan  0428  #428

Tx  882.87   Rx  837.87  Chan  0429  #429

Tx  882.90   Rx  837.90  Chan  0430  #430

Tx  882.93   Rx  837.93  Chan  0431  #431

Tx  882.96   Rx  837.96  Chan  0432  #432

Tx  882.99   Rx  837.99  Chan  0433  #433

Tx  883.02   Rx  838.02  Chan  0434  #434

Tx  883.05   Rx  838.05  Chan  0435  #435

Tx  883.08   Rx  838.08  Chan  0436  #436

Tx  883.11   Rx  838.11  Chan  0437  #437

Tx  883.14   Rx  838.14  Chan  0438  #438

Tx  883.17   Rx  838.17  Chan  0439  #439

Tx  883.20   Rx  838.20  Chan  0440  #440

Tx  883.23   Rx  838.23  Chan  0441  #441

Tx  883.26   Rx  838.26  Chan  0442  #442

Tx  883.29   Rx  838.29  Chan  0443  #443

Tx  883.32   Rx  838.32  Chan  0444  #444

Tx  883.35   Rx  838.35  Chan  0445  #445

Tx  883.38   Rx  838.38  Chan  0446  #446

Tx  883.41   Rx  838.41  Chan  0447  #447

Tx  883.44   Rx  838.44  Chan  0448  #448

Tx  883.47   Rx  838.47  Chan  0449  #449

Tx  883.50   Rx  838.50  Chan  0450  #450

Tx  883.53   Rx  838.53  Chan  0451  #451

Tx  883.56   Rx  838.56  Chan  0452  #452

Tx  883.59   Rx  838.59  Chan  0453  #453

Tx  883.62   Rx  838.62  Chan  0454  #454

Tx  883.65   Rx  838.65  Chan  0455  #455

Tx  883.68   Rx  838.68  Chan  0456  #456

Tx  883.71   Rx  838.71  Chan  0457  #457

Tx  883.74   Rx  838.74  Chan  0458  #458

Tx  883.77   Rx  838.77  Chan  0459  #459

Tx  883.80   Rx  838.80  Chan  0460  #460

Tx  883.83   Rx  838.83  Chan  0461  #461

Tx  883.86   Rx  838.86  Chan  0462  #462

Tx  883.89   Rx  838.89  Chan  0463  #463

Tx  883.92   Rx  838.92  Chan  0464  #464

Tx  883.95   Rx  838.95  Chan  0465  #465

Tx  883.98   Rx  838.98  Chan  0466  #466

Tx  884.01   Rx  839.01  Chan  0467  #467

Tx  884.04   Rx  839.04  Chan  0468  #468

Tx  884.07   Rx  839.07  Chan  0469  #469

Tx  884.10   Rx  839.10  Chan  0470  #470

Tx  884.13   Rx  839.13  Chan  0471  #471

Tx  884.16   Rx  839.16  Chan  0472  #472

Tx  884.19   Rx  839.19  Chan  0473  #473

Tx  884.22   Rx  839.22  Chan  0474  #474

Tx  884.25   Rx  839.25  Chan  0475  #475

Tx  884.28   Rx  839.28  Chan  0476  #476

Tx  884.31   Rx  839.31  Chan  0477  #477

Tx  884.34   Rx  839.34  Chan  0478  #478

Tx  884.37   Rx  839.37  Chan  0479  #479

Tx  884.40   Rx  839.40  Chan  0480  #480

Tx  884.43   Rx  839.43  Chan  0481  #481

Tx  884.46   Rx  839.46  Chan  0482  #482

Tx  884.49   Rx  839.49  Chan  0483  #483

Tx  884.52   Rx  839.52  Chan  0484  #484

Tx  884.55   Rx  839.55  Chan  0485  #485

Tx  884.58   Rx  839.58  Chan  0486  #486

Tx  884.61   Rx  839.61  Chan  0487  #487

Tx  884.64   Rx  839.64  Chan  0488  #488

Tx  884.67   Rx  839.67  Chan  0489  #489

Tx  884.70   Rx  839.70  Chan  0490  #490

Tx  884.73   Rx  839.73  Chan  0491  #491

Tx  884.76   Rx  839.76  Chan  0492  #492

Tx  884.79   Rx  839.79  Chan  0493  #493

Tx  884.82   Rx  839.82  Chan  0494  #494

Tx  884.85   Rx  839.85  Chan  0495  #495

Tx  884.88   Rx  839.88  Chan  0496  #496

Tx  884.91   Rx  839.91  Chan  0497  #497

Tx  884.94   Rx  839.94  Chan  0498  #498

Tx  884.97   Rx  839.97  Chan  0499  #499

Tx  885.00   Rx  840.00  Chan  0500  #500

Tx  885.03   Rx  840.03  Chan  0501  #501

Tx  885.06   Rx  840.06  Chan  0502  #502

Tx  885.09   Rx  840.09  Chan  0503  #503

Tx  885.12   Rx  840.12  Chan  0504  #504

Tx  885.15   Rx  840.15  Chan  0505  #505

Tx  885.18   Rx  840.18  Chan  0506  #506

Tx  885.21   Rx  840.21  Chan  0507  #507

Tx  885.24   Rx  840.24  Chan  0508  #508

Tx  885.27   Rx  840.27  Chan  0509  #509

Tx  885.30   Rx  840.30  Chan  0510  #510

Tx  885.33   Rx  840.33  Chan  0511  #511

Tx  885.36   Rx  840.36  Chan  0512  #512

Tx  885.39   Rx  840.39  Chan  0513  #513

Tx  885.42   Rx  840.42  Chan  0514  #514

Tx  885.45   Rx  840.45  Chan  0515  #515

Tx  885.48   Rx  840.48  Chan  0516  #516

Tx  885.51   Rx  840.51  Chan  0517  #517

Tx  885.54   Rx  840.54  Chan  0518  #518

Tx  885.57   Rx  840.57  Chan  0519  #519

Tx  885.60   Rx  840.60  Chan  0520  #520

Tx  885.63   Rx  840.63  Chan  0521  #521

Tx  885.66   Rx  840.66  Chan  0522  #522

Tx  885.69   Rx  840.69  Chan  0523  #523

Tx  885.72   Rx  840.72  Chan  0524  #524

Tx  885.75   Rx  840.75  Chan  0525  #525

Tx  885.78   Rx  840.78  Chan  0526  #526

Tx  885.81   Rx  840.81  Chan  0527  #527

Tx  885.84   Rx  840.84  Chan  0528  #528

Tx  885.87   Rx  840.87  Chan  0529  #529

Tx  885.90   Rx  840.90  Chan  0530  #530

Tx  885.93   Rx  840.93  Chan  0531  #531

Tx  885.96   Rx  840.96  Chan  0532  #532

Tx  885.99   Rx  840.99  Chan  0533  #533

Tx  886.02   Rx  841.02  Chan  0534  #534

Tx  886.05   Rx  841.05  Chan  0535  #535

Tx  886.08   Rx  841.08  Chan  0536  #536

Tx  886.11   Rx  841.11  Chan  0537  #537

Tx  886.14   Rx  841.14  Chan  0538  #538

Tx  886.17   Rx  841.17  Chan  0539  #539

Tx  886.20   Rx  841.20  Chan  0540  #540

Tx  886.23   Rx  841.23  Chan  0541  #541

Tx  886.26   Rx  841.26  Chan  0542  #542

Tx  886.29   Rx  841.29  Chan  0543  #543

Tx  886.32   Rx  841.32  Chan  0544  #544

Tx  886.35   Rx  841.35  Chan  0545  #545

Tx  886.38   Rx  841.38  Chan  0546  #546

Tx  886.41   Rx  841.41  Chan  0547  #547

Tx  886.44   Rx  841.44  Chan  0548  #548

Tx  886.47   Rx  841.47  Chan  0549  #549

Tx  886.50   Rx  841.50  Chan  0550  #550

Tx  886.53   Rx  841.53  Chan  0551  #551

Tx  886.56   Rx  841.56  Chan  0552  #552

Tx  886.59   Rx  841.59  Chan  0553  #553

Tx  886.62   Rx  841.62  Chan  0554  #554

Tx  886.65   Rx  841.65  Chan  0555  #555

Tx  886.68   Rx  841.68  Chan  0556  #556

Tx  886.71   Rx  841.71  Chan  0557  #557

Tx  886.74   Rx  841.74  Chan  0558  #558

Tx  886.77   Rx  841.77  Chan  0559  #559

Tx  886.80   Rx  841.80  Chan  0560  #560

Tx  886.83   Rx  841.83  Chan  0561  #561

Tx  886.86   Rx  841.86  Chan  0562  #562

Tx  886.89   Rx  841.89  Chan  0563  #563

Tx  886.92   Rx  841.92  Chan  0564  #564

Tx  886.95   Rx  841.95  Chan  0565  #565

Tx  886.98   Rx  841.98  Chan  0566  #566

Tx  887.01   Rx  842.01  Chan  0567  #567

Tx  887.04   Rx  842.04  Chan  0568  #568

Tx  887.07   Rx  842.07  Chan  0569  #569

Tx  887.10   Rx  842.10  Chan  0570  #570

Tx  887.13   Rx  842.13  Chan  0571  #571

Tx  887.16   Rx  842.16  Chan  0572  #572

Tx  887.19   Rx  842.19  Chan  0573  #573

Tx  887.22   Rx  842.22  Chan  0574  #574

Tx  887.25   Rx  842.25  Chan  0575  #575

Tx  887.28   Rx  842.28  Chan  0576  #576

Tx  887.31   Rx  842.31  Chan  0577  #577

Tx  887.34   Rx  842.34  Chan  0578  #578

Tx  887.37   Rx  842.37  Chan  0579  #579

Tx  887.40   Rx  842.40  Chan  0580  #580

Tx  887.43   Rx  842.43  Chan  0581  #581

Tx  887.46   Rx  842.46  Chan  0582  #582

Tx  887.49   Rx  842.49  Chan  0583  #583

Tx  887.52   Rx  842.52  Chan  0584  #584

Tx  887.55   Rx  842.55  Chan  0585  #585

Tx  887.58   Rx  842.58  Chan  0586  #586

Tx  887.61   Rx  842.61  Chan  0587  #587

Tx  887.64   Rx  842.64  Chan  0588  #588

Tx  887.67   Rx  842.67  Chan  0589  #589

Tx  887.70   Rx  842.70  Chan  0590  #590

Tx  887.73   Rx  842.73  Chan  0591  #591

Tx  887.76   Rx  842.76  Chan  0592  #592

Tx  887.79   Rx  842.79  Chan  0593  #593

Tx  887.82   Rx  842.82  Chan  0594  #594

Tx  887.85   Rx  842.85  Chan  0595  #595

Tx  887.88   Rx  842.88  Chan  0596  #596

Tx  887.91   Rx  842.91  Chan  0597  #597

Tx  887.94   Rx  842.94  Chan  0598  #598

Tx  887.97   Rx  842.97  Chan  0599  #599

Tx  888.00   Rx  843.00  Chan  0600  #600

Tx  888.03   Rx  843.03  Chan  0601  #601

Tx  888.06   Rx  843.06  Chan  0602  #602

Tx  888.09   Rx  843.09  Chan  0603  #603

Tx  888.12   Rx  843.12  Chan  0604  #604

Tx  888.15   Rx  843.15  Chan  0605  #605

Tx  888.18   Rx  843.18  Chan  0606  #606

Tx  888.21   Rx  843.21  Chan  0607  #607

Tx  888.24   Rx  843.24  Chan  0608  #608

Tx  888.27   Rx  843.27  Chan  0609  #609

Tx  888.30   Rx  843.30  Chan  0610  #610

Tx  888.33   Rx  843.33  Chan  0611  #611

Tx  888.36   Rx  843.36  Chan  0612  #612

Tx  888.39   Rx  843.39  Chan  0613  #613

Tx  888.42   Rx  843.42  Chan  0614  #614

Tx  888.45   Rx  843.45  Chan  0615  #615

Tx  888.48   Rx  843.48  Chan  0616  #616

Tx  888.51   Rx  843.51  Chan  0617  #617

Tx  888.54   Rx  843.54  Chan  0618  #618

Tx  888.57   Rx  843.57  Chan  0619  #619

Tx  888.60   Rx  843.60  Chan  0620  #620

Tx  888.63   Rx  843.63  Chan  0621  #621

Tx  888.66   Rx  843.66  Chan  0622  #622

Tx  888.69   Rx  843.69  Chan  0623  #623

Tx  888.72   Rx  843.72  Chan  0624  #624

Tx  888.75   Rx  843.75  Chan  0625  #625

Tx  888.78   Rx  843.78  Chan  0626  #626

Tx  888.81   Rx  843.81  Chan  0627  #627

Tx  888.84   Rx  843.84  Chan  0628  #628

Tx  888.87   Rx  843.87  Chan  0629  #629

Tx  888.90   Rx  843.90  Chan  0630  #630

Tx  888.93   Rx  843.93  Chan  0631  #631

Tx  888.96   Rx  843.96  Chan  0632  #632

Tx  888.99   Rx  843.99  Chan  0633  #633

Tx  889.02   Rx  844.02  Chan  0634  #634

Tx  889.05   Rx  844.05  Chan  0635  #635

Tx  889.08   Rx  844.08  Chan  0636  #636

Tx  889.11   Rx  844.11  Chan  0637  #637

Tx  889.14   Rx  844.14  Chan  0638  #638

Tx  889.17   Rx  844.17  Chan  0639  #639

Tx  889.20   Rx  844.20  Chan  0640  #640

Tx  889.23   Rx  844.23  Chan  0641  #641

Tx  889.26   Rx  844.26  Chan  0642  #642

Tx  889.29   Rx  844.29  Chan  0643  #643

Tx  889.32   Rx  844.32  Chan  0644  #644

Tx  889.35   Rx  844.35  Chan  0645  #645

Tx  889.38   Rx  844.38  Chan  0646  #646

Tx  889.41   Rx  844.41  Chan  0647  #647

Tx  889.44   Rx  844.44  Chan  0648  #648

Tx  889.47   Rx  844.47  Chan  0649  #649

Tx  889.50   Rx  844.50  Chan  0650  #650

Tx  889.53   Rx  844.53  Chan  0651  #651

Tx  889.56   Rx  844.56  Chan  0652  #652

Tx  889.59   Rx  844.59  Chan  0653  #653

Tx  889.62   Rx  844.62  Chan  0654  #654

Tx  889.65   Rx  844.65  Chan  0655  #655

Tx  889.68   Rx  844.68  Chan  0656  #656

Tx  889.71   Rx  844.71  Chan  0657  #657

Tx  889.74   Rx  844.74  Chan  0658  #658

Tx  889.77   Rx  844.77  Chan  0659  #659

Tx  889.80   Rx  844.80  Chan  0660  #660

Tx  889.83   Rx  844.83  Chan  0661  #661

Tx  889.86   Rx  844.86  Chan  0662  #662

Tx  889.89   Rx  844.89  Chan  0663  #663

Tx  889.92   Rx  844.92  Chan  0664  #664

Tx  889.95   Rx  844.95  Chan  0665  #665

Tx  889.98   Rx  844.98  Chan  0666  #666

Upper Set Part 1 (667-799)

Tower Freq.  Mobile Freq.    11xxxx# Channel

Tx  890.01   Rx  845.01  Chan  0667  #667

Tx  890.04   Rx  845.04  Chan  0668  #668

Tx  890.07   Rx  845.07  Chan  0669  #669

Tx  890.10   Rx  845.10  Chan  0670  #670

Tx  890.13   Rx  845.13  Chan  0671  #671

Tx  890.16   Rx  845.16  Chan  0672  #672

Tx  890.19   Rx  845.19  Chan  0673  #673

Tx  890.22   Rx  845.22  Chan  0674  #674

Tx  890.25   Rx  845.25  Chan  0675  #675

Tx  890.28   Rx  845.28  Chan  0676  #676

Tx  890.31   Rx  845.31  Chan  0677  #677

Tx  890.34   Rx  845.34  Chan  0678  #678

Tx  890.37   Rx  845.37  Chan  0679  #679

Tx  890.40   Rx  845.40  Chan  0680  #680

Tx  890.43   Rx  845.43  Chan  0681  #681

Tx  890.46   Rx  845.46  Chan  0682  #682

Tx  890.49   Rx  845.49  Chan  0683  #683

Tx  890.52   Rx  845.52  Chan  0684  #684

Tx  890.55   Rx  845.55  Chan  0685  #685

Tx  890.58   Rx  845.58  Chan  0686  #686

Tx  890.61   Rx  845.61  Chan  0687  #687

Tx  890.64   Rx  845.64  Chan  0688  #688

Tx  890.67   Rx  845.67  Chan  0689  #689

Tx  890.70   Rx  845.70  Chan  0690  #690

Tx  890.73   Rx  845.73  Chan  0691  #691

Tx  890.76   Rx  845.76  Chan  0692  #692

Tx  890.79   Rx  845.79  Chan  0693  #693

Tx  890.82   Rx  845.82  Chan  0694  #694

Tx  890.85   Rx  845.85  Chan  0695  #695

Tx  890.88   Rx  845.88  Chan  0696  #696

Tx  890.91   Rx  845.91  Chan  0697  #697

Tx  890.94   Rx  845.94  Chan  0698  #698

Tx  890.97   Rx  845.97  Chan  0699  #699

Tx  891.00   Rx  846.00  Chan  0700  #700

Tx  891.03   Rx  846.03  Chan  0701  #701

Tx  891.06   Rx  846.06  Chan  0702  #702

Tx  891.09   Rx  846.09  Chan  0703  #703

Tx  891.12   Rx  846.12  Chan  0704  #704

Tx  891.15   Rx  846.15  Chan  0705  #705

Tx  891.18   Rx  846.18  Chan  0706  #706

Tx  891.21   Rx  846.21  Chan  0707  #707

Tx  891.24   Rx  846.24  Chan  0708  #708

Tx  891.27   Rx  846.27  Chan  0709  #709

Tx  891.30   Rx  846.30  Chan  0710  #710

Tx  891.33   Rx  846.33  Chan  0711  #711

Tx  891.36   Rx  846.36  Chan  0712  #712

Tx  891.39   Rx  846.39  Chan  0713  #713

Tx  891.42   Rx  846.42  Chan  0714  #714

Tx  891.45   Rx  846.45  Chan  0715  #715

Tx  891.48   Rx  846.48  Chan  0716  #716

Tx  891.51   Rx  846.51  Chan  0717  #717

Tx  891.54   Rx  846.54  Chan  0718  #718

Tx  891.57   Rx  846.57  Chan  0719  #719

Tx  891.60   Rx  846.60  Chan  0720  #720

Tx  891.63   Rx  846.63  Chan  0721  #721

Tx  891.66   Rx  846.66  Chan  0722  #722

Tx  891.69   Rx  846.69  Chan  0723  #723

Tx  891.72   Rx  846.72  Chan  0724  #724

Tx  891.75   Rx  846.75  Chan  0725  #725

Tx  891.78   Rx  846.78  Chan  0726  #726

Tx  891.81   Rx  846.81  Chan  0727  #727

Tx  891.84   Rx  846.84  Chan  0728  #728

Tx  891.87   Rx  846.87  Chan  0729  #729

Tx  891.90   Rx  846.90  Chan  0730  #730

Tx  891.93   Rx  846.93  Chan  0731  #731

Tx  891.96   Rx  846.96  Chan  0732  #732

Tx  891.99   Rx  846.99  Chan  0733  #733

Tx  892.02   Rx  847.02  Chan  0734  #734

Tx  892.05   Rx  847.05  Chan  0735  #735

Tx  892.08   Rx  847.08  Chan  0736  #736

Tx  892.11   Rx  847.11  Chan  0737  #737

Tx  892.14   Rx  847.14  Chan  0738  #738

Tx  892.17   Rx  847.17  Chan  0739  #739

Tx  892.20   Rx  847.20  Chan  0740  #740

Tx  892.23   Rx  847.23  Chan  0741  #741

Tx  892.26   Rx  847.26  Chan  0742  #742

Tx  892.29   Rx  847.29  Chan  0743  #743

Tx  892.32   Rx  847.32  Chan  0744  #744

Tx  892.35   Rx  847.35  Chan  0745  #745

Tx  892.38   Rx  847.38  Chan  0746  #746

Tx  892.41   Rx  847.41  Chan  0747  #747

Tx  892.44   Rx  847.44  Chan  0748  #748

Tx  892.47   Rx  847.47  Chan  0749  #749

Tx  892.50   Rx  847.50  Chan  0750  #750

Tx  892.53   Rx  847.53  Chan  0751  #751

Tx  892.56   Rx  847.56  Chan  0752  #752

Tx  892.59   Rx  847.59  Chan  0753  #753

Tx  892.62   Rx  847.62  Chan  0754  #754

Tx  892.65   Rx  847.65  Chan  0755  #755

Tx  892.68   Rx  847.68  Chan  0756  #756

Tx  892.71   Rx  847.71  Chan  0757  #757

Tx  892.74   Rx  847.74  Chan  0758  #758

Tx  892.77   Rx  847.77  Chan  0759  #759

Tx  892.80   Rx  847.80  Chan  0760  #760

Tx  892.83   Rx  847.83  Chan  0761  #761

Tx  892.86   Rx  847.86  Chan  0762  #762

Tx  892.89   Rx  847.89  Chan  0763  #763

Tx  892.92   Rx  847.92  Chan  0764  #764

Tx  892.95   Rx  847.95  Chan  0765  #765

Tx  892.98   Rx  847.98  Chan  0766  #766

Tx  893.01   Rx  848.01  Chan  0767  #767

Tx  893.04   Rx  848.04  Chan  0768  #768

Tx  893.07   Rx  848.07  Chan  0769  #769

Tx  893.10   Rx  848.10  Chan  0770  #770

Tx  893.13   Rx  848.13  Chan  0771  #771

Tx  893.16   Rx  848.16  Chan  0772  #772

Tx  893.19   Rx  848.19  Chan  0773  #773

Tx  893.22   Rx  848.22  Chan  0774  #774

Tx  893.25   Rx  848.25  Chan  0775  #775

Tx  893.28   Rx  848.28  Chan  0776  #776

Tx  893.31   Rx  848.31  Chan  0777  #777

Tx  893.34   Rx  848.34  Chan  0778  #778

Tx  893.37   Rx  848.37  Chan  0779  #779

Tx  893.40   Rx  848.40  Chan  0780  #780

Tx  893.43   Rx  848.43  Chan  0781  #781

Tx  893.46   Rx  848.46  Chan  0782  #782

Tx  893.49   Rx  848.49  Chan  0783  #783

Tx  893.52   Rx  848.52  Chan  0784  #784

Tx  893.55   Rx  848.55  Chan  0785  #785

Tx  893.58   Rx  848.58  Chan  0786  #786

Tx  893.61   Rx  848.61  Chan  0787  #787

Tx  893.64   Rx  848.64  Chan  0788  #788

Tx  893.67   Rx  848.67  Chan  0789  #789

Tx  893.70   Rx  848.70  Chan  0790  #790

Tx  893.73   Rx  848.73  Chan  0791  #791

Tx  893.76   Rx  848.76  Chan  0792  #792

Tx  893.79   Rx  848.79  Chan  0793  #793

Tx  893.82   Rx  848.82  Chan  0794  #794

Tx  893.85   Rx  848.85  Chan  0795  #795

Tx  893.88   Rx  848.88  Chan  0796  #796

Tx  893.91   Rx  848.91  Chan  0797  #797

Tx  893.94   Rx  848.94  Chan  0798  #798

Tx  893.97   Rx  848.97  Chan  0799  #799

Upper Set Part 2 (991-1023)

Tower Freq.  Mobile Freq.    11xxxx# Channel

Tx  869.04   Rx  824.04  Chan  0991  #800

Tx  869.07   Rx  824.07  Chan  0992  #801

Tx  869.10   Rx  824.10  Chan  0993  #802

Tx  869.13   Rx  824.13  Chan  0994  #803

Tx  869.16   Rx  824.16  Chan  0995  #804

Tx  869.19   Rx  824.19  Chan  0996  #805

Tx  869.22   Rx  824.22  Chan  0997  #806

Tx  869.25   Rx  824.25  Chan  0998  #807

Tx  869.28   Rx  824.28  Chan  0999  #808

Tx  869.31   Rx  824.31  Chan  1000  #809

Tx  869.34   Rx  824.34  Chan  1001  #810

Tx  869.37   Rx  824.37  Chan  1002  #811

Tx  869.40   Rx  824.40  Chan  1003  #812

Tx  869.43   Rx  824.43  Chan  1004  #813

Tx  869.46   Rx  824.46  Chan  1005  #814

Tx  869.49   Rx  824.49  Chan  1006  #815

Tx  869.52   Rx  824.52  Chan  1007  #816

Tx  869.55   Rx  824.55  Chan  1008  #817

Tx  869.58   Rx  824.58  Chan  1009  #818

Tx  869.61   Rx  824.61  Chan  1010  #819

Tx  869.64   Rx  824.64  Chan  1011  #820

Tx  869.67   Rx  824.67  Chan  1012  #821

Tx  869.70   Rx  824.70  Chan  1013  #822

Tx  869.73   Rx  824.73  Chan  1014  #823

Tx  869.76   Rx  824.76  Chan  1015  #824

Tx  869.79   Rx  824.79  Chan  1016  #825

Tx  869.82   Rx  824.82  Chan  1017  #826

Tx  869.85   Rx  824.85  Chan  1018  #827

Tx  869.88   Rx  824.88  Chan  1019  #828

Tx  869.91   Rx  824.91  Chan  1020  #829

Tx  869.94   Rx  824.94  Chan  1021  #830

Tx  869.97   Rx  824.97  Chan  1022  #831

Tx  870.00   Rx  825.00  Chan  1023  #832 or #0

T*R*I*K*-*C*L*I*P********************************************S*E*C*T*I*O*N***1*1

I got this from a BBS in the (708) are code.  It had no name associated

with it.  Since NOONE has mailed me any other info on it, I will keep this

in the bible until someone bitches or sends me something tangible.  Besides,

with Loadkit so readily available, who has the time to mess with it?

-ML

MOTOROLA "TRIK-CLIP"

This is the plans I received for the Flip.  Supposedly if one knew the

pinouts on the other moto phones one could transpose. (maybe!)  I never

tested this so I don't know if it works.  The chip in the flip the text

is talking about is a 32 pin square plcc

After Phone Disassembly Locate 27c512 EPROM on phone board.  This is

On The Upper Right Side Of The Display Next To The Roam Indicator.

This Is a 32 pin Square device.  **Note the dot and beveled edge

for pin orientation (the dot is pin 1) Count to the left

counter clock wise 2 3 4 5 and so on.  To the Right or clockwise

of the dot is pin 32 Vcc.  This will aid you in your count to find

pin 25 which is the EPROM output enable.  This pin is at ground or

Vss - Level.  **Note Pin 25 on EPROM in phone must be lifted from

the phone board ground or Vss state.  Use an X-acto Knife and or

soldering iron and tools to cut pin at board level where pin

narrows.  Do not bend wide part of pin up on EPROM as this could

break off of EPROM.  Also Wide Part of pin Will be used to make

contact with EPROM test clip adapter.  The EPROM test clip adapter

will take pin 25 to logic high through an 8 to 10 thousand

resistor to pin 32 Vcc.  This will Gate off all data Commands from

the phone board EPROM and allow the EPROM test clip adapter to

take over.  **Note test clip could touch narrow part of cut off

pin on board and cause phone not to power up please remove or fold

down as low as possible so test clip only touches side of EPROM.

After programming is complete put pin 25 back together or find a

suitable ground or Vss - source.  The phone will power up and work

without pin 25 put back together but for long term precaution

put back to a logic zero or ground to enable the output enable.

To use the EPROM test clip adapter pull the locking wedge on the

test clip into the upper position.  Seat the EPROM test clip adapter

onto the EPROM in the phone.  Make sure to orient the dot and

beveled edge with each other.  Push the locking wedge down to lock

the EPROM test clip adapter onto the EPROM in the phone.  Hook up

the programming cable to the computer and plug into the jack on the

base of the phone.  Also hook up the loose lead with a jumper to the

center terminal between the battery contacts.  Turn power on green

light on phone display should come on then a complete display test

will light up after that the no service will blink along with the

signal level mark in corner of display.  If the antenna is still on

the phone it could change to roam or something else.  I  suggest

remove the antenna so the cell sight will not see you.  If you do

not get a power on test with the display there are 3 possible

things (1) pin 25 on phone board is touching the test clip this

can be checked by looking with a volt meter at pin 25 where

resistor connects for 4 to 5 volts pos with reference to ground.

(2) Test clip is not sitting on chip good some times you have pull

the test clip up off of the EPROM a 64th of an inch all the way

around.  (3) there is corrupt data, Pull the EPROM test clip off

Phone check to see if power on display is there.

Computer see if data or phone number or cell sight code or data

whole is ok I've seen the cell sight ID corrupt and the phone play

dead on  the power on test. The test clip sometimes needs

maintenance look at the gold pins.

Make sure all the pins are level with the edge of it.  If not take

an X-acto or pin and lightly bend them out so they are along the

edge of the plastic of the test clip.

Always check to see if EPROM in phone contacts are clean before

putting test clip on.  **Note when test clip is on phone - only

change the ESN only. *The other data phone number lock and so on

can be changed without the test clip and should be done so.

The software version in the test clip is 9148 you will see this in

the right corner of the computer.  Sometimes the program will crash

during the ESN write this will put all zeros in the ESN field

check the test clip try again.  Sometimes I've had to do this 3 or 4

times.  Also watch the phone display for codes I've seen at the end

of a write the code (FO8) just before power down I've had no

problem there but during the key write (FO8) means I've crashed.

Also during the time when the program is counting back into the

phone I've had (F1O) show up in the display of the phone this

problem means the next time you may not get the power on display

test pull test clip read phone check data to see if cell sight

code is corrupt or some other data correct try again.  A word of

caution do not push on EPROM on top of test clip as this could

seat EPROM lower into adapter and cause bad contact.  To remove

test clip pull locking wedge up to unlock the EPROM test clip

adapter from the EPROM in the phone.  Continue pulling up to lift

the EPROM test clip adapter from the EPROM in the phone.

P*A*G*E*R*S**************************************************S*E*C*T*I*O*N***1*2

Subject: BRAVO pagers - undocumented test features

SELF TEST:

TO PUT UNIT INTO A SELF TEST TURN OFF PAGER. NOW HOLD DOWN THE

GRAY ARROW KEY AND BLACK LOCK KEY AT THE SAME TIME AND TURN ON

PAGER. THIS TELLS THE CPU IN PAGER TO GO INTO A SELF TEST. YOU WILL

GET A 2 SECOND LONG BEEP, RELEASE THE GRAY & BLACK BUTTON AND PUSH

THE GRAY BUTTON BEFORE THE 2 SECOND BEEP ENDS. IF YOU DID ALL THIS

IN TIME YOU WILL HAVE "SPL" OR "PAGING P?" AND NOT THE DOTTED LINE

YOU ARE USED TO SEEING WHEN YOU TURN ON PAGER . BY PRESSING THE

GRAY KEY IT WILL GO TO A DISPLAY TEST, PRESS AGAIN AND YOU WILL GET

THE PAGERS CAPCODE (CAPCODE IS THE UNIQUE SERIAL NUMBER WHICH THE

PAGING TRANSMITTERS TRANSMITS TO YOUR PAGER TO TURN ON YOUR PAGER

WHEN SOMEONE PAGES YOU). WAIT AND IN ABOUT 3 SECONDS IT WILL

DISPLAY YOUR SECOND CAPCODE (IF YOU HAVE ONE-MOST DON'T) PRESS THE

GRAY KEY AGAIN AND IT WILL CHECK CONTROLS, PRESS IT AGAIN AND IT

WILL TEST VIBRATOR FUNCTION (IF YOUR PAGER HAS IT). TURN OFF PAGER

AND TURN ON AGAIN TO DISABLE SELF TEST.

SPECIAL PROGRAMMED FEATURES:

TAKE OFF BATTERIES CLIP AND IN CENTER TOWARD THE FRONT OF

PAGER YOU WILL SEE A PRINTED CIRCUIT BOARD EDGE PINS (JUST LIKE THE

BACK SIDE OF A NETENDO CARTAGE. THIS EDGE PINS ARE PLUGGED INTO A

CORE PROGRAMMER. THE PROGRAMMER CAN CHANGE.

CAPCODES: SEE ABOVE

AUTORESET TO MANUAL: YOUR PAGER IN AUTORESET WILL BEEP 8 TIMES

THEN STOP BEEPING. MANUAL RESET THE BEEPER WILL KEEP BEEPING TILL

THE COWS COME HOME OR YOU PUSH A BUTTON TO LOOK AT THE MESSAGE.

DISPLAY: ENGLISH PROMPTS OR INTERNATIONAL-SYMBOL SCREENS

DISPLAYED.

SILENT MODE CHIRP: FOR A SINGLE BEEP WHEN YOUR PAGED. NOT FOR

USE ON VIBRATOR PAGERS.

BEEP ON BAD DATA: YOUR PAGER HEARS IT'S CAPCODE BUT RECEIVED

BAD DISPLAY MESSAGE, IT WILL PUT "EEE" ACROSS DISPLAY TO SHOW BAD

RECEIVE. IF THIS IS FEATURE IS NOT ENABLED AND YOU RECEIVE BAD DATA

YOUR PAGER WILL NOT BEEP AND YOU WILL HAVE NO IDEA SOMEONE TRYED TO

PAGE YOU.

******************************************************************

NOW LETS SAY YOU ARE UNHAPPY WITH YOUR PAGING COMPANY "A" BUT OWN

YOUR PAGER. YOUR $200.00+ PAGER IS TUNED TO THEIR FREQUENCY AND YOU

WANT TO GO TO ANOTHER PAwriteOMPANY BUT NOT LOSE ALL THE MONEY YOU

SPENT FOR YOUR PAGER. THE ANSWER IS TO RECRYSTAL PAGER TO THE NEW

FREQUENCY OF COMPANY "B". BUT WE MUST ANSWER SOME QUESTIONS FIRST

TO SEE WHAT IT WILL COST.

1. WHAT IS YOUR PAGERS CODING FORMAT (POCSAG) OR (GSC)

THE EASY WAY TO TELL IS TO DO A SELF TEST AND READ

CAPCODE. IF IT'S 7 NUMBERS IT'S POCSAG. IF IT'S 6 NUMBERS

AND 1 LETTER IT'S GSC. IF YOUR PAGER DOES NOT MATCH THE

SAME CODING FORMAT AS COMPANY "B" IT WILL COST MORE THEN

IT'S WORTH TO CHANGE.

2. WHAT BAUD RATE IS YOUR PAGER WORKING AT ? DO SELF TEST AND

IF DISPLAY SHOWS PAGING P1 PAGER IS WORKING AT 1200 BAUD OTHER WISE

YOU ARE SAFE TO ASSUME 512 BAUD IT MUST MATCH COMPANY "B" BAUD RATE

TO BE WORTH YOUR TIME.

3. ARE YOU IN THE SAME FREQUENCY BAND 931 MHz OR 450 MHz ETC.

IF COMPANY "A" AND COMPANY "B" ARE NOT IN SAME BAND IT WILL TAKE A

NEW RECEIVER BOARD TO CONVERT PAGER AND COST TO MUCH TO TRY.

IF ALL THE ANSWERS ABOVE SHOW YOU ARE COMPATIBLE YOU CAN CALL

COMPANY "B" AND TELL THEM YOU WANT TO DO BUSINESS WITH THEM AND

NEED A CAPCODE NUMBER SO YOU CAN GET PAGER RECRYSTALED AND HAVE A

CAPCODE PROGRAMMED AT THE SAME TIME.

NOW YOU CAN HAVE COMPANY "B" RECOMMEND A SHOP THAT WILL

RECRYSTAL PAGER OR LOOK UP ONE YOURSELF.

(sorry for the all caps, that was how I received it and I am lazy.   -ML)

D*I*S*C*L*A*I*M*E*R******************************************S*E*C*T*I*O*N***1*3

DISCLAIMER:  I, Mike Larsen, accept NO responsibility for people using any

info within this text for fraudulent purposes.  I did not intend for the info

to be used towards fraud or theft of services.  The main reason I spent

hundreds of hours creating and compiling this information is because

programming fees are BULLSHIT and they know it.  Programming fees are the

equivalent to the Refrigerator installer guy charging 50 bucks to plug the

damn thing in to the wall.

Oh, by the way, I forgot to mention in the above disclaimer that I do

nothing fraudulent with MY fone.  I pay a bill and everything and can prove

it.  So will the "feds" or whoever the Internet Gestapo is that's been sending

me mail about me being under their "watchful eye", please go for someone else

that's dealing child pornography or asking for WaReZ?  Thanks.

The sole reason I compiled this info into book form is to let people

that are capable, work on their phone.  Again, I did not compile this for the

purpose of fraud.

mjlarsen@hotmail.com

******T*H*E***E*N*D*****T*H*E***E*N*D******T*H*E***E*N*D*****T*H*E***E*N*D******