# Protein gel

Separating gel preparation

Stock prep (for about 10 gels)

|  | 12% gel | 15% | 18% | 21% |
| --- | --- | --- | --- | --- |
| Distilled water | 13.4 ml | 9.4 | 5.4ml | 1.4ml |
| 1.5M Tris-HCL, pH 8.8 | 10 ml | 10 ml | 10ml | 10ml |
| 10% (w/v) SDS stock | 400ul | 400ul | 400ul | 400ul |
| Acrylamide/bis (30% stock) | 16 ml | 20 ml | 24ml | 28ml |

For each gel, use ~5ml of “stock prep” plus

| 10% ammonium persulfate | 25ul |
| --- | --- |
| TEMED | 2.5ul |

Stacking gel preparation

Stock prep

| Distilled water | 12.8ml |
| --- | --- |
| .5M Tris-HCL, pH 6.8 | 5ml |
| 10% (w/v) SDS | 200ul |
| Acrylamide/bis (30% stock) | 2.6ml |

For each gel, use ~2ml of “stock prep” plus

| 10% ammonium persulfate | 25ul |
| --- | --- |
| TEMED | 5ul |

Stock solutions

Acrylamide/bis (30%T, 2.67%C)

87.6 g acrylamide (29.2g / 100ml)

2.4 g N’N’-bis-methylene-acrylamide (.8g/100ml)

make to 300ml with dd water.  Filter and store at 4C in the dark (30 days max)

1.5 M Tris-HCl, pH 8.8

27.23 g Tris base (18.15g / 100 ml)

~80ml dd water

adjust to pH 8.8 with 1N HCl.  Make to 150 ml with dd water and store at 4C

.5 M Tris-HCl, pH 6.8

6g Tris base

~60ml dd water

adjust to pH 6.8 with 1N HCl.  Make to 100 ml with distilled water and store at 4C

10% SDS

dissolve 10g SDS in water with gentle stirring and bring to 100 ml with dd water

5X sample buffer (SDS reducing buffer) makes 8ml

| dd water | 4ml |
| --- | --- |
| .5 M Tris-HCl, pH6.8 | 1ml |
| Glycerol | .8ml |
| 10%(w/v) SDS | 1.6ml |
| 2-b-mercaptoethanol | .4ml |
| .05% (w/v) bromophenol blue | .2ml |

Dilute the sample at least 1:4 with sample buffer, and heat at 95C for 4 min

5X electrode (running) buffer, pH 8.3 (enough for 10 runs)

| Tris base | 9g (15g/L) |
| --- | --- |
| Glycine | 56.4g |
| SDS | 3g (5g/L) |

To 600 ml with dd water

Store at 4C.  dilute 60ml 5X stock with 240 ml dd water for one run