SERVICE PROVIDER TO CUSTOMER

PERFORMANCE REPORTING
BUSINESS AGREEMENT

NMF 503

Issue 1.0

March 1997

ã Network Management Forum 1997.

Members of the Network Management Forum are granted limited copyright waiver, under the terms of the Agreement on Intellectual Property Rights, to reproduce this document.

Disclaimer and copyright statements

The Network Management Forum (“NMF”) has made every effort to assure that the contents of this document are accurate; however, due to the inherent complexity in the design and implementation of protocols, no liability is accepted for any errors or omissions or for consequences of any use made of this document.  Under no circumstances will the NMF be liable for direct or indirect damages or any costs or losses resulting from the use of this specification.  The risk of designing and implementing products in accordance with this specification is borne solely by the user of this specification.  This document may involve a claim of patent rights by one or more NMF members, pursuant to the agreement on Intellectual Property Rights between NMF and its members, and by non-members of NMF.

Acknowledgements

SMART (Service Management Automation and Re-engineering Team ) work is a genuinely collaborative effort. I would like to thank the following people for contributing their time and expertise to the production of this specification.

| Klaus Nolte | Alcatel | Bill Neale | SITA |
| --- | --- | --- | --- |
| David Milham | BT | Lucia Marchisio | STET - CSELT |
| Patrick Rhude | CrossKeys | Fabio Macuglia | STET - Sodalia |
| Marc Malaise | Digital | Alberto Pisani | Telecom Italia |
| Per-Ake Wiberg | Ericsson | Daniele Toro | Telecom Italia |
| Kidane Woldegiorgis | Ericsson | Peter Huckett | TTC |
| Frank Birch | Fujitsu Europe | Adrian Tang | UMKC/CSTP |
| Lightsey Wallace | Hekimian | Jason Chen | USWEST |
| Hal Folts | NASA | Andy McGarty | X-Cel |
| Mike Kelly | Nortel | Douglas Kearney | X-Cel |

Hans Rudolf Stucki (Unisource Business Networks Switzerland, Ltd.), team chairman.

About this document

Revision history

| Version | Date | Comments |
| --- | --- | --- |
| 0.3b | May 20, 1996 | Final Draft of Business Requirements |
| 0.4 | September 17, 1996 | Final Draft of Technical Requirements |
| 0.5 | February 28, 1997 | - Incorporated comments from informal NMF review on Business and Technical Requirements - Further developments by team members - Compiling of Business Requirements and Technical Requirements; reformatted in line with SMART template |
| 0.6 | March 10, 1997 | - Incorporated comments from Team members - Ready for informal NMF members review |

Contact for this document

The editor of this revision of the document can be contacted as follows:

Hans R Stucki

Product Manager

UBN (Switzerland) Ltd.

Schermenwaldstrasse 13

Ittigen, 3063

Switzerland

Phone	+41 31 688 8183

Fax	+41 31 688 8152

e-mail:	hans-rudolf.stucki@ubn.unisource.ch

Further information on NMF programs and documentation can be obtained by contacting the NMF office:

NMF
1201 Mt. Kemble Avenue
Morristown, NJ  07960 
USA
Tel: +1-201 425 1900
Fax: +1-201 425 1515
Web: http://www.nmf.org

How to obtain a copy

An electronic copy of this document in MS-Word V6.0 format can be obtained using ftp from the NMF server.  Alternatively, contact the NMF as indicated above. The file is located at:

199.183.141.190:specifications/service/busagreements/nmf503.doc

ftp.nmf.org: specifications/service/busagreements/nmf503.doc

Comments

Comments should be sent to the editor.

Note:  smartperf@nmf.org can be used as e-mail id for all SMART Performance Reporting Team members. Messages are distributed to all Team members automatically. The Team member list is found in the current NMF roster. In the case of questions, do not hesitate to contact one of the Team members.

Use of fonts

Italics:  Paragraphs in italics are notes by the authors to the reader to provide additional or background information.

CONTENTS

PREFACE

SMART Performance Reporting documents

1 INTRODUCTION TO THIS DOCUMENT

1.1 How this document will be used

1.2 Document structure

1.3 Key terms used in this document

2 BUSINESS OBJECTIVES

2.1 Introduction to the requirements

2.2 Current environment

2.3 Business need

2.4 Summary of business requirements

2.5 Expected beneficiaries

3 SCOPE OF INTERFACE REQUIREMENTS

3.1 Process orientation

3.1.1 Position within the NMF Service Management Business Process Model

3.1.2 Responsibility of the Performance Reporting process

3.2 Process functions addressed

3.3 How the interface is expected to be used

3.4 Scenarios to be addressed

3.4.1 Reporting with acknowledgement

3.4.2 Reporting via “mailing“ system

3.4.3 Reporting on demand

3.4.4 Modifying the reporting mode

4 FLOW OF INFORMATION

4.1 Workflow perspective

4.2 Event flow perspective

4.3 State model

4.4 Relationship between actual events and states

5 SPECIFIC REQUIREMENTS OF THE CUSTOMER TO SERVICE PROVIDER PERFORMANCE REPORTING INTERFACE

5.1 Contract (SLA) related requirements

5.2 Performance Report related requirements

5.3 Service Availability related requirements

5.4 General QoS requirements

5.5 Report generation and distribution requirements

5.6 Report frequency requirements

5.7 Report customisation requirements

6 GENERAL TECHNICAL REQUIREMENTS

6.1 Transport requirements

6.2 Connection requirements

6.3 Control and mapping requirements

6.4 Other general requirements

6.5 Security requirements

6.6 Performance of the interface

6.7 Methods of synchronisation upon session failure

6.8 Inventory synchronisation / database reconciliation

7 CONCLUSIONS

7.1 General remarks

7.2 Success criteria

7.3 Issues and risks

7.3.1 Issues

7.3.2 Risks

APPENDIX A - SUMMARY OF REQUIREMENTS

APPENDIX B - GLOSSARY OF TERMS, ACRONYMS AND ABBREVIATIONS

APPENDIX C - REFERENCES AND RELATED DOCUMENTS

APPENDIX D - STANDARDS

D.1 OVERVIEW

D.1.1 What we are looking for

D.1.2 Conditions

D.1.3 Essential findings

D.2 Document list

D.3 Document abstract

D.4 Directory

D.4.1 Management

D.4.2 Measurements

D.4.3 Reports

APPENDIX E - ASSUMPTIONS AND INFORMATION REQUIRED

E.1 Information required for Service Availability Calculation

E.2 Information required for further QoS Report Parameters

E.3 Information required from the network

E.4 The use of common objects

E.5 Initial administrations

E. 6 Data collection

E.7 Data computation and aggregation

APPENDIX F - TABLE OF FIGURES

# PREFACE

NMF is an international consortium of communications Service Providers and their suppliers, dedicated to promoting industry-wide agreements on how management information should be exchanged between a Service Provider and its Customers, its suppliers and/or other Service Providers. Such agreements are essential to achieve a seamless, cost-efficient operation where multiple companies are involved in the fulfilment of service requests, the management of services while in use by Customers, the correction of problems that might impair service quality, and the accurate billing of the Customer for services used.

These agreements are becoming an increasing necessity as regulatory forces change the landscape for Service Providers. The addition of new value-add Service Providers such as Internet Service Providers and the development of global alliances mean that the service delivery chain is being extended beyond the traditional, monopoly environment where a single company had end-to-end control over the provision and management of Customer services. At the same time, some companies are choosing to focus their business on network infrastructure, either providing high-capacity intercity links to Service Providers, or providing local access to Customer sites. Finally, as has been the case for many years, Service Providers and network operators rely on equipment suppliers’ element management systems to make it possible to manipulate the network to create Customer services, and to report meaningful information about the status of elements, in order that the network, and ultimately, Customer services, can be maintained at required levels of quality.

NMF members work together to determine how their processes should be linked, and how they should work with Customers’ and suppliers’ systems. They work together because they believe that to solve this type of problem alone requires extensive time and money (which is better spent developing new offerings) and that proprietary solutions will not be accepted in the marketplace.  They work together because they see that in these areas, there is no competitive advantage to be gained over exchanging data in a proprietary fashion, and that, conversely, they can considerably reduce both the time to development and the risk of making bad decisions.

These requirements and definitions pertain to a very specific area in which NMF members have determined that a common, industry-wide approach is warranted. They invite others to review these agreements, and to adopt the more specific information models and Solution Sets which will be developed to meet these requirements. They invite suppliers to become familiar with these requirements and definition, and to work with them on the details of how the requirements will be met through the application of standards and other industry-accepted specifications and technology.

## SMART Performance Reporting documents

When the SMART Performance Reporting Team started its work in early 1995, the individual deliverables (output documents) of the SMART process were not clearly defined. The team’s first deliverable was a “White Paper”, mainly based on the work done by the “Customer to Service Provider Trouble Administration Team”. In the mean time the SMART process became better structured and the outputs are now defined as follows:

During the requirements gathering process, the SMART Performance Reporting Team detected that there are only few (or even no) stable and unambiguous definitions dealing with service performance at the service management level. Service Providers and Customers have different views and understanding of terms such as  “Service”, “Service Availability”, and “SAP” (Service Access Point). It became clear that describing SAP concepts, service concepts, definitions of SLAs (Service Level Agreements), etc., would go far beyond the scope of a requirements document. As a result of this, the SMART Performance Reporting Team decided to develop a document dealing with terms, definitions and concepts entitled NMF Performance Reporting Definitions Document [ref  1]. The aims of the Definitions Document are to:

- serve as additional information to the requirements document

- define terms used by Service Providers and their Customers when negotiating SLA parameters

Further documents will specify the interoperable interfaces which meet the requirements outlined in this document, as follows:

- A document which defines the information model and the services to be offered by the interface in a protocol-independent format.

- A series of specifications which specify implementations of the interoperable interface in a specific protocol. At the current time of writing no specific protocol implementations are being specified

# 1	Introduction To This Document

This document defines the requirements for the exchange of management information within a model of service management business functions documented in the NMF’s Service Management Business Process Model [ref  2].

## 1.1	How this document will be used

This specification is aimed specifically at telecommunications Service Providers, their Customers and ISVs (Independent Software Vendors) providing performance management and reporting systems. The Performance Reporting functions are also sufficiently generic to be applicable to any generic Service Provider and their Customers.

- Business managers will be able to determine, using this document, whether their own requirements are fairly and completely represented by the document, such that implementation of an interface that meets the requirements would represent a good investment and a satisfactory solution to existing operational problems. For Service Provider organisations who have already implemented Performance Reporting functions, this document will provide a rationale for extending that implementation into the wider market as part of the total Service Provider’s offering.

- For the companies who worked together to identify the requirements (and others as become interested in implementing solutions that meet the requirements) this document will provide sufficient information to develop an information model and protocol-neutral interface definition that meets the requirements. This in turn will provide sufficiently detailed information to develop a detailed implementation specification at some point.

## 1.2	Document structure

Section 1	Introduces the document.

Section 2	States the business objectives and problems that are driving the need for this interface, and the high level operational requirements that must be met. Establishes the scope of the interface agreement, identifying the parties to the interface and the functions that are the subject of the interface.

Section 3	Establishes the scope of the interface agreement, identifying the specific processes that are invoked, the parties to the interface, the functions that are the subject of the interface, and the types of information that are to be exchanged.

Section 4	Describes the flow of information across the interface, using multiple diagrams to represent different perspectives.

Section 5	Contains the specific requirements, in detail, and presented in such a way as they can be traced through to implementation.

Section 6	Lists any general technical requirements that may need to be considered in the development of the Information Model or the development of a Solution Set.

Section 7	Offers conclusions and insights from the team members who prepared the requirements document, including any issues they perceive, or their expectations of companies to meet the requirements.

Appendix A	Lists the outlined requirements as a summary

Appendix B	Provides a glossary of terms, acronyms and abbreviations

Appendix C	Lists the referenced and related documents

Appendix D	Provides a summary on already reviewed standards by the team

Appendix E	Provides further details on assumptions and required information

Appendix F	Contains the table of figures

## 1.3	Key terms used in this document

Service Provider (Role)

In this document, the term SERVICE PROVIDER  refers to companies who provide telecommunications services as a business. Service Providers may operate networks, or they may simply integrate the services of other providers (who operate networks) in order to deliver a total service to their Customers.

Providing telecommunications service to any one End-Customer may involve multiple Service Providers, where one provider may "sub-contract" with other providers to fulfil the Customer's needs. When necessary to account for this relationship, the term Service Provider is used in this document to describe the company responsible for providing service to an End-Customer. The term Other Provider is used to denote companies which have a sub-contractual responsibility.

In Performance Reporting the role of a Service Provider is to provide the Performance Report to his Customer.

Customer (Role)

In this document, the term CUSTOMER  refers to companies who make use of telecommunications services provided by a Service Provider. The Customer is the ultimate buyer of a network service.

It may be that a Customer is also an organisation who provides services to their Customers. It is confirmed that the current work of the SMART Performance Reporting Team also includes Service Providers in the role of Customers, where the nature of the SLA means that these Service Providers behave equivalently to Customers.

The term “Customer” is equal to the term "End-Customer" also used in this, and in referenced [NMF] documents.

In Performance Reporting the role of a Customer is to get the Performance Report from his Service Provider.

SLA (Service Level Agreement)

In this document, the term SLA refers to the legal contractual agreement between two parties for the service under contract, specifically between the Customer and the Service Provider. Other aspects of the level of service require documentation by both the Customer and the Service Provider and may or may not become part of the contract. Performance Reporting uses the SLA as the reference and does not address the other parameters known to exist, but which are not part of the SLA.

Performance Report

In this document, the term PERFORMANCE REPORT refers to a “document” providing information on how a contracted service was:

- delivered by the Service Provider in terms of SLA fulfilment

- used by the Customer in terms of capacity, traffic volumes, etc.

Quality of Service (QoS)

In this document, the term QUALITY OF SERVICE refers to a value providing an overall view as to how close the service offered is, to the contracted service. The overall QoS, delivered to the customer, consists of two major parts:

- operational criteria (related to the performance of an organisation)

- service specific performance criteria (related to the performance of a service provider’s service platform. These criteria are typically those that are fundamental to the operation of the service.

Service Access Point (SAP)

In this document, the term SERVICE ACCESS POINT (SAP) refers to a logical element located on the interface between the Customer’s domain and the Service Provider’s domain and it represents the point to which a service is delivered.

# 2	Business objectives

This section describes the business problem to be solved, and provides a high-level view of the requirements to be satisfied through common information exchange. It is intended to make clear why investment in such a common exchange is beneficial, and how each stakeholder should expect to benefit. Finally, this section contains illustrative scenarios of the business relationships that may exist between entities, and the purposes for which the agreement might be used.

## 2.1	Introduction to the requirements

Telecommunications Service Users and managers of private corporate networks face evermounting pressures to cut costs, streamline operations, and improve the quality and delivery of their services. Increasingly, Telecommunications Service Users (hereinafter referred as “Customer(s)”) are entering into SLAs  with their Service Providers, but no industry guidelines exist to help tailor SLAs using agreed-upon elements. Service performance is one element within the SLA. Extensive research, based on an interview process at the senior business level of Service Providers in North America, Europe and Asia shows the need for the development of a common understanding of service performance and the standardisation of service performance definitions. The focus is the Customer’s business processes and the key issue is to report on service performance to support these processes.

The SMART Performance Reporting Team is considering the potential for automation of the interface between Service Provider and Customer for Performance Reporting associated with SLAs applying at that interface. The work is focused on the following areas:

- Determination of the appropriate performance measures which are relevant to support of a service, or a group of services, provided by the Service Provider to the Customer concerned. The aim is to identify generic and service-specific measures, with a view to prioritising the business benefit which support of each measure would bring.

- Modelling of the process flows which are involved in Performance Reporting and identification of functions concerned with the related collection and processing of performance statistics and data by the Service Provider, the Customer and the network. The aim is to define the business processes which are involved in supporting and using Performance Reporting.

Note: The actual design of these processes may be proprietary to the Customer and the Service Provider.  Therefore a general process will be developed in order to define the working interfaces between the Service Provider and the Customer.

- Identification of the important service descriptors which relate to the Performance Reporting task, as well as potentially to other tasks such as service order management. The aim is to ensure a close linkage between the performance measures and the services which are supplied by the Service Provider to the Customer.

There is no list of common SLA terms from which to select an appropriate subset to create a customer-specific agreement. In addition, there are no common definitions of terms to be used at the service level.  The objectives of the SMART Performance Reporting Team work include the development and definition of  such terms.

In addressing these areas, supporting activities within the Team's work include agreeing on the relevant terminology to improve co-operation of common understanding of the issues, and identifying areas of standards which relate to the work and can be used as a basis for agreements under discussion by the Team. Another area of interest is the transport or carriage mechanism which is used to communicate performance information from the Service Provider to the Customer.

There is potential for some common work between the SMART Performance Reporting Team and other Teams concerned with the Service Provider-Customer interface, in establishing a broader mechanism which can be used for a variety of process interactions across this interface. The work on SLA definition is seen as important in this respect. The specification of a list of SLA terms (see above) is intended to be developed into a common framework in which SLAs for different services can be defined. This can then be used more generally within the SMART work (i.e. by other SMART Teams), and is an important output of the work of the SMART Performance Reporting Team. Because of the fundamental significance of SLAs in the work of SMART in general, the focus of this aspect of the Performance Reporting Team's work is on definition, rather than automation.

## 2.2	Current environment

The Performance Reporting process between Customers and Service Providers is part of a business relationship in which the Service Provider has contracted to provide a set of services to a specific level of quality as defined in a Customer’s contract or Service Level Agreement (SLA).  The Performance Reporting Process is responsible for providing whatever performance reports are requested by the Customer and available from the Service Provider.  Routine reports include performance of a service against an SLA, reports of any developing capacity problems, and reports of Customer usage patterns.  In addition, this process responds to performance inquiries from the Customer. The mutual objective of the Customer and the Service Provider is to ensure that the Service Level Agreement is met. The performance report provides the needed information to determine to what extend the Service Provider has met the contracted quality of service targets.

With the advent of competition, Service Providers are increasingly providing SLAs as a way to distinguish the quality and reliability of their services from those of their competitors.  The SLAs are contractual agreements that state the terms the Service Provider will meet.  They can include many different variables, ranging from meeting committed due dates on installation, to restoring service within a stated interval in the event of an outage.  Service Providers are finding increasing difficulty managing these SLAs, for several reasons:

- There is no list of common SLA terms from which to select an appropriate subset to create a Customer-specific agreement.  For that reason, each Customer (especially a large Customer buying complex services) is likely to have its own set of favourite terms, making it difficult for the provider to keep track of the different provisions.

- There are no common definitions of terms to be used at the service level. Although many terms exist to describe network and element availability, for example, the term has widely different application at the service level. This leads to misunderstandings and dissatisfaction of the Customer. For example, if the Customer believes that "availability" means their application is running without a problem, and the Service Provider uses the same term to mean that the service is working (even if impaired), then a mismatch in expectations is inevitable. Similarly, if a Service Provider relies on multiple sub-contracting providers for pieces of a service, the lack of common performance terms makes it virtually impossible to construct a picture of end-to-end performance.

- The types of information provided to Customers is often not of interest to some or all levels of management within the Customer's organisation. A count of errored seconds may be relevant to a technician, but the person making buying decisions may only care whether the company’s critical applications are adequately supported - meaning that the service works almost all of the time and  the Service Provider makes every effort to reduce the impacts of any service outages.

(Source: [ref  3])

## 2.3	Business need

In Performance Reporting, the business need is to reach agreement on the information provided to the Customer, and on the corresponding information to be provided from the network elements.

Many Service Providers are realising that providing meaningful performance information to their Customers can improve Customer satisfaction with the overall service provided. They also see a benefit to negotiating common elements of a Service Level Agreement (SLA) to reduce the number of unique arrangements they must accommodate with individual Customers.

Some Service Providers and suppliers are reluctant to move toward common performance terms because they would prefer their Customers not be able to perform a direct comparison of their performance vis à vis their competitors. However, increasingly these providers are finding themselves playing the role of "Customer" to another provider or supplier, and the use of SLAs is extending throughout the service chain. Thus, for a variety of reasons, it is highly desirable for Service Providers to reach industry agreements in this area.

Solving Performance Reporting problems will eventually require agreements at the service, network and element layers of management, as shown in Figure 2/1.

Figure 2/1:  Performance Reporting requirements must be linked across management levels (Source: [ref  3])

Although requirements in this area are initially focused at the information to be provided to end Customers, over time these requirements will need to be matched to performance data that is available from the network level (and then from the element level) in order to construct performance statistics automatically.

## 2.4	Summary of business requirements

Business requirements within the scope of Performance Reporting include:

- Tracking delivered Service Level Quality against SLAs: Performance reports relate to the service level selected/contracted by the Customer. They report on the extent to which the Service Provider has met the pre-defined (agreed) quality of service. Guaranteed values as stated in the SLAs are part of the contract/agreements with the Customer. Therefore the Customer expects the Service Provider to report on these values.

- Ability to determine a basis for possible levels of refund under SLAs: If contracted quality of service levels are not met by the service provider, usually a compensation or discount scheme will be applied. In this case, the performance report shall provide the necessary information needed.

- Determine where service capacity needs to be increased or decreased: A Performance Report shall also provide information regarding the actual use of the service by the Customer. Typical items covered for a Packet Switched Service could be “Successful Calls”, “Unsuccessful Calls”, “Call Duration”, “Transmitted Segments”, “Received Segments”, etc., those for a PVC based Frame Relay Service could include “CIR-Rate-Usage”, “Transmitted/Received Frames”, „Discarded Frames“, etc.

- Reporting on end-to-end performance: Customers want to get an overall picture regarding the quality of the contracted service(s). There are many well developed tools addressing performance issues for numerous network elements. Customers are not interested in the summarisation of all the individual network element performance numbers to get an end-to-end view.

- Automated Performance Reporting: Customers require that the mechanisms provided for Performance Reporting are based upon industry and international standards and that they can be integrated into their current systems using available industry tools.

- Common framework and definitions for Performance Reports: Customers would like to receive reports from Service Providers that are consistent in structure so that they can easily process them both manually and electronically without excessive complexity. To facilitate the combination and processing of information from several sources it is important to the Customers that the information parameter definitions are consistent.

- An important objective is to reduce the diversity of the definitions and to place them in a framework which takes account of the need to introduce, in a controlled way, service specific performance parameters.  It is not a requirement that the information definitions for say, availability, are reduced to a single definition common to each category of service.

- Security and confidentiality: It is usual that the service contract between the Customer and the Service Provider requires that the information is transferred on a confidential basis;  that is the Customer does not transfer the information to another Service Provider, and that the Service Provider does not disclose one Customer's performance report to any other Customer.

## 2.5	Expected beneficiaries

Customers

Through the use of common terms, Customers will get performance information that is consistent.  They will be able to compare the performance of their many providers and suppliers, which today is not possible because each supplier uses different performance terms, and each term represents different measures.  Availability, for example, is a term that is used very differently by different providers and suppliers.

Customers can refer to this work in specifying the performance measures which are critical to successful operation of services bought from one, or a range of Service Providers, and the work will link the business significance of these measures with the service type or types, concerned.

Performance reports relate to the service level(s) selected (and contracted) by the Customer. A performance report is a certificate that provides information on the extent to which the Service Provider has met the pre-defined and agreed quality levels of service. Thus, Customers can see whether they are actually receiving what they contracted for and, if not, can take appropriate action.

In addition, performance reports provide Customers with insight into the actual use of their service(s). With the information found in a performance report, the Customer is able to:

- maintain control over the quality and the effectiveness of these services

- keep his telecommunications efficient and cost-effective

- observe trends concerning traffic behaviours

- analyse the telecommunications costs for allocation to branches, plants, etc. (internal billing)

- prepare reports for management meetings

Service Providers

Many Service Providers are realising that providing meaningful performance information to their Customers can improve Customer satisfaction with the overall service provided.  They also see a benefit to negotiating common elements of a Service Level Agreement (SLA) to reduce the number of unique arrangements they must accommodate with individual Customers.

For the Service Provider, the agreements reached will be a basis for standardising Performance Reporting to multiple Customers, avoiding expensive customisation demanded currently by individual Customers because of a lack of a common approach.

The benefits are potentially in several areas, including the performance measures themselves, the way that these are applied in the operational environments of the organisations involved, and the co-ordination mechanisms used for interaction between companies.

Some Service Providers and suppliers are reluctant to move toward common performance terms because they would prefer their Customers not be able to perform a direct comparison of their performance vis à vis their competitors.  However, increasingly these providers are finding themselves playing the role of "Customer" to another provider or supplier, and the use of SLAs is extending throughout the service chain.  Thus, for a variety of reasons, it is highly desirable for Service Providers to reach industry agreements in this area.

Others

There are also benefits from the work for others, including suppliers of network equipment, through better insight into the business needs which their products can address and the opportunity to maximise the impact of the network data by understanding the linkages through to End-Customers of network services.

# 3	scope of interface requirements

This section identifies the scope of these requirements, and also points out what is not covered.

## 3.1	Process orientation

### 3.1.1	Position within the NMF Service Management Business Process Model

Performance Reporting is defined as one of the five “Customer Care” processes within the Service Provider organisation, [ref.2]. The Performance Reporting process interfaces to several other business processes within the Service Provider, namely, Order Handling, Problem Handling, Service Problem Resolution, Service Quality Management, Usage/Performance Data Collection. These relationships are illustrated in  Figure 3/1.  All of these interfaces, with their associated inputs and outputs, need to be understood by architects of the Service Provider’s OSS (Operational Support System), and help to define the business context of Performance Reporting.  These internal Service Provider interfaces are out of scope for this specification of a Performance Reporting interface, which is strictly limited to exchanges between Customer and Service Provider.

Figure 3/1:  Simplified view of processes used by a service provider

### 3.1.2	Responsibility of the Performance Reporting process

The Performance Reporting Process is responsible for providing whatever performance reports are requested by the Customer and available from the Service Provider.  Routine reports include performance of a service against an SLA, reports of any developing capacity problems, and reports of Customer usage patterns.  In addition, this process responds to performance inquiries from the Customer.

## 3.2	Process functions addressed

The process functions that are addressed within the scope of this document include:

- schedule Customer reports

- receive performance data

- compile Customer reports

- deliver reports to Customer

Figure 3/2:  Performance Reporting Description, Inputs and Outputs

## 3.3	How the interface is expected to be used

Performance Reporting is not a stand alone application. It depends on other OSS functions in the TMN (Telecommunications Management Network) environment for supply of information. The following figure illustrates Performance Reporting OSS function’s dependency and interworking with other OSS functions to perform its designed tasks and supply of information:

Figure 3/3:   Interaction of Performance Reporting with other Service Functions and Scope of the Performance Reporting Interface Definition

Figure 3/3 illustrates how the Performance Reporting function interacts with the other OSS functions, it shows only the interrelationships required for Performance Reporting. Although these interactions are essential to link information to other processes, they are out of scope of the Performance Reporting interface definition.

Note: It is assumed that the interface definition for Performance Reporting will make use as much work already done by other SMART teams (e.g. Customer to Service Provider Trouble Administration). The focus of the team’s work is therefore not only on the interface definition, but also on the information exchanged across this interface.

## 3.4	Scenarios to be addressed

Message sequences are presented below corresponding to different scenarios passing a performance report across the interface in scope.

### 3.4.1	Reporting with acknowledgement

The first scenario assumes that an acknowledgement of the report by the Customer is provided, allowing the Service Provider to time-out this response and re-transfer the report if the acknowledgement is not received. This scenario is typically applied when the report is sent on-line (even by fax) to the Customer as soon as it becomes available.

The scenario also points out that information sent to the Customer additionally is to be stored at the Service Provider’s side.

This does not mean that the stored information is on-line accessible for the Customer. Ways to retrieve this information as well as the length of the time-interval for which the information is available are subject of agreement between the Service Provider and the Customer).

Figure 3/4:  Sequence Chart of Performance Reporting with Acknowledgement

### 3.4.2	Reporting via “mailing“ system

The second scenario assumes that a “with a delay service“ is being used (e.g. e-mail, provisioning of an HTML file to a Web server, etc.) between Service Provider and Customer. The Customers access to that service and the data management in that “mailbox“ are assumed not to be in the Service Provider’s scope (although they could in a certain realisation).

Figure 3/5:  Sequence Chart of Performance Reporting without Acknowledgement

### 3.4.3	Reporting on demand

The third scenario assumes that there is a log service available at the Service Provider’s side where the performance reports are located. The Customer is provided with an access to that log. This access is typically on-line.

After being acknowledged the reports are deleted from the Service Provider’s report log.

Figure 3/6  : Sequence Chart of Performance Reporting with Acknowledgement and optional logging

### 3.4.4	Modifying the reporting mode

The fourth scenario describes the way a Customer may change the Performance Reporting mode (in the example below the reporting interval is changed).

Note that this is not in the current scope of the SMART Performance Reporting Team’s work.

Figure 3/7:   Sequence Chart of on-line Performance Reporting Control

# 4	Flow of Information

This section describes the dynamic aspects of the Performance Reporting process in terms of an event flow diagram and a state model. The workflow diagram used to capture aspects of most SMART processes is omitted as it is not considered to be relevant to Performance Reporting.

## 4.1	Workflow perspective

Performance Reporting is a periodic activity which collects data during each reporting cycle and at its end generates a report for the customer and a summary of any SLA violations that have occurred so that the customer can be billed accordingly. This is illustrated in section 4.2 below.

The workflow diagrams within SMART documents are intended to focus on the decisions and actions that need to take place within a SMART process. The emphasis is on understanding the actions resulting from decisions and their potential impact on the external interfaces of the process. As there are no internal decisions to be taken within Performance Reporting no diagram is included.

## 4.2	Event flow perspective

The event flow diagram below, shows the events that affect and are generated by Performance Reporting.

Figure 4/1 - Performance Reporting Event Flow Overview

## 4.3	State model

The performance report can be visualised as a dynamic object which is constructed from data collected during the reporting period, and is then transferred from the Service Provider to the Customer on a scheduled basis. If the specific report thus transferred is successfully received (indicated by acknowledgement, in cases where this is supported), it can then be considered to be discarded within the Service Provider. This does not preclude separate arrangements to archive this information for subsequent recovery by the Service Provider or the Customer. Following this, a new performance report for the next reporting period can be constructed. There is therefore a need to support concurrently both a current performance report (normally, the only report in use, but a new current report must be initiated while the previous report is under transferral) and a previous performance report (until this is successfully transferred).

In terms of a state model for the performance report, this gives rise to the following model:

Figure 4/2 : Performance Report State Model

## 4.4	Relationship between actual events and states

The following table shows the relationship between Performance Reporting Events and the states in which a Performance Report can exist.

| Event | Performance Report State |
| --- | --- |
| A new service is activated that supports generation of Performance Reports | An initialised Performance Report moves into the Incomplete Report State. It remains in this state for the duration of the Performance Reporting Cycle |
| Outage Data is received by Performance Reporting. This is accumulated for presentation within the complete report | The state of the Performance Report is not affected by this event |
| End of Performance Reporting Period event. This is generated on a periodic basis which could be weekly, bi-weekly, monthly, etc. | Performance Report moves from being an incomplete report, to being a complete report as data collected within the reporting period is finalised and added to the report |
| Performance Report sent to Customer | Performance Report moves to Transferred Report State |
| SLA Violations sent to Billing | The state of the Performance Report is not affected by this event |
| A service, that supports generation of Performance Reports is terminated | The affect of this event is the same as the End of Performance Reporting Period Event. The Performance Report moves from being an incomplete report, to being a complete report as data collected within the reporting period is finalised and added to the report |

Table 4/1: Relationship between Events and Performance Report States

# 5	Specific Requirements of the Customer to Service Provider Performance Reporting interface

This section is intended to capture specific business requirements (as opposed to other requirements such as the systems environment in which the interface must be capable of being implemented) that must be met through the implementation of the common agreement, in text form.  Each requirement has its own number. In some cases, sub-numbers are used  to fully explain a requirement.

The SMART Performance Reporting Team has identified a lot of requirements dealing with content of information (as opposed to functional and technical requirements). Although most of these requirements will be further addressed in the NMF Performance Reporting Definitions Document [ ref 1 ] the team sees a need to list them here as well.

Core Requirements are mandatory, as they define what is necessary to be satisfied for the system interface to fulfil the basic business need.  These core requirements must always be implemented in order to have complied with the terms of the interface (as far as technical requirements are concerned, ‘core requirements’ should be those that are felt to be essential in order to promote ease of use, efficiency of operation, throughput, flexibility, or other factors which, if not met, will render the interface unusable)

Optional Requirements (sometimes termed supplementary requirements) define how ‘core’ requirements would be extended such that value added levels of service quality are provided by the interface. It is quite possible that Service Providers who regularly partner with, or who have a large dependency on certain companies, may wish to go beyond the core requirements, to monitor the progress of the order on-demand at every step of the process.  These additional monitoring requirements would be considered optional.

The numbering system NMF will use to keep track of requirements across documents will be a string of characters and numbers.

The first three characters are the team, or project identifier (the same as used to identify documents on NMF’s server).

The fourth and fifth characters identify the type of requirement, as follows:

FR for functional requirement,

TR for technical requirement, and

OR for other requirement.

Following these characters, numbers are assigned that correspond to the paragraph numbers used in the document (for ease later on in finding the full description of a requirement).

## 5.1	Contract (SLA) related requirements

The SLA means the legal contractual agreement between two parties for the service under contract, specifically between the Customer and the Service Provider.  Other aspects of the level of service require documentation by both the Customer and the Service Provider and may or may not become part of the contract.  Performance Reporting uses the SLA as the reference and does not address the other parameters known to exist, but which are not part of the SLA.

5.1.1	Core	The SLA to be a legal contractual agreement between two parties for the service under contract (PEROR1.1)

The SLA represents an integral part of the contract between the Service Provider and the Customer.

5.1.2	Core	SLAs to be numbered, paged, dated and controlled documents (PERFR1.2)

This information is essential, since the performance report refers to the SLA. The Performance Reporting process will gather SLA information out of some Customer database.

5.1.3	Core	Performance Reporting will concentrate primarily on those elements of the SLA concerned with service performance (PERFR1.3)

SLAs can cover many aspects of the relationship between the Customer and the Service Provider such as performance of services, Customer care, billing, provisioning etc.

5.1.4	Optional	SLA to be built according to a modular concept to facilitate attachment of service specific sections to the standard SLA (PEROR1.4)

5.1.5	Core	Service definitions for each service module available and uniquely identified (PERFR1.5)

The performance report process will use the service identifier(s) of the SLA as basis for the report

5.1.6	Core	Service Provider/Customer contracted boundary of responsibility to be accurately recorded in SLA (PERFR1.6)

Exclusions (e.g. announced maintenance windows, no access to Customer premises, Customer caused service break, force majeur, etc.) have clearly to be defined in the SLA. The calculation of e.g. “Service Availability” will make use of such defined parameters

5.1.7	Core	Customer responsibilities to be defined (PEROR1.7)

E.g. the preferred method of trouble reporting to the Service Provider and provision of contact details

5.1.8	Optional	Penalties and recovery periods to be defined (PEROR1.8)

5.1.9	Core	Definitions to be fully set out (PERFR1.9)

Each parameter of either the SLA or the performance report has to be defined in a clear and unambiguous way (e.g. SERVICE, SERVICE ACCESS POINT, SERVICE BREAK, SERVICE AVAILABILITY, etc.). [ ref 1 ]

## 5.2	Performance Report related requirements

5.2.1	Core	Performance Reports to be numbered, dated and controlled “documents” (PERFR2.1)

5.2.2	Optional	Standard  presentation format, allowing for company preferred fonts and regional legal requirements (PEROR2.2)

Customers would like to receive reports from Service Providers that are consistent in structure.

5.2.3	Core	Generic deliverable definitions (PEROR2.3)

To facilitate the combination and processing of information from several sources it is important to the Customers that the information parameter definitions are consistent

An important objective is to reduce the diversity of the definitions and to place them in a framework which takes account of the need to introduce, in a controlled way, service specific performance parameters

5.2.4	Core	Types of report to be defined according to a code list/table (PERFR2.4)

E.g. Service Availability, MTTR (Mean Time To Repair), TTR (Time To Repair), MTBF (Mean Time Between Failures), utilisation, etc [ ref 1 ]

5.2.5	Core	Reporting period and frequency to be defined (PERFR2.5)

The covered time span of a performance report as well as its frequency have to be known and defined in the contract (SLA)

5.2.6	Core	Measurement period to be defined (PERFR2.6)

The covered time span of the measurement has to be known. The measurement period can differ from the reporting period (e.g. a specific measurement is only performed during traffic peak periods, though the reporting period is the entire month)

5.2.7	Core	Method of measurement to be defined (PERFR2.7)

It is essential for reporting on service performance to have the measurement method clearly defined and documented

5.2.8	Core	Arbitration method to be defined (PERFR2.8)

5.2.9	Core	How to handle boundary conditions has to be defined (PERFR2.9)

E.g. where outages cross reporting period boundaries

5.2.10	Core	Guaranteed performance values (SLA values) should be included in the performance report (PERTR2.10)

A direct comparison between the measured/calculated values with the contracted/guaranteed values should be possible within the report

5.2.11	Optional	Performance Reporting  should include fault management information related to non-availability of the contracted service (PEROR2.11)

5.2.12	Optional	Performance Reporting should include reporting on the utilisation of the contracted capacity (PEROR2.12)

5.2.13	Optional	Performance Reporting should include reporting on traffic data (PEROR2.13)

## 5.3	Service Availability related requirements

5.3.1	Core	Service applicability or service cover time to be defined (PERFR3.1)

(e.g. 08:00 - 18:00 Mon. - Fri.; 24h Mon. - Sun., etc.)

5.3.2	Core	End-to-end Service Availability measurement/calculation should not be a summarisation of single network element performance measures (PERFR3.2)

To reduce the complexity and to shield the single network element performance values, Performance Reporting makes (mainly) use of Trouble Ticket information to determine and calculate the end-to-end Service Availability

5.3.3	Core	Service Availability measurements and calculations should also reflect degraded service/ degraded performance (PERFR3.3)

Service degradation and service availability are two related but in some cases separate matters. A Service Provider may establish criteria and performance credits for service degradation. Service degradation criterias relate to the service offered to the Customer.

[ ref 1 ] provides more details regarding the applicability of degraded service to different telecommunications services.

5.3.4	Optional	Where applicable, bandwidth availability should be guaranteed, measured and reported (PEROR3.4)

5.3.5	Optional	Availability definitions should be included where LOCAL SERVICE remains available when NETWORK ACCESS is not (PEROR3.5)

5.3.6	Core	SAP availability should be calculated on a SAP by SAP basis to assist granularity of reporting (PERFR3.6)

If this is not done, it is possible, in a large population and/or over a long reporting period, to have a major failure on a single connection but report the service as exceeding performance requirements.

E.g. report the global indicator but also note individual failures. SLA's guarantee for single SAP must be set accordingly to a different value (statistically lower than Overall Service Availability)

5.3.7	Core	For Service Availability calculations a Customer's connected base (SAPs) may be dynamic during the reporting period. In order to avoid contention there are two options:

- a mean number or a cut off date for an installed base calculation must be jointly agreed and noted as an attachment to period reports (PERFR3.7.1)

- the Service Availability formula has to take into account the time in which every  single SAP has been active within the reporting period (PERFR3.7.2)

## 5.4	General QoS requirements

5.4.1	Core	The scope is to report on “end-to-end” service performance (PERFR4.1)

The SMART Performance Reporting team defines that a service is “delivered” to the Customer at the so-called Service Access Point (SAP). The Service Provider’s responsibility therefore includes all parts (and sub-parts) of the service(s) between all contracted SAPs. Exceptions/exclusions have to be stated in the SLA. [ ref 1 ]

Figure 5/1:  End-to-end service performance

Figure 5/1 shows a quite simple implementation of a service. However, the service provider’s responsibility includes already at least CPE equipment, access lines (e.g. in different countries) and the service provider’s backbone network/service platform.

In case of a pure PL-LL (Private Line - Leased Line) service, figure 5/1 can be simplified to a single line connecting two customer sites.

A more complex service could include a PVC based Frame Relay implementation delivered over an ISDN access with an ATM backbone using SDH for the transport.

There are many well developed tools addressing performance issues for numerous network elements. Performance reporting is not interested in the summarisation of all the individual network element performance numbers to get an end-to-end view.

5.4.2	Core	Further quality parameters (e.g. MTTR, MTBF, TTR, etc.) should be calculated based on the same information used to calculate Service Availability (PERFR4.2)

5.4.3	Core	Technical quality parameters (e.g. Network Transit Delay, Throughput, etc.) have to be measured and calculated according to the relevant standards (PERFR4.3)

## 5.5	Report generation and distribution requirements

5.5.1	Core	The interface shall support more than one distribution method. Different possible distribution methods include:

- On-line, Client/Server Access: e.g. WWW (World Wide Web), FTP (File Transfer Protocol), etc.

- CMISE

- EDI

- E-mail

- Fax

- Mail (postal service)

(PERFR5.1)

5.5.2	Core	Once a current aggregation interval has ended, a notification shall be sent to the customer indicating the creation of a new report. The customer may then retrieve the report through the interface (PERFR5.2).

## 5.6	Report frequency requirements

5.6.1	Core	 The interface shall support different reporting frequencies. Reporting frequency is the periodicity with which Performance Reports are delivered from the Service Provider to the Customer. Different possible periodicities include:

- synchronised with the billing cycle

- on demand

- weekly

- monthly

(PERFR6.1)

## 5.7	Report customisation requirements

5.7.1	Optional	Limited report customisation allows the customer to exercise certain defined control over report attributes. Depending on the service provider’s policy and the agreement with the customer, a subset of the following customer control activities can be supported:

- Disable an active report routing

- Create a new report routing with a specific aggregation interval within a set of (predefined) choices of interval length

- Modify the report contents within the set of supported attributes

(PERFR7.1)

# 6	General Technical Requirements

In addition to the business requirements within the Performance Reporting context, there are a number of  technical requirements that must be met to provide a robust and practical solution.

## 6.1	Transport requirements

6.1.1	Core	The  interface should be defined in a network transport protocol independent manner. Customers require that the mechanisms provided for Performance Reporting are based upon industry and international standards. It should be possible for the transport to be OSI-based, TCP-based, IPX-based, or whatever.  (PERGTR1.1)

This will support the integration of performance reports into their current systems using available industry tools (i.e. that they can easily process them both manually and electronically without excessive complexity)

## 6.2	Connection requirements

6.2.1	Core	It should be possible for the connection between the Customer and the Service Provider to be either permanent or transient.  A transient connection should be supported via dial-up, Internet access, etc. (PERGTR2.1)

Since the delivery of a performance report is neither very time consuming nor high in frequency, a  Customer would  use a permanent connection only if it were established for purposes beyond just the exchange of performance data. Service Provider - to - Customer connections may be low volume. Customer does not want to pay for a permanent connection.

6.2.2	Core	The connection between a Service Provider and a Customer should provide for “reliable” transport of data.  It should be possible for a performance report or update to be selected for forwarding, have the connection be unavailable, and have the information be ”queued” for subsequent delivery. (PERGTR2.2)

Retry is an implementation issue.  Non-delivery errors need to be addressed.

6.2.3	Optional	It should be possible for one Customer to have connections with multiple Service Providers. (PERGTR2.3)

6.2.4	Core	It should be possible for one Service Provider to have connections with multiple Customers. (PERGTR2.4)

There is no requirement for multiple Customers to exchange performance reports between themselves.

Service Provider to Service Provider exchange is covered, if one of the Service Providers acts like a Customer according to the definition in chapter 1.3

Any  requirements to correlate or process performance reports between multiple Service Providers or multiple Customers is outside the scope of this document.

## 6.3	Control and mapping requirements

The issues of polling by the Customer Performance Reporting system to the Service Provider Performance Reporting system for status updates versus having the Service Provider Performance Reporting system “push” updates down to the Customer Performance Reporting system on a status/state change (e.g. new data/report available, etc.) is an implementation issue.

## 6.4	Other general requirements

6.4.1	Core	All time values (time-stamps) exchanged across the interface should be in a format that accounts for time-zone differences. For example, systems may agree to use UTC. (PERGTR4.1)

6.4.2	Core	There is a common content for performance reports on all subjects, composed of mandatory and optional attributes.  Various performance report formats may be defined which specify which attributes must be present, and which attributes may be present.  A format must be selected when creating specific performance reports.  It must be possible to determine the format of a given performance. (PERGTR4.2)

A set of pre-defined formats, as well as the method of selecting the appropriate performance report format for a given instance, is out of scope of the present document.  However, it must be agreed between the Customer and the Service Provider in a contract.  The mechanism to support variable formats will depend on the protocol used.  For practical reasons, implementations should be able to support new performance report formats with a minimum of effort.

The SMART Performance Reporting Team will develop a framework for performance reports at a later stage.

6.4.3	Core	The Service Provider must be capable of detecting when the Customer becomes unreachable at a time when the Service Provider expects to move information across the interface.  No information should be lost due to this unreachability.  When the interface subsequently becomes operational again, normal processing should be resumed. (PERGTR4.3)

Detailed recovery procedures should be defined in the protocol-dependent specifications.

6.4.4	Core	The service provider will keep the latest report until the next report is generated. The service provider may maintain more than one report on previous aggregation intervals. The customer may perform ad hoc queries on any reports maintained by the service provider as specified for the interface. (PERGTR4.4)

Archive & Retrieval must be capable of holding a ‘rolling’ n of data.  Where n is the period agreed between Customer and Service Provider. This allows long term report summaries and contract reviews

## 6.5	Security requirements

The performance report contains information that may be confidential, so unauthorised access should  be prevented.  It also contains sensitive data so access to certain parts of the performance report must be controlled.  The following security services should be provided.

No mention is made here as to how these security services should be provided, as it depends on the protocol used.

6.5.1	Core	Identification and Authentication. Identification and authentication are the processes of determining whether a session requester is truly the identity that it claims to be.  Customers and Service Providers shall be able to identify each other reliably across the interface.  A Customer must be authorised by an Service Provider before any Trouble Reports can be exchanged. (PERGTR5.1)

6.5.2	Core	Access Control. In general, Access Control is the process of establishing a session from start to end, i.e., from login to logoff.  In the context of Performance Reporting we are concerned only with control of access to resources within the Performance Reporting application itself.  Users have to be identified and authenticated before access is granted. (PERGTR5.2)

6.5.3	Core	Data Confidentiality. Confidentiality is defined in ISO 7498-2 as “the property that information is not made available or disclosed to unauthorised individuals, entities, or processes.”  Customers of one organisation should not have access to performance reports belonging to another Customer organisation unless consent has been given. (PERGTR5.3)

6.5.4	Core	Integrity. A connection-oriented integrity service, one that deals with a stream of messages, assures that messages are received as sent, with no duplication, insertion, modification, reordering, or replays.  The destruction of data is also covered under this service.  Thus, the connection-oriented integrity service addresses both message stream modification and denial of service.  On the other hand, a connectionless integrity service, one that deals with individual messages only without regard to any larger context, generally provides protection against message modification only.  As both Customer and Service Provider rely on the interface to provide them with identical copies of performance reports, an integrity should be provided. (PERGTR5.4)

The Integrity service provided by a transport layer such as TCP or OSI Transport class 4 will normally be sufficient.

6.5.5	Optional	Non-Repudiation. Repudiation is defined as “denial by one of the entities involved in a communication of having participated in all or part of the communication”.  Two forms of non-repudiation are defined: non-repudiation with proof of origin and non-repudiation with proof of delivery.  This security service may take one or both of those forms. In non-repudiation with proof origin, the recipient of data is provided with proof of the origin of data.  This will protect against any attempt by the sender to falsely deny sending the data or its contents.  In non-repudiation with proof of delivery, the sender of data is provided with proof of delivery of data.  This will protect against any subsequent attempt by the recipient to falsely deny receiving the data or its contents.  This service is obviously useful in settling any disputes that may arise between Customer and Service Provider.  Also, procedures such as clear verification, which requires that the Customer respond to the Service Provider’s request to confirm either the receipt of the performance report or to confirm the acceptance of the report content, can be made more robust with this service. (PERGTR5.5)

## 6.6	Performance of the interface

Since Performance Reporting is neither time critical, nor are large volumes of information to be transferred, there are no general performance requirements to the interface. (PERGTR6.1)

## 6.7	Methods of synchronisation upon session failure

Please refer to (PERGTR4.3)

## 6.8	Inventory synchronisation / database reconciliation

not applicable for this interface

# 7	conclusions

## 7.1	General remarks

The SMART Performance Reporting Team started its work early in 1995 with the development of a document called SMART PERFORMANCE REPORTING WHITE PAPER  [ref  4]. The document is a mix of business and technical requirements as well as performance related standards. The structure and outline was based on a development made by the SMART Customer to Service Provider Trouble Administration Team.

During the developments, the team detected

- that there are no common definitions of terms to be used at the service level.  Although many terms exist to describe network and element availability, for example, the term has widely different application at the service level. Even terms like SERVICE, SLA, SAP, etc. are used differently by different Service Providers

The SMART Performance Reporting Team therefore took the action to develop a document which deals with this topic. The document (currently under development) will provide definitions related to quality and performance applied to the service level. The document is called NMF Performance  Reporting Definitions Document  [ref  1].

This document will provide essential input  to teams and individuals dealing with the  development of the performance reporting interface.

- important relations and dependencies to other SMART activities and processes (mainly to “Trouble Ticketing” and “Ordering”).

Currently these “cross-SMART” issues are discussed within all SMART teams and together with the NMF InfoMod team.

- that Performance Reporting does not require a dedicated interface development.

Performance Reporting can use the same mechanisms to  exchange information like e.g. Trouble Ticketing. Furthermore, Performance Reporting should be one part of a “CNM-type” (Customer Network Management) interface. The focus therefore should also be on “what” information is  exchanged, not only “how” information is exchanged.

- that compared to e.g. Trouble Tickets and Orders, the content (information) of a Performance Report is nowhere clearly defined and differs from Service Provider to Service Provider

## 7.2	Success criteria

Following the business problem and the additional requirements, the SMART Performance Reporting Team has set up some criteria to be able to determine the success of the solutions being produced for the performance reporting process. These criteria are

Requirement oriented:

- Availability of requirements that are pertinent to reporting on the performance of services (rather than reporting on the performance of network  entities)

- Availability of a set of definitions of terms for service performance, used at the service level, that meets customer needs.

- Availability of a set of identified SLA components (generic and/or service specific) which are relevant to performance reporting.

- Availability of a framework for a generic Performance Report.

Based on this requirements oriented work, more technical oriented work can be carried out. The success criteria there are

Implementation oriented:

- Reducing the amount of human intervention for collecting data, generating reports and exchanging report information by using automated processes.

- Availability of a generic performance reporting interface between customer and service provider that can be used for all types of services.

## 7.3	Issues and risks

### 7.3.1	Issues

A main issue of  the SMART Performance Reporting work is the co-ordination of the different SMART activities and the need for common information between this activity and other SMART work:

- clear definition of scope and interfaces between the work areas

- common process and approach across each SMART area

- availability of developments made by other SMART teams

- development and use of common definitions, terms, information model, etc. across all SMART activities

Further issues include:

- the conflict between the delivered service and the underlying technology must be sorted out

- the solution set must be scaleable in scope as well as expandable for new technology

### 7.3.2	Risks

The main risk of the SMART Performance Reporting work is in setting the scope of work to be broad enough to be useful to the industry while narrow enough to allow timely delivery of results.

Further risks are:

- does the approach of the “Basic Delivered Services” fulfil customer needs or are developments on the protocol level needed ?

- has input been obtained from service providers in various regions so that the resulting requirements meet the needs of a most of providers ?

- the primary objective is SERVICE reporting.  The risk is that as the report becomes more technical the value to the user is diminished.  Keep it simple is the guiding rule.

# Appendix A - summary of requirements

| requirement  | label | high level explanation |
| --- | --- | --- |
| PEROR1.1 | Core | The SLA to be a legal contractual agreement |
| PERFR1.2 | Core | SLAs to be numbered, paged, dated and controlled documents |
| PERFR1.3 | Core | Performance Reporting will concentrate primarily on those elements of the SLA concerned with service performance |
| PEROR1.4 | Optional | SLA to be built according to a modular concept |
| PERFR1.5 | Core | Service definitions for each service module available and uniquely identified |
| PERFR1.6 | Core | Service Provider/Customer contracted boundary of responsibility to be accurately recorded in SLA |
| PEROR1.7 | Core | Customer responsibilities to be defined |
| PEROR1.8 | Optional | Penalties and recovery periods to be defined |
| PERFR1.9 | Core | Definitions to be fully set out |
| PERFR2.1 | Core | Performance Reports to be numbered, dated and controlled “documents” |
| PEROR2.2 | Optional | Standard presentation format, allowing for company preferred fonts and regional legal requirements |
| PEROR2.3 | Core | Generic deliverable definitions |
| PERFR2.4 | Core | Types of report to be defined according to a code list/table |
| PERFR2.5 | Core | Reporting period and frequency to be defined |
| PERFR2.6 | Core | Measurement period to be defined |
| PERFR2.7 | Core | Method of measurement to be defined |
| PERFR2.8 | Core | Arbitration method to be defined |
| PERFR2.9 | Core | How to handle boundary conditions has to be defined |
| PERTR2.10 | Core | Guaranteed performance values (SLA values) should be included in the performance report |
| PEROR2.11 | Optional | Performance Reporting should include fault management information related to non-availability |
| PEROR2.12 | Optional | Performance Reporting should include reporting on the utilisation of the contracted capacity |
| PEROR2.13 | Optional | Performance Reporting should include reporting on traffic data |
| PERFR3.1 | Core | Service applicability or service cover time to be defined |
| PERFR3.2 | Core | End-to-end Service Availability measurement/calculation |

| PERFR3.3 | Core | Service Availability should also reflect degraded service/ degraded performance |
| --- | --- | --- |
| PEROR3.4 | Optional | Where applicable bandwidth availability should be guaranteed, measured and reported |
| PEROR3.5 | Optional | Availability definitions should be included where local service remains available when network access is not |
| PERFR3.6 | Core | SAP availability should be calculated on a SAP by SAP basis to assist granularity of reporting |
| PERFR3.7.1 | Core | For Service Availability calculations a Customer's connected base (SAPs) may be dynamic |
| PERFR3.7.2 | Core | For Service Availability calculations a Customer's connected base (SAPs) may be dynamic |
| PERFR4.1 | Core | Report on “end-to-end” service performance |
| PERFR4.2 | Core | Further quality parameters should be calculated based on the same information used to calculate Service Availability |
| PERFR4.3 | Core | Technical quality parameters have to be measured and calculated according to the relevant standards |
| PERFR5.1 | Core | Distribution method |
| PERFR5.2 | Core | Notification on the existence of a new report |
| PERFR6.1 | Core | Reporting frequency |
| PERFR7.1 | Optional | Report customisation |
| PERGTR1.1 | Core | Transport mechanism/protocol |
| PERGTR2.1 | Core | Connection between Customer and Service Provider |
| PERGTR2.2 | Core | Reliable transport of data |
| PERGTR2.3 | Optional | It should be possible for one Customer to have connections with multiple Service Providers. |
| PERGTR2.4 | Core | It should be possible for one Service Provider to have connections with multiple Customers |
| PERGTR4.1 | Core | Time zones; agreement on UTC |
| PERGTR4.2 | Core | Report format |
| PERGTR4.3 | Core | Unreachable Customer |
| PERGTR4.4 | Core | Archive & Retrieval |
| PERGTR5.1 | Core | Identification and Authentication |
| PERGTR5.2 | Core | Access Control |
| PERGTR5.3 | Core | Data Confidentiality |
| PERGTR5.4 | Core | Integrity. |
| PERGTR5.5 | Optional | Non-Repudiation |
| PERGTR6.1 | n/a | Performance of the interface |

# Appendix B - Glossary of terms, ACRONYMS and abbreviations

CIR	Committed Information Rate

CNM	Customer Network Management

CPE	Customer Premises Equipment

CUS	Customer

DCE	Data Communications Equipment

DTE	Data Circuit Terminating Equipment

FTP	File Transfer Protocol

HTML	HyperText Markup Language

IPX	Internetwork Packet Exchange (Protocol)

ISV	Independent Software Vendor

MTBF	Mean Time Between Failure

MTTR	Mean Time To Repair

NM	Network Management

NMF	Network Management Forum

NML	Network Management Layer

M	Modem

MAS	Managed Access Switch

OSI	Open Systems Interconnect

OSS	Operational Support System

PVC	Permanent Virtual Circuit

PR	Performance Report(ing)

QoS	Quality of Service

SAP	Service Access Point

SLA	Service Level Agreement

SMART	Service Management Automation and Re-engineering Team

SP	Service Provider

TCP	Transmission Control Protocol

TMN	Telecommunications Management Network

TT	Trouble Ticket(ing)

TTR	Time To Repair

UTC	Universal Time Coordinated

WWW	World Wide Web

# Appendix c - references and related documents

Referenced documents

| [ref 1] | NMF Performance Reporting Definitions Document, NMF, 1997 |
| --- | --- |
| [ref 2] | A Service Management Business Process Model, NMF, 1995 |
| [ref 3] | Implementing OMNIPoint, A safer, faster path to improved service management, NMF, 1995 |
| [ref 4] | SMART Performance Reporting White Paper ISSUE 1.0, NMF SMART Performance Reporting Team, April 19, 1996 |

Note: References to standards are found in Appendix D

Related documents

| 1 | Network Performance Survey Results Report, Jeff Paschke, International Network Services, June 1995 |
| --- | --- |
| 2 | Statement of User Requirements for Managing a Networked Information System, NMF User Advisory Council, 1992 |

# Appendix D - standards

## D.1	OVERVIEW

This section references other documents that address elements related to the subject of Performance Reporting. Where possible the issue of the document has been identified.

The SMART Performance Reporting Team reviewed performance related standards from numerous sources. Document review is an ongoing process and the list found below is not deemed to be exhaustive.  Suggestions of new references (with location and availability) are welcome and should be sent to the editor.

### D.1.1	What we are looking for

A real world “model” for a PVC data service performance report should address a Frame Relay implementation delivered over an ISDN access with an ATM backbone using SDH for the transport.  The above model demonstrates that there are very few services implemented with a single “pure” technology and the solution should support the situation likely to be encountered.

Note: The above model is only an example. The Team’s work is not limited to PVC services.

### D.1.2	Conditions

There are two categories of service conditions that drive reporting: 1) loss of service and 2) degraded service.  Loss of service is somewhat more easily determined at almost any point along the service path.  Degradation of service is more difficult to determine.

### D.1.3	Essential findings

There are many well developed documents addressing Performance Reporting for numerous network elements. No standards were found that directly apply to the end-to-end service the Customer has purchased. The user is not capable of (nor interested in) summarisation of all of the individual NE (Network Element) performance numbers.  Where several networks support the service, the aggregation of all of the individual NE degradation reports is not practical. Therefore more direct measures of the user (end-to-end) service appears to be required for degradation reports.  No standards were located that address this issue directly.

This section references other documents that address elements related to the subject of Performance Reporting. Where possible the issue of the document has been identified. This list is not exhaustive and additions should be communicated to the editor.

There are three Sections:

- Document list

- Document Abstracts

- Directory of terms

First locate the term in section 8.4 of the directory then determine the document(s) that address that term.  Section 8.4 indicates the number of different extensions and subsets of the term within the ( ).

## D.2	Document list

The identifications are ITU-T unless otherwise noted.  Documents noted [*] remain to be reviewed.  The readers attention is called to the use two terms QoS (Quality of Service) and GoS (Grade of Service) which are defined within the referenced documents.

- E.540: Overall GoS of the international part of an international connection

- E.541: Overall GoS for international connections (subscriber-to-subscriber)

- E.543: GoS in digital international telephone exchanges

- E.550: GoS and new performance criteria under failure conditions in international telephone exchanges

- E.720: ISDN GoS concept

- E.721: Network GoS parameters in ISDN

- E.771: GoS for land mobile services

- E.775: UPT GoS concept

- E.7IN1: GoS parameters and target GoS objectives for IN-based services

- E.800: Terms and Definitions Related to Quality Of Service and Network Performance Including Dependability   (08/94)

- E.810: Model for the serviceability performance on a basic call in the telephone network [*]

- E.830: Models for the allocation of international telephone connection retainability, accessibility and integrity[*]

- E.845: Connection accessibility objective for the international telephone service[*]

- E.850: Connection retainability objective for the international telephone service[*]

- E.855: Connection integrity objective for international telephone service[*]

- I.350: General aspects of QoS and network performance in digital networks, including ISDN[*]

- I.351: Recommendations in other series concerning network performance objectives that apply at reference point T of ISDN[*]

- I.352: Network performance objectives for connection processing delays in an ISDN

- I.360 series

- M.3100:  03/95

- M.3400: TMN management functions 10/92

- Q.822: Performance Reporting Management functions

- X.140: General Quality Service Parameters for Communication via Public Data Networks 09/92

- X.144: User Information Transfer Performance Parameters for Data Networks Providing International Frame Relay PVC Service  DRAFT  07/95

- X.145:  Performance Parameters for Data Networks Providing International Frame Relay SVC Service	DRAFT 07/95

- X.161: Customer network management 11/94

- ISO/IEC JTC21/SC21-N9309: QoS Basic Framework 01/95 (will become CD 113236-2 in Aug. 95)

- RACE Service Management documents

- RFC 1604: Service management requirements Frame Relay Forum.

- CCITT Handbook on Quality of Service and Network Performance

- Frame Relay Forum Quality of Service Working Group (Working document August 1993)

- T1A1.3/93-011R2 -T1 Performance Standards Contribution Doc

Note:  E.3xx, E.5xx and E.8xx documents remain to be assigned.

## D.3	Document abstract

E.771

The Grade of Service (GoS) parameters for land mobile services are parameters which primarily describe the interface between Service Providers and/or a Supplier. The mobile telephone subscriber/end user normally does not ask for GoS parameters. The probability of end-to-end blocking and connection cut-off due to unsuccessful handover is highly dependent of the network resources.

E.800

Quality of Service and Dependability Vocabulary, provides a vocabulary of terms and their relationships.  376 terms are defined.  [The reader is cautioned that the 08/94 release of E.800 is substantially different from earlier editions.  Reference numbers and sections have been changed.]

- a general framework for performance concepts is shown in figure 1 which also include planning, provisioning.

- A distinction is made between QoS and Network Performance is shown in figure 1.

- a set of measures for these performances.  The method of measurement is not addressed.

E.800 appears to be a superset of 9309.  Co-ordination between E.800 and 9309 is not clear.  Annex A lists related QoS/NP handbooks and E-Series recommendations.

M.3100

This document contains definitions of object classes, and their components, intended to represent a generic network information model. These generic managed object classes are intended to be applicable across different technologies, architectures and services.

None of the included definitions are specifically aimed at performance measurement. There is an attribute "availabilityStatus", derived from X.721, but this refers to Service Availability - i.e. whether the resource is operational, degraded, in test, etc., - and is concerned with  fault status, rather than performance.

The parameter "c.2.9.1 - alarmEffectOnServiceParameter" might be considered to have some relevance to Performance Reporting, but it is not specific on the impact:

M.3400

This document contains descriptions of TMN Management Functions. Section 2 of the document is specifically on Performance Management and includes some functional descriptions of interest.

Q.822

This document is entirely concerned with performance management, and focuses on describing the parameter collection and thresholding aspects of performance management for a Q3 Interface.  This document has an NE - MIB management orientation and is too detailed for reporting service performance at the Customer interface.  An aggregation of these parameters across multiple providers will be difficult.  The majority of the document is concerned with the mechanisms of performance management, rather than the content of the specific performance measures, but this may be of interest also. Appendix 1 contains examples of how performance parameters could be defined as managed object attributes. Full GDMO definitions are provided in the document.  Extraction of even the major objects would still be a considerable body of text.

X.140

A primary value of this document is Figure 1 which defines the three phases of a communication event: Call Establishment, Data Transfer, and Disengagement.  Performance is measured during each phase by three key parameters: Speed, Accuracy and Dependability.  This 3 by 3 matrix appears to provide a strong foundational model for other documents addressing performance and availability.

Specific emphasis is made for switched data services such as X.25 and X.21.  Performance relates to where the Public Data Network terminates at the user site at physical layer one.  The OSI user perceived QoS performance is not addressed (figure 2).  References are drawn to circuit switching X.130, X.131; Packet switching X.134, X.137 and OSI network service performance X.213.  Two parameters are addressed: performance during normal service operation and  availability parameters describing frequency and duration of service outages.

X.144:

This document is very well developed for detailing the performance of PVC operation in the data transfer phase.  The reader is drawn to X.140 for the basic construction of the three phases of a data connection: Call Establishment, Data Transfer, and Disengagement.  During the data transfer phase (PVC) various parameters are discussed as  measures of performance and availability.  Of particular value are: Figure 5 showing Statistical populations used in defining selected accuracy and dependability parameters, Table 1 showing the four Outage criteria for the availability decision parameters, and Appendix I describing sampling techniques and sampling periods for availability.

X.145:

This document is a companion to X.144 and addresses the call establishment and disengagement phases for SVC operation.  Two outage criteria for the availability decision parameters for SVCs are described in table 9 in addition to the four criteria described in Table 1 of X.144 for PVCs.

X.161

X.161 defines the management services which may be provided to a Customer and collectively described as Customer Network Management (CNM). CNM is a service which provides the Customer with an ability to access (and in some cases modify) management information relating to the services provided to them by the network.

X.161 defines the management services and supporting functions for CNM. This recommendation is intended to complement TMN specifications and provide a specification for the non-TMN environment.  X.161 is related to:

- X.160 - Architecture for CNM for public data networks

- X.162/X.163 - Definition of Management Information for CNMThe CNM services defined in X.161 are classified into six groups (Fault, Accounting, Configuration, Performance and Security Management, CNM supporting services) FCAPS.

FRF-QoS

Contains definition for transfer delay on Frame Relay circuits and networks.  Suggested measures are defined.

RFC 1604

The document  is a very useful MIB for service management of Frame Relay.  However it does not contain definitions of performance measures.  The information may be useful, but the reader must abstract the MIB field contents to gain useful QoS measures.

T1A1.3/93-011R2

Definition of PVC availability and Mean time between service outages.  Same definition as X.144

21n9309   QoS - Basic Framework

This document is an excellent reference for terms and definitions related to QoS.  It is an extension to X.200 and provides a basic framework for QoS in terms of:

1) characterisation,

2) requirements specification, and

3) management.

It does not address detailed specification of QoS mechanisms. The writers have developed text that is specifically directed to the larger issues of systems management (not the specific requirements of networks).  Readers should benefit from the higher level thinking and terminology, however it should be read from the users’ point of view and the end-to-end character of the network application.  The attempt to define QoS at generic interfaces between systems (or elements) has been accomplished.  The reader should be alert to specific details in their own application that may modify the approach of 9309.

Critique:   The subject of statistics is addressed from the point of view of necessity to compress the data resulting from measurements. In very large (real) networks the amount of data potentially available will be staggering, therefore statistical reduction and summarisation of data is important.

What is missing is the economic necessity to address sampling approaches to obtain data measurements. Ubiquitous gathering and storing of data on all circuits all of the time will not become an economical reality within the foreseeable future..  With limited compute power in the network elements and the added cost of placing “probes” at many locations, the ability to obtain measures of quality requires consideration of sampling processes. Usual issues include frequency of scanning and dwell time,  as well as scheduling specified hours and days.

This document should become a starting place as a “reference check” for definitions.

## D.4	Directory

### D.4.1	Management

Fault Management						X.161

7.1.1.1  CNM Alarm Service Notification definition

Accounting							X.161

7.3.1.1  CNM Periodic Billing Service definition

Performance							X.161

7.4.1.1  CNM Traffic Information Service definition

7.4.1.2  CNM Traffic Information Service description

7.4.1.3  CNM Network Statistics Service definition

Performance Management

Generic			(14)			M.3400

Traffic			(3)			M.3400

Traffic Administration	(5)			M.3400

### D.4.2	Measurements

Alarm Indication			(2)			Q.822

Access delay							X.140

Accuracy and Dependability 		9309	FRF-QoS	X.144

Availability			(40)	E.800;		(4) 	9309 ;	 FRF-QoS

Bit-Based Conformant Traffic Distortion Ratio		X.144

Capacity - QoS			(11)			9309

Controlled Slip			(2) 			Q.822

Code Violation			(7) 			Q.822

Counts - Frame Relay activity	(24)			RFC1604

Disengagement - delay					X.140

Errored Seconds		(21) 			Q.822

Extra Frame Rate				FRF-QoS	X.144

Frame Based Conformant Traffic Distortion Ratio			X.144

Frame Loss Ratio				FRF-QoS	X.144

Integrity - QoS			(12)			9309

Interruptions							E.800

Loss Of Signal			(1) 			Q.822

Mean Time Between PVC Outages				X.144

Performance Analysis		(2)			M.3400

Performance Monitoring

Generic 			(4)			M.3400

Traffic Status Monitoring  	(8)			M.3400

Traffic Performance		(12)			M.3400

Protection Switching			(2) 			Q.822

PVC Availability							X.144

T1A1.3

Reliability - QoS				(9)			9309

Residual Frame Error Ratio			FRF-QoS	X.144

Retainability				(5)			E.800

Servability Performance			(13)			E.800

Severely Errored Seconds		(9) 			Q.822

Speed of service - Frame Transfer delay			FRF-QoS

Transmission performance		(2)			E.800

Time - QoS				(4)			9309

Unavailable Seconds			(2) 			Q.822

User Information Bit Loss Ratio					X.144

User Information Transfer Delay			X.140c2.2.1	X.144

User Information Frame Loss Ratio				X.144

### D.4.3	Reports

Alarm Report							X.161

Alarm Quality Of Service					X.161

Alarm Effect On Service Parameter c.2.9.1			M.3100

# appendix e - assumptions and information required

## E.1	Information required for Service Availability Calculation

In order to perform the Service Availability  calculation based on the formula and definitions found in [ref  1], the SMART Performance Reporting Team considered as valid only the SAP whose outage interval does not extend beyond the reporting period.

Derivation of the SAP outage interval may be done in conjunction with Trouble Ticketing, by passing Trouble Ticket information on affected SAPs from Trouble Ticketing to Performance Reporting. If this is done by notifying Performance Reporting of open and closed Trouble Tickets, an issue arises with unclosed Trouble Tickets which will distort the derived Availability calculations for affected SAPs.

Obviously the above assumption introduces a potential error in the SA determination which is directly related to the number and relative duration of the outage intervals that may be excluded. It is up to the individual service providers to determine how to handle these boundary conditions.  When an outage crosses Reporting Period boundaries the formula is indeterminate.

The essential components of the Performance Reporting data inputs are:

| Component | Description | Source |
| --- | --- | --- |
| SAP Group | SAP Group represents a group of SAPs against which Service Availability must be reported by Performance Reporting. Each SAP normally belongs to one (or more than one) SAP Group and each SAP Group contains at least one SAP. The association between SAP Groups and SAPs is defined within the associated SLA, according to the required computation and aggregation format. It is noted that some explicit or implicit SLAs may not require that all SAPs are reported against in this way | derived from Ordering or a SLA common object |
| Reporting Period | Reporting Period represents the period at which Performance Reports are generated. It is defined independently for each SAP Group within the associated SLA | derived from Ordering or from a SLA common object |
| SAP Activity Interval | SAP Activity Interval represents the time interval of a specific active period (i.e. when the Customer requires service from the SAP) within the defined Reporting Period. It must be measured separately for each active period of each SAP within a SAP Group | derived from Ordering or a SAP common object |
| SAP Activity Time | SAP Activity Time represents the total time of all SAP Activity Intervals of a specific SAP within the defined Reporting Period | derived from Ordering or a SAP common object |
| SAP Outage Interval | SAP Outage Interval represents the time interval of a specific outage period within the defined Reporting Period. An outage period is a period of service unavailability (say, due to fault) which occurs during a SAP Activity Interval for a given SAP, and which is not due to causes excluded by the associated SLA (e.g. force majeure). It must be measured for each outage period of each SAP within a SAP Group | derived from TT and Ordering (for exclusion conditions) or from a SAP common object |
| SAP Outage Time | SAP Outage Time represents the total time of all SAP Outage Intervals of a specific SAP within the defined Reporting Period | derived from TT and Ordering (for exclusion conditions) or from a SAP common object |
| SAP Weight | SAP Weight represents a value or relative priority attached to each SAP, and influences the relative significance placed on that SAP by the Service Provider (and eventually the Customer). It is defined independently for each SAP within the associated SLA | derived from Ordering (?) or from a SAP common object |

## E.2	Information required for further QoS Report Parameters

Some service providers wish to report on further QoS related parameters like:

- TTR		Time to Restore for a specific SAP

- MTTR	Mean Time to Restore for a specific SAP/SAP Group

- MTBF	Mean Time Between Failure for a specific SAP/SAP Group

- etc.

It is out of scope of the SPT’s work to standardise the above mentioned parameters in the customer to service provider contractual (SLA) environment. However, by using the proposed Service Availability calculation formula, service providers will have all information available to perform the respective calculations.

## E.3	Information required from the network

The PM-OSF (Performance Management Operations System Function) on the NML (Network Management Layer) has the overall knowledge of the network. When given the Managed Object Instance (e.g., a Leased Line circuit ID), it is capable of determining which Trail Termination Points to collect data from. The actual mechanism that a PM-OSF collects data from the physical network is beyond the scope of this document. The PR-OSSF assumes the following functionality from PM-OSF:

- The PM-OSF on NML must be able to maintain and/or retrieve PM history data for a particular network connection on an end-to-end basis.

- The number of instances for performance history data and aggregation intervals for data collection may be set by the PR-OSSF based on the performance reporting aggregation intervals for each service.

- The PM-OSF should support a registration capability to allow PR-OSSF to register a list of Managed Object Instances for periodical reporting. The registration process should allow the specification of the managed object instance and the set of performance monitoring data attributes. Upon the completion of a collection interval, the PM-OSF should report the data to the PR-OSSF.

- The PM-OSF must allow ad hoc query of history data.

## E.4	The use of common objects

Performance Reporting also expect to make use of common objects for:

- Customer (to associate all the SLAs for that Customer)

- Service Type (to identify service characteristics which may be referenced by individual SAPs)

- Service (to associate all the SAPs for that Service (e.g. for a multi-party connection)

## E.5	Initial administrations

The Performance Report OSSF depends on information from Service Ordering OSSF. The dependencies are illustrated in figure 3/3. Once the service order information is received and Performance Reporting Request is indicated, the customer account will be established if not existing, the service record and associated SLA ID will be stored. Customer log-in and access control procedures will also be initialised.

## E. 6	Data collection

Upon the service activation time, the PR-OSSF will initiate the data collection process. There are two sources for service performance data: the TT-OSSF for service availability and PM-OSF on the NML for network impairment and traffic data.

The data collection process involves setting up any registration and periodical reporting mechanism in the TT-OSSF and PM-OSF if supported. In the absence of a registration and reporting capabilities,  the PR-OSSF will set up a data polling mechanism.

## E.7	Data computation and aggregation

Upon receiving data from PM-OSF or TT-OSSF, the raw data will be computed to derive the corresponding service performance measurements. For example, the TT-OSSF will provide the outage duration associated with each Trouble Ticket for a particular service, it is incumbent on the PR-OSSF to calculate and aggregate the Service Availability (and if desired TTR, MTTR and MTBF) for a particular aggregation interval.

The customer may choose to receive reports over various aggregation intervals. For implementation consideration, the service provider may choose an “atomic” aggregation interval for each report type. For example, the QoS measurements are only meaningful over a longer period of time while traffic data are much more dynamic and require better resolution. The service provider may choose the month as the atomic aggregation interval for QoS reports and the hour as the atomic aggregation interval for traffic data reports. Reports based on longer aggregation intervals must be aggregatable from the atomic aggregation intervals (e.g., quarterly and annual reports for QoS; daily and monthly reports for traffic data).

The number of history reports available for customer access for each report type is a local policy issue.

# Appendix f - Table of figures

| Figure |  | Page |
| --- | --- | --- |
| Figure 2/1 | Performance Reporting requirements must be linked across management levels | 8 |
| Figure 3/1 | Simplified view of processes used by a service provider | 13 |
| Figure 3/2 | Performance Reporting Description, Inputs and Outputs | 14 |
| Figure 3/3 | Interaction of Performance Reporting with other Service Functions and Scope of the Performance Reporting Interface Definition | 15 |
| Figure 3/4 | Sequence Chart of Performance Reporting with Acknowledgement | 16 |
| Figure 3/5 | Sequence Chart of Performance Reporting without Acknowledgement | 17 |
| Figure 3/6 | Sequence Chart of Performance Reporting with Acknowledgement and optional logging | 18 |
| Figure 3/7 | Sequence Chart of on-line Performance Reporting Control | 19 |
| Figure 4/1 | Performance Reporting Event Flow Overview | 22 |
| Figure 4/2 | Performance Report State Model | 23 |
| Figure 5/1 | End-to-end service performance | 30 |