- LYXASS

LYXASS is a symbolic macro assembler accepting the hole _65SC02_ code !

(Yes, the Lynx only has a 65SC02 not a 65C02 (no BBRn or BBSn !!) )

LYXASS reacts as normal assembler, only that code-generation starts after

the RUN op-code; these means :

ORG $0          ; set PC at start of zero-page

dummy           dw 0            ; reserve 2 bytes for dummy => PC = 2

NameOfPlayer    ds 10           ; reserve 10 bytes => PC = 12

** above, only the PC advances, the lables get a value but no code

RUN $1000       ; set PC to $1000 and start code-genesis

sei             ; disable IRQs

...

See MAC- and VAR-files for more information.

- The assembler LYXASS creates absoluted based programs with following header:

dc.w $8008      ; BRA *+8

dc.w Start

dc.w Len        ; _with_ Header

dc.l "BS93"

- The ComLynx-Loader wants :

start-sequence : $81,"P"                        ; command : load program

init-sequence  : LO(Start),HI(Start)            ; start address = dest.

LO((Len-10) XOR $FFFF),HI((Len-10) XOR $FFFF)

; and len of data

Xmit-sequence  : ....                           ; data

checksum       : none at all !!