BVS  Digital Recorder Specification

Power Requirement :		AC 120 V / AC 240 V, 50 ~ 60 Hz

Operating Temperature :		-10℃ to 50℃

Approx. Weight :			8 Kg

Dimension:				460 D * 350 W * 90 H  mm

Signal System :			NTSC-type color signal and EIA monochrome signal

Video Input :				CVBS or S-Video (1 Vpp, 75 Ohm composite or Y/C)

Video Output :				CVBS or S-Video (1 Vpp, 75 Ohm composite or Y/C)

Video Compression : 		Industry standard JPEG compression

Recording Resolution :		640 * 480,  640 * 240,  320 * 480,  320 * 240

Recording Quality :			8 ~ 300 KB/Frame User programmable

Recording Frame Rate :		1 ~ 30 Frame/Sec and user programmable time lapse

Recording Media :			Hard-Disk

Recording System :			VDFS	Video Disk File System

Recording Method :		Recycle  FIFO  First In First Out

Time Code :				Every Frame with Time Code , Year 2000 available

PLAY SPEED :			1/90 ~ 300 X  Play Bidirection non noise , within Single Frame

Search Method :			Realtime Scrolling Playing , Date , Time , Session

( The Fastest Search in the WORLD )

Digital ZOOM :			32X  ~  1/32  with  FOLPPY DISK  output  BMP file

Video Editing :			Mark in , Mark out , Tag Target , Copy ( more than 2 HDD )

User Interface :			On screen display and mouse control

Remote Interface :			RS-232C, RS-422, RS-485, EtherNET 10BaseT / 100BaseT

Flash System :				All system in flash , just Video DATA in hard disk

Turnkey System :			Just one Button to turn power on  Easy to USE