Digital Recorder Specification

Power Requirement :		AC 90 V ~ AC 240 V, 50 ~ 60 Hz

Operating Temperature :		-10℃ to 60℃

Approx. Weight :			13 Kg

Signal System :			NTSC-type color signal and EIA monochrome signal

PAL-type color signal and EIA monochrome signal

Video Input :				CVBS or S-Video (1 Vpp, 75 Ohm composite or Y/C)

Video Output :				CVBS or S-Video (1 Vpp, 75 Ohm composite or Y/C)

Video Compression : 		Industry standard JPEG compression

Recording Resolution :		640 * 480,  640 * 240,  320 * 480,  320 * 240

Recording Quality :			8 ~ 300 KB/Frame User programmable

Recording Frame Rate :		1 ~ 30 Frame/Sec and user programmable time lapse

Recording Media :			Hard-Disk, DVD-RAM, DAT-Tape

Time Code :				Every Frame with Time Code , Year 2000 available

Control Input :				User programmable, DB-25 connector

Control Output :			User programmable, DB-25 connector

Remote Interface :			RS-232C, RS-422, RS-485, EtherNET 10BaseT / 100BaseT

PLAY SPEED :			1/30~300 X  Play Bidirection non noise , within Single Frame

User Interface :			On screen display and mouse control

Application Example :

Disk Capacity	(GB)			9		9		9 		9 		9		64

Rec. Quality (KB/Frame)		15		15		15		20		30		20

Rec. Frame Rate (Frame/Sec)		5		15		30		30		30		15

Rec. Capacity (KB/Sec)			75		225		450		600		900		300

Max. Rec. Time (Hour)			34.95	11.65	5.55		4.16		2.78		59.25