Protected Mode programming for the  intel386tm, intel486tm                                       	ANd pentiumtm processors 							(PART 1 of 3)

Title:		Protected Mode programming for the  						intel386, intel486 and pentium 								processors (PART 1 of 3)

Intel Ref. No: 	[7143.doc] [ ] [mfg_PB]

Faxback No:	7143

Prods. covered:	 80386, 80486, Pentium

Date:  		10/11/93

Related Info:	See Faxback documents [7144], [7145][7146]

Keywords:		Protected Mode, 386, 486, Pentium, software

FaxBACK* Service:

Quick, Simple, Easy. Available 24 hours a day!

1-800-628-2283 or (916)356-3105 (US or Canada)

+44(0)793-496646 (Europe)

Bulletin Board Services (BBS):

Latest Product Information and Support, 24 hours a day!

(916)356-3600 (US or Canada)

+44(0)793-496340 (Europe)

Download the Latest FaxBACK Catalogs and FaxFind Windows Utility

1-800-897-2536 (US or Canada)

Europe, call BBS # above and look under FaxBACK area in File Locator

Literature Hotline:

For Current Literature Guide  Order No. 210620

1-800-548-4725 (US or Canada)

Europe, contact local Intel office

* Other brands and names are the property of their respective owners.

for the

Intel386ä,

Intel486ä, and

Pentiumä Processors

September 1993,
Version 1.0

Intel Corporation makes no warranty for the use of its products and assumes no responsibility for any errors which may appear in this document, nor does it make a commitment to update the information contained herein.

Intel retains the right to make changes to these specifications at any time, without notice.

Contact your local Intel sales office or your distributor to obtain the latest specifications before placing your product order.

MDS is an ordering code only and is not used as a product name or trademark of Intel Corporation.

Intel Corporation and Intel's FASTPATH are not affiliated with Kinetics, a division of Excelan, Inc. or its FASTPATH trademark or products.

*Other brands and names are the property of their respective owners.

© INTEL CORPORATION, 1993

Contents

1.	Overview		1

1.1.	32-Bit Protected Mode	1

1.2.	Protected-Mode Environments	2

1.2.1.	DOS and DOS Extenders	2

1.2.2.	Windows and Win32s	2

1.2.3.	UNIX, OS/2 2.x, and Windows NT	3

1.3.	Development Tools for Protected Mode	4

1.4.	Tools Standardization	5

2.	The Example Application	6

3.	Timer Module	8

3.1.	Applying Protection to Interrupt Procedures	8

3.2.	Example Code	8

4.	Scheduler Module	10

4.1.	Applying Protection to the Scheduler	11

4.2.	Example Code	11

(See FaxBACK document#7144)

5.	Exception Handlers	13

5.1.	Applying Protection to Exception Handlers	13

5.2.	Example Code	14

6.	Sound Module	16

6.1.	Applying Protection to a Device Driver	16

6.2.	Example Code	16

7.	The Experiment Task	19

7.1.	Applying Protection to Application Programs	19

7.2.	Example Code	19

8.	Protected Mode Startup and Termination Module	21

8.1.	Applying Protection to Kernel Tasks	21

8.2.	Example Code	21

9.	Real Mode Startup and Termination Module	22

9.1.	Applying Protection to Startup and Termination	23

9.2.	Example Code	23

(See FaxBACK document #7145)

10.	Compiling and Executing the Example	33

11.	Where to Go From Here	34

12.	About the Authors	35

13.	Bibliography	35

List of Figures

1	Duty Cycle of Sound Waves	6

List of Tables

1	Tools Alternatives	5

(See FaxBACK document #7144)

2	Protection Structure	23

(See FaxBACK document #7145)

3	Tested Configurations	34

List of Examples

1	Jump Macros	7

2	Timer Module	9

3	Scheduler	11

(See FaxBACK document #7144)

4	Exception Handlers	14

5	Sound Device Driver	17

6	Experiment Task	19

7	Kernel Task	21

8	Protected Mode Equates	23

9	Protected Mode Macros	25

(See FaxBACK document #7145)

10	Startup and Shutdown	27

Since its introduction in 1985, the Intel386™ processor and later processors (the Intel486™ processor and the Intel Pentium™ processor) have operated in one of two modes: real-address mode and protected virtual-address mode. The real-address mode provides compatibility with earlier processors. Protected mode promotes increased performance and reliability in most software applications that are designed to use that mode. To help you capture the power of 32-bit protected mode in your own programming, this application note illustrates a number of protected-mode programming techniques via an example application that you can duplicate on your 32-bit, AT-compatible PC using MS-DOS* and the Microsoft* assembler MASM.

You will find this application note useful if you:

·	Are a programmer of custom operating systems or real-time executives.

·	Need to modify or enhance existing protected-mode operating systems, such as UNIX*, Windows NT*, or OS/2*.

·	Need to develop DOS memory managers or DOS extenders.

·	Just want to understand better how the use of protected mode can enhance your software application.

We assume that you have access to the basic facts about protected mode as presented in an Intel Programmer's Reference Manual or User's Manual for your processor or in one of the many third-party reference books (refer to the Bibliography for a list of some possibilities). You will probably want to have one of these books at hand as you read through this application note.

# 1	Overview

It is almost as easy to program applications for protected mode as to program them for real mode—even easier in some cases—however, systems level programming for protected mode can seem a little off-putting until you get some experience and actually feel the new power at your disposal. So, this note presents a simple application that allows you to take the processor for a test drive at full speed. In a short trip such as this, it is not possible to explore all the features of protected mode nor try out all the tools and environments that aid protected-mode programming. We will concentrate on protection, segmentation, and multitasking. Along the way, however, we will point out some other territories that you will want to return to later and explore on your own.

## 2	32-Bit Protected Mode

The first thing to note about protected mode is that it is the "native mode" of the processor. The processor starts executing in real-address mode (called just "real mode" by most people) because that mode is capable of executing unchanged all software designed for the 16-bit 8088 processor that was used in the original PC models. But most of the architectural enhancements made to processor generations since the 8088 are available only in protected mode. Following is a list of the most important of these enhancements:

·	32-bit architecture. The 32-bit registers and internal data paths double the speed at which data can be moved internally.

·	Extended addressing. Programs can address up to four gigabytes of physical memory.

·	Flexible processor-enforced protection. Two types of memory partitioning (segmentation and paging) create independent protected address spaces that promote protection and reliability. The two types of partitioning are both under control of the operating system, allowing it to select from a wide range of protection models.

·	Virtual memory. Programs can transparently address more memory than physically available on a given machine.

The 32-bit architecture provides the programming resources required to directly support "large" applications—those characterized by large integers, large data structures, large programs (or large numbers of programs), and so on.

## 3	Protected-Mode Environments

Applications programmers can use 32-bit protected mode easily if they are developing in a protected-mode environment. Following are the most widely known, commercially available protected-mode operating environments:

·	DOS extenders

·	Microsoft Windows* 3.x

·	UNIX SVR4, OS/2 2.x, and Windows NT

The following sections expand on each of these environments.

### 4	DOS and DOS Extenders

DOS does not and cannot execute in 32-bit, protected mode. However, DOS extenders can act as a mini-operating-system, and execute application programs in 32-bit protected mode. DOS extenders provide operating system services to programs in one of three ways:

·	By trapping DOS/BIOS calls, translating them, and delivering them to DOS/BIOS in either real mode or virtual 8086 mode.

·	By trapping DOS/BIOS calls and emulating them in the extender.

·	By providing extended (non-DOS) services via an extended API in the extender itself.

While there are many similarities among the various DOS extender products, there are also many differences. If you choose to use a DOS extender, you should arrange to use compilers, linkers, and debuggers that are compatible with the extender and with each other.

### 5	Windows and Win32s

Microsoft Windows 3.1 is not, strictly speaking, a 32-bit environment. When executing in its enhanced mode, Windows is a 16-bit DOS extender with an extensive API that isolates programs from DOS and from the AT hardware, and provides a uniform graphical user interface. It also provides nonpreemptive multitasking services.

Windows is important to 32-bit programming, however, because it weans applications from their dependency on the DOS API and the AT platform. Windows provides two migration paths for 16-bit applications to future 32-bit environments:

1.	The Windows 16-bit API is supported by 32-bit operating systems Windows NT and OS/2 2.1. In both cases, 16-bit API calls are converted to 32-bit API calls by interface procedures called "thunks," so it is likely that 16-bit applications will execute less efficiently in the 32-bit environment than they did in their original 16-bit environment.

2.	The Win32 API, the "native" API of Windows NT, is supported on Windows 3.1 in the form of the Win32s SDK.

The following excerpts about Win32s are taken from "An Overview of Win32s" in file W32SWH.TXT of the MSWIN32 forum on Compuserve:

Win32s is an extension of Windows version 3.1 that allows applications using a subset of the Win32 Application Programming Interface (API) to run unmodified on both Windows version 3.1 and Windows NT. The Win32s subset consists of Win32 equivalents of Win16 functions (Win16 is the 16-bit API offered in Windows version 3.1) as well as flat memory management and structured exception-handling features. Win32s programs will take full advantage of the 32-bit capabilities of the Intel 80x86 microprocessors. Using the Win32s API, developers can compile and run 32-bit code today that will be supported across Microsoft's 32-bit platforms tomorrow.

Regarding performance, the same source says:

Win32s is a perfect candidate for applications that are memory-intensive or calculation-intensive, such as CAD packages, paint packages, image manipulation tools, spreadsheet programs, and simulation software. Manipulating data in 32-bit mode improves the performance of these applications significantly. Win32s applications will also see performance improvements in Windows version 3.1, despite the slight overhead caused by 32-bit to 16-bit translation (thunking) that occurs during API calls. On Windows NT, Win32s applications will call Win32 functions directly; for this reason, a Win32s application will run faster than its Win16 counterpart on both Windows version 3.1 and on Windows NT.

Refer to this and other files in the MSWIN32 forum for more information on Win32s.

### 6	UNIX, OS/2 2.x, and Windows NT

UNIX is not just one operating system. There are many 32-bit versions of UNIX: UnixWare*, SCO* Open Desktop*, Dell UNIX, Interactive, NeXTStep*, and Solaris*, to name a few. All these versions of UNIX, as well as OS/2 version 2.x and Windows NT, release the full power of 32-bit protected mode. The 32-bit protected mode is their normal mode of operation. All provide full 32-bit memory management, preemptive multitasking, and virtual memory. All utilize a page-based, two-layered protection model with a separate linear address space for each task, and support the "flat" memory model for application programs.

Multiprocessor capability is another attraction of these operating systems. Many of them (Windows NT, OS/2 2.1, and several of the UNIX variants) are ready for symmetric multiprocessing (SMP) platforms. Each SMP operating system is designed so that it can be executed by any of the processors in a multiprocessor system. An SMP operating system offers multithreading features, so that both operating-system and application processes can be executed by more than one processor simultaneously. Thus, multithreaded software is as scalable as the hardware platform; increasing system performance becomes a matter of adding processors.

While the advantages of converting applications to 32-bit protected mode are many, these operating systems have not neglected 16-bit applications. In addition to their native, 32-bit execution environments, OS/2, Windows NT, and most of the UNIX variants have execution environments that support 16-bit DOS applications. Typically, this consists of executing DOS and its application program as a virtual 8086 mode task. Further, OS/2, Windows NT, and some of the UNIX variants have execution environments that support Windows 3.x applications. Here the variety of implementations is greater:

·	Windows NT executes each Windows 3.1 application as a thread within a single Windows compatibility process.

·	OS/2 can either execute each Windows 3.1 application as a separate OS/2 task or execute all Windows 3.1 applications as a single OS/2 task.

·	Some of the UNIX variants execute Windows 3.1 only in its standard mode treating it as any other DOS application.

For a more extensive overview of these operating systems, refer to the group of operating-system articles in PC Magazine, June 15, 1993.

## 7	Development Tools for Protected Mode

The tools formerly used for programming 16-bit processors and the 16-bit compatibility mode of the newer processors are not, in general, adequate for 32-bit, protected-mode programming. Following is a list of tools categories, each with a brief explanation of the new features needed to support 32-bit, protected-mode programming.

Object Format	Needs to accommodate 32-bit addresses, and new protected-mode data structures.

Operating System	Needs to manage 32-bit address space, new protected-mode data structures, new exceptions, and load new object formats.

Compiler 	Needs to generate new addressing modes and instructions, calls to new operating systems, and new object formats.

Linker 	Needs to deal with 32-bit addresses, and new object formats.

Debugger 	Needs to handle processors' new debugging features, exceptions, disassembly of new instructions, viewing of new data structures, and observe protection rules.

This example uses the minimum possible set of tools, so that you can run the example on your own PC without having to invest in additional tools. For real applications, however, you will probably need a more extensive toolbox. Table 1 lists the tools used by the example, along with the corresponding tools available for more sophisticated applications.

Table 1.  Tools AlternativesTC "1	Tools Alternatives" \F T \L 2§

| Tool | Used by Example | Alternatives |
| --- | --- | --- |
| Assembler/Compiler | Microsoft MASM | A variety of 32-bit compilers for languages such as C, Pascal, FORTRAN, COBOL, Lisp, etc. are available from the vendors listed in the file TVEND1.TXT1. |
| Linker | MASM Linker; some self-binding within the example program. | 32-bit linkers are usually provided by vendors of compilers or operating systems. |
| Loader | COMMAND.COM, the DOS loader, loads the program in real mode | Loaders for 32-bit object files are customarily built into 32-bit operating systems. DOS extenders are often loaded by DOS, after which the extender uses its internal loader to load the protected-mode program. |
| Operating system | Uses no run-time operating-system services. | UNIX, OS/2, Windows NT, DOS Extenders |
| Debugger | Royer Associates' debugger was used for the development of this example. | Refer to the file TVEND1.TXT1 for a list of debuggers or debugging environments for protected mode. |

NOTE 1. The file TVEND1.TXT contains a list of tool vendors. This file is located in the libraries of the Intel ACCESS forum (GO INTELACCESS) on Compuserve.

## 8	Tools Standardization

With the growing number of 32-bit operating environments—32-bit extended DOS, Windows, UNIX, and OS/2—comes the potential for proliferation of object file formats and fragmentation of the development tools that use these formats, namely compilers, loaders, and debuggers. To control such proliferation and fragmentation, a number of leading industry vendors  have joined together to form the Tool Interface Standards (TIS) committee. The charter of the TIS committee is "to standardize sets of technical specifications for interfaces visible to development tools and compilers." Tool standardization offers the following benefits to software developers:

·	Improved productivity by standardization of the development environment.

·	"Mix and match" tools leading to more choices and improved productivity. The developer can mix a favorite linker with a favorite debugger without regard to where those tools come from.

·	Improved "time to market" for multiplatform products.

·	Efficient development. Because there is less need to focus on porting, more R&D dollars can be used to add value.

The TIS standards developed so far are available from Intel Literature Distribution (refer to the bibliography) and from the Intel ACCESS forum on Compuserve (GO INTELACCESS).

# 9	The Example Application

To illustrate protected-mode programming, we have selected a simple psychoacoustical application. The application is the implementation of an experiment to answer the question "Do changes in the duty cycle of a sound wave produce any acoustical effect that can be perceived by the human ear?" Duty cycle refers to the proportion of time spent at positive air pressure compared to the total length of a sound wave, as Figure 1 shows.

Figure 1.  Duty Cycle of Sound WavesTC "1	Duty Cycle of Sound Waves" \F F \L 2§

To carry out the experiment, we will implement a protected-mode, real-time software system that runs on an AT-compatible system with an Intel386, Intel486, or Pentium processor. This software system has three general parts:

1.	Kernel. The kernel provides low-level, operating-system-like services. It comprises:

·	Start-up and shut-down procedures.

·	A timer manager that uses the programmable interval timer to maintain a software clock.

·	A scheduler that uses the software clock to dispatch tasks at requested intervals.

·	Exception handlers.

2.	Device driver. The example has one device driver that moves the diaphragm of the speaker of the AT-compatible system in and out repetitively at variable frequencies with variable duty cycle. The frequency and duty cycle are system parameters that can be changed by application tasks. The device driver calls on the scheduler to help the device driver make changes to the state of the speaker at appropriate time intervals.

3.	Application task. The application program varies the sound parameters to create various frequencies and various duty cycles at each frequency. The application also calls on the scheduler to help the application make changes to the sound parameters at specific time intervals.

We will run the experiment and listen to the sounds that the speaker creates.

The example has some of the features of a DOS extender, and therefore can execute in a DOS environment without the aid of any other software. But it is not a complete DOS extender. It does execute on an AT equipped with DOS. We use DOS to load the experiment, but otherwise we do not use any of the features of DOS. A real DOS extender, on the other hand, would help the 32-bit application access many of the services provided by 16-bit DOS. Another major function of a DOS extender is to allow programs to access extended memory (memory above one megabyte). The example, by placing the processor in protected mode, does enable its applications to access extended memory; however, the application programs do not take advantage of that ability, and the kernel provides no memory space management functions. The subject of extended addressing in protected mode has been well covered by other documentation (refer to the bibliography); the purpose of the example is to illustrate other features of protected mode: protection and task switching.

Example 1 shows the macros used throughout the example code to implement intersegment jumps. These macros provide the correct override prefixes for jumping between the various combinations of 16-bit and 32-bit segments.

Example 1.  Jump MacrosTC "1	Jump Macros" \F X \L 2§

;The following macro is used to execute a far call when in protected

;mode.  You must supply the desired offset and selector.  It also

;assumes 32-bit mode because we want the DD to give a 32-bit offset.

;If using this in 16-bit mode, you must change the DD to DW.

;This can be used for task switches or calling gates.

PCALLF32_32   MACRO   SEL1,OFF1

DB      9AH

DD      OFF1

DW      SEL1

ENDM

;This macro is for making a far call from a 32-bit segment to a

;16-bit segment.  The 66H tells the CPU to use a 16-bit operand

;instead of a 32-bit.  Assumes OFF1 is defined in a USE16 segment.

PCALLF32_16      MACRO   SEL1,OFF1

DB      66H

DB      09AH

DW      OFF1

DW      SEL1

ENDM

;This macro is for making a far jump from a 32-bit segment to a

;16-bit segment.  The 66H tells the CPU to use a 16-bit operand

;instead of a 32-bit.  Assumes OFF1 is defined in a USE16 segment.

PJMPF32_16      MACRO   SEL1,OFF1

DB      66H

DB      0EAH

DW      OFF1

DW      SEL1

ENDM

;This macro is for making a far jump from a 16-bit segment to a

;32-bit segment.  The 66H tells the CPU to use a 32-bit operand

;instead of a 16-bit.  Assumes OFF1 is defined in a USE32 segment.

PJMPF16_32      MACRO   SEL1,OFF1

DB      66H

DB      0EAH

DD      OFFSET OFF1

DW      SEL1

ENDM

;This macro is for making a far jump from a 32-bit segment to a

;32-bit segment.  This can be used for task switches.  Assumes OFF1 is

;defined in a 32-bit segment.

PJMPF32_32      MACRO   SEL1,OFF1

DB      0EAH

DD      OFF1

DW      SEL1

ENDM

;This macro is for making a far jump from a 16-bit segment to a

;16-bit segment.  This can be used for task switches.  Assumes OFF1 is

;defined in a 16-bit segment.

PJMPF16_16      MACRO   SEL1,OFF1

DB      0EAH

DW      OFF1

DW      SEL1

ENDM

The following sections explain each module in detail and discuss its protection needs. After all the modules have been presented, we will show how the descriptors of the segments of those modules are placed in descriptor tables. The order of presentation is from low-level modules to high-level modules.

# 10	Timer Module

The example application controls the movement of the cone of the AT's speaker in real time at audible frequencies; therefore, the application needs a very high-resolution clock. The clock is based upon interrupts from one of the timers in the AT's programmable interval timer. These interrupts illustrate the use of an interrupt procedure.

## 11	Applying Protection to Interrupt Procedures

Interrupt handlers can be implemented either as tasks or procedures. The interrupt task provides greater protection, while the interrupt procedure provides greater efficiency. In this case, we have chosen to make the timer interrupt handler a procedure because the application demands a very high-frequency clock tick. An interrupt procedure must execute at privilege level zero (PL0), because use of any other privilege level results in an illegal outward call when the timer interrupt occurs while the processor is already executing a PL0 procedure. The timer procedure's descriptors must be in the GDT, so that the timer interrupt can occur while any task is executing. The timer procedure's data is likewise stored in a global, PL0, data segment.

## 12	Example Code

The timer module contains several timer-related subroutines and the timer interrupt service routine. All are located in the kernel code segment.

·	A compile-time variable TIMER_FREQUENCY that specifies the timer tick interval so that it can be easily changed. This variable is available to other kernel procedures so that they can convert TIMER_COUNT into seconds.

·	A 32-bit real-time clock variable TIMER_COUNT that holds a count of clock ticks since program start-up.

·	A timer interrupt procedure TIMER_ISR. This procedure is very short, so that timer interrupts do not consume processor cycles excessively. It increments TIMER_COUNT and checks for overflow. Overflow of TIMER_COUNT causes the example to terminate. TIMER_COUNT is never reset.

·	An initialization procedure TIMER_INIT. This sets the programmable interval timer to interrupt at TIMER_FREQUENCY.

·	A shutdown procedure TIMER_RESTORE that restores the programmable interval timer to its original state, the state required for returning control to DOS.

·	A subroutine ENABLE_LEVEL_0 that enables the timer interrupt by programming the PIC.

Example 2.  Timer ModuleTC "2	Timer Module" \F X \L 2§

;**************** TIMER module

;It uses two segments: kernel code and kernel stack.

KERNEL_STACK32       SEGMENT

KERNEL_STACK32_BEGIN LABEL WORD

DD      400H     DUP(0)

KERNEL_STACK32_END LABEL WORD

KERNEL_STACK32       ENDS

;The _BEGIN and _END labels are used later to calculate

; the segment limit for the segment descriptor.

KERNEL_CODE32  SEGMENT USE32 PUBLIC

ASSUME  CS:KERNEL_CODE32,DS:KWAIT_DATA32,SS:KERNEL_STACK32

KERNEL_CODE32_BEGIN:

TIMER_FREQUENCY EQU     20000   ;units, cycles/sec

TIMER_VALUE     EQU     1193182/TIMER_FREQUENCY

TIMER_INIT      PROC

;This proc reprograms the CT0 on the 8254 Programmable Interval

;Timer, to count with the value, TIMER_VALUE. On the AT platform,

;the input clock of the 8254 is 1.193182 MHz, so a value of 119

;in TIMER_VALUE gives an output frequency of about 10 KHz.

;In general, frequency = 1.193182*10^6/TIMER_VALUE

MOV     AL,36H          ;Command: select timer 0

OUT     43H,AL          ;Write control word

MOV     AX,TIMER_VALUE  ;Clock divisor

OUT     40H,AL          ;Low byte of counter 0

MOV     AL,AH

OUT     40H,AL          ;High byte of counter 0

RET

TIMER_INIT      ENDP

TIMER_RESTORE       PROC

;This sets the timer back to the original frequency.

CLI                     ;Disable interrupts

MOV     AL,36H          ;Command: select counter 0

OUT     43H,AL          ;Write control word

MOV     AX,0FFFFH       ;Clock divisor used by DOS

OUT     40H,AL          ;Low byte of counter 0

MOV     AL,AH

OUT     40H,AL          ;High byte of counter 0

RET

TIMER_RESTORE   ENDP

ENABLE_LEVEL_0  PROC

;Enable the timer interrupt, level 0 on the master 8259.

IN      AL,PICA01       ;Get inerrupt mask

AND     AL,0FEH         ;Clear bit 0

OUT     PICA01,AL       ;Reprogram mask

RET

ENABLE_LEVEL_0  ENDP

;The timer interrupt routine simply increments TIMER_COUNT,

; setting the OVERFLOW_FLAG if necessary.

TIMER_ISR:              ;Interrupts are disabled upon entry

PUSH    EAX             ;Save registers

PUSH    DS

MOV     AX,KWAIT_DATA32_SEL  ;Load segment descriptor

MOV     DS,AX                ; of TIMER_COUNT

ADD     TIMER_COUNT,1   ;ADD is necessary because

JNC     TIMER_ISR_EXIT  ; INC does not set overflow.

INC     OVERFLOW_FLAG   ;Record overflow condition

TIMER_ISR_EXIT:

MOV     AL,EOI          ;PIC End-Of-Interrupt command

OUT     PICA00,AL       ;Send EOI to PIC

POP     DS              ;Restore registers

POP     EAX

IRETD               ;Return, restoring interrupt flag

KERNEL_CODE32  ENDS

# 13	Scheduler Module

A task (also called a process in UNIX and some other operating systems) is an instance of a program loaded into memory, together with its current state, which includes a pointer to the current instruction in the program. In the Intel 32-bit architecture, a task is represented by a task state segment (TSS), a processor-recognized memory structure that stores the state of the task when it is not being executed. The processor switches from the task that it is currently executing to a new task by loading the processor state from the TSS of the new task and beginning to execute the instructions of that task at the point indicated by the new instruction pointer. An operating system triggers a task switch by jumping or calling to the TSS of the new task or to a task gate for that TSS.

This scheduler is designed to cause events to happen at specified time intervals. The timer module gives the scheduler its sense of time; the scheduler has access to the real-time clock variable TIMER_COUNT. Events are implemented as tasks.

The mechanism for scheduling tasks for execution is round-robin and nonpreemptive; that is, each task in turn is given an opportunity to run and each task runs until it gives up the processor. Such a mechanism works as long as each task always gives up the processor voluntarily and uses little time before doing so.

## 14	Applying Protection to the Scheduler

The scheduler is an example of a critical OS kernel procedure that needs maximum protection; therefore, the scheduler is stored in a global, PL0 segment. The scheduler must be callable from any privilege level, so it has PL3 call gates, which ensure that other programs enter the procedure only at its intended entry points. The scheduler's data structures are stored in a global, PL0 segment so that they are accessible only by trusted PL0 procedures.

## 15	Example Code

The scheduler consists of

·	A circular linked list of all tasks. The entry for each task has the following fields:

1.	A 32-bit WAKE_UP_TIME with the same units as the real-time clock. WAKE_UP_TIME is initially set to zero.

2.	A pointer to the task's TSS.

3.	A link to the next entry in the list.

·	A pointer to the current task in the list.

·	Two wait procedures WAIT_TICKS(INTERVAL) and WAIT_MSECS(INTERVAL). These are actually two entry points to the same procedure, and each has a call gate. The two are identical, except for the units of INTERVAL — either timer ticks, or milliseconds which are converted to timer tick units. The common procedure sets the task's WAKE_UP_TIME to TIMER_COUNT + INTERVAL. Then it steps through the list of tasks, starting with the next task in turn, looking for the next ready task by comparing each task's WAKE_UP_TIME to TIMER_COUNT. A task is ready if its WAKE_UP_TIME is less than or equal to TIMER_COUNT. A task is not ready if its WAKE_UP_TIME is greater than TIMER_COUNT. If no tasks are ready, the scheduler loops indefinitely through the task list. Sooner or later some task's waiting period will expire as TIMER_COUNT advances. When WAIT finds a ready task, it checks whether it is the current task. If so, WAIT just returns. If it is another task, WAIT jumps to the TSS of that task, causing the processor to perform a task switch.

Example 3.  SchedulerTC "3	Scheduler" \F X \L 2§

;***************** WAIT (SCHEDULER) module

;The data segment contains the linked list of tasks

;and several other variables.

;The code segment contains the wait function

KWAIT_DATA32  SEGMENT USE32   PUBLIC

KWAIT_DATA32_BEGIN:

TASK_LINK       STRUC

WAKE_UP_TIME        DD      0       ;Wakeup value

THIS_TASK_OFFSET    DD      0       ;Dummy offset placeholder

THIS_TASK_SEL       DW      0       ;Selector to this task

NEXT_TASK_LINK      DD      0       ;Pointer to next task link

TASK_LINK       ENDS

CURRENT_TASK_LINK   DD      KERNEL_TASK_LINK

OLD_TASK_LINK       DD      0

TASK_LIST       LABEL   FWORD   ;List of scheduled tasks

APP_TASK_LINK    TASK_LINK <0,0,APP_TSS_SEL,SOUND_TASK_LINK>

SOUND_TASK_LINK  TASK_LINK <0,0,SOUND_TSS_SEL,KERNEL_TASK_LINK>

KERNEL_TASK_LINK TASK_LINK <0,0,KERNEL_TSS_SEL,APP_TASK_LINK>

TIMER_COUNT     DD      0       ;Count of clock ticks

OVERFLOW_FLAG   DB      0       ;Set when TIMER_COUNT overflows

KWAIT_DATA32_END:

KWAIT_DATA32        ENDS

KWAIT_CODE32  SEGMENT USE32 PUBLIC

ASSUME  CS:KWAIT_CODE32,DS:KWAIT_DATA32

KWAIT_CODE32_BEGIN:

WAIT_TICKS   PROC    FAR

;This is called when a task is done for the time being.

;It executes in the context of the calling task.

;EAX = WAIT TIME in clock tick units.  If EAX = -1,

;we have reached the end of the SOUND_TABLE

PUSH    EAX                     ;Save regs

PUSH    ESI

PUSH    EBX

PUSH    DS

MOV     BX,KWAIT_DATA32_SEL

MOV     DS,BX                   ;Load data segment of KERNEL

MOV     ESI,CURRENT_TASK_LINK   ;Get current task_link

MOV     OLD_TASK_LINK,ESI       ;Save task_link to current context

CMP     EAX,-1                  ;Check for end of sound-list

JNE     WAIT1                   ;Not reached

INC     OVERFLOW_FLAG           ;End of sound list

JMP     WAIT2

WAIT1:

ADD     EAX,TIMER_COUNT         ;Form wakeup time

WAIT2:

MOV     [ESI].TASK_LINK.WAKE_UP_TIME,EAX  ;Save new wakeup time

CHECK_NEXT:

CMP     OVERFLOW_FLAG,0         ;Time to shut down?

JE      CHECK1                          ;No

PJMPF32_32 KERNEL_TSS_SEL, 0 ;Yes, switch to kernel task

CHECK1:

MOV     EAX,[ESI].TASK_LINK.NEXT_TASK_LINK  ;Get ptr for next

MOV     CURRENT_TASK_LINK,EAX           ;Store ptr

MOV     ESI,CURRENT_TASK_LINK           ;Point to next task_link

MOV     EAX,[ESI].TASK_LINK.WAKE_UP_TIME  ;Get wake up time

CMP     TIMER_COUNT,EAX                 ;Time to wake up?

JB      CHECK_NEXT                      ;No

DISPATCH_THIS_ONE:

CMP     ESI,OLD_TASK_LINK       ;Are we in the task already?

JE      SAME_TASK               ;Yes, return to caller

; Switch to new task by jumping to its TSS

JMP     FWORD PTR [ESI].TASK_LINK.THIS_TASK_OFFSET

SAME_TASK:  ;returns here next time this task is dispatched

POP     DS  ;pop registers and return

POP     EBX

POP     ESI

POP     EAX

RET

WAIT_TICKS   ENDP

MIN_PERIOD      EQU     1

DEFAULT_PERIOD  EQU     1000

WAIT_MSECS       PROC    FAR ; Alternate entry point to WAIT_TICKS.

;This wait function is called by the application, it uses application

;units of duration, i.e. milliseconds. It converts to clock ticks,

;then jumps to the normal wait.  EAX = duration in milliseconds.

CMP     EAX,-1

JE      WAIT_TICKS          ;If -1, do not convert

MOV     EBX,1193            ;Timer input frequency

MUL     EBX                 ;EAX = EAX * EBX

MOV     EBX,TIMER_VALUE

DIV     EBX                 ;EAX = EAX / EBX

CMP     EAX,MIN_PERIOD

JA      CP2

MOV     EAX,DEFAULT_PERIOD  ;Use default

CP2:

JMP     WAIT_TICKS

WAIT_MSECS       ENDP

KWAIT_CODE32_END:

KWAIT_CODE32  ENDS