Protected Mode programming For the  intel386, intel486 ANd pentium processors 	(PART 3 of 3)

_______________________________________________________________________

Example 10.  Startup and Shutdown

tc "10	Startup and Shutdown"

; ************** Define the real-mode segments **************

STACK     SEGMENT   STACK USE16 ;Real-mode stack segment

DD   100H  DUP(0)

STACK_END       LABEL   WORD

STACK     ENDS

DATA      SEGMENT   USE16       ;Real-mode data segment

DATA_BEGIN      LABEL   WORD

;Define the GDT using equates. Each entry starts with an equate

;that defines the selector for that entry, and ends with a

;structure that puts most of the values in the descriptor. The

;base addresses are not filled in here; they are added at run time.

GDT_BEGIN       LABEL   WORD

NULL_SELECTOR EQU $-GDT_BEGIN   ;SELECTOR 00H

GDT     DD      0               ;THIS IS THE BEGINNING OF THE GDT

DD      0               ;NULL DESCRIPTOR

RET_CODE16_SEL EQU $-GDT_BEGIN

B   =   PRESENT+DPL0+ER_SEG+SEGMENT_DESC

RET_CODE16_DESC        S_DESC_386      <0FFFFH,0,0,B,0,0>

RET_DATA16_SEL EQU $-GDT_BEGIN

B   =   PRESENT+DPL0+RW_SEG+SEGMENT_DESC

RET_DATA16_DESC S_DESC_386      <0FFFFH,0,0,B,0,0>

DUMMY_TSS_SEL   EQU  $-GDT_BEGIN

B   =   PRESENT+DPL0+TSS_AVAIL_386+SYSTEM_DESC

DUMMY_TSS_DESC       S_DESC_386      <67H,0,0,B,D_BIT,0>

KERNEL_TSS_SEL   EQU  $-GDT_BEGIN

B   =   PRESENT+DPL0+TSS_AVAIL_386+SYSTEM_DESC

KERNEL_TSS_DESC       S_DESC_386      <67H,0,0,B,D_BIT,0>

SOUND_TSS_SEL   EQU  $-GDT_BEGIN

B   =   PRESENT+DPL0+TSS_AVAIL_386+SYSTEM_DESC

SOUND_TSS_DESC       S_DESC_386      <67H,0,0,B,0,0>

APP_TSS_SEL   EQU  $-GDT_BEGIN

B   =   PRESENT+DPL0+TSS_AVAIL_386+SYSTEM_DESC

APP_TSS_DESC       S_DESC_386      <67H,0,0,B,0,0>

SOUND_LDT_SEL   EQU  $-GDT_BEGIN

A   =   SOUND_LDT_END-SOUND_LDT_BEGIN-1

B   =   PRESENT+DPL0+LDT_DESC+SYSTEM_DESC

SOUND_LDT_DESC       S_DESC_386      <A,0,0,B,0,0>

APP_LDT_SEL   EQU  $-GDT_BEGIN

A   =   APP_LDT_END-APP_LDT_BEGIN-1

B   =   PRESENT+DPL0+LDT_DESC+SYSTEM_DESC

APP_LDT_DESC       S_DESC_386      <A,0,0,B,0,0>

WAIT_TICKS_GATE_SEL  EQU     $-GDT_BEGIN+3

B   =   PRESENT+DPL3+CALL_G_386+SYSTEM_DESC

A   =   OFFSET WAIT_TICKS

WAIT_TICKS_GATE    G_DESC_386   <A,KWAIT_CODE32_SEL,0,B,0>

WAIT_MSECS_GATE_SEL  EQU     $-GDT_BEGIN+3

B   =   PRESENT+DPL3+CALL_G_386+SYSTEM_DESC

A   =   OFFSET WAIT_MSECS

WAIT_MSECS_GATE    G_DESC_386   <A,KWAIT_CODE32_SEL,0,B,0>

CHANGESOUND_GATE_SEL  EQU     $-GDT_BEGIN+3

B   =   PRESENT+DPL3+CALL_G_386+SYSTEM_DESC

A   =   OFFSET CHANGESOUND

CHANGESOUND_GATE    G_DESC_386   <A,SOUND_CODE32_G_SEL,0,B,0>

KERNEL_CODE32_SEL EQU $-GDT_BEGIN

A   =   KERNEL_CODE32_END-KERNEL_CODE32_BEGIN-1

B   =   PRESENT+DPL0+ER_SEG+SEGMENT_DESC

KERNEL_CODE32_DESC       S_DESC_386      <A,0,0,B,D_BIT,0>

KERNEL_STACK32_SEL   EQU $-GDT_BEGIN

A   =   KERNEL_STACK32_END-KERNEL_STACK32_BEGIN-1

B   =   PRESENT+DPL0+RW_SEG+SEGMENT_DESC

KERNEL_STACK32_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

KWAIT_CODE32_SEL EQU $-GDT_BEGIN

A   =   KWAIT_CODE32_END-KWAIT_CODE32_BEGIN-1

B   =   PRESENT+DPL0+ER_SEG+SEGMENT_DESC

KWAIT_CODE32_DESC       S_DESC_386      <A,0,0,B,D_BIT,0>

KWAIT_DATA32_SEL   EQU $-GDT_BEGIN

A   =   KWAIT_DATA32_END-KWAIT_DATA32_BEGIN-1

B   =   PRESENT+DPL0+RW_SEG+SEGMENT_DESC

KWAIT_DATA32_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

SOUND_CODE32_G_SEL EQU  $-GDT_BEGIN+1

A   =   SOUND_CODE32_G_END-SOUND_CODE32_G_BEGIN-1

B   =   PRESENT+DPL1+ER_SEG+SEGMENT_DESC

SOUND_CODE32_G_DESC       S_DESC_386      <A,0,0,B,D_BIT,0>

SOUND_DATA32_G_SEL   EQU $-GDT_BEGIN+1

A   =   SOUND_DATA32_G_END-SOUND_DATA32_G_BEGIN-1

B   =   PRESENT+DPL1+RW_SEG+SEGMENT_DESC

SOUND_DATA32_G_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

GDT_END LABEL   WORD

;Define the LDT for the sound task

SOUND_LDT_BEGIN LABEL   WORD

SOUND_CODE32_L_SEL EQU $-SOUND_LDT_BEGIN+5

A   =   SOUND_CODE32_L_END-SOUND_CODE32_L_BEGIN-1

B   =   PRESENT+DPL1+ER_SEG+SEGMENT_DESC

SOUND_CODE32_L_DESC       S_DESC_386      <A,0,0,B,D_BIT,0>

SOUND_STACK32_L_SEL   EQU $-SOUND_LDT_BEGIN+5

A   =   SOUND_STACK32_L_END-SOUND_STACK32_L_BEGIN-1

B   =   PRESENT+DPL1+RW_SEG+SEGMENT_DESC

SOUND_STACK32_L_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

SOUND_STACK32_L_PL0_SEL   EQU $-SOUND_LDT_BEGIN+4

A   =   SOUND_STACK32_L_PL0_END-SOUND_STACK32_L_PL0_BEGIN-1

B   =   PRESENT+DPL0+RW_SEG+SEGMENT_DESC

SOUND_STACK32_L_PL0_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

SOUND_LDT_END LABEL   WORD

;Define the LDT for the application task

APP_LDT_BEGIN LABEL   WORD

APP_CODE32_SEL EQU $-APP_LDT_BEGIN+7

A   =   APP_CODE32_END-APP_CODE32_BEGIN-1

B   =   PRESENT+DPL3+ER_SEG+SEGMENT_DESC

APP_CODE32_DESC       S_DESC_386      <A,0,0,B,D_BIT,0>

APP_DATA32_SEL   EQU $-APP_LDT_BEGIN+7

A   =   APP_DATA32_END-APP_DATA32_BEGIN-1

B   =   PRESENT+DPL3+RW_SEG+SEGMENT_DESC

APP_DATA32_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

APP_STACK32_SEL   EQU $-APP_LDT_BEGIN+7

A   =   APP_STACK32_END-APP_STACK32_BEGIN-1

B   =   PRESENT+DPL3+RW_SEG+SEGMENT_DESC

APP_STACK32_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

APP_STACK32_PL1_SEL   EQU $-APP_LDT_BEGIN+5

A   =   APP_STACK32_PL1_END-APP_STACK32_PL1_BEGIN-1

B   =   PRESENT+DPL1+RW_SEG+SEGMENT_DESC

APP_STACK32_PL1_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

APP_STACK32_PL0_SEL   EQU $-APP_LDT_BEGIN+4

A   =   APP_STACK32_PL0_END-APP_STACK32_PL0_BEGIN-1

B   =   PRESENT+DPL0+RW_SEG+SEGMENT_DESC

APP_STACK32_PL0_DESC   S_DESC_386      <A,0,0,B,D_BIT,0>

APP_LDT_END LABEL   WORD

;Define the interrupt descriptor table (IDT)

IDT_BEGIN       LABEL   WORD

B   =   PRESENT+DPL0+INT_G_386+SYSTEM_DESC

A   =   OFFSET EXCEPTION_HANDLER0

IDT0_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER1

IDT1_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER2

IDT2_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER3

IDT3_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER4

IDT4_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER5

IDT5_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER6

IDT6_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER7

IDT7_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER8

IDT8_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER9

IDT9_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER10

IDT10_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER11

IDT11_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER12

IDT12_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER13

IDT13_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER14

IDT14_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER15

IDT15_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER16

IDT16_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

A   =   OFFSET EXCEPTION_HANDLER17

IDT17_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

DQ      (2FH-17)    DUP(0)

B   =   PRESENT+DPL0+INT_G_386+SYSTEM_DESC

A   =   OFFSET TIMER_ISR

TIMER_ISR_GATE_DESC   G_DESC_386      <A,KERNEL_CODE32_SEL,0,B,0>

DQ      (37H-30H)    DUP(0)

IDT_END LABEL WORD

INITIAL_MASK    DB  0       ;Initial interrupt mask storage

OLD_IDT_BASE_LIMIT  LABEL FWORD ;Real mode vector table address

OLD_IDT_LIMIT   DW  3FFH        ;Real mode limit

OLD_IDT_BASE    DD  0           ;Real mode base

;Define the data structures used by LGDT and LIDT instructions.

GDT_BASE_LIMIT       LABEL   FWORD      ;fword means 6 bytes

GDT_LIMIT       DW      (GDT_END-GDT_BEGIN)-1   ;THE GDT LIMIT

GDT_BASE        DD      ?       ;AND BASE ADDRESS OF THE GDT

IDT_BASE_LIMIT       LABEL   FWORD

IDT_LIMIT       DW      (IDT_END-IDT_BEGIN)-1   ;THE IDT LIMIT

IDT_BASE        DD      ?       ;AND BASE ADDRESS OF THE IDT

DUMMY_TSS     TSS_STRUC <>  ;dummy TSS used for 1st task switch

; Define task state segments (TSSs) for all tasks

INIT_FLAGS EQU 1202H        ;Initial contents of EFLAGS for

; each task.  IOPL = 01, IF = 1

KERNEL_TSS  TSS_STRUC     <,,,,,,,,,,\

,,OFFSET KERNEL_START32,INIT_FLAGS,,,,,\

OFFSET KERNEL_STACK32_END,,,,,\

,KERNEL_CODE32_SEL,,KERNEL_STACK32_SEL,\

,KWAIT_DATA32_SEL,,,,,,,,,>

SOUND_TSS   TSS_STRUC     <,,OFFSET SOUND_STACK32_L_PL0_END,\

SOUND_STACK32_L_PL0_SEL,,,,,,,\

,,OFFSET SOUND_START32,INIT_FLAGS,,,,,\

OFFSET SOUND_STACK32_L_END,,,,,\

,SOUND_CODE32_L_SEL,,SOUND_STACK32_L_SEL,\

,SOUND_DATA32_G_SEL,,,,,,SOUND_LDT_SEL\

,,,>

APP_TSS     TSS_STRUC     <,,OFFSET APP_STACK32_PL0_END,\

APP_STACK32_PL0_SEL,,\

OFFSET APP_STACK32_PL1_END,\

APP_STACK32_PL1_SEL,,,,\

,,OFFSET APP_START32,INIT_FLAGS,,,,,\

OFFSET APP_STACK32_END,,,,,\

,APP_CODE32_SEL,,APP_STACK32_SEL,\

,APP_DATA32_SEL,,,,,,APP_LDT_SEL\

,,,>

DATA_END        LABEL   WORD

DATA      ENDS

CODE      SEGMENT USE16

ASSUME    CS:CODE,DS:DATA,SS:STACK

CODE_BEGIN:

PICA00 EQU 20H  ;master 8259 port 0

PICA01 EQU 21H  ;master 8259 port 1

EOI    EQU 20H  ;End-of-interrupt command

REAL_8259       PROC

;This proc restores the master pic to the original setting.

MOV     AL,11H

OUT     PICA00,AL

MOV     AL,08H          ;Base vector = 8

OUT     PICA01,AL

MOV     AL,4            ;Slave on ir2

OUT     PICA01,AL

MOV     AL,1

OUT     PICA01,AL

MOV     AL,INITIAL_MASK ;Restore original mask

OUT     PICA01,AL

IN      AL,60H          ;Must clear keybd interrupt on

;some machines

RET

REAL_8259       ENDP

PROTECTED_8259     PROC

;This proc moves the 8259 to vector 30-37H and mask everything

;but the keyboard , IRQ1

IN      AL,PICA01       ;Save current mask

MOV     INITIAL_MASK,AL

MOV     AL,11H

OUT     PICA00,AL

MOV     AL,30H          ;New type = 30H

OUT     PICA01,AL

MOV     AL,4            ;Slave on level 2

OUT     PICA01,AL

MOV     AL,1

OUT     PICA01,AL

MOV     AL,11111111B    ;Mask all for now

OUT     PICA01,AL

RET

PROTECTED_8259  ENDP

START:

MOV     AX,DATA

MOV     DS,AX           ;SET UP DATA ADDRESSABILITY

;Put base addresses in all segment descriptors

PUT_BASE_IN_DESCRIPTOR DUMMY_TSS,DUMMY_TSS_DESC

PUT_BASE_IN_DESCRIPTOR KERNEL_TSS,KERNEL_TSS_DESC

PUT_BASE_IN_DESCRIPTOR KERNEL_CODE32_BEGIN,KERNEL_CODE32_DESC

PUT_BASE_IN_DESCRIPTOR KERNEL_STACK32_BEGIN,KERNEL_STACK32_DESC

PUT_BASE_IN_DESCRIPTOR KWAIT_DATA32_BEGIN,KWAIT_DATA32_DESC

PUT_BASE_IN_DESCRIPTOR KWAIT_CODE32_BEGIN,KWAIT_CODE32_DESC

PUT_BASE_IN_DESCRIPTOR SOUND_CODE32_G_BEGIN,SOUND_CODE32_G_DESC

PUT_BASE_IN_DESCRIPTOR SOUND_CODE32_L_BEGIN,SOUND_CODE32_L_DESC

PUT_BASE_IN_DESCRIPTOR SOUND_STACK32_L_BEGIN,SOUND_STACK32_L_DESC

PUT_BASE_IN_DESCRIPTOR SOUND_STACK32_L_PL0_BEGIN,SOUND_STACK32_L_PL0_DESC

PUT_BASE_IN_DESCRIPTOR SOUND_DATA32_G_BEGIN,SOUND_DATA32_G_DESC

PUT_BASE_IN_DESCRIPTOR SOUND_LDT_BEGIN,SOUND_LDT_DESC

PUT_BASE_IN_DESCRIPTOR SOUND_TSS,SOUND_TSS_DESC

PUT_BASE_IN_DESCRIPTOR APP_CODE32_BEGIN,APP_CODE32_DESC

PUT_BASE_IN_DESCRIPTOR APP_STACK32_BEGIN,APP_STACK32_DESC

PUT_BASE_IN_DESCRIPTOR APP_STACK32_PL1_BEGIN,APP_STACK32_PL1_DESC

PUT_BASE_IN_DESCRIPTOR APP_STACK32_PL0_BEGIN,APP_STACK32_PL0_DESC

PUT_BASE_IN_DESCRIPTOR APP_DATA32_BEGIN,APP_DATA32_DESC

PUT_BASE_IN_DESCRIPTOR APP_LDT_BEGIN,APP_LDT_DESC

PUT_BASE_IN_DESCRIPTOR APP_TSS,APP_TSS_DESC

INIT_GDTIDT_ADDRESS    GDT_BEGIN,GDT_BASE

INIT_GDTIDT_ADDRESS    IDT_BEGIN,IDT_BASE

PUT_BASE_IN_DESCRIPTOR CODE_BEGIN,RET_CODE16_DESC

PUT_BASE_IN_DESCRIPTOR DATA_BEGIN,RET_DATA16_DESC

LGDTD    GDT_BASE_LIMIT     ;Load GDT register

CLI

LIDTD    IDT_BASE_LIMIT     ;Load IDT register

CALL    PROTECTED_8259      ;Set up 8259

MOV     EAX,CR0     ;Fetch control register 0

OR      EAX,1       ;Set protection enable (PE) bit

MOV     CR0,EAX     ;Go to protected mode

JMP     S1          ;Flush instruction prefetch queue

S1:

MOV     AX,DUMMY_TSS_SEL ;Set up dummy TSS to receive

LTR     AX               ;Undefined state from 1st switch

; ************** First task switch **************

PJMPF16_32  KERNEL_TSS_SEL,0 ;switch to kernel task

RET_REAL:                       ;Shutdown, back to real mode

CLI                         ;Disable interrupts

MOV     AX,RET_DATA16_SEL   ;Load seg regs to set limit

MOV     DS,AX               ;to real mode 64K value

MOV     ES,AX

MOV     SS,AX

MOV     FS,AX

MOV     GS,AX

LIDTD   OLD_IDT_BASE_LIMIT  ;Set to vector table address

MOV     EAX,CR0             ;Fetch control register 0

AND     EAX,07FFFFFFEH      ;Clear PE bit

MOV     CR0,EAX             ;Go back to real mode

JMP     FAR PTR S16         ;Far jump to reload CS

S16:

MOV     AX,DATA             ;Load segment registers

MOV     DS,AX               ; with real mode values.

MOV     AX,STACK

MOV     SS,AX

MOV     SP,OFFSET STACK_END

CALL    REAL_8259           ;Restore 8259 to real mode values

STI                         ;Enable interrupts

MOV  AX,4C00H               ;Back to DOS

INT  21H                    ;DOS function call

CODE_END:

CODE      ENDS

AUTONUMLGL

Compiling and Executing the Example

The example runs on any minimally configured AT-compatible PC with a 32-bit processor. No extended memory is required, nor any peripherals beyond those found on all AT-class machines.

The example runs under DOS. It would be more precise to say that it runs alongside DOS, because, although DOS loads the example into memory, the example does not call on any DOS services after it is loaded. No other software is required for execution.

Table 3 shows the configurations on which the example has been tested. Because it is not very demanding of the assembler, of DOS, or of the hardware system, we expect that it will execute correctly on most other AT-compatible configurations as well.

Table 3.  Tested Configurations

| Processor | DOS Version | MASM Version |
| --- | --- | --- |
| 33 MHz Intel486 DX Microprocessor | 5.0 | 6.1 |
| 25 MHz Intel386 DX Microprocessor | 5.0 | 6.1 |
| 25 MHz Intel486 DX Microprocessor | 5.0 | 6.1 |
| 50 MHz Intel486 DX2 Microprocessor | 5.0 | 6.1 |
| 60 MHz Pentium | 5.0 | 6.1 |
| 33 MHz Intel486 DX Microprocessor | 5.0 | 6.0 |

The source code for this example is available from the Intel ACCESS forum on the Compuserve network (GO INTELACCESS) as file PMSND1.EXE. A command such as the following will compile and link the example program with MASM 6.0 or 6.1:

ML /Fl PMSOUND.ASM

Note that some versions of MASM (version 6.1, in particular) use a DOS extender and require the presence of a memory manager, such as EMM386. To execute the example, however, no programs may be present that use protected mode; for example DOS extenders, such as Microsoft Windows, memory managers, such as EMM386 and QEMM-386, and RAM disk programs. Such a software configuration is perhaps most easily obtained by preparing a bootable floppy disk with minimal AUTOEXEC.BAT and CONFIG.SYS files. Reboot the system from the floppy disk just before executing the example program.

We hope you enjoyed your test drive in protected mode. The techniques shown in this example are those that any 32-bit, protected-mode operating system needs to use. (The only basic techniques not illustrated are paging and virtual 8086 mode.) The example could serve as the seed for a real-time or embedded kernel, a full fledged OS, or a DOS extender. Possible territories for further exploration might include:

Real device drivers—for example, keyboard, video, disk, instrumentation, and appliances.

Expanded kernel functions—for example, dynamic memory allocation; priority-based, preemptive scheduler; intertask data sharing, message passing, and synchronization.

Exception handlers that provide more information about the location and possible cause of the exception.

Debugger.

Paging.

Dynamic task creation with loader.

Virtual memory with swap file.

Virtual-8086 mode and monitor, to extend DOS or to create multiple V86 tasks.

DOS extender.

Whether you use the example application either as a starting point or as a model for further development, or whether you choose to take advantage of one of the commercially available, 32-bit, protected-mode environments, we wish you a good trip.

About the Authors

Jeff Royer, Royer Associates; (415) 326-8079, Compuserve 71170,2452

Contact Royer Associates for more information about the debugger used for this example and on-site classes covering protected mode.

Loren Stafford, Dynalogic, Inc.; (415) 967-6355, Compuserve 72260,3045

For more information on protected-mode programming, refer to the following documents from Intel Corporation and from other sources. If you can't find a book in your local bookstore, you will be happy to know that many of these books are available from on-line bookstores on Compuserve. Enter FIND BOOK at any "!" prompt to obtain a list of on-line book sources.

To order Intel literature or to obtain literature pricing information in the U.S. and Canada, call or write Intel Literature Sales. In Europe and other international locations, please contact your local sales office or distributor. A list of sales offices and distributors can be found at the back of most Intel publications.

Intel Literature Sales

P.O. Box 7641

Mt. Prospect, IL 60056-7641

In the U.S. and Canada

call toll free

(800) 548-4725

80386 System Software Writer's Guide, Intel order number 231499.

Brey, Barry B. 1993. The Advanced Intel Microprocessors. New York: Macmillan.

Crawford, John H. and Patrick P. Gelsinger. 1987. Programming the 80386. San Francisco: Sybex.

Custer, Helen. 1992. Inside Windows NT. Redmond, WA: Microsoft Press.

Deitel, H.M. and M.S. Kogan. 1992. The Design of OS/2. Reading, MA: Addison-Wesley.

DOS Protected Mode Interface (DPMI) Specification, version 1.0, Intel order number 240977.

Duncan, Ray, ed. 1991. Extending DOS: A Programmer's Guide to Protected-Mode DOS. Reading, MA: Addison-Wesley.

Ezzel, Ben. 1993. The power under the hood—Review: Windows NT. PC Magazine. June 15.

Linthicum, David S. and Steven J. Vaughan-Nichols. 1993. The beast turns beauty—UNIX on Intel. PC Magazine. June 15.

Pappas, Chris H. and William H. Murray III. 1988. 80386 Microprocessor Handbook. Berkeley, CA: Osborne McGraw-Hill.

Pentium™ Processor User's Manual, Vol. 3: Architecture and Programming Manual, Intel order number 241430.

Prosise, Jeff. 1993. How DOS programs can use over 1MB of RAM. PC Magazine. June 29.

Robinson, Phillip, ed. 1988. Dr. Dobb's Toolbook of 80286/80386 Programming. Redwood City, CA: M&T Publishing.

Salemi, Joe. 1993. IBM's 32-bit challenger revisited—Preview: OS/2 2.1. PC Magazine. June 15.

Schulman, Andrew. 1990. Stalking GP faults. Dr. Dobbs Journal. January and February.

The source code for these articles is available on Compuserve in the Dr. Dobbs forum (GO DDJFORUM) as GPFLT2.ZIP.

TIS committee. Portable Formats Specification and Format Specification for Windows, Intel order number 241597.

Williams, Albert A. 1990. Roll your own DOS extender. Dr. Dobbs Journal. October and November.

The source code for these articles is available on Compuserve in the Dr. Dobbs forum (GO DDJFORUM) as PROT2.ASC.

Williams, Albert A. 1993. DOS and Windows Protected Mode: Programming with DOS Extenders in C. Reading, MA: Addison-Wesley.

In addition to the above printed literature, much information can be obtained on-line in the following Compuserve forums: