/*

v1.0 Royce Liao liaor@iname.com 07/21/2000

http://members.tripod.com/~liaor

Reconstructing the forward_DCT_row_1 function from inverse_DCT_row_1, AP-922

----------------------------------------------------------------------------

# Introduction

AP922 describes an algorithm for precise forward & inverse DCT(s).  The complete AP922 forward_DCT function requires the following steps :

1) forward_dct_col_4() // apply forward_dct to columns 0-3

2) forward_dct_col_4() // apply forward_dct to columns 4-7

3) for( i = 0; i < 8; i=i+1 )

forward_dct_row_1(i);     // apply forward_dct to row #i

AP-922 already gives a complete code listing for the forward_dct_col_4 operation, but leaves forward_dct_row_1 as an exercise to the reader.  This document describes the process used to use the information in AP-922 to convert the inverse_DCT_row_1 macro into forward_DCT_row_1 macro.

References : Intel Application Note AP-922   http://developer.intel.com/vtune/cbts/appnotes.htm - Applicaton Note 922

http://www.elecard.com/peter/idct.shtml - AP-922 implementation of inverse_dct

Equation (6) C8 = ½ * P8 * M8 * A8

(underlined items denote matrices)

Equation-6 shows the factorization of the forward_dct operator, C8.  The inverse_dct operator is C8-1 .  But C8 , is an orthogonal matrix, which has this convienent property : C8-1 = C8t (transpose.)

Therefore, the inverse_dct operator C8-1 = C8t = ½ * A8t * M8t * P8t.  We are given three items :

(1) the exact matrix-values for A8, M8, and P8.

(2) final MMX implementation code for the inverse_dct row function.

(3) Matrix expressions for fdct and idct : C8 and C8-1 respectively

Our objective is to derive the proper forward_dct row function (MMX implementation code), given the above items (1,2,3).  Our task is aided by several clues buried in AP-922.  But the basic approach is to work backward from the inverse_dct row function:

step 1, expand the idct operator C8-1

step 2, compare step(1) results with the MMX code (AP-922)

step 3, expand the fdct operator C8

step 4, compare step(3) reeults with steps (1) & (2)

step 5, write the fdct mmx code!

# Step 1 : Expansion of inverse-dct operator C8-1

C8-1 = C8t = ½ * A8t * M8t * P8t

Rewrite as C8-1 = (A8t * M8t)* P8t * ½

[A8t * M8t] = [1.pa] (partial result #1a)

/1  0  0  0  1  0  0  0\   /4  2  4  6  0  0  0  0\  /4  2  4  6  1  3  5  7\

| 0  1  0  0  0  1  0  0 | | 4  6 -4 -2  0  0  0  0 | |4  6 -4 -2  3 -7 -1 -5|

| 0  0  1  0  0  0  1  0 | | 4 -6 -4  2  0  0  0  0 | |4 -6 -4  2  5 -1  7  3|

| 0  0  0  1  0  0  0  1 | | 4 -2  4 -6  0  0  0  0 | |4 -2  4  6  7 -5  3 -1|

|                        |*|                        |=|                      |

| 0  0  0  1  0  0  0 -1 | | 0  0  0  0  1  3  5  7 | |4 -2  4  6 -7  5 -3  1|

| 0  0  1  0  0  0 -1  0 | | 0  0  0  0  3 -7 -1 -5 | |4 -6 -4  2 -5  1 -7 -3|

| 0  1  0  0  0 -1  0  0 | | 0  0  0  0  5 -1  7  3 | |4  6 -4 -2 -3  7  1  5|

\1  0  0  0 -1  0  0  0/   \0  0  0  0  7 -5  3 -1/  \4  2  4  6 -1 -3 -5 -7/

= [1.pa] (partial result #1a)

Let's redo the calculation, this time with position-placeholders in M8t.

#M8t =  /#00 #01 #02 #03 -   -   -   -  \

| #04 #05 #06 #07 -   -   -   -   |

| #08 #09 #10 #11 -   -   -   -   |

| #12 #13 #14 #15 -   -   -   -   |

|                                 |

| -   -   -   -   #16 #17 #18 #19 |

| -   -   -   -   #20 #21 #22 #23 |

| -   -   -   -   #24 #25 #26 #27 |

\-   -   -   -   #28 #29 #30 #31/

[ A8t * M8t(#) ] =

/1  0  0  0   1  0  0  0\   /0  1  2  3   -  -  -  -\

| 0  1  0  0   0  1  0  0 | | 4  5  6  7   -  -  -  - |

| 0  0  1  0   0  0  1  0 | | 8  9 10 11   -  -  -  - |

| 0  0  0  1   0  0  0  1 | |12 13 14 15   -  -  -  - |

|                         |*|                         | =

| 0  0  0  1   0  0  0 -1 | | -  -  -  -  16 17 18 19 |

| 0  0  1  0   0  0 -1  0 | | -  -  -  -  20 21 22 23 |

| 0  1  0  0   0 -1  0  0 | | -  -  -  -  24 25 26 27 |

\1  0  0  0  -1  0  0  0/   \-  -  -  -  28 29 30 31/

/  0   1   2   3   16  17  18  19\

|  4   5   6   7   20  21  22  23 |

|  8   9  10  11   24  25  26  27 |

| 12  13  14  15   28  29  30  31 |

|                                 |

| 12  13  14  15  -28 -29 -30 -31 |

|  8   9  10  11  -24 -25 -26 -27 |

|  4   5   6   7  -20 -21 -22 -23 |

\  0   1   2   3  -16 -17 -18 -19/

From this matrix, it is apparent that A8t mirrors the 1st 16 coefficents are mapped into the left half of the matrix.  The 2nd 16 coefficients are likewise mirrored and into the right quadrant, with negation on the lower-right quadrant.

Now it's time to add the input (X) and output (Y) vectors.

Let X = 1x8 column vector, representing one row of DCT coefficients

Let Y = 1x8 column vector, representing one row of raw pixel values

Y = (C8-1)*(X) = [½ * A8t * M8t * P8t * (X)] = [½ * (A8t * M8t)*(P8t * X)]

[P8t * X] = [1.pb] (partial result #1b)

/1 0 0 0  0 0 0 0\  / x0 \   / x0 \

| 0 0 1 0  0 0 0 0 | | x1 |   | x2 |

| 0 0 0 0  1 0 0 0 | | x2 |   | x4 |

| 0 0 0 0  0 0 1 0 | | x3 |   | x6 |

|                  |*|    | = |    | = (P8t * X) [1.pb] (partial result #1b)

| 0 1 0 0  0 0 0 0 | | x4 |   | x1 |

| 0 0 0 1  0 0 0 0 | | x5 |   | x3 |

| 0 0 0 0  0 1 0 0 | | x6 |   | x5 |

\0 0 0 0  0 0 0 1/  \ x7 /   \ x7 /

Now we combine the partial results #1a and #1b :

Y = C8-1final(X) = (A8t * M8t) * (P8t * X) =

/4  2  4  6  1  3  5  7\    /x0\     /a0+b0\     /y0\

|4  6 -4 -2  3 -7 -1 -5|   | x2 |   | a1+b1 |   | y1 |

|4 -6 -4  2  5 -1  7  3|   | x4 |   | a2+b2 |   | y2 |

|4 -2  4  6  7 -5  3 -1|   | x6 |   | a3+b3 |   | y3 |

|                      | * |    | = |       | = |    |

|4 -2  4  6 -7  5 -3  1|   | x1 |   | a3-b3 |   | y4 |
|4 -6 -4  2 -5  1 -7 -3|   | x3 |   | a2-b2 |   | y5 |

|4  6 -4 -2 -3  7  1  5|   | x5 |   | a1-b1 |   | y6 |

\4  2  4  6 -1 -3 -5 -7/    \x7/     \a0-b0/     \y7/

Where do a0,a1,a2,a3,b0,b1,b2,b3 come from?  A realization, that the expression {A8t * M8t} merely copies & reorders the 4x4 subcomponents of M8t.  The constant of ½ disappears into the table of precalculated coefficients.

/  0   1   2   3   16  17  18  19\     /x0\     /a0+b0\

|  4   5   6   7   20  21  22  23 |   | x2 |   | a1+b1 |

|  8   9  10  11   24  25  26  27 |   | x4 |   | a2+b2 |

| 12  13  14  15   28  29  30  31 |   | x6 |   | a3+b3 |

|                                 | * |    | = |       |

| 12  13  14  15  -28 -29 -30 -31 |   | x1 |   | a3-b3 |

|  8   9  10  11  -24 -25 -26 -27 |   | x3 |   | a2-b2 |

|  4   5   6   7  -20 -21 -22 -23 |   | x5 |   | a1-b1 |

\  0   1   2   3  -16 -17 -18 -19/     \x7/     \a0-b0/

# Step 2: Comparison of mathematical-expansion and idct_row_mmx_code

The above matrix expression can be rewritten as a system of scalar equations:

a0 = (mt00*x0) + (mt01*x2) + (mt02*x4) + (mt03*x6);

a1 = (mt04*x0) + (mt05*x2) + (mt06*x4) + (mt07*x6);

a2 = (mt08*x0) + (mt09*x2) + (mt10*x4) + (mt11*x6);

a3 = (mt12*x0) + (mt13*x2) + (mt14*x4) + (mt15*x6);

b0 = (mt16*x1) + (mt17*x3) + (mt18*x5) + (mt19*x7);

b1 = (mt20*x1) + (mt21*x3) + (mt22*x5) + (mt23*x7);

b2 = (mt24*x1) + (mt25*x3) + (mt26*x5) + (mt27*x7);

b3 = (mt28*x1) + (mt29*x3) + (mt30*x5) + (mt31*x7);

a[3..0], b[3..0] = matrix multiplication results

y[7..0] = output of {(C8-1)*(X)}

y[0] = a0+b0    y[7] = a0-b0

y[1] = a1+b1    y[6] = a1-b1

y[2] = a2+b2    y[5] = a2-b2

y[3] = a3+b3    y[4] = a3-b3

where mt[] is the numbered element taken from #M8t

From the scalar equations, it is apparent that the [8x8] * [1x8] matrix calculation produces 8 unique results, dividied into 2 groups of 4 elements (a[3..0], b[3..0]).  This is important, because later on we will expand the fdct operator (C8), and notice

A short C-language listing precedes the MMX dct_row_1 listing.  Our matrix expansion yields the same equations *exactly.*  The final idct_row outputs are computed with some post-multiplication ('a±b') arithmetic, and a final shift&round operation.  Later on, we will refer back to the MMX code comments in AP-922.

# Step 3 : expansion of fdct operator C8

Once again, we start with Equation (6) C8 = ½ * P8 * M8 * A8

First, we multiply the first two matrix terms : (P8 * M8)

(P8 * M8) = [3.pa] (partial result #3a)

( M8 has been changed to show only the active element#s.)

/1 0 0 0  0 0 0 0\   /0  1  2  3   -  -  -  -\   /0  1  2  3  -  -  -  -\

| 0 0 0 0  1 0 0 0 | | 4  5  6  7   -  -  -  - | | -  -  -  - 16 17 18 19 |

| 0 1 0 0  0 0 0 0 | | 8  9 10 11   -  -  -  - | | 4  5  6  7  -  -  -  - |

| 0 0 0 0  0 1 0 0 | |12 13 14 15   -  -  -  - | | -  -  -  - 20 21 22 23 |

|                  |*|                         |=|                        |

| 0 0 1 0  0 0 0 0 | | -  -  -  -  16 17 18 19 | | 8  9 10 11  -  -  -  - |

| 0 0 0 0  0 0 1 0 | | -  -  -  -  20 21 22 23 | | -  -  -  - 24 25 26 27 |

| 0 0 0 1  0 0 0 0 | | -  -  -  -  24 25 26 27 | |12 13 14 15  -  -  -  - |

\0 0 0 0  0 0 0 1/   \-  -  -  -  28 29 30 31/   \-  -  -  - 28 29 30 31/

The result [3.pa] is staggered in a checkerboard like arrangement.

Now it's time to add the input (X) and output (Y) vectors.

Let X = 1x8 column vector, representing one row of raw pixel values

Let Y = 1x8 column vector, representing one row of DCT coefficents

Y = (C8)*(X) = [½ * P8 * M8 * A8 * (X)] = [½ * (P8 * M8) *(A8 * X)]

Now (A8 * X) = [3.pb] (partial result #3b)

/1  0  0  0  0  0  0  1\     /x0\     /x0+x7\

| 0  1  0  0  0  0  1  0 |   | x1 |   | x1+x6 |

| 0  0  1  0  0  1  0  0 |   | x2 |   | x2+x5 |

| 0  0  0  1  1  0  0  0 |   | x3 |   | x3+x4 |

|                        | * |    | = |       |

| 1  0  0  0  0  0  0 -1 |   | x4 |   | x0-x7 |

| 0  1  0  0  0  0 -1  0 |   | x5 |   | x1-x6 |

| 0  0  1  0  0 -1  0  0 |   | x6 |   | x2-x5 |

\0  0  0  1 -1  0  0  0/     \x7/     \x3-x4/

Combining the partial results :

Y = [(P8 * M8) * (A8 * X)]

/0  1  2  3  -  -  -  -\     /x0+x7\     /a0\     /y0\

| -  -  -  - 16 17 18 19 |   | x1+x6 |   | b0 |   | y1 |

| 4  5  6  7  -  -  -  - |   | x2+x5 |   | a1 |   | y2 |

| -  -  -  - 20 21 22 23 |   | x3+x4 |   | b1 |   | y3 |

|                        | * |       | = |    | = |    |

| 8  9 10 11  -  -  -  - |   | x0-x7 |   | a2 |   | y4 |

| -  -  -  - 24 25 26 27 |   | x1-x6 |   | b2 |   | y5 |

|12 13 14 15  -  -  -  - |   | x2-x5 |   | a3 |   | y6 |

\-  -  -  - 28 29 30 31/     \x3-x4/     \b3/     \y7/

a0 = m00*(x0+x7) + m01*(x1+x6) + m02*(x2+x5) + m03*(x3+x4);

a1 = m04*(x0+x7) + m05*(x1+x6) + m06*(x2+x5) + m07*(x3+x4);

a2 = m08*(x0+x7) + m09*(x1+x6) + m10*(x2+x5) + m11*(x3+x4);

a3 = m12*(x0+x7) + m13*(x1+x6) + m14*(x2+x5) + m15*(x3+x4);

b0 = m16*(x0-x7) + m17*(x1-x6) + m18*(x2-x5) + m19*(x3-x4);

b1 = m20*(x0-x7) + m21*(x1-x6) + m22*(x2-x5) + m23*(x3-x4);

b2 = m24*(x0-x7) + m25*(x1-x6) + m26*(x2-x5) + m27*(x3-x4);

b3 = m28*(x0-x7) + m29*(x1-x6) + m30*(x2-x5) + m31*(x3-x4);

# Step 4: Comparison of fdct and idct expansions

inverse_dct operation : Yi = (C8-1)*(Xi) = [½ * A8t * M8t * P8t] * (Xi)

/ 0   1   2   3   16  17  18  19\     /x0\     /a0+b0\      /y0\

|  4   5   6   7   20  21  22  23 |   | x2 |   | a1+b1 |    | y1 |

|  8   9  10  11   24  25  26  27 |   | x4 |   | a2+b2 |    | y2 |

| 12  13  14  15   28  29  30  31 |   | x6 |   | a3+b3 |    | y3 |

|                                 | * |    | = |       | -> |    |

| 12  13  14  15  -28 -29 -30 -31 |   | x1 |   | a3-b3 |    | y4 |

|  8   9  10  11  -24 -25 -26 -27 |   | x3 |   | a2-b2 |    | y5 |

|  4   5   6   7  -20 -21 -22 -23 |   | x5 |   | a1-b1 |    | y6 |

\ 0   1   2   3  -16 -17 -18 -19/     \x7/     \a0-b0/      \y7/

forward_dct operation : Yf = (C8)*(Xf) = [½ * P8 * M8 * A8] * (Xf)

/0  1  2  3  -  -  -  -\     /x0+x7\     /a0\      /y0\

| -  -  -  - 16 17 18 19 |   | x1+x6 |   | b0 |    | y1 |

| 4  5  6  7  -  -  -  - |   | x2+x5 |   | a1 |    | y2 |

| -  -  -  - 20 21 22 23 |   | x3+x4 |   | b1 |    | y3 |

|                        | * |       | = |    | -> |    |

| 8  9 10 11  -  -  -  - |   | x0-x7 |   | a2 |    | y4 |

| -  -  -  - 24 25 26 27 |   | x1-x6 |   | b2 |    | y5 |

|12 13 14 15  -  -  -  - |   | x2-x5 |   | a3 |    | y6 |

\-  -  -  - 28 29 30 31/     \x3-x4/     \b3/      \y7/

Comparing the forward_dct and inverse_dct matrix expressions, we see three key differences :

1) forward_dct : input data requires reordering

inverse_dct : input data requires reordering + arithmetic

2) forward_dct : mult-matrix is 4x4 mirrored (and negated for lower-right)

inverse_dct : mult_matrix is checkerboard staggered

3) forward_dct : output result is directly obtainable from {a[3..0],b[3..0]}

inverse_dct : output result requires 'a±b' arithmetic

The first difference is easy to see.  The inverse_dct's input_operator shuffles the input X.  The forward_dct's input_operator requires additional arithmetic to prepare the input data (prior to multiplication.)  Conclusion : The final MMX-code for forward_dct_row will require some extra arithmetic instructions at the front-end.

The 2nd and 3rd differences are examined together.  The result of (C8*Xf) is a vector with 8 unique elements.  Each element corresponds to one output element y[7..0].  Therefore, the forward_dct and inverse_dct produce the output-result vector in *identical* form.  However, closer examination of the mmx_idct_row function reveals 'paddd/psubd' instructions for performing 'a±b' arithmetic.  This operation is absent from the forward_dct.  Conclusion : The final MMX-code for forward_dct_row will remove final 'a±b' arithmetic (paddd/psubd) instructions.

As a final note, the coefficient table ('w[]') must be transposed.  The transpose is actually a convenient optimization.  M8 is orthogonal implies : M8-1 = M8t.

# Step 5 : Reconstructed forward_dct_row()

We are now ready to write some code.  Let's list the necessary changes to convert the idct_row_mmx() code into fdct_row_mmx() :

1) The input vector x[7..0] requires some arithmetic preparation :

INPUT    Permuted        Permuted

A8                   X      Input Xft       Input Xit

-----   forward_dct     inverse_dct

/1  0  0  0  0  0  0  1\     /x0\     /x0+x7\          /x0\

| 0  1  0  0  0  0  1  0 |   | x1 |   | x1+x6 |        | x2 |

| 0  0  1  0  0  1  0  0 |   | x2 |   | x2+x5 |        | x4 |

| 0  0  0  1  1  0  0  0 |   | x3 |   | x3+x4 |        | x6 |

|                        | * |    | = |       |        |    |

| 1  0  0  0  0  0  0 -1 |   | x4 |   | x0-x7 |        | x1 |

| 0  1  0  0  0  0 -1  0 |   | x5 |   | x1-x6 |        | x3 |

| 0  0  1  0  0 -1  0  0 |   | x6 |   | x2-x5 |        | x5 |

\0  0  0  1 -1  0  0  0/     \x7/     \x3-x4/          \x7/

As a note of caution, the inverse_dct_mmx() code already performs a permutation on the input vector X.  The forward_dct_mmx()'s permutation must simultaneously undo the inverse_dct's permutation while applying its own.

2) the tables of coefficients must each be transposed.

The upper 16 coefficients, w[15..0] corresponding to a 4x4 submatrix, must be transposed.  The lower 16 coefficients are diagonally symmetric.  Since the transpose of a diagonally symmetric matrix is equal to itself, there is no need to transpose the lower 4x4 submatrix (coefficients w[31..16]).

3) The row_idct_mmx()'s 'a±b' arithmetic should be removed.

The multiplication output (a[3..0], b[3..0]) are directly mapped to the output vector Y.

Below is a C-equivalent function for the mathematics described in this document.  The MMX implementation reuses 90% of the AP-922's mmx_dct_row() code listing.

*/

void

dct_8_frw_row_1( short * blk )

{

static short xt[8];

static *sptr, *optr, *tf;

static short a3, a2, a1, a0, b3, b2, b1, b0;

int j;

// FDCT row transform

sptr = (short *) blk;

optr = ( short * ) blk;

for ( j = 0; j < 8; j=j+1 )

{

// inverse_dct_row input-shuffling, pre-multiplication

// xt[0] = sptr[0];

// xt[1] = sptr[2];

// xt[2] = sptr[4];

// xt[3] = sptr[6];

// xt[4] = sptr[1];

// xt[5] = sptr[3];

// xt[6] = sptr[5];

// xt[7] = sptr[7];

// forward_dct_row input arithmetic + shuffle

xt[3] = sptr[3] + sptr[4];

xt[2] = sptr[2] + sptr[5];

xt[1] = sptr[1] + sptr[6];

xt[0] = sptr[0] + sptr[7];

xt[7] = sptr[3] - sptr[4];

xt[6] = sptr[2] - sptr[5];

xt[5] = sptr[1] - sptr[6];

xt[4] = sptr[0] - sptr[7];

//  Let w[32] = {   // from AP-922, table for *INVERSE TRANSFORM*

//      ; FIX(cos_4_16), FIX(cos_2_16),  FIX(cos_4_16),  FIX(cos_6_16),

//      ; FIX(cos_4_16), FIX(cos_6_16),  -FIX(cos_4_16), -FIX(cos_2_16),

//      ; FIX(cos_4_16), -FIX(cos_6_16), -FIX(cos_4_16), FIX(cos_2_16),

//      ; FIX(cos_4_16), -FIX(cos_2_16), FIX(cos_4_16),  -FIX(cos_6_16),

//      ;

//      ; FIX(cos_1_16), FIX(cos_3_16),  FIX(cos_5_16),  FIX(cos_7_16),

//      ; FIX(cos_3_16), -FIX(cos_7_16), -FIX(cos_1_16), -FIX(cos_5_16),

//      ; FIX(cos_5_16), -FIX(cos_1_16), FIX(cos_7_16),  FIX(cos_3_16),

//      ; FIX(cos_7_16), -FIX(cos_5_16), FIX(cos_3_16),  -FIX(cos_1_16) };

//

//  Then w'[32] = {   // transpose of 4x4 submatrixes of w[32], same as M_8

//      ; FIX(cos_4_16), FIX(cos_4_16),  FIX(cos_4_16),  FIX(cos_4_16),

//      ; FIX(cos_2_16), FIX(cos_6_16),  -FIX(cos_6_16), -FIX(cos_2_16),

//      ; FIX(cos_4_16), -FIX(cos_4_16), -FIX(cos_4_16), FIX(cos_4_16),

//      ; FIX(cos_6_16), -FIX(cos_2_16), FIX(cos_2_16),  -FIX(cos_6_16),

//      ;

//      ; FIX(cos_1_16), FIX(cos_3_16),  FIX(cos_5_16),  FIX(cos_7_16),

//      ; FIX(cos_3_16), -FIX(cos_7_16), -FIX(cos_1_16), -FIX(cos_5_16),

//      ; FIX(cos_5_16), -FIX(cos_1_16), FIX(cos_7_16),  FIX(cos_3_16),

//      ; FIX(cos_7_16), -FIX(cos_5_16), FIX(cos_3_16),  -FIX(cos_1_16) };

//

// table_inverse_w = { // (matrix M_8_transpose from AP-922)

//    w06 w04 w02 w00

//    w07 w05 w03 w01

//    w14 w12 w10 w08

//    w15 w13 w11 w09

//

//    w22 w20 w18 w16

//    w23 w21 w19 w17

//    w30 w28 w26 w24

//    w31 w29 w27 w25

//   };

//

// table_forward_w = { // (matrix M_8 from AP-922)

//    w09 w01 w08 w00

//    w13 w05 w12 w04

//    w11 w03 w10 w02

//    w15 w07 w14 w06

//

//    w22 w20 w18 w16

//    w23 w21 w19 w17

//    w30 w28 w26 w24

//    w31 w29 w27 w25

// };        //

//

// There are 8 tables total (table_forward[7..0]), where

//   tables 0 & 4 = table_forward_w[] * cos_4_16

//   tables 1 & 7 = table_forward_w[] * cos_1_16

//   tables 2 & 6 = table_forward_w[] * cos_2_16

//   tables 3 & 5 = table_forward_w[] * cos_3_16

//tf = &tab_inv_01234567[ 32*j ]; // idct_row load table_inverse_w

tf = &tab_frw_01234567[ 32*j ]; // fdct_row load table_forward_w

a3 = ( xt[0]*tf[10]+ xt[1]*tf[14]) + ( xt[2]*tf[11]+ xt[3]*tf[15]);

a2 = ( xt[0]*tf[8] + xt[2]*tf[9] ) + ( xt[1]*tf[12]+ xt[3]*tf[13]);

a1 = ( xt[0]*tf[2] + xt[2]*tf[3] ) + ( xt[1]*tf[6] + xt[3]*tf[7] );

a0 = ( xt[0]*tf[0] + xt[2]*tf[1] ) + ( xt[1]*tf[4] + xt[3]*tf[5] );

tf += 16;  // increment table pointer

b3 = ( xt[4]*tf[10]+ xt[6]*tf[11]) + ( xt[5]*tf[14]+ xt[7]*tf[15]);

b2 = ( xt[4]*tf[8] + xt[6]*tf[9] ) + ( xt[5]*tf[12]+ xt[7]*tf[13]);

b1 = ( xt[4]*tf[2] + xt[6]*tf[3] ) + ( xt[5]*tf[6] + xt[7]*tf[7] );

b0 = ( xt[4]*tf[0] + xt[6]*tf[1] ) + ( xt[5]*tf[4] + xt[7]*tf[5] );

tf += 16;  // increment table pointer

// apply rounding constants to scaled elements

a3 = a3 + RND_FRW_ROW;

a2 = a2 + RND_FRW_ROW;

a1 = a1 + RND_FRW_ROW;

a0 = a0 + RND_FRW_ROW;

b3 = b3 + RND_FRW_ROW;

b2 = b2 + RND_FRW_ROW;

b1 = b1 + RND_FRW_ROW;

b0 = b0 + RND_FRW_ROW;

// note, the MMX implementation applies the shifts after all other

// arithmetic operations.  In inverse_dct_row, the shift is performed

// *after* the 'a+b' arithmetic.

a3 = a3 >> SHIFT_FRW_ROW;

a2 = a2 >> SHIFT_FRW_ROW;

a1 = a1 >> SHIFT_FRW_ROW;

a0 = a0 >> SHIFT_FRW_ROW;

b3 = b3 >> SHIFT_FRW_ROW;

b2 = b2 >> SHIFT_FRW_ROW;

b1 = b1 >> SHIFT_FRW_ROW;

b0 = b0 >> SHIFT_FRW_ROW;

// inverse_dct_row : post-multiplication arithmetic

// *** NOT USED BY forward_dct ***

// optr[ 0 ] = a0+b0;

// optr[ 1 ] = a1+b1;

// optr[ 2 ] = a2+b2;

// optr[ 3 ] = a3+b3;

// optr[ 4 ] = a3-b3;

// optr[ 5 ] = a2-b2;

// optr[ 6 ] = a1-b1;

// optr[ 7 ] = a0-b0;

// forward_dct_row, assignment outputs

optr[ 3 ] = b1;

optr[ 2 ] = a1;

optr[ 1 ] = b0;

optr[ 0 ] = a0;

optr[ 7 ] = b3;

optr[ 6 ] = a3;

optr[ 5 ] = b2;

optr[ 4 ] = a2;

sptr += 8;   // increment source pointer +1 row

optr += 8;   // increment output pointer +1 row

} // end for ( j = 0 ...

}