This is casport                                                 Version 0.5.0

===============             U S E R   M A N U A L               =============

casport is (c) Copyright 1996 Tom Wheeley, All Rights Reserved.

Author : tomw@tsys.demon.co.uk

Support: support@tsys.demon.co.uk

=============================================================================

Table of Contents

=================

: 1 :  Overview

. Rationale

. Capabilities

: 2 :  Basic operation of casport

. Receiving data

. Transmitting data

: 3 :  Command line switches

. Display options

. COM port settings

. Overwriting programs

: 4 :  Troubleshooting

: 5 :  Bugs and Limitations

: 1 :  Overview

===============

Rationale

~~~~~~~~~

casport is a small add-on utility to cas to enable users to transfer

programs and data to and from the calculator.

Capabilities

~~~~~~~~~~~~

casport can:

. Transmit and receive any cas file

: 2 :  Basic operation of casport

=================================

Receiving Data

~~~~~~~~~~~~~~

1)  Make sure the calculator is connected to the computer

2)  Type `casport -r wibble.cas', followed by any neccesary communications

options (see section 3).  casport is now ready to receive data.

3)  From the Link option of the calculator, choose Transmit, and select

which type of data to send, and send it.

4)  casport should now receive the data, and save it in wibble.cas.

Transmitting Data

~~~~~~~~~~~~~~~~~

1)  Make sure the calculator is connected to the computer

2)  From the Link option of the calculator, choose Receive, and

then select the type of data which you will be sending.  Type

`cas wibble.cas' to see the type of file if you are unsure.

Set the calcualtor to start receiving

3)  Type `casport -t wibble.cas' followed by any neccesary communications

options (see section 3).

4)  casport should now transmit the data to the calculator .

: 3 :  Command line switches

============================

Display options

~~~~~~~~~~~~~~~

Using the -v switch will display verbose information on what casport

is doing.  Using -d as well will display more (debug) information.

If you are having problems, then you should invoke casport with both these

switches.

COM port settings

~~~~~~~~~~~~~~~~~

casport allows you to set the following settings:

COM port

Baud rate

Parity checking

The COM port can be any one of 1, 2, 3 or 4.

The baud rate and parity can be set to the same range as supported by the

calculator.

* COM port

This must be set to the same value as the port which your cable is

plugged  into.  The most common value is COM2, (as mice are generally in

COM1).

To specify the comport, use the -c=X switch, where X is 1,2,3 or 4.

* Baud rate

This must match the baud rate for which the calculator is set up.  Generally

you should have no problems with 9600 baud, but if strange checksum errors

occur, try a lower rate.

To specify the baud rate, use -b=XXXX, where X is 1200, 2400, 4800 or 9600

* Parity

This must also match the parity set up in the calculator.  To specify the

parity,use  -p=Odd, -p=Even or -p=None.

* Defaults

If you omit any or all of these switches, the unspecified communication

parameters revert to the following values:

-c=2      Cable plugged in on COM2

-b=9600   Running at 9,600baud

-p=Even   Even parity

Overwriting programs

~~~~~~~~~~~~~~~~~~~~

To specify that you want to overwrite any existing programs in the slot

you chose, use the -o switch.  If you don't then cas will exit with an error

if a program exists in the slot you choose.

If you are transmitting a Multiple Program file, then unfortunately

*every* program is overwritten, even if the overwriting program is blank.

If you want to `merge' the new set of programs into the calculator you can

either:

a) Upload every program separately

b) Use the following steps:

. Download the current set of programs, then use cas to convert them to

text.

. Convert your new set of programs to text

. Add the new set of programs onto the end of the first using a text

editor

. Remove any duplicate programs in the text file

. Use cas to convert the text file into a Multiple Program cas file

. Transmit the new cas file to the calculator, using the -o switch

Future versions of cas/casport will probably do this automatically.

: 4 : Troubleshooting

=====================

Communications are an error prone business, here are some suggestions if you

get an error:

* Calculator says RECEIVE ERROR immediately as I send data to it.

. Are you running Windows95?  If any open program has been using the COM

port, then you will get this error.

. Are the communication settings set up properly? (see section 3)

. Is your cable plugged in properly?

* "Checksum error"

Either the cas file you are sending is corrupted, or

* "Program area already in use"

The program area is full.  Use the -o switch (see section 3)

* "Calculator is out of free memory"

The calcualtor has not got enough memory to load the program.  Remove

some programs/data in the calcualtor and retry.

* "Wrong kind of data"

The calculator is expecting some other kind of data tahn you are

transmitting.  Did you send the right cas file?  Did you tell the

calculator the right type of file?

: 5 :  Hacking Utilities

========================

casport comes with two hacking utilities, designed to send and receive

anything from the casio calculator.

Getcas

~~~~~~

This will receive any .cas file from the calculator.  The syntax is

identical to casports, except for the -o and -r/-t switches are missing.

getcas filename [-v] [-d] [comms settings]

Using -v or -d will show progress indicators, too.

Getcas is not particularly well written, so it may be too slow on slower

computers, or not function properly on too fast computers.  (Runs ok on

my P133, 16550).

Putcas

~~~~~~

Putcas will send any .cas file to the calculator.  It is invoked in the

same way as getcas.

Putcas is *extremely* slow, and you should use casport whenever possible.

Putcas does not have status indicators at present.

: 6 :  Bugs and Limitations

===========================

. casport can only cope with Single Program and Multiple Program

communications

*Do* let me know if you find any bugs, especially the nasty ones which

display half a screen of numbers.  Look for the line saying `Exiting due to

to signal....'.  Send me the command line, files used as well as debug and

verbose output!  Send bug reports to <support@tsys.demon.co.uk>

--------

TW 96/08/13