# Procedural Texturing Applied to the Radiosity Method

Ricardo Birtel Mendes de Freitas 1

Marcelo Dreux 2

Mechanical Engineering Department,

Pontifícia Universidade Católica do Rio de Janeiro - PUC-Rio

1 birtel@geocities.com

2 dreux@mec.puc-rio.br

Key Words:  Radiosity, Procedural Texturing

Abstract

Diffuse reflections from a given surface can be accurately modeled by considering energy transference between surfaces, subject to conservation of energy laws. The technique which implements that concept is generally known as Radiosity method. Realistic images of a scene that contains mostly diffuse surfaces can be produced using that technique.

On the other hand, the original method is suited mostly to the fair representation of smooth surfaces.  However,  most objects do not have smooth, even surfaces. Modifying the Radiosity original technique to comprise those surfaces would be an extremely hard task to apply on the global illumination model level. A good compromise in order to avoid that difficulty would be to pos-render  the surfaces on a local illumination model, just after the application of the global illumination model. Such techniques to add surface detail on a local illumination level are known as Texture methods, of which Surface mapping, Bump mapping and Procedural texturing are examples.

The present work is an attempt to produce realistic images using the blending of the two described techniques. Using an existing Radiosity software package as a starting point, the authors have incorporated procedural texturing techniques on a pos-render base. The result was the production of   images, that though not entirely accurate (from the energy conservation point of view), convey a more "realistic" sense to the scene.

1   Introduction

Radiant energy transfer between surfaces has been an object of study since the 1960’s  [1] [2]. The coupling of Radiant Energy Transfer theory with realistic image synthesis techniques is very recent though, as of 1984 [3]. The search for a Global Illumination Model that could simulate with reasonable precision, the behavior and distribution of light in a given environment has given birth to the method known as Radiosity.

Radiosity is a Global Illumination Model that aims to simulate the quantitative distribution

of light in a given environment. This is accomplished by dividing its comprising surfaces into

small patches, evaluating on each of those the diffuse light reflection phenomenon [4][5]. Each patch is then compared to every other patch in the environment to identify all possible reflection interactions. For each and every patch where the interaction possibility is identified, a parameter is calculated, namely the form factor [6][7]. This parameter is dependent upon the relative orientation of the patches and their geometries. The form factor between two patches  represents the radiant energy fraction that leaves the source patch and reaches the other patch. Once calculated for all environment, those form factors are used to solve the matrix simultaneous equations that model the diffuse optical environment. The solution is the environment stability condition - in the end nothing more than the energy conservation principle applied to the given environment [9]. Once the stability condition is obtained, the amount of light on each patch is interpolated across the original surface, allowing the final image rendering.

However, considerable time is needed to solve the matrix simultaneous equations, but once the radiant energy for each patch is calculated (from now on referred to as radiosity), in some processors the image can be rendered on real time.  And because the technique is not viewpoint dependent (as long as only diffuse light phenomena is accounted for and the objects present in the scene are not moving), applications such as virtual reality, illumination and architecture projects can potentially produce real time animations [8].

Nothing prevents other phenomena other than diffuse reflection to be modeled using the Radiosity Method (specular reflection, for instance). However, the application of the energy conservation principle could easily became a hard task to accomplish, not to speak of processor increase in demand. To reduce these difficulties, a number of hybrid techiques have been proposed and developed [9][10][11][12][13]. On the other hand, most of the objects we usually deal with are diffuse reflections predominant. The diffuse reflection phenomena can be associated with the surface opacity. Surfaces such as wood, fabric and papers are diffuse predominant: it means that incoming light is reflected evenly on random directions. For an observer it looks that those kind of surfaces have a constant light intensity, not mattering the viewpoint. To simulate that behavior in Computer Graphics, the observer position does not matter - all it counts is the angle between light source and surface.

Several add ons to the basic Radiosity Method were proposed during the last few years, from which we could highlight the Progressive Refinement Radiosity [14][15], Adaptative Subdivision [16], Hierarchical Radiosity [17][18], Stochastic Radiosity Techniques [19], Accelerated Convergence [20] and Discontinuity Analysis [21].

One of the biggest Radiosity limitations is the difficulty found when trying to represent non-regular surfaces (rough). Since almost every surface we deal with on everyday basis has somehow this characteristic, this could be a major drawback for realistic rendering using this technique (the basic method allows for environment discretization on meshings composed of plane and convex patches only).

On the other hand, local illumination models, such as found in [22], can simulate rough surfaces with high degree of realism. This work attempts to increase the realism perception of radiosity rendered scenes, through the additional use of a local illumination model (procedural texturing techniques).

2   Procedural Methods

In order to achieve the proposed hybrid solution (and ultimately more realistic scenes), one possible alternative would be to deploy texturing techniques as the local illumination model, and later merge it to the radiosity results.

Among the texturing techniques, procedural methods would be a very good choice if one wants (or needs) to avoid all the trouble of transforming 2D texture map coordinates to 3D space points (on the object surface) [22].

Whenever the intensities of a given texture pattern can be expressed as a function of 3D space coordinates, the name solid textures apply. Usually solid textures imply the use of procedural methods to describe then, because it is virtually impossible to store texture values for each and every point in a given 3D region. This does not exclude the possibility of using procedural methods to evaluate texture values on bi-dimensional surfaces.

One clear advantage of solid textures is the possibility of applying the same texture pattern to interior surfaces of an object. For instance, a brick rendered using a solid texture function would not produce the aliasing artifacts a texture mapped brick would, in the case of a three-dimensional cut view (sectioning the brick).

Wood and Marble are very good examples of textures that can be reproduced fairly easy (and realistic) using procedural methods. Harmonic functions (senoidal curves), adjusted and modified to avoid obvious pattern repetition would accomplish part of the task, while pseudo random variation, namely a Noise function [22], would wrap it up.

The noise function must produce apparently random values,  actually a repetitive function of a certain number of inputs. This is preferable, because a true random function would produce different results if the viewer position was altered (a very unpleasant effect).

An ideal Noise function must have the following properties [22]:

- be a pseudo random function, repetitive of its input parameters;

- produce results inside a known range, typically between -1 and 1;

- be a limited bandwidth function, with a maximum frequency of 1;

- do not exhibit obvious or regular patterns (the function must be periodic, but its period should be long enough to avoid detectable repetition);

- be a stationary function (translationally invariant);

- be an isotropic function (rotationally invariant);

3   Procedural Functions used in this work

Procedural methods can produce a large number of different results with few changes on the function parameter values.

The most famous procedural function is definitely the Noise function, by Ken Perlin [22]. Some of its variations are described in the following section.

3.1   Lattice Noise

Lattice Noise begins with one or more pseudo random numbers assigned to each point on the texture map space whose coordinates are integers. This set of points is called the Integer Lattice.

High frequencies filtering is accomplished through a smooth pseudo random value interpolation, generally done using Catmull Splines [22] (higher frequencies are to be avoided, because of aliasing artifacts generation).

Refer to [22] for a Lattice Noise function implementation.

3.2   Gradient Noise

It is a Lattice Noise variation, where instead of generating the pseudo random function through the integer lattice, the integer lattice pseudo random gradients are used. The stochastic function is generated using those gradients (calculated around each integer lattice).

The actual procedural functions used to produce this work results were based upon a basic gradient noise function. (the generated procedural functions, using gradient noise, produce more limited bandwidth results).

4   Combining Radiosity and Procedural Methods

4.1   Marble Function

This function,  proposed in [22], is here implemented with four 4 (four) octaves for noise generation (and some variations, like using gradient noise and different parameters).

The pursued effect is the reproduction of a certain marble surface, through spectral synthesis.

Significantly different results are produced with only one parameter variation, as shown in figures 1 and 2 , and is true to all procedural functions implemented in this work.

Its source code is reproduced bellow:

Spectra ProcTextToObject::blue_marble(Point3 PP)

{

float marble,i,f;

double x,y,z;

marble = 0;

f = 1;

for (i = 0; i < NNOISE; i += 1) {

x = PP.GetX() * f *50*sliderFactor;

y = PP.GetY() * f *50*sliderFactor;

z = PP.GetZ() * f *50*sliderFactor;

marble += gnoise(x,y,z)/f;

f *= 2.17;

}

return marble_color(marble*25);

}

Notice that the parameter sliderFactor (which alters the argument passed to gnoise), results in reconstructed functions totally distinct). It is one of those parameters that could be part of an user interface - it would allow the user to produce a wide range of results with a simple change on the parameter.

To combine the Radiosity results with the texturing effect produced by the procedural function, the code bellow was used, calculated for each point on the object surface.

BlendColor= Blend(MapColor,pelem->GetExitance(),0.9);

(where MapColor receives the color calculated by the procedural function and pelem->GetExitance() returns the radiosity result)

The color calculated by the Radiosity technique has a weight of 90% (ninety percent), while the procedural function contributes with the remaining 10% (ten percent). These are empirical values, and maybe one could find a little odd that the procedural function has so small contribution, but it is worth noticing that the stochastic function perturbs the radiosity result itself (which makes the procedural method contribution more significant than what it seems at first).

4.2   Wavy Function

This function is also based on spectral synthesis, but this time using 3 (three) octaves for noise generation.

To combine Radiosity with the texturing method, the code bellow is used:

BlendColor= Blend(MapColor,pelem->GetExitance(),0.2);

In this case, the procedural method contribution is 80% (eighty percent), an empirical value that makes the local illumination method predominant (more even when one takes into account that the perturbation is based on the radiosity result). What happens is that the color channels easily reach the maximum possible value if the surface receives more than a certain amount of radiant energy (i.e, the surface is very illuminated or too bright).

It  is also possible to interactively alter certain parameters and produce very distinct results. Notice the number 100 present on the code bellow:

float ProcTextToObject::wavy(Point3 PP)

{

float turb = 0, a = 1, i;

double x,y,z;

x = PP.GetX()*100*sliderFactor;

y = PP.GetY()*100*sliderFactor;

z = PP.GetZ()*100*sliderFactor;

for (i = 0;  i < 3;  i += 1)

{

turb += a * gnoise(x,y,z);

x = PP.GetX() * 2*100*sliderFactor;

y = PP.GetY() * 2*100*sliderFactor;

z = PP.GetZ() * 2*100*sliderFactor;

a /= 2;

}

return turb*10;

}

That is one of those "magic" numbers, typical of procedural texturing, and produced empirically during the present work. This characteristic is related to the fact that scale variation on stochastic functions, such as noise, produce very different functions (the effects generated by the function are amplified or not, based on scale variation).

4.3   Borg Metal Function

Borg Metal is a spectral synthesis procedure, using 6 (six) octaves for noise generation (figure 8)

The sliderFactor parameter in this case also provides a wide range of different results.

Weight between Radiosity and Procedural results is the same as in the function Wavy.

Implementation code is shown bellow:

float ProcTextToObject::borg_metal(Point3 PP)

{

float size = 1,mag = 0,i,teste;

double x,y,z;

x = PP.GetX()*100*sliderFactor;

y = PP.GetY()*100*sliderFactor;

z = PP.GetZ()*100*sliderFactor;

for (i = 0;  i < 6;  i += 1)

{

x = PP.GetX() * size*100*sliderFactor;

y = PP.GetY() * size*100*sliderFactor;

z = PP.GetZ() * size*100*sliderFactor;

teste = gnoise(x,y,z);

mag += teste/size;

size *=2;

}

return mag*10;

}

4.4   Multi-Fractal Function

Spectral synthesis using 4 (four) octaves, with parameters usually present in fractals generation.

To combine the radiosity value with the one returned by the procedural function, the code bellow was used:

BlendColor=Blend(MapColor,pelem->GetExitance(),0.8);

In this case the Radiosity contribution is very significant (80%). It produces a very smooth result, where the texturing method contribution is hardly noticed. See figure 3

Source code for this procedural function:

Spectra ProcTextToObject::multif_marble(Point3 PP)

{

float octaves = 4.0,    /* # of frequencies to add in       */

lacunarity = 2.0, /* frequency displacement           */

offset = 0.2;             /* multifractal offset              */

float   i,          /* loop control var.        */

fsum = 1,   /* sum of fBm               */

freq = 1;   /* frequencies              */

double x,y,z;

x = PP.GetX();

y = PP.GetY();

z = PP.GetZ();

freq *= pow(lacunarity, octaves);

/* Accumulate multifractal fBm. */

for(i = 0; i < octaves; i += 1.0)

{

fsum *= offset * gnoise(x,y,z) * freq;

x = PP.GetX() * lacunarity;

y = PP.GetY() * lacunarity;

z = PP.GetZ() * lacunarity;

}

return marble_color(fsum*20*sliderFactor);

}

5   Result Analysis

All the experiments were made using an Intel Compatible computer, with IDE interface, 64 Mb RAM EDO, 4M DRAM video adapter (chipset Virge S3, with no hardware device accelerator attached), CYRIX processor 133 MHZ  (P166+), and running WINDOWS 95.

A brief description of the generated images is presented bellow:

Figures 1 and 2:

The distinct results are due solely to the different perturbation factors passed as arguments to the Marble procedural function. In figure 1 the detail richness produced by the procedural function is worth noticing.

Figure 3:

A very well behaved function, Multi-Fractal lends a very subtle pattern to the rendered object. This is due both to the low discretization level of the model, and to the function implementation (in procedural texturing, scale factors alter very significantly the function behavior -  the effects on this one are not amplified on purpose).

Figures 4 and 5:

Two images produced by the same procedural method, in two distinct objects. The huge difference between the samples is due mostly to the discretization level applied to each.

Based on figure 4, one could question the name Marble associated to the procedural method. The color bleeding,  characteristic of marble, is correctly reproduced by the function. The same cannot be said about the specular reflections, but that is a Radiosity limitation. A Hybrid solution would be welcome in this case, to complete the aimed effect [9][10].

On figure 5 the procedural function has more space for development, as it is applied to a more discretized object (more elements and patches than the object rendered in figure 4). The result is a clear example of how proper sampling can affect experiments.

Figure 6 and 7:

Again very distinct results, caused solely by the different perturbation factors used.

Figure 8:

The color channels are “blown” on some areas, meaning the maximum possible values were reached (black areas on the figure). The Borg function could be worked upon to minimize these effects, but the example has been included as it is to show the potential drawbacks of the procedural technique.

Figure 1: Function. Marble - perturbation 1

(generated in  20 steps, 15'' elapsed time - 10208 patches, 10696 elements)

Figure 2:  Function Marble - perturbation 10

(generated in 21 steps, 17'' elapsed time - 10208 patches, 10696 elements)

Figure 3: Function Multi-Fractal - perturbation 9

(generated in 170 steps, elapsed time 1 15'' - 2187 patches, 2708 elements)

Figure 4: Function Marble - perturbation 1

(generated in  250 steps, elapsed time 1' 12'' - 2187 patches, 2708 elements)

Figure 5:  Function Marble - perturbation 1

(generated in 81 steps, elapsed time 1' 58'' - 20178 patches, 20666 elements)

Figure 6:  Function. Wavy - perturbation 1

(generated in 22 steps, elapsed time 33'' - 20178 patches, 20666 elements)

Figure 7:  Function Wavy - perturbation 10

(generated in 22 steps, elapsed time 33'' - 20178 patches, 20666 elements)

Figure 8:  Function Borg Metal - perturbation 1

(generated in 5 steps, 10'' elapsed time - 10208 patches, 10696 elements)

6   Conclusion and Future Work

Combining the Radiosity technique with procedural texturing methods has proven to be an empirical task, where adjusting parameters values and discovering which weight should be applied to each technique was paramount to achieve presentable results.

Some unexpected aliasing artifacts appeared also on the experimental phase. These artifacts have proven to be useful for the analysis of  the advantages and drawbacks of each implemented technique.

As to future work, experimenting combinations of the Radiosity technique with HyperTextures [23] would certainly convey a new dimension to the variety of environments that could be realistic rendered.

Volume, gases and liquids animation, such as proposed in [22], could also be incorporated to the hybrid solution.

Reaction-Diffusion techniques, like in [24] [25], could also be incorporated, but with significant changes on the solid model used in this work.

Considerable improvements could also be made focusing solely on the user interfaces. A carefully studied user interface could take advantage of the enormous number of different results that can be produced with few parameters variation (that is, the user having control of these parameters interactively - notice that this could prove to be a difficult task for certain functions and parameters).

Another possible improvement would be to combine the procedural texturing results with texture mapping methods. If the output of one technique is used as input to the other, complex scenes can be rendered.

Acknowledgments

This research was partially supported by a research grant from the CNPq, a Brazilian Government Research Council. The authors would like to thank Ian Ashdown for the invaluable helpful discussions.

REFERENCES

- Dunkle, R.V.'' Radiant Interchange in an Enclosure with Specular Surfaces and Enclosures with Window or       Diathermanous Walls'', H.A. Johnson ed., Heat Transfer, Thermodynamics and Education (Boelter Anniversary Volume), McGraw Hill, New York, 1964.

- Eckert, E.R.G., E.M. Sparrow,'' Radiative Heat Exchange Between Surfaces with Specular Reflection'',     International Journal of Heat and Mass Transfer,  vol. 3,  1961, pp. 42-54.

- Goral, Cindy M., Kenneth E. Torrance, Donald P. Greenberg, Bennett Battaile,'' Modelling the Interaction of Light Between Diffuse Surfaces'', Computer Graphics (SIGGRAPH '84 Proceedings), vol. 18,  no. 3, July 1984, pp. 212-22.

- Cohen, Michael, “A Radiosity Method for the Realistic Image Synthesis of Complex Diffuse Environments'',      Master's Thesis, Program of Computer Graphics, Cornell Univ.,  Aug. 1985.

- Cohen, Michael F., ''Radiosity'', David E. Rogers ed.,     Ray A. Earnshaw ed., State of the Art in Computer Graphics: Visualization and Modeling, Springer-Verlag, New York, 1991, pp59-90.

- Schroder, Peter, Pat Hanrahan, ''On the Form Factor Between Two Polygons'', Computer Graphics Proceedings, Annual Conference Series, 1993, pp. 163-164, 1993.

- Shao, Min-Zhi,  Qun-Sheng Peng, You-Dong Liang, '' Form Factors for General Environments'', Proceedings of Eurographics '88,D.A. Duce ed., P. Jancene ed., Elsevier Science Publishers,  pp. 489-510, 1988.

- Baum, Daniel R., John R. Wallace, Michael F. Cohen,      Donald P. Greenberg, ''The Back-Buffer Algorithm: An Extension of the Radiosity Method to Dynamic Environments'', The Visual Computer,  vol. 2, no. 5, Sept. 1986, pp. 298-306.

- Chen, Hong, En-Hua Wu, ''An Adapted Solution of Progressive Radiosity and Ray-Tracing Methods for Non-diffuse Environments'', T.S. Chua ed., Tosiyasu L. Kunii ed., CG International '90: Computer Graphics Around the      World, Springer-Verlag, Tokyo, 1990, pp. 477-90.

- Lange, Brigitta, '' The Simulation of Radiant Light Transfer with Stochastic Ray-Tracing'', Photorealistic Rendering in Computer Graphics (Proceedings of the Second Eurographics Workshop on  Rendering), Springer-Verlag,  New York, 1994, pp. 30-44.

- Paulin, M., J-P. Jessel, ''Adaptative Mesh Generation for Progressive Radiosity: A Ray-tracing Based Algorithm'',      Computer Graphics Forum (Eurographics '94       Proceedings), vol. 13, no. 3, 1994.

- Nigri, Helio,  Dreux, Marcelo, Melo, Rubens, "Um algoritmo híbrido de Radiosidade e Ray-Tracing, para geração de aimgens realísticas", SibGrapi92,  pp . 11-18, 1992.

- Nigri, Helio,  "Uma solução híbrida para síntese de imagens realistas combinando o método da Radiosidade com o Ray-Tracing", Dissertação de Mestrado - Depto Informática PUC-RIO, 1991.

- Airey, John M., M. Ouh-young, ''Two Adaptive Techniques Let Progressive Radiosity Outperform the Traditional Radiosity Algorithm'', Technical Report TR89-020,       Univ. of North Carolina Department of Computer        Science, 1989.

- Chalmers, Alan G., Derek J. Paddon, ''Parallel Processing of Progressive Refinement Radiosity Methods'',       Photorealistic Rendering in Computer Graphics       (Proceedings of the Second Eurographics Workshop       on  Rendering), Springer-Verlag,  New York, 1994, pp. 149-159.

- Gatenby, Neil, W.T. Hewitt, ''Optimizing Discontinuity Meshing Radiosity'', Fifth Eurographics Workshop on Rendering, Darmstadt, Germany,  June 1994,  pp. 249-258.

- Bhate, N., ''Application of Rapid Hierarchical Radiosity to       Participating Media'', Proceedings of ATARV-93: Advanced Techniques in Animation, Rendering, and Visualization,       Bilkent University, Ankara, Turkey,  July 1993,  pp. 43-53.

- Carter, Michael B.,John L. Gustafson, '' An Improved Hierarchical Radiosity Method'', Tech. Report IS-J 4881, Ames Laboratory, Jan 1993.

- Cook, Robert L., '' Stochastic Sampling in Computer Graphics'', ACM Transactions on Graphics, vol. 5,  no. 1, Jan. 1986,  pp. 51-72.

- Littlefair, P.J., ''Interreflection Calculations: Improving Convergence”, Lighting Research and Technology,  vol. 23, no. 4, pp. 175-177, 1991.

- Heckbert, Paul, ''Discontinuity Meshing for Radiosity'',       Third Eurographics Workshop on Rendering,  Bristol,       UK,   May 1992,  pp. 203-226.

- Ebert, David S., F.  Kenton Musgrave, Darwyn        Peachey, Ken Perlin, Steven Worley, ''Texturing and Modeling: a procedural approach'' , Academic Press  ed., London, 1994.

- Perlin, Ken, "Hypertexture",  SigGraph 91, 1991.

- Turk, Greg, "Generating Textures on Arbitrary Surfaces Using Reaction-Diffusion", SigGraph 91, 1991.

- Witkin, Andrew, Kass, Michael, "Reaction-Diffusion Textures", SigGraph 91, 1991.