You must following these simple instruction in order for the VR Gun system to initialize and boot along with your PC correctly.  Any questions or comments relating to this procedure should be directed to WWW.CWONLINE.COM .

- Shut down your PC and plug the VR Gun mouse and keyboard cables into the appropriate ports.

- Ensure that the tracking activate push button is up (depressed).

- Turn on your PC and allow it to boot up properly.

- Upon boot up some keyboard keys may be repeating.  Stop this by pushing the slide switch forward and then back to the middle position.  The middle position is the slide switch’s neutral position.

- Press and hold the red button located on the VR Gun enclosure for approx. 2 seconds.  The right LED should start blinking and the left LED should be off.  Push the black button briefly.  Do not hold it down.  The right LED should blink rapidly and the return to a slow blink.  Press and hold the red button again for approx. 2 seconds.  Your VRGun system is now synchronized.

- Activate the tracking by pressing the ‘tracking activation push button’.  This should activate the gun and it should now be tracking.  You can see this by watching the response of the mouse cursor in correlation with your VR Gun movements.  After this initialization of the VR Gun system the VR Gun can be awaken from sleep mode by either pressing the tracking activation push button or by pulling the trigger.

- When the system is tracking the right LED will be blinking rapidly and the left LED will be constantly lit.

NOTICE: IF THE VRGUN AT ANYTIME DRIFTS OR ACTS ERRACTICALLY PLEASE REPEAT THE ABOVE INITIALIZATION PROCESS.