Creative Voice File (VOC) Format

�������������������������������������������������������������������������������

The Creative Voice file is organized into two main blocks, the Header

Block and Data Block. The Header Block contains identifier, version number

and pointer to the start of the Data Block.

The Data Block is divided into various sub-blocks of various types such as

Voice Data, Silence, ASCII text, Repeat and End Repeat, and Terminator.

The CT-VOICE driver only processes the Data Block. It is important that

you pass the address of the Data Block and not the Voice File when calling

the driver to perform voice output.

Header Block

�������������������������������������������������������������������������������

�����������������������������������������������������������������������������Ŀ

� Offset  � Description                                                       �

�����������������������������������������������������������������������������Ĵ

� 00h-13h � File type description.                                            �

�         � The following message is stored here: "Creative Voice File",1Ah   �

�         �                                                                   �

� 14h-15h � Offset of Data Block from the start of the voice file             �

�         � This word points to the Data Block. It helps the application      �

�         � programs to locate the Data Block in case the size of header      �

�         � block is changed.  For this version, the value here is always     �

�         � 1A hex.                                                           �

�         �                                                                   �

� 16h-17h � Voice file format version number                                  �

�         � This version number allows your program to identify different     �

�         � organization formats of voice data file in case of future         �

�         � enhancement. LSB and MSB are the minor and major version number   �

�         � respectively.  Current version is 1.10.                           �

�         �                                                                   �

� 18h-19h � Voice file identification code                                    �

�         � This code allows your program to check that this file is a VOC    �

�         � file.                                                             �

�         �                                                                   �

�         � Its content is the complement of the file format version number   �

�         � then plus 1234 hex.  For current version of 1.10, its complement  �

�         � (010Ah) +1234h = 1129h                                            �

�������������������������������������������������������������������������������

Data Block

�������������������������������������������������������������������������������

The Data Block is sub-divided into multiple sub-blocks of data.

a)  The first byte of each sub-block is called the Block Type (BLKTYPE).

It indicates the type of data contained in the sub-block.

b)  The next three bytes is the 24-bit Block Length (BLKLEN). It is the

number of bytes in the sub-block _excluding_ the BLKTYPE and BLKLEN

fields. Only the Terminator sub-block does not have a BLKLEN field.

c)  Depending on BLKTYPE, the rest of the data in the sub-block may be

voice attributes (such as sampling rate and packing), voice data or

other information (such as Marker and ASCII text).

Your program needs not interpret all the BLKTYPE. For those BLKTYPEs it

does not recognizem it can move to next sub-block based on the BLKLEN

easily.  In most of the applications, your program can simply pass the

address of the Data Block to the CT-VOICE driver which will interpret the

Voice file and perform the output accordingly.

Type 0 : Terminator

�������������������������������������������������������������������������������

����Ŀ

� 00 �

������

This 1-byte sub-block terminates the entire Data Block. It indicates that

there is no other sub-block after it. CT-VOICE driver terminates voice

output when this BLKTYPE is encountered.

This BLKTYPE must be the last block in the voice file.

Type 1 : Voice Data

�������������������������������������������������������������������������������

��������������������������������������Ŀ

� 01 � BLKLEN � SR � PACK � VOICE DATA �

����������������������������������������

This BLKTYPE indicates that this sub-block is a new set of voice data. It

may use different sampling rate or packing method.

The SR field(1 byte) is the Sampling Rate. It is defined as

SR = 256 - 1000000/sampling rate

For instance, if the sampling rate is 10,000 Hz, SR is set to 156.

The PACK field (1 byte) is the Packing method used for this sub-block of

voice data. It is defined as:

�������������������������������ͻ

� PACK � Description            �

�������������������������������͹

�   0  � 8-bit raw or unpacked  �

�   1  � 4-bit packed           �

�   2  � 2.6-bit packed         �

�   3  � 2-bit packed           �

�������������������������������ͼ

The voice data length is calculated as follows:

voice data length = BLKLEN - 2

Type 2 : Voice Continuation

�������������������������������������������������������������������������������

����������������----------Ŀ

� 02 � BLKLEN � VOICE DATA �

����������������----------��

This sub-block is a continuation of the voice data from the last Voice

Data sub-block.

The length of the voice data is BLKLEN.

Type 3 : Silence

�������������������������������������������������������������������������������

��������������������������Ŀ

� 03 � BLKLEN � PERIOD� SR �

����������������������������

This sub-block defines silence period in the voice data.

The length of the silence period (period, 2 bytes) is stored after the

BLKLEN field in units of sampling cycle. Its value is one less than the

silence period. The sampling rate (SR) is defined as above.

The BLKLEN for this sub-block is always 3.

Type 4 : Marker

�������������������������������������������������������������������������������

���������������������Ŀ

� 04 � BLKLEN � MARKER�

�����������������������

This is a special sub-block that specifies a Marker (MARKER, 2 bytes) in

the voice data.

During voice output, the CT-VOICE driver updates the status word

ct_voice_status with this marker value. Your program may check for the

desired MARKER value to perform synchronization with the voice output

process.

The Marker value must be within the range of 1 to 0FFFE hex. Note that 0

and FFFF hex are reserved by the CT-VOICE driver. Also refer to the

following section Essentials of using the CT-VOICE driver.

The BLKLEN field is always set to 2.

Type 5 : ASCII Text

�������������������������������������������������������������������������������

���������������������������������Ŀ

� 05 � BLKLEN � ASCII DATA � NULL �

�����������������������������������

This sub-block specifies a null-terminated ASCII string in the voice data

file.

Most programs choose to ignore this BLKTYPE. It is defined for a program

which requires ASCII information on the voice such as name, type or

remarks on the voice data.

Type 6 : Repeat Loop

�������������������������������������������������������������������������������

���������������������Ŀ

� 06 � BLKLEN � COUNT �

�����������������������

This sub-block specifies the beginning of a repeat loop. The data

sub-blocks between this block and the next End-Repeat Loop sub-block is

repeated by the CT-VOICE driver for (COUNT+1) times.

The COUNT (2 bytes) field takes a value from 1 to FFFE hex as a loop

count. If COUNT is FFFF hex, then it is to be an endless loop. In this

case, refer to the following section Reference fir SBC Voice functions on

how to break out of the endless loop.

The BLKLEN field is always 2.

Type 7 : End Repeat Loop

�������������������������������������������������������������������������������

�������������Ŀ

� 07 � BLKLEN �

���������������

This sub-block specifies the end of a Repeat Loop. Refer to Block Type 6

for details.

The BLKLEN field is always 0.