GENERIC UART MANUAL

DISCLAIMER

All source code and documents provided here can be freely copied and distributed. The design has been simulated, implemented and tested, however the author takes no responsibility whatsoever for any damages and/or losses arising as a result of the use of these files. Please email  me if you have any comments or find any errors.

uart features	3

port definitions	4

Host Interface Ports	4

Serial Interface Ports	4

host interface	4

baud rate generation	5

transmitter	9

receiver	10

register descriptions	12

Addresses	12

Control Register	12

Baud Rate Select Register #0	13

Baud Rate Select Register #1	13

Status Register	13

Interrupt Mask Register	14

operating modes	14

Normal	14

Autoecho	14

Internal Loopback	14

implementation in target device	14

Requirements of target device	14

modifying the design	15

Wait states for host interface	15

Multiple channels	15

Providing a three-state ACK	15

State machine encoding	15

appendix a –vhdl source files	16

Coding Style	16

appendix B –vhdl testbench files	17

## change history

| Manual version | Change Description | Applies to source file versions: |
| --- | --- | --- |
| 0 | Initial draft. | all files – ver A. |

## uart features

- Synchronous host interface.

- Host clock used for baud rate generation - separate crystal not required.

- Uses a maximum of 2 global clock buffers in target device.

- 8 bits per character.

- Odd, even or no parity modes.

- Normal, autoecho or internal loopback modes.

- Maskable interrupt on character received and transmitter empty.

## port definitions

### Host Interface Ports

| Port name | DIRECTION | DESCRIPTION |
| --- | --- | --- |
| hclk | IN | Host clock, all host interface timings synchronous to this clock. Also used for baud clock generation. |
| RESETH | IN | Active high asynchronous reset |
| Addr(2:0) | IN | Address inputs for register selection. |
| Data(7:0) | IN/OUT | Bidirectional data bus (three state). |
| rw | IN | High for host reads, low for host writes. |
| cs | IN | Active low chip select. |
| iRQ | OUT | Active low interrupt request. |
| ack | OUT | Active low transfer acknowledge. |

### Serial Interface Ports

| Port name | DIRECTION | DESCRIPTION |
| --- | --- | --- |
| TxD | OUT | Transmitted serial data. |
| Rxd | IN | Received serial data. |
| cts | IN | Active low Clear-To-Send. |
| rts | OUT | Active low Ready-To-Send. |

## host interface

The host interface is a simple synchronous interface with the host clock input (HCLK) providing the base for all timings:

Host read cycle

Host write cycle

The host interface samples the CS input with the rising edge of HCLK. The access to the UART’s internal resources starts when the chip select (CS) input is recognised as asserted (low) and the internal state machine moves from the ‘idle’ state to state ‘s1’.

At the second clock edge, the RW and ADDR(2:0) inputs must be valid. The RW input must be high for reads and low for writes. The host must maintain RW and ADDR(2:0) stable until it recognises the ACK output asserted.

When the host recognises the ACK output (asserted low), it must latch the data for read cycles or release the data bus for write cycles.

The number of HCLK periods (‘wait states’) inserted between the edge where CS is recognised low (the start of the access) to the edge where ACK is asserted (end of access) is determined by the state machine and can be modified as required, see the Target Implementation chapter.

## baud rate generation

The transmitter and receiver have a common clock which is controlled by the BAUD RATE SELECT registers, Baud Rate Select #0 (BRSEL0) and Baud Rate Select #1 (BRSEL1), which together form a 16 bit integer ‘N’. BRSEL1 is the most significant byte of the integer, BRSEL0 the least significant byte.

Baud Rate Generator

The values in these registers are used to increment an accumulator that is clocked by HCLK. The most significant bit of the accumulator register is output as a clock (BCLKX16) which is 16 times the actual required baud rate. The following equation gives the baud rate for any value of N programmed into BRSEL1 and BRSEL0:

Baud rate = (N x FreqHCLK  / 16 x 2n)

where:

Baud rate – bits/sec.

FreqHCLK – Frequency of HCLK in Hertz

N – decimal value to program into BRSEL1 & BRSEL0

n – accumulator width in bits

Solving for N:

N = (16 x 2n x BaudRate / FreqHCLK)

The size of the accumulator is controlled by a generic (accum_width) and the user can change this as required. Note that larger width accumulators will require larger values of N and may mean that some higher speed baud rates cannot be generated.

The user may prefer to instantiate a macro from a library provided by the target device manufacturer (such as Xilinx Logiblox) instead of using the generic VHDL code in the baudgen.vhd file if the synthesis tool used cannot infer an accumulator.

Using the host clock to generate the baud rate instead of a standard frequency such as 3.6864MHz means that one less port is required and, more importantly, one less internal global clock buffer. However this does mean that there will be some error in the actual baud rate generated. The following example shows how to calculate the value to program into BRSEL0 and BRSEL1 and the error in the baud rate:

Maximum required Baud rate is 9600 baud, accum_size generic is 16 (i.e. accumulator is 17 bits wide) and host clock frequency is 40MHz:

N = (16 x 217 x 9600 / 40 x 106) = 503.3 rounded to 503

The actual baud rate obtained is:

Baud rate = (503 x 40 x 106 / 16 x 217) = 9593.96 bits/sec.

The baud rate error is:

Error % = ((Actual baud rate  - Required baud rate) / Required baud rate) x 100

error % = ((9593.96  - 9600) / 9600) x 100 = 0.063%

Converting N from decimal to its 16 bit hexadecimal equivalent gives $01F7. Therefore BRSEL1 must be programmed with the value $01 and BRSEL0 is programmed with $F7.

The tables below show the values of N required to produce some common baud rates for various HCLK frequencies, assuming that the accum_size generic is set to 16:

HCLK = 40 MHz

| Required Baud Rate (bit/sec) | N (decimal) | Actual baud rate (bit/sec) | Error % |
| --- | --- | --- | --- |
| 115200 | 6040 | 115203.857 | 0.003 |
| 76800 | 4027 | 76808.929 | 0.012 |
| 57600 | 3020 | 57601.929 | 0.003 |
| 38400 | 2013 | 38394.928 | 0.013 |
| 28800 | 1510 | 28800.964 | 0.003 |
| 19200 | 1007 | 19207.001 | 0.036 |
| 14400 | 755 | 14400.482 | 0.003 |
| 9600 | 503 | 9593.964 | 0.063 |
| 7200 | 377 | 7190.704 | 0.129 |
| 4800 | 252 | 4806.519 | 0.136 |
| 3600 | 189 | 3604.889 | 0.136 |
| 2400 | 126 | 2403.259 | 0.136 |
| 1800 | 94 | 1792.908 | 0.394 |
| 1200 | 63 | 1201.630 | 0.136 |
| 900 | 47 | 896.454 | 0.394 |
| 600 | 31 | 591.278 | 1.454 |

HCLK = 20 MHz

| Required Baud Rate (bit/sec) | N (decimal) | Actual baud rate (bit/sec) | Error % |
| --- | --- | --- | --- |
| 115200 | 12080 | 115203.857 | 0.003 |
| 76800 | 8053 | 76799.393 | 0.001 |
| 57600 | 6040 | 57601.929 | 0.003 |
| 38400 | 4027 | 38404.465 | 0.012 |
| 28800 | 3020 | 28800.964 | 0.003 |
| 19200 | 2013 | 19197.464 | 0.013 |
| 14400 | 1510 | 14400.482 | 0.003 |
| 9600 | 1007 | 9603.500 | 0.036 |
| 7200 | 755 | 7200.241 | 0.003 |
| 4800 | 503 | 4796.982 | 0.063 |
| 3600 | 377 | 3595.352 | 0.129 |
| 2400 | 252 | 2403.259 | 0.136 |
| 1800 | 189 | 1802.444 | 0.136 |
| 1200 | 126 | 1201.630 | 0.136 |
| 900 | 94 | 896.454 | 0.394 |
| 600 | 63 | 600.815 | 0.136 |

HCLK = 2 MHz

| Required Baud Rate (bit/sec) | N (decimal) | Actual baud rate (bit/sec) | Error % |
| --- | --- | --- | --- |
| 115200 | - | - | - |
| 76800 | - | - | - |
| 57600 | 60398 | 57600.021 | 0.000 |
| 38400 | 40265 | 38399.696 | 0.001 |
| 28800 | 30199 | 28800.011 | 0.000 |
| 19200 | 20133 | 19200.325 | 0.002 |
| 14400 | 15099 | 14399.529 | 0.003 |
| 9600 | 10066 | 9599.686 | 0.003 |
| 7200 | 7550 | 7200.241 | 0.003 |
| 4800 | 5033 | 4799.843 | 0.003 |
| 3600 | 3775 | 3600.121 | 0.003 |
| 2400 | 2517 | 2400.398 | 0.017 |
| 1800 | 1887 | 1799.583 | 0.023 |
| 1200 | 1258 | 1199.722 | 0.023 |
| 900 | 944 | 900.269 | 0.030 |
| 600 | 629 | 599.861 | 0.023 |

Note that in the 2MHz example, the higher baud rates are not available as they require a value of N that is greater than 16 bits. When this occurs and the higher baud rates are required, the user must reduce the width of the accumulator (by reducing the generic accum_size), remembering that the width of N must be less than or equal to the width of the accumulator.

## transmitter

The transmitter section consists of a transmitter holding register that is written to via the host interface and a transmitter shift register. The shift register will be loaded whenever there is data in the transmitter holding register (providing it has finished shifting out the previous data and the TXEN bit in the control register is set) and then shifts the data out to the TXD pin using the transmitter clock, TXCLK (whose frequency is one 16th that of the BCLKX16 clock).

The format of a complete frame with parity is shown below:

Frame with parity

The start bit is transmitted first, then the data field starting with the LS data bit. If parity has been enabled in the control register, then a parity bit will be transmitted before the stop bit, otherwise the stop bit will be transmitted directly after the MS data bit.

There are two status bits associated with the transmitter, TXRDY and TXEMPTY, which can be read from the status register.

Whenever the transmitter holding register is empty, the TXRDY (transmitter ready) status bit will be set. When the TXRDY bit is set, it will generate an interrupt (if not masked by the interrupt mask register and not in autoecho mode).

TXEMPTY (transmitter empty) is set whenever both the transmitter holding register and the transmitter shift register are empty.

With the ‘Tx inhibit by CTS’ bit set in the control register, the transmitter will check its CTS input before starting the transmission of a frame. If CTS is negated (logic high), the transmitter will not transmit. To stop the transmitter sending the following frame, the CTS input must be negated before the middle of the current stop bit. The transmitter will finish the frame it is currently sending, but will wait until CTS goes active (logic low) before resuming transmission with the next frame.

In autoecho mode, the transmitter will retransmit the data received on the RXD input. Writes to the transmitter holding register will have no effect. The TXRDY and TXEMPTY status bits will both be read from the status register as inactive (i.e. cleared) and no interrupt will be generated even if the interrupt mask register bit #1 is set. The transmitter doesn’t have to be enabled for autoecho mode.

The transmitter is enabled using the TXEN bit in the control register. When the transmitter is first enabled, an interrupt will be generated (if the corresponding bit in the Interrupt mask register is set and the UART operating mode is not ‘autoecho’) indicating that the transmitter holding register is empty. This interrupt can only be removed by writing a new byte for transmission to the transmitter holding register or by disabling the transmitter using the TXEN bit in the control register. Note that if the TXEN bit is cleared (transmitter disabled) while a transmission is in progress, the transmitter will finish the transmission (i.e. the data in the transmitter shift register will be shifted out) but no new transmissions will occur. Any data in the transmitter holding register will remain pending and when the transmitter is enabled again, the data will be passed to the transmitter shift register and transmitted. Writes to the transmitter holding register while the transmitter is disabled will have no effect and the data in the holding register will not be overwritten.

When the transmitter is idle or disabled or in internal loopback mode, the TXD pin is held at a logic high.

## receiver

The receiver section also has a shift register and a holding register. The incoming serial data is sampled at 16 times the baud rate and a start bit will only be recognised as valid if it stays at a logic low for at least half a bit period. The 4-bit counter which generates the receiver clock enable (s_rxclken) is synchronously reset when a falling edge occurs on the serial data input (RXD), this positions s_rxclken in the ideal centre of the start bit.

The serial data is shifted in from the RXD pin to the receiver shift register. When the shift register is full, the data is loaded into the receiver holding register where it can be read by the host.

When data is loaded into the holding register, the RXRDY (receiver ready) flag is set in the status register which will generate an interrupt if not masked. This flag will be cleared (and the interrupt removed) when the RX holding register is read.

If another frame is received into the receiver shift register before the receiver holding register has been emptied by a read, the new data will be held in the shift register until the holding register is read and the RXFULL (receiver full) flag will be set in the status register. The RXFULL flag will be cleared when the data in the shift register is transferred to the holding register.

If a new start bit is received on the RXD input while the receiver shift and holding register are both full, the overrun error flag will be set in the status register. This flag will be reset when the ‘Reset Status’ command is sent to the UART by writing to the status register (the data written is not important as it is not used).  Note that the data in the shift register will not be overwritten when an overrun occurs - the host can always read the data in the holding register and assume it has been received correctly (apart from parity and frame errors).

Parity will be calculated for the received data only if even or odd parity modes have been selected by the parity mode bits in the control register. If the received data parity is incorrect, the parity error flag will be set in the status register. This flag will be reset when the ‘Reset Status’ command is sent to the UART (by writing to the status register) or when another character is received with no parity error.

If the received stop bit is not set to a logic level 1, a framing error is generated and the framing error flag will be set in the status register. This flag will be reset when the ‘Reset Status’ command is sent to the UART (by writing to the status register) or when another character is received with no framing error.

The parity and frame error flags are associated with the data in the receiver holding register and should be checked before the holding register is read. When the holding register is read, any data pending in the shift register will be written to the holding register and the parity and frame error flags will be updated. The parity and frame error flags are only valid  when the rxrdy flag is set.

The receiver will negate (logic level 1) the RTS output when the receiver holding register is full (and the ‘RX RTS enable’ bit is set in the control register) or when the receiver is disabled, so blocking transmissions from an external transmitter which is using hardware handshaking (generally done by connecting a receiver’s RTS output to a transmitter’s CTS input).

## register descriptions

### Addresses

| ADDRESS | READ/WRITE | REGISTER |
| --- | --- | --- |
| 0 | R/W | Control |
| 1 | R/W | Baud Rate Select 0 |
| 2 | R/W | Baud Rate Select 1 |
| 3 | R | Status |
| 3 | W | Reset RX status flags (parity,frame,overrun) |
| 4 | R/W | Interrupt Mask |
| 5 | R | RX Holding |
| 5 | W | TX Holding |

### Control Register

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 |  | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANNEL MODE 00 – Normal 01 – Internal loopback 10 – Auto echo 11 – Reserved (don’t use) |  | Tx inhibit by CTS 0 – No inhibit 1 – Inhibit | Rx RTS enable 0 – Disabled 1 - Enabled | PARITY MODE 00 - No parity 01 - Even parity 10 - Odd parity 11 – Reserved (don’t use) |  |  | RXEN 0 - Disabled 1 - Enabled | TXEN 0 - Disabled 1 - Enabled |
| STATE AFTER RESET |  |  |  |  |  |  |  |  |
| 00 |  | 0 | 0 | 00 |  | 0 |  | 0 |

All bits are read / write. The control register should not be modified unless both the transmitter and receiver are disabled, otherwise unexpected operation might occur.

### Baud Rate Select Register #0

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N(7) | N(6) | N(5) | N(4) | N(3) | N(2) | N(1) | N(0) |
| STATE AFTER RESET |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

All bits are read / write.

### Baud Rate Select Register #1

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N(15) | N(14) | N(13) | N(12) | N(11) | N(10) | N(9) | N(8) |
| STATE AFTER RESET |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

All bits are read / write.

### Status Register

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Reserved (always 0) | Framing Error 0 - No error 1 – Error | Parity Error 0 - No error 1 - Error | Overrun Error 0 - No error 1 - Error | TXEMPTY 1 – Tx hold register & shift reg empty 0 – Tx hold register or shift reg full | TXRDY 1 - Tx hold register empty 0 - Tx hold register full | RXFULL 0 - Rx shift reg empty 1 - Rx shift & hold reg full | RXRDY 0 - Rx hold reg empty 1 - Rx hold reg full |
| STATE AFTER RESET |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |

All bits are read only. Note that the framing error and parity error bits are only valid while RXRDY is set. The overrun error can be reset by the ‘Reset Status’ command. The ‘Reset Status’ command is executed by writing to the status register address – no data is transferred.

In autoecho mode, the TXRDY and TXEMPTY flags will remain cleared.

### Interrupt Mask Register

| BIT7 | BIT6 | BIT5 | BIT4 | BIT3 | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Reserved (always 0) | 0 Reserved (always 0) | 0 Reserved (always 0) | 0 Reserved (always 0) | 0 Reserved (always 0) | 0 Reserved (always 0) | TXRDY Interrupt Mask 1 - Enabled 0 - Masked | RXRDY Interrupt Mask 1 - Enabled 0 - Masked |
| STATE AFTER RESET |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Bits (1:0) are read/write, the remaining bits cannot be written and always read as ‘0’.

## operating modes

### Normal

In normal mode the UART functions as described in the Transmitter and Receiver chapters.

### Autoecho

In autoecho mode the serial data received on RXD will be retransmitted without modification on the TXD output. The receiver must be enabled, but the transmitter may be disabled. The TXRDY and TXEMPTY flags in the status register will be read as cleared and will not cause an interrupt to be generated, even if the interrupt mask register bit#1 is set. Writing data to the transmitter holding register will have no effect – the data will not be placed in the transmitter holding register and will not be transmitted.

### Internal Loopback

Internal loopback mode is used for testing the UART. Internally, TXD is connected to RXD and RTS is connected CTS. Externally, the TXD and RTS output pins are held at a logic high. Any data received on the RXD pin is ignored as is any signal connected to the CTS input. Apart from these differences the UART functions exactly as for normal mode. Both the transmitter and receiver must be enabled.

## implementation in target device

### Requirements of target device

The UART model was originally written for a Xilinx Spartan device and so uses both set and presets on the flip-flops that are inferred.  The global set/reset routing resource should be used for the reseth input.

The global clock buffers should be used as much as possible, certainly for the host clock input (hclk) and the bclkx16 clock.

The output pins require tristate capability for the bidirectional data bus pins - note that the output enable signal (dataoe in host.vhd) is active low.

## modifying the design

### Wait states for host interface

The number of clock cycles required for a host read or write can be changed by inserting extra states into the state machine in the host interface block (host.vhd).

### Multiple channels

A single serial channel is made up of the baudrate generator (baudgen.vhd), the receiver (receive.vhd) and transmitter (transmit.vhd) and any number of these can be used without modification. The host interface (host.vhd) will however require some modifications, in particular a complete register set may needed for each channel.

### Providing a three-state ACK

Some users may require the ACK output to have three-state capability. This may require an extra state to be inserted in the state machine between the last state and the IDLE state in which ACK can be driven high before being released.

### State machine encoding

The state machines (in receive.vhd and host.vhd) are encoded using an explicity binary coding. Other coding styles may be more appropriate for different target technologies. For an explanation of state machine encoding with VHDL, visit my website and follow the VHDL information links.

## appendix a –vhdl source files

The UART is built up from 5 different source files. The top level file (uart.vhd) instantiates the host interface (host.vhd), receiver (receive.vhd), transmitter (transmit.vhd) and baud clock generator (baudgen.vhd) design entities as components. The source files are listed in appendix A of this document and are available for download from the Geocities web site http://www.geocities.com/SiliconValley/Screen/2257/ .

| File | Description |
| --- | --- |
| uart.vhd | top level design entity, instantiates the other design entities. |
| baudgen.vhd | Baudrate generation |
| host.vhd | host interface control; registers |
| transmit.vhd | transmitter |
| receive.vhd | receiver |

### Coding Style

All VHDL signals are prefixed with ‘s_’ . For example:

SIGNAL s_signalname : std_logic;

All VHDL constants (apart from explicit state machine coding constants) are prefixed with ‘c_’ . For example:

CONSTANT c_constantname : std_logic;

All VHDL variables are prefixed with ‘v_’ . For example:

VARIABLE v_variablename : std_logic;

All VHDL aliases are prefixed with ‘a_’ . For example:

ALIAS a_aliasname : std_logic_vector(1 DOWNTO 0) IS register(7 DOWNTO 6);

## appendix B –vhdl testbench files

The testbench files are available as a single zipped file from my website.

| Testbench | Description | Log files output |
| --- | --- | --- |
| tb1_uart.vhd | Verifies the state (i.e. logic level) of all outputs after a reset. Reads all registers to ensure that they are correctly reset & then performs a read/write test on all registers. The external host is emulated by the host_access procedure. | tb1_uart.log |
| tb2_uart.vhd | Verifies receiver section using a procedure (transmit) which emulates an external transmitter. | tb2_uart.log |
| tb3_uart.vhd | Verifies transmitter section using a process (receive) which emulates an external receiver. | tb3_uart.log |
| tb4_uart.vhd | Verifies autoecho mode. | tb4_uart.log |
| tb5_uart.vhd | Verifies internal loopback mode. | tb5_uart.log |
| tb6_uart.vhd | Connects TXD output to RXD input & RTS output to CTS input to test external loopback. The UART transmits & performs handshake to itself. | tb6_uart.log |

Note that all the testbenches use the sim_util library which is available for download.