# Infrared DCC Control of a Model Railroad

This paper describes how I control my DCC model railroad with an inexpensive infrared remote control.  This builds on the work of Mike Brandt that he describes on the web at

http://web.syr.edu/~mobrandt/dcc-mb/dccmbhom.htm

Mike describes how to control a DCC layout with an old PC XT.  Here, I will describe the infrared remote control that I use to control the PC, and hence the DCC layout.

Overview

I use an RCA SystemLink 3 Universal Remote Control that I bought at Wal-Mart for $10.  I built a simple infrared receiver with $10 in parts from Radio Shack, and I fed that into the PC’s serial port.  I wrote software that reads the infrared codes through the serial port, makes calls to Mike’s driver to control the trains, and updates the PC’s display.  Here is a sample of the screen display:

| Loco Number | Description | Direction | Speed |
| --- | --- | --- | --- |
| 1905 | NYC Mogul | Forward | 6 |
| 2375 | UP GP-9 | Reverse | 3 |
| 2378 | UP GP-9 | Forward | 4 |

This shows three locomotives (please don’t ask why I run Union Pacific diesels with New York Central steam), each identified by the locomotive number painted on the side of the loco.  I change the speed and direction of each loco by pressing buttons on the remote.  At any given time, exactly one locomotive is selected, and that row is painted inverse video.  I change the selected locomotive by changing the “channel”, and I control the speed by changing the “volume”.

Here is how I map the buttons on the remote and the keyboard:

| Button | Keyboard Key | Meaning |
| --- | --- | --- |
| Channel Up | Up arrow | Select the previous locomotive in the display |
| Channel Down | Down arrow | Select the next lower locomotive in the display |
| Previous Channel | P | Select the previously selected locomotive |
| Digits 0-9 | 0-9 | Select the locomotive with that number* |
| Volume Up | Right arrow | Increase the DCC speed by one |
| Volume Down | Left arrow | Decrease the DCC speed by one |
| Stop or Pause | S | Stop this engine |
| On/Off | Esc | Stop all engines |
| Fast Forward | F | Set direction to Forward |
| Rewind | R or B | Set direction to Reverse |
| Play | F5 | Toggle Function 0 on or off (usually headlight) |
| Enter | F1 | Toggle Function 1 on or off (usually taillight) |
| Mute | F2 | Toggle Function 2 on or off (sound?) |
| TV/VCR | F3 | Toggle Function 3 on or off |
| Record | F4 | Toggle Function 4 on or off |

* - You can select a locomotive by keying in the minimal unique locomotive number.  For example, in the above display, if you pressed “1” then you would select the Mogul.  If you pressed “2”, the “2”s on the GP-9s would both highlight, then when you pressed “3”, the “23”s would highlight, then when you pressed “7”, the “237”s would highlight, then when you pressed “5” or “8”, that locomotive would be selected.  This way you get instant feedback on the buttons being received, and you can select locomotives by the number painted on the side of them, and you don’t need to remember DCC addresses.

Notice that the buttons map pretty well until you get to Functions.  My software can handle 25 locomotives.  The locomotive description, direction, and speed all fit into 40 chars, so the display can be put in 40 column mode, which makes the font twice as big, which means that I can read it from about 10 feet away.  I get more than 20 feet range on the remote.  I could get more by adding more receivers.

To exit the program, press any key on the keyboard.

Why would I want to control a model railroad with a TV remote control?

This solution is very inexpensive, and it allows me to walk around with the trains, and it allows me to use a very well usability engineered handheld remote, and I don’t have to remember DCC addresses, and I don’t have to plug and unplug handheld throttles.

Some disadvantages are:  the throttle does not have a knob, the buttons do not match exactly (rewind instead of reverse?), you have to build some electronics yourself, and you can have only one operator.  The software to date only allows one locomotive to be selected at a time, so two operators using two remotes would be controlling the same train.  This, along with the limitation that my software does not support consists, could be overcome with more design work, but I really need to finish my layout and run some trains!  I’m not likely to fix these problems soon.

How does the software work?

I describe each locomotive in a file called “LOCOS.TXT”.  It looks like this:

DCC Address	DCC Mode	Loco Number	Loco Description

3		2		1906		NYC Mogul

41		2		2375		UP GP-9

55		2		2378		UP GP-9

This file describes each locomotive, including its DCC address, DCC speed mode, number, and description.  This file is read by the software, used to paint the screen, and used to interpret the button pushes on the remote.  If you know the “C” programming language, the program in run.c is quite straightforward.

How does the infrared work?

This was the tricky part.  When you push a button the remote, it sends out a stream of “packets”, each one of which has 29 pulses in it.  (This is a “universal” replacement remote control.  It’s output probably varies as you set up different brands of TVs, VCRs, etc.  I’m assuming that you put the batteries in and just start pushing the buttons, without setting it up to any particular brand of TV.  Mine comes up wanting to talk to an RCA XL100 TV.  Don’t touch the VCR, Cable, TV, Code Search, or any other “setup” buttons, since they will change the codes that the remote sends.)  Each pulse is either 1500 microseconds or 700 microseconds depending upon whether it’s a zero or a one.

If you fed that signal into a serial port set to 9600 baud, then each pulse of 700 microseconds would look like a start bit (lasting roughly 100 microseconds) and then 5 or 6 “one” data bits, with the last several data bits being “zero”.  On the other hand, the 1500 microsecond pulses would look like a start bit and then all “one” bits.  So I wrote a serial port driver that reads bytes, ignores framing, overrun, and parity errors, and returns a string of ones and zeros, where the ones are short pulses and the zeroes are long pulses.  For example, holding down the “Channel Up” key sends:

000001101001011110010110100000011010010111100101101000000110100101111001011

while holding down “Channel Down” sends:

000001101001111110010110000000011010011111100101100000000110100111111001011

Notice that the shortest repeating identifying pattern for “Channel Up” is

0110100101111001011010

while the pattern for “Channel Down” is

01101001111110010110

Once these patterns are identified, run.c can read the stream of bytes from the serial port, search the list of patterns, figure out what buttons were pressed, call Mike’s driver to run the trains, and update the display.

I wrote a program called learn.c that prompts you to press each button on the remote, and writes the long strings to a file.  I then manually edited that file down to the shortest repeating identifying pattern.  The program run.c then reads this file, known as “KEYS.TXT”.  It simply describes the pattern for each key.

If you run learn.exe, you can remap the keys any way you like.

If you wanted to use a different remote, you could try running learn.exe and see if it works for your remote.  If you buy the one I use (or the SystemLink 4) then you don’t have to worry about any of this – it should just work.

How do I build the infrared receiver?

See the attached bitmap.  You can buy the infrared receiver module (Radio Shack part number 276-137), the DS1488 rs-232 driver chip, and the 7404 TTL inverter at Radio Shack or at Digi-Key.  If Radio Shack will only sell you a 7404 with a “C” in the middle of the part number, like a 74HCT04, be aware that this is a CMOS part and that all of the unused inputs should be tied to ground (or +5V).  I was able to find +5V, +12V, and -12V in the PC.

So how do I pull all this together?

Download my “RUN.EXE”, “KEYS.TXT”, and “LOCOS.TXT”, and Mike’s software to your PC.  Build Mike’s circuit and a booster and run Mike’s DCCTEST.EXE to make sure everything works.  (Or build my Centronics to DCC circuit instead of Mike’s).  Then build my infrared circuit and connect it to the first serial port.  Then edit LOCOS.TXT to describe your locomotives.   Add a line in CONFIG.SYS:

DEVICE=C:\DOS\ANSI.SYS

(The ANSI.SYS driver came with your DOS disks.  Typically it will be installed in the C:\DOS directory, but you may have to hunt around for it.  If you skip this step, the screen display will be garbled.)  Reboot the computer after changing CONFIG.SYS.

Then say

MODE COM1:    9600,n,8,1

to set up the port for 9600 baud, no parity, 8 data bits, and one stop bit.  Then say

MODE CO40

to turn on 40 column mode to get a larger font that can be read at a distance.  If you would prefer words other than “Forward” and “Reverse” on the screen, edit the file WORDS.TXT.  Then type

DCC-MB

RUN

to load Mike’s driver and start my program.  Fire your handheld remote control at the receiver and get back to railroading!

Copyright 1996 Chuck Heller