TMW DCC     ver1.32

This program is an application written by Andreas Hansson, with the specifications and features

by Lars Lundgren at Tillorps Mekaniska, to fit the needs for a DCC system at Skovde

Model Railroad Club (The Westergotland-Nerikes Jernvagar WNJ) in Sweden. It was originally

written as an application program to run under a driver released on the Internet by

Michael Brandt

http://web.syr.edu/~mobrandt/dcc-mb/dccmbhom.htm.

The original driver was modified, direct addressing new programming modes and a new throttle handler was added, and the

name altered from dcc-mb.com to dcc-mbX.com.

The program is compatible with the original Michael Brandt hardware (see NOTE).

| This version requires the DCC-MB17.COM driver.

The program has two screens. The   Operate  Mode  Screen and the Service Mode Screen.

THE OPERATE MODE SCREEN ( Blue headline.)

This screen is used when operating your railroad. The screen is divided into three

windows, a gray help window, a black console window and a light blue status window.

THE GRAY HELP WINDOW

The window shows the available commands.

L) Locomotive		Allows you to:

1)   	Assign a  locomotive to a throttle. You can assign multiple locos

to one throttle, (MU) but a loco can be assigned to only one throttle.

If a loco in a MU is going to run backwards, you can reverse the

direction by selecting a negative loco number. E.g. if loco #5 is going

to run backwards together with loco #7 (running forward) you assign

loco #5 as -5. If assigning a loco that is already assigned to

another throttle, you steal it from the first throttle.

2)	Activate a loco and assign it to its latest used throttle.

3)	Deactivate a loco.

4)	Select speed mode for that locomotive.

T)  Throttle 		Allows you to:

1)	Assign a locomotive to the selected throttle.

2)	Deactivate a locomotive.

3)	Calibrate the selected throttle.

4)	Select between throttle TypeA, TypeB or Type C.

(See note about the C throttle.)

S)	Servicemode		Switches the program into Service Mode

Q)   Quiet mode.		Blanks and deactivates the console status

display.

U)  Autoload mode.		Toggles auto load of register values from a loco

positioned on the servicetrack, when entering

service mode. Autoload uses the latest

selected decoder type.

C) Clear console screen.	Does just that.

SPACE BAR		Emergency stops all locos. Resets with HOME.

The command removes the voltage from the track after 5 sec.

HOME		Resets the Emergency status.

M)   addressing Mode	Address mode enables page mode, direct mode,

register mode, Old mode or Rpg mode, when

using autoload.

F)   reFresh Functions	Enables a feature that updates the function

settings to all active locomotives every 2

seconds.

A)   Accessory switch	Here you can switch on and off accessories connected to

an accessory decoder. The syntax is <address> <ID><+|->

This needs an explanation. "Address" is the decoder

address.

The first accessory decoder has address 1, the second

address 2 the third address 3. Each decoder has 8

outputs (IDs). They are numbered 0 - 7. The plus and

minus are used to switch on and off an output.

Example:  1 3+ turns on output number 3 on decoder 1.

5 0-  turns off output 0 of decoder number 5.

W)   sWitch throw		With the switch command you can throw a switch (turnout.)

The syntax is <address> <+|-> where address is the actual

turnout address and plus or minus is throwing or clearing

the turnout.

Example  6+ throws turnout #6 , and 6- clears

turnout #6. The numbering of switches goes like this.

A turnout decoder can usually control four

turnouts. The first decoder controls turnout number

1 - 4, the second number 5 - 8 and so on.

R)   Routes		Routes or turnout ganging is a way of preparing a train

route. When you press "R" you will be prompted to enter a

route name. The Route name is the file name, less the

extension .rot of the file corresponding to the route.

O)   On The Main Programming	With the OTM command you can alter any CV content while the

locomotive is on the main track.

X)   eXit		Exits the program.

THE BLACK CONSOLE WINDOW

The window shows program status codes and provides input and output

between you and the program.

THE LIGHT BLUE STATUS WINDOW

A throttle line appears when connecting a throttle to the game port, and

disappears when disconnecting the throttle. If a throttle controlling a

loco is disconnected, the loco maintains its speed, direction and aux-

settings. The keyboard line is always there. (see NOTE)

Th		Throttle number 1-8 +keyboard

T		Throttle type  K=keyboard A=TypeA B=TypeB C=TypeC

Spd		Speed step

Dir		Direction

F0-F4		Status of AUX functions. Toggles with throttle #

and F1-F5 keys. E.g.  2 F1  toggles F1 on the loco

controlled by throttle #2.

Controlled Locos		Shows locos assigned to this throttle.

Auto, Quiet, Refresh   	Shows the status of these features.

Reg, Roc, Dir, Old		Shows in which mode the Autoload runs. This is also

Pag, Rpg		the mode used in Service Mode

THE  SERVICE MODE SCREEN  ( Red Headline).

This screen is used when setting up a locomotive. The screen is divided into

three windows, a gray help window, a black console window and a light blue

status window. When entering Service mode the entire layout is disconnected

and the only track under power is the Service Track. Put the locomotive to program

on the Service track prior to entering Service mode.

THE GRAY HELPWINDOW.

The help window shows the available commands;

R)  Read	 	Reads a single CV from the loco.

W)  Write		Writes a single CV to the loco.

E)  Edit		Lets you edit contents in status window. Edit ends with ESC.

When exiting you can save the content of the Status window into

a specified file on your hard disk, or into the loco.

V)  Verify		Verifies the contents of a selected CV.

L)  Load		Loads a specified file into the loco.

S)  Save		Saves the content of the status window into a specified

file or into the loco.

C)  Compare		Compares the loco CV contents with the contents of a

specified file.

F)  reFresh		Dumps CV contents from loco to status window.

X)  eXit		Exits Service mode and sends you back to operate mode.

M)  Mode		Toggles between dir, old, page, reg or rpg mode.

The manual of the decoder you are going to program

will tell you which mode to use. The Old mode is only

used with ZTC decoders. They do not confirm with the

NMRA standard regarding direct mode addressing of the

CV:s. RPG mode is used with CVP AD4 accessory decoder.

P)  Power cycle		When you press "P" You will remove the power from the

service track for 2 seconds.

0-9  Decoder Type 		Pressing 0-9  switches  the  program  to  deal  with

different types of decoders. The data displayed in the

status window depend on the decoder type selected. The

program is looking for .DEC files in your working

directory. Line one, if beginning with a *, is the

addressing mode that TMWDCC selects for you. If not

beginning with a * it is the headline in the helpwindow.

The lines after specify what CV:s to be dealt with.

The decoder type NMRA is a fake decoder with all possible

CV:s  displayed.

T)		T is a feature used to calibrate the acknowledge hardware.

When typing T the program writes 1 to register 6 (the

page register) and after that confirming that the

register contains the value 1. The confirm is done about once

every second.

Each time this is done, the decoder responds with an

acknowledge, and the motor will jump for every ack.

The program senses the ack and displays "Ack

received #". If the hardware fails to detect the

acknowledge from the decoder, the program does not

display anything. If the hardware detects a false,

continuous ack, it will make the motor "jump" once,

and then display "Ack recieved #" continuously. See the

adjustment procedure on the hardware web page for correct

adjustment of the ack detector..

THE BLACK CONSOLE WINDOW

Provides input and output between you and the program.

THE LIGHT BLUE STATUS WINDOW

The window shows data read from loco or file. You can edit

the data and save it to a loco or to a file on your hard disk..

FILETYPES

DCC-MBX.com is the driver. It loads with the command DCC-MBX [lpt #]

Lpt1 is default. The driver is a driver originally written by

Michael Brandt and modified to fit TMWDCC at Tillorps Mekaniska.

TMW130.EXE is the application program.

.DEC  files are decoder files telling the program which CV:s to display. Check

with your decoder manual when creating a new .DEC file. Use the Template.txt as a

pattern and edit it to fit your decoder type, edit line one to get a new heading

and number in the gray help window, and save the file as "yourdecodertype.dec".

You can have maximum ten decoder files present.

.ROT  files are route files.  They consist of four commands:

*   Prints this line on the screen.	e.g.  *Johns test route  will print "Johns test route   on the screen.

s   Throw a switch (turnout)	e.g.  s1+ throw turnout #1    s3- clear turnout #3

w   Wait for the decoder to recharge  	e.g.  w1000    wait 1000 ms  (1sec)

w1500    wait 1500 ms  (1.5sec)

a   Accessory control  	 e.g.  a1 0+    switches  ON accessory 0 on decoder 1

a3 4-    switches OFF accessory 4 on decoder 3

Tmw131.cfg is a file in which the program saves its configuration at exit.

You can delete this file to get a "fresh start" if you mess up something.

The first time you exit TMWDCC after you have deleted the file Tmw130.cfg,

you get an error message (Null pointer assignment) in the DOS window. This

is normal.

Files with no extension are files saved from locos, under e.g. the loco's name.

Manual.doc is this file you are reading now.

Tmw100.hex  The microcode to load in the TMW01 processor board.

December 4 1999

Lars Lundgren, Tillorps Mekaniska

lars.lundgren@mailbox.swipnet.se

Note:	Throttle type A is a throttle with a knob for controlling the speed and a

toggle switch for controlling the direction.

Throttle type B is a throttle with one knob for controlling the speed, one

toggle switch for controlling the direction and one toggle switch each for the

five (only for working at the moment) functions F1 - F4 and F0/FL.  The ROCO

Wreck crane controller is a type B throttle with only F0 and F1 controlled.

The type B throttle takes two throttle numbers, thus the throttle after a

type B throttle is unavailable to be used as a separate throttle. This means

that you can have max for type B throttle in a "8 throttle system"

The Type C is the most powerful TMW throttle, and with its knob you can control

the speed and with the a push button you can control the five functions

F1 - F4 and F0/FL.  The push button together with the knob also lets you assign

and free locomotives and to build and clear consists

The C-throttle is operated like this:

When the speed is zero, a short push on the button the button toggles the

direction.

When the speed is zero and you press and hold down the button, you will

enter a locomotive selection screen .If you now turn the speed knob up to

half speed, the big numbers on the screen will start browsing through

all available locomotives. The more speed the faster it browses. If you

slow down the speed you will start to browse backwards. When the locomotive

you like to select is displayed on the screen, a short push on the button will

toggle a minus sign before the locomotive number. The minus sign means that the

locomotive will run backwards in e.g. a consist. If you now press and hold the

button the selected locomotive will be acquired.

You can now select another locomotive to the same throttle and build a consist.

If you select locomotive Zero, you will free all locomotives controlled by the

throttle.

When the speed is NOT zero, one short press on the

button toggles F1. Two short presses toggles F2 and so on. Five pushes toggles

F0/FL. If you press the button once and keep it pressed, you will toggle F1, and

when you let the button up it will toggle back again. If you press the button

twice, and keep the second press down, it will toggle F2, and when you let the

button up, it will toggle back, and so on to five presses and keeping the fifth

down for operating F0/FL.

lars.lundgren@mailbox.swipnet.se