# TMWACC1 decoder ver 1.0

Operates four Bang-Bang type Turnout Motors.

The outputs can be operated individually.

The following NMRA defined CV's and ranges are supported...

CV513	Lower 6 address bit  00h - 3Fh

CV514	Output enable.           00h - FFh  Bits 0 to 7.Each bit sets an output.

CV515	F1   Function 'on' time   00h – FFh  10ms / steps. 0 is continuous

CV516	F2   Function 'on' time   00h – FFh  10ms / steps. 0 is continuous

CV517	F3   Function 'on' time   00h-- FFh  10ms / steps. 0 is continuous

CV518	F4   Function 'on' time   00h – FFh  10ms / steps. 0 is continuous

CV521 	Upper 3 address bits	00h -07h

Read only CVs.

CV519	Version Number       = 10 (ver 1.0)		Cannot be changed

CV520	Manufacturers ID     = 13 (DIY decoder)	Cannot be changed

CV541	Configuration=80 (Accessory decoder)	Cannot be changed

No other CVs are supported.