1.0   Introduction :

Bunkers are vessels designed for storage of material  such as coal, coke, sinter, iron ores,  manganese ores, cement, grains, any other bulk materials, over a  temporary  period of time. They may  be  rectangular  or circular in plan. This software is  for rectangular steel bunkers which  are   commonly used in Steel Plants,  Power  Plants, Coal Handling plant and in different Raw Material  Handling Plants.

Design of bunkers is very time consuming task.  This involves lot of repetitive calculations. Before arriving at any final  design value, numerous trial  calculations  are required to be performed. Calculation of various forces for different variant cases takes many engg.-hours. To minimise time consumption  and to achieve accuracy,  a  software  is required  to perform analysis and design of steel  bunkers.

This  software  has  been developed so that  the  User  can design  the  bunker easily and effortlessly.  The software includes Online Help in Input Mode an Input Editor, a Result Viewer and a Result Printer and use of DOS Shell option all within SOFTDOSS environment.

***

- 2.0 Design Concept :

Rectangular  steel  bunkers  are analysed  using  Rankine's Earth  Pressure Theory assuming that filling materials  are cohesionless. The  height  of vertical wall of  bunkers   is generally   less  than the least dimension  of  bunkers  in plan.  In  such  cases wall friction on  vertical  wall  is negligible. The forces developed in a rectangular bunker will induce  direct, shear and bending stresses in  the  various  parts.

fig-1 General Arrangement of Bunker

The   pyramidal  bottom  portion   called   hopper consists   of   skin  plates  supported   over   horizontal ribs/stiffeners. The skin plate which forms the hopper wall is designed to span between suitable placed horizontal ribs and it is subjected to bending, some shear and longitudinal tension. Normally the plate is designed for the bending and  the longitudinal tension and shear is neglected.The  trough stiffeners  or horizontal ribs are required to support  the skin plate which carries the load. These ribs act as  beams subjected  to  bending, shear and direct  tension.  In  the actual  design  of rib only bending and direct  forces  are considered  and  part  of skin plate  is  included  in  the strength calculation of ribs.

The  top or straight portion of bunker forms  bunker  beam. The  web along with vertical stiffeners forms the  wall  of bunker. This portion is designed to accomodate its share of  total  vertical load, the horizontal side  pressure  within its own depth plus the horizontal reactions from the hopper portion.   In   actual  design  the  side   pressures   are transferred  proportionately at the top and bottom  flanges and  the beam is considered to act as an  equivalent  truss system.  The web plate is designed as tension diagonal  and vertical   stiffeners  as  compression  members   of   this equivalent  truss. All other forces are combined with  this to  design the beam components. This has been discussed  in detail in Design Procedure chapter.

- 3.0   Utility:

- This software has got following utilities.

- 1.  Detail Engineering:

Analysis   and  design   of  steel  bunker is very tedious and time consuming job.  It takes a whole day to design a simple bunker and at  least a  week  to  design manually total  bunkers  of  a  Stock house/Raw  Material  plant.  This  software  designs   15  bunkers at a time in no time and gives the results  which  can  used  for  preparation of  design   and  fabrication drawings.

- 2.  FR preparation :

SOFTDOSS flashes total  tonnage estimate on the screen after completion  of design.  This value can be used for costing/estimating purpose  without taking any print or going into result file.

- 3.  Absolute Economic Design for Research/Academic Purpose:

For   a  constant   geometrical parameter of a bunker an absolute economic design can  be  achieved  by this software. The input parameter  for  the placements  of  vertical stiffeners  in  web,  horizontal stiffeners in hopper and depth of vertical wall beam  can  be  changed using SOFTDOOSS EDIT option.For each run  the displayed  tonnage estimate can be compared  to get  then minimum poossible tonnage.

- 4.  A Marketable Software Product:

The SOFTDOSS   package  has   been developed  as  MENU DRIVEN Stand Alone Package and  needs only DOS (3.0 or above) for its exection. Any PC-AT  with VGA  card  (which is needed for graphics  generation  in draw option) can be used. SOFTDOSS is fully user friendly product  provided  with   Input  Editor,   Input  Drawer,  Result  viewer   Result Printer and  DOS  Shell  options. SOFTDOSS traps almost all the errors and gives its User a chance  to  rectify them. The User will not  require  any other  WordPrcessor  like  Wordstar, Norton  Editor,   MS-editor,etc  to  edit the input or to view  or  print  the results.  The screen getup has been developed  in  trendy professional  manner so that User never assumes that  the System  has  haulted. These features make  this  software package a truly marketable product.

- 4.0   Setting Up:

To setup SOFTDOSS System, insert SETUP DISK in Drive. Suppose you want to  setup in X: drive on your hard disk. X: may C: or D: or E: etc.

Goto that drive by typing X:  or you may be already  in that  drive. Type A:SETUP or B:SETUP (as per  your  drive specification)at DOS prompt(X:>) and press Enter.

SOFTDOSS  SETUP  will create a new drectory as  per  path

specified  by you and will load the SOFTDOSS software  in

that  directory. Your specified path will be  in  x:drive

from  where  you  invoke SETUP. SETUP will  not  work  if

invoked from floppy disk drive.

Otherwise  you can use DOS COPY commands to  copy all the files of DISK into  your directory.

CONTENTS OF SETUP DISK (11 files)

SETUP.EXE

SOFTDOS1.EXE

SOFTDOS2.EXE

SOFTDOS3.EXE

SOFTDOS4.EXE

SOFTDOS5.EXE

SOFTDOS6.EXE

SOFTDOS7.EXE

SOFTDOS8.EXE

SOFTDOSS.EXE

SOFTDOSS.TBL

SOFTEXMP.EDT

If all the files are not found ask for the rest of files or new setup disk.

- 5.0   Getting Started:

Type  SOFTDOSS and Press ENTER. It will take you  to  the following screen.

fig-2 Opening Screen

Strike  any  KEY.  It  will take  you  to  the  following  screen.  This is  the Main Program Driver. To begin  with you can enter file name = SOFTEXMP. This file is supplied  with this package. Select Run option to create result file.

SOFTDOSS 1.0(1996) MODULE: SBAD STEEL BUNKERS ANALYSIS & DESIGN 1 RUN 2 EDIT 3 VIEW 4 DRAW 5 PRINT 6 DOS 7 END Selection is 1 Enter File/path name : F1=Help F2=GA of Bunker Use Arrow Keys & Press Enter Or Cilck Mouse to Select/Invoke the Option fig-3

SOFTDOSS provides folllwing Menu Driven Facilities to the users.

- A. RUN OPTION: To search and run the existing input file or

to create input data for a new file.

- To  run this option, enter a  valid  Filename without extesion   and   press   ENTER.The SOFTDOSS    will    search    for    a    file "Filename.EDT". If it finds this file it  will read  the  input  from the  file  and  do  the analysis  and  design.If does  not  find  this file,  it will ask you   to enter input  data.

To input data refer next chapter.

- B. EDIT OPTION: To edit input file.

To  edit  input file,select this  option  and press  ENTER.  This  option  provides  you  an editor to edit your input file. You   can  use following keys :

ESC + Y  :  To exit with save.

ESC + N  :  To exit without save.

DEL      :  To delete character right.

BkSpc    :  To delete character left.

INS      :  To Change mode of editing.

Insert "ON"/"OFF".

Tab      :  To insert/shift by  5-columns to right.

Ctrl + Y :  To delete line.

ESC + ESC:  To continue editing.

ENTER    :  To insert line when  Insert is On.

To go to next line when Insert is Off.

HOME     :  To go to begining of the line.

END      :  To go to end of the line.

PgDn     :  To scroll down one full screen page.

PgUp     :  To scroll up one full screen page.

Up ARROW :  To scroll up one line.

Dn ARROW :  To scroll down one line.

-> ARROW :  To go to one column right.

<- ARROW :  To go to one column left.

Ctrl+END :  To go to the end of file.

Ctrl+HOME:  To go to the beginning of file.

F1       :  To see help menu.

F10      :  Save and Exit.

F2-F9    :  Not defined here.

It  may  be  noted  that comma(,) has  been  used  as  data separator which shall not be deleted or erased because this may generae error such as "INPUT PAST END OF FILE". Make  a practice of editing numerical value only.

- C. VIEW OPTION: To view Result file.

To view result file, select this option and press ENTER.This  option provides facility to view  and  browse  through  the  result file.  You  can  use following keys :

HOME     :  To go to begining of the file.

END      :  To go to end of the file.

PgDn     :  To scroll down one full screen page.

PgUp     :  To scroll up one full screen page.

Up ARROW :  To scroll up one line.

Dn ARROW :  To scroll down one line.

Ctrl+END :  To go to the end of file.

Ctrl+HOME:  To go to the beginning of file.

F1       :  To see help menu.

F10      :  Exit.

F2-F9    :  Not defined here.

OtherKeys: Disabled.

D. RAW OPTION  : To draw & check the input.

SOFTDOSS provides following Menu driven  facility for this option.

1.EXIT

2.DRAW

3.LABEL

4.ZOOM---------->1. PLAN

5.EXIT          	      2. LEV.

3. EXIT

DRAW   :  Draws plan & elevation on Scale 1:300 (initially).

LABEL  :  Marks  wall  No.  &  Bunker  No.  in  plan &  Sheathing/Skin plates in elevation.

ZOOM   :  Has three options.

PLAN   :  Asks for new scale.

First  enter new scale such as 50,100,150  etc and  press  Enter. Then  select  this option.  For  labelling  exit  ZOOM  menu  and select LABEL. ELEV.  :  same  as  PLAN. Enter valid  scale  and  press Enter.

EXIT   :  To exit from current menu.

***

6.0   DATA INPUT :

  - GENERAL TERMS AND NOTATIONS USED:

Olx Oly Lx Ly 4 3 2 1 1 fig-4 Typical plan

  = Angle of repose of filling material.

Y  = Bulk density of filling material.

W  = Floor area load.

Lx = Length of bunker.

Ly = Width of bunker.

HT = Total Depth of bunker.

HP = Depth of hopper portion.

OLx= Opening size of hopper bottom along length of bunker.

OLy= Opening size of hopper bottom along width of bunker.

Ex = Eccentricity  of centre of opening  of  bunker  from centre of top plan of bunker along length.

Ey = Eccentricity  of centre of opening  of  bunker  from centre of top plan of bunker along width.

Ex,Ey  are taken as coordinates of centre  of  opening assuming  the centre of top plan as origin. The  right shift  for Ex is +ve, and left shift is -ve,  similary the upward shift is +ve and downward shift is -ve  for            Ey in top plan of bunker.

OEX= Same as EX.

OEY= Same as EY.

NHR= No. of horizontal divisions on hopper plate. (first  division will determine the bottom   of bunker beam.NHR=3 for fig-5.)

  = Slope of bunker wall.

Sv = Spacing of vertical stiffeners in the web  of  bunker beam.

( This shall be determined taking even  divisions of length web of bunker.)

Ex/Ey Sh1 Sh2 Sh3 Sh4 Sh5 R1 R2 R3 R4 Ht Hp fig-5 Typical Elevation of a Bunker Sh1 to Sh5 : Sheathing Nos R1 to R4 : Rib Nos

  - Wall no. and Bunker no.

A   fixed  clockwise  marking  has   been followed  for walls.  Bunker nos  marking have been displayed in the bottom opening space as shown  in fig-4 & 6.

  - Sheathing no.:

The  inclined  portion  of  bunker   i.e, hopper  is to be divided  into  different divisions. The spacing of these divisions will determine the spacing of  horizontal   ribs. The sheathing numbering starts from  top.  The  first spacing  determines  the location of bottom flange of bunker beam.

So the depth of bunker beam = HT-HP + SH1

Refer fig-5 also see fig-1.

Normally  values for rib spacing is  kept   between 400 to 600 mm. There  are Five sheathing  divisions  in fig-5.

  - Rib no.:

Rib  marking has been done from the top. The  first rib is the rib just below  the  bottom flange of bunker beam.

Total no of  ribs = NHR-1.

The  bottom inverted rib is not  designed  it  is  provided as  per  the  functional requirement. However,the last rib section may  be  used  safely  if there  is   no information provided. There are four  ribs

in fig-5.

3 4 2 1 3 3 3 3 4 1 2 1 4 1 4 2 3 4 2 1 C1 C2 C3 C4 Column Support (Typ) fig-6 (a) Bunker/Wall Numbering System & Beam Wall Connectivity for Common Beams

  - Beam Wall Connectivity:

For  two  adjacent walls of bunkers being  supported  on  a       common  beam  the connectivity data is to be given  as  per       following. Refer fig-6.

- Total no. of common beam = 4  as marked C1,C2,C3 and C4.

Connectivity for common beam no. 1 (C1)

bunker no = 1

wall no   = 4

bunker no = 2

wall no   = 2

likewise connectivity for common beam no.2 (C2)

bunker no = 1

wall no   = 4

bunker no = 2

wall no   = 2

3 4 2 1 3 3 3 3 4 1 2 1 4 1 4 2 3 4 2 1 C1 C2 C3 C4 Column Support (Typ) fig-6(b) Multispan Bunker Beams

  - Multispan Bunker Beams:

In  fig-6 the middle columns have  been  removed. This may be due to functional requirement of  the buliding.  In  such cases single beam  spans  for  both   the  bunkers.  These  beams   are   called multispan beam. These beams has been further categorised into two category.

    - Multispan Edge Beam:

Refer  fig-6,there  are  two  multi  span  edge beams.  The edge beams marked 3 for  bunker  no.1 and  2 constitute a multispan edge beam and  edge beams  marked 1 for bunker no 3 and 4  constitute another  multi  span edge beam.  For  input  data entry  you  will have to enter all such  type  of edge beams and related data as follows:

Total no. of multispan edge beam = 2

For edge beam no.=1 correspoding wall/edge no=4

no of constituent beam = 2

for  constituent beam no.1  corresponding  bunker no=1

for  constituent beam no.2  corresponding  bunker no=2

For edge beam no.=2 correspoding wall/edge no=1

no of constituent beam = 2

for  constituent beam no.1  corresponding  bunker no=3

for  constituent beam no.2  corresponding  bunker no=4

    - Multi Span Common Beam:

In  figure  5,  common beam marked  c3  &  c4  constitute one multispan common beam. For  input data entry you will have to  enter total  no. of such common beams  and  related information as follows:

for fig.5

Total no of multispan common beams =1

for multispan common beam no =1

total no of constituent beams = 2

for constituent beam no = 1 corresponding common beam no = 3

for constituent beam no = 2 corresponding common beam no = 4

6.2   Sequence for Input Data Entry:

Enter  data as it is asked. You will come across  following sequences. "xx.." indicates the value to be entered by  the user  and "II.." shows computer responded value  for  which the data is to given.

First Screen:

ENTER PROJECT TITLE = xxxxxxxxxxxxxxxxxx TOTAL NO. OF BUNKERS = XX DATA FOR BUNKER NO.......I 01.0 ANGLE OF REPOSE(Degree) OF FILLING MATERIAL...... =XX 02.0 BULK DENSITY(T/M3) OF FILLING MATERIAL...........Y =XX 03.0 FLOOR LOADING(T/M2)..(DL+LL).....................W =XX 04.0 LENGTH AND WIDTH OF BUNKER (M)...............Lx,Ly =XX 05.0 TOTAL DEPTH OF BUNKER(M)........................HT =XX 06.0 DEPTH OF HOPPER PORTION(M)......................HP =XX 07.0 OPENING SIZE L,B(M)........................OLx,OLy =XX 08.0 ECCENTRICITIES OF OPENING ...................Ex,Ey =XX 09.0 SPACING(MM) OF VERTICAL STIFFENERS IN WEB...............................SV =XX 10.0 TOTAL NO. OF HORIZONTAL DIVISINS ON HOPPER PLATE.......................NHR =XX DATA O.K <Y/N> X

Next Screen:

- DO YOU WANT TO KEEP UNIFORM SPACING FOR HORZ. RIB.<Y/N>= X

| If  you enter "Y" for the  above

| choice  then it  completes  the

| data  entry  for  bunker   under

| consideration.

| If  you enter "N" for the  above

| choice then enter spacing one by

| one as it is being asked.

Next Screen:

- ENTER SPACING (MM) FOR DIVISION NO. I = ? XX - ENTER SPACING (MM) FOR DIVISION NO. I = ? XX - ENTER SPACING (MM) FOR DIVISION NO. I = ? XX ENTER SPACING (MM) FOR DIVISION NO. I = ? XX - ABOVE DATA OK <Y/N> X

| After data confirmation SOFTDOSS

| checks the correctness of    the

| entries   and  suggest   correct

| uniform    spacing.   In    case

| spacings   are   not   correctly

| entered, following message  will

| flash  on  screen.  It  provides

| opportunity to re-enter  correct

| data once again.

- SM,HHP(BN)=II; HHOP(BN)=II

- INPUT ERROR IN RIB SPACING  SHOULD BE .. PL. CHECK III

.........RE-ENTER  DATA AGAIN FOR RIB SPACING .........

- PRESS ANY KEY WHEN READY

Next Screen:

- DATA ENTRY COMPLETE FOR BUNKER NO. PRESS ANY KEY WHEN READY FOR NEXT INPUT

Next Screen:

- IS DATA SAME FOR OTHER BUNKERS ? - EXCEPT 1.0 OPENING-ECCENTRICITIES 2.0 DENSITY OF FILLING MATERIAL ENTER........<Y/N>..........XX

Next Screen:

ENTER FOLLOWING THREE DATA <DENSITY(T/M3),EX(M),EY(M)> FOR BUNKER NO= I DENSITY(T/M3),OEX(M),OEY(M) ? XX,XX,XX FOR BUNKER NO= I DENSITY(T/M3),OEX(M),OEY(M) ? XX,XX,XX - ABOVE DATA OK (Y/N)

Next Screen:

- ENTER DATA FOR BUNKERS WALL/BEAM CONNECTIVITIES TOTAL NO OF COMMOM BEAMS ......=XX ¦ In case all bunkers are - ¦ are isolated and there ¦ is no common beam, press ¦ <Enter> to proceed. - DATA FOR COMMON BEAMNO..............= I - FIRST BUNKER NO=XX - FIRST WALL NO=XX - SECOND BUNKER NO=XX - SECOND WALL NO=XX - ABOVE DATA OK (Y/N) X

Next Screen:

- ENTER DATA FOR MULTISPAN EDGE BEAMS - TOTAL NO. OF MULTISPAN EDGE BEAMS xx ¦ In case there is no such ¦ beams press <Enter> to ¦ proceed. - FOR MULTISPAN BEAM NO= I EDGE/WALL NO = XX - TOTAL NO. OF CONSTITUENT BEAMS = xx - FOR CONSTITUENT BEAM NO = J CORRESPONDING BUNKER NO = xx - ABOVE DATA OK (Y/N) X

Next Screen:

- ENTER DATA FOR MULTISPAN COMMON BEAMS ¦ In case there is no such ¦ beams press <Enter> to ¦ proceed. - TOTAL NO. OF MULTISPAN COMMON BEAMS =XX - FOR MULTYSPAN BEAM NO = I - TOTAL NO. OF CONSTITUENT BEAMS = XX - CONSTITUENT BEAM NO = I CORRESPONDING COMMOM BEAM NO = xx - ABOVE DATA OK (Y/N) X

- Data entry is complete.

- 7.0   Result Output:

SOFTDOSS  creates  a result file listing the  input  data provided  by  the User and giving the analysis  and  design results in tabular form. Results are given for all four walls of bunkers as follow:

7.1   Results for Hopper Portion

- Results Sheathing Plates

Results are tabulated in following form:

| SH.NO. | Z | Pl.Th. | Mn,cal | Ptn,cal | fb,cal | fat,cal | f ,cal | ELE.WT. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

where,

| SH.NO. = | Sheathing no. |
| --- | --- |
| Z = | Depth(M) of c.g. of sheathing plate under consideration. |
| Pl.Th. = | Design Plate thickness in MM. |
| Mn,cal = | Calculated Bending Moment(T-M) at the center of sheathing plate under consideration due to normal presuure. |
| Ptn,cal = | Calculated Longitudinal Tensile Force(T) acting on sheathing plate. |
| fb,cal = | Calculated Bending Stress in Kg/cm2 due to Mn,cal |
| fat,cal = | Calculated tensile stree in Kg/cm2 due to Ptn,cal. |
| f,cal = | fb,cal + fat,cal < 1500 Kg/cm2 |
| ELE.WT. = | Weight of sheathing plate in Kg. |

Note : Mild Steel fy : 250 has been assumed.

- So, Permissible Stresses has been taken as 1500 Kg/cm2.

- Results for Hopper Ribs

Results are tabulated in following form:

| RIBNO | Z | SECTION | Mn,cal | Pat,cal | fb,calS | fb,calR | fat,cal | ELE.WT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

where,

| RIBNO = | Rib no. starting from top. |
| --- | --- |
| Z = | Depth(M) of rib under consideration |
| SECTION = | Designed Section may be one of the three i. ISA angle sections. ii. Half cut ISMB sections. iii. Built up Tee/Half-cuts with results for thicknesses and depths for flange and web with: - dw = web depth in MM. tw = web thickness in MM. df = Flange width/depth in MM. |
| Mn,cal = | Calculated Bending Moment(T-M) at center of span due to normal pressure. |
| Pat,cal = | Calculated Axial Tensile Force(T) acting on rib due to normal pressure on adjecent walls. |
| fb,calS = | Calculated Bending Stress in Kg/cm2 due to Mn,cal at the sheathing edge. |
| fb,calR = | Calculated Bending Stress in Kg/cm2 due to Mn,cal at the rib edge. |
| fat,cal = | Calculated tensile stress in Kg/cm2 due to Pat,cal. |
| ELE.WT. = | Weight of rib section in Kg. |

Note :  Though f,cal is not figured in table due to constraint of

space, following has been ensured.

- f,cal = rb,calR + rat,cal < 1500 Kg/cm2

- Permissible Stresses has been taken as 1500 Kg/cm2 for Fy 250:

- 7.2   Results for Beam Portion

Bunker beams are designed for various variant cases. Results  are  listed  for all  six/thirtysix  variants  and  finally  the  most critical section is chosen  as  designed  value  alongwith  their weight in kg. First  web  plate  is designed.  The  design  thickness of  web  is  subsequently  considered  for design of top flange and bottom flanges  of  beam.  The  permissible   stresses has been  taken  as  1500 Kg/cm2  assuming grade of steel as Fy 250.

- Web Plate :

The results are tabulated in following way : -

| VARIANT | Tw | Pat,cal | Pn,cal | fb,cal | fa,cal | f,cal |
| --- | --- | --- | --- | --- | --- | --- |

where,

| VARIANT = | No of variant case for which design is done. |
| --- | --- |
| Tw = | Design thickness in MM of web plate for the above variant no. |
| Pat,cal = | Calculated Axial Tensile Force(T) due to Total vertical force on beam. |
| Pn,cal = | Calculated Normal Pressure(T-m) on web inducing bending in horizontal plane. |
| fb,cal = | Calculated Bending Stress in Kg/cm2 due to Pn,cal. |
| fa,cal = | Calculated Tensile stress in Kg/cm2 due to Pat,cal. |
| f,cal = | fb,cal + fac,cal < 1500 Kg/cm2 |

- Top Flange

| VARIANT | Ttf | Btf | Mx,cal | Pac,cal | fb,cal | fa,cal | f,cal |
| --- | --- | --- | --- | --- | --- | --- | --- |

where,

| VARIANT = | No of variant case for which design is done. |
| --- | --- |
| Ttf = | Design thickness in MM of top flange for the variant no. |
| Btf = | Width for Ttf in MM. |
| Mx,cal = | Calculated Bending Moment(T-M) due to Normal pressure on web part of beam. |
| Pac,cal = | Calculated axial Compressive Force(T) due to Bending Moment due to Total vertical force on beam. |
| fb,cal = | Calculated Bending Stress in Kg/cm2 due to Mx,cal. |
|  | fa,cal = Calculated Compressive stress in Kg/cm2 due to Pac,cal. |
| f,cal = | fb,cal + fac,cal < 1500 Kg/cm2 |

- Bottom Flange with Vertical Edge Stiffener

---------------------------------------------------------------

| VARIANT | Tbf | Bbf | Tvbf | Bvbf | Mx,cal | Pat,cal | fb,cal | fa,cal | f,cal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

where,

| VARIANT = | No of variant case for which design is done. |
| --- | --- |
| Tbf = | Design thickness in MM of bottom flange for the above variant no. |
| Bbf = | Width for Tbf in MM. |
| Tvbf = | Thickness in MM of Vertical plate stiffener welded at the outer edge of bottom flange. |
| Bvbf = | Width for Tvbf in MM. |
| Mx,cal = | Calculated Bending Moment(T-M) due to Normal pressure on web part of beam. |
| Pac,cal = | Calculated axial Compressive Force(T) due to Bending Moment due to Total vertical force on beam. |
| fb,cal = | Calculated Bending Stress in Kg/cm2 due to Mx,cal. |
| fac,cal = | Calculated tensile stree in Kg/cm2 due to Pac,cal. |
| f,cal = | fb,cal + fac,cal < 1500 Kg/cm2 |

- Bottom Flange of Common Beams with bunker on both sides

| VARIANT | Tbf | Bbf | Mx,cal | Pat,cal | fb,calL | fb,calR | fa,cal | f,cal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

where,

| VARIANT = | No of variant case for which design is done. |
| --- | --- |
| Tbf = | Design thickness in MM of bottom flange for the above variant no. |
| Bbf = | Width for Tbf in MM. |
| Mx,cal = | Calculated Bending Moment(T-M) due to Normal pressure on web part of beam. |
| Pac,cal = | Calculated axial Compressive Force(T) due to Bending Moment due to Total vertical force on beam. |
| fb,cal = | Calculated Bending Stress in Kg/cm2 due to Mx,cal. |
| fac,cal = | Calculated tensile stree in Kg/cm2 due to Pac,cal. |
| Pac,cal = | Calculated axial Compressive Force(T) due to Bending Moment due to Total vertical force on beam. |
| fb,calR & |  |
| fb,calL = | Calculated Bending Stresses in Kg/cm2 due to Mx,cal on both the sheathing plates. |
| fac,cal = | Calculated tensile stress in Kg/cm2 due to Pac,cal. |

Though f,cal is not figured in table due to constraint of  space, following has been ensured.

- f,cal = fb,cal + fac,cal < 1500 Kg/cm2

- Web Vertical Stiffener

| VARIANT | Tvs | Bvs | Mn,cal | Pac,cal | fb,cal | fa,cal | f,cal |
| --- | --- | --- | --- | --- | --- | --- | --- |

where,

| VARIANT = | No of variant case for which design is done. |
| --- | --- |
| Tvs = | Design thickness in MM of stiffener plate for the above variant no. |
| Btf = | Width for Tvs in MM. |
| Mn,cal = | Calculated Bending Moment(T-M) due to pressure on stiffener. |
| Pac,cal = | Calculated axial Compressive Force(T) due to Bending Moment due to Total vertical force on beam. |
| fb,cal = | Calculated Bending Stress in Kg/cm2 due to Mn,cal. |
| fa,cal = | Calculated Compressive stress in Kg/cm2 due to Pac,cal. |
| f,cal = | fb,cal + fac,cal < 1500 Kg/cm2 |

***

8.0   Design Procedure:

- 8.1   Design of Sheathing Plates of Hopper

The  the wall of hopper is designed for all the  horizontal divisions between horizontal ribs. The ribs act as  support points for sheathing plate.The horizontal ribs are  assumed  non-translatory  and  equivalent  to  intermediates between restrained  support and hinged support. For calcutation  of  sheathing   thickness   we   assume   hinged   support(non-translatory).

The load acting on sheathing  (Fig.7) due to filling of dry  materials  are:

pvz = Vertical load on unit length of sheathing &

pnz = Normal pressure on unit length of sheathing

Bending stress(fb,cal) in sheathing due to normal  pressure

(fig.7)  is the main stress (is equal to 80 to 95%  of  the

total stress) and direct longitudinal tensile stree due  to

slope component of pvz and are computed as follows:

fb,cal = Mn,cal/Zs

pnz pvz Z a fig - 7

- Where,

| Mn,cal = | Span moment computed us a single span beam with hinged supports. |
| --- | --- |
| Mn,cal = | Pnz.l2/8 |
| l = | a/sina |
| a = | vertical spacing of ribs. |
| let A = | unit width of sheathing |
| ts = | Thickness of sheathing. |
| Zs = | A.ts2/6 |
| Normal pressure Pnz is determined as follows: - |  |
| pnz = | m.Y.z |
| m = | cos2 + K. sin2 |
| where, K is Co-efficient of lateral pressure (i.e, ratio of horizontal pressure to vertical pressure) and is equal to : |  |
| K = | tan2(45-/2) |
| In above formulae |  |
| Y = | volumetric wt. of filled material |
| z = | distance from upper surface of bunker to the centre of sheathing under consideration. |
|  | angle of inclination of plane of wall of sheathing. |
|  = | Angle of internal friction of filling material. |

Tensile  stress  rat,cal in sheathing  (fig 7 & 8)  due  to  vertical pressure pvz (i.e, Ptn,cal=slope component pvz) is small and is not more than 10 to 20 % of the total stess ie  sheathing.

pat,cal = Ptn,cal/ts

Ptn,cal = pvz.sina

pvz = pz/r  (on unit length of sheathing)

where,

pz = wt. of shaded part of filled material

r =  perimeter of sheathing wall to hopper at a distance z

from  upper surface of bunker (fig. 8)

The total stress in sheathing is kept less than 1500 kg/cm2

- f,cal = fat,cal + fb,cal  1500

fig-8

- 7.2    Design of horizontal ribs on hopper

- The  horizontal  ribs  consists  of  ISA sections  or

halfcut ISMB section or builtup sections or combination  of

these  sections,  as design is done for the  most  economic

section.  Ribs are jointed together so as to  form a  close

rigid contour.

- Design  of  ribs  (fig-9) is carried  out  due  to  normal

pressure pnz of filling material.

Force  arising in horizontal flange due to normal  pressure

pnz are

- (a) Bending moment, Mn,cal due to normal pressure of filled

material

- (b)  Tensile force, Ptn,cal due to force N

The stress rb,cal due to bending moment Mn,cal  is the main

stess in the horizontal ribs and is about 80 to 95% of  the

total stress in the ribs.

In  designing ribs due to bending moment , we  assume  that

the  rib  is  supported such  that  suports  are  partially

restrained and fixed.

w1 = Load on rib under consideration

w2 = Load on rib giving normal tensile force on the rib

under consideration.

w1 = pnz.(l1+l2)/2

w2 = pnz.(l1'+l2')/2

Mn,cal = w1.Lx2/10

Pat,cal = w2.Ly/2

where  l1  and l2 are  sloping  distances  between

consecutives  ribs, l1' and l2' are the  same  for

ribs on adjacent walls.

pnz l2 Z l1 fig – 9

Lx Ly w2 w1 w1 w2 Pat,cal Pat,cal Pat,cal Pat,cal fig-9

Bending Stresses:

Compressive  bending stress in sheathing  plate portion :

fb,calS = Mn,cal/Zsh

Tensile bending stress in rib portion :

fb,calR = Mn,cal/Zrib

Tensile axial force:

fat,cal = Pat,cal/A

Where  Zrib, Zshth are Section modulii of  rib  and sheathing  taken  together  about  axis  u-u   with   respect  to  the edge points of sheathing  and  rib  respectively,  and  A is the area of rib angle sheathig  plate.

In  determining Z angle and F, the area of plate  is  taken equal to 30.ts where ts= thickness of wall sheathing.

For design:

fb,calR <1500 Kg/cm2

fb,calS <1500 Kg/cm2

fat,cal <1500 Kg/cm2

and total stress

f,cal = fb,calR + fat,cal  <1500    Kg/cm2

- 7.3   Design of the bunker beam

- 7.3.1 General analysis of forces acting on bunker beam

In  designing  the bunker beam, the  following  assumptions have  been  made.Restraining of support  is  neglected multi-span beams are designed as single span.

- Load of design of bunker beam is shown in fig-11.

h2 h1 Fxt Fxb pnz fig – 11

- Total vertical load (on unit length of bunker beam)

w  =  wn + wv

Where,

| wn = | vertical load on beam due to self wt. of beamself wt. of flooring, live load on flooring and so on. |
| --- | --- |
| wv = | vertical load due to self wt. of filling of material. |
| wv = | Wt/r |
| Wt = | wt. of filling material in bunker |
| r = | perimeter of bunker beam. |

- Load of normal presure due to filling material, causing  horizontal  bending moment in top and bottom flange  of   beam are contributed from three parts marked a,b and  c  in fig-11 and as mentioned below:

- Normal  pressure  due to filled material  acting  on  vertical web of bunker beam.

- Normal  pressure due to filled  material  action  on sheathing of hopper along the boundary of bunker beam

- Normal  pressure due to filling material, acting  on  the  sheathing  of hopper along the  boundary,  from  bottom flange to upper horizontal stiffening rib. Horizontal forces Fxt and Fxb are calculated due  to the  above  forces as support reactions at  top  and  bottom flange assuming the web of beam is acting  as  beam simply supported over flanges.

- Horizontal  forces  Fxt  &  Fxb  from  component  of  vertical  load due to  self wt. of  filled  material

Refer(Fig-12).

- Fy = wv/tan

Fxt = Fy.h2/H

Fxb = Fy.h1/H

h2 h1 Fxt Fxb b Fy H fig – 12

Besides  the loads indicated above, on the top  and  bottom flanges  of  bunker  beams acts  normal  force,  which   is  determined  as reactions Fxt and Fxb of girders,  adjoining  the bunker beam under consideration .

- Fnt  = Ly/2 .Fxt - in top flange

Fnb = Ly/2. Fxb - in bottom flange

where,

Lx =Span of beam under consideration

Ly =Span of adjoining beam

Fxt  and  Fxb  =  Total  value  of  horizontal   foorce  calculated according to formulae for II[ (a); (b); (c); (d)]   for   the   beam  adjoining   the   beam   under consideration.

The  normal  force  Fnt &  Fnb  depending  on direction of H1 and H2 may be compressive or tensile.

The total load on the bunker beam may be summarised  as

follows :-

1.  Vertical loads :-w

2.  Horizontal loads :- Fxt

Fxb

3.  Normal force    :- Fnt

Fnb

In  designing  the bunker beam, it  is  necessary  that combination  of EFx and Fn shall be  most  unfavourable for  the  beam.To determine this, it  is  necessary  to calculate the sum of stresses due to different variants of filling of the bunker.

Following  six variant cases have been  considered  for  beam design.

Variant-1: bunker is fully filled.

Variant-2: bunker is full upto 1/3 rd of h1 from top.

Variant-3: bunker is full upto 2/3 rd of h1 from top.

Variant-4: bunker is full upto waist level i.e.  upto  h1 from top.

Variant-5: bunker is full upto bottom chord level of beam.

Variant-2: bunker is full upto the topmost rib level.

In  case of adjoining bunkers on both the sides of  bunker beam,  the total stress in the bunker beam  is  calculated  for 36 different variants for filling of bunker on  either side as below.

Variant-1 :  first  bunker  corresponds to  Variant-1  and  second bunker corresponds to Variant-1.

Variant-2 :  first  bunker  corresponds to  Variant-1  and  second  bunker corresponds to Variant-2.

Variant-3 :  first  bunker  corresponds to  Variant-1  and  second  bunker corresponds to Variant-3.

Variant-4 :  first  bunker  corresponds to  Variant-1 and  second bunker corresponds to Variant-4.

Variant-5 :  first  bunker  corresponds to  Variant-1 and  second bunker corresponds to Variant-5.

Variant-6 :  first  bunker  corresponds to  Variant-1 and  second bunker corresponds to Variant-6.

Variant-7 :  first  bunker  corresponds to  Variant-2 and  second  bunker corresponds to Variant-1.

Variant-8 :  first  bunker  corresponds to  Variant-2 and  second bunker corresponds to Variant-2.

Variant-9 :  first  bunker  corresponds to  Variant-2 and  second bunker corresponds to Variant-3.

Variant-10:  first  bunker  corresponds to  Variant-2 and  second  bunker corresponds to Variant-4.

Variant-11:  first  bunker  corresponds to  Variant-2 and  second  bunker corresponds to Variant-5.

Variant-12:  first  bunker  corresponds to  Variant-2 and  second bunker corresponds to Variant-6.

Variant-13:  first  bunker  corresponds to  Variant-3  and  second  bunker corresponds to Variant-1.

Variant-14:  first  bunker  corresponds to  Variant-3  and  second  bunker corresponds to Variant-2.

Variant-15:  first  bunker  corresponds to  Variant-3  and  second  bunker corresponds to Variant-3.

Variant-16:  first  bunker  corresponds to  Variant-3 and  second  bunker corresponds to Variant-4.

Variant-17:  first  bunker  corresponds to  Variant-3 and  second   bunker corresponds to Variant-5.

Variant-18:  first  bunker  corresponds to  Variant-3 and  second  bunker corresponds to Variant-6.

Variant-19:  first  bunker  corresponds to  Variant-4 and bunker corresponds to Variant-1

Variant-20:  first  bunker  corresponds to  Variant-4 and  second bunker corresponds to Variant-2.

Variant-21:  first  bunker  corresponds to  Variant-4 and  second  bunker corresponds to Variant-3.

Variant-22:  first  bunker  corresponds to  Variant-4 and  second bunker corresponds to Variant-4.

Variant-23:  first  bunker  corresponds to  Variant-4 and  second bunker corresponds to Variant-5.

Variant-24:  first  bunker  corresponds to  Variant-4 and  second bunker corresponds to Variant-6.

Variant-25:  first  bunker  corresponds to  Variant-5  and  second bunker corresponds to Variant-1.

Variant-26:  first  bunker  corresponds to  Variant-5  and  second  bunker corresponds to Variant-2.

Variant-27:  first  bunker  corresponds to  Variant-5  and  second

bunker corresponds to Variant-3.

Variant-28:  first  bunker  corresponds to  Variant-5 and  second

bunker corresponds to Variant-4.

Variant-29:  first  bunker  corresponds to  Variant-5 and  second

bunker corresponds to Variant-5.

Variant-30:  first  bunker  corresponds to  Variant-5 and  second

bunker corresponds to Variant-6.

Variant-31:  first  bunker  corresponds to  Variant-6 and  second

bunker corresponds to Variant-1.

Variant-32:  first  bunker  corresponds to  Variant-6 and  second

bunker corresponds to Variant-2.

Variant-33:  first  bunker  corresponds to  Variant-6 and  second

bunker corresponds to Variant-3.

Variant-34:  first  bunker  corresponds to  Variant-6 and  second

bunker corresponds to Variant-4.

Variant-35:  first  bunker  corresponds to  Variant-6 and  second

bunker corresponds to Variant-5.

Variant-36:  first  bunker  corresponds to  Variant-6 and  second

bunker corresponds to Variant-6.

where  first and second bunkers are  adjacent  bunker

connected to either side of the beam under consideration.

- 7.3.2 Design of Web and Web Vertical Siffeners:

In  practical designs, the stability of the web of  bunker

beam  is not checked and therefore in  design,the  section

comprises  only of top flange and bottom flange.  In  such

case, the design scheme of bunker beam has been  converted

into a truss (Fig 13)"conditionally".

The verticals of bunker beam are considered as  posts

Sv

a

# Equivalent Truss Diagram for Web Design

H

8@SV =Lx

fig : 13

of truss, and tension diagonals are with part width of web of  beam -'a'(shaded in fig 16). The determination of  the value of 'a' is rather complicated procedure as the web is under  complex  stress-condition. Therefore  in  practical design  of  the  bunkers,  the  value  'a'  is  determined depending  on  the  height  of  the   bunker  beam  'H' and  distance between vertial ribs 'sv'and is equal to 30.tw to 60.tw  where  tw = thickness of web of bunker  beam.

Here

a = 30.tw has been considered.

The  web of bunker beam is designed  considering  the combined effect of force Pn,cal due to normal pressure due to  filled  material  and force  Pat,cal  in  the  tension diagonal in the equivalent truss due to vertical load (Fig : 13). The total stress in the web shall be :-

- f,cal  = fa,cal + fb,cal < 1500 Kg/cm2

The axial and bending stresses are calculted as below:

fa,cal = Pat,cal/Ae < 1500 Kg/cm2

fb,cal = 0.6.Pn,cal.(sv/tw) < 1500 Kg/cm2

Where,

Pat,cal = Ew.Lx/(2.sinB)

Pn,cal = normal pressure due to filled material.

Ae = area of tension diagonal in the  equivalent  truss

= 30.tw

sv = distance between vertical ribs.

tw = thickness of web of bunker beam.

B = arctan(h/sv)

h = total depth of bunker beam.

The  vertical  stiffeners  receive  load  from  normal pressure due to filling of material from the web of bunker beam and transmit the load to the top and bottom flanges. Besides  the load due to normal pressure  as  mentioned below  due to filling material the vertical ribs  acts  as posts   of  the  equivalent  frame and is also designed for transverse force due to total vertical loads.

- Normal pressure due to filled material, acting on  the  vertical web of bunker beam.

- Normal  pressure due to filling on  the  sheathing  of funnel along the boundary of the bunker beam.

- The  vertical  component of load due to  self  wt.  of  filled material

- The loads calculated according to the three cases cause  bending moment, (Mn,cal) in the ribs and are   designed as  a  single  span girder with hinged  supports  at  ends (ie.at flanges of bunker beam).

Besides   bending    moment(Mn,cal)  in  the   ribs    acts transverse   force as axial compression(Pac,acl)   due   to total vertical loads.

- Pac,cal =w.Lx/2

where

w = Total vertical udl on bunker beam.

Lx = span of  beam under consideration.

The verical ribs have been made from   plates with  their outstanding projecion as  12.tv on either  side of  web  an  total 40.tw length of web  is  assemed  to  be contributing to the sectional  properties of stiffener. (where  tv  is the thickeness of stiffener plate  as  shown below.)

Stresses are calculated as below:

Bending stress:

fb,cal = Mn,cal/Zsv  1500 Kg/cm2

Axial Stress:

fa,cal = Pac,cal/Asv  fac

Comined Stress :

f,cal = fb,cal + fa,cal  1500 Kg/cm2

where,

Zsv = tv.(24.tv)2/6

=  Section  modulus of the stiffener  about  web-

plane.

Asv = (24.tv).tv +(40.tw).tw

rac = Allowable axial compressive stress calculted

as  per  function of Slenderness ratio  of  web

stiffeners which has been kept as < 180.

In  SOFTDOSS, first web plate is designed then  its  design  thickness  is used in sectional properties of  top  flange and  bottom flange design. The web vertical stiffeners  are designed  at the end after design of flanges,  however  the design has been discussed earlier in this manual.

Design of Top and Bottom Flanges :

After  completion  of design of web for  variant  cases, the final  design  thickness  of  web  is   used  for  strength calculation of flanges.  15.tw length of the web is assumed to  be  acting  with  the  flanges  contributing  to  their  sectional  properties.  For bottom flange  30.tsh  part  of sheathing plate connected to the edge of flange  is assumed  to  be  contributing  to  the  sectional  properties.   The  flanges  have been designed as simply supported  beams the horizontal forces Fxb/Fxt. The bottom flange with the edge stiffener acts as beam with flange of sheathing  plate  and  edge  stiffener. For common beam the  sheathing  plate acts  as  flanges  of beam. The  edge  stiffener  has connected  at 15.Tbf from centre of beam so that  the  edge stiffener  can contribute to resist the axial force due  to  total  vertical  udl on bunker beam.  The  contribution  of   sheathing  plate to resist axial force has been  considered only wheb they are connected to the bottom flanges at  less than   15.Tbf  from  centre  of  beam.  The  stresses   are  calculated as follows:

Bending Stress:

fb,cal = Mx,cal/Z  1500 Kg/cm2

fa,cal = Pa,cal/Ae 1500 kg/cm2

where,

Pa,cal = Pat,cal = Pac,cal = (w.Lx2/8)/H

Mx,cal = (Fx).Lx2/8

(Fx  is EFxb for bottom flange and Fxt  for  top  flange.)

Mx,cal  is calculated bending moment on the flanges due  to Fx.  and Pac,cal is axial force due to total udl  on  beam transferred through bending of the beam.

Effect Of Eccentricity of Opening at Bottom:

The  above case is for asymmetrical bunkers. The  load  pvz calculated for design of sheathing plates of hopper portion and  the  Ew  for design of bunker beam is  modified  by  a factor Fec, which is calculated as follows:

- Fec = Fecx.Fecy and

Where,

- Fecx = 1  6.Yc(lxz + lyz)/(lyz(lyz +3.lxz))

- Fecy = 1  6.Xc(lxz + lyz)/(lxz(lxz +3.lyz))

Xc,Yc = absolute values of coordinates of  centre  of gravity of the volume of bunker.

lxz, lyz = are hoppers dimension in plan at depth z from top.

'+'  sign has been taken for wall nearest to the centre  of

gravity.  '-' sign is for the wall which is at  a  distance

more  than  lxz/2 or lyz/2 from centre of  gravity  of  the

volume of bunker.

***

9.0    Error Handler:

- Error no = 4 : Out of data.

Solution:

------------------------------

| This  Error  not   encountered

| during   development    stage.

| However the case may  reported

| to the SOFTDOSS Designer.

- Error no = 5 : Illegal function call.

| SOFTDOSS.DRAW   requires   VGA

| card for its graphics display.

| Without  VGA the  draw  option

| can not be invoked.

- Error no = 6 : Overflow.

| Check  input data. Wall  slope

| may be equal/tending to 90*.

- Error no = 7 : Out memomry.

| Switch   Off  the  PC(or   use

| (Ctrl+Alt+Del)  Key) to  clear

| computer  memory. This may  be

| due  to some  Memory  Resident

| programs  already  loaded   in

| computer memory.

- Error no = 24: Device time out.

| Printer may not available  for

| the printing. Set the  printer

| ON, and ensure that printer is

| not   out  of   paper   before

| selection SOFTDOSS.PRINT.

- Error no = 25: Device fault.

- Also refer Error no = 24.

- Error no = 27: Out of paper.

| Put paper and set the printer

| at top of the paper.

| Also refer Error no = 24.

- Error no = 51: Internal Error.

- Refer error no 4.

- Error no = 52: Bad File name or no.

| Enter  valid  file name  of  8

| characters without extension.

| It should not contain dots  or

| commas.

- Error no = 53: File not found.

| There may be following cases.

|

| i)  Enter  file  name   before

|     selecting   any   SOFTDOSS

|     option.   Without  it   no

|     option  can be  invoked  /

|     selected.

|

| ii) If any  SOFTDOSS*.exe  or

|     SOFTDOSS.TBL is not  found

|     Refer  setup  to   install

|     SOFTDOSS correctly.

|iii) SOFTDOSS.EDIT  &

|     SOFTDOSS.DRAW requires

|     *.EDT file to proceed.

|      Use  SOFTDOSS.RUN  option

|     to  create  it  and   then

|     select these options.

|iv)  SOFTDOSS.VIEW       &

|     SOFTDOSS.PRINT   requires

|     *.BNK  files  to  proceed.

|     Use SOFTDOSS.RUN option to

|     create it and then  select

|     these options.

- Error no = 54: Bad File mode.

| your file name may be matching

| with any existing file.

| Change  your filename  to  any

| other valid name.

- Error no = 55: File already open.

- Not encoutered.

- Error no = 57: Device I/O error

| Where might be bad sectors  in

| your floppy/disk. Replace your

| floppy.

- Error no = 58: File already exists.

| You should not enter file name

| similar to the Reserved Words of

| SOFTDOSS.

| DATA

| TEMPO

| ANALYSIS

| SOFTDOSS

Error no = 59: Bad record length.

Refer Error no = 4

- Error no = 61: Disk full.

| SOFTDOSS  doesnot find  enough

| space  to  store  results   on

| specified  disk.  Create  disk

| space and reuse SOFTDOSS.RUN.

- Error no = 62: Input past end of file.

| This may happen during reading

| *.EDT files. Use SOFTDOSS.EDIT

| option to rectify the  error,

| if you can. Otherwise re-run  to

| create new EDT file.

- Error no = 63: Bad Record no.

- Refer Error no = 4

- Error no = 64: Bad file name.

- Refer Error no = 52

- Error no = 67: Too many files.

- Refer Error no = 4

- Error no = 68: Device not availale.

| Specified  drive name may  not

| exist. Use valid drive name.

- Error no = 71: Disk not ready.

| Specified  drive  may  not  be

| properly  placed with  floppy.

| Disk may not be formatted. Use

| formatted floppy.

- Error no = 74: Rename across disk.

- Refer error no 76.

Error no = 75: Path file access error.

Refer error no 76.

- Error no = 76: Path not found.

| Incorrect path entered.  Check

| path   name.   The   specified

| directory in the path may  not

| exist.   First   create   this

| directory  and then enter  the

| path.

********

- For any clarifiction regarding this software, please

contact:-

- Binay Kumar Pandey

Design Engineer

Structural Section

MECON (I) LTD.

Ranchi-834002

Phone - (0651)2480229(O)

(0651)2503730(R)

***