WELL: BR-001

X:   184256

Y:   154086

ELEV:     38.7

DR.DATES: Apr. 1974

DR.METH: Percussion

SWL:     8.18

VSCALE:      600.0

HSCALE:       50.0

LITH:

20.000 CLAY CLAY, brown

36.000 CLAY CLAY, yellow

95.000 CLAY CLAY, brown and MARL

110.000 MARL

185.000 LIMEH LIMESTONE, hard\(soft in upper 18 ft)

204.000 LIMEH LIMESTONE, hard

212.000 SAND SAND pockets

235.000 LIMEH

305.000 LIMEH LIMESTONE, hard \with sand

330.000 LIMEH

345.000 MARL

350.000 LIMEH LIMESTONE, hard with clay

HOLE:

170.000       24.000

350.000       16.000

CASING:

170.000       24.000

SCREEN:

100.000      170.000

ANNULUS:

1.000 CEMENT

WELL: BR-002

X:   180354

Y:   168138

ELEV:    200.0

DR.DATES: Apr. 1970

DR.METH: Percussion

SWL:   135.33

HSCALE:       50.0

LITH:

20.000 CLAY CLAY, brown

35.000 CLAY-LST Clay, brown and Limestone

55.000 LIMEH LIMESTONE, hard,white

70.000 LIME LIMESTONE, white, \softer than above

175.000 LIMEH LIMESTONE, hard,white

192.000 CAVITY CAVITY

245.000 LIMEH LIMESTONE, hard,white \with clay joints\water struck at 210 ft.

280.000 LIME-SD LIMESTONE and SAND

HOLE:

175.000       10.000

280.000        8.000

CASING:

30.000       10.000

192.000        8.000

SCREEN:

122.000      192.000

WELL: BR-003

X:   182396

Y:   152075

ELEV:     35.1

DR.DATES: Aug. 1958

DR.METH: Percussion

SWL:     9.14

VSCALE:      200.0

HSCALE:       20.0

LITH:

19.000 CLAY BAUXITE

50.000 MARL MARL and loose stones

96.000 LIME LIMESTONE\water struck at 93 ft.

120.000 LIMEH LIMESTONE, white to buff\ very hard, \few fossils

HOLE:

120.000        6.000

CASING:

112.200        6.000

SCREEN:

92.200      112.200

WELL: BR-004

X:   174654

Y:   164571

ELEV:     35.0

DR.DATES: Nov. 1985

DR.METH: Percussion

SWL:    19.20

VSCALE:      350.0

HSCALE:       50.0

LITH:

5.000 SOIL SOIL, clayey, with lst chips

50.000 LIMES LIMESTONE, soft,white,\with minor clay \impurities

150.000 LIMEH LIMESTONE, hard

200.000 LIMEH LIMESTONE, very hard, \white recrystallized

HOLE:

70.000       24.000

200.000       20.000

CASING:

200.000       16.000

SCREEN:

50.000      200.000

WELL: BR-005

X:   172155

Y:   158597

ELEV:     15.2

DR.DATES: Dec. 1991

DR.METH: Percussion

SWL:    11.58

VSCALE:      400.0

HSCALE:       50.0

LITH:

5.000 SOIL Red clay, top soil

20.000 CLAY CLAY with \limestone at bottom

25.000 CONGLO Limestone pebbles, loose

30.000 CLAY-LST CLAY, with lst lumps

40.000 LIME-SD LIMESTONE, loose, with sand

55.000 LIME-SD LIMESTONE and SAND, \red clay stains

65.000 LIMEH LIMESTONE, hard, marly

115.000 LIME LIMESTONE, chalky at top\partly recrystallized\at base-bioclastic

140.000 LIME-SD LIMESTONE, white, sand

150.000 LIME LIMESTONE, moderately hard

160.000 LIME-SD LIMESTONE, white, sand

170.000 MISSING

175.000 LIMEH LIMESTONE, white,hard

195.000 MISSING

210.000 LIME LIMESTONE, white, with\foraminifera

215.000 LIME-SD LIMESTONE and SAND

HOLE:

215.000       16.000

CASING:

215.000       16.000

SCREEN:

45.000      215.000

WELL: BR-006

X:   176179

Y:   151955

ELEV:    101.1

DR.DATES: July 1966

DR.METH: Joy Core

SWL:     3.85

VSCALE:      650.0

HSCALE:       10.0

LITH:

65.000 LIME LIMESTONE,hard,partly\recrystallized. Rich in\solution vugs through\whole core.

70.000 LIMES

335.000 LIME LIMESTONE,hard,partly\recrystallized. Rich in\solution vugs through\whole core.

365.000 LIME L'STONE, softer than above.\Porous,less vugs.

HOLE:

365.000        2.500

CASING:

362.000        1.500

SCREEN:

311.000      331.000

341.000      362.000

WELL: BR-007

X:   169563

Y:   157588

ELEV:     36.1

ELEVM:     36.1

DR.DATES: June 1966

DR.METH: Joy Core

SWL:     1.69

VSCALE:      280.0

HSCALE:       10.0

LITH:

15.000 LIME LIMESTONE, with some vugs

35.000 LIME LIMESTONE, \largely recrystallized\only 1 ft recovery

65.000 LIME LIMESTONE, poor recovery\vug permeability\only 2 ft recovered

105.000 LIME LIMESTONE, WELL developed\skeletal vug porosity

135.000 LIME LIMESTONE, patchily \recrystallized\with coral debris

155.000 LIME LIMESTONE, poor recovery,\only 6"

HOLE:

155.000        2.500

CASING:

149.500        1.500

SCREEN:

109.500      149.500

WELL: BR-008

X:   160207

Y:   161798

ELEV:    100.0

DR.DATES: Mar. 1990

DR.METH: Percussion

SWL:    27.43

VSCALE:      700.0

HSCALE:       50.0

LITH:

35.000 CLAY

120.000 LIMEH

175.000 LIME LIMESTONE, hard and soft

315.000 LIMEH LIMESTONE hard,\water struck at 235'

400.000 LIME-SD LIMESTONE, hard \with sand

HOLE:

400.000       20.000

CASING:

250.000       14.000

400.000       12.000

SCREEN:

230.000      400.000

WELL: BR-009

X:   159384

Y:   158933

ELEV:     40.0

DR.DATES: May 1954

DR.METH: Cable tool

SWL:    37.55

HSCALE:       50.0

LITH:

102.000 LIMEH LIMESTONE, hard,\with marly bands

181.000 CHERT FLINTSTONE, \with marly bands

187.000 MARL MARL with stones

193.000 CHERTT FLINTSTONE with marl

250.000 LIMEH LIMESTONE, hard,\with marly bands

HOLE:

250.000       12.000

CASING:

99.000       12.000

SCREEN:

70.000       99.000

WELL: BR-010

X:   160938

Y:   151953

ELEV:      0.8

DR.DATES: May 1966

DR.METH: Joy Core

SWL:    -0.12

VSCALE:      280.0

HSCALE:       10.0

LITH:

12.000 SAND SAND, grey

30.000 LOAM LOAM, clay, sandy

35.000 SAND SAND, yellow

45.000 CLAY CLAY, yellow-red

65.000 LIMEH LIMESTONE, hard, with\solution vugs

75.000 SANDF SAND, fine,white-yellowish

95.000 SANDF SAND, fine, grey

160.000 LIMEH LIMESTONE, hard,grey, \carbonaceous solution vugs \not common, poor recovery

HOLE:

160.000        2.500

CASING:

20.000        1.500

SCREEN:

10.000       20.000

WELL: BR-011

X:   153562

Y:   156464

ELEV:     30.0

DR.DATES: 1963

DR.METH: Cable tool

SWL:    18.71

VSCALE:      180.0

HSCALE:       50.0

LITH:

1.000 CLAY CLAY, black

3.500 CLAY CLAY, red

14.000 MARL MARL and stone

20.000 MARL

34.000 MARL MARL, hard

48.000 LIMEH LIMESTONE, white, hard

51.000 LIME-SD L'STONE,white,hard & sand

75.000 LIMEH LIMESTONE, white, hard

93.000 MISSING

HOLE:

93.000       20.000

CASING:

8.000       20.000

76.000       18.000

SCREEN:

36.000       76.000

WELL: BR-012

X:   153989

Y:   166796

ELEV:    365.8

DR.DATES: May 1968

DR.METH: Percussion

SWL:   247.80

VSCALE:      900.0

HSCALE:       50.0

LITH:

5.000 CLAY CLAY, red

15.000 CLAY CLAY, brown

27.000 CLAY CLAY, red

50.000 LIME LIMESTONE, yellow

150.000 LIME LIMESTONE, white

170.000 LIME-CL LIMESTONE, white & clay

200.000 LIMEH LIMESTONE, white,hard

230.000 LIME L'STONE,hard & soft, yellow

250.000 LIME

330.000 LIME LIMESTONE, white

415.000 LIME LIMESTONE, yellow

505.000 LIME LIMESTONE, white,hard

HOLE:

505.000       10.000

WELL: BR-013

X:   174045

Y:   143053

ELEV:     26.2

DR.DATES: Mar. 1965

DR.METH: Cable tool

SWL:     1.12

VSCALE:      350.0

HSCALE:       50.0

LITH:

8.000 SOIL Earth, red

14.000 CLAY CLAY, brown

22.000 LIME-CL CLAY,brown & L'stone,white

35.000 LIME

38.000 MARL

85.000 LIMES

101.000 LIMEH

120.000 LIME-CL

122.000 MARL MARL with CLAY

177.000 MARL MARL, with some clay\and stone

200.000 MARL

HOLE:

200.000       20.000

CASING:

152.000       20.000

SCREEN:

80.000      152.000

WELL: BR-014

X:   185383

Y:   141254

ELEV:    297.5

DR.DATES: Apr. 1987

DR.METH: Percussion

SWL:    38.40

VSCALE:     2000.0

HSCALE:       50.0

LITH:

60.000 CLAY Clay,red,bauxite,l'st frag.

110.000 LIMEH LIMESTONE fragments,hard

410.000 LIME LIMESTONE,white to off \white, fine grained, marly\indurated.\High primary porosity

710.000 LIME LIMESTONE,white, moder. \indurated marly, with\higher crystalline\coarse grain fraction

820.000 LIME LIMESTONE,fine grain,\powdery,brownish \poorly indurated,marly

1008.000 LIME LIMESTONE,white sandy\to powdery marly

HOLE:

1008.000       12.000

CASING:

150.000       12.000

SCREEN:

149.900      150.000

WELL: BR-015

X:   171911

Y:   147076

ELEV:     15.0

DR.DATES: May 1974

DR.METH: Percussion

SWL:     8.90

VSCALE:      260.0

HSCALE:       50.0

LITH:

6.000 CLAY CLAY, brown

15.000 CLAY CLAY, yellow

24.000 MARL MARL and CLAY

28.000 LIMES

52.000 LIMEH LIMESTONE,very hard,\white

55.000 MARL MARL, loose, hard

58.000 ROCK2 Hard rock - not limestone!

62.000 LIME-CL LIMESTONE with clay

68.000 LIME LIMESTONE, white

98.000 LIMEH LIMESTONE,very hard,\white

117.000 CLAY CLAY,white,sticky \with limestone

131.000 CWIOS CLAY,white and SAND

136.000 LIMEH LIMESTON, white,hard

145.000 LIME-SD LIMESTONE,hard with sand

151.000 CLAY CLAY,with limestone gravel

HOLE:

42.000       18.000

151.000       16.000

CASING:

42.000       18.000

151.000       16.000

SCREEN:

104.000      151.000

WELL: BR-016

X:   172704

Y:   145339

ELEV:     22.7

ELEVM:     22.7

DR.DATES: Apr. 1965

DR.METH: Joy Core

SWL:     1.28

VSCALE:      550.0

HSCALE:       20.0

LITH:

60.000 LIME LIMESTONE,white,recrystal.\molluscan moulds,\skeletal and vug porosity

100.000 LIME LIMESTONE,white patchily\recrystallized

150.000 LIME LIMESTONE,white,soft,\recryst. patches

200.000 LIME LIMESTONE,light greyish,with\molluscs and foraminifera

250.000 LIME LIMESTONE,nodular,\recryst. patches

300.000 LIME LIMESTONE,light greyish

HOLE:

300.000        2.500

CASING:

130.000        1.000

SCREEN:

120.000      130.000

WELL: BR-017

X:   171515

Y:   144729

ELEV:     18.2

ELEVM:     18.2

DR.DATES: May 1965

DR.METH: Joy Core

SWL:     0.97

VSCALE:      270.0

HSCALE:       10.0

LITH:

4.000 DEBRIS BAUXITE

40.000 LIMEH LIMESTONE, hard,\micro-grained\white, molluscas

80.000 LIME LIMESTONE, dense,recryst.,\poor solution vugs

120.000 LIME LIMESTONE, recryst. with\a few specimens of\Amphorisorus

150.000 MISSING No recovery

HOLE:

150.000        2.000

CASING:

150.000        1.250

SCREEN:

130.000      150.000

WELL: BR-018

X:   167553

Y:   151343

ELEV:      5.0

DR.DATES: May 1967

DR.METH: Joy Core

SWL:     4.08

VSCALE:      580.0

HSCALE:       20.0

LITH:

5.000 CLAY CLAY, dark brown

100.000 LIME LIMESTONE, buff grey,\medium grained\partly recrystallized

150.000 LIME LIMESTONE, grey, \much coral debris

200.000 LIME LIMESTONE, passing into\fine marly limestone

250.000 LIME LIMESTONE, buff carbonate\sand

320.000 LIME LIMESTONE, medium grained,\compact but not WELL\cemented

HOLE:

320.000        2.500

WELL: BR-019

X:   171210

Y:   145643

ELEV:     15.5

DR.DATES: May 1975

DR.METH: Percussion

SWL:     1.15

VSCALE:      300.0

HSCALE:       50.0

LITH:

58.000 MARL

95.000 LIME

120.000 LIMEH LIMESTONE,hard with\sand pockets

160.000 LIMES

HOLE:

6.000       24.000

160.000       20.000

CASING:

6.000       24.000

158.000       20.000

SCREEN:

59.000      158.000

WELL: BR-020

X:   171180

Y:   141742

ELEV:     22.1

ELEVM:     22.1

DR.DATES: Jan. 1967

DR.METH: Rotary

SWL:     1.70

VSCALE:      850.0

HSCALE:       50.0

LITH:

16.000 CLAY CLAY, yellowish, bauxite

150.000 LIMEH LIMESTONE, hard,\light grey\partly recrystallized,with\moluscan & coral remains\Solution vugs abundant.

503.000 LIME LIMESTONE, \greyish white,\recrystallized nodules \in soft whitish marl.\Fossiliferous horizon.

HOLE:

141.000       20.000

200.000       12.000

503.000        5.600

CASING:

141.000       16.000

SCREEN:

92.000      116.000

WELL: BR-021

X:   167705

Y:   144211

ELEV:     44.2

ELEVM:     44.2

DR.DATES: Mar. 1967

DR.METH: Rotary

SWL:     0.99

VSCALE:     1000.0

HSCALE:       50.0

LITH:

240.000 LIME LIMESTONE, white,\partly recrystallized

550.000 LIME LIMESTONE, greyish, soft\with recrystallized patches.\Fossiliferous horizon.

600.000 LIME LIMESTONE, grey, poorly\cemented, no recryst.

HOLE:

254.000       15.000

408.000        8.600

CASING:

234.000       10.750

WELL: BR-022

X:   169259

Y:   147655

ELEV:      7.5

ELEVM:      7.5

DR.DATES: June 1967

DR.METH: Rotary

SWL:    -1.33

VSCALE:     1800.0

HSCALE:       50.0

LITH:

12.000 CLAY CLAY,brown with lst chips

50.000 LIME LIMESTONE,white,chalky

130.000 LIME LIMESTONE, buff,soft

550.000 LIME LIMESTONE, grey,marly,\compact,\partly recrystallized.

1010.000 LIME LIMESTONE, grey,soft,\loosely cemented,\fossiliferous\Also buff, finegrained,\compact, impermeable.

WELL: BR-023

X:   172917

Y:   138602

ELEV:     94.4

ELEVM:     94.4

DR.DATES: Feb. 1967

DR.METH: Rotary

SWL:     0.94

VSCALE:     1800.0

HSCALE:       50.0

LITH:

5.000 CLAY Terra Rossa

310.000 LIME LIMESTONE,white,dense,\compact,recrystallized,\high skeletal and vug\permeability

680.000 MISSING NO RECOVERY

690.000 LIME LIMESTONE,white,dense,recryst.

1015.000 LIME LIMESTONE\greyish,compact\partly recrystallized\marked softer and less\recrystallized than in\interval 5-310 ft.

HOLE:

366.000       18.500

673.000        8.600

1015.000        5.600

CASING:

366.000       13.400

673.000        6.600

SCREEN:

366.000      673.000

WELL: BR-024

X:   170326

Y:   143327

ELEV:     22.5

DR.DATES: Apr. 1965

DR.METH: Percussion

SWL:     1.44

VSCALE:      280.0

HSCALE:       50.0

LITH:

4.500 SOIL EARTH, red

7.000 ROCK2 ROCK

66.500 MARL MARL,hard;last foot soft

80.000 LIMEH

97.000 LIME-CL LIMESTONE,soft with clay

103.000 LIMEH LIMESTONE,very hard

130.000 LIME-CL LIMESTONE,soft with clay

142.000 CLAY

146.000 LIMEH

151.000 MARL MARL with CLAY

HOLE:

151.000       20.000

CASING:

142.000       20.000

SCREEN:

70.000      142.000

WELL: BR-025

X:   166547

Y:   148082

ELEV:     15.1

ELEVM:     15.1

DR.DATES: May 1965

DR.METH: Joy Core

SWL:     2.68

VSCALE:      350.0

HSCALE:       10.0

LITH:

90.000 LIME LIMESTONE,dense,compact,\recrystallized

140.000 LIME LIMESTONE,buff-grey,\poorly consolidated,\high porosity

200.000 LIME LIMESTONE,same as above;\limestone is more clastic\with less cementing material

HOLE:

200.000        2.500

CASING:

200.000        1.000

SCREEN:

180.000      200.000

WELL: BR-026

X:   167370

Y:   144241

ELEV:     50.0

DR.DATES: Apr. 1965

DR.METH: Joy Core

SWL:     7.86

HSCALE:       20.0

LITH:

30.000 LIME LIMESTONE,firm,white,\large solution cavities

70.000 LIMES LIMESTONE,soft,very porous\friable

90.000 LIME LIMESTONE,uncrystallized\porous

200.000 LIME LIMESTONE,porous,white,low \density,solution cavities

220.000 LIME LIMESTONE,cream-coloured,\med-coarse,poorly sorted

240.000 LIME LIMESTONE,soft,chalky,vugs

HOLE:

240.000        2.500

CASING:

240.000        1.000

SCREEN:

219.000      240.000

WELL: BR-027

X:   167126

Y:   141681

ELEV:     61.2

DR.DATES: Apr. 1967

DR.METH: Joy Core

SWL:     0.77

VSCALE:      650.0

HSCALE:       20.0

LITH:

10.000 CLAY CLAY,yellowish-red,white

120.000 LIMEH LIMESTONE,hard,white,\compact,recrystallized\with many solution vugs

130.000 LIMES LIMESTONE,soft,white,sandy\medium grained,no fossils

335.000 LIME LIMESTONE,white,compact,\recrystallized. Becoming\softer with depth\and less cemented.

HOLE:

335.000        2.500

CASING:

327.000        1.250

SCREEN:

291.000      327.000

WELL: BR-028

X:   164200

Y:   149758

ELEV:      1.6

DR.DATES: June 1967

DR.METH: Joy Core

SWL:     2.13

VSCALE:      550.0

HSCALE:       10.0

LITH:

10.000 CLAY CLAY

20.000 SANDM SAND,med. grained,buff

50.000 CLAY CLAY, tan, tight, calc.

65.000 LIME-SD LIMESTONE,grey and SAND

160.000 LIME LIMESTONE, grey,hard,\vugular permeability\argillaceous

180.000 SANDF SAND,grey,fine grained

210.000 SANDC SAND,coarse,grey,with\consolidated limestone

230.000 SANDF SAND,grey,fine-medium

270.000 LIME LIMESTONE,grey,high \silt and clay fraction

300.000 SANDF SAND,fine,calcareous

HOLE:

188.000        2.500

300.000        1.500

CASING:

200.000        1.500

SCREEN:

15.000      200.000

WELL: BR-029

X:   185383

Y:   146649

ELEV:    169.1

DR.DATES: 25 Jan 1967

DR.METH:

SWL:    12.50

VSCALE:     1200.0

HSCALE:       50.0

LITH:

5.000 CLAY BAUXITE

20.000 LIME-CL BUAXITE and white limestone

50.000 LIMES LIMESTONE, soft,white

170.000 LIME LIMESTONE,med. hard\white

235.000 LIMEH LIMESTONE, hard,white

258.000 LIME LIMESTONE,med. hard\white

277.000 LIMEH LIMESTONE, hard,white

308.000 LIMEH LIMESTONE,hard,fissured

612.000 LIMEH LIMESTONE, hard to \very hard\white

638.000 LIMEH LIMESTONE, hard,fissured\pale buff

674.000 LIMES LIMESTONE,soft,porous

HOLE:

308.000       24.000

600.000       21.000

638.000       16.000

674.000       12.000

CASING:

638.000       16.000

643.000       12.000

SCREEN:

543.000      643.000

WELL: BR-030

X:   183378

Y:   151404

ELEV:     64.8

ELEVM:     64.8

DR.DATES: 1970

DR.METH: Rotary

VSCALE:      650.0

HSCALE:       50.0

LITH:

50.000 LIME LIMESTONE,medium hard\white

70.000 LIME LIMESTONE,med-hard,\white to grey

100.000 LIME LIMESTONE, clay at 87-90

120.000 LIME LIMESTONE,medium hard,\grey to white

145.000 LIME LIMESTONE,hard,greyish

209.000 LIME LIMESTONE,medium hard,\white to grey

220.000 LIME LIMESTONE,very hard,grey-yellow

277.000 LIME LIMESTONE,tough,greyish

288.000 LIME LIMESTONE,hard,grey to yellow

400.000 LIME LIMESTONE,medium hard\grey

WELL: BR-031

X:   189346

Y:   137962

ELEV:    104.9

DR.DATES: Dec. 1965

DR.METH: Rotary

SWL:     1.83

VSCALE:      700.0

HSCALE:       50.0

LITH:

20.000 CLAY CLAY,red

34.000 CLAY CLAY,red and STONE

44.000 LIME-CL LIMESTONE with clay pockets

104.000 LIMEH

176.000 MARL MARL, with limestone and\white clay

254.000 LIME MARL and LIMESTONE

287.000 LIME

316.000 MARL

326.000 CLAY MARL and CLAY

340.000 LIME

353.000 LIME-SD LIMESTONE,hard and sand

400.000 LIMEH

HOLE:

289.000       24.000

400.000       20.000

CASING:

189.000       24.000

310.000       20.000

374.000       20.000

400.000       16.000

SCREEN:

360.000      400.000

WELL: BR-036

X:   183585

Y:   153934

ELEV:     31.2

DR.DATES: 19 Dec. 1974

DR.METH: Percussion

SWL:     7.44

VSCALE:      650.0

HSCALE:       50.0

LITH:

22.000 CLAY

30.000 CWG CLAY with GRAVEL

87.000 MARL

100.000 LIMES LIMESTONE,soft,white

105.000 LIME-SD LIMESTONE,white with sand

202.000 LIMEH LIMESTONE,hard,white

206.000 CWIOS CLAY with SAND

277.000 LIMEH LIMESTONE,hard to very\ hard, white

350.000 LIMES LIMESTONE,soft,white

HOLE:

150.000       24.000

350.000       16.000

CASING:

150.000       24.000

SCREEN:

90.000      150.000

ANNULUS:

40.000 CEMENT

WELL: BR-037

X:   184225

Y:   153812

ELEV:     40.0

DR.DATES: 26 Apr. 1975

DR.METH: Percussion

SWL:     7.14

VSCALE:      650.0

HSCALE:      100.0

LITH:

350.000 LIME LIMESTONE\3 permeable zones:\  111-115 ft.\  135-142 ft.\  150-176 ft.

HOLE:

176.000       24.000

350.000       16.000

CASING:

176.000       24.000

SCREEN:

116.000      176.000

WELL: BR-043

X:   175142

Y:   155854

DR.DATES: Apr. 1961

DR.METH: Percussion

SWL:    13.15

VSCALE:      350.0

HSCALE:       50.0

LITH:

15.000 CLAY Red earth (bauxite?)

20.000 MARL CLAY,red with MARL

35.000 LIME LIMESTONE, white

37.000 MARL MARL, loose

134.000 LIME LIMESTONE, white\water struck at 57 ft.

144.000 MARL

195.000 LIMEH LIMESTONE,hard,white

200.000 LIME-SD LIMESTONE,white w/sand

HOLE:

60.000       20.000

200.000       16.000

CASING:

199.500       16.000

SCREEN:

69.500      199.500

WELL: BR-044

X:   178464

Y:   151709

ELEV:     95.5

DR.DATES: May 1983

DR.METH: Rotary

SWL:     4.33

VSCALE:      650.0

HSCALE:       50.0

LITH:

375.000 MISSING Unknown lithology. \Principal aquifer: limestone.

HOLE:

375.000       17.000

CASING:

375.000       14.000

SCREEN:

295.000      375.000

WELL: BR-051

X:   182122

Y:   155519

ELEV:     11.7

ELEVM:     11.7

DR.DATES: June 1965

DR.METH: Digging

SWL:     6.71

VSCALE:      100.0

HSCALE:       50.0

LITH:

19.600 LIME

WELL: BR-060

X:   182152

Y:   156159

ELEV:     10.7

ELEVM:     10.7

DR.DATES:

DR.METH:

SWL:     6.55

VSCALE:      100.0

HSCALE:      100.0

LITH:

18.750 LIME

WELL: BR-073

X:   185079

Y:   146649

ELEV:    182.1

DR.DATES: 4 Oct. 1967

DR.METH: Cable tool

SWL:    11.58

VSCALE:     1400.0

HSCALE:       50.0

LITH:

30.000 LIME LIMESTONE, white

181.000 LIMEH LIMESTONE, white\hard to very hard

196.000 LIMEH LIMESTONE,brown,weathered\very hard

250.000 LIMEH LIMESTONE, white\hard to very hard

315.000 LIMEH LIMESTONE, very hard\fissured

327.000 LIME LIMESTONE,buff coloured

460.000 LIMEH LIMESTONE,white,\medium hard

485.000 LIME LIMESTONE,white,w/shells

540.000 LIME LIMESTONE,white

585.000 LIMEH LIMESTONE,very hard,\white, fissured

620.000 LIMEH LIMESTONE,white,hard

655.000 LIME LIMESTONE,white,very porous

721.000 LIME LIMESTONE,med.hard,white

HOLE:

655.000       24.000

721.000       12.000

CASING:

30.000       24.000

655.000       18.000

SCREEN:

575.000      655.000

WELL: BR-074

X:   185140

Y:   146283

ELEV:    163.5

DR.DATES: 10 Jan. 1968

DR.METH: Cable tool

SWL:    11.13

VSCALE:     1200.0

HSCALE:      100.0

LITH:

5.000 CLAY BAUXITE

25.000 LIMES LIMESTONE,soft,white

515.000 LIMEH LIMESTONE, white\medium to hard

557.000 LIME LIMESTONE, white

587.000 FRACTURE LIMESTONE,white, fissured

672.000 LIMEH LIMESTONE, white\hard to very hard

HOLE:

617.000       24.000

672.000       12.000

CASING:

30.000       24.000

617.000       18.000

SCREEN:

537.000      617.000

WELL: BR-075

X:   185261

Y:   146466

ELEV:    165.2

DR.DATES: 20 June 1968

DR.METH: cABLE TOOL

SWL:    11.28

VSCALE:     1200.0

HSCALE:      100.0

LITH:

28.000 LIMES LIMESTONE,soft,white

185.000 LIMEH LIMESTONE,hard,white\with solution channels

250.000 LIMEH LIMESTONE,very hard,white\fissured

310.000 LIMEH LIMESTONE,white,med.hard

330.000 LIMEH LIMESTONE,white,very hard

495.000 LIME LIMESTONE,white\med. to hard

620.000 LIME-CAV LIMESTONE,fissured,buff\with solution channels

680.000 LIME LIMESTONE,white,med.hard

HOLE:

620.000       24.000

680.000       12.000

CASING:

20.000       24.000

620.000       18.000

SCREEN:

540.000      620.000

WELL: BR-076

X:   185292

Y:   146192

ELEV:    159.7

DR.DATES: 1970

DR.METH: Rotary

SWL:    12.50

VSCALE:     1200.0

HSCALE:      100.0

LITH:

700.000 LIME

HOLE:

20.000       18.000

700.000       12.000

WELL: BR-077

X:   184407

Y:   151004

ELEV:     78.3

DR.DATES: 13 Aug. 1970

DR.METH: Rotary

SWL:     7.62

VSCALE:      800.0

HSCALE:      100.0

LITH:

96.000 LIMEH LIMESTONE, white\med.hard to hard

174.000 LIMEH LIMESTONE, buff,hard

212.000 LIMEH LIMESTONE, white\med. to hard

261.000 LIME LIMESTONE,white,broken

400.000 LIMEH LIMESTONE, white\very hard

HOLE:

20.000       18.000

400.000       12.000

WELL: BR-078

X:   184621

Y:   151313

ELEV:     67.1

DR.DATES: 1970

DR.METH: Rotary

SWL:     8.53

VSCALE:      800.0

HSCALE:      100.0

LITH:

400.000 LIME

HOLE:

20.000       18.000

400.000       12.000

WELL: BR-079

X:   184408

Y:   151587

ELEV:     56.0

DR.DATES: June 1972

DR.METH: Rotary

SWL:     7.92

VSCALE:      800.0

HSCALE:      100.0

LITH:

86.000 LIMEH LIMESTONE,med.hard,\white to grey

95.000 LIMEH LIMESTONE,white,hard

260.000 LIME-CAV LIMESTONE,hard,fractured\greyish

280.000 LIMEH LIMESTONE,grey,very hard

300.000 LIME-CAV LIMESTONE,hard,grey,fract.

320.000 LIMEH LIMESTONE,grey,hard

400.000 LIME-CAV LIMESTONE,very hard\grey,partly fractured

HOLE:

270.000       22.000

400.000       12.000

CASING:

270.000       18.000

ANNULUS:

400.000 TEXT

WELL: BR-080

X:   184560

Y:   151069

ELEV:     67.1

DR.DATES: June 1972

DR.METH: Percussion

SWL:     2.83

VSCALE:      800.0

HSCALE:      100.0

LITH:

400.000 LIME

HOLE:

320.000       22.000

400.000       12.000

CASING:

320.000       18.000

SCREEN:

235.000      320.000

WELL: BR-081

X:   184041

Y:   149023

ELEV:    147.1

DR.DATES:

DR.METH: Rotary

SWL:     7.84

VSCALE:     1100.0

HSCALE:       50.0

LITH:

8.000 CLAY CLAY, red

110.000 LIME-CL MARL and LIMESTONE

215.000 LIME

265.000 LIMEH

270.000 LIME-CAV LIMESTONE, porous

475.000 LIME LIMESTONE, med. hard

555.000 LIME-CAV LIMESTONE, with crevices

580.000 LIME-CAV LIMESTONE, porous

603.000 LIME

HOLE:

603.000        8.250

CASING:

530.000        6.000

SCREEN:

480.000      530.000

WELL: BR-082

X:   185169

Y:   147377

ELEV:    171.9

DR.DATES:

DR.METH: Rotary

SWL:    11.28

VSCALE:     1400.0

HSCALE:       50.0

LITH:

1.000 CLAY CLAY, red

3.000 CLAY-LST CLAY w/limestone boulders

38.000 LIMEH

64.000 LIME-CL LIMESTONE with MARL

72.000 LIME-CAV LIMESTONE,hard,w/crevices

185.000 LIME-CL LIMESTONE with MARL

215.000 FRACTURE LIMESTONE, with crevices

375.000 LIMEH

545.000 LIME LIMESTONE, buff coloured

748.000 LIME-CAV LIMESTONE,hard,w/crevices

751.000 LIMEH LIMESTONE, very hard

HOLE:

751.000        9.800

CASING:

600.000        6.000

SCREEN:

550.000      600.000

WELL: BR-083

X:   184468

Y:   147651

ELEV:    178.7

DR.DATES: 6 Jan. 1982

DR.METH: Rotary

SWL:    17.37

VSCALE:     1200.0

HSCALE:       50.0

LITH:

10.000 MARL MARL with BAUXITE

70.000 DEBRIS MARL\with boulders and gravel

410.000 MISSING No recovery

440.000 MARL MARL, fine w/little CLAY

630.000 MISSING No recovery

650.000 MARL MARL, with CLAY

700.000 MISSING No recovery

HOLE:

700.000       11.000

CASING:

615.000        6.000

SCREEN:

565.000      615.000

WELL: BR-084

X:   185932

Y:   146040

ELEV:    198.1

DR.DATES:

DR.METH: Rotary

SWL:    13.67

VSCALE:     1200.0

HSCALE:       50.0

LITH:

64.000 LIME-CL LIMESTONE and MARL

90.000 LIME

275.000 LIME-CAV LIMESTONE with crevices\and MARL

340.000 LIME LIMESTONE, buff coloured

440.000 LIME-CAV LIMESTONE with crevices\and MARL

510.000 LIME-CL LIMESTONE, hard and MARL

535.000 LIMEH

605.000 LIME-CL LIMESTONE and MARL

701.000 LIME-CAV LIMESTONE with crevices

HOLE:

701.000       10.000

CASING:

658.000        6.000

SCREEN:

608.000      658.000

WELL: BR-085

X:   185626

Y:   148078

ELEV:    166.3

DR.DATES: 17 Mar. 1979

DR.METH: Rotary

SWL:     8.84

VSCALE:     1600.0

HSCALE:       50.0

LITH:

2.000 SAND SAND, red,fine grained

380.000 LIME LIMESTONE, tan to white\mod.hard to hard,\various traces of iron stains,\clay,white;chalky lst.

410.000 CLAY CLAY,white with limestone

460.000 CLAY CLAY,tan,chalky,w/lst

475.000 LIME LIMESTONE, tan to white\mod.hard to hard,\various traces of iron stains,\clay

485.000 CLAY

900.000 LIME LIMESTONE,white with red\iron staining,some vugular\and pinhole porosity\hard to mod. hard\slightly fossiliferous\partly recrystalized

902.000 MISSING

HOLE:

20.000       12.000

902.000       11.000

CASING:

645.000        6.000

SCREEN:

595.000      645.000

WELL: BR-086

X:   184803

Y:   148505

ELEV:    183.2

DR.DATES: 24 Mar. 1979

DR.METH: Rotary

SWL:     8.53

VSCALE:     1200.0

HSCALE:       50.0

LITH:

20.000 LIME LIMESTONE,white,trace clay

40.000 LIME LIMESTONE,tan,chalky

260.000 LIME LIMESTONE,tan to white,\fine to med. grained\vugs and pinholes\mod. hard to hard

280.000 CLAY CLAY,white,soft,plastic

320.000 LIME LIMESTONE,dark tan,vugs

360.000 CLAY-LST CLAY,tan to white,soft\plastic,w/limestone

390.000 LIME LIMESTONE,tan,vugs,hard

410.000 LIME LIMESTONE,white,chalky,vugs

420.000 CLAY

530.000 LIME LIMESTONE,white,chalky\w/some vugs\moderately hard

540.000 CLAY CLAY, some Limestone

650.000 LIME-CL LIMESTONE and CLAY\white to tan\hard to mod. hard

725.000 LIME LIMESTONE,white to tan\low porosity

HOLE:

20.000       12.000

725.000       11.000

CASING:

18.000       12.000

675.000        6.000

SCREEN:

635.000      675.000

WELL: BR-087

X:   184011

Y:   147987

ELEV:    192.3

DR.DATES: 20 Apr. 1979

DR.METH: Rotary

SWL:    17.07

VSCALE:     1500.0

HSCALE:       50.0

LITH:

10.000 LIME LIMESTONE, and terra rosa sands

90.000 LIME LIMESTONE,tan-white,chalky

210.000 LIME LIMESTONE,white to grey,\low porosity

330.000 LIME LIMESTONE,grey to tan,\low porosity

370.000 LIME LIMESTONE,tan,white,vugs

450.000 LIME LIMESTONE,grey to tan,\low to zero porosity

560.000 LIME LIMESTONE,white,vuggy,\chalky

660.000 LIME LIMESTONE,white to tan,\pinholes,low porosity

802.000 LIME LIMESTONE,white,chalky,\weathered,mod. hard to soft\some iron stains

HOLE:

20.000       12.000

802.000       11.000

CASING:

20.000       12.000

640.000        6.000

SCREEN:

590.000      640.000

WELL: BR-088

X:   182334

Y:   148108

ELEV:    125.8

DR.DATES: 1 Apr. 1979

DR.METH:

SWL:    17.98

VSCALE:     1000.0

HSCALE:       50.0

LITH:

2.000 CLAY TERRA ROSA

90.000 LIME LIMESTONE,white,vugs \and pinholes\med. porosity,chalky

150.000 LIME LIMESTONE,white,granular\moderate porosity

320.000 LIME LIMESTONE,white to grey,\chalky\with vugs and pinholes\mod. hard to hard\low porosity

530.000 LIME LIMESTONE,white to grey,\chalky\granular,fair porosity

HOLE:

20.000       12.000

530.000       11.000

CASING:

18.000       12.000

475.000        6.000

SCREEN:

425.000      475.000

WELL: BR-089

X:   183554

Y:   147499

ELEV:    148.2

DR.DATES: 24 Apr. 1979

DR.METH: Rotary

SWL:    17.68

VSCALE:     1300.0

HSCALE:       50.0

LITH:

10.000 LIME-CL LIMESTONE and TERRA ROSA

150.000 LIME LIMESTONE,white,with vugs\and pinholes;mod.hard\mod. porosity

180.000 MISSING No samples collected

220.000 LIME LIMESTONE,as above

360.000 LIME-CL LIMESTONE and CLAY,\white to tan,chalky\crumbly,mod. porosity

490.000 LIME LIMESTONE,white,brittle\with vugs and pinholes\low porosity

585.000 LIME-CL Claystone and Limestone\grey to white,crumbly

702.000 MISSING No samples collected. Lost circ.\Drilling blind with mud.

HOLE:

700.000       12.250

CASING:

470.000       10.000

520.000        6.000

SCREEN:

470.000      520.000

WELL: BR-090

X:   183858

Y:   147194

ELEV:    128.7

DR.DATES: 4 May 1979

DR.METH: Rotary

SWL:    16.76

VSCALE:     1100.0

HSCALE:       50.0

LITH:

50.000 LIME LIMESTONE, white to \light cream

100.000 LIME LIMESTONE, tan to white

140.000 CLAY-LST CLAY with limestone

290.000 LIME LIMESTONE,white and grey\low porosity

510.000 LIME LIMESTONE, white to tan,\fine grained,brittle\vugs and pinholes\hard,trace of white clay

625.000 MISSING No samples collected,lost circ.\drilling blind

HOLE:

20.000       12.000

625.000       11.000

CASING:

503.000        6.000

SCREEN:

453.000      503.000

WELL: BR-091

X:   182914

Y:   146371

ELEV:    115.6

DR.DATES: 10 May 1979

DR.METH: Rotary

SWL:    20.12

VSCALE:     1000.0

HSCALE:       50.0

LITH:

60.000 LIME LIMESTONE,white to cream

240.000 LIME LIMESTONE,white to cream\fine grained,mod.hard\low to moderate porosity\trace of white,chalky clay

260.000 LIME-CL LIMESTONE and CLAY

300.000 LIME-CL CLAYSTONE and LIMESTONE\crumbly

380.000 LIME LIMESTONE,fine grained,vugs\low porosity

490.000 LIME-CL CLAYSTONE,crumbly,silty,clay\soft,w/limestone

550.000 LIME LIMESTONE,hard,low porosity\interbeds of clay

HOLE:

20.000       12.000

550.000       11.000

CASING:

20.000       12.000

403.000        6.000

SCREEN:

353.000      403.000

WELL: BR-092

X:   184438

Y:   148017

ELEV:    168.7

DR.DATES: 13 May 1979

DR.METH: Rotary

SWL:    12.19

VSCALE:     1200.0

HSCALE:       50.0

LITH:

30.000 LIME LIMESTONE,beige to white

120.000 LIME LIMESTONE,beige to tan,\fine grained,mod.hard\to hard,low porosity\and white chalky limestone

130.000 MISSING No samples collected

200.000 LIME LIMESTONE,tan to grey,vugs\pinholes,low porosity

210.000 MISSING No samples collected

230.000 LIME LIMESTONE,white to light beige

290.000 LIME LIMESTONE,beige to light tan\mod.soft to mod.hard

310.000 LIME LIMESTONE,grey/brown

440.000 LIME LIMESTONE,tan to beige;\tan is hard,zero or low porosity;\beige low porosity

490.000 LIME LIMESTONE,beige,brittle\low porosity

570.000 LIME LIMESTONE,beige to tan,\low porosity

590.000 MISSING No samples collected

660.000 LIME LIMESTONE,as above

702.000 LIME LIMESTONE,beige,tan,grey

HOLE:

20.000       12.000

702.000       11.000

CASING:

20.000       12.000

591.000        6.000

SCREEN:

541.000      591.000

WELL: BR-093

X:   189010

Y:   137688

ELEV:    126.4

DR.DATES: 13 June 1951

DR.METH:

SWL:     1.09

VSCALE:     1200.0

HSCALE:       50.0

LITH:

3.000 CLAY BAUXITE

35.000 BOULDER

244.000 LIMEH

430.000 LIME LIMESTONE, medium hard

500.000 LIME LIMESTONE, very porous,\with small water inflows

550.000 LIME LIMESTONE, with water

625.000 LIME-SD LIMESTONE,sandy green

HOLE:

180.000       19.000

320.000       16.000

625.000       12.000

CASING:

200.000       16.000

WELL: BR-094

X:   185718

Y:   153564

ELEV:     78.1

DR.DATES: 20 Oct. 1951

DR.METH: Cable tool

SWL:    11.03

VSCALE:      900.0

HSCALE:       50.0

LITH:

30.000 CLAY BAUXITE, with rock 15-25'

37.000 BOULDER

44.000 ROCK2 VERY HARD ROCK

80.000 LIMEH LIMESTONE, very hard

300.000 LIMEH

350.000 LIME LIMESTONE,loose,broken

415.000 LIME LIMESTONE, hard\to medium hard

505.000 LIMES

HOLE:

350.000       20.000

460.000       16.000

505.000       12.000

CASING:

370.000       16.000

WELL: BR-107

X:   189559

Y:   136713

ELEV:     11.0

DR.DATES: 6 Sep. 1974

DR.METH: Percussion

SWL:     1.68

VSCALE:      200.0

HSCALE:       50.0

LITH:

4.000 CLAY CLAY and loose stone

18.000 BOULDER LIMESTONE boulders,hard\ loose

30.000 LIME-CL LIMESTONE,hard with marl

37.000 LIME-SD LIMESTONE,hard,white,w/sand

105.000 LIME-SD LIMESTONE,hard,white, \with marl and sand pockets

HOLE:

105.000       16.000

CASING:

105.000       16.000

SCREEN:

55.000      105.000

WELL: BR-118

X:   189346

Y:   137962

ELEV:    104.4

DR.DATES: June 1941

DR.METH: Cable tool

SWL:     1.64

VSCALE:      700.0

HSCALE:       50.0

LITH:

6.000 CLAY

400.000 LIMEH

HOLE:

189.000       24.000

400.000       20.000

CASING:

175.000       24.000

377.000       20.000

SCREEN:

329.000      377.000

WELL: BR-121

X:   185231

Y:   146619

ELEV:    176.2

DR.DATES: 10 May 1967

DR.METH:

SWL:    14.33

VSCALE:     1200.0

HSCALE:      100.0

LITH:

8.000 CLAY BAUXITE

55.000 LIMES LIMESTONE,soft,white

77.000 LIMEH LIMESTONE,hard,white

450.000 LIMEH LIMESTONE,med. to hard\white

480.000 LIME LIMESTONE,porous,white

540.000 LIME LIMESTONE,med.hard,white

640.000 LIME LIMESTONE,med. hard,\fissured, white

700.000 LIMEH LIMESTONE,hard,white

722.000 MISSING

HOLE:

648.000       24.000

722.000       12.000

CASING:

30.000       24.000

648.000       16.000

SCREEN:

568.000      648.000

WELL: BR-122

X:   186511

Y:   151861

ELEV:     77.6

DR.DATES: 21 Dec. 1970

DR.METH:

SWL:    -2.29

VSCALE:      600.0

HSCALE:       50.0

LITH:

1.000 CLAY CLAY, red

21.000 LIMEH

24.000 CWG

107.000 LIMEH LIMESTONE, hard,white

270.000 LIME-CL LIMESTONE, soft \and MARL

280.000 LIMEH LIMESTONE,hard,crystalline

301.000 LIME-CL LIMESTONE, soft,w/MARL

320.000 LIMEH LIMESTONE,hard,white

350.000 LIMEH LIMESTONE,hard,crystal.\white

360.000 LIME-CL LIMESTONE,soft with clay

HOLE:

360.000       12.000

CASING:

360.000       12.000

SCREEN:

270.000      360.000

WELL: BR-123

X:   183707

Y:   155580

DR.DATES: 18 May 1979

DR.METH: Percussion

SWL:    11.06

VSCALE:      600.0

HSCALE:       50.0

LITH:

19.000 CLAY CLAY, red

50.000 MARL CLAY, red and MARL

66.000 MARL MARL and CLAY

83.000 LIME-CL MARL and LIMESTONE

95.000 LIMEH

108.000 LIME-SD LIMESTONE,hard and SAND

155.000 LIMES LIMESTONE,soft.\Water struck at 121 ft.

234.000 LIME-SD LIMESTONE,hard and SAND\Water struck at 165,223 ft.

242.000 LIME-CL LIMESTONE,hard and CLAY

350.000 LIME-SD LIMESTONE,hard and SAND\Water struck at 244, 302 ft.

HOLE:

148.000       20.000

350.000       18.000

CASING:

153.750       18.000

SCREEN:

72.250      153.750

WELL: BR-124

X:   184835

Y:   157195

DR.DATES: 21 July 1956

DR.METH: Cable tool

SWL:    37.49

VSCALE:      250.0

HSCALE:       50.0

LITH:

5.000 MARL

15.000 CLAY

40.000 LIME LIMESTONE, yellowish

45.000 LIMES LIMESTONE,white,soft

60.000 LIME-CL LIMESTONE and MARL

65.000 LIMEH LIMESTONE, hard

85.000 LIMES LIMESTONE,white,soft

90.000 LIME LIMESTONE, yellowish

145.000 LIMEH LIMESTONE,hard,yellow

HOLE:

60.000       20.000

145.000       16.000

CASING:

122.000       12.750

WELL: BR-125

X:   184347

Y:   156738

DR.DATES: 1966

DR.METH: Percussion

SWL:    31.00

VSCALE:      250.0

HSCALE:       50.0

LITH:

20.000 CLAY

23.000 CLAY CLAY and STONES

33.000 MARL

60.000 LIMEH LIMESTONE,hard,white\water struck at 33.5 ft.

70.000 LIME-CL LIMESTONE,as above\with MARL

150.000 LIMEH LIMESTONE,hard,white

WELL: BR-126

X:   179927

Y:   155184

ELEV:     32.7

DR.DATES:

DR.METH: Digging

SWL:     6.16

VSCALE:      200.0

HSCALE:       50.0

LITH:

16.000 CLAY CLAY, brown

90.000 LIME

WELL: BR-127

X:   178556

Y:   141163

ELEV:    762.0

DR.DATES:

DR.METH:

VSCALE:    15000.0

LITH:

6451.000 LIME LIMESTONE, white, \Newport Formation

7910.000 ROCK2 Santa Cruz volcanics\andesitic tuff

8732.000 ROCK4 Basement metamorphics

ANNULUS:

8732.000 TEXT1

WELL: BR-128

X:   188675

Y:   148143

ELEV:    253.0

DR.DATES: 14 Sep. 1951

DR.METH: Cable tool

SWL:    10.67

VSCALE:     1800.0

HSCALE:       50.0

LITH:

16.000 CLAY CLAY and BAUXITE

515.000 LIME LIMESTONE, medium soft

925.000 LIME LIMESTONE, med.soft to\hard and very hard

HOLE:

600.000       20.000

925.000       12.000

CASING:

578.000       16.000

WELL: BR-129

X:   190291

Y:   142047

ELEV:    201.0

DR.DATES: May 1989

DR.METH: Percussion

SWL:     6.54

VSCALE:     1400.0

HSCALE:       50.0

LITH:

680.000 LIMEH

710.000 LIME-SD LIMESTONE and SAND

HOLE:

710.000       12.000

CASING:

70.000       12.000