# @BEGIN_TEXTDEF

CS2CS	cs2cs is an XBit object for different coordinates system transformation based on the PROJ.4 library (Copyright of Frank Warmerdam, http://www.remotesensing.org/proj).  To use a cs2cs object, we need to properly set a system environment vairable PROJ_LIB to point to the original PROJ.4 package's nad directory.  For example, if the original PROJ.4 package is located in c:/proj445, we need to set PROJ_LIB=c:/proj445/nad.  In XBit, this directory is located at $gimgHome/doc/proj/nad.

SYNOPSIS

cs2cs.create objName ?options?

objName cget option

objName class

objName clone

objName config ?options?

objName count

objName csconfigure ?options?

objName cs2cs x y z

objName csget option

objName iinfo

objName iparams

objName oinfo

objName oparams

objName save pathname

objName type

objName version

STANDARD CONFIGURATION OPTIONS

-file			specifies a proj specification file

-update			specifiies a flag to update clients

CSCONFIG OPTIONS

-ics			specifies a projection definition for an input coordinate system, e.g., "proj=utm zone=10 datum=NAD27",  the default input projection definition.

-ocs			specifies a projection definition for an output coordinate system, e.g., "proj=utm zone=10 datum=NAD83", the default output projection definition.

-ix			specifies an input value of x or longitude coodindate.

-iy			specifies an input value of y or latitude coordinate.

-iz			specifies an input value of z.

-ox			refers an output value of x or longitude coordinate.

-oy			refers an output value of y or latitude coordinate.

-oz			refers an output value of z.

-fmt			specifies a format string for calculated x and y conversion.  The default is "%.12g".

CS2CS OBJECT COMMANDS

cs2cs.create objName ?options? creates a projection object which can be configurated to perform coordinate system transformation.

objName cs2cs x y z		returns a triple {u v w}, which are coordinates of the projection specified by -ocs option.

objName csconfig	?options? returns option of or configurates the cs2cs object (see CSCONFIG OPTIONS).

objName csget option	returns option value of the cs2cs object.

objName iinfo			returns infomation of the input projection, such as semi axis, eccentricity, central meridian and latitude, false easting and northing, general scaling, cartesian scaling and etc.

objName iparams			returns a list of the input projection definition parameters including internal default params.

objName oinfo			returns infomation of the current projection, such as semi axis, eccentricity, central meridian and latitude, false easting and northing, general scaling, cartesian scaling and etc.

objName oparams			returns a list of the output projection definition parameters including internal default params.

CS2CS COMMAND

cs2cs [ -eEfIlrstvwW [ args ] ] [ +opts[=arg] ] [ +to [+opts[=arg]] ] file[s]

CS2CS COMMAND OPTOINS		For details and the original document, see cs2cs man.1.

-I 			method to specify inverse translation, convert from +to coordinate system to the primary coordinate system defined.

-ta 			A specifies a character employed as the first character to denote a control line to be passed through without processing. This option applicable to ascii input only. (# is the default value).

-e string			String is an arbitrary string to be output if an error is detected during data transformations. The default value is: *\t*. Note that if the -b, -i or -o options are employed, an error is returned as HUGE_VAL value for both return values.

-E 			causes the input coordinates to be copied to the output line prior to printing the converted values.

-l[p|P|=|e|u|d]id 			List projection identifiers with -l, -lp or -lP (expanded) that can be selected with +proj. -l=id gives expanded description of projection id. List ellipsoid identifiers with -le, that can be selected with +ellps, -lu list of cartesian to meter conversion factors that can be selected with +units or -ld list of datums that can be selected with +datum.

-r 			This options reverses the order of the expected input from longitude-latitude or x-y to latitude-longitude or y-x.

-s 			This options reverses the order of the output from x-y or longitude-latitude to y-x or latitude-longitude.

-m mult			The cartesian data may be scaled by the mult parameter. When processing data in a forward projection mode the cartesian output values are multiplied by mult otherwise the input cartesian values are divided by mult before inverse projection. If the first two characters of mult are 1/ or 1: then the reciprocal value of mult is employed.

-f format 			Format is a printf format string to control the form of the output values. For inverse projections, the output will be in degrees when this option is employed. If a format is specified for inverse projection the output data will be in decimal degrees. The default format is "%.2f" for forward projection and DMS for inverse.

-[w|W]n 			N is the number of significant fractional digits to employ for seconds output (when the option is not specified, -w3 is assumed). When -W is employed the fields will be constant width and with leading zeroes.

-v 			causes a listing of cartographic control parameters tested for and used by the program to be printed prior to input data. Should not be used with the -T option.

-V 			This option causes an expanded annotated listing of the characteristics of the projected point. -v is implied with this option.

# @END_TEXTDEF

# @BEGIN_TAGDEF

# @BEGIN_FONT

# "-*-Times-Bold-r-normal-*-*-140-*-*-*-*-*-*" 1.0 1.5 1.183 1.188 1.302 1.305 3.0 3.8 5.0 5.12 6.8 6.12 7.8 7.13 8.8 8.13 9.8 9.14 10.8 10.13 11.8 11.19 12.8 12.13 13.8 13.13 14.8 14.13 15.8 15.15 16.8 16.13 17.8 17.15 18.8 18.12 19.8 19.12 20.8 20.15 22.0 22.8 22.9 22.30 24.0 24.5 25.0 25.7 27.0 28.0 29.0 29.4 31.0 31.4 33.0 33.3 35.0 35.3 37.1 37.3 39.0 39.3 41.0 41.3 43.0 43.4 45.0 45.4 47.0 47.21 49.0 49.12 51.8 51.13 51.100 51.104 53.8 53.16 53.65 53.70 53.83 53.99 55.8 55.13 55.49 55.54 57.8 57.13 59.8 59.15 61.8 61.13 63.8 63.15 66.0 66.13 68.0 68.5 71.0 71.21 73.0 73.3 75.1 75.2 77.0 77.3 79.0 79.6 81.0 81.2 81.3 81.14 83.0 83.2 85.0 85.3 87.0 87.2 88.0 88.2 90.0 90.1 90.2 90.5 92.1 92.3 94.0 94.3

# "-*-Times-Medium-I-normal-*-*-140-*-*-*-*-*-*" 5.13 5.20 5.21 5.30 6.0 6.7 6.13 6.19 7.0 7.7 8.0 8.7 9.0 9.7 9.15 9.24 10.0 10.7 11.0 11.7 11.20 11.29 12.0 12.7 12.14 12.19 13.0 13.7 13.14 13.20 14.0 14.7 15.0 15.7 16.0 16.7 17.0 17.7 18.0 18.7 18.13 18.21 19.0 19.7 20.0 20.7 35.58 35.59 37.33 37.35 39.32 39.33 41.32 41.33 43.32 43.33 49.13 49.20 49.21 49.30 51.0 51.7 51.14 51.19 51.39 51.44 53.0 53.7 53.17 53.26 55.0 55.7 55.14 55.20 57.0 57.7 59.0 59.7 61.0 61.7 63.0 63.7 75.2 75.3 77.3 77.9 81.15 81.18 87.3 87.7 88.3 88.9 90.6 90.7

# "-*-Times-Medium-R-normal-*-*-140-*-*-*-*-*-*" 1.467 1.489

# "-*-Times-Medium-r-normal-*-*-140-*-*-*-*-*-*" 81.2 81.3 81.14 81.15 90.1 90.2 90.5 90.6

# @END_FONT

# @BEGIN_LMARGIN1

# "0i" 1.5 1.106 1.108 5.0 22.9 23.0 27.2 28.0 47.0 47.21 71.0 72.0

# "0.5i" 5.0 22.0 23.0 27.0 28.0 47.0 49.0 49.128 51.0 51.38 51.39 51.44 51.45 66.0 68.0 70.0 72.0 96.0

# @END_LMARGIN1

# @BEGIN_LMARGIN2

# "0.5i" 1.5 1.106 1.108 5.0 22.9 23.0 27.2 28.0 47.0 47.21 68.0 70.0 71.0 72.0

# "1.5i" 5.0 22.0 23.0 27.0 28.0 47.0 49.0 49.128 51.0 51.38 51.39 51.44 51.45 66.0 72.0 96.0

# @END_LMARGIN2

# @BEGIN_RMARGIN

# "0.5i" 1.5 1.106 1.108 51.38 51.39 51.44 51.45 66.0 71.0 96.0

# "0.5i " 68.0 70.0

# @END_RMARGIN

# @END_TAGDEF

# @BEGIN_CONFIG

# "-font -*-Times-Medium-r-normal-*-*-140-*-*-*-*-*-* -tabs {0.5i 1.0i 1.5i 2.0i 2.5i 3.0i 3.5i 4.0i 4.5i 5.0i} -background #c0c0c0c0c0c0 -foreground #000000000000 -wrap word -spacing1 0 -spacing2 0 -spacing3 0"

# @END_CONFIG