# @BEGIN_TEXTDEF

GEOD	geod is an XBit object for geodesic calculation based on the PROJ.4 library (Copyright of Frank Warmerdam, http://www.remotesensing.org/proj).  To use a geod object, we need to properly set a system environment vairable PROJ_LIB to point to the original PROJ.4 package's nad directory.  For example, if the original PROJ.4 package is located in c:/proj445, we need to set PROJ_LIB=c:/proj445/nad.  In XBit, this directory is located at $gimgHome/doc/proj/nad.

SYNOPSIS

geod.create objName ?options?

objName calcarc lon lat A S n dA

objName calcgeod lon lat A S n

objName cget option

objName class

objName clone

objName config ?options?

objName count

objName geodconfigure ?options?

objName geodfwd lon lat A S

objName geodget option

objName geodinv lon1 lat1 lon2 lat2

objName save pathname

objName type

objName version

STANDARD CONFIGURATION OPTIONS

-file			specifies a proj specification file

-update			specifiies a flag to update clients

GEODCONFIG OPTIONS

-geod			specifies a parametric string to describe the ellipsoid for geodesic calculation.  The defult is "ellps=clrk66"

-inv			specifies a flag of inverted conversion: 0 for forward conversion, 1 for inverted conversion.

-lon_1			specifies a longitude of the first point.

-lat_1			specifies a latitude of the first point.

-lon_2			specifies a longitude of the second point.

-lat_2			specifies a latitude of the second point.

-A			specifies an azimuth from the first pont to the second point in unit of degrees.

-S			specifies a distance between the first and second points in unit of meters.

-fmt			specifies a format string print calculated results.  The default is "%.12g".

Note: When option -inv is '0', values of -lon_2, -lat_2 will be calculated using values of -lon_1, -lat_1, -A and -S.  Otherwise, -A and -S will be calculated using values of -lon_1, -lat_1, -lon_2 and -lat_2.  The option -inv does not have any impact on geod object commands calcarc, calcgeod, geodfwd and geodinv.

GEOD OBJECT COMMANDS

geod.create objName ?options? creates a geod object which can be configurated to calculate geodecis of various cooridinate systems.

objName calcarc lon lat A S n dA		returns a list of vertices in longitude and latifude of arc segments defined by {lon lat A S n dA} where n is the number of vertices excluding the start point (lon,lat), dA is the azimuth increment in unit of degrees at each vertex.

objName calcgeod lon lat A S n		returns a llist of vertices in longitude and latitude of great circle segments defined by {lon lat A S n} where n is the number of vertices excluding the start point (lon,lat)

objName cget ?option?	retrieves the value of the projection object configuration.

objName config ?options?	configurates the object if options are provided.  Otherwise it retrieves a list of configuration options of the projection object.

objName geodconfig		?options? returns option of or configurates the geod object (see GEODCONFIG OPTIONS).

objName geodfwd lon lat A S		returns a triple of {longitude latitude azimuth} at the other end of a great circle segment defined by {lon lat A S}.

objName geodget ?option?	returns option value of the proj object.

objName geodinv lon1 lat1 lon2 lat2		returns triple of {A1 A2 S} of a great circle segment defined by {lon1 lat1 lon2 lat2}.

GEOD COMMAND

geod [ -afFIlptwW [ args ] ] [ +args ] file[s]

GEOD COMMAND OPTOINS		For details and the original document, see geod man.1.

-I 			specifies that the inverse geodesic computation is to be performed. May be used with execution of goed as an alternative to invgeod execution.

-a 			Latitude and longitudes of the initial and terminal points, forward and back azimuths and distance are output.

-ta 			a specifies a character employed as the first character to denote a control line to be passed through without processing.

-le 			Gives a listing of all the ellipsoids that may be selected with the +ellps= option.

-lu 			Gives a listing of all the units that may be selected with the +units= option.

-[f|F] format 			Format is a printf format string to control the output form of the geographic coordinate values (f) or distance value (F). The default mode is DMS for geographic coordinates and "%.3f" for distance.

-[w|W]n 			N is the number of significant fractional digits to employ for seconds output (when the option is not specified, -w3 is assumed). When -W is employed the fields will be constant width with leading zeroes.

-p 			This option causes the azimuthal values to be output as unsigned numbers between 0 and 360.

The +args run-line arguments are associated with geodetic parameters for specifying the ellipsoidal or sphere to use. See proj documentation for full list of these parameters and contrl. The options are processed in left to right order from the run line. Reentry of an option is ignored with the first occurance assumed to be the desired value.

One or more files (processed in left to right order) specify the source of data to be transformed. A - will specify the location of processing standard input. If no files are specified, the input is assumed to be from stdin.

For direct determinations input data must be in latitude, longitude, azimuth and distance order and output will be latitude, longitude and back azimuth of the terminus point. Latitude, longitude of the initial and terminus point are input for the inverse mode and respective forward and back azimuth from the initial and terminus points are output along with the distance between the points.

Input geographic coordinates (latitude and longitude) and azimuthal data must be in DMS format and input distance data must be in units consistent with the ellipsoid major axis or sphere radius units. Output geographic coordinates will be in DMS (if the -f switch is not employed) to 0.001" with trailing, zero-valued minute-second fields deleted. Output distance data will be in the same units as the ellipsoid or sphere radius.

The Earth's ellipsoidal figure may be selected in the same manner as program proj by using +ellps=, +a=, +es=, etc.

Geod may also be used to determine intermediate points along either a geodesic line between two points or along an arc of specified distance from a geographic point. In both cases an initial point must be specified with +lat_1=lat and +lon_1=lon parameters and either a terminus point +lat_2=lat and +lon_2=lon or a distance and azimuth from the initial point with +S=distance and +A=azimuth must be specified.

If points along a geodesic are to be determined then either +n_S=integer specifying the number of intermediate points and/or +del_S=distance specifying the incremental distance between points must be specified.

To determine points along an arc equidistant from the initial point both +del_A=angle and +n_A=integer must be specified which determine the respective angular increments and number of points to be determined.

# @END_TEXTDEF

# @BEGIN_TAGDEF

# @BEGIN_FONT

# "-*-Times-Bold-r-normal-*-*-140-*-*-*-*-*-*" 1.0 1.4 1.158 1.162 1.276 1.279 3.0 3.8 5.0 5.11 6.8 6.15 7.8 7.16 8.8 8.12 9.8 9.13 10.8 10.13 11.8 11.14 12.8 12.13 13.8 13.21 14.8 14.15 15.8 15.15 16.8 16.15 17.8 17.12 18.8 18.12 19.8 19.15 21.0 21.8 21.9 21.30 23.0 23.5 24.0 24.7 26.0 27.0 28.0 28.5 30.0 30.4 32.0 32.6 34.0 34.6 36.0 36.6 38.0 38.6 40.0 40.2 42.1 42.2 44.0 44.4 46.0 46.5 46.18 46.22 46.41 46.48 46.49 46.55 46.91 46.97 46.99 46.105 46.108 46.109 46.115 46.116 46.131 46.132 46.138 46.139 46.176 46.181 46.184 46.189 46.192 46.197 46.203 46.208 46.222 46.226 46.276 46.283 46.285 46.293 46.295 46.302 46.307 46.314 48.0 48.20 50.0 50.11 52.8 52.15 54.8 54.16 56.8 56.12 58.8 58.14 60.8 60.18 60.68 60.72 60.85 60.103 62.8 62.15 64.8 64.15 66.8 66.15 68.0 68.12 70.0 70.4 72.0 72.20 74.1 74.2 76.1 76.2 78.0 78.2 80.0 80.2 82.0 82.2 84.2 84.5 86.2 86.5 88.1 88.2

# "-*-Times-Medium-I-normal-*-*-140-*-*-*-*-*-*" 5.12 5.19 5.20 5.29 6.0 6.7 6.15 6.32 7.0 7.7 7.16 7.30 8.0 8.7 8.13 8.19 9.0 9.7 10.0 10.7 11.0 11.7 11.15 11.24 12.0 12.7 13.0 13.7 13.22 13.31 14.0 14.7 14.16 14.27 15.0 15.7 15.16 15.22 16.0 16.7 16.15 16.35 17.0 17.7 17.13 17.21 18.0 18.7 19.0 19.7 50.12 50.19 50.20 50.29 52.0 52.7 52.15 52.32 52.115 52.131 52.139 52.140 52.194 52.201 52.204 52.206 54.0 54.7 54.16 54.30 54.123 54.136 54.144 54.145 54.199 54.206 56.0 56.7 56.13 56.21 58.0 58.7 58.15 58.24 60.0 60.7 60.20 60.29 62.0 62.7 62.16 62.27 62.50 62.76 62.133 62.144 64.0 64.7 64.16 64.24 66.0 66.7 66.16 66.35 66.56 66.63 66.103 66.122 78.2 78.8 84.7 84.13 86.6 86.7

# "-*-Times-Medium-R-normal-*-*-140-*-*-*-*-*-*" 1.441 1.463

# "-*-Times-Bold-R-normal-*-*-140-*-*-*-*-*-*" 80.2 80.3 82.2 82.3

# @END_FONT

# @BEGIN_LMARGIN1

# "0i" 1.0 5.0 21.9 22.0 26.4 27.0 48.0 48.20 72.0 73.0

# "0.5i" 5.0 21.0 22.0 26.0 27.0 48.0 50.0 68.0 70.0 70.46 73.0 107.0

# @END_LMARGIN1

# @BEGIN_LMARGIN2

# "0.5i" 1.0 5.0 21.9 22.0 26.4 27.0 46.0 48.20 70.0 70.46 72.0 73.0

# "1.5i" 5.0 21.0 22.0 26.0 27.0 46.0 50.0 68.0 73.0 107.0

# @END_LMARGIN2

# @BEGIN_RMARGIN

# "0.5i" 1.0 46.0 48.0 68.0 72.0 107.0

# "0.5i " 46.0 48.0 70.0 70.46

# @END_RMARGIN

# @END_TAGDEF

# @BEGIN_CONFIG

# "-font -*-Times-Medium-r-normal-*-*-140-*-*-*-*-*-* -tabs {0.5i 1.0i 1.5i 2.0i 2.5i 3.0i 3.5i 4.0i 4.5i 5.0i} -background #c0c0c0c0c0c0 -foreground #000000000000 -wrap word -spacing1 0 -spacing2 0 -spacing3 0"

# @END_CONFIG