# EuroDirect British Aerospace 146 Operating Manual v1.0

8th November 2000

This operating manual is written for use with the following aircraft:

The EuroDirect BAe 146 by Bill Alderson, Sam Chinn & Tony Madge

I-EDME

Original Aircraft created by Bill Alderson.

Flight Dynamics by Sam Chinn.

Texture and moving parts by Tony Madge

Re-Paint for EuroDirect by Chris Williams.

Despatch sheets by Scott Clifford

#### EuroDirect would like to thank the original authors for allowing us to use their aircraft for this project. Without their work, permission and help the EuroDirect BAe 146 would not have been possible.

Welcome to the BAe 146 operating manual.  I hope you get as much enjoyment from its use as I have had, writing it. Please let me know if you have any feedback regarding the material in this manual. I would like to make corrections and release updates but I will only be able to do this with your help.

BAe 146 Technical Information:

First Flight: 			3rd September 1981

Certified: 			13th June 1983

Flight Crew: 			2

Passengers: 			85-100 maximum in single class layout

##### Powerplants

Engines: 			4 Allied Signal ALF 505 R-5(Turbine engines)

Engine Rating			6970 lbs take-off,  6550 lbs max cruise

Specific Fuel Consumption: 	473 kg/h normal cruise

Fan case diameter		3ft 6in (1.06m)

Bypass ratio			5:1

Dry Weight			1385 lb (628 kg)

## Weights

Maximum take-off:		93000 lbs (42184 kg)

Maximum landing:		81000 lbs (36740 kg)

Maximum zero fuel:		75000 lbs (34019 kg)

## Performance

Cruise Speed:			432 kts max, 390 kts normal, 356 kts economy

Cruise Altitude:			FL250 to FL350

Range:				1620 nautical miles (1865 miles or 3001 km)

Take-off field length:	5400 ft (1646 m) @ MTOW, sea level, ISA + 10

Landing field length:		3910 ft (1192 m) @ sea level, ISA.

Picture courtesy of BAe Systems

### EURODIRECT OPERATIONS AS OF October 2000

Currently the EuroDirect maintains two standard regional jet (category 2) aircraft.  These are the BAe 146 and the Bombardier CRJ 200.

The BAe 146 was selected for it's excellent STOL capability.  This allows it to serve airports which could otherwise only utilise regional turboprop aircraft.  Customer response to the comfort levels provided by this aircraft also influenced the choice.

Recently the CRJ 200 offered by Bombardier greatly increased competition in this market sector.  The more modern airframe design coupled with lower maintenance costs for 2 engines as opposed to 4 for the BAe 146 means that each route must be reviewed to determine the most cost effective aircraft choice.

It is envisaged that EuroDirect will maintain fleets of both aircraft types for the foreseeable future.  Together both aircraft types offer a mix of attributes which are necessary for operating a European regional jet service.

PILOT INFORMATION

Note: Flight Simulator uses flight dynamics scalars that do not accurately represent those present in real world aviation. While the flight dynamics of the BAe 146 have been modelled as closely as possible on it’s real world counterpart the figures shown below are those of the FS98 EuroDirect BAe 146 on routes typically carrying 85 passengers in a two class layout. This equates to a zero fuel operating weight of 32547 kg. With the ALF 505 R-5 engines the BAe 146 may take off at any fuel load at this weight although the pilot must be careful to ensure that maximum landing weights are adhered to.

Flight Planning

Please use the following table for flight planning (it is designed to make the Hub Captain’s life easier). Distance is the total distance travelled in nautical miles. The remaining columns indicate the estimated time taken to make the journey in hours and the total fuel (including reserves, taxi requirements etc.) needed to make the journey. The figures are based upon MTOW/MLW and possible fuel loads. Obviously weather and ATC can cause these figures to vary and range can be extended by increasing altitude in the latter stages of the journey. Take-off and climb to FL250 should take approximately 13 minutes and cover about 65 nm. Descent and approach from FL250 should take about 11 minutes and cover 60 nm. Take-off, climb, descent and approach should consume no more than 1480kg of fuel. A Final note, please remember the rules governing altitude for east/west flight, and rules regarding holding fuel (675 kg) and alternate fuel (745 kg typical for Europe).

IMPORTANT: Please help by reporting errors when using this table. If you cannot make the destination using the fuel load listed in this table please let me know. It could be for any number of reasons, including a miscalculation or two on my part!

| Distance nm (km) | Recommended Flight Level | IAS (kts) | Time (hrs) | Fuel Load |
| --- | --- | --- | --- | --- |
| 200 | FL210-230 | 291-280 | 0:35 | 4344 kg |
| 400 | FL270-310 | 262-244 | 1.09 | 4956 kg |
| 600 | FL270-310 | 262-244 | 1:43 | 6076 kg |
| 800 | FL270-310 | 262-244 | 2.17 | 7197 kg |
| 1000 | FL270-310 | 262-244 | 2:52 | 8317 kg |
| 1200 | FL270-310 | 262-244 | 3.26 | 9437 kg |

The above table shows the typical fuel burn for the BAe 146.  Note that while the BAe 146 has a maximum quoted range of 1620 nm, the Euro Direct configuration with a full load of passengers has a maximum range of about 1000 nm.

This range covered by the Euro Direct BAe 146 is more than sufficient for all current routes operated using these aircraft.  Typically routes above about 800 miles will be flown by larger Category 3 aircraft such as the A320, A321 and MD83.  The final selection of appropriate aircraft is determined by the operations director.

For more exact determination of fuel load please use the BAe 146 flight despatch sheet.  This sheet can also be used to record actual fuel burn for later analysis by the Division Director.

Take-off and Climb Data

Warning: The ALF 505 R-5 engines are rated for full power for short periods only. N1 should not exceed 96% for greater than five minutes. Continuous use of high power settings should be reported as an inspection of the engines may be required.

Take off conditions: ISA, configure flaps at 24 degrees.

| Take Off Weight | Runway Length | V1 | VR | V2 |
| --- | --- | --- | --- | --- |
| 42184 kg | 1646m | 110 | 119 | 127 |

Retract gear and flaps at 155 kts and 165 kts respectively, while accelerating to 270 kts IAS. Commence climb on instruction from ATC.

| Climb Altitude | Rate Of Climb | Throttle | Fuel Use | Total Time | IAS |
| --- | --- | --- | --- | --- | --- |
| Initial Climb to FL110 | 2750 feet/minute | 92% | 743 kg | 4m 00 | 275 kts |
| Climb to FL210 | 1900 feet/minute | 92% | 371 kg | 9m 15 | 255 kts |
| Climb to FL230 | 1200 feet/minute | 92% | 72 kg | 10m 40 | 215 kts |
| Climb to Cruise | 1000 feet/minute | 92% | 72 kg* | 12m 35 | Cruise IAS |

* Assumes FL250, FL270=150kg, FL290=222kg, FL310=372kg

Cruise Data

The cruise tables are based on an economical cruise giving a true airspeed of between 380 and 385 kts. The cruise table shows the typical throttle settings required in order to maintain economic cruising speed and the fuel consumption/hour. Note that weather can have a significant effect on these figures.

| Altitude IAS TAS | FL210 291kts 385kts | FL230 280kts 385kts | FL250 271kts 385kts | FL270 262kts 385kts | FL290 253kts 385kts | F310 244kts 385kts |
| --- | --- | --- | --- | --- | --- | --- |
| Power Fuel Use | 82% 2851kg/hr | 82% 2506kg/hr | 82% 2215kg/hr | 82% 1961kg/hr | 82% 1788kg/hr | 82% 1580kg/hr |
| Nm/500 kg fuel | 68nm | 77nm | 87nm | 98nm | 108nm | 122nm |

Descent and Approach

During descent from cruise altitude the pilot should maintain cruise IAS with the engines on idle power until IAS reaches 230 kts. Typical rate of decent during this time will be 2400 fpm. The rate of descent of approximately 2400 fpm should continue. ATC may instruct the pilot to maintain altitude or a lower airspeed according to conditions.

Vref approach speeds are as follows:

| Flaps at 0 degree | 230 kts | Approach |
| --- | --- | --- |
| Flaps at 18 degree | 175 kts | Approach |
| Flaps at 24 degree | 160 kts | Approach |
| Flaps at 30 degree | 145 kts | Approach and Landing |
| Flaps at 33 degree | 125 kts | Landing |

Landing should normally occur at 115 to 125 kts.

Maximum Airspeed Limits

Maximum hull airspeed is 435 kts

Flaps limitations are 	210 kts IAS for 18 degrees,

180 kts for 24 degrees,

170 kts IAS for 30 degrees

145 kts for 33 degrees extension.

Landing Gear limitations are 205 kts IAS for retraction and 205 kts IAS for extension.

Maximum Tyre limit speed (ground speed) 175 kts

Notes on Flying the BAe 146

The BAe 146 was developed out of a Hawker Siddeley project in the 1960's to replace the DC3.  It was designed to have STOL capability coupled with 747 like comfort.  After much work the BAe 146 was realised.  It's unique high wing, 4 engine design gives some different flying characteristics.

Takeoff

Use the recommended flaps setting of 24 degrees for takeoff unless STOL requirement is necessary.  If operating from city airports (such as London city Airport), at night hold takeoff power to 92% if runway lengths permit to minimise the aircraft noise signature.

Cruise

Occasionally reports of engine choking due to icing have been reported with this aircraft.  Reports have identified the condition as most likely to occur at high altitude (> FL250) and when flying through cloud.  The symptoms are a loss of engine power on multiple engines over a short time period.

If such an incident occurs, turn off any air bleed from the affected engine(s) if possible.  Inform ATC and descend to a lower altitude.  In the incidents reported engine power returned when lower altitudes were reached.

Landing

If STOL landing is required, set flaps to full (33 degrees) for landing.  Approach to airports with restricted access should be on a maximum glideslope of 6 degrees.

Many airports with a STOL requirement have limited overrun capacity.  It is imperative that the pilot adhere to the quoted approach speeds (Vref).  Approach speeds more than 5-10 knots above these can cause serious problems should any of the aircraft brake systems fail during landing.

Pilot practice in a simulator is advisable to perfect steep glideslope approaches at Vref together with the runway threshold touchdowns demanded by STOL.

Note that braking on this aircraft is via spoilers and wheel brakes.  There is no engine reverse thrust capability.

FINAL NOTES

Please remember that these figures are NOT intended to be identical to the real world BAe 146. They are, however, the closest that we can get with the limitations of FS98 as they are. The statistics for fuel use etc. have been obtained using weather settings that accurately represent those of Italy and Southern France in Summer and Winter 2000 (but with no wind) at all altitudes.

Also please consider that there are differences between the flight (and weather) dynamics of FS98 and FS2K. Please let me know your thoughts on the flight dynamics in operational use.

Picture courtesy of BAe Systems

I would like again to thank Bill Alderson, Sam Chinn and Tony Madge for their work in designing this aircraft and for their kind permission for us to use it as part of the EuroDirect fleet. It really is their aircraft!

Copyrights:

This manual is free. It may not be uploaded or placed in a compilation without the written permission of one of the EuroDirect management team. It can be distributed so long as it is not altered in any way and it remains free. You may use and reproduce the information in whatever way you wish but please give credit to Bill, Sam and Tony, and to EuroDirect.

Simon Bradshaw: EuroDirect Fleet Operations Development:

ed_fleet@yahoo.co.uk

Chris Williams EuroDirect Fleet Consultant

The EuroDirect Fleet Website:

http://www.geocities.com/ed_fleet/

Bill Alderson:

http://www.geocities.com/~freefltdesign/

Tony Madge

tonymadge@yahoo.com