# EuroDirect	 A330-200 Operating Manual v1.2

23rd May 2000

This operating manual is written for use with the following aircraft:

The EuroDirect Airbus A330-200 ProMax 2000

by Camil Valiquette

Created with moving parts and Night Lighting by Camil Vailiquette.

Re-Painted for EuroDirect by Chris Williams.

#### Many thanks for your kind permission Camil!

Welcome to the EuroDirect A330-200 operating manual. I hope you get as much enjoyment from its use as I have had, writing it. Please let me know if you have any feedback regarding the material in this manual.  I would like to make corrections and release updates but I will only be able to do this with your help.

D - AEGE

Technical Information:

First Flight: 			13th August 1997

Certified: 			April 1998

Launch Customer:		Canada 3000 (leased from ILFS)

Flight Crew: 			2

Passengers: 			198 in three-class layout (see EuroDirect Operations, below)

##### Powerplants

Engines: 			2 Rolls Royce Trent 772B-60 (triple-shaft turbofans)

Engine Rating			72450lbf @ 25 C

Fuel System: 			139090 litres

Specific Fuel Consumption: 	0.565lb/lbf/hr

Dry Weight			14350lb (6509 kg)

Service Entry:			October 1998

## Weights

Maximum take-off:		507000lbs (230000 kg)

Maximum landing:		405650lbs (184000 kg)

Maximum zero fuel:		265150lbs (120250 kg)

## Performance

Cruise Speed:			Mach 0.82 to 0.84 (465 to 475 kts TAS)

Cruise Altitude:			FL370 achievable at all weights.

FL410 on sectors of up to nautical miles

Range:				6450 nautical miles (11850 km)

Take-off field length:	minimum of 7350ft (2350 m), sea level, ISA+15

Landing field length:		minimum of 5365ft (1635 m) at MLW, sea level, ISA+15

### EURODIRECT OPERATIONS AS OF JUNE 2000

In November of 1999 the EuroDirect Management Team began their planned redevelopment of the EuroDirect fleet. In the course of the next two months CEO, Paolo Patat, met and discussed EuroDirect's future requirements with both Airbus Industrie and Boeing. By the end of the year it was evident that in order to meet the requirement to strengthen the long range low density market requirements the A330-200 was the ideal solution. The increased efficiency offered by the operation of a new aircraft sharing a large number of components with EuroDirect's large existing A330-300 and A340-300 fleet, and the benefits of a single type rating for crews were the deciding factor after prolonged negotiations.

Consequently, 32 A330-200 were ordered, all with Rolls Royce Trent 772 engines. Deliveries of the new aircraft began at the end of May, the aircrafts initially being restricted to European Routes while crews familiarised themselves with the new aircraft. Initially, each of the Divisions will operate with it's own seating arrangement. Central Europe are currently operating with a 270 seat 'high density' layout and operating the aircraft on core routes. UK & Ireland Division continue with the EuroDirect 'standard' of a three class 198 seat configuration and the Italian Division operate with a 202 seat two class layout more suited to the local market.

PILOT INFORMATION

Note: Flight Simulator uses flight dynamics scalars that do not accurately represent those present in real world aviation. While the flight dynamics of the EuroDirect A330-200 have been modelled as closely as possible on it’s real world counterpart, the figures shown below are those of the FS98 EuroDirect A330-200 on routes typically carrying 190 to 200 passengers with approximately 10 tonnes of cargo for the standard aircraft. In the case of the Central European high weight version carrying 260 to 270 passengers and 12 tonnes cargo the figures are identical for the standard aircraft at the same take off weight. Zero fuel operating weights of about 150000kg can be expected for the standard version and 160000kg for the high weight version. With the Trent 772 engines, the EuroDirect A330-200 may take off at a fuel loading of up to 81500kg (72000kg for high weight version) at these weights. The pilot must be careful to ensure that the aircraft is below a gross weight of 174000kg in both cases prior to landing. To ensure MTOW is not exceeded the flight dynamics have been modelled such that 82. tonnes represents 100% fuel on the standard version and 72 tonnes on the high weight version.

Flight Planning

Please use the following table for flight planning (it is designed to make the Hub Captain’s life easier). Distance is the total distance travelled in nautical miles. The remaining columns indicate the estimated time taken to make the journey in hours and the total fuel (including reserves, taxi requirements etc.) needed to make the journey. The figures are based upon MTOW/MLW and possible fuel loads. Obviously weather and ATC can cause these figures to vary and range can be extended by increasing altitude in the latter stages of the journey. Take-off and climb to FL370 should take up to 25 minutes and can cover up to 200nm, depending on Take-Off Weight. Descent and approach from FL370 should take about 20 minutes and cover 160nm. Take-off, climb, descent and approach should consume no more than 15000kg of fuel. A Final note, please remember the rules governing altitude for east/west flight, and rules regarding holding fuel and alternate fuel (about 7000kg typical).

IMPORTANT: Please help by reporting errors when using this table. If you cannot make the destination using the fuel load listed on this table please let me know. It could be for any number of reasons, including a miscalculation or two on my part!

| Distance nm (km) | Recommended Flight Level | IAS (mach) | Time (hrs) | Fuel Load |
| --- | --- | --- | --- | --- |
| 400 (740) | FL350-370 | 0.78-0.80 | 1:00 | 8350kg |
| 600 (1110) | FL390-410 | 0.81-0.82 | 1:25 | 10000kg |
| 800 (1480) | FL390-410 | 0.81-0.82 | 1:45 | 11900k g |
| 1000 (1850) | FL390-410 | 0.81-0.82 | 2:15 | 13900kg |
| 1200 (2220) | FL390-410 | 0.81-0.82 | 2:40 | 16000kg |
| 1400 (2590) | FL390-410 | 0.81-0.82 | 3:00 | 18000kg |
| 1600 (2960) | FL390-410 | 0.81-0.82 | 3:30 | 20000kg |
| 2000 (3700) | FL390-410 | 0.81-0.82 | 4:15 | 24500kg |
| 2400 (4440) | FL390-410 | 0.81-0.82 | 5:00 | 29000kg |
| 2800 (5180) | FL390-410 | 0.81-0.82 | 5:45 | 33500kg |
| 3200 (5920) | FL390-410 | 0.81-0.82 | 6:30 | 38500kg |
| 3600 (6660) | FL370-410* | 0.81-0.82 | 7:15 | 43500kg |
| 4000 (7400) | FL370-410* | 0.81-0.82 | 8:00 | 46000kg |
| 4400 (8140) | FL370-410* | 0.81-0.82 | 8:45 | 53000kg |
| 4800 (8880) | FL370-410* | 0.81-0.82 | 9:30 | 58000kg |
| 5200 (9620) | FL370-410* | 0.81-0.82 | 10:15 | 63500kg |
| 5600 (10360) | FL370-410* | 0.81-0.82 | 11:00 | 69000kg |
| 6000 (11100) | FL370-410* | 0.81-0.82 | 11:45 | 74000kg |
| 6400 (11840) | FL370-410* | 0.81-0.82 | 12:30 | 80000kg |

FL350-410* Indicates that the pilot should fly the initial stage of the route at FL370- FL390. Once 3200nm of the route remain eastbound pilots should climb make a gentle climb to FL410 (subject to clearance from ATC).

Take-off and Climb Data

Take off conditions: ISA, configuration flaps 1+F (two ‘notches’).

Note: In order to make a 'clean' take-off, add a couple of notches of trim (make sure that NumLock off and hit the '1' key on the keypad a few times).

| Take Off Weight | Runway Length | Flaps | V1 | VR | V2 |
| --- | --- | --- | --- | --- | --- |
| 170000kg | 2350m | 1+ (2 notches) | 155 | 160 | 165 |
| 190000kg | 2650m | 1+F (3 notches) | 160 | 165 | 170 |
| 210000kg | 2950m | 2 (4 notches) | 160 | 170 | 175 |
| 230000kg | 3250m | 3 (5 notches) | 165 | 170 | 175 |

Maximum takeoff power should not be maintained in excess of 5 minutes unless in the event of an engine outage, in which case the maximum operating time is 10 minutes.

Retract gear and flaps at 190kts and 200kts respectively, while accelerating to 250kts. Commence climb on instruction from ATC. Below FL100 maximum rate of climb at low weights is 4000fpm. 2500fpm is normal at high weights. After FL100 is reached level off and increase N1 to 90% until 320kts IAS is reached. While maintaining 320kts and 90% N1 proceed to climb to cruise altitude. Rate of climb will now be about 2000-2500fpm, depending on fuel load. Continue to climb, reducing rate of climb so that 320kts IAS and 90% N1 is maintained. At about FL270 the A330-200 will reach 0.78 mach and climb rate will be 1400-2000fpm. From this point maintain 0.78 mach until cruise altitude is reached, maintaining 90% N1 and continuing to reduce rate of climb as required. On reaching cruise altitude level off, maintaining 90% N1 until the A330-200 reaches 0.82 mach.

| Climb Altitude | Typical Rate Of Climb (MTOW) | Throttle | Fuel Use | Total Time | IAS |
| --- | --- | --- | --- | --- | --- |
| Initial Climb to FL100 | 2400 feet/minute | 90% | 5500kg | 5m | 250kts |
| Climb to FL270 | see above | 90% | 9000kg | 15m | 320kts |
| Climb to Cruise* | 1000 feet/minute | 90% | 10600kg | 25m | 0.78 mach |

* Assumes FL370

Cruise Data

The cruise tables are based on an economical cruise at 0.82 mach giving a true airspeed of between 460 and 475kts. On longer routes an altitude of FL370 will need to be maintained before climbing to FL390 or FL410 for the remainder of the journey. The cruise table shows the typical throttle settings that are required in order to maintain an economic cruising speed and typical fuel consumption/hour. Note that weather and load can have a significant effect on these figures.

| Altitude IAS TAS | FL310 0.76 mach 450kts | FL330 0.79 mach 460kts | FL350 0.80 mach 460kts | FL370 0.81 mach 465kts | FL390 0.81 mach 460kts | FL410 0.82 mach 470kts |
| --- | --- | --- | --- | --- | --- | --- |
| Power Fuel Use | 76% 5720 kg/hr | 76% 5160 kg/hr | 78% 4960 kg/hr | 80% 4750 kg/hr | 79% 4660 kg/hr | 84% 4640 kg/hr |
| Nm/1000 kg fuel | 79nm | 89nm | 93nm | 98nm | 100nm | 102nm |

Single Engine Operations

In the event of engine outage descend and maintain FL150 and reduce speed to 300kts. Dump fuel if over 185 tonnes gross weight and divert to nearest alternate. The A330-200 is certified for ETOP's operations of up to 180 minutes, meaning that throughout any route the aircraft must be within 180 minutes of a suitable alternate facility.

Descent and Approach

During descent from cruise altitude the pilot should maintain 0.78 mach with the engines on idle power until IAS reaches 290kts. At this point 290kts should be maintained down to FL100. Typical rate of decent during this time will be 2500fpm. Below FL100 the pilot should reduce IAS to 250kts or less, depending on ATC instruction. The rate of descent of approximately 2000fpm should be normal, although ATC may instruct the pilot to maintain altitude or a lower airspeed according to conditions. Vref approach speeds are as follows:

| Flaps at 1 (1 notch) | 230kts (max 240kts) | Intermediate Approach |
| --- | --- | --- |
| Flaps at 2 (4 notches) | 190kts (max 195kts) | Approach |
| Flaps at 3 (5 notches) | 180kts (max 180kts) | Approach |
| Flaps at 4 (6 notches) | 170kts (max 170kts) | Landing |

Touchdown should be at 200fpm for a smooth landing.

Landing gear should not be extended above speeds beyond 250kts or retracted at speeds in excess of 220kts. Tire Limit speed is 200kts.

FINAL NOTES

Please remember that these figures are NOT intended to be identical to the real world A330-200. They are, however, the closest that we can get with the limitations of FS98 as they are. I would like again to thank Camil Valiquette for his excellent work in designing this aircraft and for his kind permission for us to use it as part of the EuroDirect fleet. Camil has provided us with an excellent flight model to work with that required very little effort to modify for the EuroDirect operation. Congratulations Camil, a great aircraft!

Note to Virtual Airline's: If you want to upload or use this manual for your own aircrafts please contact EuroDirect. This way we can notify you of revisions and future releases:-)

Copyright: This manual is and will remain completely free. It may not be uploaded or placed in a compilation for which money is charged without the written permission of one of the EuroDirect management team.

Chris Williams: EuroDirect Fleet Planning and Development: Fleet.EuroDirect@ntlworld.com

EuroDirect Fleet WebSite: http://www.geocities.com/ed_fleet

Camil Valiquette: http://pages.infinit.net/camsim