# EuroDirect Dehaviland Dash 8 Q400 Operating Manual v1.0

20th October 2000

This operating manual is written for use with the following aircraft:

The EuroDirect Dehaviland Dash 8 Q400 by Barry Blaisdell & Jens Borgstroem

I-EDME

Original Aircraft created by Barry Blaisdell.

Flight Dynamics by Jens Borgstroem.

Re-Paint for EuroDirect by Chris Williams.

Updated Flight Dynamics Simon Bradshaw

Despatch sheets by Scott Clifford

Special Thanks to Helge Bast for Help and Advice!

#### EuroDirect would like to thank the original authors for allowing us to use their aircraft for this project. Without their work, permission and help the EuroDirect Q400 would not have been possible.

Welcome to the EuroDirect Dash 8 Q400 operating manual.  I hope you get as much enjoyment from its use as I have had, writing it. Please let me know if you have any feedback regarding the material in this manual. I would like to make corrections and release updates but I will only be able to do this with your help.

Dehaviland Dash 8  Q400 Technical Information:

First Flight: 			13th December 1997

Certified: 			26th January 2000

Flight Crew: 			2

Passengers: 			68-78 in single class layout

##### Powerplants

Engines: 			2 Pratt & Whitney Canada PW150A (free turbine turboprop)

Engine Rating			4580 shp take-off,  3947 shp max cruise

Propeller: 			Dowty R408 six blade, reversible pitch, composite.

Specific Fuel Consumption: 	0.481 lb/shp/hr at take off

Dry Weight			1 lb (4 kg)

Service Entry:			1995

## Weights

Maximum take-off:		63750 lbs (28917 kg)

Maximum landing:		61250 lbs (27783 kg)

Maximum zero fuel:		56500 lbs (25628 kg)

## Performance

Cruise Speed:			350 kts max, 320 kts normal, 310 kts economy

Cruise Altitude:			FL150 to FL250

Range:				1296 nautical miles (1492 miles or 2401 km)

Take-off field length:	minimum of 4430 ft (1350 m) @ MTOW, sea level, ISA + 10

Landing field length:		minimum of 4250 ft (1295 m) @ sea level, ISA.

Picture courtesy of Bombardier Aerospace

### EURODIRECT OPERATIONS AS OF October 2000

Currently the EuroDirect maintains two standard regional turboprop (category 1) aircraft.  These are the the Dehaviland Dash 8 Q400 and the ATR72-210.  This followed a decision to streamline the previous fleet and maintain only the the ATR72 in the 70 seat regional turboprop airliner category. Additional aircraft were leased and refitted at the new EuroDirect Maintenance Centre at Shannon to cover the shortfall created by the release of other aircraft in this category.

Following expansion in the Central European division to include the Berlin mini hub, further ATR72 were added together with the introduction of the Dash 8 Q400.

The Q400 was chosen after careful testing and analysis which showed it to be the best of the current fast turboprop designs available.  Customers interviewed following test flights gave very favourable feedback.  Coupled with excellent aircraft economics made the Q400 the natural choice for the Berlin mini hub expansion.

Depending on the results of ongoing cost and reliability analysis, the current mix of aircraft may be changed in the future.

PILOT INFORMATION

Note: Flight Simulator uses flight dynamics scalars that do not accurately represent those present in real world aviation. While the flight dynamics of the EuroDirect Dash 8 Q400 have been modelled as closely as possible on it’s real world counterpart the figures shown below are those of the FS98 EuroDirect Q400 on routes typically carrying 60 to 70 passengers in a single class layout. This equates to a zero fuel operating weight of 21910 kg. With the PW150A engines the EuroDirect Dash 8 Q400 may take off at any fuel load at this weight although the pilot must be careful to ensure that maximum landing weights are adhered to.

Flight Planning

Please use the following table for flight planning (it is designed to make the Hub Captain’s life easier). Distance is the total distance travelled in nautical miles. The remaining columns indicate the estimated time taken to make the journey in hours and the total fuel (including reserves, taxi requirements etc.) needed to make the journey. The figures are based upon MTOW/MLW and possible fuel loads. Obviously weather and ATC can cause these figures to vary and range can be extended by increasing altitude in the latter stages of the journey. Take-off and climb to FL190 should take approximately 12 minutes and cover about 60 nm. Descent and approach from FL190 should take about 8 minutes and cover 35 nm. Take-off, climb, descent and approach should consume no more than 775kg of fuel. A Final note, please remember the rules governing altitude for east/west flight, and rules regarding holding fuel (350 kg) and alternate fuel (375 kg typical for Europe).

IMPORTANT: Please help by reporting errors when using this table. If you cannot make the destination using the fuel load listed in this table please let me know. It could be for any number of reasons, including a miscalculation or two on my part!

| Distance nm (km) | Recommended Flight Level | IAS (kts) | Time (hrs) | Fuel Load |
| --- | --- | --- | --- | --- |
| 100 | FL110-150 | 275-257 | 0:25 | 1268 kg |
| 200 | FL110-150 | 275-257 | 0.52 | 1573 kg |
| 300 | FL110-210 | 275-234 | 1:13 | 1942 kg |
| 400 | FL190-250 | 241-217 | 1.44 | 2225 kg |
| 500 | FL210-250 | 234-217 | 2:05 | 2486 kg |
| 600 | FL230-250 | 226-217 | 2.25 | 2638 kg |

The above table shows the typical fuel burn for the Dash 8 Q400.  Note that while the Q400 has a maximum quoted range of 1296 nm, the Euro Direct configuration with a full load of passengers has a maximum range of about 600 nm.

Given issues such as comfort and speed, it is not realistic to operate the Q400 over distances greater than about 500nm.  For routes covering greater distances, the CRJ200 or the BAe 146 regional jets would be used.

For more exact determination of fuel load please use the Dash 8 Q400 flight despatch sheet.  This sheet can also be used to record actual fuel burn for later analysis by the Division Director.

Take-off and Climb Data

Warning: The PW150A engines are rated for full power for short periods only. N1 should not exceed 90% for greater than five minutes. Continuous use of high power settings should be reported as an inspection of the engines may be required.

Take off conditions: ISA, configure flaps at 15 degree.

| Take Off Weight | Runway Length | V1 | VR | V2 |
| --- | --- | --- | --- | --- |
| 24432 kg | 1200m | 120 | 125 | 135 |

Retract gear and flaps at 135 kts and 145 kts respectively, while accelerating to 270 kts IAS. Commence climb on instruction from ATC.

| Climb Altitude | Rate Of Climb | Throttle | Fuel Use | Total Time | IAS |
| --- | --- | --- | --- | --- | --- |
| Initial Climb to FL110 | 1900 feet/minute | 80% | 280 kg | 5m 30 | 275 kts |
| Climb to FL150 | 1500 feet/minute | 82% | 78 kg | 8m 10 | 257 kts |
| Climb to FL170 | 1200 feet/minute | 82% | 48 kg | 9m 30 | 249 kts |
| Climb to Cruise | 1000 feet/minute | 80% | 30 kg* | 11m 30 | Cruise IAS |

* Assumes FL190, FL210=78kg, FL230=102kg, FL250=150kg

Cruise Data

The cruise tables are based on an economical cruise giving a true airspeed of between 310 and 315 kts. The cruise table shows the typical throttle settings required in order to maintain economic cruising speed and the fuel consumption/hour. Note that weather can have a significant effect on these figures.

| Altitude IAS TAS | FL150 257kts 315kts | FL170 249kts 315kts | FL190 241kts 315kts | FL210 234kts 315kts | FL230 226kts 315kts | FL250 217kts 315kts |
| --- | --- | --- | --- | --- | --- | --- |
| Power Fuel Use | 70% 1021kg/hr | 70% 945kg/hr | 70% 869kg/hr | 71% 794kg/hr | 71% 718kg/hr | 71% 643kg/hr |
| Nm/500 kg fuel | 154nm | 166nm | 181nm | 198nm | 219nm | 245nm |

Descent and Approach

During descent from cruise altitude the pilot should maintain cruise IAS with the engines on idle power until IAS reaches 230 kts. Typical rate of decent during this time will be 2400 fpm. The rate of descent of approximately 2400 fpm should continue. ATC may instruct the pilot to maintain altitude or a lower airspeed according to conditions.

Vref approach speeds are as follows:

| Flaps at 0 degree | 200 Kts | Approach |
| --- | --- | --- |
| Flaps at 5 degree | 180 kts | Approach |
| Flaps at 10 degree | 165 kts | Approach |
| Flaps at 15 degree | 150 kts | Approach and Landing |
| Flaps at 35 degree | 130 kts | Landing |

Landing should normally occur at 120 to 125 kts.

Maximum Airspeed Limits

Maximum hull airspeed is 371 kts

Flaps limitations are 	200 kts IAS for 5 degrees,

181 kts for 10 degrees,

172 kts IAS for 15 degrees

158 kts for 35 degrees extension.

Landing Gear limitations are 200 kts IAS for retraction and 215 kts IAS for extension.

Maximum Tyre limit speed (ground speed) 165 kts

Notes on Flying the Q400

The Dash 8  Q400 is one of a new breed of fast regional turboprops .  It derives its performance with a combination of high tech airframe design coupled with powerful engines and efficient propellers.

Takeoff

Maintain the maximum power rating at takeoff (100%) until you have taken off and reached about 200 knots (or approximately 3 minutes has passed), whichever occurs sooner.

Do not hold the engines on high power (80% and above), more than is necessary.  The engines have more than ample power at the lower power settings.  Higher power settings just wastes fuel and creates excessive air speed which will need to be bled off.

FINAL NOTES

Please remember that these figures are NOT intended to be identical to the real world Dash 8 Q400. They are, however, the closest that we can get with the limitations of FS98 as they are. The statistics for fuel use etc. have been obtained using weather settings that accurately represent those of Italy and Southern France in Summer and Winter 2000 (but with no wind) at all altitudes.

Also please consider that there are differences between the flight (and weather) dynamics of FS98 and FS2K. Please let me know your thoughts on the flight dynamics in operational use.

Picture courtesy of Bombardier Aerospace

I would like again to thank Barry Blaisdell, Jens Borgstroem and Helge Bast for their work in designing this aircraft and for their kind permission for us to use it as part of the EuroDirect fleet. It really is their aircraft!

Copyrights:

This manual is free. It may not be uploaded or placed in a compilation without the written permission of one of the EuroDirect management team. It can be distributed so long as it is not altered in any way and it remains free. You may use and reproduce the information in whatever way you wish but please give credit to Barry, Jens and Helge, and to EuroDirect.

Simon Bradshaw: EuroDirect Fleet Operations Development:

ed_fleet@yahoo.co.uk

Chris Williams EuroDirect Fleet Consultant

The EuroDirect Fleet Website:

http://www.geocities.com/ed_fleet/

Barry Blaisdell:

http://www.flightsimmers.net/premaircraft

Jens Borgstroem: Duckling Air

http://www.danbbs.dk/~jbdk/duckling/