W.S. DARLEY & CO

FIRE APPARATUS CONTRACT

ENGAGER 1250 GPM PUMPER/TANKER

for

RATTLESNAKE FIRE PROTECTION DISTRICT

PARKER, COLORADO

TYPE OF APPARATUS

W.S. Darley & Co. is pleased to present to your department

for consideration our:

Darly Engager 1250 gpm 2400 gallon Pumper/Tanker Model fire

apparatus on a HME 1871 LFD 6 X 6 Custom Chassis.

Included on this apparatus shall be the:

Darley Champion Model LDM 1500 GPM Fire Pump

The apparatus meets the following design requirements:

The wheelbase shall be...............................243.00"

The Cab to Axle shall be.............................175.00"

The angle of departure shall be approximately........18 deg.

The overall length shall not exceed..................39'-7"

The overall height shall not exceed..................11'-11"

The water tank capacity shall be...................2400 Gal.

NFPA 1901 COMPLIANCE

The apparatus shall meet all current applicable NFPA 1901

guidelines. It is the intent of these guidelines to provide

for increased vehicle safety. These standards cover a wide

range of emergency vehicle safety related issues.

DELIVERY

W.S. Darley shall have this apparatus completed and ready

for delivery in November of 1999 as per bid specifications.

It shall be W.S. Darley & Co.'s responsibility to deliver

the completed apparatus to the Fire Department. It shall be

delivered under the vehicle's own power.

TRAINING

W.S. Darley & Co. shall provide training for fire department

personnel after acceptance of apparatus. Training shall be

conducted at the fire department's facilities. Training

shall be provided by factory trained personnel. The training

schedule will be worked out with the fire department

training office.

Training shall include all aspects on apparatus operation

and maintenance.

Training will consist of two (2) days of hands on

instruction at eight (8) hours per day if necessary.

ADDITIONAL TRAINING

Additional training specifically geared towards Darley fire

pump operation, maintenance, and repair shall be provided at

no cost to the Fire Department. This class covers in detail

CAFS (compressed air foam systems), chassis and pump

driveline, pumper service testing, and also includes a tour

of all Darley facilities. The W.S. Darley & Co. Pump School

is available twice a year; once in Spring, and again in the

Fall.

Lodging and Transportation to Chippewa Falls, Wisconsin are

the responsibility of the Fire Department.

PAYMENT TERMS

Payment for the value of the chassis, provided by W.S.

Darley & Co. is due at the within 30 days from execution of

this contract. The balance is due at time of delivery or

pick-up.

APPARATUS PRICE: The total for the three vehicles is

One Million, Six Hundred, Eighty-Five, Three Hundred, Forty

Three and 00/100 dollars($1,685,343.00), which Buyer agrees

to pay in accordance with the following payment terms. No

applicable, Federal, State, Parish or other taxes included.

A 5.33% charge will be added to base price if the decision

is to purchase one prototype vehicle.

PAYMENT TERMS: Payment for the value of the chassis, is due

within 30 day after contract exacution to the chassis

manufacturer. Upon receipt of payment for value of three

(3)HME Custom chassis in the amount of:

Four Hundred, Eight-Six thousand, Nine Hundred and 00/100

dollars ($486,900.00) by Seller, the Agreement balance due

is then:

One Million, One Hundred, Ninety-Eight Thousand, Four

Hundred, Forty-Three and 00/100 ($1,198,443.00). This

balance is due immediately upon delivery of apparatus by

Seller.

In the event there are shortages or deficient items upon

delivery, the Buyer may elect to hold back an equitable

amount equal to a maximum of two times the value of the

items deficient.

Reasonable progressive payments are due the Seller at

reasonable intervals as shortages and/or deficient items are

rectified by the Seller. The buyer will not put the

apparatus on line, in service, or use the apparatus without

the consent of the Seller at any point prior to receipt of

final payment by Seller.

This hold back does not change the terms of this Agreement

for any and all balances due.

Should payments, as a part of this agreement, not be

received by Seller, interest at the rate of 1-1/2% per month

will be due and payable the Seller, for each day receipt of

payment is delayed.

The Manufacturers Statement of Origin, or Title, shall be

held by the Seller until final payment has been received.

The cash payments made by Buyer (in the amount specified in

the preceding sentence) shall be to Seller or its authorized

agent; provided that no payment shall be made to an agent

except on presentation, in writing, of an express power of

attorney to accept payment.

The Buyer shall supply the seller with lending institution

information with returned contracts.

LIABILITY

W.S. Darley & Co. shall defend any and all suits and assume

liability for use of any patented process, device or article

forming part of the apparatus, or any compliance under the

contract.

PRODUCT LIABILTY INSURANCE

W.S. Darley & Co. has included in their Contract, a

Certificate of Product Liability Insurance for a minimum of

five (5) million dollars.

This certificate is an original and is made out to the

purchaser.

INSPECTION TRIPS

W.S. Darley & Co. is pleased to include the following FIVE

(5) inspection trip(s) for THREE (3) member(s) of the

Rattlesnake Fire Department per trip. W.S. Darley & Co.

shall bear all costs associated with the trips including

lodging and transportation.

CHASSIS INSPECTION

A chassis inspection trip to the chassis manufacturer to

verify chassis is built as required by the contract shall be

supplied.

PRE-PAINT BODY INSPECTION

A pre-paint body inspection trip to the manufacturer to

verify body is constructed as required by the contract shall

be supplied.

PRE-CONSTRUCTION MEETING

A pre-construction meeting shall be held at the

manufacturer's facilities within 30 days of contract

execution. Meeting shall be attended by representatives of

both the department and manufacturer. Manufacturer shall

supply approval drawings at this time, detail the

construction process, and review specifications.

MID-CONSTRUCTION INSPECTION TRIP

A mid-construction inspection trip to the manufacturer to

verify all construction details are correct as required by

the contract shall be supplied.

FINAL CONSTRUCTION INSPECTION TRIP

A final construction inspection trip to the manufacturer to

verify all construction details are correct as required by

the contract shall be supplied.

WARRANTY INFORAMTION

DARLEY WARRANTIES

W.S. Darley & Co. is pleased to submit the following

warranty terms.

APPARATUS WARRANTY

The apparatus, plating, paint, and all items furnished on

this apparatus under this specification shall be guaranteed

against defective workmanship and materials by the

contractor for a period of one (1) year from date of

acceptance by the Fire Department.

CAB & CHASSIS WARRANTY

The cab and chassis furnished on this apparatus under this

specification shall be guaranteed against defective

workmanship and materials by the contractor for a period of

one (1) year from date of acceptance by the Fire Department.

BODY WARRANTY

The Compolymer body of the apparatus including the structure

and paint furnished on this apparatus under this

specification shall be guaranteed against defective

workmanship and materials by the contractor for a period of

three (3) years, Rust & corrosion for service life of

vehicle, and mechanical hardware for one (1) from date of

acceptance by the Fire Department. A copy of this warranty

in full shall be provided to department.

DARLEY APPARATUS LIFETIME PUMP WARRANTY

The pump shall be delivered with a copy of the W.S. Darley

and Co. apparatus lifetime warranty. This warranty shall be

provided at no additional charge to the customer.

WATER TANK WARRANTY

The poly or fiberglass water tank provided shall be

warranted for the life off the apparatus as detailed in

the tank manufacturers warranty document.

A copy of the warranty statement shall accompany the

apparatus specifications.

PARTS

Replacement parts shall be made available within

approximately 72 hours for the life of the apparatus.

Manufacturer literature must show this.

DARLEY NETWORK SERVICE BLANKET

The W.S. Darley Company is committed to total customer

satisfaction and service of your new fire apparatus.

W.S. Darley & Co. has established service centers from coast

to coast and world wide to offer the support necessary to

maintain pump performance and also to take care of any

chassis or body related challenges.

In order to receive the most rapid response and proper

referral to your challenges call the Toll-Free Darley

Warranty/Service line at 1-800-323-0244 and ask for Howard.

LOCAL SERVICE

W.S. Darley is pleased to include in their specifications

the location of the nearest service center(s) to your

department in the event of required apparatus service.

The name of the service center is:    Advanced Fabrication

The distance from the Department is ________________________

The service center contact is:       Jerry Fullerton

Phone # is       303-469-1763       Fax # is        303-460-1365

Your contact at Darley MP is: Howard Kremske at 708-345-8050

or phone 1-800-323-0244    Fax # is 708-345-8993

Your contact at Darley CF is: Terry Hughes at 715-726-2645

or phone 1-800-634-7812    Fax # is 715-726-2648

SEATING CAPACITY

The seating capacity in the chassis cab shall accommodate

7 personnel.

PAINT NUMBER - THREAD DETAILS

The Buyer has carefully selected the color of the apparatus

to be:

(    CARDINAL RED S.H. 32256  SNOW WHITE S.H. 14600        )

Paint Color and Number if possible (Both if Two-Tone)

The Buyer relieves W.S. DARLEY & CO. of any obligation or

liability regarding the choice of color by the Buyer.

The Buyer is also aware of the recommendations of NFPA 1901

regarding colors and opinions expressed by other authorities

in the field of visibility, etc.

The specific thread details to be supplied on the fire pump

discharges and inlets to the fire pump are as follows: NST

is standard if left blank or not specified.

1" discharges are to have       NST      type threads

1-1/2" discharges are to have       NST      type threads

2-1/2" discharges are to have       NST      type threads

Small inlet connections are to have       NST      type threads

Large inlet connections are to have       NST      type threads

DETAILED APPROVAL DRAWINGS

Prior to construction, a set of drawings shall be supplied

to the Fire Department. These drawings shall include a left

and right side view along with a rear view of the complete

vehicle. The drawings shall include notations of all

critical dimensions including overall height, overall

length, width, pump panel width, and body dimensions. The

drawings shall also state water tank size and compartment

sizes. The drawings shall also contain proposed pump panel

layout and partial pump plumbing design configuration. One

of the drawings shall be signed and returned to the factory

indicating approval by the Fire Department for body

construction details.

REQUIRED DELIVERY INFORMATION

The manufacturer shall supply two copies of the following

information with the completed apparatus at the time of

delivery.

1. Owner's Name and address

2. Apparatus manufacturer, model and serial number

3. Chassis manufacturer, make, model, and serial number

4. Certificate of origin

5. GVWR of front and rear axles

6. Front tire size and total rated capacity in lbs.

7. Rear tire size and total rated capacity in lbs.

8. Chassis weight distribution in pounds with water and

manufacturer mounted equipment (front & rear)

9. Engine manufacturer, make, model, serial number, # of

cylinders, bore, stroke, displacement and compression ratio,

rated horsepower and related speed, and no-load governed

speed

10. Type of fuel, and fuel tank capacity

11. Electrical system voltage and alternator output in amps

12. Battery make, model, and capacity in CCA

13. Transmission make, model, and type

14. Pump to drive through transmission ( Yes or No )

15. Engine to pump gear ratio, and transmission gear ratio

used

16. Pump manufacturer, model, rated capacity in gallons per

minute, serial number, number of stages, and impeller

diameter in inches

17. Pump transmission manufacturer, model, serial number

18. Pump priming device

19. Type of pump pressure control system

20. Auxiliary pump manufacturer, model, rated, capacity in

gallons per minute, serial number, number of stages, and

impeller diameter in inches

21. Water tank manufacturer, certified capacity in gallons,

and warranty information

23. Paint numbers

24. Company name and signatures of responsible company

representatives

25. All applicable test results required by NFPA 1901-1996

edition Standard for Automotive Fire Apparatus

26. Engine manufacturer's certified brake horsepower curve

for engine supplied showing the maximum no-load governed

speed, required on apparatus with pump

27. A copy of apparatus manufacturer's approval for

stationary pumping

28. Weight documents from a certified scale - showing actual

loading on front and rear axle(s), and overall vehicle (with

water tank empty and full) Does not include personnel or

equipment

30. A complete book and video on pump operation and

maintenance

31. Wiring schematic for chassis and complete apparatus

32. Warranty information on complete apparatus, including

any components in which warranty period is beyond the

standard one year warranty

33. Unless otherwise specified, a minimum of one copy of

complete, as delivered, chassis operation and general

maintenance instructions, including, but not limited to

chassis, engine, transmission, axles, and lubrication

charts, shall be supplied.

CUSTOM CHASSIS TECHNICAL

SPECIFICATION SECTION

CHASSIS VOCATION AND BASIC ATTRIBUTES

BUILD CHASSIS TO NFPA 1996 STANDARDS

When completed this chassis will have the following

attributes:

Order Information:

Apparatus Builder: W.S. DARLEY & CO.

P.O.  Number: _____________________

Selling Dealer: FACTORY

Apparatus Builder Shop Order Number: 82137, 82138, 82139

User Information:

End User: RATTLESNAKE FIRE PROTECTION DIST.

Mailing Address: 46220 COAL CREEK DRIVE

City: PARKER

State: CO              Zip Code: 80134

F.D. Contact: ASST.CHIEF DALE GOETZ

Phone Number: 303-841-8111

Fax Number: 303-841-8111

Vocation:

_X_ - Pumper (With mid-ship water pump) *PTO DRIVEN PUMP

___ - Rescue (No water pump)

___ - Aerial (No water pump)

___ - Aerial (With mid-ship water pump)

___ - Aerial (Is HME supplying a Boom Support)

(When a water pump other than a conventional mid-ship type

is to be installed HME requires details of the installation

that would be necessary for interface with the chassis.)

(When an aerial device is installed over the cab, a

dimension of the width of the ladder over the cab roof must

be supplied for HME to insure clearance of all roof mounted

devices.  The ladder is __N/A__ inches wide over the entire

cab roof.

Also, when an aerial boom support is selected from HME a

drawing indicating the boom support interface dimensions

must be provided.)

Chassis Frame Dimensions:

256" - Wheelbase (centerline of front axle to centerline

of rear axle)

188.5" - Cab to Axle (rear edge of cab to centerline of

rear axle does not include clearance for cab tilt.)

94" - Rear Frame Overhang (centerline of rear axle to

the end of the frame)

Bumper Layout Information:

28" - Extension length

Speaker Quantity & Location:   1

Location of speakers:

___ - "SA" - Officer's side, extreme outside

___ - "SB" - Officer's side, next to frame rail

___ - "SC" - Officer's side, inboard of rail

___ - "SD" - Centered

_X_ - "SE" - Driver's side, inboard of rail

___ - "SF" - Driver's side, next to frame rail

___ - "SG" - Driver's side, extreme outside

Note: "SA" position cannot be used with a front suction

option.

Air Horns

There will be _0_ air horns mounted in the front bumper.

Location of air horns:

___ - "S1" - Officer's side, extreme outside

___ - "S2" - Officer's side, next to frame rail

___ -      - One horn, centered (21" extension required)

___ - "S3" - Driver's side, next to frame rail

___ - "S4" - Driver's side, extreme outside

Note: S1 & S2 locations cannot be used with a front suction

option.

Dome Light Locations

SEE NOTE - Standard locations - ONE LIGHT TO BE LOCATED

IN POSITION H THE OTHER LIGHT IS TO BE PLACED OVER THE

DOGHOUSE ON DRIVERS SIDE.

_____ - Additional Clear Dome Lights

C & L - Additional BLUE Dome Lights

ONE BLUE DOME LIGHT TO BE LOCATED OVER THE DOGHOUSE ON THE

OFFICERS SIDE.  THIS LIGHT ALONG WITH ONE CLEAR DOME ARE TO

BE 6 TO 8 INCHES APART CENTERED OVER THE DOGHOUSE.

If more than one hosewell is ordered indicate on a separate

piece of paper the information for the other well.

Fuel Tank Inlet Position

Fuel filler location: (50 gallon tank only)

_X_ - 28" behind the centerline of rear axle

___ - 29" behind the centerline of rear axle

___ - 30" behind the centerline of rear axle

Fuel filler location: (68 gallon tank only)

___ - 30" behind the centerline of rear axle

Is there is a dimension from the centerline of rear axle to

the back edge of the fuel tank that is necessary for the

body installation. If so what is the dimension ____ inches.

Water Pump:

___ - Pump installed at    HME

___ - Temporary driveline installed by    HME   for pump

_X_ - Installed at apparatus OEM - NOTE HME TO PROVIDE

PRE-DRILLED PUMP MOUNTING HOLES PER WS DARLEY DRAWING

___ - None installed on finished apparatus

Pump Data:

Manufacturer: DARLEY  PTO

Model: PSP  1500 GPM

Ratio: 1.66

Location ahead of the C/L of Rear Axle: ________

If Hale Pump:  Long ______ or Short _____

If Darley Pump: Forward ______ or Aft _______

PTO installation:

___ - No PTO provisions will be used

___ - A PTO will be installed by the apparatus builder.

_X_ - TWO PTO's will be installed by HME.

(When a PTO is to be installed at the apparatus builder the

following information is required with the order.)

Location: 1 O'CLOCK / WATER PUMP

Ratio: 858XGFJP-B5XV  108%

Output:  DIRECT MOUNT

Location: 8 O'CLOCK / AMPS GENERATOR

Ratio: 266XDFJP-M5XD 123%

Output: DIRECT MOUNT

Estimated in-service weight including equipment, water and

firefighters:

Front Axle - _______________ lbs.

Rear axle(s) - _______________ lbs.

Does chassis have an overall height restriction? If so fill

in the blank.

Overall height restriction - ______________

___ - Inches ground to the top of the frame at the

centerline of the front axle when loaded to _______ lbs.

___ - Inches ground to the top of the frame at the

centerline of the rear axle when loaded to ________ lbs.

Are there minimum angle of approach or departure angle

requirements?  If so fill in the blank.

Minimum angle of approach - __________ degrees

Minimum angle of departure - __________ degrees

Paint Information

Paint Manufacturer: PPG

(Note: Insure you have selected 40-Q0-18 for paint other

than Dupont Imron)

Single Color: 40-Q0-15

Primary color: ___N/A___

Primary paint code: ___N/A___

Two Tone Color: 40-Q0-16

Upper paint color: SOFT WHITE

Upper paint code: DBHS 8000

Lower paint color: RED

Lower paint code: DBHS 71660

Paint Break Line (Scheme #): 3A

(Note: If option 40-Q0-19 (Custom Two-Tone Paint) is

selected a drawing, with dimensions, must be attached for

order entry.)

Interior Paint Code: * ___N/A___

*Required if option 40-QA-14 is selected

Color Painted Frame Code: *___N/A___

*Required if option 40-Q0-11 is selected

Color Painted Rims Code: *___N/A___

*Required if option 40-Q0-17 is selected

High Idle

Standard Preset 1400 RPM or  ________  RPM

(Note:  Standard preset will used unless otherwise

specified.)

Load Management

The following chart must be filled in if a load management

system has been selected on the chassis by HME.

Switch        Function           Load        Turn-On

Pos.         (Describe Use)     Managed     Sequence

M         _________________    __Y__N     ________

1         _________________    __Y__N     ________

2         _________________    __Y__N     ________

3         _________________    __Y__N     ________

4         _________________    __Y__N     ________

5         _________________    __Y__N     ________

6         _________________    __Y__N     ________

7         _________________    __Y__N     ________

8         _________________    __Y__N     ________

9         _________________    __Y__N     ________

10         _________________    __Y__N     ________

11         _________________    __Y__N     ________

12         _________________    __Y__N     ________

13         _________________    __Y__N     ________

Load managers can load manage up to a certain number of

loads.  Check the option you have provided on this chassis

and select the correct number of managed loads.  Also,

sequencing of loads is only available on certain options

check if your option offers this feature.

Review your specifications for switches required by HME for

items such as heated mirrors, Jake brakes, etc.  These

options will in some cases require the use of overhead

switches.  If these switch positions are required it will be

indicated in the written description of the option.

CUSTOM FIRETRUCK CHASSIS

The chassis shall be designed and manufactured by a custom

chassis manufacturer. The manufacturer shall demonstrate

evidence of manufacturing similar custom vehicles for at

least fifty (50) years.  The chassis must be the apparatus

manufacturer's top of the line custom chassis.

The chassis shall be designed and manufactured for heavy

duty fire service with adequate strength and capacity for

all components as detailed within these specifications.

CHASSIS FRAME

The frame shall be designed to industry standards.  The

manufacturer shall provide a life time warranty to the

original purchaser of the chassis.  The frame rails shall be

10.5" X 3.5" X .375" heat treated steel.

A 3/4 length inner frame liner with dimensions of 9.687" x

3.125" x .3125" shall be provided for additional strength

and reduce deflection. The frame liner shall extend from the

centerline of the front axle and taper 45 degrees forward

and shall extend to the rear of the main frame rail.

The rails shall be 110,000 psi minimum yield and shall have

a minimum section modulus of 30.38 cu. in., in the frame

liner area, calculated by using the square corner shape

method.  The resulting frame rail resistance to bending

moment shall be 3,341,800 in. lb. per rail.  The

crossmembers shall be bolted in place using grade 8 bolts,

hardened washers, and grade "C" distorted thread locknuts.

Flanged head fasteners and nylon locking nuts are not

acceptable.  The top of the frame rails shall be free of

bolt heads.

Frame engine cutouts shall be made with a plasma torch to

minimize the heat affected zone of the cut.  All cutouts

shall have a minimum of 6 inch transitions between rail

flange cut depths to reduce the stress concentrations

throughout the cutout area.  The root of all transition

areas shall have a minimum of a 2 inch radius to reduce

stress concentrations at the root.

There shall be one (1) additional channel type crossmember

mounted in the wheelbase section of the chassis.

BUMPER EXTENSION

The front frame extension shall be bolted directly to the

main rail.  The extension and main rail joint shall have a

3/8" thick side plate for reinforcement.  The completed

apparatus must be able to be lifted at the front bumper

without structural damage to the front extension for towing

of a disabled vehicle.

The front bumper shall have a structural steel backing

channel reinforcement.  This reinforcement must be bolted to

the front frame extension and the bumper bolted to the

bumper backing reinforcement.

The front bumper face shall extend 28 inches ahead of the

front face of the cab skin.

FRONT BUMPER

A 12" high heavy-duty 10 gauge, polished stainless steel,

wrap around, 2-rib front bumper shall be provided the full

width of the cab.

TOW HOOKS

Two (2) chromed tow hooks shall be provided and shall be

attached directly to the front frame extension under the

bumper.  These tow hooks shall be attached with two Grade 8

bolts with hardened washers and Grade "C" distorted thread

locknuts.

FRONT DRIVE STEER FOR ALL WHEEL DRIVE CHASSIS

The front axle shall be an MARMON-HERRINGTON model "MT-22"

with a 20,000 lb. capacity.

The front drive steer axle shall have a built in caster of

three degrees, a camber of 1/2 degree, and a toe-in of

1-1/16 inch.

The steering joints shall be a "Double Cardan" type

connection.

The front axle shall be equipped with oil bath type oil

seals.

The front brakes shall be EATON 16-1/2" x 7", S-Cam, air

operated with 24 sq in. chambers and automatic slack

adjusters.

The front axle assembly shall be capable of a thirty five

(35) degree cramp angle.  Actual front cramp angle to be

limited by tires installed on the axle.

Front springs shall be a minimum of ten (10) leaf elliptical

type, 53" x 3-1/2" x .499" forged steel.  Capacity at ground

of 18,000 lbs.  The spring rate shall not exceed 3,000

lbs/in deflection.  The front springs shall have a military

wrapper for safe operation.

All front spring pins shall have grease fittings for

lubrication.

Double acting hydraulic shock absorbers are to be installed.

These shocks shall have an effective piston diameter of

1.48"

The steering shall be equipped with a dual ROSS TAS-65 /

RCS-65 integral power steering gears.

The rims attached to this axle shall have 10-Stud x 13.188"

bolt circle.

CRAMP ANGLE

The chassis shall have a turning cramp angle of 35 degrees.

WARRANTY STANDARD AXLE WARRANTY

The Marmon Herrington axle shall have a standard 1 year

12,000 mile parts and labor warranty.

REAR TANDEM AXLE

The rear axle shall be ROCKWELL model "RT-46-160" wide

track, for super single tires, in a tandem axle assembly.

An inter-axle differential lock shall be provided with an

air operated control on the driver's dash.  This control

shall include a light to indicate the inter-axle

differential lock is engaged.

The rear axles shall be equipped with oil bath type wheel

end seals.

VEHICLE TOP SPEED

The rear axle shall be geared for a top speed of 62 to 65

mph at engine governed RPM.

TANDEM AXLE REAR SUSPENSION

The rear suspension shall be HENDRICKSON "RT" model, rated

to match the capacity of the rear axle.  The suspension

shall have 52 inch long walking beams.  The suspension shall

have fore-and-aft torque rods for axle housing stability.

The rear suspension crossmember shall be a 1/4" thick 50,000

psi yield full liner through the suspension area with a

double channel torque rod crossmember bolted into the liner

to form a rear suspension "H-member".

DRIVER CONTROLLED DIFFERENTIAL

A ROCKWELL driver controlled locking differential shall be

supplied for the tandem rear axles.  Both differentials in

the "145" or "160" tandem axle shall have the locking

differential.

A single control on the driver's dash shall operate the

locking differential.  A lamp on the dash shall illuminate

to indicated that the differentials are locked.

REAR AXLE BRAKES

The rear brakes shall be 16-1/2" X 7", S-Cam, air operated.

The rear brakes shall be equipped with automatic slack

adjusters.

The rear axle shall be equipped with automatic slack

adjusters.

ROCKWELL STANDARD AXLE WARRANTY

The Rockwell axle shall have a standard three (3) year part

and labor warranty.  The axle will also have an additional

two (2) year parts only coverage.  Rockwell will provide a

one (1) year parts and labor warranty for wheel seals.

AIR SYSTEM

An air brake system meeting the requirements of the FMVSS -

121 shall be provided.  The system shall consist of three

(3) reservoirs with a 4,362 cu. in. volume.  The air system

shall consist of the following components:

Dual air system with a dual needle gauge, warning light and

buzzer.  A spring actuated parking brake built into the rear

axle brakes with a manual control and warning light the in

cab.  These shall automatically apply in case of air system

failure.  A mechanical means of releasing the spring brake

shall be provided in the event of total loss of air

pressure.

A quick build up system shall be provided, capable of

building enough air pressure to release the spring brake in

less than thirty (30) seconds, when starting with the entire

air system at zero pounds pressure.

The brake system shall be a split system.  One (1) system

serving the rear brakes and one (1) system serving the front

brakes.  The two (2) systems shall be connected with a

double check valve that will automatically shuttle air from

the front system to the rear system should loss of air

pressure occur.  This system shall also modulate the amount

of air so the spring brakes will apply in direct

relationship to the amount of pressure applied to the

treadle valve.

The spring brakes shall be piped in such a manner that if

the treadle valve is depressed while the spring brakes are

applied, the spring brakes will release and remain released

as long as the treadle valve is depressed.  They shall

reapply immediately when the treadle valve is released.

The piping in the air system shall be 2-ply nylon reinforced

color coded tubing for all stationary lines.

All air reservoirs shall have manual drain valves with

rubber seats to reduce air valve leaks.

AIR DRYER

The air system shall include a ROCKWELL / WABCO System Saver

1200 air dryer.  The dryer shall have a capacity of 30 CFM

of air flow.

The air dryer shall have a spin on dessicant cartridge for

ease in servicing the dryer dessicant.

The air dryer shall incorporate an integral turbo cut-off

valve.  The turbo cut-off valve shall close the path between

the air compressor and the air dryer purge valve during the

compressor "unload" cycle.  This will allow the air dryer to

purge the water and contaminates without any loss of turbo

boost or engine horsepower.

A 12 volt, 100 watt heated moisture ejector shall be an

integral part of the air dryer.  This heater shall be

thermostatically controlled.  The electrical connection for

the heater shall use a sealed electrical connector to

protect against moisture and corrosion.

ROCKWELL/WABCO ABS BRAKE SYSTEM

A four channel, single rear axle model, ROCKWELL/WABCO ABS

Braking System shall be supplied.

This electronic system shall monitor and control wheel speed

during braking.  This ABS system shall be divided in two

circuits or    diagonals   to control specific areas of the

vehicle.     Diagonal 1   shall control the right front and the

left rear wheels.     Diagonal 2   shall control the left front

and the right rear wheels.

If a fault occurs in one diagonal, the other diagonal shall

continue to provide the ABS function.  If the ABS system

should fail completely, the brake control shall be returned

to normal (non-ABS) braking.

If the chassis is equipped with an electronically controlled

engine, this 4-wheel system shall also have the ATC

anti-slip feature.  The ATC shall prevent the drive wheels

from spinning when starting from a standing position, or

when accelerating.

An ABS warning light shall be installed on the driver's

dash.  This warning light shall cycle through a test stage

at the point of ignition turn on and remain illuminated

until the vehicle reached approximately four (4) MPH.  The

light shall illuminate in other conditions to warn of an ABS

system failure.

With electronic engines an ATC (spin-indicator) light shall

be installed on the driver's dash.  This light shall

illuminate when the drive wheels spin during acceleration.

An ABS system blink code light shall be installed on the ABS

power panel.  This light shall illuminate when the

diagnostic function is activated.  This feature shall

indicate what ABS failure is occurring.

ON-BOARD ELECTRICAL AIR COMPRESSOR AND BATTERY CHARGER

A KUSSMAUL AUTO AIR model 091-9-1200 air compressor with an

40 amp automatic battery charger shall be supplied on the

chassis.

A pressure switch shall be installed on the air compressor

to sense the system pressure.  This switch shall start the

compressor which runs until pressure is restored.

The battery charger is permanently wired to the chassis

batteries to provide battery charging and operation of the

on board air compressor.

A 120 VAC electrical receptacle connection shall be

installed beneath the driver's seat next to the battery

electrical disconnect switch.

TANDEM AXLE AIR TANK

One (1) additional 2181 cubic inch air reservoir shall be

added to the chassis for the tandem axle brake requirements.

FRONT TIRES

The front tires shall be 385/65R22.5-18PR (J) GOODYEAR G-286

all weather tread, tubeless radial tires. These tires shall

be mounted on 22.5" x 12.25" rims.

Front axle GAWR shall be 20,000 LBS. @ 120 psi.

REAR TIRES

The rear tires shall be 385/65R22.5-18PR (J) GOODYEAR G-286

all weather tread, tubeless radial tires. These tires shall

be mounted on 22.5" x 12.25" rims.

Rear axle GAWR shall be 37,480 LBS. @ 120 psi.

FRONT STEEL RIMS

ACCURIDE steel disc wheels shall be supplied on the front

axle.

REAR STEEL RIMS

"ACCURIDE" steel disc wheels shall be supplied on the rear

axle.

ENGINE COOLANT RADIATOR

The engine coolant radiator shall have sufficient capacity

to perform under the engine manufacturer's installation

requirements.  The chassis manufacturer shall demonstrate

the ability to meet this requirement with the submittal of

an approved EPQ to the fire department for the apparatus.

This radiator shall have stainless steel top and bottom

tanks. These tanks shall have a material thickness of 16

gauge. The top and bottom tanks shall be attached to the

header assemblies with a minimum of forty-two (42)

fasteners. These fasteners shall not exceed a center

distance of 1-3/4 inches to reduce the possibility of tank

leaks.  These fasteners shall be torqued to a value of 10 to

12 ft-lbs.

The header plates shall be made of 16 gauge brass.

The radiator tubes shall be constructed of .0068 inch thick

brass and have a dimensional size of .076 inch x .625 inch.

These radiator tubes shall have welded tube seams.

The radiator shall contain four (4) rows of tubes arranged

in an inline profile across the radiator core.  The entire

radiator shall a contain (184) tubes.  These tubes shall

have a smooth bore to allow for radiator cleaning.

In the critically stressed area, where the radiator tubes

are attached to the header plates, this joint shall be

accomplished with a welding process on the coolant side.  In

addition to the welded joint a solder fillet joint shall

occur on the air side of the core creating a continuos dual

bond.

The radiator shall have a louvered serpentine type core that

contains fins constructed of .003 inch thick copper.  These

fins shall be spaced to a maximum density of 14 fins per

inch of radiator tube.  Each fin shall have a louvered

surface for high cooling efficiency.

The radiator shall contain an     integral     coolant de-aeration

tank.  This tank shall be designed to remove entrapped air

or gas from the coolant side of the radiator.

The radiator side rails shall have integrally designed

support gussets for the transition to the header attachment.

The bottom tank of the radiator shall have a drain valve for

coolant removal.  The top tank of the radiator shall have a

glass sight glass for coolant level inspections.  This sight

glass shall be able to be seen without tilting the cab.

The coolant system shall contain an ethylene glycol and

water mixture to keep the coolant from freezing to a

temperature of -34 degrees F.

The bottom tank of the radiator shall have a transmission

cooler with a plate-type design.  The plates shall have

internal turbulators to break up laminar oil flow across the

surface.  The cooler shall have 1175 square inches of

surface area for water surface contact and heat transfer.

The radiator system shall be a pressurized with a cap rated

per the cooling system requirements of the specific engine

manufacturer.

The high efficiency engine fan shall be encompassed with a

radiator shroud to provide the proper air flow from the fan

blade to the radiator.

The perimeter of the radiator shall have recirculation

baffles to eliminate the possibility of recirculation of

"hot" air to the face of the radiator core.  The bottom of

the radiator shall have a recirculation baffle from the

radiator to the frame rails.

CHARGE AIR COOLER RADIATOR

The engine charge-air cooler shall have sufficient capacity

to perform under the engine manufacturers installation

requirements.  The chassis manufacturer shall demonstrate

the ability to meet this requirement with the submittal of

an approved EPQ to the fire department for the apparatus.

This radiator shall have cast aluminum side tanks.  These

tanks shall have a material thickness of .200.  These tanks

shall be attached to the charge-air core with the ALBRAZE

construction technique.

The external air fins shall be louvered serpentine and

constructed of .006 inch thick aluminum.

The internal air fins shall be of the lance-and-offset

design for greater air turbulence and higher efficiency. The

internal fins are to be constructed of .010 inch thick

aluminum.

The charge-air cooler shall be mounted directly in front of

the engine coolant radiator.  To reduce vibration rubber

"iso" mounts shall be used for mounting of the charge-air

cooler to the engine radiator.

The charge air cooler shall contain (12) rows of internal

fins within a .313 x 2.632 aluminum tube assembly.  This

tube assembly shall be constructed of .025 thick aluminum.

The charge-air cooler shall contain thermal expansion slots

to allow the expansion and contraction of the charge-air

core over the wide range of temperatures that are expected

in operation.

The charge air piping between the engine and charge-air

cooler shall be aluminum tubing with a wall thickness of

.065 inch.  The system shall utilize four (4) ply silicone

rubber woven Nomex hoses with stainless steel pressure

bands.  These bands are designed to maintain the hose shape

under the pressure of the turbocharger boost air.  All

clamps used on the charge air piping are to be stainless

steel constant torque and shall be installed at each joint.

DIESEL ENGINE

The chassis shall be powered by a DETROIT DIESEL engine as

described below:

MODEL: Series 60 11.1L

NUMBER OF CYLINDERS: Six

BORE AND STROKE: 5.12" X 5.47"

DISPLACEMENT: 677 cu. in.

RATED BHP: 350 @ 2100 RPM

TORQUE: 1350 @ 1200

COMPRESSION RATIO: 16.0:1

GOVERNED RPM: 2100

PEAK HP: 365 @ 1650

Standard Equipment on the engine to include the following:

GOVERNOR: Electronic Controls

TURBOCHARGER: Wastegate design for increased boost at lower

engine speed. Ceramic turbine wheel

INJECTORS: Electronically controlled unit type

AIR CLEANER: Donaldson or equal with fresh air intake

OIL FILTER: A full flow and by-pass

LUBE OIL COOLER: Non-drainback, thermostatically controlled

with full flow cooling

FUEL FILTERS: Two fuel filters providing 30 micron primary

and 12 micron secondary absolute filtration with check

STARTER: A Delco, 12 volt, 42 MT type 400 starter motor

AIR COMPRESSOR: A 16 cfm compressor will be provided.

The chassis shall be equipped with electric controls for

vernier throttle operation.  The chassis shall be supplied

with a vernier throttle wiring harness to the pump panel

area.  This system when completed by the apparatus

manufacturer shall be in compliance with NFPA 1901.

ENGINE WARRANTY

The engine shall have a 5 year warranty provided by the

engine manufacturer.

ENGINE BRAKE

A JACOBS engine brake shall be supplied.

The Driver's dash shall include an ON/OFF and 3 position

brake control switch.

The brake lights shall illuminate when the Jacobs Brake is

in operation.

The Jacobs Brake shall be inoperative when the chassis is in

pump mode.

ENGINE FAST IDLE

The chassis must be equipped with an Electronic Idle Control

(EIC) for the electronic engine.  Preset speed is factory

adjustable.

The fast idle provision will only function when the parking

brake is set and the transmission is in neutral.

Control of the fast idle is by an overhead console mounted

switch.

DETROIT PRESSURE SENSITIVE GOVERNOR CONTROL SYSTEM

The chassis shall be equipped with DETROIT DIESEL Pressure

Sensitive Governor (PSG) system.  A pressure transducer is

installed in the fire pump discharge chest.  This transducer

shall continuously monitor pump pressure and send a signal

to the engine Electronic Control Module (ECM).  The PSG

system shall directly control engine rpm or pump discharge

pressure.  In the pump pressure mode the Fire Commander

shall vary the engine RPM to maintain a constant pump

discharge pressure.

The "Electronic Fire Commander" control unit shall be

mounted on the apparatus pump panel that enables the

operator to select either engine speed or pressure operation

of the electronic pump control system.

The "Electronic Fire Commander" operates the PSG and

displays all vital engine operating parameters such as

engine temperature, oil temperature, engine RPM, voltage and

fuel level.

CORRISON INHIBITOR

Nalcool corrison inhibitor will be installed in the chassis

cooling system.

COOLANT HOSES

The entire chassis cooling system shall have premium rubber

hoses. All clamps to be stainless steel worm drive type

clamps.

COOLANT ADDITIVE

The cooling system shall have NALCOOL Coolant additive

protection.

EXHAUST SYSTEM

A single exhaust pipe shall be provided for the engine.  The

aluminized muffler shall be located under the frame on the

right side of the apparatus.  The tail pipe shall terminate

on the right side directly ahead of the rear wheels.  The

tube shall be cut on the end with a 45 degree cut.

CHROME TAILPIPE

A chrome plated exhaust tailpipe tip shall be furnished. The

tip is to be clamped to the end of the tailpipe and have a

45 degree bend to direct exhaust downward.

TRANSMISSION

The transmission shall be an Allison HD4060(P) automatic

transmission with electronic controls.

The transmission shall be equipped with a lock-up control

circuit that will automatically shift the transmission into

4th gear lock-up when the pump is shifted into gear.

TRANSMISSION COOLER

An automatic transmission cooler shall be provided as an

integral part located in the bottom tank of the radiator. It

shall be designed to withstand 165 psi working pressure and

an intermittent pressure of 250 psi.  The cooler shall be of

sufficient size to maintain the operating temperature within

the recommended limits of the transmission manufacturer. Due

to the complex nature of today's engine cooling systems.

FIVE SPEED HD TRANSMISSION

The transmission shall programmed for five speeds. The

transmission shall have the following gear ratios.

First - 3.51

Second - 1.91

Third - 1.43

Fourth - 1.00

Fifth - 0.74

Reverse - 4.80

The chassis will be geared for the top speed in 5th gear.

TRANSMISSION SHIFTER

The transmission shall be controlled by a lever type shift

control.  It shall be internally illuminated for night

operation and have an internal lock to securely hold it in

the position selected.

TRANSMISSION PTO

A clutched drive Chelsea 10-bolt standard duty engine driven

power take off shall be furnished by the chassis

manufacturer.  It shall be installed in the left-down

position on the HD transmission.  The PTO ratio shall be

determined by the apparatus manufacturers application.

The output side of the PTO shall have a 1410 Series Spicer

flange.

A PTO engagement switch shall be furnished in the cab.  It

shall have a neutral switch interlock to prevent engagement

unless the transmission is in the neutral position.

Available         Maximum

Ratios             Torque

109%             460 lb-ft

140%             390 lb-ft

165%             335 lb-ft

TRANSMISSION PTO

A clutched drive Chelsea 10-bolt standard duty engine driven

power take off shall be furnished by the chassis

manufacturer.  It shall be installed in the left-down

position on the HD transmission.  The PTO ratio shall be

determined by the apparatus manufacturers application.

The output side of the PTO shall have an SAE "B" pad or a

keyed 1.25" shaft.

A PTO engagement switch shall be furnished in the cab.  It

shall have a neutral switch interlock to prevent engagement

unless the transmission is in the neutral position.

Available         Maximum

Ratios             Torque

103%             268 lb-ft

123%             260 lb-ft

147%             240 lb-ft

184%             212 lb-ft

DRIVELINES

Universal joints and driveshafts shall be SPICER 1810

series.  The driveshaft tube shall be a minimum of 4.5"

diameter with a .259" tube wall thickness.  All drive-lines

shall be balanced to prevent driveline vibration.

ALL WHEEL DRIVE TRANSFER CASE

A MARMON-HERRINGTON transfer case shall be installed into

the all-wheel-drive drivetrain.  This transfer case shall be

a two (2) speed case with ratios of 1.91:1 for low range and

1.00:1 for high range.

The transfer case shall include an integral lube pump for

pressure lubrication of the gears.

Internal gears shall be helical in design for a constant

mesh and to minimize operating noise.

A front axle disconnect shall be supplied for dis-engagement

of the driveline to the front axle.

The cab shall have two selector air valves for operation of

the transfer case range and front axle disconnect.  Two (2)

red warning lights shall be installed on the cab dash, one

(1) to indicate low range in operation and one (1) to

indicate front axle driveline engaged.

STANDARD TRANSMISSION WARRANTY

The chassis shall have a five (5) year unlimited

mileage/Parts & Labor warranty for the Allison HD

transmission.

FUEL TANK

The fuel tank shall have a capacity of 50 gallons (US) and

be D.O.T. certified.  It shall be mounted with straps bolted

to the bottom frame flange to allow for easy removal.  The

tank construction shall be of 12 gauge steel.  The baffled

tank shall be vented to prevent low vacuum and facilitate

rapid filling.  A drain plug shall be provided in the bottom

of the tank.  The tank shall have a 2" NPT fill to the

driver's side of the chassis.

The fuel tank sending unit is to be mounted to the Driver's

side of the fuel tank for easy replacement.

FUEL LINES

NYLON reinforced rubber fuel lines with push-on fittings

shall be used.

FUEL FILTER

The standard Detroit Diesel primary fuel filter shall be

installed on the engine.  This filter shall provide 30

microns of filtration.  This filter does not contain a

fuel/water separator.

CHASSIS CAB STEPS

The steps for all four doors shall be

lowered to ease entrance and exit.  These steps shall be

exposed and constructed of a bright aluminum non-skid

surface.

CUSTOM FIRETRUCK CAB

The cab shall be capable of seating up to ten (10) fire-

fighters and be of a one-piece tilting, contoured front,

fully enclosed design.  It shall have four (4) side doors

and be a 96" wide custom aluminum cab-over engine forward

style.  The cab shall have an "Open Space" design, free of

interior walls or obstructions. A 24" high raised roof shall

be supplied over the crew area of the tilting cab.  The

raised section shall be encompassed with fixed windows,

three on the cab sides and two across the front face of the

raised section.

CAB MATERIALS

The cab construction shall have the following material

gauges as a minimum:

Cab floor - 3/16" (.190") aluminum

Front skin - 3/16" (.190") aluminum

Cab side panels - 3/16" (.190") aluminum

Cab rear wall - 3/16" (.190") aluminum

Cab roof - 3/16" (.190") aluminum

Cab doors - 3/16" (.190") aluminum

Roof Panel Rails - The roof panel assembly shall have

extruded hat section supports welded to the roof skin. These

roof hat sections shall be joined to the Cab Roof Rail

Section to complete the upper cab skeletal structure. These

completed Roof Panel Rails shall provide a grid for maximum

roof strength.  The roof shall support a minimum weight of

250 lbs. / sq. ft. without permanent roof deformation.

Rear Wall Rails - The rear wall assembly shall have extruded

hat section supports welded to the wall skin.  These

sections shall be joined to provide a rear wall grid

structure for maximum strength.

Cab Front Wall - The front wall of the cab shall be designed

with a double wall construction to reduce the effects of

exterior noise in the crew and operator compartment.

Engine Enclosure - The engine doghouse will be constructed

of welded aluminum and shall be welded into the cab as an

integral part of the cab.

CAB DIMENSIONS

The cab shall have the following overall dimensional

requirements:

Overall Width - 96 inches minimum

Center of front axle to back of cab - 67-1/2"

Center of front axle to front of cab - 74 inches

Windshield area - 3624 sq. inches minimum

Front Grille Opening - 478 sq. inches minimum

Side Grille Opening - 63 sq. inches each minimum

Cab full tilt angle - 45 degrees minimum

Cab full tilt height - 196 inches maximum

Cab interior dimensions shall be provided as a minimum in

the following chart:

Driver's Lower Step Size - 10-1/2" deep minimum

Driver's Lower Step Size - 30" front to back

Officer's Lower Step Size - 10-1/2" deep minimum

Officer's Lower Step Size - 30" front to back

Crew cab seating position:

Cushion to Roof - 62-1/4 inches minimum

FRONT CAB DOORS

The forward cab doors shall be 74" high x 37" wide and will

have roll down windows and fixed front corner windows.  The

front door windows shall have a minimum of 702 square inch

area of viewing glass per door.  Each window shall have an

exterior glass weather seal to prevent the influx of exterior

air.  The doors will have exterior and interior paddle

latches for ease of opening with a gloved hand.  The paddle

latches are to have a rubber gasket, on the outside,

separating the handle from the finished painted surface.

Each door shall be of the flush mounted design having

exposed, polished, one-piece, 12 gauge stainless steel piano

hinges with 1/4" hinge pins.

The cab doors shall be covered with an automotive styled ABS

panel that is covered with vinyl to match the interior trim

color.  The front door panels shall have map pockets in each

door.  The lower section of each door panel is to have a

diamond plate insert providing a "kick plate" for the

firefighter.

REAR CAB DOORS

The rear cab doors shall be of the "NO KNOCK" type and shall

be located directly behind the front wheel well area. These

doors shall be 84-3/4" high x 30" wide and will be a flush

type door with exposed, polished, full length 12 gauge

stainless steel piano hinges with 1/4" hinge pins.  Each

door shall have roll down rear windows.  The rear doors

shall have a minimum of 546 square inches of viewing area

per door.  Each window shall have an exterior glass

weather seal to prevent the influx of exterior air.  The

doors shall have interior and exterior paddle latches, and

shall be mounted in an easy to reach location.  Interior

latch shall not be blocked by the seat occupant. The paddle

latches are to have a rubber gasket, on the outside,

separating the handle from the finished painted surface.

INTERIOR DOOR LOCKS

All doors shall have interior door locks and exterior keyed

door lock controls.  The door locks and the finished door

assemblies shall be in conformance with FMVSS 206, with

specific adherence to 49 CFR 571.206 Section 4.1.3 requiring

that "Each door shall be equipped with a locking mechanism

with an operating means in the interior of the vehicle". All

doors shall be keyed alike.  The doors shall be equipped

with appropriate safety interlocks to prevent accidental

locking of the doors when closed.

WHEEL WELL LINERS

To reduce road splash underneath the cab and allow for easy

cleaning, aluminum front wheel well liners are to be

installed in the wheel wells.   The wheel well liners are to

be a minimum of 22 inches in width.

STAINLESS CAB FENDERETTES

To reduce road splash on the cab sides, polished stainless

steel fenderettes shall be installed across the top of the

wheel openings. An extruded rubber gasket is to be installed

between the fenderette and the cab to reduce the possibility

of electrolysis between the dissimilar materials.

CAB TILT, LOCK AND SUSPENSION SYSTEM

The cab shall tilt a minimum of 45 degrees for ease of

servicing.  Tilting will be accomplished by means of a tilt

pump connected to two (2) heavy duty lift cylinders.  It

shall be equipped with a positive locking mechanism (service

lock) to hold the cab in the full tilt position.  Release of

the service lock shall be by means of a pull type cable

assembly.  The cylinders shall have velocity fuses at the

base to prevent the cab from falling in the event of a

hydraulic hose failure.  The cab shall be capable of tilting

90 degrees for major engine service, if necessary.  The 90

degree cab tilt shall be accomplished by removing the cab

cylinder pins, removing one bolt in the steering shaft, and

removing the front bumper and tread plate.

The cab shall have a three (3) point cab locking system.  To

prevent undue stresses in the cab, the cab mounting shall

incorporate a five (5) point load mounting system.

The front cab pivot/lock assemblies shall utilize four (4)

radially loaded, bonded rubber, axial mounts.  These mounts

shall have a maximum radial load rating of 925 pounds each

and a torsional rating of 25 lbs-in/deg.  Two one (1) inch

diameter cab pivot pins shall be installed at the front of

the cab.  Each pivot pin shall have a grease fitting to

allow for lubrication to the pivot area.

The rear cab lock shall be center point mounted to prevent

normal twist of the chassis from affecting the cab mounting,

cab structure, and windshield areas of the cab.  This rear

cab lock shall be mounted on a chassis crossmember to

provide a stable platform for the locking system.  This

locking system shall automatically open prior to the cab

tilting and automatically relatch when the cab is lowered

completely into the travel position.

Two (2) outboard frame mounted urethene "V" blocks shall be

provided at the rear of the cab.  These dual purpose mounts

shall align the cab upon lowering as well as provide

non-latching support for the cab in the down position.  With

this system, extreme chassis twist will allow the cab to

move independently of the rear cab supports, reducing the

structural stress damage often caused by outboard dual cab

locking systems.

INTERIOR CAB STEP TRIM

The cab steps will be completely enclosed behind each door

and the toe kick surface and the horizontal step area shall

be covered with aluminum tread plate trim.

INTERIOR CAB TRIM

The cab front interior will have a one-piece, removable,

sound absorbing headliner to cover all wiring and tubing

used for lights and antenna leads.  The rear headliner shall

be a two-piece design similar to the front.

The rear interior wall of the cab shall have a one-piece,

removable, wall covering to finish the interior trim and

cover all wiring and tubing used for lights and antenna

leads.

The cab dash will be an ABS automotive styled housing with

vinyl covering.  An 18" long grab handle for the Officer

will be furnished on the right hand side of the cab.  The

instrument housings will be symmetrical in design for a

pleasant appearance.

CAB GLASS

AS-1 safety laminate glass shall be used in a two piece,

wrap around design with a minimum 3624 square inches of

windshield area for maximum visibility.

The windshield shall be a type which is readily available

from a nationally recognized automotive glass manufacturer

that maintains local distribution outlets.

All glass shall be tinted.

All fixed glass shall be installed with a one-piece triple

locked rubber lacing material.  Due to long term appearance

two-piece chrome trim lock lacing is not desired.

SUNVISORS

Two (2) 17-1/2" by 9" black padded sun visors shall be

supplied, one on each side of the windshield.  Vertical

adjustment shall be a minimum of 15" to allow maximum

coverage.

RADIO / STORAGE COMPARTMENT

Beneath the officer's seat there shall be a storage/radio

compartment approximately 19-1/2" wide x 17" long x 7" high.

The compartment shall have a diamond plate door mounted on a

piano hinge to prevent stored materials from spilling out.

Access through this door shall be from the side of the seat

box. An identical compartment shall be provided beneath the

driver's seat with the door on the front of the seat box.

SEAT BELTS

All seats shall be equipped with metal to metal quick

release seat belts with automatic seat belt retractors.

These seat belts and retractors shall be black in color.

HEATER / DEFROSTER

A 57,600 BTU heater with a three speed fan shall be mounted

in the front of the cab, centered over the windshield.  This

heater shall have six (6) adjustable vents to assure

windshield defogging.

WINDSHIELD WIPERS

Two speed electric pantograph wipers shall be installed.

These wipers shall have minimum 24" blades and have 28 1/2"

wet arm electric pump washers.  A 70 oz. minimum windshield

washer reservoir shall be furnished.  The reservoir shall be

mounted     inside the cab     to eliminate the need to tilt the

cab to fill it.

DRIVER INSTRUMENTATION AND CONTROLS

The cab dash panel will have black textured anti-glare

surface.  This panel will have a wrap around feature for

easy viewing of the instrumentation.  The dash panel shall

be a three piece design.  Each panel shall be removable for

access to the gauges and wiring.  The instrument panel will

include the following gauges and indicators:

Electronic speedometer with trip odometer

Dual air pressure gauge with low air pressure warning lights

and buzzer

Fuel level gauge

Air cleaner restriction indicator light

High beam indicator light

Parking brake set light

Turn signal indicator lights

High beam indicator light

"Cab Door Open" indicator light

The lighting control panel is to be located to the left side

of the instrument panel.  This panel will have a black

textured anti-glare surface.  The lighting control panel

shall include the following:

Dash rheostat for instrumentation lighting control

Wiper and washer control switches

The engine control panel is to be located beneath the

instrument panel on the driver's right hand side.  The panel

shall have a black textured anti-glare surface.  The engine

control panel shall include the following:

Keyless ignition switch with a green pilot light

Ignition switch(s) with battery green indicator light(s)

Parking brake control valve

The apparatus control panel is located beneath the

instrument panel on the driver's left hand side.  The panel

shall have a black textured anti-glare surface.  The

apparatus control panel be utilized for the pump shift

controls.

Due to the request of customer to remove certain gauge and

indicator lights, HME will take out these items as long as

they are not a requirement of D.O.T. of FMVSS.

STEERING WHEEL AND COLUMN

The steering column shall be a DOUGLAS tilt / telescopic

type with an integral high beam / turn signal control

switch.  The column shall have self canceling design for

the turn signal switch.  A 4-way warning "Hazard" light

switch shall be mounted on the column.  For safety, a rubber

boot shall be installed to cover the steering shaft from the

dash to the floor.

The steering wheel shall be a minimum of 18 inch diameter,

covered with a padded absorbite finish.  The telescopic

feature of the steering column shall be controlled by a

lever on the left side of the steering column.

EXTERIOR GRAB HANDLES

The cab shall have a bright anodized extruded aluminum 24"

grab handle with extruded rubber inserts at each door

position.  The aluminum shall be bright anodized for long

service.  Molded rubber gaskets shall be installed under the

grab handles to protect the painted surface of the cab.

EXTERIOR FASTENERS

All cab exterior fasteners shall be stainless steel type

fastened to the cab with nut serts.

ENGINE "WIDEMOUTH" DOGHOUSE

The doghouse shall have access through the top to perform

routine maintenance.  Through the opening, the engine oil,

transmission fluid, power steering fluid, engine coolant,

and fan belt tension can be checked, refilled and/or

adjusted.  The door shall open 90 degrees from the rear and

swing to the front of the cab. This door, shall be assisted

by high temperature gas springs.  The door shall be latched

by a positive locking mechanism located on each side of the

doghouse door.  The doghouse doors shall be sealed with the

same sealing method as the exterior doors of the cab.  The

entire doghouse inside the cab will be padded with foam and

covered with heavy duty vinyl upholstery.

The under side of the engine enclosure is to be covered with

a "Whispermat Barrier" for interior cab noise and heat

rejection.  This sandwiched acoustical material shall have

one layer of 1/8" foam, a 3/16" single barrier septum and a

7/8" layer of foam to provide an overall thickness of

1-3/16".  The sandwich material shall be chemically bonded

to prevent layer separation.  A finished surface treatment

of metalized film shall be provided on the engine side of

the barrier.  The acoustical barrier shall be held in place

with mechanical fasteners in addition to adhesive.

BATTERY ACCESS

The rear cab steps shall be hinged, providing access to the

batteries for routine maintenance and inspection.

CAB CORROSION TREATMENT

The cab shall have a corrosion preventative material

conforming with Mil Spec C-16173-C, Grade 1, applied during

and after construction.  A 10 year warranty against

perforation due to rust or corrosion shall be furnished for

the cab.

CAB GRILLE

All cab exterior grilles shall be bright finished stainless

steel.  The front grille shall have a radiator rock guard to

assist in preventing damage to the radiator core.

The cab shall have one (1) engine "hot" air exhaust and one

(1) engine air cleaner intake, on each side of the cab.

These openings shall be covered with a bright polished

stainless steel grille.

CAB GROUND LIGHTING

One light shall be mounted beneath each door.  These lights

shall be designed to provide illumination on areas under the

driver and crew riding area exits.  All cab ground lights

will automatically activate when any cab door is opened.

BACK-UP ALARM

An electronic Back-up Alarm shall be installed on the rear

of the apparatus and wired to the back-up light circuit.

Minimum decibel rating of 97 dba is required.

ELECTRIC HORN

An additional electric horn shall be mounted on the chassis

MIRRORS

Two (2) single piece bright finished stainless steel West

Coast Mirrors with 16-1/2" x 7" flat mirror heads shall be

supplied an additional 5-1/2" x 8-1/2" rectangular lower

convex mirror shall also be supplied.  These mirror heads

shall be mounted on spring loaded RETRAC arms.

Two (2)       INTERIOR CONVEX MIRROR

A rectangular convex mirror shall be mounted in the front on

the cab, to allow the driver to view the rear crew area.

RIGHT SIDE FRONT VIEWING MIRROR

A mirror shall be provided on the right front of the cab to

allow the driver to view the front bumper area.

CAB SIDE WINDOWS

Two AS-2 tempered glass, fixed side windows, 26-1/2" high x

16" wide will be furnished, one on each side behind the

forward doors. All glass shall be tinted. These windows

shall be installed with a one-piece triple locked rubber

lacing material.

COMPARTMENT OPEN LIGHT

A WHELEN Cadet revolving red light shall be mounted overhead

wired as an open door warning light.

This door open light is wired to the power panel for

bodybuilder completion to the compartment door open circuit.

The light shall be wired through the parking brake circuit

so that the light illuminates when a cab door is open and

the parking brakes are released.

INTERIOR CAB LIGHTING

Two (2) cab interior swivel map lights shall be supplied

mounted in the front overhead console, one each side of the

heater.  These lights shall be controlled by individual

switches mounted on each light.

Four (4) step well lights shall be supplied (SFD cabs shall

have two (2).  These lights, one in each step well, shall

have a light hood to direct the light downward toward the

step and ground.  All step well lights shall be illuminated

when any door is opened and the battery selector switch is

on.

Two (2) seven inch clear dome lights shall be supplied.  One

light shall be installed in the front of the cab centered

over the "Widemouth Doghouse".  One light shall be installed

centered over the rear crew area.  These lights shall be

illuminated when any door is open or individually operated

with a switch mounted on the light and the battery switch is

in the on position.

ADDITIONAL DOME LIGHTS

Four (4) additional seven inch blue dome lights shall be

supplied.  These lights shall be operated by individual

switch mounted on each light.

UNDER CAB ENGINE MAINTENANCE LIGHTS

Two (2) engine maintenance lights shall be supplied beneath

the cab.  These lights shall illuminate automatically when

the cab is tilted to the full tilt position.

ICC MARKER LIGHTS

Five (5) cab face mounted clearance lights shall be

supplied, mounted above the windshield, in conformance with

FMVSS 108.

ICC MARKER LIGHTS

Two (2) side combination clearance / turn signal lights

shall be supplied, one (1) each side mounted ahead of the

front door.

HEADLIGHTS

Four (4) rectangular halogen headlights shall be supplied

mounted in a chrome plated bezel.  These headlights will be

mounted in the upper position on the front of the cab.

TURN SIGNALS

Two (2) rectangular halogen turn signal lamps shall be

mounted on the bumper face. These lights shall be supplied

with an amber arrow cover.

FRONT WARNING LIGHTS

Two (2) RED Code 3 Oscilaser warning lights shall be

supplied mounted above the headlights in a chrome plated

bezel.  These lights shall be supplied with red transparent

covers.

CAB FRONT WARNING LIGHT CONTROLS

Control of the Oscilaser warning lights shall be by the

Master Switch for both "Calling of Right of Way" and

"Blocking Right of Way" modes.

HEAD LAMPS "ON" IGNITION CONTROL

When the ignition switch is in "on" the head lamps shall be

illuminated to 80% brilliance as required by Canadian Law.

REAR WALL LIGHT

A white light shall be provided on the rear wall of the cab.

Light shall be centered on rear wall and mounted just below

drip rail.

Light shall be activated thru the parking brake.

HIGH BEAM HEADLIGHT WIG-WAG FLASHER

A high beam headlight flasher shall be wired into the

headlight system and shall include an override system if the

high beam headlights are required.

EXTERIOR WALL DIAMOND PLATE

The cab exterior rear wall shall be covered with a single

sheet of bright aluminum tread plate to protect the back of

the cab from scratches.

TILT PUMP

An electric-over-hydraulic cab tilt pump shall be supplied.

This pump shall have a remote control for cab tilting

operation.  The control shall be "safety-yellow" in color.

ACRYLIC URETHANE FINISHES

The manufacturer shall warrant the Acrylic Urethane finishes

on a fire and emergency vehicle for a period of five years

from its date of delivery.

This warranty shall apply only to the finished areas for the

following defects:

- Cracking or Checking.

B. A total loss of gloss caused by chalking or fading.

C. Peeling of the top coat or all layers included in the

process from the substrate.

D. Spot or, random discoloration in the overall finish.

This warranty does not apply to:

A. Corrosion originating from within the apparatus.

B. Hazing of the paint caused by improper abrasive

detergents, or by incidental chemical exposure.

C. Scratches, abrasions, or stone chips caused intentionally

or accidentally.

D. Homogenous darkening of the finished color.

E. Blemishes or blistering caused by corrosion originating

from within the apparatus.

F. Blistering or peeling of lettering, and stripping.

G. Multi-stage fluorescent finishes.

H. Accidents.

In the case of warranty claim, repair of all non-warranty

blemishes shall be negotiated prior to the warranty refinish

or repair.  Transportation of the vehicle to the factory

authorized repair center shall be the responsibility of the

owner.

CHASSIS PAINT

The frame and running gear shall be painted gloss black

enamel.  The running gear shall consist of the axles,

drivelines, air tanks, steering gear, frame mounted

brackets, draglink(s), and fuel tank.

The air system piping and electrical harnesses shall not be

installed in the frame at the time of the frame painting.

This will insure complete coverage of paint behind those

areas, as well as to insure that the air piping and wiring

harnesses do not have paint applied to them, hindering

troubleshooting.

INTERIOR FINISH

The interior of the cab shall be painted with spatter paint,

textured gray in color. The spatter paint is selected for

ease of repairs when the interior is scratched.

The exterior doors and all fixed cab glass is to be removed

from the cab prior to the painting process beginning.

The cab metal finish shall be covered with one coat of base

self-etching primer to fill the small surface imperfections.

Then the interior of the cab is to be blocked and a coat of

sealer-primer is to be sprayed to the exterior finish.

Next a sealer-primer is applied and will be sanded to a

smooth finish ready for final color coat application.

Two (2) coats of finished paint is to be applied to a final

thickness of 4 mills.

The following interior components shall be covered in gray

vinyl to match the interior paint color:

- All seating

- Headliner

- Rear wall padding

- Door Panels-ABS

- Doghouse covering

- Floor covering (mats)

CAB EXTERIOR FINISH

The exterior doors and all fixed cab glass is to be removed

from the cab prior to the paint and body process beginning.

The final finish of the cab shall be to fire apparatus

standards;  exhibiting excellent gloss durability and color

retention properties.

PREPARATION

The removal of all contaminates and oxidation is essential

to the final effect of a finish system, the cab shall be

precleaned with a Wax and Grease Remover and prior to

evaporation, towel dried.

To remove all oxidation and foreign materials, the cab shall

be sanded with a 180 grit abrasive using an orbital type

disc sander.

All weld marks and other major surface imperfections shall

be filled with a polyester type body filler, prior to body

filler application special attention shall be given to the

areas requiring filler again sanding and cleaning.

The body fillers shall be thoroughly mixed in accordance

with the manufacturers directions.

After the final coat of filler is sanded a spray polyester

shall be applied in sufficient amounts as to provide a final

base and sanded with an abrasive paper.

PRECLEAN

Within 45 minutes of pretreat the cab must be again washed

with a Wax and Grease Remover using a "Scotch brite pad".

Towel dry prior to evaporation.

Special precaution shall be taken     NOT     to saturate any

polyester body fillers with the cleaning solvents.

PRETREAT AND PRIMERS

The pretreat and primer applications shall be made in two

independent steps.  A combined pretreat/primer one product

application will not be allowed as a substitute.

The prepared substrate shall be pretreated with an acid

curing 2-component Transparent Primer.  This pretreat shall

be designed to provide corrosion protection and to create an

adhesive bond between the substrate and the surface

applications.

It is critical that the body fillers not receive a

saturation of solvents associated with the pretreat

application.  Only the pretreat over spray resulting from

product application to the adjacent metal areas should be

allowed to come in contact with the body fillers.

All polyester body fillers are porous, and will absorb

liquids.  Solvents when absorbed not only soften but will

create swelling of the polyester filler.  After sanding and

later shrink the fillers will create blemishes in the

painted surfaces.

Prior to complete primer application, each area with applied

body fillers be precoated with a 2-dry applications of

primer (sander surfacer) of which shall be allowed to "Touch

Dry" between coats.  This procedure will isolate the filled

areas and protect them from subsequent product applications.

The primer (sander surfacer) shall be a poly-acrylic resin,

zinc and chromate free surfacer that is designed to create a

superb surface smoothness, increase the depth of color, and

insure top coat gloss.

The cab after pretreat and precoat shall be primed with a 3

to 4 medium applications of a Hi-Build Tintable Surfacer.

To create a finish base that meets the rigid requirements of

the fire and emergency service;  the primed surface shall be

dry sanded smooth thus removing all texture and surface

imperfections with a 320 grit (minimum) sanding abrasive.

FINISH AND COLOR COATS

The color coat application shall consist of one of one or

two applications of acrylic urethane color coat.  The final

finish shall be free of dirt and sags and shall meet a

minimum grade of 5 when compared to the "ACT" general orange

peel standards by "ACT" Laboratories, Inc. of Hillsdale, MI.

FLOOR COVERING

The front and rear floor areas of the cab shall be covered

with "HUSHCLOTH" sound barrier floor mats.  This floor mat

shall be a three ply material with a 3/16" thick open cell

isolation barrier of Polyurethane, a 3/32" thick closed cell

Nitrile mid barrier for section reinforcement, and a 1/16"

thick embedded pebbled grain wear surface.

SEAT BELT WARNING LABELS

The cab shall be equipped with two (2) seat belt warning

labels.  These labels are to be in full view of the

occupants in the seated position.

DRIVER'S SEAT

The driver's seat shall be a Seats, Inc. 911 air ride

driver's seat.  The seat shall be of the low-back style. The

seat shall have adjustments for height and ride adjustment.

The fore / aft adjustment of the seat shall be six (6)

inches and contain a fore / aft shock absorber.  The bottom

seat cushion shall contain a contoured thigh support. The

seat covering shall be heavy duty vinyl upholstery.

OFFICER'S SEAT

A SEATS, INC. SCBA Officer's Fixed Seat with split head rest

shall be supplied.  The seat shall contain a S.C.B.A. filler

pad for when the bottle is not in use. The seat upholstery

shall be heavy duty vinyl.

TWO REAR/THREE FORWARD SCBA/TWO REAR NON SCBA

The two (2) outboard rear facing canopy seats shall be

SEATS, INC. 911 Series Self-Contained Breathing Apparatus

(SCBA) type seats.

The two (2) outboard forward facing canopy seats shall be

SEATS, INC. 911 Series Self-contained Breathing Apparatus

(SCBA) flip bottom seats.

The one (1) inboard forward facing canopy seats shall be

SEATS, INC. 911 Series Self-contained Breathing Apparatus

(SCBA) type seats.

The SCBA seats shall have split head rest.

The seat covering shall be heavy duty vinyl upholstery.

SHOULDER HARNESS

Shoulder harness seatbelts for Driver/Officer positions

shall be provided.  The shoulder harnesses shall be black in

color.

40,000 BTU AIR CONDITIONING

A climate control system shall be furnished in the cab.  The

system shall consist of a 40,000 BTU air conditioning

evaporator with a 36,000 BTU auxiliary heater.

The system is to have a 12.6 cu. in. minimum compressor

mounted on the engine to provide the compressed refrigerant

to the system.  The compressor is to be plumbed to a heavy

duty truck air conditioning condenser mounted in a concealed

location ahead of the radiator.  There shall be an extended

life filter receiver/dryer with a pressure relief valve

installed to protect the system from contaminates, moisture,

and high pressure.  It is to have a sight glass for visual

inspection and ease of service.

The air conditioning evaporator shall be centrally mounted

on the rear of the engine enclosure in the cab.  The

evaporator shall have an externally equalized expansion

valve and be thermostatically protected to prevent freeze

up.  Dual high performance 3-speed blowers shall provide a

minimum of 700 CFM air flow.  Each blower is to be

controlled separately. Four (4) forward facing and three (3)

rear facing full adjustable diffusers with shut-off

capability shall be utilized to direct the air flow through

the cab.

The air conditioning on-off switch, thermostat control, and

blower switches shall be located on the evaporator unit.

The air conditioning system shall use R134A freon.

36,000 BTU AUXILIARY HEATER

A 36,000 BTU auxiliary heater is also to be furnished inside

the conditioning evaporator unit to provide additional cab

heating during cooler weather.  The heater core is to be

plumbed to the water lines of the engine cooling system.

LOW COOLANT ALARM

A low coolant light & alarm shall be furnished.  A test

button is not available.

TRANSMISSION OVERHEAT WARNING LIGHT

A Transmission temperature light & buzzer shall be provided

on the dash panel.

SPEEDOMETER ACTIVATED IN PUMP MODE

The speedometer and odometer shall be activated while in

pumping mode.

LOW VOLTAGE WARNING

A low voltage indicator light shall be installed on the

dash.  An alarm and the dash indicator light shall activate

when the system voltage drops below 11.8 volts.

DRIVERS SIDE OVERHEAD SWITCH PANEL

The apparatus warning light panel shall be mounted above the

driver in the overhead console.  The panel will have a black

anti-glare surface, and be angled for easy viewing of the

driver.  The panel shall include the following switches:

One (1) lighted master control switch to allow for

pre-selection of the other switches.

The master lighting control switch shall be wired to a 50

amp circuit breaker and a 75 amp relay.  A 10 gauge wire is

powered by this circuit and run to the roof for lightbar

power.  The remaining switches shall be wired to 20 amp

circuit breakers and relays.

TOTAL SYSTEM MANAGER

The apparatus shall be equipped with a Class 1 Total System

Manager (TSM) for performing electrical load management.

The TSM shall have outputs to supply warning and load

switching requirements.

Outputs 1-12 shall be independently programmable to activate

during the scene mode, the response mode, or both.  These

outputs can also be programmed to activate with the ignition

or warning master switch, or to sequence and shed along with

the priority.

Output 15 is a user configurable output and shall be

programmable for activating between 10.5 and 15 volts.

The TSM shall be protected against reverse polarity and

shorted outputs, and be enclosed in a metal enclosure to

enhance EMI/RFI protection.

INTERMITTENT WIPER CONTROL

A rotary combination intermittent electric wiper / washer

switch shall be provided on the left hand side of the

Driver's dash.

PUBLIC BROADCAST RADIO

The cab shall be equipped with an AM/FM Cassette Stereo

Radio with four (4) speakers.

OFFICER'S DASH

The dash on the Officer's side of the cab shall be a low

profile flat surface with the ability of mounting a map box

or a computer terminal.  The "Data Dash" shall have a

minimum twelve (12) inch deep x twenty-four (24) inch wide

flat work surface.  The dash shall be covered with vinyl to

match the interior of the cab.

EMI/RFI PROTECTION

The apparatus shall incorporate the latest designs in the

electrical system with state of the art components to insure

that radiated and conducted electromagnetic interference

(EMI) and radio frequency interference (RFI) emissions are

suppressed at the source.

The apparatus proposed shall have the ability to operate in

the environment typically found in fire ground operations

with no adverse effects from EMI/RFI.

EMI/RFI susceptibility is controlled by utilizing

components that are fully protected and wiring that utilizes

shielding and loop back grounds where required.  The

apparatus shall be bonded through wire braided ground

straps.  Relays and solenoids that are suspect to generating

spurious electromagnetic radiation are diode protected to

prevent transient voltage spikes.

In order to fully prevent the radio frequency interference

the purchaser will be requested to provide a listing of the

type, power output, and frequencies of all radio and bio

medical equipment that is proposed to be used on the

apparatus.

ALTERNATOR

A LEECE-NEVILLE model LN4864J 270 Amp alternator shall be

installed on the engine.  This alternator is internally

rectified and regulated.

CHASSIS ELECTRICAL SYSTEM

The apparatus "Electrical Distribution System" (EDS) shall

be mounted inside the cab to prevent moisture from entering

the area.  It shall be mounted under the dash on the

officer's side behind a diamond plate cover.

The EDS shall be fed by two power studs:

One (1) battery positive

One (1) ignition

The battery positive stud is to be controlled by the master

disconnect switch and the ignition switch mounted on the

lower right dash panel.  A green light shall indicate when

the ignition circuit(s) are energized.

EDS MODULE

The EDS system shall be designed with locally available

plug-in circuit breakers and plug-in relays.  Each

component position shall be labeled to indicate it's

function.   All electrical connections shall be insulated

and secured behind the panel face to eliminate the chance of

accidental electrical shorts while performing electrical

system service.

The EDS shall control a minimum of thirteen (13) low

voltage, analog switched, high amperage electrical loads.

Provision for a minimum of thirty-one (31) automatic reset

circuit breakers is required to protect the vital circuits

of the apparatus.

The EDS system shall be removable with only four (4)

fasteners for major electrical service or modifications.

The EDS panel will have two (2) lamps for illumination of

the panel during service.

CHASSIS COLOR CODED WIRING

All chassis wiring shall be type "GXL" in accordance with

S.A.E. J1128 and NFPA-1901.  ALL wiring shall be        COLOR

CODED       and continuously marked with the circuit number and

function.

All wiring to be covered in nylon heat resistant "HTZL" loom

rated at a minimum of 300 degrees F exceeding the heat

requirements of NFPA-1901. A battery "loop back" ground circuit

shall be supplied for the EDS system to reduce the possible

effects of Electromagnetic and Radio Frequency Interference.

The chassis cab, engine and transmission shall be

electrically bonded to the chassis frame rails with braided

ground straps.

ELECTRICAL SYSTEM CONNECTORS

All multiple conductor electrical connections shall be made

with Packard electrical connectors.  The Packard connectors

shall become mechanically locked when mated.

All single wire terminations requiring special connectors

with A ring or spade terminal shall be crimped, and wrapped

with heat shrink tubing.

BATTERY BANK

A single battery system shall be provided, utilizing six (6)

high cycle type Group 31 batteries.

This system shall be capable of engine start after

sustaining a continuos 150 amp load for 10 minutes with the

engine off (NFPA-1901, 1996 edition S9-4.2).

A battery disconnect switch (Rated at not less than 250 amps

continuos) shall be used to activate the system and provide

power to the power panel.  A green pilot light will

illuminate to indicate which battery bank is activated.

BATTERY CABLES

All battery wiring shall be "GXL" battery cable capable of

handling 125% of the actual load.  It shall be run through a

heat resistant flexible nylon "HTZL" loom rated at a minimum

of 300 degrees Fahrenheit.  All cable connections shall be

machine crimped and soldered.

STARTING CIRCUIT

One (1) engine start button is to be located on the lower

right dash panel.  It shall be wired to heavy duty solenoid

rated at not less than 1100 amps.  The battery indicator

light is to be located directly above the start button to

indicate that the battery bank is on.

Two (2)      SHORELINE AUTO-EJECT

A KUSSMAUL Super Auto Eject, model O91-55-20-120, with

weatherproof yellow cover shall be mounted on the cab

exterior immediately adjacent to the rear of the driver's

door.

The Super Auto Eject is to be completely sealed to prevent

internal contamination of the working components.

The internal switch arrangement of the Super Auto Eject

shall be designed to close and open the 120 vac A.C. circuit

after the mating connector is inserted and before the

connector is removed.  This design shall prevent arcing at

the connector contacts to provide long life.

The electrical connection shall be provided as a 120 vac -

20 amp type using a NEMA 5-20P connector.

OPERATOR'S MANUAL

Operator's Manual w/Parts List-TWO Sets shall be provided

with the chassis.

CHASSIS MODIFICATIONS

The following modifications and installations shall be

performed on the chassis upon delivery to the apparatus

manufacturer.

Six (6)      AIR PACK BRACKETS

Apollo SCBA air pack bracket(s) shall be mounted in the

backs of the SCBA seat backs in the cab with quick release

collision restraint release straps.

EXTENDED FRONT BUMPER

A front bumper extension shall be supplied at the front of

the cab from chassis manufacturer.

The open area between the bumper and the cab front shall be

covered with 28" bright aluminum tread plate to form a

"gravel shield".  The tread plate shall be boxed in at the

bumper ends to cover the vertical area between the bumper

and the cab as required.

The bumper shall be extended approximately 28" from the

front of the cab.

Hose wells and equipment and accessories to be mounted on

bumper extension to be described in other portion of this

specification.

Two (2)      HOSE WELL

A of hose wells shall be provided in the extended front

bumper gravel pan to hold 200' of 2 inch attack line in donut

rolls. Wells shall be located outboard frame rails.

The hose well shall be made of 1/8" unpainted smooth

aluminum.  The hose well shall be approximately 17" long x

14" deep x 19" wide. Well shall have a rounded bottom to

follow contour of hose donut roll. A drain hole shall be

provided in each low spot of the well. An adjustable divider

shall be provided in each well.

One attack line shall be made up of 150' of 1-3/4" hose and

50' of 1" hose with TFT Mid Force nozzle with MX tips

attached.

A total of two wells shall be provided in bumper. See print

for exact location and dimensions.

Two (2)      PORTABLE FLASHLIGHT

A Stream light Vulcan portable flashlight with charger shall

be mounted in cab as directed by department.

Eight (8)    PORTABLE FLASHLIGHT

A Stream light Survivor flashlight with charger shall be

mounted in cab as directed by department.

LOCK PLATE FOR TRANSMISSION SHIFT

A hinged polished stainless steel guard shall be installed

for the transmission shift lever.  It shall be notched to

allow the plate to swing over the lever shaft and hold the

shift lever in the pumping gear position.

MAP BOOK HOLDER

A parts book style map book holder with swivel shall be

provided and installed on engine cover. Size of holder

needed to be finalized at reconstruction meeting.

VIDEO SYSTEM

An Intec Cab mounted thermal imager and color rear camera

shall be provided and installed in cab.

Intec Rear Vision System shall include the following:

One (1)  CVC450 Color Camera 1/3" CCD 127 FOV. Camera shall

be mounted in protective housing centered on rear body face

below hose bed.

One (1) CVM560LCD 5.6" Color LCD Display mounted on driver's

side on ceiling above windshield in upper right corner.

One (1) CVCOAX18M 18M Coax Cable. This shall be used to

hook camera to monitor.

The system shall include Back Talk system to allow driver to

communicate with personnel at rear of apparatus.

Intec 200 Series Thermal Imager System shall include the

following:

One (1) #3202720-2 Thermal Imager Camera 12 FOV with pan,

tilt and wiper. Camera shall be mounted on bracket fastened

to front face of cab above windshield.

One (1) #3202675-1 Joystick controller. Controller to be

mounted as directed by customer.

One (1) #3202711-2S Custom hook-up cable.

One (1) CMV560LCD 5.6" Color LCD display. Display shall be

mounted on passenger side on ceiling above windshield in

upper left corner.

One (1) 12VDC industrial mobile video recorder to record

video information that is being displayed at one monitor.

One (1) Custom Switching System. This switching system shall

allow for complete video integration. This shall have

Four-Input, Dual record output, distribution amp dual switch

auto call for rear camera and all cables and material.  Two

sets of four button controls shall be provided, one for the

driver and the other for the officer.

VIDEO SYSTEM OPERATION

This system shall allow for viewing of rear camera, cab

mounted thermal imager and hand held thermal imager.

Switching allow for independent viewing of both displays.

Passenger side display shall be connected to VCR.

Receiver for hand held thermal imager shall be connected to

switching system.

PUMP CONTROLS IN CAB

The following pump controls shall be located in cab on a

panel fabricated to provided ease of operation. Final layout

and exact location to be determined during inspection trips.

The following controls shall be located on panel:

Pump Engage Switch

Generator Switch (Flood Master)

Pump engaged lights - part of Detroit Fire Commander

120 volt lighting switches

Light switches for lights on bumper turrets

Bumper turrets remote control tethers

Bumper turrets on/off switch - part of turret control

Dump valve chute extension switches

Dump valve control switch

MC Accusonic water level gauge

Water level on/off switches

Class One digital suction & pressure gauges

Detroit Fire Commander - pressure governor

AUTOCafs controls

On/Off switch for warning system and water level

Panel shall be constructed from stainless steel with letting

and labeling provided by Vision Mark.

Panel shall be hinged at the front with a hold open device

to provide access to access cover on engine cover.

Department to approve layout and final location.

RADIO MOUNTING

Customer supplied radio will be mounted on the engine dog house

above the in cab pump panel. Two radio speakers shall be

mounted at the center of the cab ceiling. One dual portable

radio charger, four antennas and GPS receiver shall be mounted

in cab as directed by department.

SPECIAL WARNING SYSTEM

A special warning system shall be provided and installed.

System shall consist of Code 3 red strobe light and 8" gong.

Strobe lights shall be mounted on the center post of raised

roof section and gong shall be mounted on rear wall of cab.

Red strobe light and MC Products 8" gong shall be wired to

warnings for Detroit Fire Commander for engine, transmission,

compressor, pump, low oil pressure, tank low level, T-Pass,

overheats, and all alarms in cab.

INTERIOR CAB REAR WALL

Inside custom chassis rear wall the following modifications

shall be provided and installed.

Provided by the chassis manufacturer shall be three scba

seats. One mounted in left corner and two mounted to the

right side. The wall shall be reinforced by chassis

manufacturer in space between scba seat and along upper

portion of rear wall.

The space between the seats shall be used to install an RV

type refrigerator/freezer. On top of the refrigerator shall be a

Grindmaster model PAPS400 coffee maker. A water spigot shall

be provided on wall with water supply coming from a 12

gallon insulated water jug mounted in cabinet over left side

scba seat. Mounted above coffee maker shall be a small

microwave oven. Water jug shall be constructed from poly and

have quick disconnect for removal and refilling.

Two pair of 120 volt household type outlets shall be mounted

in rear wall. Both outlet shall be wired to 120 volt

electrical system. One outlet shall also be wired to

shoreline with a changeover relay to change outlet over to

truck generator system when disconnected from shoreline. This

outlet shall be used for the refrigerator.

A water cup and styrofoam cup dispensers shall be supplied

and installed one rear wall.

Mounted above scba seats shall be cabinets constructed from

aluminum with plexiglass doors, Cabinet above right side

seats shall be approximately 40" wide x 16" high x 12" deep

with an adjustable shelf and sliding doors. An aluminum

shall be provided over refrigerator for microwave. The

cabinet above the left side scba seat shall be approximately

20" wide x 16" high x 12" deep with a hinged door. This

cabinet shall hold a removable water jug.

This option was provided at No Charge.

Final layout of wall shall be provided with approval

drawings.

All item provided shall be mounted to withstand driving

conditions. Darley to fabricate any brackets to hold down

appliances.

STAINLESS STEEL WHEEL COVERS

One (1) set of stainless steel Phoenix wheel covers shall be

installed on the front wheels.  Stainless steel lug nut

covers shall be included.

STAINLESS STEEL WHEEL LINERS

One (1) set of stainless steel Phoenix wheel liners shall be

installed on the rear wheels.  Stainless steel lug nut

covers shall be included.

FRONT MUDFLAPS

A pair of black hard rubber mud flaps shall be installed

securely behind the front wheels, one each side.

REAR MUDFLAPS

A pair of black hard rubber mud flaps shall be installed

securely behind the rear wheels, one each side.

The apparatus company logo shall be displayed on each rear

mud flap.

FLUID DATA PLATE

A permanently mounted plate in the drivers compartment

specifying the quantity and types of the following fluids

(as applicable) used in the vehicle shall be provided:

A.  Engine Oil

B.  Engine Coolant

C.  Chassis Transmission Fluid

D.  Pump Transmission Lubrication Fluid

E.  Pump Primer Fluid

F.  Drive Axle Lubrication Fluid

G.  Air Conditioning Refrigerant

H.  Air Conditioning Lubrication Oil

I.  Power Steering Fluid

J.  Cab Tilt Mechanism Fluid

K.  Transfer Case Fluid

L.  Equipment Rack Fluid

M.  Air Compressor System Lubricant

N.  Generator System Lubricant

OCCUPANT PLATE

A permanent plate shall be provided and installed in the

driver's compartment specifying the authorized number of

personnel allowed to ride in the apparatus.

PAINT CODE INFORMATION

A label specifying the paint code(s) used on the apparatus

shall be installed on or near the area of the driver door

opening.

TRAVEL HEIGHT SIGN

An engraved sign shall be mounted in the cab in view of

driver. Lettering height shall be a minimum of 1" high. Sign

shall be worded as follows:

"TRAVEL HEIGHT"

"OF APPARATUS"

"__'-__"

FIRE PUMP & BODY TEHNICAL

SPECIFICATION SECTION

NFPA 1901 COMPLIANT PUMP

The fire pump and related plumbing on the specified

apparatus shall be installed in accordance with applicable

NFPA 1901 guidelines at the time the contract was placed.

U.L. CERTIFICATION (1250) GPM

The fire pump shall meet and perform the following tests to

receive a U.L. certification.

100% of rated capacity at 150 PSI net pump pressure

100% of rated capacity at 165 PSI net pump pressure

70% of rated capacity at 200 PSI net pump pressure

50% of rated capacity at 250 PSI net pump pressure

PUMP PANEL PLATE - SPECIAL

A separate plate shall be mounted at pump operator's panel

with peak performance of pump. This plate shall be in

addition to UL plate provided.

EM TWO STAGE FIRE PUMP - PTO - CAFS COMPATIBLE

Pump Casing  - Vertically split type high strength ductile

iron, bronze fitted.  Has a cored heating jacket that can

be optionally hooked into the vehicle antifreeze system to

protect pump from freezing in cold climates.

Impeller  - Accurately balanced and splined to pump shaft

for precision fit and durability.  Opposed suction inlet

design eliminates end thrust.

Impeller Seal Rings  - Renewable double labyrinth bronze

type are standard.

Pump Shaft  - Precision ground stainless steel with long

wearing ceramic hard coating under packing glands. Shaft is

splined for broached impeller hubs for greater resistance to

wear, torsional vibration and torque imposed by engine.

Stuffing Box  - Packed with plastallic packing injected into

box from external supply cylinder by a single injection

screw. Injection pressures are equalized around shaft.

Short length of packing area minimizes friction and heat

generation.  Packing can be renewed by removing the plunger

and inserting pellet form packing as needed.

Transmission Case  - Alloy cast iron with adequate oil

reserve capacity for low operating temperature. An auxiliary

transmission will mount to the side of the pump gear case

transmission. With the use of a cab mounted engagement

control an air compressor can be driven off of this

supplementary gear case. Both gear cases utilize the increased

oil reserve capacity and surface area for improved cooling.

Drive Shafts  - Precision ground heat treated alloy steel

with minimum 2-1/2" - 10 Spline standard input and output.

Gears  - 3-1/2" face helical.  Precision cut from heat

treated alloy steel for quiet operation and long life.

Pump Shift  - Pump to be placed in gear from chassis cab.

Pump shift to be clearly labeled. Pump shall be driven by

transmission PTO. Gear ratios of the pump and PTO to be

individually engineered for engine selected.

Compressor Gear Shift  - A sliding, heat treated, helically

cut gear couples pump drive train to air compressor drive

train.

Bearings  - Heavy duty, deep groove, radial type, ball

bearings, oversized for long life.  Bearings are protected

at all openings from road dirt and water splash with oil

seals and water slingers.

Tachometer Drive  - 52" cable and panel mounted hand

tachometer drive turns at engine speed.

Transfer Valve - Series or parallel operation shall be

selected through a quarter turn, all bronze transfer valve

operating in a renewable bronze sleeve.

Discharge Outlet - Discharge extensions have companion

flange openings for auxiliary service.

Master Drain - Single master pump drain of push-pull type,

bronze construction to be provided on the pump. Control for

this valve to be located so operator does not have to crawl

under the apparatus.  Master pump drain control is to be

operated from left side of apparatus.  All pump passages,

including relief valve, shall be connected to master drain.

Two UL test plugs shall be pump panel mounted for UL testing

of vacuum and pressures.  Metal

rust proof Underwriter's Performance Data Plate to be

furnished and installed.  Hand speed-counter outlet to be

supplied on the pump and cable furnished for connecting to

the pump panel.

Power to drive the fire pump shall be provided by the same

engine-transmission used to propel the apparatus.  The pump

shall be located near the middle of the chassis ("mid-ship

mounted") and driven through an integral transmission PTO.

Two (2) Shop Manuals covering the pump, pump transmission

and necessary working parts of the pump shall be provided

with the apparatus.

TRANSFER VALVE CONTROL

An additional transfer valve control shall be provided and

installed on passenger side pump panel.

PUMP OPERATION VIDEOTAPE

A VHS videotape that explains proper fire pump operating

procedures and maintenance shall be included at the time of

delivery. This videotape shall be provided by the same

company that manufactures the fire apparatus.

PUMP SHIFT

A PTO shall be installed to drive the fire pump. An

electrically activated switch shall be installed in the cab

to engage the fire pump complete with indicator lights

located on the cab dash and pump panel. Pump shifting

instructions shall be provided at the pump shifting

location.

PRIMER - ELECTRIC

A 12 volt positive displacement rotary vane primer shall be

installed complete with its own lubricant reservoir tank.

The push-pull control valve shall be mounted on the pump

operators panel in a bezel that reads "Pull to Prime, Push

to Close".

Three primer control handles shall be provided on both pump

panels.

The primer system shall be fully functional even with the

discharge valves left open on the pump, due to the check

valve design in the discharge head of the pump.

One primer control valve will control pickup from outside of

butterfly valve to the squirrel tail suction hose only, another

will control pickup from inside the master intake valve and

one will control pickup from outside the master intake valve.

A control  for the valve shall be provided on both panels.

Primer exhaust shall be tee'd and extend below running

boards on both officer's and driver's side.

HEAT EXCHANGER/HEATED PUMP CORE

The fire pump shall be equipped with an automatic heat

exchanger system. Antifreeze from the vehicle engine shall

flow through the pump core jacket. This feature shall use

water flow from the fire pump to cool the engine antifreeze.

This feature shall also assist to protect against the pump

freezing in cold climates.

PUMP COOLER

A pump cooler, consisting of a 5/8" line direct from the

discharge side of the fire pump to the water tank.

PUMP FREEZE PROTECTION

A pair of gated 5/8” re-circulating line shall run from

the front and rear of the apparatus to the pump suction to

allow constant water motion for freezing protection of

the front and rear plumbing.

PLUMBING SYSTEM

The plumbing system shall consist of hard piping, or

flexible high pressure hose with stainless steel ends, as

deemed necessary for the application.  Upon completion, the

entire system shall be tested for leaks and weaknesses.  Any

leaks or weaknesses shall be corrected prior to delivery.

The plumbing and valve arrangement shall be capable of

delivering water to the pump at a minimum flow rate of 500

GPM while pumping at 150 psi pressure.

A tank fill shall be provided as described elsewhere in

these specifications.

When any 3" or larger intake or discharge is gated (except

tank to pump valve), the valve shall have a mechanism to

allow the valve to fully open or fully close no faster than

3 seconds.

MASTER DRAIN - PUSH-PULL

A master drain control shall be installed on the lower

portion of the side control panel. It is plumbed to drain

all of the major portions of the pump including, but not

limited to; the tank to pump line, the relief valves, the

pump core, the discharge head, and the volute. It shall be

of two-piece brass construction with special o-ring seals

and use a push-pull type mechanism complete with a black

plastic knob. It shall be labeled on the panel as "Pull to

Drain/Push to Close".

Two     DRAIN - AUTOMATIC

A 3/4" automatic drain valve shall be installed where

indicated. The automatic drain shall have an all brass body

with stainless steel check assembly. The drain shall be

normally open that closes when the pressure is greater than

6 psi.

Two (2)      PUMP INLETS

Two (2) 6" suction steamer inlets shall be provided, one (1)

on the left side pump panel and one (1) on the right side

pump panel.

6" BUTTERFLY VALVE - RIGHT SIDE

One (1) 6" manually operated butterfly valve shall be

installed in the right side suction sleeve casting,

completely behind the panel.  A hand wheel control

shall be provided through the side panel.

6" BUTTERFLY VALVE - LEFT SIDE

One (1) 6" manually operated butterfly valve shall be

installed in the left side suction sleeve casting,

completely behind the panel.  A hand wheel control shall be

provided through the side panel.

Two (2)      PRECONNECTED SQUIRREL TAIL HOSE

The 6" NST side suction inlet to the pump on each side shall

be provided with a 90 degree 5" NST lightweight swivel with

Bray butterfly valve with manual throttle handle. A single

section of flexible hard suction hose shall be preconnected

to the inlet swivel. Swivel and valve shall be connected to

inlet by a minimum of a 6" weld "T" with pipe threads.

Manual twist bleeders shall be provided on each side. A 6"

NST F LH Swivel x 5" Storz 30 degree elbow with cap and

chain shall be mounted on intake. A push pull drain shall

be provided ahead of the valve.

The preconnected "squirrel tail" suction hose shall be bent

upward and routed to a hose trough mounted near the top of

the body.  At the rear of the trough, the hose shall be bent

downward and routed to a  above the lower compartment. Hose

shall be attached to a low flow strainer and strainer to

mounted in slide bracket mounted to apparatus body. Hose

shall lay in trough, velco straps shall be provided to

secure strainer in place.

A 3" Kochek suction hose with    COLOR CODED   CAM-LOCK

couplings and connected to a Kochek foot valve low lift

strainer and mounting bracket shall be provided. Length of

hose shall be determined based on final layout of apparatus.

This hose shall be mounted on left side of apparatus.

A 5" Kochek suction hose with COLOR CODED CAM-LOCK

couplings and connected to a Kochek foot valve low lift

strainer and mounting bracket. Length of hose shall be

determined based on final layout of apparatus. This hose

shall be mounted on right side of apparatus.

A Kochek 5" NST F Swivel to Cam lock shall be provided for

each suction hose to adapt to thread on swivel.

All hose couplings shall be stenciled RATTLESNAKE F.D.

Color codes shall be as follows:

One rig shall be RED

One rig shall be BLUE

One rig shall be GREEN

2-1/2" RIGHT REAR TANK FILL

There shall be one (1) 2-1/2" rear tank fill at the right

rear of the tank.  The threads on the valve shall be 2-1/2"

National Standard. The valve control shall be vertically

mounted along side the valve body. Valve shall have a 2-1/2"

NST F swivel.

ELECTRIC TANK TO PUMP LINE

One (1) 3" tank to pump line shall be provided between the

tank and the fire pump.  It shall have an electric over air

actuated 3" bronze, quarter turn ball valve.  An electric

toggle switch and open - closed indicator lights shall be

located on both pump panels.

CHECK VALVE

There shall be NO check valve supplied for tank to pump

line.

An additional plate shall be provided in tank to pump

opening to protect tank when back filled.

TANK FILL

There shall be a 3" pump to tank fill line installed

with a 3" inline bronze valve.  The valve shall be

electrically controlled at both left and right pump panels

AutoCAFS - COMPRESSED AIR FOAM SYSTEM

This highly advanced fire apparatus shall be equipped with a

high energy, automatic compressed air foam system (AutoCAFS)

which will be sized to provide compressed air foam to at

least three (3) 1-1/2" discharge hand lines. The system

shall be sized to provide a minimum of 60 gallons per minute

water flow and 60 cubic feet per minute air flow to each

line. Minimum total system flow of 200 GPM, 200 CFM shall be

provided.

The air compressor shall be of a high quality, industrial

rated, modulating, continuous duty, rotary screw design. The

air compressor shall be mechanically driven by the main pump

gear case and shall be so designed as to provide optimum

performance at 70% of rated engine RPM. Air compressor gear

train shall provide a means to engage and disengage air

compressor as required.

Air compressor system shall include a pressurized oil

lubrication system, oil separator, oil filter, inlet air

filter, and modulating inlet air control. The air compressor

shall be provided with an (Auto/Manual) air pressure control

valve located at the panel to permit either automatic

balancing of air to water pressure or, if desired, manual

air pressure adjustment. The air compressor system shall

also have mounted on the panel a compressor engaged light,

and a high temperature oil warning light/buzzer mounted in

the cab on pump panel.

The AutoCAFS system shall incorporate an automatic discharge

side foam concentrate proportioner. The proportioner shall

be adjustable and shall be capable of maintaining a solution

ratio of as low as 0.1 percent to at least 1.0 percent Class

A foam at flow rates of near 20 GPM to a minimum of 250 GPM.

The proportioner shall be capable of using different types

of class "A" and "B" foam concentrates at up to 1 percent

concentration. Unless specified in the foam proportioner

section of this document, this unit will only be provided

for use with Class A foam. The specific foam concentrate

proportioner description shall be provided upcoming in this

document.

The discharges shall be controlled by switch mounted on cab

pump panel. Switch shall be provided for on/wet/dry foam.

Flow and pressure gauges shall be provided to monitor CAFS

system total air and water discharge conditions. Gauges and

controls shall be positioned and clearly marked so as to

provide simple and easy operation.

Each of the components of this Automatic Compressed Air Foam

System - (air compressor, drive system, foam proportioner,

control and instrumentation system) shall be sized, driven

and controlled to produce a well operating and reliable CAFS

unit.

This automatic compressed air foam system (AutoCAFS) shall

be completely installed and tested before delivery by the

factory.

No air flow meter shall be provided.

The compressed air foam system shall utilize an air pressure

control system. The system includes a 1/4 turn auto/manual

selector valve and an adjustable pressure regulator. It

allows the pump operator to choose between automatic air

pressure balancing of air and water pressure or manual

regulating of the air compressor pressure. It shall be

mounted inside the pump compartment behind the operator's

control panel with air pressure gauge.

An air pressure test port shall be supplied inside the pump

compartment for periodic testing of the pressure settings.

The compressed air foam system shall utilize an air

compressor pressure gauge mounted inside the pump

compartment. It shall be installed near the air compressor

pressure control valve and shall read the air compressor

output pressure.

AIR OUTLET 1/4"

There shall be brass 1/4" female air hose quick-disconnect

fittings mounted near portable air hose reel . The fitting

shall be connected to the apparatus CAFS air compressor.

There shall be an adjustable regulator installed on the

discharge side of the separator to provide clean oil free

compressed air to the air outlet. Access to regulator shall

be from driver's side panel.

COMPRESSED AIR FOAM OPERATORS MANUALS

Two (2) Compressed     Air Foam System (CAFS)

instruction manuals shall be provided with this unit. These

manuals must be supplied as a guide to proper operation and

maintenance of the CAFS system and are to be written and

supplied by the manufacturer of this apparatus.

TESTING REQUIREMENTS

The compressed air foam system shall be tested and run for a

minimum of eight (8) hours prior to delivery. After testing

is completed, the foam system and CAFS oil system strainers

shall be removed and flushed. The system shall then be

delivery tested once more to ensure that all strainers,

fittings, and components are installed properly.

FOAM SYSTEM

A FoamPro 1600 Class A automatic, electronic, direct

injection, foam proportioning system will be installed on

the discharge side of the pump. It will provide foam to

pre-determined foam discharge(s). This foam system is

completely automatic and requires only a flip of a switch to

turn it on before the system is functioning.

The system will incorporate a 3" paddle wheel flow meter to

measure the water flow, and based on the foam percentage

selected on the controller dial, the direct injection pump

will inject the proper amount of foam into the foam

discharge(s).

The system is capable of providing precise foam solution

concentration rates from 0.1% to 1% and are easily operator

adjustable with the rotary dial on the control panel.

System Capacity - Foam Pump is capable of 1.7 gpm of foam

output @ 200 psi. Pump motor is 1/3 HP 12 volt.

Foam Concentration    Water Flow Range

0.1%                20-1700 gpm

0.2%                20-850 gpm

0.3%                20-566 gpm

0.5%                20-340 gpm

1 %                 20-170 gpm

A 3" Technocheck check valve will be installed between the

flowmeter and the injection fitting to avoid foam

contamination back into the rest of the pump.

System Features:

System will be provided with a low foam tank level switch

which will alert operator of low foam concentrate level by

flashing a red warning light at the controller and will

automatically shut unit off after two minutes if the foam

supply is not replenished.

Foam system flushing is achieved by simply turning off the

unit and flowing water out of the discharge(s) that were

previously flowing foam solution.

System will be completely installed inside pump compartment,

with control unit and instruction plate mounted on the pump

operators panel and will be tested before delivery.

FOAMPRO DISCHARGES

The following discharges shall be supplied by the FoamPro

system. They shall be provided as part of a foam manifold.

Front and Rear 3" discharges.

Additional discharge information detailed elsewhere in these

specifications.

3" DISCHARGE - FRONT BUMPER

An un-gated 3" discharge pipe shall be plumbed to front

bumper. Plumbing shall supply an Elkhart Sidewinder mounted

in at each side of bumper behind water thiefs, and two Akron

Water Thiefs, one located to each side of bumper. An

Electric valve shall be installed at tee for each bumper

turret. Valve shall be controlled from inside cab. A pick-up

tube for foam system shall be provided. Water thiefs shall

be mounted to 90 degree elbows. All discharges shall be

supplied by same pipe. A 5/8 valved cold weather water

recirculating line shall be plumbed to discharge pipe.

A total of (2) Akron 1573 water thiefs shall be provided for

both discharges. Water Thief shall have a 2-1/2" NST Female

and two (2) 1-1/2" NST male outlets and One (1) 2-1/2" NST

male outlet. All outlets shall be gated. NO LEATHER STRAP

SHALL BE PORVIDED.

3" DISCHARGE - REAR BUMPER

An un-gated 3" discharge shall be plumbed to rear body face.

Plumbing shall supply to Akron gated water thiefs. A 5/8 valved

cold weather water recirculating line shall be plumbed to

discharge pipe.

A total of two (2) Akron 1573 water thiefs shall be provided

for both discharges. Water thief shall have a 2-1/2" NST

Female swivel inlet, two (2) 1-1/2" NST male outlets and One

(1) 2-1/2" outlet. All outlets shall be gated. NO LEATHER

STRAP SHALL BE PORVIDED.

Two (2)      SIDEWINDER BUMPER TURRET

There shall be 2" plumbing supplied from 3" the front bumper

plumbing for a turret discharge.  The turret shall be an

Elkhart Model #8494-01 Sidewinder remote control monitor.

The discharge outlet shall be controlled by an electric 2"

inline valve.  The valve shall be a quarter turn ball type

of fixed pivot design and be constructed of bronze. The

discharge control shall be located in the cab.

This discharge shall be capable of delivering water, foam

solution.

Controls for bumper turret shall be mounted on cab pump

panel or in an accessible location for operation from driver

and passenger seats.

A Collins FX-12 flood/spot light shall be mounted to turret.

A switch shall be provided on cab pump panel to control

light.

Turret shall extend above bumper approximately 16".  Pipe

and gun shall be installed in a manner to allow for easy

removal to allow for tilting of chassis. Wiring harness

shall have easy disconnect plugs or a long enough harness to

allow gun to be removed at set off to side of vehicle.

Darley to provide construction details at reconstruction

meeting.

Two (2)      BUMPER TURRET NOZZLE

A TFT model H-TO-ER remote control nozzle tip shall be

provided and installed on bumper turret.

DECK GUN DISCHARGE

There shall be 3" plumbing supplied for a deck gun

discharge, installed above pump. The discharge shall be

controlled by a 3" manual bronze self-locking valve.

The control shall be located in the pump dunnage area.

Deck gun shall be an Akron #3423 manually operated with 3405

standpipe and mounting bracket for pipe.

A 3/4" Automatic drain shall be supplied.

MASTER STREAM NOZZLE

A TFT model MR master stream nozzle shall be provided for

master stream device.

Two (2)      CAFS TIP

An Elkhart model 187 CAFS tip shall be provided for each

turret specified.

PUMP OPERATION & PANELS

PUMP MODULE

A pump module shall be provided. It shall be assembled and

mounted independently from both the chassis and the body to

properly allow sufficient flexing and prevent component

fatigue.

The module shall be constructed with extruded 3" x 3"

aluminum tubing with 3/16" radius corners.  The welded ends

of the extrusions shall be chamfered prior to welding and

shall be ground smooth before finishing.

The exterior module shall be sanded, prepped, primed using

paint manufacturer's recommendations.  The module shall be

finish painted to match the body and chassis cab.

A heavy duty rubber isolation material shall be provided

between dissimilar metals during the mounting process.  Any

steel substructure used in addition to the aluminum

extrusions shall be painted and undercoated before bolting

to the chassis frame.

The module shall be a minimum of 67.5" long x approximately

81" tall x 74" wide.  The side operators panels shall be a

minimum of 62".  The panel shall be approximately 56" high.

SIDE PUMP PANELS

The operator's control panel shall be located in the cab on

top of doghouse.

PANELS

The pump panels shall be removable and shall be constructed

from brushed stainless steel sheeting.

A vertically hinged left side gauge panel with two (2) push

to open latches, and a vertically hinged right side pump

access panel with two (2) push to open latches shall be

located above the main panels.  They shall also be

constructed of brushed stainless steel, unless detailed

elsewhere in these specifications.

TRIM RINGS

All ports shall be fitted with removable polished stainless

steel trim rings.

GAUGES AND CONTROLS

Any control required to be mounted on side panels shall be

mounted as low as possible.

PUMP PANEL LIGHT  (LEFT SIDE)

The left side pump panel shall be illuminated using a hooded

light shield assembly mounted near the top of the panel. The

light fixtures shall be sealed in an enclosed weatherproof

housing.

PUMP PANEL LIGHT  (RIGHT SIDE)

The right side pump panel shall be illuminated using a

hooded light shield assembly mounted near the top of the

panel. The light fixtures shall be sealed in an enclosed

weatherproof housing.

RUNNING BOARDS

Running boards shall be installed on each side of the pump

compartment module. The running boards shall be constructed

of 1/8" bright aluminum tread plate. Each shall be a minimum

of approximately 11" deep x the width of the side panel

module. The running boards shall have a 1-1/4" upward bend

on the inside edge to act as a kick plate. The running

boards shall be attached to a frame mounted outrigger

support structure. Each running board have a 3" downward

bend on the front and side faces with a 1" underside return

for superior strength.

RECESSED DUNNAGE AREA COMPARTMENT

A recessed dunnage (storage) compartment shall be provided

below the pump compartment. It shall be formed by using 1/8"

aluminum diamond plate and shall be approximately 10" - 18"

deep, and shall be full width of the pump compartment.

This area shall hold following components.

A manual cord reel with a portable light, one on each side

of apparatus. On right side next to cord reel

shall be a hydraulic hose reel plumbed to power unit and an

air hose reel plumbed to CAF compressor. One additional card

reel shall be provided on each side for Tripod quartz lights

mounted to each side of cab.

A hydraulic generator with reservoir.

Phoenix electric simo hydraulic pump

3" deck gun discharge

ACCESS LADDER

A pool style access ladder shall be provided and installed on

right side of pump module.

PUMP PANEL ID PLATE

An identification plate, stamped out by the fire pump

manufacturer, shall be permanently installed on the pump

operator's control panel. It shall identify the fire pump

serial number, model number, and performance points.

TANK LEVEL GAUGE

A MC Products Accusonic ultra-sonic water level detection

system shall be provided and installed.

System shall consist of ultra-sonic sensor located in water

and foam tanks. A display shall be mounted on operator's

panel in cab, and connected to telescoping water level

lights.

Cab display shall have an analog and LCD readout . Lecd

shall be able to rear both tanks.

MAST MOUNTED WATER LEVEL LIGHTS

The water level gauge shall be (4) Whelen RB6 revolving lights

mounted to the side of the 15 foot telescopic light mast.

Light shall be mounted to a bracket system constructed of aluminum

plate and tube and shall be bolted to together to allow for service of

lights.

Two (2)      CLASS 1 MASTER PRESSURE/COMPOUND SUCTION GAUGE

A Class 1 master pressure/suction gauge will be installed on

the operators panel. This gauge will read a pressure range

of 0 - 600 psi. This gauge will utilize a pressure sensor

and a vacuum diaphragm sensor mounted in the water pump.

This gauge measures 4-1/4" square and will be mounted on the

pump panel.

PRESSURE GOVERNOR

A Detroit Diesel Fire Commander electronic pressure governor

system shall be installed on the pump operators control

panel inside the cab. This unit shall be supplied with

chassis and engine to be programmed by HME prior to delivery

of chassis to Darley.

AUDIBLE/VISUAL WARNING DEVICES

Audible and visual warning devices shall be provided for

high engine temperature and low oil pressure at the pump

panel.  Gauges to be grouped together with the tachometer.

A special light and gong shall be mounted to roof of cab.

See part #07-44-00 in chassis modification section for

additional information.

2400 GALLON TANK (T) POLYPROPYLENE

Booster tank shall be constructed of Polypropylene baffled

to meet the requirements of NFPA #1901 and constructed

separately from the body for easy maintenance and repair.

Tank to be "T" shaped for deeper side compartments.

The tank shall be provided with at least one (1) full-length

swash partition (baffle) and a sufficient number of

widthwise baffles so that the maximum dimension of any

spaces in the tank, either transverse or longitudinal, shall

not exceed 46", and not less than 23".

Baffles shall have openings at both the top and bottom to

permit movement of air and water between spaces to allow

maximum flow requirements.  Baffles shall form an integral

part of the tank, and design shall be to provide and

maintain safe road stability regardless of water level.

Tank shall have 4" minimum overflow and air vent designed to

prevent damage to the tank under high flow conditions and

enclosed in front tank filler.  Tank filler to extend upward

from hose bed the same height as body sides.  Overflow is to

be designed and located to prevent water loss on fast stops

or starts, and is also to be located so as not to affect

traction on the rear tires per NFPA #1901.

Tank fill opening shall be 16" x 16".   Cover for fill shall

be able to flip-back clear of opening for overhead filling.

Tank outlet connection shall be designed with a 12"

anti-swirl baffle plate above tank outlet to prevent air

from mixing with the  water when pumping from the tank.

Tank to be designed to be removable from the body.

Removable cover for inspection and maintenance purpose shall

be provided in tank top.  Minimum eighty percent (80%) of

tank top will be removable for access to the tank.

A second vent shall be installed to exhaust entrapped air at

the rear of the tank, to allow the entire tank to be filled

when apparatus is parked in the forward sloping position.

Fill tower shall be installed on front corner of the tank in

tank top, not to interfere with removability of the lid.  It

shall be of adequate size, minimum 16" X 16", to accommodate

overflow and vents, to have a hinged cover and screen

installed.

Tank mounting shall be so the booster tank can be removed

without dismantling the body. The tank shall be mounted to

the chassis frame, per manufacturers requirements.

A lifetime warranty shall be provided from the tank

manufacturer.

FOAM TANK - 45 GALLON, POLY

Foam tank shall be 45 gallons and be molded Polypropylene.

It will be built integral with the water tank and have a

rectangular fill tower approximately 7" x 13" with removable

screen to keep debris out of foam while filling. Tank will

be provided with a drain inside the pump compartment.

Nine (9)     CLASS A FOAM

Five (5) gallon pail(s) of Chemguard Class A Plus foam shall

be delivered with this apparatus. It is specified to be used

between 0.1% and 1% concentration.

WATER TANK DUMP VALVES

The water tank shall be equipped with the following dump

valve mounted on the rear of the water tank.

The sides of the water tank shall be equipped with the

following dump valves.

DUMP VALVE - NEWTON

There shall be one (1) model 1060 10" x 10" Newton dump

valve with a manual valve handle mounted on the left side of

the dump provided. The dump valve shall be mounted to the

water tank at the rear of the body.

Two (2)      DUMP VALVE - NEWTON

There shall be one (1) model 1085 10" x 10" Newton dump

valve with an electric actuator mounted on the right side of

the dump provided. The dump valves shall be mounted to the

right and left side of body behind rear wheel well and ahead

of rear compartment. Controls with indicator lights for

valve shall be mounted on the front and rear face of body.

Dump Valve shall be constructed from stainless steel.

EXTENSION CHUTE

The Newton dump valve provided shall have a model #4036

two-piece 36" manual telescoping extension chute provided

and mounted on the dump valve.

Two (2)      ELECTRIC EXTENSION CHUTE

The Newton dump valve provided shall have a model #5018

two-piece 18" electric telescoping extension chute provided

and mounted on the dump valve.

BODY CONSTRUCTION

The body shall be designed and fabricated using a high

strength copolymer material, a combination of polypropylene

and polyethylene, providing a durable, impact resistant,

corrosion resistant, and lightweight body. All seams shall

be welded.

Due to the importance of the strength and impact resistance

of the copolymer material, there shall be no exception to

the above paragraph.

The body shall be attached to and supported by a heavy duty,

spring loaded, steel sub frame bolted to the truck frame. The

sub frame shall be spring mounted to the chassis frame to

allow for independent flexing of the body in relation to the

chassis frame. The sub frame shall be powder coated. No

welding shall be allowed to the truck frame.

Due to the importance of the sub frame flexibility and

corrosion resistance, there shall be no exception to the

above paragraph.

There shall be an air space between the body sides and the

booster tank to allow circulation and minimize moisture and

condensation conditions in the compartments.

Compartment floors shall be constructed of the same heavy

duty smooth copolymer material. All seams shall be

completely welded.

Compartment interiors shall be provided in their natural

unpainted finish.

Each compartment shall be properly louvered for adequate

ventilation and air displacement for closing of the

compartment doors.

All compartments shall be sweep out with no lip at bottom

edge.

FENDER PANELS

Side fender panels above the rear wheels shall be heavy duty

smooth copolymer material. Each shall be painted the same

color as the rest of the exterior body.

FENDERETTES

Bright anodized aluminum fenderettes shall be bolted to the

wheel well openings.

FENDER LINERS

Copolymer fender liners shall be welded into the wheel well

above the rear wheels for ease of cleaning. Adequate

clearance shall be provided for the installation of single

tire chains.

HOSE BED

The hose bed walls shall be of unpainted smooth copolymer

material, reinforced at the corners.

The hose bed sides shall be of the same material. The upper,

outer edge shall have a solid tube type design for strength

and stiffness.

The hose bed shall be smooth and free from all projections

which may interfere with the unloading of hose.

Hosebed capacity shall be approximately 134 cu. ft.

capacity. Hose load requirement of 2800' of 3" DJ hose.

The hose bed floor shall be removable and constructed of

black copolymer material, designed to provide for drainage

and ventilation of hose.

DOORS

The body compartment doors shall be flush pan aluminum or

poly.

TREADPLATE AND TRIM

All treadplate shall be bright aluminum. All walking

surfaces shall be a minimum  of 1/8" or as designated

elsewhere in these specifications.

Compartment tops shall be over laid with 1/8" bright

aluminum treadplate.

The outer edge of the treadplate compartment tops shall be

broken down 45 degrees over the compartment sides.

The front face of the body sides shall be overlaid with

bright aluminum treadplate.

The rear of the apparatus inside of the extending side

compartments, and the rear panels below the hose bed shall

be covered with bright aluminum treadplate.

There shall be stainless steel trim installed over the rear

edge of the extending side compartments to protect the body

paint when fire hose is being deployed.

REAR STEP

The rear step shall be bolted to a heavy duty steel support

assembly attached to the chassis frame. An angle of

departure of at least 8 degrees is desired at the rear of

the vehicle.

The rear step shall be 10" deep and constructed of bright

aluminum open grating to promote safe footing and prevent

build up of debris. The rear step shall be full width

between the extended side compartments. The rear step shall

be bolted to the support assembly with a drain gap between

the rear body and the rear step.

REAR STANCHIONS

There shall be two stanchions, one each side, bolted to

uprights at the rear of the body for mounting of deck lights

and/or warning lights. Each shall be a bright polished

material and shall be designed to enclose the 12 volt wiring

of any lighting mounted to the stanchion.

RUB RAILS

Rub rails shall be installed using solid black rubber

material designed to help protect the lower body and cushion

against accidental contact. Each shall be mounted below the

lower side compartments and at the vehicle rear. Each end

shall have a hard black rubber end cap.

There shall be bright polished scuff strips mounted between

the body surface and the rub rails.

Per D.O.T., a marker light shall be mounted on the lower

front corner each side of the body, just ahead of each rub

rail.

HAND RAILS

Body handrails shall be 1-1/4" aluminum tubing mounted in

chrome-plated brackets. Each shall have rubber grip inserts.

Handrails shall be mounted as follows:

One (1) horizontal rail mounted at the rear just below the

hosebed.

Two (2) handrails shall be mounted on the hosebed sides at

the rear of the body.

Four (4) handrails mounted, two each side, at the forward

portion of the hosebed sides, at the pump compartment area.

COMPARTMENTATION LEFT

Ahead of rear wheels, approximately 62" wide x 40" high x

28" deep with vertically hinged double doors.

Door opening approximately 56" wide x 38" high.

Behind rear wheels, approximately 34" wide x 40" high x 24"

deep with vertically hinged single door.

Door opening approximately 28" wide x 38" high.

Above wheel wells shall be two compartments. The compartment

above the forward wheel well shall be approximately 54.5"

wide x 14.75" high x 28" deep with a horizontally hinged

drop-down door.

Door opening approximately 50" wide x 12" deep

Above the rear wheel well shall be a compartment

approximately 45" wide x 14.75" high x 28" deep with a

horizontally hinged drop-down door.

Door opening approximately 43" wide x 12" high.

COMPARTMENTATION RIGHT

Ahead of rear wheels, approximately 62" wide x 40" high x

28" deep with vertically hinged double doors.

Door opening approximately 56" wide x 38" high.

Behind rear wheels, approximately 34" wide x 40" high x 24"

deep with vertically hinged single door.

Door opening approximately 28" wide x 38" high.

Above wheel wells shall be two compartments. The compartment

above the forward wheel well shall be approximately 54.5"

wide x 14.75" high x 28" deep with a horizontally hinged

drop-down door.

Door opening approximately 50" wide x 12" deep

Above the rear wheel well shall be a compartment

approximately 45" wide x 14.75" high x 28" deep with a

horizontally hinged drop-down door.

Door opening approximately 43" wide x 12" high.

REAR STEP

The following accessories shall be mounted on rear step and

body face.

Three (3) hose wells constructed from aluminum, two (2)

shall hold 350' of 1-3/4" hose lines and one 50' of 1" hose

with TFT Mid Force nozzle with MX tips attached. One mounted

in center shall hold 150' of 1" hose. The hose shall be

stored in donut rolls on end. Bottom of hose wells shall be

rounded to follow contour of donut rolls.

Mounting shall be supplied for 6' piercing nozzle, flat head

axe, and 5' hook.

Two 3" discharges with Akron water thiefs shall be mounted.

DOORS

Aluminum doors shall be provided.

Doors shall be double pan, lap construction, with

full-length polished Stainless Steel piano-type hinges.

Doors including pans shall be approximately 1-5/8" thick.

Doors are to be completely weather-stripped.  Sealing of

door opening against entrance of water to be close cell,

Neoprene rubber, self-adhesive design gasket.

Polished Stainless Steel recessed "D" ring handle locks

shall be of slam-type, separate from door to permit

adjustment, thus assuring close-fitting doors.  All doors

shall be designed with two-point slam latches to prevent

door from accidentally opening. "D" ring locks shall be

screwed to compartment doors.  Latch catch to be screwed to

compartment to allow for future adjustment.

Free doors of any double-door compartments shall be latched.

Door hinges to be attached and bolted to the exterior door

jam to allow maximum door opening and to allow for

adjustment, if desired.

Spring hold open device shall be the positive holding

type, Cleveland or equal, requiring only one hand to shut.

HOSE BED FLOORING

A hose bed floor shall be provided and constructed of

textured black copolymer slat style material.  The slats

shall be properly spaced to allow passage of air between the

hose and the booster tank.

HOSE BED DIVIDERS

Three (3) adjustable copolymer hose bed dividers shall be

provided and installed in extruded tracks to allow

adjustment from side to side for alternate hose capacities.

The dividers shall have a textured black finish, and shall

have a radius corner on the rear portion.

The center divider shall be utilized to support aluminum

hose bed cover.

ALUMINUM HOSE BED COVER

A 1/8" pan formed aluminum tread plate hose bed cover will

be provided. Cover shall be two (2) door type with

continuous stainless steel hinge provided along each side.

Cover doors shall have door hold open device. Cover shall be

reinforced to allow firefighters to walk on.

FUEL FILL DOOR

There shall be a polished aluminum access door mounted on

the side of the body to cover the fuel cap and fuel fill

inlet.

ADJUSTABLE SHELVING

Four (4) heavy duty adjustable black copolymer shelves, with

edges turned up approximately 1" for added strength, shall

be provided.

Extruded aluminum tracks shall be provided to allow shelf

adjustment.  The tracks shall run the full height of the

respective compartment.

Shelving shall be located as follows:

ZICO VERTICAL LADDER RACK

A ZICO Quick-Lift aluminum ladder rack system shall be

provided on the side of the apparatus, above the body

compartments.  The rack shall store ground ladders in a

vertical position.

The rack shall be operated electrically, using two cast

assemblies with Warner 12V linear actuators.  The rack shall

be actuated by a switch located at the pump panel or the

apparatus rear, in view of the rack operator.

When activated, the rack shall lower the ladders to a

vertical position, along the side of the body, close to the

body.

Unless specified, the rack shall not operate with

compartment doors open on the side of the ladder rack.

Audible warning and visual lighting (including rack not

nested) shall be supplied on and for the rack as recommended

by NFPA, current at the time of order entry.

Two (2)      HARD SUCTION TROUGH

Unpainted, aluminum trough(s) with a sanded finish shall be

provided above the body side(s) to hold up to 24'ft. x 6"

hard suction hose(s).  There shall be two heavy duty velcro

straps used on each trough specified to hold the hose(s) in

a secure position.

Two (2)      SUCTION HOSE STORAGE

A compartment with open front shall be provided on side of

apparatus above low side compartments. This compartment shall

be constructed from polypropylene with aluminum diamond

plate tops. This compartment shall support a Zico ladder

bracket on one side and a Zico Portable tank bracket on the

other side. Velcro straps shall be provide to secure suction

hose in place. A hose trough shall be provided in

compartment to hose. Each suction hose shall have a strainer

connected to hose and a mounting bracket shall be for

strainer.

Right side shall hold a 14' section of 5" suction hose and

14' folding ladder. Left side shall hold two (2) 14'

sections of 3" suction hose, one with Kochek low flow

strainer, and a 14' folding ladder.

One (1)      PORTABLE FOLDING TANK MOUNT

A Zico Quick-Lift system, designed to carry a portable

folding tank (not included), shall be mounted on the side of

the apparatus. The folding tank rack is designed to hold the

portable folding tank in the upside down position when in

the raised position and right side up as it is lowered into

the unloading position. A suitable locking device to hold

tank in a stable position and to hold rack in the up/stored

position shall be provided. The rack shall be provided with

an aluminum diamond plate cover frabrticated to cover

portable tank. A bracket shall be fabricated to hold on 14'

roof ladder.

AIR HOSE REEL

A hose reel designed to hold 3/8" air supply hose shall be

mounted in the shelf over pump compartment. It shall be

designed to hold 150 feet of air hose. The hose reel, rated

at 250 psi, shall include a roller guide system and shall be

equipped with a manual crank rewind system. The reel shall

be supplied with air from the CAFS air compressor system on

the pump. Reel shall included carry handles and color coded

to match rigs.

AIR HOSE

150' of 3/8" air supply hose shall be supplied. Air hose

shall be rated at 250 PSI working pressure.

RUB RAIL

This unit will be equipped with a full body length removable

bolt on rubber rub rail. It will be mounted on the sides of

the body below the compartments, along the pump compartment

running boards, and the rear of the apparatus. It will be

designed to absorb small impacts with out transferring

damage to the body structure.

REAR TOW EYES

Two (2) heavy duty laser cut steel tow eyes, 3/4" thick with

a 3" I.D. cutout, shall each be bolted to the rear chassis

frame rails with four 3/4" grade 8 bolts.  The tow eyes

shall extend through the rear panel (or into the back wall

of the rear compartment, if present).

The tow eyes shall be painted black unless specified

otherwise.

FINISH

The apparatus shall be finish painted with a high luster

color.  The compartment doors, if painted, shall be painted

separately to assure proper paint coverage on the body

edges.

Any metal to metal surfaces not painted during final

painting shall be painted before assembly to assure proper

coverage and corrosion prevention.

The apparatus shall be sanded and filled as needed.  The

apparatus shall be properly prepped, primed, and finish

painted per the paint manufacturer's recommendations for the

type of material being painted.

The final color coats shall be Dupont Chrome and shall be a

high luster finish.  No exceptions.

The compartment interiors shall be unpainted and in their

natural finish.

A pint of touch up paint shall be provided for each color

used.

LETTERING

Lettering on apparatus shall be  as follows:

On removable panels:

E-# 6" RED reflective block letters with a "white"

reflective back ground on panel.

"Rattlesnake Fire Department" shall be in "gold bronze"

reflective 3" block letters on cab front doors.

"ISO Class # Fire Department" shall be in "gold bronze"

reflective 3" block letters on rear cab doors. Letters shall

be in two lines.

"RATTLESNAKE, CO. FPD" shall be in SMART GOLD Script Style

lettering on front of chassis below windshield. Maximum

height of letters to be 3". Customer to approve lettering

size and location prior to ordering.

Four (4)     LETTERING PANELS

A 12" x 18" removable aluminum lettering panel with aluminum

bracket shall be provided. Bracket shall be constructed from

smooth plate 1/8" aluminum. Lettering panel shall be

constructed from 1/8" smooth plate aluminum and slide into

bracket. Panel shall be finished per instructions in

lettering section,

6" REFLECTIVE STRIPE

A "white" 6" reflective stripe shall be provided on each

side of the apparatus, on the painted surface.

The center of the stripe shall be    50"   above ground level.

To comply with current NFPA standards, reflective striping

shall be applied to the side of the vehicle chassis and body

on at least 50% of the overall length of the vehicle. At

least 50% of the rear and 25% of the front of the vehicle

width shall have reflective striping applied.

ELECTRICAL SYSTEM - 12 VOLT

GENERAL

The 12 Volt DC electrical system shall conform to all

requirements as stated in Chapter 9 of the NFPA 1901-1996

edition standard for automotive fire apparatus.  All

required testing shall be performed before the apparatus is

delivered.  All required test documents shall be supplied at

the time of apparatus delivery.

All wiring for the apparatus shall be installed in a

workmanship-like manner, protected in loom or conduit.

Grommets shall be installed where wire passes through body

panels.  All wiring shall be function and color-coded and

connections made at pre-insulated terminals.

Automatic circuit breakers, located in a readily accessible

and protected location, shall be provided.  All circuits

added by apparatus builder shall originate from this single

source.  A separate circuit shall be provided for each

category of items.

A junction block shall be located in a readily accessible

and protected location at the apparatus rear.  This shall

accommodate wiring to all rear electrical devices including

stop, tail and turn lights.

WIRING DIAGRAM

Electrical diagrams of apparatus wiring shall be supplied at

time of delivery which include standard wiring color code

key and gauge of wire used.

SOLENOIDS/RELAYS

All high amperage draw lighting shall be wired to

incorporate either a solenoid or a relay to limit the

amperage through a switch to maximize the switch life.

All electrical circuit wiring installed by the apparatus

manufacturer shall be stranded copper alloy conductors of a

gauge rated to carry 125% of the maximum current for which

the circuit is protected.  Wiring shall be colored and/or

printed with circuit function code over each conductors

entire length.

ROCKER SWITCHES

A set of rocker switches with pilot lights shall be provided

in the cab for functioning of warning lights.

A master switch, with pilot light, to energize all the

rocker switches shall be provided on the rocker switch

panel.

Rocker switches shall be labeled.

HI TEMP-FUNCTION CODE

Apparatus body wiring to be high temperature compatible

wire, insulated with chemically cross-linked Polyethylene

and withstands prolonged temperatures of up to 350 degrees

F. without wilting or fusing. Wire shall be highly resistant

to grease, oil, fluids, break fluid and abrasion. Wire shall

meet of exceed S.A.E. certifications J1128.  Wire shall be

individually color coded and function name labeled every

three (3) inches on the insulation.

REAR DIRECTIONALS

Rear directional lighting shall be supplied as follows:

Two (2) Whelen 6" x 4" stop and taillights, one each side,

red lens.

Two (2) Whelen 6" x 4" turn signals, one each side, amber

lens.

Two (2) Whelen 6" x 4" back up lights, one each side, clear

lens.

BACK-UP ALARM

There shall be an electronic beeper that sounds when the

truck is placed in reverse.  The beeper shall be heard over

all engine noise, to warn persons near or on the truck.

WARNING LIGHT SYSTEM

The following warning lights shall include lights arranged

to be compliant with the four zones as recommended by the

1996 edition of NFPA 1901.  These warning lights shall be

configured as packages and certified by the respective light

manufacturer.

The upper and lower level zones shall be provided as one

certified package by one light manufacturer, or as two

packages, with one manufacturer certifying the upper zone

and another manufacturer certifying the lower zone.

CODE 3 WARNING LIGHT SYSTEM

LIGHT BAR

Zone A (Front-Upper)  One (1) pair of model #XL5100AF, 21"

lightbar two 50 watt standard rotators  with a diamond

mirror between rotators.

All red lenses.

This satisfies the requirement for the forward 1/2 of

side-upper zones B and D.

FRONT WARNING

Zone A (Front-Lower)  Two (2) model #OL13F, 35 watt

Oscilasers mounted, one each side, on the front face of the

cab. These lights shall be supplied with chassis.

Red lenses shall be provided.

SIDE WARNING

Zone B  (Right Side-Lower)  Two (2) model #40BZ, 7" x 3"

perimeter strobe lights located at the lower front side

corner on bumper extension, and rear side of the apparatus

near rear wheel well.

Red lens shall be provided.

Zone D  (Left Side-Lower)  Two (2) model #40BZ, 7" x 3"

perimeter strobe lights located at the lower front side

corner on bumper extension, and rear side of the apparatus

near rear wheel well.

Red lens shall be provided.

REAR WARNING

Zone C (Rear-Upper) Two (2) model #550F rotating beacons

with one 50 watt fast rotator in each, red lens, mounted one

each side on the rear lighting stanchions.

One (1) model #4135BZ, 35 watt halogen 7" x 3" perimeter

light, red lens, mounted below the center area of the

hosebed.  A #710 flasher shall be installed.

This satisfies the requirement for the rearward 1/2 of

side-upper zones B and D.

Zone C (Rear-Lower)  Two (2) model #4135BZ, 7" x 3" strobe

perimeter lights with bezels mounted, one each side, at the

lower rear of the apparatus.

Red lenses shall be provided.

POWER SUPPLIES

Two (2) #PSE475 power supplies shall be installed for the

eight (6) strobes.

WIRING REQUIREMENTS

All warning lights shall be wired to a single master switch

inside cab. All lights required to be shut down in blocking

mode, shall be deactivated thru parking brake.

SIREN CONTROL

A Code 3 model 3000 undercover siren control shall be

provided and installed. Siren controls shall be a single switch

located in cab in overhead switching and siren amplifier.

shall be mounted under dash or in a seat riser.

SIREN SPEAKER

A Federal Dynamax speaker shall be provided and recessed in

front bumper.

AUTOMATIC DOOR ACTIVATED COMPARTMENT LIGHTING

All apparatus compartment doors shall be provided with a

door activated compartment light switch. Each switch shall

operate that compartment's interior lighting. A red

indicator light, located in the driving compartment, shall

be illuminated automatically whenever any compartment door

is open and the parking brake is released.

HAZARD LIGHT/WARNING INDICATOR

There shall be a red flashing or rotating light, located in

the driving compartment. This light shall be illuminated

automatically whenever the apparatus's parking brake is not

fully engaged and any passenger or equipment door is open,

any ladder or equipment rack is not in the stowed position,

a stabilizer system is deployed, a powered light tower is

extended, or any other device is opened, extended, or

deployed that creates a hazard or is likely to cause damage

to the apparatus if the apparatus is moved.

The light shall be marked "Do Not Move Apparatus When Light

Is On."

Four (4)     RECESSED STEP LIGHT

There shall be a recessed step light mounted in such a

manner as to light the area around the step.

PUMP COMPARTMENT LIGHTING

Inside the pump compartment shall be installed one pair of

12 volt lights with switches that shall illuminate the pump

compartment area.

UNDER BODY GROUND LIGHTS

There shall be ten (10) under body ground lights provided.

The lights shall have clear lenses and shall be mounted

beneath the apparatus body to provide lighting under and

surrounding the apparatus. The lights shall be turned on

when parking brake is engaged.

Lights shall be mounted in the following locations:

(2) under front bumper extension

(1) under each side of pump panel

(1) under each front body compartment

(1) under each rear body compartment

(2) under rear step.

FOUR DOOR CAB GROUND/STEP LIGHTING

There shall be a light mounted to illuminate the ground and

stepping area of each entry door on the 4-door chassis cab.

Each light shall be automatically controlled by the cab door

switch.  Light shall be provided and installed by chassis

manufacturer.

LOOK DOWN LIGHTS

A pair of look down lights shall be mounted on front of cab

and shine down to front bumper. Light shall be switched via

parking brake. Lights to be switched on when parking brake

is engaged.

Light shall be a Weldon corrosion proof step well lamp. One

shall be mounted on each upper corner of grille.

SCENELIGHTS 13 DEGREE

Two (2) Whelen scenelights, each with 13 degree downward

angled optics and clear lens shall be provided.  The lights

shall be controlled in the cab.

The lights shall be located at the rear as dual backup lights.

120 VOLT ELECTRICAL SYSTEM

The 120 Volt AC electrical system shall conform to Chapter

19 of the NFPA 1901-1996 edition standard for automotive

fire apparatus.  All required testing shall be performed

before the apparatus is delivered.  All required test

documents shall be supplied with the owners manual at the

time of delivery.

All wiring for the fire apparatus shall be installed in a

workmanship-like manner, protected in loom or conduit.

Grommets shall be installed where wire passes through body

panels.  All wiring to be of adequate size to carry the

required necessary load.

A power source system specification label shall be provided

and permanently attached near the operator's control

station. It shall contain the following information:

Rated Voltage(s) and Type (AC or DC)

Phase

Rated Frequency (at rated voltage(s)

Rated Amperage

Continuous Rated Watts

Power Source Engine Speed

The power source shall meet all installation and testing

requirements as listed in the appropriate section of NFPA

1901-1996 Chapter 19-Line Voltage Electrical Systems.

Electrical diagrams of standard apparatus wiring to be

supplied at time of delivery, without exception, including

gauge of wire used.

HYDRAULIC GENERATOR

This advanced fire fighting apparatus is to be equipped with

an AMPS 25 KW modular hydraulic generator system. It is

intended to be installed in a manner that will not interfere

with any usable compartment storage space. The entire unit

is enclosed within a stainless steel enclosure and can be

mounted inside or over the pump compartment area.

ELECTRICAL INSTALLATION  120/240 VOLTS

This unit is capable of 120/240 volts and 250/125 Amps.

HYDRAULIC HOSES

All hydraulic hoses shall be high pressure type, approved

for this application.  The hoses shall be wire braid with

steel crimp-on couplings.  The hoses shall be securely held

in place and protected from oil, heat and /or other

mechanical damage.

MOUNTING

The hydraulic generator module shall contain all system

components and related items necessary to comprise a

complete hydraulic generating system with exception of the

system pump and hoses. The system shall be assembled and

tested prior to installation into fire apparatus.  The

components shall be grouped and assembled into a compact

modular unit with overall dimensions of 47.5" Long x 21"

High x 19.3" deep.

TRANSMISSION, POWER-TAKE-OFF DRIVEN:  The hydrostatic pump

shall be driven by a power-take-off at the truck

transmission.  The hydraulic pump shall be close-coupled to

the PTO or driven by balanced propeller shaft as

installation requires.

A modified load sensing variable displacement pump is used

that produces the exact flow to maintain 60 HZ regardless of

RPM changes. There is to be NO wasted flow or horsepower.

The remote control combined with the constant engagement PTO

allow you to turn a pre loaded AMPS system off and on at any

RPM, any time.

A quad meter to display system information will also be

installed near circuit box.

GENERATOR PERFORMANCE

Manufacturer                       Amps

Rating                             25,000 Watts

Volts                              120/240

Phase                              Single or three

Cycles                             60 Hertz

AMP Rating                         250 AMPS at 120 Volts

125 AMPS at 240 Volts

Weight                             425 Lbs.

Nine    110V TWIST LOCK RECEPTACLE

There shall be 110v twist lock dual port receptacle mounted as

directed by the Fire Department.

Any outlet 15 amps shall be dual receptacles

Two dual outlet with standard 3-prong receptacles shall be

mounted on rear wall of cab.

SHOREINE RELAY

A heavy duty 30 amp change-over relay shall be installed to

switch over outlets in cab powered by shoreline to switch

over to generator power when apparatus is started.

Six (6)      1000 WATT PERMENANT MOUNT QUARTZ LIGHT

A Tele-Lite model F-10 1000 Watt 110 volt permanent mount

quartz light. Light shall be wired to circuit box and

controlled by 12 volt switch located in cab.

Two (2)      1000 WATT TELESCOPING QUARTZ LIGHT W/TRIPOD

A Tele-Lite model TT-10 1000 watt, 120 volt telescoping

quartz light with tripod base shall be supplied and

installed. A 120 volt twist lock outlet shall be provided

near light mounting location. A model TTVM mounting bracket

shall be provided. An on/off switch shall be provided on

lighthead, to control light when removed from vehicle.

The base for lighthead shall include and outlet with weather

proof cover and recessed plug.

Three (3)    750 WATT PORTABLE QUARTZ LIGHT

A Tele-Lite model PR portable quartz light base shall be

supplied and mounted as directed. Light shall include a

model PB-103 portable light mounting base. Base of light

shall include an outlet with weatherproof cover and recessed

plug.

Mounted to base shall be a Kwik-Raze model 27 Alpha 2000 750

watt quartz light with light switch mounted on lighthead.

TELE-LITE WARRANTY

Tele-Lite products provided in this proposal shall be

covered by Tele-Lite Full Three Year Guarantee.

For Three years from original date of purchase, TELE-LITE

Inc. will provide, free of charge parts and labor that is

required due to a manufacturing defect. Proof of original

purchase is required.

Lamp is guaranteed for 30 days.

Two (2)      RECESSED QUARTZ LIGHTS

A pair of Fire Research model FC300-M12 1000 watt 120 volt

Focus recessed quartz lights shall be provided and

installed in a housing mounted to face of cab above

windshield above officer and driver. Lights shall be

switched in the cab.

The housing shall be constructed from smooth plated aluminum

and painted to match color of cab roof.

Five (5)     12 VOLT SWITCH W/RELAY

There shall be a switch with a 12 volt DC operated relay to

operate the 120/240 volt accessories.  The switch shall be

mounted in the cab.

Switches shall control the following items:

Front quartz lights mounted on cab roof

Quartz lights mounted on each side of apparatus

Rear telescoping quartz light

Electric rescue tool hydraulic pump

NIGHT-SCAN ROOF MOUNTED LIGHT TOWER

Provide and install one (1) extendable roof mounted lighting

system. Unit to be self-contained, with RCP directional

lighting system mounted at top of mast, with 385 degree

rotation and 330 degree tilt. Unit to have "Auto Stow"

feature and wired to hand-held remote control. Unit to be

connected to vehicle generator system. Light tower to extend

11 ft. above mounted location. Four 1500 watt quartz lights

equaling 6000 watts to be mounted at top of mast to

directional system.

A look-up light shall be provided with the unit.

STROBE LIGHT

A 12 volt Red strobe light shall be provided and installed

on top of pan & tilt box at top of light tower. Light shall

have switch located in cab.

WARNING LIGHT AND ALARM FOR LIGHT TOWER

A flashing warning light shall be installed in the cab to

inform the driver that the light tower or telescoping lights

are not in their stowed position. An audible alarm shall

sound if the driver releases the parking brake while this

light is flashing. A warning label shall be posted near the

light indicating to the driver not to move the apparatus

while the light is flashing.

110 VOLT BREAKER PANEL WITH 8 BREAKER

A twenty-two (22) place weather proof type breaker box with

twelve (12) 20 amp 120 volt circuit breakers will be

installed.

Breaker box shall be mounted in pump compartment or in right

front. A stainless steel access door shall be provided to

provide access to breaker box without removing panel.

Five (5)     GROUND FAULT CIRCUIT BREAKER - 20 AMP 110 VOLT

There shall be one (1) 20 AMP 110 volt A/C, single pole

ground fault interrupter (GFI) circuit breaker, provided in

lieu of the standard 20 AMP breaker.  The GFI breaker will

be wired to only one (1) electric receptacle or electric

cord reel.

Eight (8    GROUND FAULT CIRCUIT BREAKER - 20 AMP 220 VOLT

There shall be one (1) 20 AMP 220 volt A/C, double pole

ground fault interrupter (GFI) circuit breaker provided in

lieu of the standard 20 AMP breaker.  The GFI breaker will

be wired to only one (1) electric receptacle or electric

cord reel.

Five (5)     MANUAL REWIND CORD REELS

Manual rewind, portable cord reel(s) with capacity for 150

ft. of 12 gauge, 3 conductor electric cable shall be

supplied.  Reel(s) shall be located at order approval. Reels

shall be painted to match color code of each apparatus

ordered.

Fifteen (15)

ELECTRIC CABLE

Each cord reel specified shall have 50 ft. section(s) of

12/3 yellow electrical cable installed on the reel.

Cord shall be mounted on each reel in 50' sections

Thirty (30)

110V TWIST LOCK RECEPTACLES

Each reel specified, with cord, shall have a 110v twist lock

plug mounted on the end of the cable.

Five (5)     PIGTAIL ADAPTER

A 15 amp 3 prong twist x 3 prong household male plug pigtail

shall be provided. Pigtail shall have a minimum of 8" 12/3

yellow cable between plugs.  A mounting bracket shall be

provided for pigtails.

FIRE EQUIPMENT

SPECIFICATION SECTION

EQUIPMENT REQUIREMENTS

The following list of equipment shall be supplied and

installed on apparatus, unless specified otherwise.

Brackets not included in these specifications but needed to

mount equipment shall be supplied, if mounted by Darley.

Equipment listed shall be provided for each truck ordered

unless specified otherwise.

Final mounting location of equipment shall be determined by

department during inspection trips.

Any equipment not mounted will be shipped loose with

completed apparatus

COLOR CODED ACCESSORIES

Some items specified are to be color coded. The colors

requested by customer shall be as follows:

One shall be RED

One shall be BLUE

One shall be GREEN

Equipment that requires color coding shall be specified with

that particular equipment.

EQUIPMENT LABELS

Reflective labels shall be provided to mark department

equipment. Allowance for 275 labels with reflective

backgrounds per rig is provided per vehicle ordered. Label

background colors shall match colors specified for each rig.

Labels shall rear as follows:

"Property of"

"RATTLESNAKE, CO. FIRE DISTRICT"

"303-841-8111

10 FT. FOLDING LADDER

10 ft. Duo Safety aluminum folding ladder shall be

provided with rubber feet.  Ladder(s) shall meet or exceed

the latest NFPA standards.

Two (2)      14 FT. FOLDING LADDERS

Two (2) 14 ft. Duo Safety aluminum folding ladders shall be

provided with rubber feet.  Ladders shall meet or exceed the

latest NFPA standards.

One ladders shall be mounted in each suction hose storage

above compartments.

14 FT ROOF LADDER

14 ft. Duo-Safety aluminum single section roof ladder(s)

with folding steel roof hooks on one end and steel spikes on

the other end shall be provided.  Ladder(s) shall meet or

exceed latest NFPA standards.

24 FT EXTENSION LADDER

24 ft. Duo Safety 2-section aluminum extension ladder(s)

with steel spikes shall be provided.  Ladder(s) shall meet

or exceed NFPA standards.

35 FT 3-SECTION EXTENSION LADDER

35 ft. Duo Safety 3-section aluminum extension ladder(s)

with  steel spikes shall be provided.  Ladder(s) shall meet

or exceed the latest NFPA  standards.

Two (2)      HARD SUCTION FLEXIBLE HOSE

Section(s) of 14 ft. x 3" Kochek flexible suction hose

shall be provided.  The hard suction hose shall be the

flexible type with light weight CAM-LOCK couplings. Coupling

shall be    COLOR CODED   and stenciled with    RATTLESNAKE F.D.

HARD SUCTION FLEXIBLE HOSE

Section(s) of 14 ft. X 5" lengths of Kochek suction hose

shall be provided.  The hard suction hose shall be the

flexible type with lightweight CAM-LOCK couplings. Couplings

shall be    COLOR CODED   and be stenciled with    RATTLESNAKE

F.D.

FOL-DA-TANK

A 3500 gallon "Fol-Da-Tank" portable folding water tank

shall be provided. The frame shall be constructed from

resistance welded 14-gauge 7/8" steel tubing with a snag

free finish. The frame features die-cut cadmium-plated

hinges, brass grommets, and heavy-duty bolts. The liner

shall be made from 22 oz Hypalon material. Two drains,

two connector clamps and two sets of bar clamps shall be

provided with the tank.

Colors of hypalon for tanks shall be as follows:

RED, BLUE and GREEN.

Each tank supplied shall be stenciled with "RATTLESNAKE

F.P.D. PARKER. CO.".

Twenty Two (22)

DARLEY 1-3/4" HOSE - 50' LENGTHS

50-ft. length(s) of x 1-3/4" Darley Cord 800 psi fire hose

with 1-1/2" NST couplings. Hose shall be color coded with

Kochek hard coat colored anodized couplings  . Coupling

shall be laser etched with    RATTLESNAKE F.D.  2 inch hose

Hose color shall be as follows:

400' of RED

400' of BLUE

150' of ORANGE

150' of YELLOW

1100' TOTAL PER RIG

This amount of hose is required per rig ordered.

Fifty Seven (57)

3" DARLEY CORD 800 - 50'

50-ft. length(s) of X 3" Darley Cord 800 fire hose with

2-1/2" NST couplings.  Hose shall be color  coded with

Kochek hard coat colored anodized couplings  .  Couplings

shall be laser etched with    RATTLESNAKE F.D.  .

Hose color shall be as follows:

1400' of YELLOW

1400' of ORANGE

20' of RED

2820' TOTAL PER RIG

Fifteen (15)

1" FIRE HOSE - 50' LENGTHS

50-ft. length(s) of X Darley Cord 800 psi 1" fire hose

colored with 1" NST Kochek hard coat colored anodized

couplings. Couplings shall be laser etched with    RATTLESNAKE

F.D.

Colors for hose shall be as follows:

50' of RED

50' of BLUE

50' of YELLOW

50' of ORANGE

50' Of TAN

500' of Color of Choice

750' TOTAL PER RIG

This amount of hose is required for each rig ordered.

HOSE JACKET

A Darley leather hose jacket shall be supplied.

One (1)      HOSE GASKETS

A set of hose gaskets shall be provided. The set shall

consist of the following gaskets.

(3) each 1" hose gasket

(3) each 1-1/2" hose gasket

(3) each 2-1/2" hose gasket

(3) each 5" storz draft gasket

(3) each 5" storz hose gasket

GASKET HOOK

A coat hook style hook shall be provided for mounting hose

gaskets.  Seven hooks shall be provided.

HOSE CLAMP

A Darley U875 hose clamp for 3" hose with mounting bracket

shall be provided.

HYDRANT STRAP

A Darley #AC960 hydrant strap shall be provided.

3" LOW LEVEL STRAINER

A Kochek 3" low level strainer with foot valve shall be

provided and installed on apparatus. Strainer shall have 3"

hard coat colored anodized CAM-LOCK male coupling. Strainer

shall be laser etched with Rattlesnake F.D.

STRAINER BRACKET

A fabricated mounting bracket for a Kochek 3" low level

strainer shall be provided and installed.

Four (4)     REDUCER

A TFT model B-A 1-1/2" to 1" shaped reducers shall be

supplied.

NOZZLE

An Akron #489 Akron smooth bore tip with 2-1/2" F NSt

base and 2" orifice shall be provided.

APOLLO DECK / PORTABLE MONITOR - AKRON 3423

An Akron 3423, Apollo Deck/Portable monitor complete with

pyrolite stacked tips and stream shaper shall be provided

and mounted as noted on print. A portable ground base shall

be provided with fold-away legs and 2-1/2" female swivel

inlets. The Apollo lift off deck gun is rated for up to 1250

GPM and shall be removable with the use of pins from the

direct mount base on the truck. Any plumbing shall be noted

in plumbing and pump section.

DECK GUN EXTENSION PIPE

An Akron #3405 18" deck pipe extension shall be provided and

mounted with bracket near deck gun. Extension pipe shall

include indicator switch with warning light mounted in cab.

Seven (7)    NOZZLE

A TFT model HD-TO Dual Force nozzle with color coded H-VO

handles to match hose lines.

Colors to be Green, Red & Blue.

Two (2)      NOZZLE TIPS

A TFT FJ-MX-HM MX tips shall be provided for Mid Force

nozzle.

One (1)      PIERCING NOZZLE

A Superior Flamefighter #2500 piercing nozzle with 2 6'

color coded extension shall be supplied and installed on each

vehicle ordered.

Colors shall be as follows:

One rig shall be black.

One rig shall be gold.

One rig shall be Red. (4) 3' Nozzle Pieces for Red.

One (1)      PIERCING NOZZLE

An Akron style 1088 piercing nozzle shall be provided.

Seven (7)    FALLON TIPS

An Iowa American set of Fallon Tips shall be provided. Set

shall include the following:

(1) IA-A-NYMT-15 New York Style Main tip 1-1/2"NST inlet &

15/16" Outlet

(1) IA-A-NYOT-50 New York Style outer tip 1/2"

(1) Ia-A-MIT-15 Miami Style Stub Tip 1-1/2"F X 1-1/2"M X

1-1/8" tip.

Seven (7)    FALLON NOZZLE ADAPTER

An TFT VOF Fallon nozzle adapter and 1-1/2" valve shall be

provided.

One (1)      CELLAR NOZZLE

A 1-1/2" Cellar nozzle, catalog # AF242 shall be provided.

Two (2)      AXES

An Iowa American 8 lb stainless steel flat head axe with a

fiberglass handle will be provided and installed.

An Iowa American 8 lb stainless steel pick head axe with a

fiberglass handle will be provided and installed.

One of each type to be provided per truck.

Two (2)      AXE BRACKET

A chrome plated Axe blade bracket and handle clamp will be

provided.

Two (2)      5' HOOK

An Iowa-American all steel roof hook with pinch ends and

Hawk 2 Stainless steel heads with lock pullers, key

(1)IAEOTSK tool (1)AEOTK1 and mounts.

PIKE POLE

An Iowa-American 8' fiberglass pike pole with Super Hawk

head and D-handles will be provided and installed as

directed.

An Iowa-American 10' fiberglass pike pole with Super Hawk

head end and D-handle will be provided and installed as

directed.

An Iowa-American 12' fiberglass pike pole with Super Hawk

head and D-handle will be provided and installed as

directed.

An Iowa-American 14' fiberglass pike pole with Super Hawk

head and D-handle will be provided and installed as

directed.

Six (6)      PIKE POLE BRACKETS

A set of Zephyr pike pole brackets will be provided and

mounted as directed.

Twelve (12) COVER 12' X 18' SALVAGE

A 12' x 18' vinyl-nylon, fire resistant, salvage cover will

be provided.  Salvage covers will be color coded to the vehicles,

Red, Blue and Green.

BOLT CUTTER

A 36" Fire Hooks Unlimited bolt cutter will be provided.

BOLT CUTTER BRACKET

A bolt cutter bracket will be provided.

GARAGE DOOR TOOL

A Fire Hooks Unlimited keyless garage door entry tool shall

be provided.

RESCUE KNIFE

A V-Blade rescue knife, catalog #L470 shall be provided.

SPANNER WRENCHES

A pair of Akron #46 Fitzall combination spanner wrench shall

be provided

HYDRANT WRENCHES

An Akron model 15 combination hydrant wrench shall be

provided.

Three (3)    HYDRANT WRENCH/SPANNER COMBO

A set of Akron model 2442 triple wrench holder with wrenches

shall be provided. Set shall include a pair of 46 Fitzall

combination wrench and one 15 combination hydrant wrench.

SPANNER WRENCH HOLDER AND 2 WRENCHES

An Akron holder with two (2) 46 Combination Spanner wrenches

will be provided.

CAP WRENCH

A cast aluminum alloy, cap wrench shall be provided which

aids in the loosening/tightening of 70mm foam container

caps. Wrench is 9-1/2" long with an outside diameter of

3-1/4".

RESCUE TOOL

A Phoenix 3525 Combination Cutter/Spreader shall be provided

and installed on apparatus.

A total of ONE TOOL for entire order shall be supplied.

POWER UNIT

A Phoenix  220 volt simo tool power unit shall be supplied.

Power unit shall be plumbed to the reel mounted in the dunnage

area and right side of pump compartment.

An electric control shall be provided to control the front and

side extrication tool reel diverter valves for power unit.

RAM

A Phoenix #2560 Ram shall be provided and installed.

Two (2)      HOSE REEL

A Phoenix Hydraulic dual hose reel with manual rewind shall

be supplied and installed. Reel shall include 150' hose.

Provide proper connection and guide rollers.  Reel to plumbed to

power unit mounted above pump.

MOUNTING BRACKETS

Mounting brackets for Phoenix rescue equipment shall be

supplied and/or fabricated as required to meet installation

requirements of department.

Four (4)     AIR BAGS

Four (4) Maxiforce 20" x 20" air bags shall be provided.

AIR BAR CONTROL

A control package for air bags shall be provided. Package

shall include One (1) pressure regulator, One (1) dual

deadman safety relief & control valve w/16' of black hose.

(2) 16' hoses. Two unique colors of hose for each rig.

(4)  ParaTek #890490 - air bag on/off inline relief valve.

AIR HAMMER

One (1) Standard Duty Air Hammer Kit for use with 4500 psi

tanks, catalog # S878A shall be provided.

Kit shall include the following item:

(1) Air Hammer

(1) Retainer

(1) Regulator - 4500 psi

(1) Pry Bar 15"

(2) Double Blade Panel Cutter

(2) Kwik Cutter

(2) Kwik Cutter - 18"

(1) 15' 1/4" Hose

(1) 1/4" Male Connector

(1) 1/4" Male Coupler

(1) 1/4" Female Coupler

(1) Gauge Guard

(1) Steel Box

(1) 4 oz. Oil

AIR VALVE

A 3-Way air valve shall be provided for use with air tools

and air bags.

GAS DETECTOR

An AIM 600 #953-600-503 gas detector with charger shall be

supplied and installed.

Two (2)      RESCUE SAW

Two (2) Partner K650 12" 70cc rescue saw shall be provided.

BLADE MOUNTING

A bracket for mounting saw blade for K650  saw shall be

provided. Bracket shall consist of marine grade plywood

mount to wall or shelf of compartment. A bolt shall be

installed thru plywood with a black plastic knob to hold

blade in place.

Two (2)      SAW BLADE

A Da Ax carbide tip blade shall be provided.

Two (2)      SAW BLADE

A 12" Dynabalde shall be provided for each saw. Blade shall

be a carbide chip blade with white edges. Blade dimensions

shall be 12" x 250 x1". Blade shall be manufactured by Team

Equipment part #DYNA12.

ELECTRIC POWER SAW

A Milwaukee Sawzall or equal reciprocating saw shall be

provided. Saw shall include two (2) packages of Stanett .050

vari-pitch demolition blades.

FUEL CAN

A one gallon safety fuel can shall be supplied and installed

as directed.

Two (2)      SAW SLING

A Fire Hooks Unlimited Power Saw Sling and Safety Strap

shall be provided.

20LB ABC EXTINGUISHER

ABC stored pressure dry chemical extinguisher shall have a

heavy steel wall shell and corrosion resistant aluminum

handles.  All extinguishers have a pressure gauge and come

pressurized with a Triplex dry chemical.

EXTINGUISHER BRACKET

There will be a fire extinguisher bracket provided and

mounted as  directed.

2-1/2 GALLON PRESSURE WATER EXTINGUISHER

There shall be a 2-1/2 gallon pressure water extinguisher

with Iowa American shoulder strap mounted as directed by the

Fire Department.

2-1/2 GALLON EXTINGUISHER MOUNTING BRACKET

Vehicle extinguisher brackets shall be the hinged latch type

with rubber bumpers to protect extinguisher.  Brackets shall

have a high corrosion and impact resistant finish.

Seven (7)    SCBA SYSTEM

A Scott AIR-PACK FIFTY shall be provided and installed as

directed. System shall include 30 minute 4.5 carbon bottle,

integrated PASS device and nose cups.

GRACE T-PASSACCOUNTABILITY COMMUNICATION SYSTEM

A Grace T-Pass II System shall be provided and installed. The

system Shall include 8 transmitters per rig for turnout mounting

and a LCD receiver with charger mounted in cab for two rigs.

Seven (7)    SPARE SCBA BOTTLE

A Scott 4.5 30 minute Air-Pack Fifty carbon bottle shall be

supplied.

Seven (7)    MASK BAGS

Seven air mask bags #AF108 shall be supplied.

SMOKE EJECTOR/MISTER FAN

A 16" Mister Fan, catalog #AK067, 1/3 horsepower motor rated

at 3450 RPM 5200 C.F.M. 115V 60 cy AC with dimensions of 13L

x 18-3/4W x 19-3/4"H

Smoke ejector to include 50' of cord w/3-prong twist plug.

BLANKET

A Team fireproof blanket shall be provided.

THERMAL IMAGER

A Bullard Thermal Imager camera with data transmitter shall

be provided.

One Imager shall be provided for each rig ordered.

Darley to install receiver and antenna in cab.

Colors to be (1) Blue & (1) green & (1) Red

ADAPTER GENREAL NOTES

The following list of adapters shall be supplied by Kochek

unless noted otherwise.

Adapter quantities specified a per each vehicle ordered

unless noted otherwise.

All adapters shall be hard coat anodized color coded.

All adapters shall be laser etched with    RATTLESNAKE F.D.

Seven (7)    ADAPTER

A Kochek 1-1/2" NST Male x 1" NST Female Swivel adapter

shall be provided.

Three (3)    ADAPTER

A Kochek #54R 2-1/2" NSt male x 1-1/2" NST female rocker

lug swivel adapter shall be provided.

Three (3)    ADAPTER

A Kochek #P_1552 1-1/2" NST rocker lug plug and chain shall

be provided.

Four (4)     ADAPTER

A Kochek 6" NST F SW x 5" Cam Lock shall be provided.

Three (3)    ADAPTER

A Kochek 5" M Cam Lock x 3" F Cam Lock adapter shall be

provided.

Four (4)     ADAPTER

A Kochek 3" M Cam Lock x 2-1/2" NST F Swivel adapter shall

be provided.

Adapter - 2-1/2" NST F LH SW x 3" M Cam Lock

Seven (7)    ADAPTER

A Kochek 3" F Cam Lock x 2-1/2" NST Male adapter shall be

provided.

Six (6)      ADAPTER

A Kochek #37R 1-1/2" NST female rocker lug x 1" male

adapter shall be provided.

Two (2)      ADAPTER

A Kochek 3" male plug for cam lock fittings shall be

provided.

Two (2)      ADAPTER

A Kochek 5" male plug for cam lock fitting shall be

provided.

Eleven (11)

ADAPTER

A Kochek # 37R 2-1/2" NST female rocker lug x 1-1/2" NST

male adapter shall be provided.

Three (3)    ADAPTER

A Kochek #C_105_ 1" cap with chain shall be provided.

ADAPTER

A garden hose wye, catalog #AJ09901 shall be provided.

Two (2)      ADAPTER

A Kochek #36R 1" NST double male adapter shall be provided.

Two (2)      ADAPTER

A Kochek #36R 1-1/2" NST double male adapter shall be

provided.

Four (4)     ADAPTER

A Kochek #36R 2-1/2" NST double male adapter shall be

provided.

Two (2)      ADAPTER

A Kochek #35R 1" NST double female adapter shall be

provided.

Two (2)      ADAPTER

A Kochek #35R 1-1/2" double female adapter shall be

provided.

Four (4)     ADAPTER

A Kochek #35R 2-1/2" NST double female adapter shall be

provided.

One (1)      ADAPTER

A Kochek 2-1/2" NST F x 5" Storz adapter shall be provided.

SIAMESE

An Akron 1262 Siamese – 2  x2-1/2" NST F inlet x (2) 2-1/2"

NST M outlet with clapper valve shall be provided.

VALVE

A 2-1/2" nozzle ball valve Elkhart #275B shall be provided.

NOZZLE

One 2 ½” base with 2 inch smooth bore tip