DréCB

CB/HAM RADIO

Information Site

http://go.to/dre-cb

www.cbradio.myweb.nl

http://members.tripodnet.nl/DreCB/

___________________________________

___________________________________

ICOM IC-751

MODIFICATION 37 MHz

Modify your Icom 751 to operate at an extended frequency range of

up to 37 MHz.

This is how :

Mark developed a clever way to read and interpret the contents of

the Icom RAM chip (the old battery backed-up one) and alter its

frequency limits in the GENeral coverage and HAM modes.  This was

done in conjunction with the modification of his 751A to operate

six meters.  My interests are in listening to the low band skip

present above the ten meter band during band openings, so I asked

Mark for simply a new upper frequency limit, leaving the six

meter mods to the very ambitious.

First, I changed the battery on my RAM board since mine was eight

years old, and I didn't want Icom messing up my custom program if

my battery died later.

I then shipped the board off to Mark, and he changed my GEN mode

limits from .10-30 MHz, to .01 to 60 MHz (subject to VCO and RF

limits, naturally).  He also customized my HAM mode limits as

shown:

Before                  After

1.8-2.0               	27.98-30.62   Default HAM mode selection

changed from 160M

3.45-4.1              	31.98-33.02

6.95-7.5              	33.98-35.02

9.95-10.5             	 1.78- 2.02

13.95-14.5             3.48- 4.02

17.95-18.5             6.98- 7.32

20.95-21.5             9.98-10.17

24.45-25.1            13.98-14.37

27.95-30.0            18.05-18.18

none                  	20.98-21.47

none                  	24.86-25.01

These new HAM mode limits provide tighter control while hamming.

Note the tight 12M and 17M limits.  Much more realistic.

One problem encountered during the mod was that the rig's

processor (IC-14) instructs the rig to select the proper VCO and

bandpass filter for a given frequency, and it uses the highest

BPF up until 31 MHz, then must be further instructed as to what

to do at frequencies > 31 MHz.

IC-14 has several pins which turn on and off to do this

instructing.  Mark found that pin 18 (originally not used) of

IC-14 turns on at >31 MHz.  This could be used to turn on the

highest BPF and VCO, thus allowing operation to higher

frequencies.  This was done physically by cutting the trace from

pin 17's normal route, adding a diode to each pin 17 and 18 of

IC-14, and joining the anodes of the two diodes to the circuit

originally driven by pin 17, but now isolated by the trace cut.

The effect is the use of the highest BPF and VCO until 31 MHz

(pin 17) , and then the same BPF and VCO from 31 MHz and beyond

(pin 18).

As expected, VCO4, as originally adjusted, dropped out at a low

frequency (32.33 MHZ).  Since the VCO has a range of about 15 MHz

(and was originally employed to cover only 8 MHz with 3.5 MHz of

lagniappe on each end), and kicks in at 22 MHz, I adjusted it for

a comfortable low limit of 22 MHz (with little margin), which

lifted the upper range to a maximum observed limit of 37.175 MHz.

This was done by giving C107 on the VCO unit 1/4 turn CW.

I haven't yet done any sensitivity measurements, as I'll have to

borrow the equipment from my employer when its convenient for

him.  I don't know where, or to what extent the bandpass filter

will cut off on the way to 37 MHz, if it does at all.

I believe the rig "hears itself" on a few certain frequencies,

with strong signals at 30.715, 31.33, and 30.95.  I believe this

is normal as Icom probably moved these birdies to >30 MHz

purposely.  There are few other spurs.

CONCLUSIONS: I borrowed an R-7000 from a friend some months ago,

and found that my favorite listening was in the lower 30's.  Now,

my 751 is as good to me as having my own R-7000 for what I would

use it for.  In the ham mode, its much easier to stay in the ham

bands (especially WARC bands) with the new program, and the

default band of 10M much more suits my tastes than the old 160M

default band.

===============================

ICOM IC-751

SPLIT MODE MODIFICATION

GOAL OF MODIFICATION:

Allow CW Xmit & USB recieve split mode to still use the CW VOX. This allows

one to operate with limited privaledges when communicating with SSB stations

above 10MHz. Below 10MHz, the CW receiver is actually LSB so split operation

in that case is effortless and does not require the split oepration to be

invoked. The original bogus operation can be confirmed by placing the rig

into CW Tx and USB Rx split operation and operate the key. A more `graphic'

example of this operation is CW Rx and USB Tx split and operate the key

PARTS REQUIRED:

1 4.7Kohm resistor and 2 1N914/1N4148/1N4448 diodes

SCHEMATIC:         +8V (Pin 14 of IC1)

|

>

<

> 4.7Kohm

<

>

|

+-------------//-----------------+

|                                |

+-------+------(----------+                     o

|       |      |          |                    /

|       |1N914 | 1N914    |                   /

|      _|__   _|__        |               +--o   Split Switch

|      \  /   \  /        |            ___|___

D11  |     __\/__ __\/__       |            / / / /  o-----> to `split' display

| /| |       |      |          |     |                       segment driver

|/ |_|_\/____|______|______\/__|____ |

|\ |   /\    |      |      /\       \|

| \|         >      >                |

< R29  < R26

>5.6K  > 4.7K

<      <

|      |

DESCRIPTION:

On the Main PC board, there is only one cut trace because R29 & R26 are

both fed from 1 trace and isolated from the rest of the circuitry. One of

the diodes is surface mounted on the solder side of the board over the

cut. R29 is unsoldered so that a thin wire (from w/w or pulled from a

ribbon cable) is then subsequently inserted with the R29 lead back into the

hole to bring R29+R26 common point up to the top of the main board. The

4.7Kohm resistor is soldered to pin 14 of IC1, the other end of the resistor

is soldered to the anode of the other diode's anode and another long wire

to be routed to the split switch. The cathode of the diode is attached to

the wire inserted earlier with R29. All the components may be surface mounted

if desired on the bottom of the main board, but I chose to perform this mod

on the top of the board to allow easier removal of the main board for

servicing. The long wire that was attached to the second diode and the

resistor is routed to the empty pin on the split switch that is driven to

ground when the switch is in the OFF position.

BUGS:

The CW VOX will be active when using split mode, even if neither the

transmit or receive mode are the CW mode.

==================================