CUSTOMER TO SERVICE PROVIDER TROUBLE ADMINISTRATION 
INFORMATION AGREEMENT

NMF 601

Issue 1.0

March 1997

ã NMF 1997.

Members of the NMF are granted limited copyright waiver, under the terms of the Agreement on Intellectual Property Rights, to reproduce this document.

DISCLAIMER AND COPYRIGHT STATEMENTS

The Network Management Forum (“NMF”) has made every effort to assure that the contents of this document are accurate; however, due to the inherent complexity in the design and implementation of protocols, no liability is accepted for any errors or omissions or for consequences of any use made of this document.  Under no circumstances will the NMF be liable for direct or indirect damages or any costs or losses resulting from the use of this specification.  The risk of designing and implementing products in accordance with this specification is borne solely by the user of this specification.  This document may involve a claim of patent rights by one or more NMF members, pursuant to the agreement on Intellectual Property Rights between NMF and its members, and by non-members of NMF.

ACKNOWLEDGMENTS

The following NMF SMART CTT team members have contributed their expertise to the production of this document:

Adam More Gordon (chair), SITA

Gary Guidon, Bell Sygma,

Lorraine Kevra, AT&T

Tim Le Thorp, Remedy

Anil Prasad, Cisco

Kirk Shrewsbury, Worldcom

David Tootill, Computer Associates

Adrian Tang and Anna Uy at University of Missouri-Kansas City

Dan Folts, NASA

Liz Zeisler, Dan Bolton and John DeCarlo at MITRE

Lindsay Carpenter and Bill Reed at DISA

Also many thanks to the following students at University of Missouri-Kansas City: Deokjai Choi, Taesang Choi, Frank Kalich, Oh Young Kang and Hai Feng Weng.

Document Status

•	Document History

version 1.0 (July, 1995)

This version was presented in the NMF SMART TT meeting in Ottawa.  It includes the P (Protocol independence) interface specification (using Abstract Service Definition Convention), CMIP[8] Ensemble, SNMP Trouble Ticketing MIB and Remote Procedure Call Specification of the P Interface.

version 2.0 (October, 1995)

This version was presented in the NMF SMART TT meeting in London.  It includes the Object Definition Language[7] interface specification of the Customer Trouble Ticketing (CTT) interface types (the ASDC specification technique is no longer used).  It also includes an information modeling using the quasi-GDMO+GRM language[6].

version 3.0 (January, 1996)

This version was presented in the NMF SMART TT meeting in Colorado.  It has minor changes from version 2.0.

version 4.0 (May, 1996)

This version was presented in the NMF SMART TT meeting in Orlando.  It has minor changes from version 3.0.

version 5.0 (July, 1996)

This version was presented in the NMF SMART TT meeting in Heidelberg. It includes an object analysis of CTT using OMT[5].  It keeps the informal information and interface specifications of version 4.0, but removes the formal specifications.

version 6.0 (February, 1997)

This version is a reformatted version of version 5.0 in accordance with NMF Solution Set Analysis Document Template[4].

version 7.0 (March, 1997)

This version is a revised version of version 6.0 based on the team review.

•	Contacts

The editors of this revision of the document can be contacted as follows:

Adrian Tang/Anna Uy

Computer Science and Telecommunications,

University of Missouri-Kansas City,

4747 Troost, Kansas City,

Missouri 64110, USA

Tel: (816) 235-2357

Fax: (816) 235-5159

e-mail: tang@cstp.umkc.edu, auy@cstp.umkc.edu

Further information on NMF programs and documentation can be obtained by contacting the NMF office:

NMF

1201 Mt. Kemble Avenue

Morristown, NJ  07960, USA

Tel: +1 201 425 1900

Fax: +1 201 425 1515

Web site: http://www.nmf.org

• 	How to obtain a copy

An electronic copy of this document can be obtained by using ftp from the NMF server.  The file is located at:

199.183.141.190:SPECIFICATIONS/SERVICE/INFOAGREEMENT/NMF601.DOC

• 	How to comment on the document

Comments should be sent electronically to the editors.

TABLE OF CONTENTS

Preface

1.0	Introduction to this document	1

1.1	How this document should be used	1

1.2	Document structure	1

1.3	Key terms/definitions used in this document	2

1.4	Identity of the requirements definitions document	3

2.0	Summary of Requirements	3

2.1	Introduction	3

2.2	Current environment	3

2.3	Summary of business requirements	3

2.4	Summary of technical constraints	4

2.5	System Overview	4

3.0	Modeling Approach	8

3.1	Foundation	8

3.2	Customization of conventional approaches	8

4.0	Analysis Model	9

4.1	Object Model	9

4.1.1  Object Class Descriptions	10

4.1.2  Data Dictionary	12

4.1.3  Object Class Relationships	26

4.1.4  Exported Interfaces	26

4.2	Dynamic Model	27

4.2.1	 Scenario Descriptions and Interaction Diagrams	27

4.2.2	Collaboration Diagrams	31

4.2.3  State Transition Diagrams	35

4.3	Functional Model	36

4.3.1  Operations and Exported Interfaces	37

4.3.1.1  Customer_TTR Interface Type	37

4.3.1.2  TTR_Notification Interface Type	42

4.3.1.3  PTR_Notification Interface Type	44

5.0	Design Considerations	46

6.0	Requirements Satisfaction Analysis	46

7.0    Conclusions		55

Appendix A	Glossary of Terms and Acronyms 	56

Appendix B	References	57

Appendix C	Relevant Standards	58

TABLE OF FIGURES

FIGURE 1	CTT Domain	5

FIGURE 2	Usage of the CTT Interface	5

FIGURE 3	CTT Object Model	10

FIGURE 4	TR and TTR Object Classes	11

FIGURE 5	Customer-TTR Interface Type	26

FIGURE 6	TTR-Notification Interface Type	27

FIGURE 7	PTR-Notification Interface Type	27

FIGURE 8	An Event Trace Diagram for TTR	30

FIGURE 9	An Event Trace Diagram for PTR	31

FIGURE 10	Collaboration Diagram for TTR Class:  Case 1	32

FIGURE 11	Collaboration Diagram for TTR Class:  Case 2	32

FIGURE 12	Collaboration Diagram for TTR Class:  Case 3a	32

FIGURE 13	Collaboration Diagram for TTR Class:  Case 3b	33

FIGURE 14	Collaboration Diagram for TTR Class:  Case 4	33

FIGURE 15	Collaboration Diagram for TTR Class:  Case 5	33

FIGURE 16	Collaboration Diagram for TTR Class:  Case 6	33

FIGURE 17	Collaboration Diagram for TTR Class:  Case 7	34

FIGURE 18	Collaboration Diagram for TTR Class:  Case 8	34

FIGURE 19	Collaboration Diagram for PTR Class:  Case 1	34

FIGURE 20	Collaboration Diagram for PTR Class:  Case 2	34

FIGURE 21	Collaboration Diagram for PTR Class:  Case 3	35

FIGURE 22	Collaboration Diagram for PTR Class:  Case 4	35

FIGURE 23	Collaboration Diagram for PTR Class:  Case 5	35

FIGURE 24	State Diagram for the TTR Class	36

# Preface

NMF is an international consortium of communications service providers and their suppliers, dedicated to promoting industry-wide agreements on how management information should be exchanged between a service provider and its customers, its suppliers and/or other service providers or network operators.  Such agreements are essential to achieve a seamless, cost-efficient operation where multiple companies are involved in the fulfillment of service requests, the management of services while in use by customers, the correction of problems that might impair service quality, and the accurate bulling of the customer for services used.

These agreements are becoming an increasing necessity as regulatory forces change the landscape for service providers.  The addition of new value-add service providers such as Internet Service Providers and the development of global alliances mean that the service delivery chain is being extended beyond the traditional, monopoly environment where a single company had end-to-end control over the provision and management of the customer services.  At the same time, some companies are choosing to focus their business on network infrastructure, either providing high-capacity intercity links to service providers, or providing local access to customer sites.  Finally, as has been the case for many years, service providers and network operators rely on equipment suppliers’ element management systems to make it possible to manipulate the network to create customer services, and to report meaningful information about the status  of elements, in order that the network, and ultimately, customer services, can be maintained at required levels of quality.

NMF members work together to determine how their processes should be linked, and how they should work with customers’ and suppliers’ systems.  They work together because they believe that to solve this type of problem alone requires extensive time and money (which is better spent developing new offerings) and that proprietary solutions will not be accepted in the marketplace.  They work together because they see that in these areas, there is no competitive advantage to be gained over exchanging data in a proprietary fashion, and that, conversely, by working together they can considerably reduce both the time to development and the risk of making bad decisions.

This analysis pertains to a very specific area in which NMF members have determined that a common, industry-wide approach is warranted.  They invite others to review these agreements, and to adopt the information model reflected in this document to meet these requirements.  They invite suppliers to become familiar with these requirements, and to work with them on the details of how the requirements will be met through the application  of standards and other industry-accepted specifications and technology.

# 1.0        Introduction to this document

This document defines the functionality and information for the exchange of management information to meet the requirements defined in the Customer-SP Trouble Administration Requirements Document.  While it contains sufficient information to be a ‘stand alone’ document, it is better read together with Customer-SP Trouble Administration Requirements Documents to gain a full understanding of the information it contains as an analysis of a solution to the  set of requirements.

## 1.1   How this documents will be used

It is expected that this document will be used in two ways:

1.	System design managers will be able to determine, using this document, whether the analysis modeling performed by the NMF is a substantial contribution that can be used in the development of an implementation of an interface that meets the requirements.

2.	For the companies who worked together to perform the analysis (and others as become interested in implementing solutions that use the results) this document will provide sufficient information to develop a detailed technology specific solution that meets the requirements.

## 1.2   Document Structure

This document is structured in seven major sections.  Sections 2 and 3 are considered informative and provide information to allow the reader to understand the models produced in Section 4.  The following reflects what you will find in each section:

Section 1  	Introduces the document

Section 2	Summarizes the business objectives that are driving the need for this interface, and the high level operational requirements that must be met.  It provides an overview of the system that will support the interface in question and other interfaces required to achieve automation of service management or network management functions.

Section 3  	Informs the reader about the techniques used by the analysis team by presenting (mainly through references) the basic modeling language used and the restrictions on the use of the language adopted by the team.

Section 4  	Describes the information to be represented by the system and highlights the information flowing across the interface.  It also describes the behavior of the system as seen across the interface.  Multiple diagrams are used to represent different perspectives.

Section 5 	Contains advice from the analysis team to those making the detailed protocol dependent technical specifications.

Section 6  	Analyzes the requirements and shows how they have been satisfied or otherwise by the analysis results.  It should draw attention to the steps taken within the models to cater for optionality expressed within the requirements.

Section 7  	Offers  conclusions and insights from the team members who prepared the analysis document, including any issues they perceive, or their expectations  of companies to implement the models.

Appendix A	Glossary of terms and acronyms, using precise definitions where they have been agreed upon by NMF.

Appendix B	Detailed information (date, document number, source) regarding any document referenced in the analysis and design Document.

## 1.3   Key terms used in this document

Contact - a person who can provide additional information about the trouble on behalf of either the customer or service provider depending on which domain the person belongs.

Customer - an organization that contracts with a service provider for service(s).

Customer Trouble Ticketing (CTT) domain - the domain within which electronic trouble ticketing between the customer and the service provider takes place.

CTT-Customer  - one of the systems within the CTT domain which is used by the customer to communicate electronically with the service provider using the CTT interface.

CTT Interface - defines the set of interface types which are used by the CTT-Customer and CTT-Service Provider systems for trouble administration.

CTT-Service Provider - one of the systems within the CTT domain which is used by the service provider to communicate electronically with the Customer using the CTT interface.

Provider Trouble Report (PTR) - represents a trouble report which is created by the service provider to inform the customer of planned maintenance on a telecommunications service.

Service - is a telecommunications capability that the customer buys or leases from the service provider.

Service Level Agreement (SLA) - set of contractual agreements between a customer and a service provider.

Service Provider - an organization that provides telecommunications services.

Telecommunications Trouble Report (TTR) - represents a trouble report which is created by the service provider on a telecommunications service trouble which is reported by either the customer or the service provider.

Trouble - any cause that may lead to or contribute to an end-user perceiving a failure or degradation on the quality of service of a telecommunications service.

Trouble Administration - a set of functions that enable troubles to be reported and their status tracked.

Trouble Tracking - the ability to follow the progress of a trouble report from its creation through its closure.

Trouble Reporting - the act of communicating that a trouble has been detected so that it could be resolved.

## 1.4    Identity of the requirements definition document

This document is bound to Customer to Service Provide Trouble Administration Interface: Requirement Specification[3] which is the requirements definition document for CTT.  The said document can be obtained by using ftp from the NMF server.  The file is located at:

199.183.141.190:SPECIFICATIONS/SERVICE/INFOAGREEMENT/NMF601.DOC

It can also be downloaded from the NMF web site at:

http://www.nmf.org

# 2.0       Summary of Requirements

This section summarizes the requirements on the CTT interface defined in the requirements definition document.  Readers needing to have a full appreciation of the requirements should go to the original source document.   It is included to allow the reader to avoid obtaining the original source until he/she is sure that the analysis presented here is relevant to their interests.

## 2.1   Introduction to the requirements

The requirements definition document[3] contains the business and technical requirements for Customer-to-Service Provider trouble administration.  These requirements are specified in a manner that is independent of the protocol used for communication between the customer and the service provider.  A summary of the business and technical requirements is given in sections 2.3 and 2.4.

## 2.2   Current environment

In today's business environment, the availability and quality of communications services are of increasing importance as businesses automate and rely heavily on computer-based applications.  The quest for improvement in availability and the quality of communications services is now focusing on the rapid, accurate and reliable exchange of trouble administration information between the customer and its service provider.  Today, trouble administration interfaces between the customer and the service provider tend to be manual.  However, a number of more sophisticated customers are demanding visibility of, or an interface with, the service provider's internal trouble administration system.  This is typically to enable customers to support their own internal Help Desk type functions, and to provide a check of the service provider's performance against SLAs.  The provision of a standard method of exchanging trouble administration information is seen as an immediate requirement by some customers and at least a path for future migration by many others.

## 2.3   Summary of Business Requirements

The main business level requirements within the scope of this document are the following:

•	Customers need to receive prompt and relevant notification of problems affecting telecommunications service, including scheduled maintenance from service providers.

•	Customers need to report problems requiring remedial action to their service providers.

•	Customers need to track progress of trouble resolution.

•	Customers and service providers both need to exchange trouble management information to authorize or expedite the repair activity, and to confirm trouble clearance.

## 2.4    Summary of technical constraints

The following technical requirements explain how the business level requirements are expected to be met in the following areas:

•	Transport:

The interface should be defined in a network transport protocol independent manner.

•	Connection:

The connection between the customer and the service provider should permit both permanent and transient connections.  It should provide for reliable transport of data.  Connections between one customer and multiple service providers  as well as one service provider with multiple customers should be allowed.

•	Security:

Security services like identification/authentication and integrity are mandatory.  Optional security services include access control, data confidentiality and non-repudiation.

•	Other general requirements:

All time values exchanged across the interface should follow a format that considers time-zone differences.

A format must be determined in creating a trouble report.  This format specifies which attributes must be present and which are optional.

No information should be lost in case the customer becomes unreachable.  Normal processing should be resumed as soon as the interface becomes operational again.

## 2.5   System Overview

The system overview describes the scope of the system modeled, the system boundary, and elements outside the system that interact with it.

The CTT domain (Figure 1) has two systems:  CTT-Cust and CTT-SP systems which are defined in section 1.3.  The CTT-Cust and CTT-SP systems interact with each other using the CTT interface which is the subject of this document.  An end-user of the CTT-Cust system is typically a person within the customer who is concerned with the resolution of a reported trouble.  An end-user of the CTT-SP system is typically a technician within the SP whose job is to clear the trouble.

This document defines three interface types for the CTT interface.  Each interface type is characterized by a set of interface operations which are exported to an external system which is either the CTT-Cust system or the CTT-SP system within the CTT domain.

The Customer-TTR interface type consists of interface operations which can be invoked by the CTT-Cust system for the purpose of entering a telecommunications trouble report and tracking the status of trouble resolution.  These operations are provided by the CTT-SP system.

The TTR-notification interface type consists of operations which can be invoked by the service provider to inform the customer of events (e.g., status change, attribute value change, etc.) related to telecommunications trouble reports.  These operations are provided by the CTT-Cust system.

The PTR-notification interface type consists of interface operations which can be invoked by the service provider to inform the customer of events (e.g., status change, attribute value change, etc.) related to provider trouble reports, i.e., reports created to schedule maintenance on a service.  These operations are provided by the CTT-Cust system.

During the resolution of the trouble, the CTT-SP system may need to interface internally with other service/network management processes in the Service Management Business Process Model[2] and externally with systems at other service providers.  How the CTT-SP system interfaces with these internal processes or external systems is outside the scope of this document.  As far as the CTT-Cust system is concerned, the CTT-SP system is the only system which it would interface for the purpose of trouble administration.

Figure 1:  CTT Domain

## 2.5.1   How the interface is expected to be used

The two sides of the CTT interface are the CTT-Cust system (shown as Customer Trouble Administration in Figure 2) and the CTT-SP system  (shown as Outsourcing Organization Trouble Administration in Figure 2).  The CTT interface is used to allow end-users of the CTT-Cust system and the CTT-SP system to report troubles, to schedule service maintenance, to enable the service provider to keep the customer informed of trouble resolution, and to facilitate the cooperation of service providers acting as suppliers or maintenance organizations to the outsourcing company.

Figure 2:  Usage of the CTT Interface

Use Case:	Customer delegates to one provider - no feedback

Use Case Identifier:	CTTUC1

Summary:	This case covers the situation where a customer identifies a problem and determines that it is not related to services or equipment within their domain of control and further knows which service provider is responsible for the repair of the problem.  The customer will delegate from their internal trouble ticketing system to the service provider, and has no desire for information flow back into their tracking system.  This case may be common with larger customers when the problem is well understood, and would certainly be common with smaller customers that do not have extensive trouble tracking requirements.

Preconditions:	A trouble was detected and the trouble object as well as the service provider responsible for it is determined.

Postconditions:	A trouble report is created but no feedback is sent to the customer.

Use Case:	Customer delegates to one provider and requires feedback

Use Case Identifier:	CTTUC2

Summary:	In this case the customer identifies a problem and determines that it may involve a service provider’s service, but continues to monitor the progress on the ticket.  The customer will delegate a trouble ticket to the specific service provider that they think is responsible for the problem, but demand status back to their trouble tracking system.  The requirements for feedback are varied.  In some cases the customer may only be interested in an update when the ticket is closed, or may be interested in intermediate status.  Also since the customer is unsure whether or not the problem is with services of the service provider, the customer may want to be able to update information on the trouble ticket in their tracking system as well as the delegated trouble ticket to insure that all information regarding the trouble is disseminated.

Preconditions:	A trouble was detected and the trouble object as well as the service provider responsible for it is determined.

Exceptions:	As soon as the customer detects that the reported problem is internal, it sends a request to the suspected service provider to have the trouble report canceled.  Or as soon as the suspected service provider has detected that it is their problem, it will inform the customer who then proceeds to stop all the related trouble resolution procedures internally.

Postconditions:	The customer finds out that the reported problem is either an internal problem, a problem with the service of the suspected service provider, or a problem with another service provider to be determined.

Use Case:	Delegation to multiple providers

Use Case Identifier:	CTTUC3

Summary:	When a customer has problems with service and is unsure of the source, tickets may be delegated to multiple service providers simultaneously.  This will typically occur when both local carriers and long distance carriers are involved in providing service.  As in scenario 2 the feedback requirements to the customer are varied, and trouble ticket updates are expected to be bi-directional between customer and each service provider.  This scenario is expected to be the most prevalent scenario with large customers, as voice and data requirements become more complex and specialized support expertise within companies is reduced.

Preconditions:	A trouble was detected but the customer is not sure which service provider is responsible.

Exceptions:	As soon as the customer detects that the reported problem is internal, it sends a request to all the service providers to have their trouble reports canceled.  Or as soon as one of the service providers has detected that it is their problem, it will inform the customer who then proceeds to stop all the related trouble resolution procedures internally and to inform other service providers to cancel their trouble reports.

Postconditions:	When the reported problem has been fixed, the customer finds out that the reported problem is either an internal problem or a problem with one of the service providers.

Use Case:	Delegation between service providers

Use Case Identifier:	CCTUC4

Summary:	This scenario is essentially the same as the scenarios above, with the service provider (taking the role of customer) delegating in turn to another provider.  However the problems are undoubtedly better qualified when they are delegated from one provider to another.  Service providers will typically have high expertise in localizing the problem prior to delegation, reducing the number of times that tickets are delegated to multiple providers simultaneously.  The feedback requirements and need for simultaneous updates to delegated tickets remains the same. Note, this scenario is included for completeness’ sake, but is out of scope of the requirements definition.

Preconditions:	A trouble was detected but customer is not sure which service provider is responsible.

Exceptions:	If the service provider finds out that it is not their problem, it will delegate to another service provider and await for a response.  Depending on its SLA with the customer, it may inform the customer of the delegation.  If the second service provider finds out that it is their problem, it will try to resolve the problem and keep the first service provider informed of the status of trouble resolution.  If the second service provider finds out that it is not their problem, it may either inform the first service provider or take the role to continue the delegation, depending on the joint implementation agreements between the two service providers.

Postconditions:	When the reported problem has been fixed, the customer finds out that the reported problem is either an internal problem or a problem with one of the service providers who are involved in the delegation chain.

Use Case:	Provider informs Customer - end user originated

Use Case Identifier:	CTTUC5

Summary:	Sometimes the end-users of a network report problems directly to the service provider via a non-standard interface, e.g. the telephone, but thereafter the provider continues to report to the customer trouble administration center as for CTTUC2.  Electronic Bonding trouble administration refers to this as “late bonding”. This may be the preferred method of working between the customer and service provider (especially if it reduces service down-time), or indeed the end-users of the network may not be part of the customer organization which pays for it.

Preconditions:	same as CTTUC2

Exceptions:	same as CTTUC2

Postconditions:  same as CTTUC2

Use Case:	Provider informs Customer - provider originated

Use Case Identifier:	CCTUC6

Summary:	The provider may detect service affecting faults before the customer reports them.  These are notified by the provider to the customer.  Thereafter they may be tracked in the normal way as for CCTUC2.

Preconditions:	The service provider has detected an internal problem which has negative impact on the service provided to a customer.  It creates a trouble report and notifies the customer of the creation.

Postconditions:	The service provider has fixed the problem and informs the customer that the trouble report is cleared.

Use Case:	Provider informs Customer - scheduled down-time

Use Case Identifier:	CCTUC7

Summary:	The trouble reporting mechanism may be used by the service provider to notify the customer of scheduled down-time.  The customer may track the “trouble” as normal, although the customer does not control the trouble management process in the ordinary way, but merely observes passively.

Preconditions:	The service provider wants to schedule planned maintenance.  It creates a provider trouble ticket and informs the customer of the creation.

Exceptions:	During the service maintenance, a trouble is detected which may require the creation of a trouble report; this trouble ticket has to be closed first before the provider trouble report can be closed.

Postconditions:	The service provider has finished the service maintenance and informs the customer that the provider trouble ticket is closed.

# 3.0	    Modeling Approach

This section has two parts.  The first informs the reader about the foundations of the modeling approach adopted and the second describes variations from the conventional approach to modeling selected/defined by the modeling team.

## 3.1   Foundation

The modeling approach chosen for this analysis document is mainly OMT.  However, in representing interface objects and collaboration diagrams, the UML methodology was used.

## 3.2   Customization of conventional approaches

Since UML (version 1.0) does not provide a template for the definition of the syntax and semantics of interface operations, the team devised the following template:

In defining the syntax of  an interface operation, the following items are specified:

•	Input parameters

•	Output parameters

•	Exceptions

In defining the semantics of an interface operation, the states of the relevant information objects (see Appendix A for definition of terms) are noted before and after the execution of the operation.

# 4.0       Analysis Model

This section contains the results of the analysis in the form of an Object Model, a Dynamic Model and a Functional Model.

The Object model identifies and describes the ‘things’ of concern to the users of the interface.  Such things are defined as objects.  Relevant relationships between objects are also defined.

The dynamic model describes the most important interactions between the objects themselves and between the objects and external systems.

The Functional Model defines object signatures and operations particularly relevant to the interfaces.

## 4.1   Object Model

The Customer Trouble Ticketing (CTT) Object Model is the result of object modeling of the CTT domain which has defined object classes, object class relationships and an associated data dictionary.  This subsection also highlights the interfaces the system exports.

## 4.1.1   Object Class Descriptions

Figure 3:  CTT Object Model

The diagram in Figure 3 shows the CTT object classes and their relationships.  See Figure 4 for details of TR and TTR object classes.

Figure 4:  TR and TTR Object Classes

## 4.1.2   Data Dictionary

The concepts and terms within the class diagram are defined as follows.  Note that detailed definitions of the enumerated-type attributes can be found in ITU-X.790[9].

Object Class - CTT-Cust

The CTT-Cust object class is an abstraction of a customer's trouble administration system that the customer uses to communicate with the SP within the CTT domain.  This is a computation object class.

Attributes:

Name:  This identifies an instance of the CTT-Cust object class.

Operations:

The operations here are exported as interface operations which can be invoked by a SP's trouble administration system.  They are related to either TTRs or PTRs.

TTRCancellationNotify:  The SP invokes this operation to inform the customer of the cancellation of trouble resolution on a TTR.

TTRCloseOutNotify: The SP invokes this operation to inform the customer of the closure of a TTR.

TTRCreationNotify:  When the SP has detected a service failure, it initiates to create a TTR and invokes this operation to inform the customer of the creation.

TTRDeletionNotify:  The SP invokes this operation to inform the customer of the deletion of a closed TTR.

TTRModificationNotify:  After the SP has initiated to modify one or more TTR attributes, it invokes this operation to inform the customer of the modification.

TTRStatusUpdate:  The SP invokes this operation to inform the customer of the status change of a TTR.

PTRCancellationNotify:  The SP invokes this operation to inform the customer of the cancellation of trouble resolution on a PTR.

PTRCreationNotify:  After the SP has created a PTR, it invokes this operation to inform the customer of the creation.

PTRDeletionNotify:  The SP invokes this operation to inform the customer of the deletion of a closed PTR.

PTRModificationNotify:  After the SP has modified one or more PTR attributes, it invokes this operation to inform the customer of the modification.

PTRStatusUpdate:  The SP invokes this operation to inform the customer of the status change of a PTR.

Object Class -  CTT-SP

The CTT-SP object class is an abstraction of a SP's trouble administration system that the SP uses to communicate with the customer within the CTT domain.  This is a computation object class.

Attributes:

Name:  This identifies an instance of the CTT-SP object class.

Operations:

The operations here are exported as interface operations which can be invoked by a customer's trouble administration system.

TTRCancel:  The customer invokes this operation to request cancellation of trouble resolution on a TTR.

TTRCreate:  The customer invokes this operation to enter a TTR.

TTRStatusTrack:  The customer invokes this operation to track the status of a TTR.

TTRVerify:  The customer invokes this operation to convey its disposition of the verification on a TTR.

TTRView:  The customer invokes this operation to view one or more attributes of a TTR.

TTRModify:  The customer invokes this operation to request modification of one or more attributes of a TTR.

Object Class - Account

The Account object class is an abstraction of information on a customer held by a SP's trouble administration system within the CTT domain.  An Account object may reference the billing account for the customer.  This information object class ensures that each and every TR created is related to an existing service, and belong to a single customer.

Attributes:

AccountId:  This uniquely identifies an instance of the Account object class.

Operations:

AccountCreate:  This is used to create an Account object for the purpose of trouble administration.

AccountDelete:  This is used to delete an Account object.

Object Class - SLA

The SLA (Service Level Agreement) object class is an abstraction of a contract between the customer and the service provider.  This object represents the TR formatting rules and policies that govern  trouble administration like the TR escalation, cancellation, closure etc.  This information object class is bound to a customer's account within the CTT domain.

Attributes:

SLAId:  This uniquely identifies an instance of the SLA class.

Operations:

SLACreate:  This is used to create an SLA object to which trouble administration is applicable.

SLADelete:  This is used to delete an SLA object.

Object Class - Service

The Service object class is an abstraction of a telecommunications service which is bound to a customer's account within the CTT domain.  This information object class ensures that the TR created is related to an existing service offered by the SP.

Attributes:

ServiceId:  This uniquely identifies an instance of the Service object class.

Operations:

ServiceCreate:  This is used to create a Service object to which trouble administration is applicable.

ServiceDelete:  This is used to delete a Service object.

Object Class - Trouble Report

The Trouble Report (TR) is a superclass of the Telecommunications Trouble Report (TTR) and Provider Trouble Report (PTR) object classes.  It holds information common to the TTR and PTR object classes.  This is an information object class.

Attributes:

ActivityDuration:  This indicates the time spent on billable and non-billable repair activities.  It is represented by a set of activity duration sequences which specify the time spent on an activity type and a flag that indicates whether the activity is billable or not.  The computation of the time would use a standardized algorithm or an algorithm which is mutually agreed upon by the customer and the SP.

The following is a list of possible activity types:

after-hours repair

standby

after-hours standby

test

customer-initiated test

dispatch

no-access

delayed-maintenance

release

AdditionalTroubleInfoList:  The customer uses this to further describe the reported trouble.  It can be represented by a set of graphic strings.

CloseOutNarr:  This provides a place for the person who resolved the problem to document additional information regarding the trouble report closure.

ReceivedTime:  This indicates when the TR was created.

RelatedTRList:   This is a list of related TRs.

RepairActivityList: This contains parameters and text describing the specific repair functions performed, who performed them, and when they were performed.  This can be represented by a list of repair activity sequences where each repair activity sequence provides the following information for a single repair activity:

-	entry time

-	activity information

-	activity person

-	activity code which identifies the activity performed such as approved, assign, cancel, clear, close, defer, dispatch, refer, release, re-open, repair, test, or transfer.

RestoredTime:  This indicates when the trouble was cleared.

SPRoleAssignment:  This attribute assigns a person to each SP role established for CTT.  These persons can be contacted by the customer during trouble resolution.  Examples of SP roles are primary contact person and alternate contact person.  The attribute can be represented by a set of role assignments.

TRDescription:  This gives a summary of the TR.

TRID:  This is the identifier assigned to the TR at creation time.

TroubleClearancePerson:  This identifies a person in the customer's organization who either verified repair completion or requested to cancel the TR.

TroubleFound:  This is used to identify the problem resolved. Values for this attribute are defined in ITU-X.790[9].

TroubleLocation:  This indicates where the trouble is.  This information may not be known at the time when the TR was created.

TRNumList:  This represents a list of internal TR alias identifiers presently being used to identify the TR within each of the SP's trouble administration system working on the reported trouble.  These TR alias identifiers are needed by the customer when trouble resolution requires telephone conversations between the customer and a person working at a particular SP's trouble administration system.

TroubleObject:  This identifies the service against which the TR is created.

TroubleType:   	This identifies the category of trouble that is being reported on the trouble object.  Values for this attribute are defined in ITU-X.790[9].

TRState:  This indicates the current state of the TR (Section 4.2.3).  A TR may be in one of the following states:

-	queued:  when the TR has been created but the trouble resolution process has not been initiated,

-	open/active:  when appropriate actions to resolve the trouble are being carried,

-	deferred:  when the corrective action on the TR has been postponed.  A deferred report can become open/active when specific conditions are met,

-	cleared:  when the trouble has been corrected.  If the customer needs to verify that the trouble has been resolved, verification may optionally be awaited by the SP prior to closure of the TR,

-	closed:  when the trouble has been corrected, and

-	disabled:  when the TR information cannot be updated due to local conditions.  In this state, only read operations can be performed on the TR.

TRStatus:  This indicates the current status of trouble resolution on the TR.  The following shows the status values which are allowed in a given state.

open/active:

-	screening,

-	testing,

-	dispatchedIn,

-	preassignedOut,

-	bulk DispatchedOut,

-	startRepair,

-	pendingTest,

-	pendingDispatch,

-	requestedRepair,

-	referMtceCenter,

-	referVendor,

-	stopNoAccess,

-	stopDelayMtce,

-	troubleEscalated,

-	craftDispatched,

-	cableFailure,

-	originatingEquipFailure,

-	backOrder.

deferred:

-	noAccessOther,

-	startNoAccess,

-	stateDelayedMtce.

cleared:

-	temporaryOK,

-	clearedCustNotAdvised,

-	clearedCustAdvised,

-	clearedAwaitingCustVer.

closed:

-	closedOut,

-	closedOutByCustReq,

-	closedOutCustVerified,

-	closedOutCustDenied,

-	canceledPendingWorkInProgress,

-	canceledPendingTestCompletion,

-	canceledPendingDispatchCompletion.

TRStatusTime:  This indicates when the status was last changed or validated.

Operations:

/* the following operations are used to create or delete a TR */

TRCreate:  This is used to create a TR object.  The TR is created in either the queued or open/active state.  The following TR attributes are mandatory at the creation time:

-	AdditionalTroubleInfoList,

-	ReceivedTime,

-	TRID,

-	TroubleFound,

-	TroubleObject,

-	TroubleType,

-	TRState,

-	TRSatatus and

-	TRStatusTime.

TRDelete:  This is used to delete a TR object.  The operation can be executed only when the TR is closed.

/* the following operations are related to TR attributes */

ActivityDurationGet:  This is used to retrieve the sequences in ActivityDuration.

ActivityDurationAdd:  This is used to add an activity duration sequence.

AdditionalTroubleInfoListGet:  This is used to find out the information in AdditionalTroubleInfoList which describes the reported trouble.

AdditionalTroubleInfoListAdd:  This is used to add additional trouble information to the TR.

CloseOutNarrGet:  This is used to retrieve the close-out narrative when the TR is either cleared or closed.

CloseOutNarrReplace:  This is used to modify the close-out narrative.  This operation can be executed only when the TR is either cleared or closed.

ReceivedTimeGet:  This is used to find out when the TR was created.

RelatedTRListGet:  This is used to retrieve the related TRs.

RelatedTRListReplace:  This is used to replace RelatedTRList.

RelatedTRListAdd:  This is used to add a TR to RelatedTRList.

RelatedTRListRemove:  This is used to delete a TR from RelatedTRList.

RepairActivityListGet:  This is used to retrieve one or more repair activity sequences from RepairActivityList.

RepairActivityListReplace:  This is used to replace RepairActivityList.

RepairActivityListAdd:  This is used to add a repair activity sequence to RepairActivityList, e.g., after a repair has been performed.

RepairActivityListRemove:  This is used to remove a repair activity sequence from RepairActivityList.

RestoredTimeGet:  This is used to find out when the trouble was cleared.

RestoredTimeReplace:  This is used to record when the trouble was cleared.  This operation can be invoked only when the TR is either cleared or closed.

SPRoleAssignmentGet:  This is used to retrieve role-assignments from SPRoleAssignment.

SPRoleAssignmentReplace:  This is used to replace SPRoleAssignment.

SPRoleAssignmentAdd:  This is used to add a role-assignment to SPRoleAssignment.

SPRoleAssignmentRemove:  This is used to remove a role-assignment from SPRoleAssignment.

TRDescriptionGet:  This is used to retrieve a summary of the TR.

TRNumListGet:  This is used to get the items in the TRNumList.

TRNumListReplace:  This is used to replace the TRNumList.

TRNumListAdd:  This is used to add an internal TR alias identifier to TRNumList.

TRNumListRemove:  This is used to remove an internal TR alias identifier from TRNumList.

TroubleClearancePersonGet:  This is used to find out the identity of the person who either verified repair completion or requested to cancel the TR.

TroubleClearancePersonReplace:  This is used to enter the identity of the person who either verified repair completion or requested to cancel the TR.  This operation can be executed only when the TR is not closed.

TroubleFoundGet:  This is used to find out the problem which was found during trouble resolution.

TroubleFoundReplace:  This is used to enter the problem which was found during trouble resolution.  This operation can be executed only when the TR is either cleared or closed.

TroubleLocationGet:  This is used to find out the location of the trouble.

TroubleLocationReplace:  This is used to enter the location of the trouble after the location has been identified.

TroubleObjectGet:  This is used to find out the trouble object based on which this TR was created.

TroubleTypeGet:  This is used to find out the category of trouble that is being reported on the trouble object.

TRStateGet:  This is used to find out the current state of the TR.

TRStateReplace:  This is used to modify the state of the TR.  The state modification should obey the invariants described in the TR state diagram (Section 4.2).  This operation has a side-effect on the TRStatus and TRStatusTime attributes.

TRStatusGet:  This is used to find out the current status of trouble resolution.

TRStatusReplace:  This is used to modify the status of trouble resolution.  The new status is constrained by the current state.  This operation has a side-effect on the TRStatusTime attribute.

TRStatusTimeGet:  This is used to find out when the status was last changed or validated.

TRStatusTimeReplace:  This is used to enter the time when the status was changed or validated.

/*	the following operations are notifications which can be emitted by a TR */

TRAVCNotification:  This is used to emit notification when at least one TR attribute has been modified.  It returns a list of TR attribute(s) which were modified.

TRCreateNotification:  This is used to emit notification when a TR has been created.  It returns a list of attributes of the created TR.

TRDeleteNotification:  This is used to emit notification when a TR has been deleted.  It returns the identity of the deleted TR.

Object Class - Telecommunications Trouble Report

The Telecommunications trouble Report (TTR) object class is a subclass of TR.  It represents reported trouble(s) on telecommunications services within the CTT domain.  TTR is an information object class.

Attributes:

Note:  In addition to the attributes listed below, the TTR object class also includes the attributes inherited from TR.

AfterHrsRepairAuthority:  This indicates whether permission has been given by the customer to repair the service outside normal business hours (e.g., 9am-5pm Monday through Friday).  This attribute and the authorizationList attribute are supplied mutually exclusive of each other at the creation time by the customer.

AuthorizationList:  This attribute is used to specify the level of authorization requested by the customer or granted by the SP.  It can be represented by a set of authorization sequences.  Each authorization sequence has the following components:

-	request state indicates whether the authorization is requested or granted.

-	activity type which includes all the activities either authorized or requiring authorization depending on the type of authorization.  A list of activity types can be found in ITU-X.790[9].

-	authorization time which indicates the time from when authorization takes place, and

-	authorization person which indicates the person who grants the authorization.

The last two items, authorization time and person, are optional and are filled only if the request state indicates grant authorization.

CancelRequestedByCustomer:  This indicates whether the customer has requested to cancel the TTR.  Cancellation request is disallowed if the TTR is already cleared or closed.  The TTR closes when a cancellation request is granted.

CloseOutVerification:  	This indicates whether the customer has verified repair completion, denied repair completion, or taken no action.  This attribute can be initialized with the "noAction" value.  The customer can verify only after TRStatus has been changed to "clearedAwaitingCustVerification", i.e., the SP is awaiting customer’s verification.

CommitmentTime:  This indicates either the on-site or trouble cleared time given to the customer.  The on-site time refers to the time which the SP has agreed to go to the customer's organization to fix the problem.

CommitmentTimeRequest:  This indicates either the on-site or trouble cleared time requested by the customer.

CustomerRoleAssignment:  This attribute is used to assign a person to each customer role established for CTT.  These persons can be contacted by the SP's organization during trouble resolution.  The attribute can be represented by a set of role assignments.  Examples of customer roles are:

-	primary contact person,

-	alternate contact person,

-	person to be contacted when a TR has been escalated,

-	person to be called back before any automatic tests are performed on the service, and

-	person to be called back when the trouble is cleared.

Note that there is no role for the person who has requested for cancellation or verified repair completion.  However, we do have the TRClearancePerson attribute to specify this person for this role.

CustomerTRNum:  This specifies the customer's internal number (e.g., one which is assigned by a customer's trouble administration system) of the TTR.  It allows the customer to access the TTR with this internal number.

Dialog:  This enables interaction to take place among the end-users (e.g., a person at the customer and a person at the SP) during trouble resolution.  It is in free format text.

EscalationList:  This is used to indicate whether escalation is requested or granted.  It can be represented by a set of escalation sequences.  An escalation sequence has the following components:

-	escalation state which indicates whether the escalation is requested or granted,

-	escalation time which indicates the time when escalation takes place,

-	request person which indicates the person who requests the escalation,

-	escalation level which indicates the requested escalation level, and

-	escalation person which indicates the person who has granted the escalation.

When escalation is requested, it is optional to supply the request person and escalation level components.  When an escalation is granted, it is optional to supply the escalation time, level and person.  Escalation levels can only increase but not decrease.

InitiatingMode:  The creation of the TTR may be initiated by either the customer or the SP.  This attribute is used to qualify who initiates the creation.  Creation of TR may be initiated through one of the following ways:

-	carrier-originated where the SP detected the problem and locally created the TTR,

-	alarm-originated where the TTR was automatically created by the network/equipment because of the alarm,

-	directly by the customer through the use of the CTT interface, and

-	indirectly by the customer where the trouble was reported by the customer to the SP (via email, fax or phone) and the SP created the TTR in the CTT system.

LastUpdateTime:  This indicates the time of the most recent update made to the TTR.

MaintServiceCharge:  This indicates whether the customer will be charged for repairs performed on the service.

OutageDuration:  This indicates the amount of time between the TTR clearing time and the TTR received time, excluding the times spent for delayed maintenance or the times when the service could not be accessed by the SP for repair.

PerceivedTroubleSeverity:  This allows the customer to indicate the effect of the reported trouble.  Its value may be out of service, back in service, service impairment or non-service affecting trouble.

PreferredPriority:  This defines the urgency level with which the customer requires resolution of the problem.  Its value may be undefined, minor, major or serious.

RepeatReport:  This indicates whether there has been a provisioning/installation or a trouble activity on the trouble object in the recent past, e.g., within the last 30 days.  Its value may be unspecified, recent installation, repeat trouble or both installation and repeat trouble.

SuspectObjectList:  This is a list of suspect objects, where a suspect object may be the underlying cause of the trouble.

TroubleDetectionTime:  This indicates when the trouble was detected.

TroubleLocationInfoList:  This provides information (such as address, accessible hours and location person) on the possible locations of a trouble object (e.g., the A-location and the Z-location of a leased circuit).

TroubleObjectAccessFromTime:  This identifies the beginning of the time frame during which the service personnel can have access to the trouble object.  Access restrictions within this time frame can be specified by the TroubleObjectAccessHours attribute.

TroubleObjectAccessHours:  This defines the specific hours for each day of the week during which access to the trouble object is available.  It further delimits the time frame by the TroubleObjectAccessFromTime and TroubleObjectAccessToTime attributes by specifying the access availability intervals for each day of the week.

TroubleObjectAccessToTime:  This identifies the end of the time frame during which the service personnel can have access to the trouble object.  Access restrictions within this time frame can be specified by the TroubleObjectAccessHours attribute.  The continual choice is made if the end of the time frame is not specified.

Operations:

Note:  In addition to the operations listed below, the TTR object also includes all the operations inherited from TR.

/* the following operations are related to TTR attributes */

AfterHoursRepairAuthorityGet:  This is used to find out whether permission has been given to repair the service outside normal business hours.  The usage of this operation assumes that the AuthorizationList attribute is not supplied at the creation time of the TTR.

AfterHoursRepairAuthorityReplace:  This is used to modify AfterHoursRepairAuthority.  This operation can only be executed when the TTR is neither cleared nor closed.  The usage of this operation assumes that the AuthorizationList attribute is not supplied at the creation time of the TTR.

AuthorizationListGet:  This is used to retrieve authorization sequences from AuthorizationList.

AuthorizationListReplace:  This is used to modify AuthorizationList.  This operation can only be executed when the TTR is neither cleared nor closed.

AuthorizationListAdd:  This is used to add an authorization sequence to AuthorizationList.  This operation can only be executed when the TTR is neither cleared nor closed.

CancelRequestedByCustomerGet:  This is used to find out whether the customer has requested to cancel the trouble resolution.

CancelRequestedByCustomerReplace:  This is used to modify CancelRequestedByCustomer.  This operation can only be executed when the TTR is neither cleared nor closed.

CloseOutVerificationGet:  This is used to find out the customer's disposition on the verification of the trouble resolution.

CloseOutVerificationReplace:  This is used to enter the customer's disposition on the verification of the trouble resolution.  This operation can only be executed when the status is "clearedAwaitingCustVerification".

CommmitmentTimeGet:  This is used to retrieve the on-site or trouble cleared time given to the customer.

CommitmentTimeReplace:  This is used to modify the on-site or trouble cleared time given to the customer.  This operation can only be executed when the TR is neither cleared nor closed.

CommitmentTimeRequestGet:  This is used to retrieve the on-site or trouble cleeared time requested by the customer.

CommitmentTimeRequestReplace:  This is used to modify the on-site or trouble cleared time requested by the customer.  This operation can only be executed when the TTR is neither cleared nor closed.

CustomerRoleAssignmentGet:  This is used to retrieve role assignments from CustomerRoleAssignment.

CustomerRoleAssignmentReplace:  This is used to replace CustomerRoleAssignment.

CustomerRoleAssignmentAdd:  This is used to add a role assignment to CustomerRoleAssignment.

CustomerRoleAssignmentRemove:  This is used to remove a role assignment from CustomerRoleAssignment.

CustomerTRNumberGet:  This is used to retrieve the customer's internal number for the TTR.

DialogGet:  This is used to retrieve the Dialog attribute.

DialogReplace:  This is used to replace the Dialogue attribute as interaction takes place among the CTT end-users during the trouble resolution.

EscalationListGet:  This is used to retrieve escalation sequences from EscalationList.

EscalationListAdd:  This is used to add an escalation sequence to EscalationList, when escalation is either requested or granted.  This operation can only be executed when the TTR is neither cleared nor closed.

InitiatingModeGet:  This is used to find out who initiated to enter the TTR.

LastUpdateTimeGet:  This is used to find out the time of the most recent update made to the TTR.

LastUpdateTimeReplace:  This is used to replace the time of the most recent update made to the TTR.  This operation can be executed only when one or more TTR attributes have been modified.

MaintServiceChargeGet:  This is used to find out whether the customer will be charged for the repairs.

MaintServiceChargeReplace:  This is used to modify the MaintServiceCharge attribute when decision has been made whether the customer should be charged for the repairs.

OutageDurationGet:  This is used to find out the amount of time between the TTR clearing time and the TTR received time.

OutageDurationReplace:  This is used to specify the amount of time between the TTR clearing time and the TTR received time.  This operation can be executed when the TTR is either cleared or closed.

PerceivedTroubleSeverityGet:  This is used to retrieve the customer's perceived severity of the TTR.

PerceivedTroubleSeverityReplace:  This is used to modify the customer's perceived severity of the TTR.  This operation can be executed only when the TTR is neither cleared nor closed.

PreferredPriorityGet:  This is used to find out the customer's urgency level in having the problem resolved.

PreferredPriorityReplace:  This is used to modify the customer's urgency in having the problem resolved.  This operation can only be executed when the TTR is neither cleared nor closed.

RepeatReportGet:  This is used to find out whether there has been a provisioning/installation or a trouble activity on the trouble object in the recent past.

RepeatReportReplace:  This is used to update the RepeatReport attribute when a recent provisioning/installation or a trouble activity on the trouble object has occurred.

SuspectObjectListGet:  This is used to retrieve the suspect object(s) which may be the underlying cause of the trouble.

TroubleDetectionTimeGet:  This is used to find out when the trouble was detected.

TroubleDetectionTimeReplace:  This is used to modify the time when the trouble was detected.

TroubleLocationInfoListGet:  This is used to retrieve the possible locations of the trouble object.

TroubleLocationInfoListReplace:  This is used to replace the list of possible locations of the trouble object.

TroubleLocationInfoListAdd:  This is used to add a location to TroubleLocationInfoList.

TroubleLocationInfoListRemove:  This is used to remove a location from TroubleInfoList.

TroubleObjectAccessFromTimeGet:  This is used to find out the beginning of the time frame during which the service personnel can have access to the trouble object.

TroubleObjectAccessFromTimeReplace:  This is used to replace the beginning of the time frame during which the service personnel can have access to the trouble object.  This operation can be executed only when the TTR is neither cleared not closed.

TroubleObjectAccessHoursGet:  This is used to find out the specific hours for each day of the week during which access to the trouble object is available.

TroubleObjectAccessHoursReplace:  This is used to replace the special hours for each day of the week during which access to the trouble object is available.  This operation can be executed only when the TTR is neither cleared nor closed.

TroubleObjectAccessHoursAdd:  This is used to add additional hours to TroubleObjectAccessHours.  This operation can be executed only when the TTR is neither cleared nor closed.

TroubleObjectAccessHoursRemove:  This is used to remove one or more hours from TroubleObjectAccessHours.  This operation can be executed only when the TTR is neither cleared nor closed.

TroubleObjectAccessToTimeGet:  This operation is used to find out the end of the time frame during which the service personnel can have access to the trouble object.

TroubleObjectAccessToTimeReplace:  This operation is used to replace the end of the time frame during which the service personnel can have access to the trouble object.  This operation can be executed only when the TTR is neither cleared nor closed.

/*	the following operations are notifications which can be emitted by a TTR */

TTRCloseOutNotification:  This is used to emit notification when a TTR is closed.  It returns the history information of the closed TTR.  The TTR attributes in the history information may include the following:

-	ActivityDuration,

-	AdditionalTroubleInfoList,

-	AuthorizationList,

-	CancelRequestedByCustomer,

-	CloseOutNarr,

-	CloseOutVerification,

-	CommitmentTime,

-	CustomerTRNum,

-	PerceivedTroubleSeverity,

-	ReceivedTime,

-	RestoredTime,

-	TRDescription,

-	TRID,

-	TRNumList,

-	TroubleClearancePerson,

-	TroubleFound,

-	TroubleObject and

-	TroubleType.

Object Class - Provider trouble Report

The Provider trouble Report (PTR) object class is a subclass of TR.  It represents a report generated by the SP to inform the customer of planned maintenance on a service.  This is an information object class.

Attributes:

Note:  In addition to the attributes listed below, the PTR object class also includes the attributes inherited from TR.

BeginTime:  This is used to indicate the beginning of the time frame for which the service will be unavailable.

EndTime:  This is used to indicate the end of the time frame for which the service will be unavailable.

Operations:

Note:  In addition to the operations listed below, the PTR object also includes all the operations inherited from TR.

BeginTimeGet:  This is used to find out the beginning of the time frame for which the service will be unavailable.

BeginTimeReplace:  This is used to modify the beginning of the time frame for which the service will be unavailable.  This operation can be executed only when the PTR is in the queued state.

EndTimeGet:  This is used to find out the end of the time frame for which the service will be unavailable.

EndTimeReplace:  This is used to modify the end of the time frame for which the service will be unavailable.  This operation can be executed only when PTR is in the queued state.

4.1.3   Object Class Relationships

A CTT-Cust object may be associated with more than one CTT-SP object and vice versa.

A CTT-SP object manages the other CTT information objects (i.e., Service, TTR and PTR) through an Account object .  It may manage more than one Account object.

An Account object may have one or more Service objects associated with it.  Whenever there is a failure of a Service object, a TTR object referencing the Service object is created.  Furthermore, whenever there is planned maintenance on a Service object, a PTR referencing the Service object is created.

Associated with every Account object is one or more SLA objects which contain the policies that regulate the trouble management of the Service object(s), as well as the formatting requirements of the TR created for the Service object(s).

4.1.4    Exported Interfaces

The following interface objects are identified in the CTT domain.

Customer_TTR Interface:  This interface type (Figure 4) consists of interface operations which can be invoked by the CTT-Cust system for the purpose of entering a telecommunications trouble report and tracking the status of trouble resolution.  These operations are provided by the CTT-SP system.  See section 4.3.1.1 for details of these interface operations.

Figure 5: Customer_TTR Interface type

TTR_Notification Interface:  This interface type (Figure 5) consists of operations which can be invoked by the service provider to inform the customer of events (e.g., status change, attribute value change, etc.) related to telecommunications trouble reports.  These operations are provided by the CTT-Cust system.  See section 4.3.1.2 for details of these interface operations.

Figure 6:  TTR_Notification Interface type

PTR_Notification Interface:  This interface type (Figure 6) consists of interface operations which can be invoked by the service provider to inform the customer of events (e.g., status change, attribute value change, etc.) related to provider trouble reports, i.e., reports created for scheduled maintenance on a service.  These operations are provided by the CTT-Cust system.  See section 4.3.1.3 for details of these interface operations.

Figure 7:  PTR_Notification Interface Type

## 4.2   Dynamic Model

The dynamic model shows the time-dependent behavior of the system and the objects in it.

The dynamic model is expressed in scenario descriptions, interaction diagrams and state transition diagrams.  Operations at the exported interfaces complete this sub-section.

4.2.1   Scenario Descriptions and Interaction Diagrams

Here we show the behavior of the system in terms of sequences of objects interacting with each other and with the outside world.  We describe scenarios which are useful for understanding how objects work together to perform a task or reach a goal.  Complementary to these scenarios are interaction diagrams (message trace/event trace diagrams).

Introduction

Objects in a system exchange events, resulting in a possible state change.  The CTT Dynamic Model gives the state diagram for each information object class in the CTT Object Model.  Furthermore, it gives sequence diagrams which summarize the events between object classes.

A scenario is a sequence of events that can occur during a particular execution of the system.  The next section examines scenarios in the CTT System.

Scenarios

A.	Scenarios for the TTR Class

CTT use cases were given in the interface requirements specification document[3].  Some of these use cases are rephrased using the language of our object model to illustrate CTT scenarios below.

•  Scenario 1

The event trace diagram in Figure 7 illustrates a majority of the events which can occur to a TTR.  In the description below, we assume that A is a CTT-Cust object and B is a CTT-SP object.  Numbered groups of related events are explained as follows:

1.	A has detected trouble on a subscribed service and requests B to create a TTR.  Upon receiving the request event, B creates a TTR within the guidelines of the specific SLA between A and B.  After B has received the creation notification from the TTR, it returns pertinent information (such as trouble report id and status) about the TTR to A.

2.	A wants to track the status of the TTR.  Upon receiving the request event, B sends an event to the TTR to retrieve the status.  The returned status is then passed to A.

3.	A wants to query or modify the TTR.  Upon receiving the request event, B sends an event to the TTR to get or modify one or more TTR attributes.  For a query request, the returned attribute(s) are passed to A.

4.	B sends a modification event to modify the TTR.  Upon receiving an AVC (Attribute Value Change) notification from the TTR, B sends a modification notification to A.

5.	The trouble has been cleared.  B sends an event to the TTR to set the status to "customerAwaitingVerification", i.e., awaiting for the customer to verify.  Upon receiving an AVC notification from the TTR, B sends a status update notification to A.

6.	The customer sends an event to verify repair completion.  B sends an event to the TTR to modify those attributes which are related to verification.

7.	When the state of the TTR is changed to closed, the TTR emits a closeOut notification to B.  Upon receiving the notification, B passes the closeOut information to A.

8.	Upon deletion, the TTR emits a deletion notification to B.  Upon receiving the notification, B sends a deletion notification to A.

•  Scenario 2

Unlike Scenario 1, the trouble is first detected by B, so B initiates to create the TTR.  Upon receiving the creation notification from the TTR, B sends a creation notification to A so that A knows that a trouble has been detected by B.

•  Scenario 3

Before the trouble is cleared, A decides to cancel resolution on the TTR and hence sends a cancellation event to B.  Upon receiving the cancellation event, B updates those attributes of the TTR which are related to cancellation, i.e., setting the cancelRequestedByCustomer value to TRUE, and the TRState value to closed.

•  Scenario 4

Unlike step 6 of scenario 1, A sends an event to B to deny repair completion.  Upon receiving the event, B sends an event to the TTR to re-open the TTR by setting the state to open/active. Upon receiving the AVC notification from the TTR, B sends a status update notification to A.

•  Scenario 5

Before the trouble is resolved, B decides to defer resolution on the TTR, i.e., this can occur when B is not granted authority to a requested activity type.  It sends an event to the TTR to change the state to deferred.  Upon receiving the AVC notification from the TTR, B sends a status update notification to A.

•  Scenario 6

This last scenario shows an abnormal situation when update on the TTR cannot be performed due to local conditions.  B first updates the state value of the TTR to disabled.  Upon receiving the AVC notification from the TTR, B sends a state update notification to A.

Figure 8:  An Event Trace Diagram for TTR Scenario 1

B.  Scenarios for the PTR Class

•  Scenario 1

Scenario 1 in Figure 8 illustrates a majority of the events which can occur to a PTR.  Groups of related events are explained below:

1.	B has created a PTR for planned maintenance.  Upon receiving a creation notification from the PTR, B passes pertinent information about the PTR to A.

2.	A wants to query the PTR.  Upon receiving the request event, B sends an event to the PTR to retrieve the requested attribute(s).  The returned attribute(s) are passed to A.

3.	B sends a modification event to modify the PTR.  Upon receiving an AVC notification from the PTR, B sends a modification notification to A.

4.	Planned maintenance has been completed and no trouble was found.  B changes the state of the PTR to closed.  Upon receiving an AVC notification from the PTR, B sends a status update notification to A.

5.	Upon deletion, the PTR emits a deletion notification to B.  Upon receiving the event, B sends a deletion notification to A.

•  Scenario 2

Before maintenance is completed, B decides to cancel maintenance.  It sends an event to the PTR to modify those attributes which are related to cancellation. Upon receiving an AVC notification from the PTR, B sends a status update notification to A.

•  Scenario 3

This is similar to Scenario 5 for TTR.

•  Scenario 4

This is similar to Scenario 6 for TTR.

Figure 9:  An Event Trace Diagram for PTR

4.2.2	Collaboration Diagrams

This section presents the collaboration diagrams that corresponds to the event trace diagrams described in section 4.2.1.  The section is divided into 2 parts, the first part presents collaboration diagrams for the scenario 1 cases of the TTR class and the second part presents collaboration diagrams for the scenario 1 cases of the PTR class.

A.  Scenario 1 of TTR Class

Figure 10:  Collaboration Diagram for TTR Class: Case 1

Figure 11:  Collaboration Diagram for TTR Class: Case 2

For case 3, one collaboration diagram is presented for customer requesting query (3a) and one for customer requesting modify (3b).

For the collaboration diagram of case 3a, we let the customer query a specific attribute which in this case, we use the CloseOutNarrative.

Figure 12:  Collaboration Diagram for TTR Class: Case 3a

For the collaboration diagram of case 3b, we let the customer request the modification of a specific attribute which in this case, we use the EscalationList.

Figure 13:  Collaboration Diagram for TTR Class: Case 3b

For the collaboration diagram of case 4, we let the Service Provider modify a specific attribute which in this case, we use the TroubleLocation.

Figure 14:  Collaboration Diagram for TTR Class: Case 4

Figure 15:  Collaboration Diagram for TTR Class: Case 5

Figure 16:  Collaboration Diagram for TTR Class: Case 6

Figure 17:  Collaboration Diagram for TTR Class: Case 7

Figure 18:  Collaboration Diagram for TTR Class: Case 8

B.  Scenario 1 of PTR Class

Figure 19:  Collaboration Diagram for PTR Class: Case 1

For the collaboration diagram of case 2, we let the customer query a specific attribute which in this case, we use the BeginTime.

Figure 20:  Collaboration Diagram for PTR Class: Case 2

For the collaboration diagram of case 3, we let the Service Provider modify a specific attribute which in this case, we use the EndTime.

Figure 21:  Collaboration Diagram for PTR Class: Case 3

Figure 22:  Collaboration Diagram for PTR Class: Case 4

Figure 23:  Collaboration Diagram for PTR Class: Case 5

4.2.3    State transition diagrams

State transition diagrams provide information on the states that each class seen at the CTT interface can occupy and the way that the objects in those classes transition from one state to the next.

The next section describes the state diagrams for the TTR and PTR object classes.

•  State Diagram for the TTR Class

After a TTR is created, it goes through a state change because of interactions between the CTT-Cust and the CTT-SP.  The state values are queued, open/active, deferred, cleared, closed and disabled.

Referring to Figure 9, the TTR may be created in the queued state, meaning that trouble resolution has not yet been initiated on the reported trouble.  It may be created in the open/active state if appropriate actions to resolve the reported trouble are being carried out.  During trouble resolution, the state of the TTR may be changed to deferred if the corrective action has been postponed, or closed if trouble resolution is canceled.  After the trouble has been corrected, the state is changed to cleared, awaiting for verification from the CTT-Cust.  After the CTT-Cust has verified repair completion, the state is changed to closed.  If the CTT-Cust does not verify in due time, the CTT-SP has the option to close the TTR.  During the lifetime, there may be a period when the state of the TTR is disabled because its information cannot be updated.

Figure 24:  State Diagram for the TTR Class

•  State Diagram for the PTR Class

The state diagram for PTR is almost identical to that for TTR (Figure 9).  The state values are identical.

When a PTR is created, it is in the queued state if maintenance has not started.  It is in the open/active state if maintenance has started.  If maintenance is postponed after it has started, the state is changed to deferred.  When maintenance is completed, the state is changed to cleared.  If maintenance completion is verified or canceled, the state is changed to closed.  During its lifetime, there may be a period when the state of the PTR is disabled because its information cannot be updated.

In the state diagram for PTR, cancellation of maintenance can only be initiated through a CTT-SP.  This is the only subtle difference from the state diagram for TTR.

## 4.3   Functional Model

The functional model provides specifications of the operations at the exported interfaces.  Such interfaces have been presented in section 2.5.

4.3.1   Operations at the exported interface

The following operation specifications are provided for the interfaces exported to the users at the system boundary.  The notion of interface types is used to collect operations together on a logical basis.

The following items are specified for each operation included in the interface types:

-	Input parameters:  The set of attributes expected by the operation when it is invoked.

-	Output parameters:  The set of attributes returned by the operation upon successful execution.

-	Exceptions:  The set of attributes returned by the operation upon failed execution.

-	Description of the semantics of the operation by specifying the preconditions required in order to successfully invoke the operation and the postconditions as a consequence of such invocation.

4.3.1.1   Customer_TTR Interface Type

This interface type is provided by the CTT-SP system to the CTT-Cust system.  The operations of the Customer_TTR interface type, as a subset of the CTT_SP operations, provide the capabilities for the customer to:

-	cancel trouble resolution on (TTRCancel),

-	request the creation of (TTRCreate),

-	modify (TTRModify),

-	track status of (TTRStatusTrack),

-	verify (TTRVerify) or

-	view (TTRView)

a TTR.  Detailed descriptions of the interface operations are given below:

•	TTRCancel

Inputs:

-	ClearPerson:  This is used to identify the person who requests for the cancellation.

-	TRID:  This is used to identify a TTR.

Outputs: none

Exceptions:

-	CannotCancelAtThisTime:  This exception is raised if the TTR is either cleared or closed at the time of invocation.

-	UnknownTRID:  This exception is raised if the TRID value is unknown.

The customer invokes this operation to request cancellation of trouble resolution on a TTR.  On successful invocation, the CancelRequestedByCustomer value is changed to TRUE and the TroubleClearancePerson value is updated by the ClearPerson parameter value.  Thus this operation has a side-effect on the CancelRequestedByCustomer and TroubleClearancePerson attributes.

•	TTRCreate

Inputs:

-	AdditionalTroubleInfoList:  This is used to pass additional trouble information.

-	InputTTRAttributeList:  This is used to specify additional TTR attributes which can be used to initialize a TTR.

-	TroubleObject:  This is used to specify the service against which a TTR is requested to be created.

-	TroubleType:  This is used to specify the trouble type.

Outputs:

-	OutputTTRAttributeList:  This is used to return additional  information of the created TTR to the customer.

-	TRID:  This is used to identify the created TTR.

Exceptions:

-	FallbackTTRingUsed:  The trouble facility in the SP's organization is either partially failed or temporarily unavailable for receiving TTR creation requests.

-	TTRAlreadyExists:  The creation is disallowed because there is an existing TTR on the same trouble object.

-	UnknownTroubleObject:  The TroubleObject parameter value is unknown, inapplicable or invalid.

-	UnknownTroubleType:  The TroubleType parameter value is unknown, inapplicable or invalid.

-	TRAttributeNotPresent:  The attribute expected is not supplied.

-	UnknownTRAttribute:  The attribute supplied is unknown.

-	InvalidTRAttribute:  The attribute supplied is invalid or not applicable.

The customer invokes this operation to enter a TTR.  A successful invocation will trigger the creation of a TTR.  The TTR is initialized by attributes which are supplied by either the customer or the SP.  The customer may supply the following TTR attributes in the InputTTRAttributeList parameter:

-	AuthorizationList,

-	CommitmentTimeRequest,

-	CustomerRoleAssignment,

-	CustomerTRNum,

-	Dialog,

-	EscalationList,

-	PerceivedTroubleSeverity,

-	PreferredPriority,

-	RepeatReport,

-	SuspectObjectList,

-	TRDescription,

-	TroubleDetectionTime,

-	TroubleLocationInfoList,

-	TroubleObjectAccessFromTime,

-	TroubleObjectAccessHours or

-	TroubleObjectAccessToTime.

The use of the outputTTRAttributeList parameter is optional.  It is implementation specific and is further constraint by the SLA.  The developer of a CTT solution set may determine which of the following TTR attributes should appear in the outputAttributeList parameter:

-	ActivityDuration,

-	AfterHrsRepairAuthority,

-	CancelRequestedByCustomer,

-	CloseOutNarr,

-	CloseOutVerification,

-	CommitmentTime,

-	InitiatingMode,

-	LastUpdateTime,

-	MaintServiceCharge,

-	OutageDuration,

-	ReceivedTime,

-	RelatedTRList,

-	RepairActivityList,

-	RestoredTime,

-	SPRoleAssignment,

-	TRNumList,

-	TroubleFound,

-	TroubleLocation or

-	TroubleClearancePerson.

-	TRState and

-	TRStatusTime.

•	TTRModify

Inputs:

-	TRID:  This is used to identify a TTR.

-	AddList:  This indicates the components which are to be added to the specified TTR attributes.

-	RemoveList:  This indicates the components which are to be removed from the specified TTR attributes.

-	ReplaceList:  This indicates the TTR attributes which are used to replace the specified TTR attributes.

Outputs: none

Exceptions:

-	AddDenied:  This exception is raised if the specified component cannot be added to an attribute value.  This can occur if the type of the attribute component is incorrect or the attribute is not set/list-valued.

-	ChangeDenied:  This exception is raised if the requested change is denied, e.g., because the pre-conditions of the triggered attribute-operations are invalid at the time of invocation.

-	RemoveDenied:  This exception is raised if the specified component cannot be removed from an attribute value.  This can occur if the type of the component is incorrect or the attribute is not set/list-valued.

-	ReplaceDenied:  This exception is raised if the existing attribute value cannot be replaced by the new attribute value.  This can occur if the type of the new attribute value is incorrect.

-     UnknownTRID:  This exception is raised if the TRID value is unknown.

-    UnknownTRAttribute:  This exception is raised if the attribute specified in the AddList, RemoveList or ReplaceList is unknown.

-   InvalidTRAttribute: This exception is raised if the attribute specified in the AddList, RemoveList or ReplaceList is invalid or not applicable.

The customer invokes this operation to request the modification of one or more TTR attributes.  A TTR attribute can be modified in three ways: adding a component, replacing a component or replacing the entire attribute (the first two cases are allowed if the attribute is set/list-valued).  Three input parameters are used to specify what the invoker requests to modify.  On a successful invocation, the TTR attributes are modified as requested.  Thus this operation has a side-effect on the specified TTR attributes.

The following describes what the customer can request to modify.  The customer can request to add a component to the following TTR attributes (including the attributes inherited from TR):

-     AdditionalTroubleInfoList,

-	AuthorizationList,

-     CustomerRoleAssignment

-	EscalationList,

-	TroubleLocationInfoList and

-	TroubleObjectAccessHours.

The customer can request to replace the following TTR attributes:

-	AfterHrsRepairAuthority,

-	CancelRequestedByCustomer,

-	CloseOutVerification,

-	CommitmentTimeRequest,

-	CustomerRoleAssignment,

-	Dialog,

-	PerceivedTroubleSeverity,

-	PreferredPriority,

-	RepeatReport,

-	TroubleClearancePerson,

-	TroubleDetectionTime,

-	TroubleLocationInfoList,

-	TroubleObjectAccessFromTime,

-	TroubleObjectAccessHours and

-	TroubleObjectAccessToTime.

The customer can request to remove a component from the following TTR attributes:

-     CustomerRoleAssignment

-	TroubleLocationInfoList and

-	TroubleObjectAccessHours.

•	TTRStatusTrack

Inputs:

-	TRID:  This is used to identify a TTR.

Outputs:

-	Status:  This gives the current status of a TTR.

-	StatusTime:  This indicates when the status a TTR was last changed or validated.

Exceptions:

-	UnknownTRID:  This exception is raised if the TRID value is unknown.

The customer invokes this operation to track the status of a TTR.  On a successful invocation, the TRStatus and TRStatusTime attributes are returned.  The TRStatus attribute may be validated as the result of the invocation.  Thus this operation may have a side-effect on the TRStatusTime attribute.

•	TTRVerify

Inputs:

-	TRID:  This is used to identify a TTR.

-	Verification:  This gives the disposition of the person who performed the verification.  The disposition value can be "noAction", "verified", "denied", "deniedActivityDurationDisputed" or "deniedCloseOutNarrDisputed".

-	VerifyPerson:  This is used to identify the person who performed the verification.

-    AdditionalInfo:  This is used to input additional information related to the verification.  This allows the VerifyPerson to enter remarks or explanations regarding the verification.  For instance, the customer may enter the reason for denying the verification.

Outputs: none.

Exceptions:

-	CannotVerifyOrDeny:  This exception is raised if the TRStatus value is not "clearedAwaitingCustVerification" at the time of invocation.

-	UnknownTRID:  This exception is raised if the TRID value is unknown.

The customer invokes this operation to convey the disposition of verification.  On a successful invocation, the closeOutVerification value is updated by the Verification input parameter value and the Dialog value is updated by the AdditionalInfo if the latter is not empty.  Thus, this operation has a side-effect on the CloseOutVerification attribute and optionally on the Dialog attribute.

•	TTRView

Inputs:

-	TRID:  This is used to identify a TTR.

-	TTRAttributeIdList:  This is used to specify the TTR attribute(s) which the customer requests to view.

Outputs:

-	TTRAttributeList:  This is used to return the TTR attribute(s) 	which 	the customer has requested to view.

Exceptions:

-	UnknownTTRAttributeId:  This exception is raised if one of the TTR attribute ids in the TTRAttributeIdList parameter is unknown.

-	UnknownTRID:  This exception is raised if the TRID value is unknown.

The customer invokes this operation to view one or more TTR attributes.  These attributes are returned in the TTRAttributeList output parameter.  This operation has no side-effect on the specified TTR.

TTR_Notification Interface Type

This interface type is provided by the CTT-Cust system to the CTT-SP system.  The operations in the interface type, as a subset of the CTT_Cust operations, are invoked by the SP to convey notifications of a TTR.  They are:

-	cancellation of trouble resolution (TTRCancellationNotify),

-	closeOut (TTRCloseOutNotify),

-	creation (TTRCreationNotify),

-	deletion (TTRDeletionNotify),

-	modification (TTRModificationNotify) and

-	status change (TTRStatusUpdate).

None of the operations in this interface has any side-effect.

•	TTRCancellationNotify

Inputs:

-     TRID: This is used to identify a TTR.

-     CloseOutNarrative:  This is used to convey the reason for the cancellation of the TTR.

The SP invokes this operation to inform the customer of the cancellation of trouble resolution on a TTR and the reason behind such cancellation.

Outputs:  none.

•	TTRCloseOutNotify

Inputs:

-	TRID:  This is used to identify a TTR.

-	CloseOutList:  This is used to convey history information of a closed TTR.

Outputs: none.

The SP invokes this operation to convey history information of a TTR whose state has been changed to closed.  The history information in the CloseOutList parameter may include the following TTR attributes:

-	ActivityDuration,

-	AdditionalTroubleInfoList,

-	AuthorizationList,

-	CancelRequestedByCustomer,

-	CloseOutNarr,

-	CloseOutVerification,

-	CommitmentTime,

-	CustomerTRNum,

-	PerceivedTroubleSeverity,

-	ReceivedTime,

-	RestoredTime,

-	TRDescription,

-	TRID,

-	TRNumList,

-	TroubleClearancePerson,

-	TroubleFound,

-	TroubleObject and

-	TroubleType.

The use of the CloseOutList parameter is optional.  It is up to the developer of a CTT solution set to determine those TTR attributes which should appear in the CloseOutList parameter.

•	TTRCreationNotify

Inputs:

-	TRID:  This is used to identify a TTR.

-	CreationList:  This is used to convey information of the created TTR.

Outputs: none

When the SP has detected a service failure, it initiates the creation of a TTR and invokes this operation to inform the customer of the creation.  The use of the CreationList parameter is optional.  It is up to the developer of a CTT solution set to determine which TTR attributes should appear in the CreationList parameter.

•	TTRDeletionNotify

Inputs:

-	TRID:  This is used to identify a TTR.

Outputs: none.

The SP invokes this operation to inform the customer of the deletion of a closed TTR.

•	TTRModificationNotify

Inputs:

-	TRID:  This is used to identify a TTR.

-	ModificationList:  This is used to convey information on the TTR attributes which have been modified by the SP.

Outputs: none.

After the SP has initiated to modify one or more TTR attributes, it invokes this operation to inform the customer of the modification.  The modified TTR attributes can include the following (modification constraints are shown after each attribute where appropriate):

-	ActivityDuration,

-	AuthorizationList (only when the TTR is neither cleared not closed),

-	CommitmentTime (only when the TTR is neither cleared nor closed),

-	Dialog,

-	EscalationList (only when the TTR is neither cleared nor closed),

-	OutageDuration (only when the TTR is either cleared or clsoed),

-	RelatedTRList,

-	RepairActivityList (only when the TTR is neithercleared nor closed),

-	RepeatReport,

-	RestoredTime (only when the TTR is either cleared or closed),

-	TRNumList,

-	SPRoleAssignment,

-	TroubleFound (only when the TTR is either cleared or closed),

-	TroubleLocation

-	TRState.

The use of the ModificationList parameter is optional.  It is up to the developer of a CTT solution set to determine which TTR attributes should appear in the ModificationList parameter.

•	TTRStatusUpdate

Inputs:

-	TRID:  This is used to identify a TTR.

-	StatusInfo:  This is used to pass the TRStatus, TRState and TRStatusTime attributes.

Outputs: none.

The SP invokes this operation to inform the customer of a status change of a TTR.  The use of the StatusInfo parameter used especially when SP is waiting for the customer to verify something.

4.3.1.3   PTR_Notification Interface Type

This interface type is provided by the CTT-Cust system to the CTT-SP system.  The operations in the interface type, as a subset of the CTT_Cust operations, are invoked by the SP to convey notifications of a PTR.  They are:

-	cancellation of maintenance (PTRCancellationNotify),

-	creation (PTRCreationNotify),

-	deletion (PTRDeletionNotify),

-	modification (PTRModificationNotify) and

-	status change (PTRStatusUpdate).

None of the notifications has any side-effect.

•	PTRCancellationNotify

Inputs:

-	TRID:  This is used to identify a PTR.

Outputs: none.

The SP invokes this operation to inform the customer of the cancellation of trouble resolution on a PTR.

•	PTRCreationNotify

Inputs:

-	TRID:  This is used to identify a PTR.

-	CreationList:  This is used to convey information of the created PTR.

Outputs: none.

After the SP has created a PTR, it invokes this operation to inform the customer of the creation.  The use of the CreationList parameter is optional.  It is up to the developer of a CTT solution set to determine which PTR attributes should appear in the CreationList parameter.

•	PTRDeletionNotify

Inputs:

-	TRID:  This is used to identify a PTR.

Outputs: none.

The SP invokes this operation to inform the customer of the deletion of a closed PTR.

•	PTRModificationNotify

Inputs:

-	TRID:  This is used to identify a PTR.

-	modificationList:  This is used to convey information on the PTR attributes which have been modified by the SP.

Outputs: none.

After the SP has modified one or more PTR attributes, it invokes this operation to inform the customer of the modification.  The PTR attributes which can be modified include the following (modification constraints are shown after each attribute where appropriate):

-	ActivityDuration (only when the PTR is either cleared or closed),

-	AdditionalTroubleInfoList,

-	BeginTime (only when the PTR is queued),

-	CloseOutNarr (only when the PTR is either cleared or closed),

-	EndTime (when the PTR is not closed),

-	TRNumList,

-	TRState,

-	TRStatus,

-	TRStatusTime (when the status is updated),

-	RelatedTRList,

-	RepairActivityList,

-	RestoredTime (only when the PTR is either cleared or closed),

-	SPRoleAssignment,

-	TroubleClearancePerson,

-	TroubleFound (only when the PTR is either cleared or closed) or

-	TroubleLocation.

The use of the ModificationList parameter is optional.  It is up to the developer of a CTT solution set to determine which PTR attributes should appear in the ModificationList parameter.

•	PTRStatusUpdate

Inputs:

-	TRID:  This is used to identify a PTR.

-	StatusInfo:  This is used to convey the TRStatus and TRStatusTime attributes.

Outputs: none.

The SP invokes this operation to inform the customer of a status change of a PTR.  The use of the StatusInfo parameter is optional to the developer of a CTT solution set.

# 5.0	Design Considerations

In general, the technical constraints as listed in section 2.4 should be met.  Special attention should be given to restart and error handling where the system has to make sure that no information is lost during the restart process.

In addition to these, the systems within the CTT domain, especially CTT-SP, must be designed to handle large numbers of simultaneous connections.

# 6.0       Requirements satisfaction analysis

This section examines the requirements addressed in this document and states which requirements have been fully satisfied and which have not.  In the case of the latter, a rationale for this short-coming is given.  Particular attention is paid as to how optionality within the requirements has been accommodated.

The requirements in the interface specification requirements document[3] are divided into general technical requirements and specific trouble administration interface functional requirements.  In this section, we demonstrate how these requirements are addressed by our interface specification.

The general technical requirements include transport requirements, connection requirements, control and mapping requirements, and security requirements.  The first three requirements are readily met because our interface specification (1) is transport protocol-independent, (2) is independent of the underlying network infrastructure, and (3) gives flexibility for a Solution Set to choose the required interface types from this document.

The security requirements can be met at different levels in a Solution Set.  It can be met at the transport/network level by selecting a transport/network protocol which could meet the security requirements.  It can be met at the application level by using a security API.  Or it can be met by adding security-related interface functions (e.g., authentication functions) to our interface types or adding security-related parameters to our interface functions.

For the rest of this section, we demonstrate how the specific requirements are addressed by our interface specification.  The name used in the interface requirements specification document[3] to identify each requirement will be indicated.  The requirement names have the format CTTFRX.X.

## 6.1	Trouble Report Subject

1.	Each Trouble Report must have a single, identified subject.  Any single resource or service may be the subject of a Trouble Report.  The resource or service which is the subject of a Trouble Report must be known to the Service Provider.  If the Customer uses the interoperable interface to create a Trouble Report, then the inventory of resources and services which may be the subject of a Trouble Report must be made known to the Customer in advance by the Service Provider. (CTTFR1.1)

The means to provide and maintain this inventory are outside the scope of this document.

The grouping of related resources and services into one Trouble Report is outside the scope of this document.

The naming convention used in the inventory may be constrained by the interface protocol.

Analysis solution: This requirement is addressed by the TroubleObject attribute.

## 6.2  Trouble Report Creation

This section covers the initial creation of Trouble Reports by either a Customer or a Service Provider.  Technically, neither side can ‘create’ a Trouble Report on the other side.  A Customer can ‘request’ that a Trouble Report be created on the Service Provider side.  A Service Provider can inform a Customer that a Trouble Report has been created by the Service Provider, and the Customer can create a Customer Trouble Report if they wish.

Note, after a Trouble Report has been created the state model allows for it to rest in a 'queued' state until it is either canceled (moved to ‘closed' state or opened (moved to 'open/active' state).  Opening does not necessarily imply human intervention;  some Service Providers may move a Trouble Report into the 'open/active' state automatically.

A Customer  request to a Service Provider could be rejected for a variety of reasons (for example, the Customer may have sent the request to the wrong Service Provider, or the resource or service specified in the request may not be in the inventory).

A Customer may initiate, via a local process, a request that a Trouble Report on a resource or service be created by a Service Provider using the interoperable interface. (CTTFR2.1)

The process by which the Customer initiates the request is a local matter outside the scope of this document.

This is a choice by the Customer.  Not every Customer Trouble Report needs to be sent to an Service Provider.  The choice is a local Customer policy.

Analysis solution:  This requirement is addressed by the TTRCreate operation in Customer_TTR interface.

2.	A Service Provider may notify a Customer about the creation of a Trouble Report on a resource or service based on one of the following scenarios:

-	a Service Provider management system detects a trouble and automatically generates a Trouble Report, for instance due to threshold limits being exceeded or alarm notifications. (CTTFR2.2a)

Analysis solution:  This requirement is addressed by the TTRCreationNotify operation in TTR_Notification interface.

-	a person in the Customer’s  organization reports a trouble to the Service Provider by a procedure other than using the interoperable interface (e.g. a telephone call), and as a result the Service Provider creates a Trouble Report. (CTTFR2.2b)

Analysis solution:  This requirement is addressed by the TTRCreationNotify operation in TTR_Notification interface.  The InitiatingMode attribute of the TTR object specifies how and perhaps why the trouble report is entered.

3.	A Service Provider may create a Trouble Report to specifically notify the Customer that planned maintenance will be carried out at a given time and that all or parts of the service(s), resource(s), network or system will be inaccessible during that time.  In this case, the trouble management function is used to notify the Customer that planned maintenance action is scheduled. (CTTFR2.3)

Analysis solution:  This requirement is addressed by the PTRCreationNotify operation in PTR_Notification interface.

The creation time should be recorded in a Trouble Report by the Service Provider. (CTTFR2.4)

Creation time is the time that the Trouble Report is accepted as valid by a Service Provider.  If the Service Provider implements a ‘queued’ state, there may be an interval between Trouble Report creation time and opening time, otherwise they are equivalent.

Analysis solution:  This requirement is addressed by the ReceivedTime attribute.

5.	If the Service Provider rejects the Trouble Report (i.e., does not create it), the Customer should be informed. (CTTFR2.5)

Analysis solution:  An exception (e.g., TTRAlreadyExists)  would be raised if the SP decides not to create the trouble report.

6.	The Trouble Report originator, either Service Provider or Customer, should be able to specify the time at which the trouble was detected, as it is likely to be different from the time at which the Trouble Report was created. (CTTFR2.6)

Analysis solution:  This requirement is addressed by the TroubleDetectionTime attribute.

7.	It should be possible to have alias(es) for the Trouble Report.  For example, there may be distinct Trouble Report IDs on the Service Provider side and the Customer side.  This allows for compatibility with existing trouble handling systems, e.g., the Trouble Report identity derived using existing internal trouble management practices may be used.  Each Trouble Report may support a Customer Trouble Report ID in addition to the Service Provider Trouble Report ID. (CTTFR2.7)

Analysis solution:  This requirement is addressed by the CustomerTRNum and TRNumList attributes.

8.	Both Customer and Service Provider should be able to group Trouble Reports together, e.g., to indicate a common problem. (CTTFR2.8)

The criteria for grouping or regrouping Trouble Reports is outside the scope of this document. This is a local matter.

Analysis solution:  This requirement is addressed by the RelatedTRList attribute.

9.	A Trouble Report used during the trouble resolution procedures may have associated with it one or more alarms triggered by a single common problem (association with alarms is a local matter). (CTTFR2.9)

This implies the presence of a separate Fault Management system that allows for the sharing of alarm information.

Analysis solution:  The SP can create a trouble report after it receives an internal alarm.  The InitiatingMode value is set to indicate that the TR creation was alarm-originated.

10.	Multiple distinct Trouble Reports can be created on the same subject if the Service Provider and Customer support it, although this does not necessarily imply Trouble Report correlation. (CTTFR2.10)

Analysis solution:  In case the SP and the customer do not allow the creation of more than one trouble report on the same trouble object, the exception TTRAlreadyExists would be raised when the TTRCreate operation is invoked.

11.	It should be possible for either the Service Provider or Customer to include comments in free format text in the Trouble Report.  Comments once included cannot be modified or removed. (CTTFR2.11)

Analysis solution:	This requirement is addressed by the AdditionalTroubleInfoList attribute.

12.	It shall be possible for the Trouble Report originator, either Service Provider or Customer, to include in the Trouble Report the trouble type and additional information about the trouble.  This represents the Customer’s viewpoint, not the Service Provider’s viewpoint. (CTTFR2.12)

Analysis solution:  This requirement is addressed by the TroubleType and AdditionalTroubleInfoList attributes.

13.	The Trouble Report originator, either Service Provider or Customer, should be able to supply the identity of the person or system who originated the Trouble Report. (CTTFR2.13)

Analysis solution:  This requirement is not addressed by the P interface specification because it is considered to be a local issue.

14.	The Trouble Report originator, either Service Provider or Customer, should be able to supply the perceived trouble severity in terms of the effect on the Trouble Report subject. (CTTFR2.14)

This severity is from the Customer’s viewpoint.  Severity is not to be confused with priority or escalation level.

Analysis solution: This requirement is addressed by the PerceivedTroubleSeverity attribute.

15.	In addition to the identity of the service or network resource on which the trouble is being reported, the Trouble Report may contain additional suspect objects and the location of the trouble.  This additional information is not necessarily present in the create request, but may be added by Trouble Report updates. (CTTFR2.15)

Analysis solution:  This requirement is addressed by the SuspectObjectList and TroubleLocation attributes.

16.	Customer should be able to supply a priority to indicate the urgency for resolution of the trouble relative to their own other active Trouble Reports. (CTTFR2.16)

Analysis solution:  This requirement is addressed by the PreferredPriority attribute.

17.	Both the Service Provider and  Customer should each be able to identify a contact person and an alternative contact person(s) from their respective organizations who can provide additional information about the trouble. (CTTFR2.17)

Analysis solution: This requirement is addressed by the CustomerRoleAssignment and SPRoleAssignment attributes.

18.	The Customer or Service Provider should be able to indicate that the subject of the Trouble Report was recently created or that another trouble was reported on this subject in the recent past. (CTTFR2.18)

This might be used to identify chronic problems and newly deployed equipment.

The mechanism used for determining this is outside the scope of this document.

Analysis solution:  This requirement is addressed by the RepeatReport attribute.

19.	The Customer should be able to request a commitment time (i.e., the time at which the Service Provider either arrives on site, or clears the trouble). The Service Provider may supply a commitment time, not necessarily the same time as requested by the Customer. (CTTFR2.19)

Analysis solution: This requirement is addressed by the CommitmentTimeRequest and CommitmentTime attributes.

20.	The Customer should be able to specify when the site is accessible to the Service Provider. (CTTFR2.20)

Analysis solution: This requirement is addressed by the TroubleLocationInfoList parameter which includes the location hours.

21.	Customer should be able to specify when the subject of the Trouble Report is accessible (i.e., when disruptive repair activity may be carried out). (CTTFR2.21)

Analysis solution: This requirement is addressed by the TroubleObjectAccessFromTime, TroubleObjectAccessToTime, and TroubleObjectAccessHours attributes.

22.	The Service Provider should be able to inform the Customer when a Trouble Report is created.

This capability may, but should not have to be, explicitly requested by the Customer.

Analysis solution:  This requirement is addressed by the ReceivedTime attribute.

## 6.3  Trouble Report Tracking

This section is concerned with the tracking of the problem resolution process by the CTT-Cust.

- A Customer should be able to track the progress of a Trouble Report to resolution.  The Customer should be able to request the Trouble Report state and/or status from the Service Provider. The Service Provider should be able to notify Customer of changes to the Trouble Report. (CTTFR3.1)

This tracking capability should not have to be explicitly requested by Customers.

The issue of whether the Customer Trouble Report system polls for status versus having the Service Provider Trouble Report system automatically update status will be explicitly determined by what transport protocol is implemented.

Some implementations may provide a periodic refresh by the Service Provider to the Customer, irrespective of whether any changes have taken place, but this mechanism is not itself a functional requirement.

Analysis solution: The customer can use the TTRStatusTrack operation to check the status of a trouble report.  In addition, the SP can invoke TTRStatusUpdate or TTRModificationNotify to inform the customer of a status change or an attribute value change.

2.	The state of the Trouble Report should be updated as the global condition of the Trouble Report changes (i.e., whether or not the Trouble Report may be changed, closed, or otherwise moved through the State Transition Model). (CTTFR3.2)

Analysis solution:  This requirement is addressed by the TRState attribute.  The SP can modify the state using TRStateReplace as the global condition of the trouble report changes.

3.	The status of the trouble repair activity should be updated as the Service Provider’s trouble resolution progresses, and the Customer should be notified of all changes. The Service Provider should maintain the time of the last change in the Trouble Report status. (CTTFR3.3)

Analysis solution: This requirement is addressed by the TTRStatusUpdate operation and the TRStatusTime attribute.

4.	The time at which the Trouble Report was last updated should be recorded. (CTTFR3.4)

Analysis solution:  This requirement is addressed by the LastUpdateTime attribute.

5.	The Trouble Report should maintain a running account of information on the activities taken to diagnose, test and repair the trouble, the repair type, the length of time spent on each activity, whether the activity is billable or not and the equipment involved in each activity.  This information is provided by the Service Provider.  In addition to the running account, it should be possible to summarize key information, such as how long was spent on an activity, whether the work is billable or not, etc. (CTTFR3.5)

Any processing that is carried out on this information is a local matter.

Analysis solution:  This requirement is addressed by the ActivityDuration and RepairActivityList attributes.

- The Service Provider should allow the Customer to retrieve a complete description of a specific Trouble Report based on the specification of the unique Service Provider Trouble Report ID. (CTTFR3.6)

This would be for the re-synchronization of “views” of Trouble Reports if there is a suspicion that they are not equivalent, or the creation of a new Customer Trouble Report instance if a Trouble Report has been missed.

Analysis solution:  This requirement is addressed by the TTRView operation.

7.	The Service Provider may allow the Customer to retrieve complete descriptions of a specific set of Trouble Reports based on ad hoc filter definition (e.g., active (not closed) Trouble Reports, updated Trouble Reports, etc.). (CTTFR3.7)

This may be implementation specific.

Analysis solution:  The interface operations do not include scoping/filtering functionalities.

8.	It is possible that the Service Provider system may not be able to update the Trouble Report information. In this case the Customer should be informed that the Trouble Report is disabled and its information cannot be updated. (CTTFR3.8)

Analysis solution:  When the TRState value is changed to disabled, the SP can invoke the TTRModificationNotify operation to inform the customer of any state change.

## 6.4  Trouble Report Management

1.	It should be possible for a Customer to escalate a Trouble Report if this is supported by the Service Provider, or for a Trouble Report to be escalated by the Service Provider.  The Customer should be informed of all changes to the escalation level.  Escalation levels can only increase, not decrease. (CTTFR4.1)

Some Service Providers  may not wish to allow Customers to escalate Trouble Reports, as it could cause confusion, but prefer to carry out escalation according to prior arrangement in a Business Level Agreement. In any case, the conditions underwhich a Trouble Report is escalated (for example, if the trouble has not been resolved in a given period of time) should be contained in a Business Level Agreement.

Analysis solution:  This requirement is addressed by the EscalationList attribute and the TTRModificationNotify operation.

2.	It should be possible for the Service Provider to defer work on a Trouble Report.  For example, it may be necessary to suspend repair work because access to the faulty resource has been prevented by the Customer.  Deferral of a Trouble Report suspends the ‘outage duration timer’.  The Service Provider should notify the Customer when a Trouble Report has been deferred and when the deferral is released. (CTTFR4.2)

Analysis solution:  When the TRState value is changed to deferred, the SP can invoke the TTRModificationNotify operation to inform the customer of the state change.

3.	The Customer should be able to provide the required repair completion time (which may be derived from an SLA, for example) and obtain from the Service Provider the expected repair time, and subsequently the actual repair time. (CTTFR4.3)

Analysis solution: The first requirement is addressed by the CommitmentTimeRequest and CommitmentTime attributes.  The actual repair time is provided by the RestoredTime attribute.

4.	The Customer should be able to update an open and active Trouble Report with new information, e.g., additional trouble related information, contact change, etc. (CTTFR4.4)

Analysis solution:  This requirement is addressed by the TTRModify operation.

5.	The Customer should be able to authorize repairs to be made at the Customer’s location outside of normal business hours, and also be able to identify the permitted access times for a given single location. (CTTFR4.5)

Analysis solution:  The first requirement on authorization is addressed by the AuthorizationList and AfterHrsRepairAuthority attributes.  The second requirement on access times is addressed by the TroubleLocationInfoList attribute.

6.	Upon receiving a request for authorization related to specific repair activities from the Service Provider, a Customer should be able to provide authorization over the interoperable interface. (CTTFR4.6)

Analysis solution:  The customer can use the TTRModify operation to modify the AuthorizationList attribute, in granting authorization to the SP.

7.	The Service Provider should be able to indicate to the Customer whether the repair activity covered by the present Trouble Report is billable. (CTTFR4.7)

Analysis solution:  This requirement is addressed by the MaintServiceCharge attribute.

## 6.5  Trouble Report Clearing and Closure

1.	A Customer should be able to request the cancellation of a Trouble Report and indicate in the Trouble Report the person making the request. (CTTFR5.1)

Analysis solution:  This requirement is addressed by the TTRCancel operation.  The ClearPerson input parameter can be used to specify the person who makes the request.

2.	If the Customer is unreachable by the Service Provider at a time when the Service Provider requests clearance verification for the Trouble Report, the activity timer should be suspended. The Service Provider should be able to detect when the Customer becomes reachable again, so normal processing may be resumed. (CTTFR5.2)

Analysis solution:  This seems to be an implementation issue.

3.	The Customer should be allowed to verify the trouble is cleared prior to closure of the Trouble Report by the Service Provider.  When the clearing of the problem related to the Trouble Report is verified by the Customer, the identity of the trouble clearance person doing the verification should be captured in the Trouble Report. In the event that the Customer denies or delays closure verification, then the Service Provider should have the option of closing the Trouble Report. (CTTFR5.3)

Analysis solution:  This requirement is addressed by the TTRVerify operation and the ClearPerson input parameter.

4.	Once an Service Provider has deemed a Trouble Report to be cleared and has notified the Customer, regardless of whether the Customer denies clearance or fails to respond, the Service Provider should be able to deny the Customer the ability to modify some of the information in the Trouble Report. (CTTFR5.4)

This is intended to prevent redefinition of the original problem.

The details of how the Service Provider ensures that the appropriate notification has been received by the Customer and the time period within which he has to respond to the request are beyond the scope of this specification.

Analysis solution:  The Service Provider can deny the Customer the ability to modify the trouble report if the Customer fails to verify in a timely manner.  If the customer tries to modify a trouble report by not following the constraints of the TR state model, the ChangeDenied exception is raised.

5.	The actual cause of the trouble found, together with the solution provided and other relevant factors, should be recorded in the Trouble Report. (CTTFR5.5)

Analysis solution:  This requirement is addressed by the TroubleFound and CloseOutNarr attributes.

6.	It should be possible to compute/determine the outage duration of the equipment/service from the repair activity information held in the Trouble Report or the Repair Activity object instance if present.  This outage duration may then be added to the Trouble Report at the time of closure. (CTTFR5.6)

The algorithm for computing/determining the outage duration is outside the scope of this document.

Analysis solution:  This requirement is addressed by the RepairActivityList, RestoredTime and OutageDuration attributes.

7.	The Customer should be able to request an indication when an instance of a Trouble Report is deleted. (CTTFR5.7)

Analysis solution:  The SP can invoke the TTRDeletionNotify operation.

8.	It should be possible to determine outage duration of the service from the repair activity information held in the Trouble Report. This outage duration may then be added to the Trouble Report at the time of closure.

Analysis solution:  This requirement is addressed by the RepairActivityList, RestoredTime, and OutageDuration attributes.

## 6.6  Trouble Report History

1.	It should be possible for either Service Provider or Customer to maintain Trouble Report history. (CTTFR6.1)

Analysis solution:  When the TRStatus value is changed to indicate closedOut, closedOutByCustReq, closedOutCustVerified or closedOutCustDenied, the SP can create a trouble history record to capture the history of the closed trouble report.  It can invoke the TTRCloseOutNotify operation to inform the customer of the history information of the closed trouble report.  After receiving the notification, the customer can use local means to store the history information.

# 7.0       Conclusions

This document defines the CTT interface types to meet the business and technical requirements in the interface requirements document[3].  All the CTT interface operations here are analyzed in a way independent of the communications protocol used between the customer and the service provider.  By giving a protocol independent analysis, this document provides a neutral platform for the development of protocol dependent CTT Solution Sets.

The analysis is found to be complete as described in section 6.

# Appendix A    Glossary of terms and acronyms

- Terms:  Section 1.3 provides the key terms specific to Customer Trouble Ticketing.  The following provides informal definition of terms which are used throughout the document to support the analysis of an interface.

computation object:  This is used to model a system in a domain which may have a need to export its operations or import operations to/from other systems in the domain.

domain:  This provides a boundary of the systems in a context.  This boundary may be defined from an administrative, functional or physical point of view.

information object:  This provides the data source for a system.  Its state may change as the result of interaction between systems.  Unlike a computation object, the operations of an information object are not visible to a system external to the system containing the information object.

interface:  This is used to describe how two systems in a domain can interact with each other.  It can be represented by a set of interface types.

interface operation:  This is a component of an interface type.  When invoked, either a normal result is returned or an exception is raised.  The semantics of an interface operation can be described by the state of the information object(s) before and after the operation is invoked.

interface type:  This is a group of related interface operations.  A computation object may export or import one or more interface type(s).

- Acronyms:

CTT	Customer Trouble Ticketing

CTT-Cust	Customer Trouble Ticketing - Customer

CTT-SP	Customer Trouble Ticketing - Service Provider

PTR:	Provider Trouble Report

SLA:	Service Level Agreement

SP:	Service Provider

TR:	Trouble Report

TTR:	Telecommunications Trouble Report

# Appendix B    References

1.	Network Management Forum: NMF 025, The "Ensemble" Concepts and Format, Issue 1.0, July 1992.

2.	Network Management Forum: A Service Management Business Process Model, 1995.

3.	Network Management Forum: Customer to Service Provider Trouble Administration Interface: Requirements Specification

4.	Network Management Forum:  NMF Solution Set Analysis Document Template, version 1.0, 1996

5.	Object-Oriented Modeling and Design: Rumbaugh, et. al., Prentice Hall, 1991.

6.	TINA-C: Information Modeling Concepts, version 2.0, 1995.

7.	TINA-C: TINA Object Definition Language (TINA-ODL) Manual, version 1.3, 1995.

# Appendix C     Relevant Standards

8.	ITU Recommendation X.711, Information Technology - Open Systems Interconnection - Common Management Information Protocol, 1991.

9.	ITU Recommendation X.790, Information Technology - Open Systems Interconnection - Trouble Management Function, 1995.