PERFORMANCE REPORTING
DEFINITIONS DOCUMENT

NMF 701

Issue 1.0

April 1997

 Network Management Forum 1997.

Members of the Network Management Forum are granted limited copyright waiver, under the terms of the Agreement on Intellectual Property Rights, to reproduce this document.

Disclaimer and copyright statements

The Network Management Forum (“NMF”) has made every effort to assure that the contents of this document are accurate; however, due to the inherent complexity in the design and implementation of protocols, no liability is accepted for any errors or omissions or for consequences of any use made of this document.  Under no circumstances will the NMF be liable for direct or indirect damages or any costs or losses resulting from the use of this specification.  The risk of designing and implementing products in accordance with this specification is borne solely by the user of this specification.  This document may involve a claim of patent rights by one or more NMF members, pursuant to the agreement on Intellectual Property Rights between NMF and its members, and by non-members of NMF.

Acknowledgments

SMART (Service Management Automation and Re-engineering Team ) work is a genuinely collaborative effort. I would like to thank the following people for contributing their time and expertise to the production of this definitions document.

| Patrick Rhude | CrossKeys | Alberto Pisani | Telecom Italia |
| --- | --- | --- | --- |
| Frank Birch | Fujitsu Europe | Daniele Toro | Telecom Italia |
| Lightsey Wallace | Hekimian | Peter Huckett | TTC |
| Hal Folts | NASA | Andy McGarty | X-Cel |
| Mike Kelly | Nortel | Douglas Kearney | X-Cel |
| Fabio Macuglia | STET - Sodalia |  |  |

Many thanks for contributions to this document also go to Elizabeth Adams and Kimberly Lewis of the NMF.

Thanks are also extended to many others who provided review comments.

Hans Rudolf Stucki

(Unisource Business Networks Switzerland, Ltd.), team chairman

About this document

Revision history

| Version | Date | Comments |
| --- | --- | --- |
| 0.1 DRAFT | January 24, 1997 | File: PERDEF01.DOC WinWord 6.0 First draft of the “NMF SMART Performance reporting definitions document”, developed by members of the SMART Performance Reporting Team during the meetings and workshops at Miami FL, January 21-24, 1997. Distribution: Team Members |
| 0.2 DRAFT | March 13, 1997 | File: PERDEF02.DOC WinWord 6.0 Update of Team members’ contributions. Consistency check across the different sections Distribution: Team Members |
| 0.3 DRAFT | April 2, 1997 | File: PERDEF03.DOC WinWord 6.0 Team members’ review feedback included: - Aknowledgements: updated - Preface: changes to the last paragraph - 4.4: changes to the 1st paragraph - 4.5: new - 4.6: new - 5.2.2: table 5/2 added - 5.2.3: SAP definition added in table 5/3 - 5.4: new - 6.2.3: further developed - 7: new - Annex A: former Appendix A - Annex A: minor edits; Annex A6 added - Appendix 1: former Appendix B - Appendix 1: further developed - Appendix 2: former Appendix C Distribution: NMF Members |

Contact for this document

The editor of this revision of the document can be contacted as follows:

Hans R Stucki

Product Manager

UBN (Switzerland) Ltd.

Schermenwaldstrasse 13

Ittigen, 3063

Switzerland

	+41 31 688 8183

	+41 31 688 8152

e-mail:	hans-rudolf.stucki@ubn.unisource.ch

Further information on NMF programs and documentation can be obtained by contacting the NMF office:

NMF
10201 Mt. Kemble Avenue
Morristown
New Jersey 07960,  USA.
Tel: (+1) 201 425 1900
Fax: (+1) 201 425 1515
Web: http://www.nmf.org

How to obtain a copy

An electronic copy of this document in MS-Word V6.0 format can be obtained using ftp from the NMF server.  Alternatively, contact the NMF as indicated above. The file is located at:

199.183.141.190:Members_Only/smart/perform/Workdocs/perdef03.doc

ftp.nmf.org:Members_Only/smart/perform/Workdocs/perdef03.doc

Comments

Comments should be sent to the editor.

Note:  smartperf@nmf.org can be used as e-mail id for all SMART Performance Reporting Team members. Messages are distributed to all Team members automatically. The Team member list is found in the current NMF roster. In the case of questions, do not hesitate to contact one of the Team members.

Use of fonts

Italics:  Paragraphs in italics are notes by the authors to the reader to provide additional or background information.

CONTENTS

PREFACE

1 EXECUTIVE SUMMARY

2 INTRODUCTION TO THIS DOCUMENT

2.1 Scope

2.2 Document Structure and Outline

2.3 Key terms used in this document

3 SERVICE DEFINITION

3.1 Service and Service Element

3.2 Service Availability

3.3 Network Topology - Containment and Management Domains

3.4 Service Access Points and Customer Premises Equipment

3.5 Regulatory Issues

4 SERVICE LEVEL AGREEMENT

4.1 What is a Service Level Agreement  (SLA)?

4.2 The role of SLAs in the evolving marketplace

4.3 List of typical SLA parameters

4.4 Focus on Quality of Service within SLAs

4.5 Grade of service

4.6 Need for performance measurement in support of SLAs

4.7 Priorities for SLA commitments

5 SERVICE AVAILABILITY DEFINITION AND CALCULATION FORMULA

5.1 Introduction

5.2 Service Availability

5.2.1 Basic Service Availability Calculation Formula

5.2.2 Service Degradation Factor (SDF)

5.2.3 The Service Access Point (SAP) Concept

5.2.4 The Service Access Point Weight in the Service Availability Calculation Formula

5.3 Information Source (Using of TT and/or Network Information)

5.4 Service Availability  Value

6 PERFORMANCE REPORTING

6.1 Reporting Frequency

6.2 Formats and Contents for Reports

6.2.1 Common Report Header

6.2.2 QoS Report

6.2.3 Service Specific Measurements

6.2.4 Traffic Data Reports

7 CONCLUSIONS

ANNEX A - EXAMPLES

ANNEX A1 - Service Availability for PL-LL circuits

ANNEX A2 - Service Availability for PVC/SDLC Service

ANNEX A3 - Service Availability for PL-LL circuits / Trouble Ticket  based

ANNEX A4 - Service Availability for PVC Frame Relay services / Trouble Ticket based

ANNEX A5 - Service Availability for SVC/LAN connectivity / Trouble Ticket based

ANNEX A6 - Note to the Service Availability calculation when using Trouble Tickets

APPENDIX 1 - GLOSSARY OF TERMS, ACRONYMS AND ABBREVIATIONS

1.1 The use of terms in this document

1.2 Abbreviations

APPENDIX 2 - REFERENCED AND RELATED DOCUMENTS

# PREFACE

NMF is an international consortium of communications Service Providers and their suppliers, dedicated to promoting industry-wide agreements on how management information should be exchanged between a Service Provider and its Customers, its suppliers and/or other Service Providers. Such agreements are essential to achieve a seamless, cost-efficient operation where multiple companies are involved in the fulfilment of service requests, the management of services while in use by Customers, the correction of problems that might impair service quality, and the accurate billing of the Customer for services used.

These agreements are becoming an increasing necessity as regulatory forces change the landscape for Service Providers. The addition of new value-add Service Providers such as Internet Service Providers and the development of global alliances mean that the service delivery chain is being extended beyond the traditional, monopoly environment where a single company had end-to-end control over the provision and management of Customer services. At the same time, some companies are choosing to focus their business on network infrastructure, either providing high-capacity intercity links to Service Providers, or providing local access to Customer sites. Finally, as has been the case for many years, Service Providers and network operators rely on equipment suppliers’ element management systems to make it possible to manipulate the network to create Customer services, and to report meaningful information about the status of elements, in order that the network, and ultimately, Customer services, can be maintained at required levels of quality.

NMF members work together to determine how their processes should be linked, and how they should work with Customers’ and suppliers’ systems. They work together because they believe that to solve this type of problem alone requires extensive time and money (which is better spent developing new offerings) and that proprietary solutions will not be accepted in the marketplace. They work together because they see that in these areas, there is no competitive advantage to be gained over exchanging data in a proprietary fashion, and that, conversely, they can considerably reduce both the time to development and the risk of making bad decisions.

The definitions contained in this document pertain to the measurement of service quality, from a customer’s point of view. NMF members have determined that a common, industry-wide approach to service measurement is needed in order to effectively manage Service Level Agreements. They invite others to review these definitions, and to adopt them in their performance reporting processes. They invite suppliers to become familiar with these definitions, and to help determine how best to apply specific performance data to achieve the types of overall service measurements defined herein.

# 1	executive summary

In early 1994, the NMF identified a need to determine what service providers require in the way of common industry interfaces to support process automation. Without this information, computing and equipment suppliers felt incapable of understanding what they would be called upon to deliver in their management systems and computing platforms. A team called Requirements Capture (RECAP) was formed and began interviewing service providers. It soon became clear that agreements on specific interfaces would be impossible to consider unless all involved service providers had a common way of viewing service management processes. The output of the RECAP activity was a document called “A Service Management Business Process Model” [ref 1].

The completion of this document marked the end of the "RECAP" project and the beginning of multiple interface requirement-specific projects within the NMF under a work group called SMART. SMART, which stands for Service Management Automation and Re-engineering Team, is using [ref 1] as a starting point.

The SMART process defines three major outputs: the Requirements Document, the Information Model and the NMF Solution Set, as illustrated in figure 1/1.

Figure 1/1:  The SMART deliverables

During the requirements gathering process, the SMART Performance Reporting Team detected that there are only few (or even no) stable and unambiguous definitions dealing with service performance at the service management level. Service Providers and Customers have different views and understanding of terms such as  “Service”, “Service Availability”, and “Service Access Point” (SAP). It became clear that describing SAP concepts, service concepts, definitions of Service Level Agreements (SLAs), etc., would go far beyond the scope of a Requirements Document. As a result of this, the SMART Performance Reporting Team decided to develop a document dealing with terms, definitions and concepts entitled NMF Performance Reporting Definitions Document. The aims of this Definitions Document are to:

- serve as additional information to the requirements document

- define terms used by Service Providers and their Customers when negotiating SLA parameters

Feedback from an interview process at the senior business level of service providers in North America, Europe and Asia, as well as SMART Performance Reporting Team members experience show that “Service Availability” is the key parameter that customers are interested in! Although many terms exist to describe network and element availability, the term has widely different application at the service level. This leads to misunderstandings and dissatisfaction of the customer. For example, if the customer believes that "availability" means their application is running without a problem, and the service provider uses the same term to mean that the service is working (even if impaired), then a mismatch in expectations is inevitable. Similarly, if a service provider relies on multiple sub-contracting providers for pieces of a service, the lack of common performance terms makes it virtually impossible to construct a picture of end-to-end performance.

This document concentrates on the definition of “Service Availability” and the application of it to different services. Although “Service Availability” is the key performance parameter in today’s telecommunications market, Customers and Service Providers would like to measure, calculate and report on further service performance parameters. In response to that need, this document provides some high level information regarding the definition and calculation of further service performance parameters. Future releases of this document will concentrate on these parameters, too.

The SMART Performance Reporting Team has developed both, a definition for “Service Availability” and a corresponding calculation formula.  Many service providers may have the feeling that a standardisation of “Service Availability” (and its calculation) will affect their competitive play-ground. The Team believes that the developed definition still leaves enough room for free competition.

# 2	introduction to this document

## 2.1	Scope

This document is aimed at clarifying all of the definitions used in the NMF Service Provider to Customer Requirements Specification [ref  2] produced by the SMART Performance Reporting Team. It also provides background information on the applications of many of these terms.

## 2.2	Document Structure and Outline

After the Executive Summary and this Introduction, the document covers the following terms and areas:

Section 3: Service Definition

This section provides the definition of the term SERVICE as a concept of layered SERVICE ELEMENTS. Furthermore general statements on SERVICE AVAILABILITY and SERVICE ACCESS POINTS are found.

Section 4: Service Level Agreements

This section provides a definition of the term SERVICE LEVEL AGREEMENT and its role in the evolving marketplace.

Section 5: Service Availability

Section 5 is the key section of this document. It provides a structured approach to define the term SERVICE AVAILABILITY and introduces the SERVICE ACCESS POINT concept.

Section 6: Performance Reporting

This section provides guidelines for content and formats of Performance Reports.

Section 7: Conclusions

This section contains conclusions.

Annex A: Examples

Annex A contains examples on the application of Service Availability to different network services.

Appendix 1: Glossary of terms, acronyms and abbreviations

Appendix 2: Referenced and related documents

## 2.3	Key terms used in this document

- Customer

- Customer to Service Provider Interface

- Performance Report

- Quality of Service

- Service and Service Element

- Service Access Point

- Service Availability

- Service Level Agreement

- Service Provider

Please refer to Appendix 1 for more details regarding the above mentioned key terms

# 3	service definition

## 3.1	Service and Service Element

In today's telecommunications environment, service provision increasingly involves knitting together components sourced within the Service Provider organisation, together with those bought-in from other Service Providers or Network Providers. The concept of a layered transport network architecture is now becoming well established, and is represented in standards such as ITU-T recommendations G.803 and G.805. Similarly, a layered service concept can be used as a basis for defining “services” and service performance such as Service Availability (SA) offered by a Service Provider to a Service User at one or more Service Access Points (SAPs) sometimes called a SAP Group. Figure 3/1 illustrates this concept.

Figure 3/1:  Layered Service Concept

In a layered service provision environment as illustrated in Figure 3/1, a Customer purchases a service from Service Provider 1, through a SLA or service contract which contains commitments on Quality of Service (QoS) and against which performance parameters such as Service Availability is reported for a SAP Group. In order to provide this service, Service Provider 1 brings together a number of Service Elements (SEs) analogous to the construction of a Network from Network Elements, links and paths. Thus, a “service” can be considered as the combination of a set of SEs co-operating in an appropriate fashion to support the service.

Each Service Element may be developed from network or service resources at the disposal of the Service Provider concerned (Service Provider 1 in Figure 3/1), or may be provided using services obtained (bought-in) from other Service Providers (e.g. Service Provider 2 in Figure 3/1).  In order to support the commitments on QoS and SA provided to the Customer within the SLA, corresponding commitments are required from the Service Providers contributing Service Elements used to construct the service concerned. The “integrating” Service Provider (Service Provider 1 in Figure 3/1) must then ensure that the commitments negotiated concerning the Service Elements are sufficient to support the commitments made to the Customer. This will require that both the nature of the commitments can be related, (in the simplest case, that the commitments are of the same kind), and the values of the QoS thresholds and of the related Service Availability for the Service Elements are adequate to ensure that the SLA commitments to the Customer are in fact achievable.

## 3.2	Service Availability

Service Availability (SA) represents a measure of the fraction of time during a defined period when the service provided is deemed to be better than a defined QoS threshold.  SA is measured in the context of a SLA between the Service Provider and the Customer concerned. The SLA will define the basis on which the QoS threshold which separates available from unavailable conditions is determined, and the measurement period over which SA is assessed.

Note that the extent and nature of the QoS problem which triggers an individual period of service unavailability will have an impact on the inconvenience for the Customer, and this will typically influence the contribution which this unavailability is judged to make to assessment of the overall SA for the measurement period concerned. This indicates a slight difference in approach from the assessment of path availability from a network or equipment-oriented perspective, where this sort of comparative judgement is typically not used. However, as experience is gained in relating network path unavailability to service availability with also the use of intelligent network equipment (e.g. self-healing SDH/SONET transport rings), comparative judgement may also be used at the network level. For example, performance parameters such as delay may be affected by path re-routing, even though path availability may be unaffected.

## 3.3	Network Topology - Containment and Management Domains

As alluded to above, the network topology and means of constructing an overall service is increasingly a layered architecture. This leads to the concept of containment and management domains. One or more parts of the network supporting the service may be in another Service Provider’s jurisdiction than the Service Provider providing the service to the Customer. Similarly the service itself is comprised of a number of SEs, each of which in turn may be a service comprised of a number of SEs. Indeed, in some cases, the service to the Customer may be provided by a Service Provider who does not have any physical network access to the Customer, but contracts with a local Service Provider who does.

For an integrated approach to service management, information transfer on service performance against the individual SLAs needs to be transferred from one Service Provider to another. In a Telecommunications Management Network (TMN) world, this is accomplished automatically across an “X-interface” between two Operations Systems (OSs) in the connecting Service Providers. In most situations today, this is still accomplished manually via telephone and/or fax, but increasingly in the future some form of “electronic bonding” will be achieved.

## 3.4	Service Access Points and Customer Premises Equipment

An integral part of defining and managing service performance is the adequate definition of what is a Service Access Point (SAP). In addition to the basic technical specification of the SAP such as performance parameters for which service has been contracted in the SLA, there are market and physical network-related aspects that have to be specified. These are as much about regulatory and equipment ownership issues as they are about physical network configuration itself.

For any End User telecommunications service, the basic network topology supporting that service comprises some kind of Customer Premises Equipment (CPE) connected to a network Serving Node (SN) over some Access Network (AN) technology. The simplest form of this is Plain Old Telephone Service (POTS) with a telephone set connected to a Serving Exchange (Central Office) over a copper access line. A more sophisticated example is a range of services provided over an ATM transport network via an ATM access switch which is a piece of CPE as shown in Figure 3/2.

Figure 3.2 ATM/SDH/PDH Network

The actual User Network Interface (UNI), and hence the SAP, between the End User and the Service Provider, depends on the ownership of the CPE. In one case, for example a fully “managed service”, the CPE is owned and maintained by the Service Provider. Here the SAP is on the customer side of the CPE. In a second case, for example a leased line service, the CPE may be owned and maintained by the End User or by a third party. In this situation, the SAP is effectively on the network side of the CPE. A third case, for example a Network Node Interface (NNI) between two SPs, also needs to be considered which is a SAP between two Service Providers where one network operator is effectively a carrier service provider for the other.

## 3.5	Regulatory Issues

The ownership of the CPE is often a regulatory issue. In some countries, the Service Provider is not allowed to own and operate electronic equipment on the customer premises. In other countries, where the network operator is still a monopoly, all the CPE may still be provided, owned and operated by the Service Provider. This regulatory situation is changing with network and service liberalisation taking place across the world.  Competitive service provision is becoming the norm and all types of situations described above exist. Thus the SAP may be defined in terms of End User service parameters related to the actual service or it may be defined purely in terms of network path performance parameters supporting a service or range of services.

# 4	service level agreement

## 4.1	What is a Service Level Agreement  (SLA)?

A Service Level Agreement is a formal negotiated agreement between two parties. It is a contract that exists (in our case) between the Service Provider and the Customer. It is designed to create a common understanding about services, priorities, responsibilities, etc.

SLAs can cover many aspects of the relationship between the Customer and the Service Provider such as performance of services, customer care, billing, provisioning etc. Performance Reporting uses the SLA as the reference and does not address the other parameters known to exist, but which are not part of the SLA.

## 4.2	The role of SLAs in the evolving marketplace

As competition is progressively introduced into all service provision markets, and customers become increasingly discerning, Service Providers are realising the need to differentiate their products through value added services. Additionally industry deregulation is transforming a traditionally monopolistic marketplace into an extremely competitive one. One prime mechanism to combat this is the provision of an ‘off the shelf’ SLA document that clearly sets out the obligations of both the Service Provider and the Customer.

SLAs, also known as Service Level Guarantees, are an excellent tool for Customer and Service Provider management.  A well crafted SLA sets, and manages, expectations for all elements of the service to which it refers. It assists the Service Provider to force operational change, improve internal measurement and reporting, assess trends (from the improved reporting), improve customer relations and provides a vehicle for potential differentiation from their natural competitors.

## 4.3	List of typical SLA parameters

An SLA should contain a number of objective, measurable, parameters which the Service Provider guarantees to their Customers. These parameters may be reported against in order to provide proof to the Customer that Service Providers are meeting their commitments. Typically SLAs include statements about:

- system/service availability

- time to identify the cause of a customer-reported malfunction

- time to repair a customer-reported malfunction

- provisioning-related time

- Quality of Service (QoS) targets

## 4.4	Focus on Quality of Service within SLAs

Quality of Service (QoS)

The QoS value provides an overall view as to how close the service as offered (actual) is to the service as contracted. The overall quality of service, as delivered to the Customer, consists of two major parts:

- operational criteria (related to the performance of an organisation)

- service specific performance criteria (related to the performance of a service provider’s service platform. These criteria are typically those that are fundamental to the operation of the service).

Figure 4/1:  Criteria contributing to the Quality of Service

Figure 4/1 provides an abstract view of the main criteria contributing to the quality of an offered service. Each of the criteria associated with a given service would be individually tracked as part of the SLA. The criteria chosen as contributors to the QoS are seen as being service specific. These criteria are typically those that are fundamental to the operation of the service. Further work is required to establish whether it is possible to define shopping lists of criteria to be associated with the various classes of service offered.

It may be possible to algorithmically combine the chosen criteria into a single quality of service value intended to provide an overall view as to how close the service offered is to the contracted service. The amount that each of the criteria contributes to the overall QoS would be defined by a weighting value which is agreed with the customer and specified in the SLA.

## 4.5	Grade of service

A very important issue is to determine wheter a fault or malfunction is causing:

- no service outage (= service fully available)

- a partial service outage (= service degraded available)

- a complete service outage (= service fully unavailable)

It will be quite difficult for Customers and Service Providers to specify the “gray zone” of degraded service in the SLA. Section 5.2 and Annex A5 of this document provide some detailed information on how to approach this issue.

## 4.6	Need for performance measurement in support of SLAs

There are many well developed documents and tools addressing Performance Reporting for numerous network elements. The Team did not found any standards that directly apply to the end-to-end service the Customer has purchased. The user is not capable of (nor interested in) summarisation of all of the individual Network Element performance numbers.  Where several networks support the service, the aggregation of all of the individual Network Element degradation reports is not practical. Therefore more direct measures of the user (end-to-end) service appears to be required for performance reports. No standards were located that address this issue directly.

The issue of performance measurements is further discussed in section 5.3 of this document.

## 4.7	Priorities for SLA commitments

Figure 4/1 illustrates the two main categories of QoS related SLA parameters. It was clearly stated by both, Customers and Service Providers that “Service Availability” is the key QoS parameter, to be supported by SLAs.

According to figure 4/1, further SLA parameters may include:

- Operational parameters

- Time to Restore a specific service (e.g. at a specific SAP),

- Mean Time to Restore a service (e.g. for a specific SAP/SAP group),

- Mean Time Between Failure (e.g. for a specific SAP/SAP group)

- Service specific parameters

- a digital non switched service such as ATM PVC might include Network Cell delay, Errored Cell Ratio, etc.

- a switched analogue service such as voice might include Call Establishment Time, Network Delay, End-to-End Distortion, etc.

- a Private Line/Leased Line service might include Bit Error Rate, Unavailable Seconds, Errored Seconds, Severely Errored Seconds, etc.

- a Frame Relay PVC service might include Cyclic Redundancy Check Errors, Committed Information Rate Quality, Discarded Frames, Network Delay, etc.

# 5	service availability Definition and
Calculation formula

## 5.1	Introduction

Extensive research, based on an interview process at the senior business level of service providers in North America, Europe and Asia shows the need for the development of a common understanding of service performance and the standardisation of service performance definitions  [ref 3 ].

Feedback from the above mentioned interview process as well as Team members’ experience show that “Service Availability” is the key parameter that customers are interested in! Although many terms exist to describe network and element availability, the term has widely different application at the service level. This leads to misunderstandings and dissatisfaction of the customer. For example, if the customer believes that "availability" means their application is running without a problem, and the service provider uses the same term to mean that the service is working (even if impaired), then a mismatch in expectations is inevitable. Similarly, if a service provider relies on multiple sub-contracting providers for pieces of a service, the lack of common performance terms makes it virtually impossible to construct a picture of end-to-end performance.

## 5.2	Service Availability

### 5.2.1	Basic Service Availability Calculation Formula

As already outlined in section 3 of this document, the telecommunications industry is moving toward solutions that allow Customers to use service packages to support their communication needs. These packages are built on Service Elements, ranging from the lower layers up to the application layers, combined in a vertical (e.g. IP over Frame Relay) and/or horizontal way (e.g. IP and Voice over ATM).

Consequently there is a need to define a methodology to measure all the features provided through a service package, in order to evaluate the Service Availability. The Service Availability (SA) is expressed as a percentage (SA%) to indicate the time during which the contracted service (e.g. SVCs, PVCs, end-to-end circuits incl. protocols, applications, etc.) at the respective Service Access Points (SAPs) is operational. Operational means that the customer has the ability to use the service as specified in the SLA.

An event affecting the service at the SAP, can be defined as an outage. The duration of this specific event is the outage interval. Ordinarily this concept is used for the Unavailability (UA%) and Service Availability percentage (SA%) calculation as follows :

|  | SA% = 100% - UA% |  |
| --- | --- | --- |

Figure 5/1:  Basic Service Availability Calculation Formula

Where Unavailability UA% is is defined as:

Figure 5/2:  Basic Service Unavailability CalculationFormula

### 5.2.2	Service Degradation Factor (SDF)

A major issue is to determine whether an event affecting the service at the SAP is causing a complete service outage (service fully unavailable) or a partial service outage (service degraded available).

In order to address this issue, a Service Degradation Factor (SDF) can be used in the calculation as follows :

Figure 5/3:  The use of the Service Degradation Factor

Where: 0  SDF  1

A list of SDF values with the corresponding event type can be defined in the SLA. This definition procedure characterises the service in the SLA and can be stated according to the Customer’s business need.

It is very important to state an agreement on the time stamps information source related to the event that causes the outage interval.

A list of SDFs is provided as an example in table 5/1 below:

| Service Degraded Factor Agreed Value | Event Type | Duration Source (Time stamps information) |
| --- | --- | --- |
| 1 | Service Fully Unavailable | System 1 or System 2 |
| 0.8 | Outage Type A | System 3 |
| 0.6 | Outage Type B | System 4 or ... |
| 0.5 | Outage Type C | System 5 |
| ... | ... | ... |
| 0 | Service Considered Available | (Customer happy with it) |

Table 5/1: SDF examples

Table 5/2 presents the relationship between Billing, Availability, and Degraded Performance.  SDF enables the definition and orderly application of measures for degraded performance.

| Billing | Full Billing | Acceptable Performance | Available |
| --- | --- | --- | --- |
|  | Conditional Billing | Degraded Performance |  |
| No | No | Unacceptable Performance | Unavailable |
| Billing | Billing | No Performance |  |

Table 5/2: Relationship of SDF (shaded section), Billing and Availability

### 5.2.3	The Service Access Point (SAP) Concept

The SMART Performance Reporting Team defines that a service is “delivered” to the customer at the so-called Service Access Point (SAP). The service provider’s responsibility therefore includes all parts (and sub-parts) of the service(s) between all contracted SAPs.

Figure 5/4:  The Service Access Point Concept

Figure 5/3 illustrates a quite simple implementation of a service. However, the service provider’s responsibility includes already at least CPE equipment, access lines (e.g. in different countries) and the service provider’s backbone network/service platform.

In case of a pure PL-LL (Private Line - Leased Line) service, figure 5/3 can be simplified to a single line connecting two customer sites.

A more complex service could include a PVC based Frame Relay implementation delivered over an ISDN access with an ATM backbone using SDH for the transport.

There are many well developed tools addressing performance issues for numerous network elements. However, an end-to-end view of performance will not necessarily result from a simple summarisation of all of the individual network element equipment performance numbers. The Performance Reporting approach described herein is specifically oriented to report end-to-end performance.

The SAP is a concept used to distinguish the boundary between the Customer domain and the Service Provider domain. A contracted service is delivered by the Service Provider to the SAP. Each service contains at least one SAP and a SAP may only be associated with one service. If a customer detects a problem with a service, he reports the problem to the service provider in terms of a faulty SAP.

Table 5/3 provides the essential SAP related components of the Performance Reporting data inputs are:

| Component | Description | Source |
| --- | --- | --- |
| SAP | A Service Access Point (SAP) is a logical element located on the interface between the Customer’s domain and the Service Provider’s domain and it represents the point to which a service is delivered. A SAP can then be weighted in accordance with a business critical factor which is defined in the SLA | derived from Ordering or a SLA common object |
| SAP Group | SAP Group represents a group of SAPs against which Service Availability must be reported by Performance Reporting. Each SAP normally belongs to one (or more than one) SAP Group and each SAP Group contains at least one SAP. The association between SAP Groups and SAPs is defined within the associated SLA, according to the required computation and aggregation format. It is noted that some explicit or implicit SLAs may not require that all SAPs are reported against in this way | derived from Ordering or a SLA common object |
| Reporting Period | Reporting Period represents the period at which Performance Reports are generated. It is defined independently for each SAP Group within the associated SLA | derived from Ordering or from a SLA common object |
| SAP Activity Interval | SAP Activity Interval represents the time interval of a specific active period (i.e. when the Customer requires service from the SAP) within the defined Reporting Period. It must be measured separately for each active period of each SAP within a SAP Group | derived from Ordering or a SLA common object |
| SAP Activity Time | SAP Activity Time represents the total time of all SAP Activity Intervals of a specific SAP within the defined Reporting Period | derived from Ordering or a SLA common object |
| SAP Outage Interval | SAP Outage Interval represents the time interval of a specific outage period within the defined Reporting Period. An outage period is a period of service unavailability (say, due to fault) which occurs during a SAP Activity Interval for a given SAP, and which is not due to causes excluded by the associated SLA (e.g. force major). It must be measured for each outage period of each SAP within a SAP Group |  |
| SAP Outage Time | SAP Outage Time represents the total time of all SAP Outage Intervals of a specific SAP within the defined Reporting Period | derived from either TTS and/or NMS (exclusion conditions from a SLA common object) |
| SAP Weight | SAP Weight represents a value or relative priority attached to each SAP, and influences the relative significance placed on that SAP by the Service Provider (and eventually the Customer). It is defined independently for each SAP within the associated SLA | derived from Ordering (?) or from a SLA common object |

Table 5/3:  SAP related terms

### 5.2.4	The Service Access Point Weight in the Service Availability Calculation Formula

Each SAP may have a weight attached to it indicating the importance of it to the customer. A service may consist of many SAPs which may not all be weighted equally.

E.g. a service consisting of a server (host) site connected to multiple client sites contains a certain number of SAPs: one for the server and one for each client. In this example the weight of the server site may be higher than the one of the clients, indicating that a problem with the server SAP has a more significant implication on the service.

If the Customer’s business need requires a weighting of SAPs, the UA% formula can be extended with a SAP weighting factor as follows:

Figure 5/5:  The Use of the Service Access Point Weighting

## 5.3	Information Source (Using of TT and/or Network Information)

One of the main issues with the Service Availability calculation is to select the appropriate information source for all the required formula elements:

- Outage Interval;

- Activity Time;

- SAP Weight;

- SDF

While SAP Weight and SDF come from the SLA common data, event driven information such as Outage Interval and Activity Time can be obtained from Network Element Managers (NEMs) and/or Trouble Ticket Administration Systems (TTSs). A Service Provider will need both systems to manage the services provided to its Customers.

Usually these two type of systems are completely unrelated. Therefore it is likely that choosing one instead of the other leads to different capabilities for Service Availability calculations and Performance Reports.

A certain type of service can be mapped onto the ISO/OSI layer model. Typical examples could include, on one end, the Service Provider to Network Provider SLA,  where a Service Provider buys transmission facilities and capacity (trunks) to build its own network. On the other end, the Customer to Service Provider SLA, where the Customer outsourced its intercompany communication needs to a Service Provider.

In the first case the service layer maps at the transmission level, in the second one at the application level.

When dealing with lower level services (according to the ISO/OSI layer model) the underlying technology drives the choice: the data source for the Service Availability calculation is obviously the Network Management System and the parameters to take into account depend on the specific service (PL-LL, ATM, Frame Relay etc.) as further developed in the Annex A of this document.

In the upper layer scenario it is not possible to refer to a specific technology, since the Customer is not interested in how the Service Provider delivers the service as long it is available.

As a guiding criteria it can be assumed that the final goal of Performance Report is to state the provided level of service as close as possible to the Customer’s perception.

A useful approach could be to use the Network Management System as a monitoring system to detect failures and initiate the creation of a related trouble ticket in the TTS. However, after the fault is cleared, the Service Provider has to check with the Customer that the service is operational again and also to agree on the service restoring time. This can be done both, via electronic bonding between the Customer’s and the Service Provider’s Customer Care Systems or traditional communications between Customer and Service Provider representatives.

In this case a report produced out of the TTS can provide information close to Customer’s perception,  while raw data from NEMs can be still used to produce technical reports about reliability, MTBF, etc.

## 5.4	Service Availability  Value

The relevant Service Availability value (SA%) is contract dependent and has to be specified in the SLA. The SA% value might be influenced by:

- type of service (e.g. PL-LL, PVC, SVC)

- type of transport protocol used (e.g. X.25, Frame Relay, ATM, etc.)

- network/service configuration (e.g. single point of failure vs. redundancy, reliable network components, etc.)

- Service Provider’s policy

- etc.

It is out of scope of this document to provide any “typical” SA% values.

# 6	performance reporting

## 6.1	Reporting Frequency

Reporting Frequency is the periodicity with which Performance Reports are delivered from the Service Provider to the Customer. Different possible periodicities include:

- synchronised with the billing cycle

- on demand

- weekly

- monthly

The time span covered within a Performance Report is called the reporting period. It has clearly to be defined and stated in the report.

## 6.2	Formats and Contents for Reports

Basically, two types of report are covered with Performance Reporting:

- QoS reports

- Traffic Data reports

The QoS report provides overall assessment of service quality according to the SLA parameters. The Traffic Data report provides usage measurement information for the service.

Information is provided in the following format using text, graphs and/or tables:

- summary information

- trends

- exceptions

- comparisons

- snapshots

### 6.2.1 Common Report Header

All reports should have a common report header providing the following information:

a) Customer Information

- Customer Name (e.g. name of the entity on the billing record)

- Customer ID (an internally assigned identifier for the customer)

- Customer Contact Name (name of the customer contact for performance reporting matters; eventually organisational unit)

- Customer Contact Information (address, phone numbers, e-mail, etc. for the customer contact person.

b) Service Provider Information

- Service Provider Name (name of the service provider for the Managed Object Instance

- Service Provider Contact Name (the contact person or organisation of the service provider for the Managed Object Instance)

- Service Provider Contact Information (address, phone numbers, e-mail, etc. for the service provider contact name)

c) Service Information

- Managed Object Instance (a unique identifier for a customer service. This identifier is a key to request performance data and outage information)

- Service Type Code (an enumerated code which identifies a service type (e.g. FR PVC, CR PVC, DS-1, E-1, etc.)

- Service Descriptions ID (an identifier which points to a textual descriptions of the service)

- Service Profile Descriptions (descriptions of configuration parameters and corresponding values)

- SAP Information (SAP’s address, SAP’s weight, etc.)

d) Report Information

- Report Type (QoS report or Traffic Data report)

- Reporting Period (start and end time of the reported interval)

- Boundary Conditions (information on addressed boundary conditions; e.g. where outages cross reporting interval boundaries)

- Suspect Flag (the flag is set if the report is suspected to contain unreliable or incomplete data)

e) SLA Information (in case of a QoS report)

- contracted/guaranteed SLA value(s) (e.g. SA%, Bit Error Rate, Cell Loss Ratio, etc.)

### 6.2.2	QoS Report

QoS reports will contain:

- service independent measurements

- service specific measurements

a) Service Independent Measurements

As already outlined earlier, the key measurement is Service Availability. Further service independent measurements could include:

- Total number of SAP outage intervals

- Time to Restore for a specific SAP

- Exceeding Committed Time-to-Restore. If the SLA specifies a Committed Time-to-Restore value for any outage, this measurement provides a percentage of outages where service was  not restored within committed time.

- Mean Time to Restore for a specific SAP/SAP Group

- Mean Time Between Failure for a specific SAP/SAP Group

- etc.

This list should be seen as information only. It is out of scope of the SMART Performance Reporting team to standardise the above mentioned parameters in the Customer to Service Provider contractual (SLA) environment. However, by using the proposed Service Availability calculation formula, Service Providers will have all information available to perform the respective calculations.

### 6.2.3	Service Specific Measurements

a) Leased Line Service

Performance measures for a PL-LL service may include:

- Bit Error Rate (BER)

- Unavailable Seconds (UAS)

- Severely Errored Second Ratio (SES)

- Errored Second Ratio (%ES)

- Severely Errored Second Ratio (%SES)

- Background Block Error Ratio (BBER)

- ...

b) ATM Cell Relay PVC Service

Performance measures for a ATM service may include:

- Network Delay

- Errored Cell Ratio

- ...

c) Frame Relay PVC Service

Performance measures for a Frame Relay PVC service may include:

- Network Delay

- CIR Quality

- CRC Errors

- Discarded Frames

- ....

d) Analogue SVC Service (e.g. voice)

Performance measures for an analogue SVC service may include:

- Call Establishment Time

- Network Delay

- End-to-end Distortion

- ...

### 6.2.4	Traffic Data Reports

Traffic Data Reports are used by Customers to determine how they are using the contracted services. This allows the Customer (and the Service Provider) to plan for future services based on traffic patterns, and to verify if they over/under utilising the services they are currently subscribing to.

a) Leased Line Service

Traffic Data reports are not applicable to PL-LL services.

b) ATM Cell Relay PVC Service

Traffic Data reports for ATM services may include:

- Cells Transmitted in either direction

- Cells Transmitted with CLP = 0 in either direction

- Cells Received in either direction

- Cells Received with CLP = 0 in either direction

- Cells Discarded in either direction

- Cells Discarded with CLP = 0 in either direction

- Average Utilisation: This is the utilisation of the PVC based upon the traffic flowing for a period time divided by the Sustainable Information Rate (SIR).  Traffic is defined as number of cells multiplied with 53 bytes per cell. This is an average for the time period determined by the service provider in the granularity of the report. This is available for each direction of a PVC.

| Utilisation = | Bytes Transferred ---------------------------- SIR |  |
| --- | --- | --- |

- Peak Utilisation = is the peak average utilisation of the PVC and the time it occurred for a fine granularity determined by the service provider during the reporting period. This peak may be an hourly peak over one month. This is available for each direction of a PVC.

c) Frame Relay PVC Service

Traffic Data reports for Frame Relay PVC services may include:

- Frames Transmitted - in either direction

- Frames Transmitted with DE = 0 - in either direction

- Frames Received - in either direction

- Frames Received with DE = 0 - in either direction

- Frames Discarded - in either direction

- Frames Discarded with DE = 0 - in either direction

- Bytes Transmitted - in either direction

- Bytes Received - in either direction

- Average Utilisation = this is the utilisation of the PVC based upon the traffic flowing for a period time divided by the Committed Information Rate (CIR). Traffic is defined as number of bytes transmitted. This is an average for the time period determined by the service provider in the granularity of the report. This is available for each direction of a PVC.

| Utilisation = | Bytes Transferred ---------------------------- CIR |  |
| --- | --- | --- |

- Peak Utilisation = is the peak average utilisation of the PVC and the time it occurred for a fine granularity determined by the service provider during the reporting period.  This peak may be an hourly peak over one month. This is available for each direction of a PVC.

# 7	conclusions

The SMART Performance Reporting Team started its work early in 1995 with the development of a document called SMART PERFORMANCE REPORTING WHITE PAPER. The document was a mix of business and technical requirements as well as performance related standards.

During the developments, the team detected that there are no common definitions of terms to be used at the service level. Although many terms exist to describe network and element availability, for example, the term has widely different application at the service level. Even terms like SERVICE, SLA, SAP, etc. are used differently by different Service Providers. The Team did not found any standards that directly apply to the end-to-end service the Customer has purchased (please refer to Appendix D of [ref 2]).

NMF members have determined that a common, industry-wide approach to service measurement is needed in order to effectively manage Service Level Agreements. The definitions contained in this document are mainly based on Team members own experience. It is therefore essential that a broad audience within and outside the NMF reviews these definitions and

- provide feedback to the NMF SMART Performance Reporting Team (all)

- adopt them in their performance reporting processes (Customers, Service Providers)

- adopt them in their developments of performance reporting tools (ISVs)

- adopt them in their developments of  network components and management systems (Network Component Suppliers)

Although this release of this document concentrates on “Service Availability”, the  SMART Performance Reporting Team welcomes also feedback to other service performance parameters. This will be the needed input for further developments of this document.

# annex a - Examples

Introduction

The Performance Report Definition document develops generic equations for performance with a focus on measurement of Service Availability. The advantages of a generic equation include: simplicity in understanding the basic concepts and thus providing a much needed similarity between measures for different implementations.  This will ease the task of comparing various performance reports.

Annex A contains measurement techniques specific to certain services identified in the Requirements document including: PVC, SVC and PL-LL.  Each Annex section will refer to the Requirements document and will provide as close to an implementable interpretation of the equations as possible.  This annex will allow the reader to better understand the generic equation and will also demonstrate the advantage of having a fundamental approach for all services.

Any values provided in this Annex are for illustrative purposes only and are not intended to represent agreement nor recommendations.  Values should only be determined by the two parties through an appropriate SLA.

This introductory section provides a placeholder for the Annex A sections to follow. This annex is not exhaustive and is expected to grow as more applications are developed.

CONTENTS

| Annex | Basic Service | SAP Interface | Measure | Source Input |  |
| --- | --- | --- | --- | --- | --- |
| A1 | PL-LL | T-n, E-n, OC-n, STM-n | Unavailable & Degraded Service | NE PM data |  |
| A2 | PVC | SDLC | Availability | TT system |  |
| A3 | PL-LL | Digital Lines | Availability | TT system |  |
| A4 | PVC | Frame Relay | Availability | TT system |  |
| A5 | SVC | LAN Connectivity | Availability | TT system |  |
| A6 | Note to the Service Availability calculation when using Trouble Tickets |  |  |  | TT system |

Table A/1:  Content of Annex A

This Annex A will be further developed in future releases of this document.

## ANNEX A1 - Service Availability for PL-LL circuits

A1.1 References

PRR: Performance Reporting Requirements document

PRD: Performance Reporting Definitions document

PRR Figure 3/3 provides for Network Element (NE) performance data as input to the Performance and Trouble Ticketing functions.  This annex describes the value of NE input to the performance reporting process.  This input also provides a mechanism to measure degraded performance for PL-LL circuits.  Degraded performance is difficult to measure with a trouble ticket mechanism.

A1.2	PL-LL Application

The main thrust of the referenced documents is directed toward end-to-end Service Availability.  This annex is directed toward the application of the equation in PRD section 5.2 to PL-LL circuits.  T1M1.3/96-017R5 demonstrates availability of path (end-to-end), line, near end and far end measures for T1/E1, T3/E3, and SONET.  Service Availability (outage) and Service Availability (degraded) can be achieved by application of the equations of PRD 5.2.  This approach is applicable for end-to-end paths (as well as links) that are within the domain of the same service provider such as the access links shown in PRD figure 5/3.  Therefore this annex would apply for customers who contract for end-to-end service or for separate links to build their own network as well as when one SP is the buyer of links from another SP.

Existing technology for transmission Network Elements provides for measures of Unavailable Seconds, Severely Errored Seconds, and Errored Seconds.  SDH provides equivalent measures based on errored blocks and frames for near end and far end.  Table A1.1 shows the correlation between the SA parameter and the basic measure from the NE.

| SA parameter | Basic Measure | Definition |
| --- | --- | --- |
| Outage | UAS | UnAvailable Seconds |
| Degraded Service | ES SES | Errored Seconds Severely Errored Seconds |

Table A1/1:  Measures used for availability and degraded service

This annex applies to PL-LL applications and will be driven by the data retrieved from NEs.  Service Availability will be automatically produced without necessarily involving manual Trouble Ticket procedures. Definition and measurement of degraded service is a distinct feature of this technique.  When the SA for degraded service falls below a given threshold, a Trouble Ticket may be opened automatically, thereby initiating maintenance procedures earlier than would have occurred considering the normal delay associated with the manual process of opening a Trouble Ticket. Trouble Ticket descriptions (class or attributes) may need further development to accommodate “degraded service” status as shown in PRD table 5/2.

A1.3	General Equation for PL-LL

Figure A1.1 is an adaptation of the Service Availability equation found in section 2.6.2 of the Performance Report Requirements document.

The original equation is general in scope and may be implemented with several sets of inputs.  The general equation defines basic events and duration.  Two parameters from PRD table  5/3 have been included: w - weight or importance and SDF - Service Degradation Factor.

SA%=[1−i=1NDtUASiwiSDF1+i=1N(Dt)SESiwiSDF2+i=1N(Dt)ESiwiSDF3j=1MDtTotalSecondsjwj]x100

Figure A1/1: General  Service  Availability Formula for PL-LL

Unavailable Seconds (UAS) define down time. Severely Errored Seconds (SES) and Errored Seconds (ES) may, in some cases, be considered as service available or service unavailable, therefore the value of SDF may be assigned for the application by the SLA or by the SP’s standard practices.

NOTES:

1. 	SDF values:

SDF2 and SDF3 may default to 0 for the most optimistic definition of Service Availability.

SDF2 and SDF3 may default to 1 for the most pessimistic definition of Service Availability.

SDF1,  SDF2 and SDF3 may be assigned a value between 0 and 1 to build a specific definition of Service Availability.

The values of SDF1, SDF2 and SDF3 are independent of each other.

2. The equation may be applied in several ways.  One technique will provide a calculation of SA “unavailable” primarily using UAS (SDF1=1, SDF2 and SDF3 = 0). Another technique will provide a calculation of SA “degraded” primarily using ES and SES (SDF1=0, SDF2 and SDF3= non zero).

3.  SONET and other transmission technologies also derive an equivalent measure of UAS, SES and ES.  This allows several PL-LL technologies to utilise the general equation with consistent results.

4.  There are a number of other parameters indicative of service degradation such as BPVs and CVs.  Any of these measures could be considered for inclusion in the above equation.  The equation therefore has no fundamental limit to three or four terms.  From a practical consideration, there is a limit to the ability to extract (and communicate economically) large amounts of MIB information from a large number of remote NEs.  There is also the consideration that not all NEs will support all MIB attributes, therefore a common (small) set of measures may offer the most beneficial implementation.  Therefore this Annex describes the most common set:  UAS, SES and ES.  Additional attributes could be added to the equation.

A1.4	Unavailable Service

Unavailable i.e. ‘out of service’ may be determined by substituting Unavailable Seconds for the duration found in 2.6.2. The primary application of the UA measure is to achieve a summary over a period of time. This measure of SA may provide validation for the TT management process. Thresholds may be established to generate alerts about the (accumulated) down time. This process may be used for opening trouble tickets, however urgent ‘out of service’ alarms are usually generated without the delay associated with waiting for a threshold to be exceeded.

Since UAS is determined within the NE automatically, this measure of SA will record short outages that may be missed by a trouble ticket system.  Summarisation of all outages is essential to an accurate measure of SA. (SDF and weight have been eliminated from the equation below for clarity, but should be considered in an implementation).

SA%=1−UASTotalSecondsx100

Figure A1/2 Unavailable Service Availability Formula

A1.5	Degraded Service

“How well did it perform when it was available?” is answered by measuring Degraded Service. Degraded Service is usually difficult to detect or measure with a Trouble Ticket system.  Therefore a measure of Degraded Service is essential to confirm conformance to a SA requirement (i.e. in an SLA). While the service may still be available, but is approaching or below a SLA performance threshold, a Degraded Service alert may be quite useful to generate an input to a Trouble Ticket system.  TotalSeconds minus UAS equals TotalAvailableSeconds. The SDF will be applicable to ES and SES. (SDF and weight have been eliminated from the equation below for clarity, but should be included in an implementation).

SA%=1−ES+SESTotalAvailableSecondsx100

Figure A1/3: Degraded Service Availability Formula

## ANNEX A2 - Service Availability for PVC/SDLC Service

The provided Service to a Customer consists of a SDLC connection via Protocol Emulation on a X.25 network.

Figure A2/1: Service Availability for PVC/SDLC Service

The data network is in the Service Provider’s domain. The Service is delivered at both ends of the given SDLC connection on two V.24 physical interfaces between the CPE (Customer Premises Equipment belonging to the Service Provider’s domain) and, respectively, the customer's Cluster Control Device (CC) and the Customer operated Front End Processor (FEP).

In this case an Integrated  Management System (IMS) can collect and correlate data from leased lines, PADs, X.25 nodes, etc. and detect failures or service degradation for each active SDLC connection.

Using data from the above mentioned IMS, the Service Provider can produce a very accurate Service Availability report. However, when this report is delivered to the Customer, many differences can be found comparing the IMS report with the data coming from the Customer's service management platform.

The reason for this mismatch is due to the different behaviours of the Data Network (Service Provider domain) and Teleprocessing Line Control.(Customer domain). E.g.a connection failure, several minutes long, can make the Front End Processor delay or suspend Cluster Control polling, until a manual or automatic Line Activation command is performed.

In this case, if the Service Provider relies on the IMS’ historical data to measure the delivered Quality of Service, some failures may have a different perception given that the Customer sees the service operational when his own system restores the SDLC connection.

The suggested solution is to use the IMS as a monitoring system to detect failures and initiate the creation of a related trouble ticket in the TT administration system. Moreover, when the  fault is cleared, the Service Provider has to check with the Customer that the service is operational again and also to agree on the service restoring time (this can be done both via electronic bonding between the Customer’s and the Service Provider’s Customer Care Systems or traditional communication between Customer and Service Provider representative).

In this case a report produced out of the Trouble Ticketing Administration System can provide information much closer to the Customer’s perception, achieving the main goal of the Performance Reporting Process, while raw data from Network Element Management Systems can be still used to produce technical bound reports about reliability, MTBF, etc.

## ANNEX A3 - Service Availability for PL-LL circuits / Trouble Ticket  based

Example 1

A single line connecting two Customer sites has two SAPs. Each SAP, in this case mapped physically with the DCE, represents the boundary between the Customer Domain and the Service Provider Domain.

Figure A3/1: Service Availability for PL-LL circuits / Trouble Ticket  based (Example 1)

Parameters:

Reporting Period

T = 30 days = 720 h = 43200 minutes

SAP Activity Time

For each SAP the Service Provider is responsible form 6.00 am to 6.00 pm for 30 days = 12 h a day for 30 days = 21’600 minutes

SAP Weighting

The SAP weight has no meaning in this case since the service is symmetrically provided at both sites (default SAP(a) weight = SAP(b) weight=1)

SDF

Not applied (default =1)

Suppose we have a fault (e.g. line down) on the 13th day from 4.30 pm to 8.30 pm. We could state that this has provided an outage at each SAP of 4 h. Since the Service Provider is responsible only from  6.00 am to 6.00 pm the amount of outage caused at each SAP to the Customer is 1h and 30min = 90 minutes.

Calculating the Service Availability according to the formula outlined in section 5:

SA = [ 1 - (1x90x1+1x90x1) / (1x21600+1x21600) ] x 100%

SA = [ 1 - 90/21600 ] x 100%

SA =  99.58%

This is equivalent considering in the general availability formula:

SA = [1 - (outage time) / ( Service Provider responsibility time)] x 100%

The Service Availability is calculated considering all the agreed (Service Provider & Customer) Trouble Ticket Time Stamps at the Service Access Points.

We can suppose that the Trouble Ticket System has an interface with other Service Monitoring Systems that provide Time Stamps and related information to Service Outage or Degradation.

Example 2

As a further step we add a new leased line to the previous example according with the figure below.

Figure A3/2: Service Availability for PL-LL circuits / Trouble Ticket  based (Example 2)

Parameters:

Reporting Period

T = 30 days = 720 h = 43200 minutes

SAP Activity Time

For SAP(a) and SAP(b)  the Service Provider is responsible 24h a day for 30 days = 43’200 minutes

For SAP(c) and SAP(d)  the Service Provider is responsible form 6.00 am to 6.00 pm for 30 days = 12 h a day for 30 days = 21’600 minutes

SAP Weighting

The SAP weight can be used to differentiate the importance of a single SAP to the Customer’s business:

SAP(a) weight = SAP(b) weight=1and SAP(c) weight = SAP(d) weight=2

SDF

Not applied (As default =1)

Suppose we have a fault (e.g. line down) between SAP(c) and SAP(d) on the 13th day from 4.30 pm to 6  pm. We could state that this has caused an outage at SAP(c) and SAP(d) of 1h and 30m = 90 minutes.

Calculating the Service Availability for all SAPs according to the formula outlined in section 5:

SA = [ 1 - (1x0x1+1x0x1+2x90x1+2x90x1) / (1x21600+1x21600+2x43200+2x43200) ] x 100%

SA = [ 1 - 360 / 216000 ] x 100%

SA = 99.82 %

We can calculate the Service Availability also for group of  SAPs if required. For example considering only SAP(a) and SAP(d) we could calculate the Service Availability at one Customer’s site. Considering SAP(a) and SAP(b) we will get the Service Availability for a single leased line. This gives a wider application range to the Service Availability concept.

## ANNEX A4 - Service Availability for PVC Frame Relay services / Trouble Ticket based

Considering a Frame Relay Service provided on two PVCs (DLCI) as shown in the figure below:

Figure A4/1: Service Availability for PVC Frame Relay services / Trouble Ticket based

In this case the SAPs are mapped on logical link ends.

Parameters:

Reporting Period

T = 30 days = 720 h = 43200 minutes

SAP Activity Time

For all SAPs  the Service Provider is responsible 24h a day for 30 days = 43’200 minutes

SAP Weighting

The SAP weight can be use to differentiate the importance of a single SAP:

SAP(a) weight = SAP(b) weight=1and SAP(c) weight = SAP(d) weight=2

SDF

Not applied (As default =1)

Suppose we have a physical medium fault on the access line connecting SAP(a)-SAP(d)’s site on the 13th day from 4.30 pm to 6  pm. We could state that this has caused an outage at all SAPs of 1h and 30m = 90 minutes.

Calculating the Service Availability for all SAPs according to the formula outlined in section 5:

SA = [ 1 - (1x90x1+1x90x1+2x90x1+2x90x1) / (1x43200+1x43’200+2x43’200+2x43’200) ] x 100%

SA = [ 1 - 540 / 259’200 ] x 100%		SA = 99.79 %

As before we can calculate the Service Availability also for groups of  SAPs if required.

## ANNEX A5 - Service Availability for SVC/LAN connectivity / Trouble Ticket based

Considering the LAN Connectivity Service provided through whatever Service Platform suitable for the LAN protocol at the Customer sites.

Figure A5/1: Service Availability for SVC/LAN connectivity / Trouble Ticket based

In this case the SAPs are mapped at the LAN interface.

Parameters:

Reporting Period

T = 30 days = 720 h = 43200 minutes

SAP Activity Time

For all SAPs  the Service Provider is responsible 24h a day for 30 days = 43’200 minutes

SAP Weighting

The SAP weight can be use to differentiate the importance of a single SAP to the Customer’s business: SAP(a) weight =3 and SAP(b) weight=2 = SAP(c) weight=1

SDF

·	Service degraded at the SAP (all other SAPs  reachable but with less bandwidth) implies SDF = 0.3

·	SAP partially isolated (not all SAPs are reachable) SDF = 0.5

·	SAP isolated implies SDF = 1

Suppose we have a logical link fault between SAP(a) and SAP(b) on the 13th day from 4.30 pm to 6  pm. Due to the fact that SAP(a) can be connected to SAP(b) through the site where SAP(c) is located, we could state that this has caused a service degradation at all SAPs (SDF=0.3)

Calculating the Service Availability for all SAPs according to the formula outlined in section 5:

SA = [ 1 - (3x90x0.3+2x90x0.3+1x90x0.3) / (3x43’200+2x43’200+1x43’200) ] x 100%

SA = [ 1 - 180 / 259’200 ] x 100%

SA = 99.93 %

As before we can calculate the Service Availability also for groups of  SAPs if required.

A further example could be the calculation of the Service Availability for all SAPs in the case when SAP(a) cannot be connected to SAP(b) through the site where SAP(c) is located.

## ANNEX A6 - Note to the Service Availability calculation when using Trouble Tickets

The definition of the Service Availability formula (section 5) makes certain assumptions when using Trouble Ticket information as input for the calculation.

- The formula is valid if SAP outage interval does not extend beyond the reporting periods (Dt).

- Individual Service Providers must devise a mechanism to address boundary conditions e.g. where SAP outage intervals cross reporting period (Dt) boundaries. An example of a potential solution to this issue is noted below:

Service Availability calculation exceptions handling (proposal)

Obviously ‘1’ introduces a potential error in the SA determination which is directly related to the number and relative duration of the outage intervals that may be excluded.

As a possible solution to handle this inaccuracy, a new attribute can be added to SA definition in the SLA: the Maximum Estimated Error. A simple way to evaluate Estimated Error could be based on the (excluded SAP outage intervals)/(Total SAP outage intervals) ratio.

These calculations and definitions are supported by the following  illustration:

Figure A6/1: Time Concept for Service Availability Calculation

Definitions for figure A6/1 are as follows:

| Term | Definition |
| --- | --- |
| Dtcumulative | Dtcumulative represents the aggregation of more than one reporting period |
| Dtperiodic | Dtperiodic represents one specific reporting period |
| SAP’s life | SAP’s life represents the total life time of a specific SAP as stated in the contract |
| SAP Activity Interval | SAP Activity Interval represents the time interval of a specific active period (i.e. when the Customer requires service from the SAP) within the defined Reporting Period. It must be measured separately for each active period of each SAP within a SAP Group |
| TT’s life | TT’s life represents the total life time of a specific Trouble Ticket |
| SAP Activity within TT’s life | SAP Activity within TT’s life represents the total time of all SAP Activity Intervals of a specific SAP within the life of a corresponding Trouble Ticket |
| No SP liability interval | No SP liability interval represents the time interval of a specific outage period, which is outside the Service Provider’s responsibility (Customer Responsibility Outage or Force Majeur) |
| SAP Outage intervals | SAP Outage Interval represents the time interval of a specific outage period within the defined Reporting Period. An outage period is a period of service unavailability (say, due to fault) which occurs during a SAP Activity Interval for a given SAP, and which is not due to causes excluded by the associated SLA (e.g. force majeure). It must be measured for each outage period of each SAP within a SAP Group. |

Table A6/1:  Definitions to time concept

# appendix 1 - glossary of terms, acronyms and abbreviations

## 1.1	The use of terms in this document

Service Provider

In this document, the term SERVICE PROVIDER refers to companies who provide telecommunications services as a business.  Service providers may operate networks, or they may simply integrate the services of other providers (who operate networks) in order to deliver a total service to their customers.

Providing telecommunications service to any one end-customer may involve multiple service providers, where one provider may "sub-contract" with other providers to fulfil the customer's needs.  When necessary to account for this relationship, the term service provider is used in this document to describe the company responsible for providing service to an end-customer. The term Other Provider is used to denote companies which have a sub-contractual responsibility.

In Performance Reporting the role of a Service Provider is to provide the Performance Report to his Customer.

Customer

In this document, the term CUSTOMER refers to companies who make use of telecommunications services provided by a service provider. The customer is the ultimate buyer of a network service.

It may be that a customer is also an organisation who provides services to their customers. It is confirmed that the current work of the SMART Performance Reporting Team also includes service providers in the role of customers, where the nature of the SLA means that these service providers behave equivalently to customers.

The term “customer” is equal to the term "end-customer" also used in this, and in referenced [NMF] documents.

In Performance Reporting the role of a Customer is to get the Performance Report from his Service Provider.

Customer to Service Provider Interface

In this document the term CUSTOMER TO SERVICE PROVIDER INTERFACE is used to identify the boundary of information exchange according to [ref 1].

A Simple Business Reference Model

Service Level Agreement

A Service Level Agreement is a formal negotiated agreement between two parties. It is a contract that exists (in our case) between the Service Provider and the Customer. It is designed to create a common understanding about services, priorities, responsibilities, etc.

SLAs can cover many aspects of the relationship between the customer and the service provider such as performance of services, customer care, billing, provisioning etc. Performance Reporting uses the SLA as the reference and does not address the other parameters known to exist, but which are not part of the SLA.

Service and Service Element

In today's telecommunications environment, service provision increasingly involves knitting together components sourced within the Service Provider organisation, together with those bought-in from other Service Providers or Network Providers. The concept of a layered transport network architecture is now becoming well established, and is represented in standards such as ITU-T Recs. G.803 and G.805. Similarly, a layered service concept can be used as a basis for defining “services” and service performance such as Service Availability (SA) offered by a Service Provider to a Service User at one or more Service Access Points (SAPs)

Service Access Point (SAP)

A Service Access Point (SAP) is a logical element located on the interface between Customer’s domain and the Service Provider’s domain and it represents the point to which a service is delivered. A SAP can then be weighted in accordance with a business critical factor which is defined in the SLA.

Service Availability (SA)

SERVICE AVAILABILITY (SA) represents a measure of the fraction of time during a defined period when the service provided is deemed to be better than a defined QoS threshold.  SA is measured in the context of a SLA between the Service Provider and the Customer concerned. It is expressed as a percentage (SA%) to indicate the time during which the contracted service (e.g. SVCs, PVCs, end-to-end circuits incl. protocols, applications, etc.) at the respective Service Access Points (SAPs) is operational. Operational means that the customer has the ability to use the service as specified in the SLA.

Performance Report

In this document, the term PERFORMANCE REPORT refers to a “document” providing information on how a contracted service was:

- delivered by the Service Provider in terms of SLA fulfilment

- used by the Customer in terms of capacity, traffic volumes, etc.

Quality of Service (QoS)

The QoS value provides an overall view as to how close the service offered is, to the contracted service. The overall quality of service, delivered to the customer, consists of two major parts:

- operational criteria (related to the performance of an organisation)

- service specific performance criteria (related to the performance of a service provider’s service platform. These criteria are typically those that are fundamental to the operation of the service.

## 1.2	Abbreviations

AN	Access Network

ATM	Asynchronous Transfer Mode

CC	Cluster Control (Device)

CIR	Committed Information Rate

CPE	Customer Premises Equipment

CR	Cell Relay

CRC	Cyclic Redundancy Check

DLCI	Data Link Connection Identifier

FEP	Front End Processor

FTP	File Transfer Protocol

FR	Frame Relay

HTML	HyperText Markup Language

IMS	Integrated Management System

MTBF	Mean Time Between Failure

MTTR	Mean Time To Repair

NM	Network Management

NMF	Network Management Forum

NML	Network Management Layer

NMS	Network Management System

NNI	Network Node Interface

OS	Operations System

OSS	Operational Support System

PR	Performance Report(ing)

QoS	Quality of Service

SAP	Service Access Point

SDH	Synchronous Digital Hirarchy

SDLC	Synchronous Data Link Control

SE	Service Element

SLA	Service Level Agreement

SMART	Service Management Automation and Re-engineering Team

SN	Serving Node

SONET	Synchronous Optical NETwork

TMN	Telecommunications Management Network

TT	Trouble Ticket(ing)

TTS	Trouble Ticket System

TTR	Time To Repair

# appendix 2 - referenced and related documents

Referenced documents

| [ref 1] | A Service Management Business Process Model, NMF, 1995 |
| --- | --- |
| [ref 2] | NMF Service Provider to Customer Performance Reporting Requirements Specification, NMF, 1997 |
| [ref 3] | Network Performance Survey Results Report, Jeff Paschke, International Network Services, June 1995 |

Note: References to standards are found in Appendix D of [ref 2]

Related documents

| 1 | Implementing OMNIPoint, A safer, faster path to improved service management, NMF, 1995 |
| --- | --- |
| 2 | Statement of User Requirements for Managing a Networked Information System, NMF User Advisory Council, 1992 |
| 3 | The Lean Communications Provider, NMF, McGraw Hill, 1996 |