TstHost version 1.43B                                      1

TstHost version 1.43B

---------------------

Features

--------

TSTHOST   is  a software  written for  TNC2 and  clones with   HOST

mode  EPROM type TF8, TF23,  TF24 etc. It  is possible   to use the

software with other TNCs set in KISS mode.

A  TNC in KISS  mode requires  a serial  driver (like TFPCX/TFPCR),

but  don't  forget to  set  the  TNC in  KISS  mode  BEFORE running

TSTHost.

A  TNC  in  WA8DED  Host  mode requires  a  serial    driver   like

COMBIOS,  MBBIOS  etc.  The  GKJBIOS    driver    is    an   MBBIOS

compatible   driver  optimised for   TSTHost,  that   detects   the

special    16550A  UART.   For  the    driver    configuration, run

GKBIOCFG.EXE and follow the instructions displayed on the screen.

TSTHOST  offers from  2 to  8 independent communication   channels,

where   every  channel has  a 300 lines  receive buffer,   circular

keyboard   buffer  to recall  the ten previous  commands, automatic

text  justification, up  to    8  simultaneous  YAPP  (with  resume

and    yappC options)  or  ASCII file  transfers,   up  to    8 PMS

channels  with  RLI-type forward with  a master  BBS. All  received

data  can  be  saved   in  a   disk file. Every  session may have a

different callsign.

There   is  a  separated window for monitoring,  and it is possible

to capture in a file all the traffic displayed in that window.

The  text entered from  the keyboard  is recognised  as commands to

run  for every COMMAND mode session, while  in CONVERSATION mode is

sent  to   the   connected   station.   For this  reason it  is not

possible  to switch in  conversation mode if  the session   is  not

connected.  The exception   is    the  Monitor   window,  where all

keyboard data  is transmitted as Unproto packets.

For   every session   there is  a receive buffer,  useful to review

all  the received data.  By receiving data  from   a   channel  not

actually  selected, the  related callsign  in the status  line will

blink.   All  unconnected  channels  are  set  by  default  to  PMS

sessions.  So, when  an user  connects your  station, he  goes into

the    personal mailbox  system,  but  if YOU  connect  a  user the

channel  is set   for  a normal  conversation mode.  Naturally, you

have  a specific command to switch  a PMS session into conversation

mode  and vice-versa. During a  PMS session, the user  can read and

write  messages, view the users directory,  up or download programs

TstHost version 1.43B                                      2

with  the YAPP  protocol. The   PMS session  can also  run external

server   programs,  send   a  Break   to other PMS  users, talk the

sysop.

Incoming  connection  in a  PMS  windows, may  be  managed  from an

external filtering program, called CON_FILT.EXE

TSTHost   automatically  recognises  the standard  password format,

and  the  extended  MD2  algorithm.  The   standard password format

accepts  from    one to    ten  letters,   but  normally    all the

programs require five letters for the password.

Other  features are the Unproto list capability  and a full forward

protocol  MBL-RLI  or FBB  COMPRESSED  type, even  if  the  the BBS

require a password for the access.

The  program can also  extract any 7plus  code from  the unproto or

pms   messages,  and  write  this  code  to   a  file,  for  furter

processing.  TstHost support remote SERVER technique and PG program

to increase the pms command for particoular applications.

At  the end,  is supported  also the  Macro  Commands, that  may be

active for any channels, without restriction.

The  communication in  HOST-mode   between  the PC  and the TNC has

a  3 seconds  timeout. After  this time,  if  the program  does not

receive  a reply from the  TNC, attempts to  synchronise the serial

port.

TstHost version 1.43B                                      3

Tsthost with TNC in KISS or BAYCOM modem  (/K)

----------------------------------------------

You   need  the driver  TFPCR or  TFKISS for  KISS mode,  TFPCX for

modem  BAYCOM.  TFPCX version 2.10 supports  both the KISS and  the

BAYCOM modes.

Using  a KISS-mode   TNC,  set the TNC  in KISS  mode manually   or

with   the    appropriate  command     line    switch     of    the

TFPCR/TFPCX/TFKISS driver.

TFPCX  uses  the IRQ  253  for the  internal  communications,   the

other drivers uses the IRQ 254.

Load the driver

Load TSTHost with the command:  TSTHOST /T /I254 (or /I253)

/T  sets TSTHost  to   use  the   TFPCX/R  driver,  and  /I254 sets

the IRQ for the driver communication.

NOTE:  when started,  TSTHost automatically  sends the  commands to

the driver:  JHOST1, USERS NbrOfChannels, @U0

Example for TFPCR or TFKISS:

TFKISS command line parameters

TSTHOST /T /I254

Example for TFPCX:

TFPCX command line parameters

TSTHOST /T /I253

WARNING:   I  have noticed   that TFPCR   version  3.30  does   not

work  properly with high speed transfers over  400  cps (G3RUH 9600

baud).  I don't  know the  reason, but version  2.10 of  TFKISS and

TFPCX  does not have this bug.

The sound and BAYCOM modem  (/Sx)

---------------------------------

Some  baycom drivers, like tfpcx, speed up  the sound timer used by

tsthost  to generate  delay and  sound temporization: the  resul is

that  the  sound  is  unaudible  or  distort.  To  avoid this,  the

parameters  /SX may be added  to the tsthost  startup command line,

where X is a value between 1 and 1000.

TstHost version 1.43B                                      4

Pratically,  this  is  a delay  multiplier  factor,  a  sound  of 1

seconds,  if the /S500 is  present, will became  lenght 500 secods!

THIS  is one of the  reasons for which this  parameters MUST NOT BE

USED  if  not  needed.  Do not  use  this  parameters  with  tnc in

hostmode,  kissmode, or generally, if the  sound is correct without

it.  General paramters, may be  /S200 for baycom at  1200 baud, and

/400 for baycom at 2400 baud, you may change to your preference.

Tsthost with the standard HOST mode  (/H)

-----------------------------------------

Configure  GKJBIOS  or  other  COMBIOS  drivers  for  the   correct

serial port parameters. Load the driver in memory

Load TSTHost with the command TSTHOST /H /Cx /Bzzzz

where   /H selects the software  to use the   standard host mode, x

is  the  serial  port  where  you   have connected  the TNC (1..8),

zzzz  is  the  baud  rate of  the  port.  TSTHost  supports  a data

transfer rate of 9600, 19200 and 38400 baud.

NOTE:  when started,  TSTHost automatically  sends the  commands to

the TNC:  JHOST1, USERS NbrOfChannels, Z0, @U0

At  the end   of  the   program, you  must  remove from  memory the

serial driver with the command:  GKJBIOS U. For instance:

GKJBIOS

TSTHOST /H /C1 /B19200

GKJBIOS U

NOTE:  You may replace  the gkjbios or  any type  of combios driver

with  the FOSSIL type  driver, the  program recognize automatically

the driver and use it.

Tsthost with DRSI driver  (/D)

------------------------------

This  interface permit to use  tsthost with RSI  driver that return

the  data in the wa8ded host compatible mode, laki DEDHOST provided

with BPQ.

- Configure your drsi driver for your work condition

- Load TstHost with the command TSTHOST /D /Ixx, where:

/D    tell tsthost to use a drsi driver

/Ixx  XX is the communication vector used by the driver, and must

be given in DECIMAL mode.

NOTE:  when started,  TSTHost automatically  sends the  commands to

the driver:  JHOST1, USERS NbrOfChannels, @U0

TstHost version 1.43B                                      5

Tsthost with 2 or more tnc

--------------------------

You   may connect   two or   more  interfaces to  the program. This

can be done  with the TFPCX v2.10 driver. This  driver supports  up

to  8 interfaces  like  KISS,  BAYCOM,  MODEM   etc. This is only a

small  description,  because  the  complete  documentation  of  the

commands  is shipped with the driver files.

Since   the driver  may support   up to  8 port,   the driver  have

a  command  to define  which  channels assign  to  which  port. The

extended  host command is @PO xxxxxxxx. Each 'x' is a value between

0  and the  number   of   installed   interfaces.   0 is  the first

interface,  1 the second, etc. TSTHost uses  8 channels so you must

define 8 digits.

Of  course,  if you   use the   driver for    one   interface only,

the  @PO  command  is  not  requested,  because  all  channels  are

assigned  to that interface.

Here are some examples:

1)   We have two TNCs,   connected to   COM1 and  COM2.   The  TNCs

are  already  set  in KISS mode,  the baud  rate is  19200 baud, we

want  to assign  channels 1234   to the TNC  on COM1, channels 5678

to the TNC on COM2.

TFPCX -PKISS1 -PKISS2 -B19200:19200 -CH8

TSTHOST /T /I253

To assign the channels type the command:   PARAM 0 @PO 00001111

(This command can also be written in the tsthost.cfg file)

2)    We  have 2  TNC  and  a BAYCOM  modem.  The  BAYCOM  modem is

connected to COM1,  1200 baud; a TNC is on COM2 at 19200 bauds, the

second  TNC is  on  COM3,  IRQ10, 38400   baud. The   two TNCs  are

already  set in KISS  mode. We to  want assign channels  123 to the

BAYCOM, 456 to the first TNC, and 78 to the second TNC.

TFPCX -PCOM1 -PKISS2 -PKISS3:03E8:10 -B1200:19200:9600 -CH8

TSTHOST /T /I253

To assign the channels, type the command PARAM 0 @PO 00011122

(This command can be executed from the tsthost.cfg file)

3)   A  TNC in  KISS mode, already  set in   KISS  mode, on   COM1,

19200 baud.

TFPCX -PKISS1 -B19200 -CH8

TSTHOST /T /I253

In this case the command @PO is not requested.

TstHost version 1.43B                                      6

TstHost and diskette usage, parameter /ND

-----------------------------------------

Even  if  tsthost is  thiked  for an  hard  disk usage,  it  may be

installed  in floppy  disk system,  with naturally  the limitations

due  to the little capacity of that  supports. TstHost every minute

check  for the free disk space, and if  the space go down under the

500  Kbytes free,  emit a  warning message  to video.  In dischette

sistem,  probably you  never arrive  to have more  than 500  Kb, so

every  minute  yuo  receive the  warning  message....  this  may be

eliminated  with the command line switch /ND, that instruct tsthost

to don't check for disk space.

NOTE:  tsthost only check for disk space, no actions occours.... is

your   responsability  to  maintain  sufficent  space  for  tsthost

operations.

Memory usage

------------

TstHost  use the  memory dinamically.  This  means that  the memory

will  be request to the  system when needed, and  released when not

more  requested. But, some type  of data, like  revision buffer and

channel structure, cannot be released.

Actually tsthost  need about 500 Kbyte of memory to run, this for a

standard  use, and all pc that have  installed almost 640 kbyte may

run tsthost. But others memory usage may be needed:

1) TFPCX-TFPCR, if used, need about 64K

2) Unproto list, (little mode), require 24K

3)  RUN  command,  and generally  any  other  application  like the

SERVER  that require a shell  to operating system,  need memory for

command.com and the program to execute.

4)  Compressed data received  with forward and  uproto list require

other 19K.

Expanded memory and /NOEMS command

----------------------------------

TstHost  detect and  use EMS  memory if  found  in the  sistem. EMS

memory may free up to 240Kbyte of conventional memory.

If  you   have  any  problem  using the  expanded  memory,  you can

disable  the  function with  the  command line  switch  /NOEMS. For

example:                      TSTHOST /H /B19200 /C1 /NOEMS

TstHost version 1.43B                                      7

Extended memory and /NOXMS command

----------------------------------

TstHost  detect and  use XMS  memory if  found  in the  sistem. XMS

memory  may  free up  to 210Kbyte  of  conventional memory,  and is

accessed  via the HYMEM.SYS driver. (or  equivalent). Note that XMS

memory management is slow referred to EMS and conventional memory.

TstHost  priority are: first  fill in EMS,  next if  needed in XMS,

next if needed in conventional memory.

Command  line  parameters  /NOXMS  tell  thsthost  to  not use  XMS

memory, for example:          TSTHOST /H /B19200 /C1 /NOXMS

Reduction of RX buffers, /Rsize command

---------------------------------------

If  you do not have memory for  all your applications, you may free

memory  reducing the  size of  the rx  buffer. Normally  any buffer

have  a capacity  of 300  lines. The  /Rsize  permit to  define the

number  of line allocated  in the review  buffer, from  100 to 750.

Any line cost 82 bytes.

For example, with TSTHOST .... /R200, you may free:

(300-200)x82x9=73800 byte, 73K

This  command do not  affect buffer  allocated in  EMS memory, that

have always 300 line.

Number of channels, /K parameters

---------------------------------

This  parameter is useful to reduce the  numeber of channel used in

tsthost. Application  may be in old eprom that not manage more of 4

channel,  or  to reduce  the memory  usage.  Is possible  to define

from2  to 8 channel, 8  default: for example, to  set up 4 channel,

add the parameters /K4 to the tsthost command line.

Unproto list size, /U parameters

--------------------------------

Tsthost  always  attempt  to  set  the  unproto  list  size to  800

messages.  If you have memory  occupation problems, this parameters

is  useful to reduce the  memory usage: Available  value start from

200  to 800, 800 default: To  set the unproto list  size to at most

400  messages,  add the  parameters  /U400 to  the  tsthost command

line.

TstHost version 1.43B                                      8

VGA mode

--------

Add  the  parameter /VGA  to  the command  line,  to get  43  or 50

screen lines on an EGA or VGA monitor.

Example: TSTHOST /H /B19200 /C3 /VGA

Extended interface for data request, switch /V

----------------------------------------------

Starting  from version 1.41  TstHost will  provide a  new method to

access  particoular information about the status of the program and

the   tasks.  Extended  data  request  will  be  done  trought  the

internally  irq service  vector, normally  101, 65Hex.  This vector

may  be redefined with the  command line parameter  /V, that accept

in input a DECIMAL value. For example:

TSTHOST ... /V100

Tsthost tell tsthost to use irq number 100, 64Hex, instead of 101

NOTE: TSTHOST REQUIRE AN EXCLUSIVE ACCESS TO THIS VECTOR.

Read also the section about TSTHOST.IRQ vector.

Programs,  servers,  pgs,  may interrogate  this  irq  in  order to

obtain  info about general parameters of tsthost  and or tasks. The

info  will be  returned in  a structure here  described, that  is a

static  area into tsthost. For this reason,  if you need to mantain

information  between two or  more calls to  the irq,  you must copy

the  previsuly information in an area of  your program, because the

new irq call will cause the overwrite of the previusly info.

Program that  require info, must be call the irq vector with the AL

register set to 0, AH set to the specified channel to investigate.

When  return  from  the  irq  routine,  if  any  error was  detect,

register  AH is set to not 0 value,  and AL will contain the number

of  channel supported by tsthost.  Normally AL is 8,  but it may be

loss  if tsthost was  start with the  /K switch in  order to reduce

the  number of channels. Actually  the unique error  possible is to

investigate a channel that not exist.

If  success,  register AH  will be  0,  AL will  contain  again the

maximum  number of available channels, and ES:BX  will point to the

info structure.

NEVER,  NEVER AND NEVER  WRITE INTO THIS  AREA. YOU  MAY ONLY READ.

WRITING  INTO  THIS  AREA PROBABLY  WILL  CAUSE  THE  CRASH  OF THE

SYSTEM!

TstHost version 1.43B                                      9

Here   is  the  format   of  the  information   structure.  Like  C

convention, all char array are null terminated.

struct info

{         // THIS VALUE ARE GLOBALS, NOT CHANNEL DEPANDANT.

//===============================================

unsigned char THVH;    //TstHost version, high value

unsigned char THVL;    //TstHost version, low value

int  MAXCHANNEL;       //Number of channel available i tsthost

char DRVTYPE;          //Driver type, 1 real host, 0 tfpc, 2 drsi

char PORT;             //If real host, com port

unsigned int BAUDRATE;//If real host, baudrate

int  INTNO;            //If tfpcx/r, irq vector used by driver

char TstHostCall[10];  //Callsign of the sistem, with ssid

char UListEnable;      //If not 0, unproto list is active

char Wpath[81];        //tsthost WorkDir

char Upath[101];       //tsthost UserDir. If more than one path

//is defined, the multiple path are

//separated by a space.

char HomeBbs[10];      //HomeBbs Callsign

char HomeAlias[10];    //homebbs alias call, null if undefined

// THIS FIELDS ARE CHANNEL DEPANDANT

//====================================================

int  chstatus;         //0 channel is disconnected

//1 standard connection, I have connect

//  another OM

//2 PMS connection, a remote user is

//  connected on my pms

//3 PMS connection, HomeBbs have connect

//  my pms to do forward.

//4 PMS connection, my pms have connect

//  HomeBbs to do forward

//5 UNPROTO connection, i have connect

//  HomeBbs to request unproto mail.

char SuppCall[10];     //If not null, extra callsign for the

//channel (command AX PORT)

char UserCall[10];     //call of the connected station, with ssid

// THIS FIELDS ARE VALID ONLY FOR USER THAT HAVE CONNECT

// MY PMS, chstatus=2 o 3

//=====================================================

char UIname[13];       //User name

long UILastConnTime;   //in sec dal 1970, last connection date

long UILastMsgList;    //in sec, last messaged listed date

long UINbrConn;        //number of connection for this user

long UIThisConnTime;   //in second, this date at connection

unsigned int SysFlag;  //actual SYS flag for the user

};

TstHost version 1.43B                                      10

TSR interface for monitor data, TSRMON command

----------------------------------------------

This  interface permits to an  external program to  manage the data

received  on the  monitor port:  this function is  an open  door to

many  possibility..... an example  may be an  external program that

monitor  a packet  cluster emission,  analize the log,  and perform

the   desidered  actions,  like  hf   shack  frequency  change,  or

other.... there are multiple applications.

The  external  manger may  be a  device  driver, or  a  tsr program

loaded  before  tsthost.  The manager  talk  with  tsthost  with an

interrupt  vector,  and  receive from  tsthost  all  the  data that

tsthost  receive  on the  monitor port:  this  means that  the data

received  by the external  manager will  reflect the  status of the

monitor  set with the AX MONITOR command inside tsthost. The switch

/F and  /T of the ax monitor command are ingnored, even if present,

the  manager will receive always  the original data.  Note that the

external  manager  do  not substitute  the  internal  management of

tsthost, all the native function will continue to work.

The  activation and the link with the external manager is performed

with the command TSRMON, with this sintax:

TSRMON IRQVECT

IrqVect  is  the  irq number,  express  in  DECIMAL  notation, that

tsthost call  to send the monitor data to the manager. To disable a

previusly enable manager, use 0 like vector parameter.

Since  tsthost cannot known how your manager  is construct, or what

interrupt  it may use, is  not possible use by  tsthost any sort of

control.  USE THIS COMMAND WITH CARE, do not set invalid irq vector

or non-existant manager, or probably your pc crash.

Tsthost   call  the  manager  loading  in   the  CPU  register  the

appropriate  paramenters, and next issue the interrupt specified in

the  tsrmon command. The manager,  will receive the  data with this

sintax:

AH:  always 0

AL:  this is the channel number, but since for the moment the

interface support only the monitor port, also this parameters

will be 0.

BX:  This register tell you the type of the packet received. Again,

since only monitor is actually supported, only 3 value are

TstHost version 1.43B                                      11

supported:

4    header without data, like control packet, RR, RNR... and

the connection/disconnection frame.

5    header with data, like UI beacon frame, or I frame. Note

that this is only the header, the effective data will be

received with the next packet.

6    data of the previusly packet type 5.

Type 4 and type 5 are the header exactly like you may see in

the monitor port, already decoded, like:

fm IK1GKJ to BEACON ctl UI^ pid F0

SI:  Tell you how many byte are present in the packet, on the other

words, the size of the packet. The max size that you may

receive are 256 bytes.

CX:DX in the form segment:offset are the address of a memory

location where the data are stored. WARNING, this is a READ

ONLY memory, do not attempt to write anything here, or you

system may crash.

And  now, some little rules  to write a good  interface program for

tsthost:

-  Tsthost is stopped while your program  is in execution, so write

fastest and little code.

-  DO NOT USE  THE TSTHOST STACK,  use you dedicated  stack in your

program.  The  tsthost  stack  is  sized  for  tsthost usage,  your

program may saturate it.

-  DO NOT USE dynamic allocation in the  far heap, you may lock the

tsthost  memory management. If  you need dinamic  memory inside you

program, allocate a zone at the starting time and use it.

- DO  NOT ALTER the tsthost register. Before modify, save it on the

stack or other area, and restore them before return to tsthost.

-  And, to  finish, DO NOT  WRITE ANYTHING  IN THE  AREA POINTED BY

CX:DX.

TstHost version 1.43B                                      12

Short cut keys

--------------

ALT F1             Automatic transmission of the password.

ALT F2             Displays the MHEARD list

(depending on the driver/TNC software)

ALT F3             Enables the monitor traffic to IUSC

ALT F4             Disables the monitor traffic

ALT F5             Set monitor traffic to U

ALT F6             Like the command ULIST L

ALT F7             Enable the file selection for file DIR

ALT F8             Display the message in the pms list

ALT F10            Turn off the new mail indicator.

CONTROL ESC or F10 Switches command<>conversation mode.

PAGE UP            Activates review buffer, or if already active

displays the previous page.

PAGE DOWN          Displays next page in review buffer

SHIFT UP           One line up in review mode

SHIFT DOWN         One line down in review mode

CONTROL PAGE DOWN  Quit from review mode

CONTROL HOME       Top of review buffer

CONTROL END        Bottom of review buffer

ENTER              In command mode: executes the command.

In conversation mode: transmits the text.

In both mode, if the revision buffer is active,

it will be disabled.

CURSOR UP          Recalls the previous line from kbd buffer.

CURSOR DOWN        Recalls the next line from the kbd buffer.

F1...F8            Switches to session 1 to 8

F9                 Switches to the Monitor window

CTRL-F1..F10       User defined.

SHIFT-F1..F10      User defined.

CTRL_BACKSPACE     Clears from cursor to the previous word.

CTRL_LEFT          Moves the cursor one word to the left

CTRL_RIGHT         Moves the cursor one word to the right

HOME               Moves the cursor at the beginning of the line

END                Moves the cursor at the end of the line

BACKSPACE          Deletes char to the left of the cursor

DELETE             Deletes char under the cursor

INSERT             Inserts mode toggle

ESCAPE             Clears the current line

CTRL-Y             Deletes the current line (internal ).

TstHost version 1.43B                                      13

Path of the files

-----------------

There  isn't a particular  directory for  the TstHost installation,

but  you MUST create,  within that  directory, three subdirectories

with the names: MAIL, PG, USERS.

This can be a standard installation:

\TSTHOST         <DIR>

|-PG           <DIR>

|-MAIL         <DIR>

|-USERS        <DIR>

|-UPLOAD    <DIR>

|-TSTHOST.EXE

|-TSTHOST.DOC

|-TSTHOST.PSW  File for automatic password

|-TSTHOST.CFG  Configuration file for the program

|-TSTHOST.ENT  Input text when connect the pms

|-???????.ENT  Selective text when connect the pms

|-TSTHOST.HLP  Help file for pms

|-TSTHOST.SYS  Password file for remote pms access.

|-TSTHOST.PGS  List/description of the available PG server

|-TSTHOST.USR  User database, managed by tsthost

|-TSTHOST.IRQ  Irq identification, managed by tsthost

|-???????.ULS  Unproto list file, created/used by TSTHOST

|-???????.CON  Connection file for Homebbs, created by the user

|-TSTHOST.HBB  Automatic routing file

|-TSTHOST.SHD  Tnc configuration file when shutdown program

|-TSTHOST.CRN  Command file executed every hours.

|-TSTHOST.IN   Mail import file

|-TSTHOST.LOG  created from TSTHOST if LOG=ON

|-TSTHOST.BID  Bid file, managed by TSTHOST

|-TSTHOST.EXT  File estension definition, created by the user

|-TSTHOST.MAC  Macro definition file, created by the user

|-TSTHOST.DMS  PMS messages database, created/used by TSTHOST

|-TSTHOST.HRT  HROUTE database file, managed by tsthost.

|-DBFMSG.OLD   Old messages database, created/used by TSTHOST

|-Files for source command (if you create source files)

Detailed description of files and directory:

PG - Server directory

You  must place the PMS server in this directory. If this directory

does not exist, the PMS PG command does not work.

The next section of the manual describes the PG Server.

TstHost version 1.43B                                      14

MAIL - Messagges directory

The  bulletins/personal   messages  of the  PMS  will be  stored in

this  directory. If this path  does not exist,  the electronic mail

section of the PMS does not work properly.

USERS - Users directory

This  is the directory   that  the   users  access with   the W, YU

YD  commands. This  is the   only   path   that can point  to other

parts  of your   hard  disk.  If  not otherwise  specified, TSTHost

uses  the path  USERS from the  directory   where   tsthost.exe  is

loaded.    You   can  specify any  other  drive/directory  with the

USERDIR command.

UPLOAD - Users upload directory

This  is the directory where will be  stored any file uploaded by a

pms  users  with  the  YU  command.  If  you  do  not  create  this

directory, the YU PMS command will not work.

WORKDIR - Standard work directory

The  default value  is the  directory where tsthost.exe  is loaded,

but  you  can select  any  other drive/directory  with  the WORKDIR

command. This  path is used for the WRITEBUF, RECORD,  SEND,  YPUT,

YREC  and  MAILCOPY  commands.  With  these  commands,  unless  the

filename does not contain a   specified  drive  and/or  path,   the

file   will  be  accessed  in  the  WORKDIR   path.  For  instance,

by   executing  the  command WRITEBUF  TEXT,  the  contents of  the

review  buffer will  be written  in  a  file   called TEXT   in the

directory that   you have specified with  the WORKDIR  command, but

WRITEBUF  E:\ASC\TEXT will write the buffer  into   a  file  called

TEXT   in E:\ASC.  The purpose of  this command is  to separate the

tsthost files  in  capture, YAPP receive and the other files.

TSTHOST.IRQ - Internal interrupt detection

Due  to the new interface for request  data in tsthost, that use an

irq  vector  to  request  the  info,  program  that  use  this  new

interface must known the vector used by tsthost.

This  file is created when tsthost start,  and deleted when tsthost

ends,  and its unique purpose  is to tell the  program that use the

new  tsthost  interface  about  the  irq  vector  used by  tsthost.

Internally,  this file, that  reside in  the tsthost.exe directiry,

have only this line:

TstHostInterruptVector=XX

TstHost version 1.43B                                      15

Where XX is  a decimal number that is the irq used by tsthost. This

file  is important,  so never  put this  file  READ ONLY  ann never

manipulate the contents written by tsthost.

TSTHOST.USR - Users database

This  sistem file is managed  from tsthost, and are  created in the

tsthost.exe  directory. Actually,  the program  store in  this file

only the name of the user and the date of the last connection.

This is the format:

struct UInfo

{

char call[7];       /* user callsign */

char name[13];      /* user name */

long LastConnTime;  /* in sec. from 1970, last conn date */

long LastMsgList;   /* date of the last message listed */

long NbrConn;       /* number of connection fo this user */

long ThisConnTime;  /* data at connection time */

char PrivDir[31];   /* user private directory */

char reserved[61];  /* riserved */

};                  /* 128 byte every record */

TSTHOST.HBB - Automatic routing file

This  simple  ascii file,  managed  directly from  sysop,  have the

purpose  to assign an  automatic routing for  all locally generated

message  that  do  not have  one.  Messege  that  have  a specified

routing will not be althered.

The  format of the file  is simple: Any invalid  line is ignored. A

valid  line have  the format  CALL @  BBS, where  CALL and  BBS are

callsign WITHOUT ssid.

If there is,  for example, a line like this: IK1MSL @ IK1ZZZ, and i

write  a message with SP  IK1MSL @ IK1ABC, the  message will not be

tuched, but, SP IK1MSL, will be transformed into SP IK1MSL@IK1ZZZ.

TSTHOST.HRT - Hierarchical routing database.

--------------------------------------------

TstHost  support the hierarchical routing  addressing. Any bbs have

a  supplementary address  that provide  a more  powerful addressing

method. An hierachical address may be in the form:

TstHost version 1.43B                                      16

IK1MSL.IPIE.ITA.EU,  where IK1MSL is the bbs callsign, and the rest

are  the hierachical address. For any pms incoming message, tsthost

detect  this address, and store it in  the binary file TSTHOST.HRT.

Next,  when a message  need to be  forwarded to homebbs,  if the h-

address  is known, it will be proposal to  the bbs with the @ route

in order to provide a complete callsign.

Any  h-address may be  volatile or permanent,  where volatile means

that  tsthost may automatically  change (default  value) the hroute

reading  the info  from the  incoming messages on  the pms.  If the

hroute  is fixed, tsthost never  change the route  even detect that

an incoming message have a different hroute for that call.

Normally  you  do  not need  to  manipulate  the  hroute  file, but

tsthost  provide the command  HROUTE (see  later) to  set or change

hroute address for a specified call.

This is the structure of the TSTHOST.HRT file:

struct hroute{

char type;        0=variable, 1=permament;

char call[7];     callsign

char hroute[35];  H address;

long sx;          left node;

long dx;          right node;

} 51 total bytes.

The file is  a binary tree, where the first entry is the radix, and

the  alphabetically lower callsign are in the  left node, the other

in  the right node. Double  key are not allowed.  A variable record

is  automatically  updated from  the information  that  the program

receive  during the reception of  a message. A  permanent record is

fixed, and may be updated only from the keyboard.

TSTHOST.PSW - Automatic password file

This   file contains  the password  for those  systems that require

an access password, like BBS and node sysop.

The  next manual section  describes this aspect  with more details.

If  you  do not  need any  password, or  do  not need  an automatic

password,  do not create  this file. Anu  line starting   with  the

'#' symbol is a remark and is ignored.

TstHost version 1.43B                                      17

TSTHOST.SYS - Password file for PMS SYS command

This file is similar to the tsthost.psw file. The format is:

CALLSIGN_without_ssid VALIDATION_FLAG PASSWORD

every  line  start with    a callsign    without SSID,  a  space, a

validation   flag  and  the  user  password  not  longer  than  255

alphanumeric  characters. Only the users write here  may do the SYS

command. For instance:

IK1GKJ 3 aabbcc

IK1MSL 1 8873mnhbkjAST

Flag is a bit field:

1  means that user may access  with the YD, W,  V to any drive

and path of your system, but may YU only in the UPLOAD path.

2 is like 1, but the user may access to any disk and path also

for write a file (YU)

4 means that the user may do the DOS command.

8  means that the user  may do yapp transfer  even the yapp is

disabled in the pms. (command PYAPP)

16  means that the user may access  to all message in the pms,

exatcly  like if hi is the sysop  on the keyboard. Access including

the editing of the mail header. (EM, FA, FR commands)

32 means that the user may do the shutdowm remote command /E?

In  the example  above, IK1GKJ may  read and  write in  any part of

your  system (1+2=3), IK1MSL instead may read in all your disk, but

write only in the UPLOAD directory.

TSTHOST.CFG - Automatic configuration file

This    is   an    ASCII   text  file,  that  contains  the command

sequence  for  the  automatic   configuration.  This   file will be

searched  by TSTHost  when   started and,  if present,  the program

executes  all  the commands   in  the  file. Every    line   is   a

command.   This   file can contain  any TSTHost command.   Any line

starting with the '#' symbol is   a remark  and  will  be  ignored.

The  line  length  MUST  not exceed 78 characters.

TSTHOST.SHD - Shutdown configuration file

Like   the  tsthost.cfg  file,  this   file  contains  the  command

sequence that is executed at the program shutdown.

TstHost version 1.43B                                      18

???????.ULS - Unproto list file

This    file  is  created and  managed  by  the  program.  When you

activate  the  unproto  list   the  program  reads   this  file, if

present,  so TSTHost   can  handle the message  list from  the last

number  received. When  you   close  the   list  or   shutdown  the

system,  the program  saves the  content of the  list in  this disk

file.  If  this  file   is not found or you  delete it, the program

will  ask to the HomeBBS  the Unproto message  starting from number

one.  The extension filename  is always ".ULS",  while the filename

is the callsign of homebbs without ssid.

TSTHOST.ENT - PMS connection text

This  is an  ASCII   text  file.   When a remote  user connects the

PMS  system,  TSTHost sends  the content of this   text   file  (if

found) to the user before the PMS command prompt.

WARNING:  Do not use the square brackets '[' and ']' in this file,

because they are used for SID forward protocol.

???????.ENT - Selective PMS connection text

????  means a callsign without ssid. When an user connect your psm,

tsthsearch  for  the  existance,  in  the  path  where  are  stored

tsthost.exe,  of that file,  and if found,  it will be  send to the

user   instead  of  the  standard  pms  text  and  tsthost.ent.  Is

poossible  create empty  files: in  this mode  at the  connect time

will be send only the pms prompt.

TSTHOST.HLP - PMS help file

This  is an  ASCII   text file.  When the remote  user connected to

the PMS issues the   HELP  command,  TSTHost  sends  the content of

this  file to the  user, otherwise sends  the message  'NO HELP' if

the file is not present on the system.

TSTHOST.PGS - PMS PG server list

Another  ASCII file.   It can be created by the sysop according  to

his  requests, because  it is  related to  the  PG section.  When a

user  connected  to   the   PMS   issues   the   command PG without

arguments,  TSTHost sends the contents of this file to the user, or

the message  NO PG AVAILABLE if the file does not exists. This file

can be used to list and describe the available PG server programs.

TstHost version 1.43B                                      19

TSTHOST.LOG - Connections log

This  file will be  created and   managed  by   TSTHost   when  the

LOG command  is  ON.  It   contains  the  list  of the  connections

with callsign, date and time.

SOURCE files

These  files are similar   to  the   tsthost.cfg  file,   but  will

be executed  only  when  you   type  the  command.  This is  useful

to  activate specific configurations  on demand.  Any line starting

with the '#' symbol is considered a remark and is ignored.

TSTHOST.CRN - Command file executed every hours

This  command file is executed, if exist,  every hours. The must be

put  in the same  directory of tsthost.exe,  and must be  edit in a

pure  ASCII form.  NOTE: any  command may  be  put into  this file,

because  tsthost do not  execute any  type of  control, but please,

put command that have a sense!

The format is simple:

- any line starting with a # is a comment and ignored.

- any hour block start with *

-  any  line  after the  hours  block  is  considered  like tsthost

command, exactly like if the command was entered from the keyboard

-  any hours block end at the  start of new hours  block, or at the

end of file.

- multiple hours interval may be specified in the form * HH-HH

- Line lenght must not exceeded the 80 characters.

Example:

* 12      <-- executed at 12:00

CMD1

* 13-15   <-- executed at 13,14,15

CMD2

CMD3

* 00-23   <-- executed every hours

CMD4

CMD5

TstHost version 1.43B                                      20

TSTHOST.BID - Bid file

This is  a very important file, and must NOT be altered or deleted.

The purpose of this  file is to have a list of the received bid and

to store the number  of the last bid used by tsthost. But what is a

BID?  Simple, think  to a bid  like a  word that  unique identify a

message.  Every time a new message will be created, from tsthost or

other  bbs, a unique bid  will be appended to  the message, this to

avoid duplicate.

For  example, assume that i forward the  message XX to homebbs, the

message  have bid YY. Next i use the  command FA to force again the

forward  of  this  message to  homebbs.  When  homebbs  receive the

proposal,  it  detect that  the bid  YY  was already  received, and

refuse the message.

The  internal bid of tsthost are a sequential numbers and a string,

you callsign: 12345-IK1GKJ

(1->2->3->..65535->1->2 etc etc).

Now, if you delete or lose this file, you obtain 2 things:

1)  will be lose  the list of  the received  bid, this  is a little

problem,  at most, if you  receive forward from more  than one bbs,

is possible that you receive a duplicate message.

2)  tsthost must  restart the  bid sequence,  and  this is  a great

problem.  For  example,  tsthost  have  created  and forwarded  ten

message,  this messages have bid  from 1 to 10.  Now, if you delete

the  file, tsthost must restart the sequence from 1, but since your

homebbs  have already received bid  1..10, it will  refuse any your

message with bid number under 11.

To  avoid this problem, you can use  the command NBID, to assign an

high  value  to  the bid  number.  This  is  a  emergency solution,

because  assume that you know what was  the last bid number used by

tstshost.....  The  best thing  is to  NOT  DELETE or  altered this

file.  Pleas  note that  the NBID  command  must be  used  ONLY for

emergency situation, normally you MUST NOT usa this command.

When  tsthost start, if the bid file  do not exist, the program ask

for  its  creation (normally  the  first time  you  start tsthost).

Insert  Y to  create the  file, (and if  the file  was lost  do the

command NBID BID_NUMBER), or N to exist and provide a backup copy.

TSTHOST.BID  a routing file of 3001 record. Every record is long 13

byte,  and is the bid  of the message (char[13]).  The first record

is  an  exception,  it  have  two  unsigned  integer number  (ascii

format),  separated by space;  the first  is the  next internal bid

that  tsthost  will  be  use, the  other  is  the  first  record to

overwrite into the tsthost.bid file.

TstHost version 1.43B                                      21

TSTHOST.IN - Mail import file

This   is  an ASCII text  file. The purpose is  to allow SERVER and

PG  programs   to   add  messages   to   the     PMS.    The   file

presence  is  tested    every  minute  and  processed, when  found.

TSTHost  searches in this  file  a  sequence of  SP or SB commands.

Note  that you  MUST use the   correct  syntax.   When processed, a

line  starting with SP or SB is searched, discarding the others. If

such  line is found, the  next one is considered  the message title

and  the others  are the message  text. A  line starting  with  /EX

ends the message.

Example:

SP IK1AAA @ IK1MSL < IK1BBB

message title

message line 1

message line 2

....

/EX

SB ALL @ EU

message title

message line 1

message line 2

....

/EX

The   TO  field   MUST be  specified, the  ROUTE  (@) and  FROM (<)

fields  are optional.  If the route field is specified, the message

will  be forwarded   to  the   HomeBBS. If  the  FROM field  is not

specified, the default value of the PMS callsign will be used.

NOTE: The line length MUST NOT exceed 80 characters.

TstHost version 1.43B                                      22

TSTHOST.DMS - PMS messages database

This   is a  binary  file,  and is   used by   TSTHost to  maintain

the  messages  list  in  the  MAIL directory. Do  not alterate this

file if you  do not know what you are doing!. The easiest method to

destroy all messages is to delete this file, so be careful  ......

This    is    the    tsthost.dms    file    structure,   for  those

programmers interested in writing server programs:

struct dirmes {

char type;                  /* type of message BPT */

char status;                /* status of message NFKYHX */

long number;                /* number of message */

int  da_year;               /* creation date, year */

char da_day;                /* day */

char da_mon;                /* mounth */

unsigned char ti_min;       /* creation time, minutes */

unsigned char ti_hour;      /* hour */

unsigned char ti_hund;      /* hundredths of seconds */

unsigned char ti_sec;       /* seconds */

char to[7];                 /* destination, TO field */

char route[7];              /* destination BBS, @ field */

char from[7];               /* sender, FROM field */

char title[61];             /* message title */

char frombbs;               /* bit field:

bit 0 =1 if received in forward, 0 if local

bit 6 =1 if the msg contain the string /ACK

bit 7 =1 if the msg contain 7plus code */

char startbbs[7];           /* BBS where the message started*/

char bid[13];               /* BID or MID of the message */

char rcvbbs[7];             /* BBS that have fwd the msg */

char free[4];               /* reserved */

};                 /* total 128 bytes */

All char fields end with a 0 byte, according to the C default.

The  message is decoded  with the number  field. The  message is in

the  MAIL   directory:   the   filename   correspond to  the NUMBER

field  and the extension  is .MES. The  filename is  always 8 chars

long,   right aligned by padding the remaining left positions  with

ASCII  '0's.  For   instance,   if  NUMBER is  4,   the     message

filename   will  be 00000004.MES

The  message  number  is consecutive    and,   when    the   server

appends  a new message,   it   has  to   read the  last record from

tsthost.dms  and  increment  the number  field  to  obtain  the new

message number.

WARNING:   all  fields in  the dirmes structure  must be  always be

set;  for instance, if the  @ route is not  requested, you MUST set

it, so *dirmes.route=0;

DO NOT ALTER THIS FILE WHILE TSTHOST IS RUNNING !

TstHost version 1.43B                                      23

TSTHOST.EXT - File extension for automatic execution

----------------------------------------------------

When  you do  the DIR  command, the  contents  of the  directory is

displayed  on the windows screen.  Now, if you press  the ENTER key

on  a filename,  tsthost will  search a corrispondance  between the

EXTENSION  of the selected file and the contents of the tsthost.ext

file.  If the corrispondance is not found,  or the file tsthost.ext

does  not exist, the pressure of the  enter key is ignred. Instead,

if  the  corrispondance exist,  tsthost,  (if you  have  the needed

memory),  will execute  the specified  ds program defined  for that

corrispondance. You may execute, .bat, .com and .exe program.

The format of the tsthost.ext file is simple:

-Any line mut be long not more of 80 charactes. ONE line for ext.

-Any  valid line start with a .  followed by the extension and next

by the instruction to execute fot that extension.

There  are two symbol that tsthost may  substitute with the name of

the selected file.

1) the character "&"

For  the first & found, tsthost will substitute the symbol with the

full  path and filename for the selected  file. The related command

is called from the tsthost directory.

2) the character "!"

For  the first ! found, tsthost will subsitute the symbol with ONLY

the  filename of the selected  file, next change to  drive and path

of the selected file, and executed the related command.

Example:  current displayed directory  is D:\BIN,  and the selected

file is MARCO.ARJ

1) assuming to have a line like this in tsthost.ext:

.ARJ C:\UTILITY\AVIEW & /K /J

When you press return, tsthost will execute:

RUN C:\UTILITY\AVIEW D:\BIN\MARCO.ARJ /K /J

2) assuming you have a line like this in tsthost.ext file:

.ARJ C:\UTILITY\AVIEW ! /K /J

When you press return, tsthost will execute:

D:

CD \BIN

RUN C:\UTILITY\AVIEW MARCO.ARJ /K /J

TstHost version 1.43B                                      24

THE MACRO COMMANDS

------------------

What is a macro?

----------------

First,  what  is macro?  A  macro  is a  sequence  of  command that

tsthost  will execute automatically, sequentially, exactly like you

type  the same commands from the keyboard.  The command may be data

to  send to the  remote station, or  command for  tsthost, or macro

relative  command. It is very important understantd the the command

will  be  executed  sequentially:  this  means  that  we need  some

interrupt  point to  stop the  sequence,  otherwise the  macro will

never stop and continue to its end without do the request result.

Again,  we  are  intelligent, but  the  macro  no,  macro  are very

stupid, so you must instruct tsthost exactly what to do.

How  start  a macro?  Simple typing  its name  in  the cmd  mode of

tsthost.  Every time you type something that is not recognized like

a  tsthost command, the program will search in the tsthost.mac file

if exist a macro  with that name. If the macro do not exist, or the

file  tsthost.exe  do not  exist,  the command  is  simple declared

invalid.  If  the  macro  exist,  the  macro  will  be executed.  A

flashing  "M" will  appear on  the status  line  to display  that a

macro is active on that channel.

How  to stop a macro?  The command is MABORT,  and have a immediate

effect:  macro will be stopped, and the channel remain in the state

that was at the moment in which the macro was stopped.

The  macro support  all tsthost  command, with a  little exception,

and  have  a subset  of  internal commands,  valid  only  for macro

execution.  Is possible insert  also a  name of  another macro like

command,  but in this case  the new macro replace  the existing one

in the execution.

The  macro support  command parameters,  up to 8.  This parameters,

are  identified by the symbol %1..%8, that  are translated into the

literal  parameters  passed  from  the  line  where  the macro  was

started.  For example, if the  macro named PIPPO, have  a line like

this: 10 SAY YGET %1

and  will be  started with  PIPPO MYPROG.ARJ, the  line 10  will be

transformed into: 10 SAY YGET MYPROG.ARJ

But,  if you  start the  macro with  PIPPO without  parameters, the

line will not be transformed, and will be processed literally.

TstHost version 1.43B                                      25

NOTE:

Any  line, in  the macro  structure, that is  preceeded by  + sign,

will  be executed ONLY  if the  channel is  connected. For example,

the  statement  +SAY  ...., will  be  executed  if  the  channel is

connected,  skipped  otherwise. SAY  .... instaed,  will  be always

executed. Do not put space between the + sign and the command.

Macro specified command

-----------------------

NOTE: the  command may be written in upper case or lower case. Some

of  those command will refer to a  line number: Yes, the macro have

the  line number, like the basic language,  but this number will be

explaned   next  in  the   relative  section   of  the  tsthost.mac

structure.  For the moment, you need to  know that if you refear to

a  line number that do not exist in  your macro, the macro will not

produce  an error, but will go to  the existent numeber immediately

highest  of  the number  that you  have  specified. This  means two

things:  if the inexisted  number is  lower of  the lowest existent

number,  the macro will be  restarted. If the  inexistant number is

higher  than the highest existent number the macro will be stopped,

beacuse you reach the end of macro.

* DELAY XX

XX  is a number  to specify how  second the macro  must wait before

process  the next instruction.  This is a  REALLY important command

in  the connection statement.  For example, assuming  you connect a

station,  and open  a record  file. The  record command  will fail,

because  you have  send the  command CONNECT, but  immediately open

the  record file. In this  very little time, probably  the tnc does

not  have tell  tsthost that  the channel  is conected,  so tsthost

will refuse to open the file.

ALWAYS ADD A LITTLE DELAY AFTER A CONNECT STATEMENT.

* DISPLAY TEXT

Simple display the message TEXT on the screen example:

10 DISPLAY Hallo boys

* IFYAPPACTIVE XX

XX  is the line number where tsthost  will go if, when process this

line, there is a yapp transfer active on the macro channel.

TstHost version 1.43B                                      26

* IFNOTYAPPACTIVE XX

XX  is the line number where tsthost  will go if, when process this

line, there is not a yapp transfer active on the macro channel.

* IFYAPPERROR XX

XX  is the linenumber where tsthost will  go if the latest executed

yapp  was stopped due to an error. NOTE: reading this variable will

clear  the yapp  error status,  so you  cannot use  it two  or more

times.  After reading the error status, you  may read it again only

after a new yapp transfer.

* IFNOTYAPPERROR XX

XX  is the linenumber where tsthost will  go if the latest executed

yapp  was succesfully terminated. NOTE:  reading this variable will

clear  the yapp  error status,  so you  cannot use  it two  or more

times.  After reading the error status, you  may read it again only

after a new yapp transfer.

* IFFILESEND XX

XX  is the linenumber where  tsthost will go if,  when process this

line, there is a file transfer active (SEND command).

* IFNOTFILESEND XX

XX  is the linenumber where  tsthost will go if,  when process this

line, there is not a file transfer active (SEND command).

* IFUNDEFPAR PARNUMBER LINENUMBER

Jump  to  line LINENUMBER  if  the parameters  PARNUMBER  (1..8) is

undefined. For example, the macro PIPPO have this lines:

10 IFUNDEFPAR 1 100

20 SAY YGET %1

If  started with PIPPO MYPROG.ARJ line 10  and 20 will be executed,

but  if started with PIPPO without  arguments, after processing the

line 10, the macro will jump to line 100.

* IFULSELECTED LINENUMEBER

Jump  execution  to  linenumber  if  esist  one  or  more  messages

selected for reception in the unproto list.

* GOTO XX

XX simple is the line number where tsthost will go.

TstHost version 1.43B                                      27

* SAY DATA DATA DATA...... [CTRL-?]

This  statem,ent tell tsthost to send DATA.. to the remote station,

is  like when you are connected and  type something to the keyboard

in the CONV  mode. DATA will be send to the remote station. You may

also  use the tsthost SEND command to  send long data to the remote

station.

A  say instruction  that start  exatcly with CTRL-?  Where ?  is an

alphabetical   character,  is   translated  by   tsthost  into  the

equivalent  CTRL code.  For example,  the line  10 SAY  CTRL-Z will

send  the  ctrl-z characters  (ascii  26) to  the  remote connected

station.  Note that you cannot  use more that  one ctrl instruction

for  line.  Again,  while tsthost  always  append  the  CR carriage

return  to any nomral  SAY instruction, the  CR is  NOT appended to

the CTRL line.

* WAITSTRING TIME (LINENUMBER) STRING TO WAIT

This   statement  is  more  complex.   Tsthost  when  process  this

instruction  wait  for not  more  of TIME  seconds  to  receive the

"STRING  TO  WAIT"  from  the tnc.  If  "STRING  TO  WAIT"  was not

received  in TIME seconds, the cprogram will  go to the line number

specified  in LINENUMBER. All field are needed in this instruction,

and () are mandatory aroud the linenumber value.

* WAITDISC [XX]

This  statement simple wait  until the channel  is disconnected. If

the  chanel is  connected, no  other macro  line will  be processed

until  the  channel  will  be  free.  If  anything follow  waidisc,

example  WAITDISC  X,  the  program  wait  until  ALL  channel  are

disconnected

* WAITCONN

Simple wait that the channel is connected before continue.

TstHost version 1.43B                                      28

TstHost.mac macro file format

-----------------------------

Another  simple file  in the  format, more  complex is  write macro

that work right hi!

- Any line MUST NOT have more of 80 characters.

- Any line that start with a # characters, and any empty line is

ignored.

A  macro start  with its  name. The  macro may  be executed  on the

cirrent  channel, or may have  a specified channel:  this is needed

on  a multiport system, where a call,  for example, may reched only

trought  a specified port.  If i hae  two radio port  in my system,

and  my bbs frequency is associated on  the channel 5,6,7,8m surely

i cannot connect it on the channel 3, no?

A  macro stop when a  new macro start or  when the end  of file was

reached.

To define  a macro name, the line must start with the ":" character

followed  by the  macro name. Macro  name may  have not  more of 20

characters,  and  may be  followed  by a  number  to  specify which

channel  will be used for that macro.  Obviusly, the channel number

must not be highest of the number of channel defined for tsthost.

Here is two example:

:PIPPO

This macro will be executed on the actually current channel.

:GIOVANNI 5

This  macro will be executed on the  channel number 5. You may also

use  channel 0, monitor, for a macro,  but remember that you cannot

do connection on the monitor channel.

Any  other line in the  macro definition start with  a line number,

that  is a sort of label,  followed by a space,  and by the command

to  execute.  NOTE: line  number must  be  in sequential  up order,

tsthost do not do a sort order for the macro line number.

Ogni  altra riga  della macro  inizia con  un  numero di  riga, che

particamente  e' una etichetta, seguito da uno spazio e dai comandi

associati  a quel  numero di  riga. Attenzione, non  potete mettere

numeri  di riga fuori  sequenza. I  numeri devono  essere in ordine

crescente, tsthost non effettua l'ordinamento.

For example:   right               bad

10 PIPPO            20 PIPPO

20 LUPO             18 LUPO

TstHost version 1.43B                                      29

You  may use line number from 1 to  32767. 0 cannot be used because

is  an internal number, and from 32768  will be considered negative

number from tsthost and are invalid in the macro.

Pratically, a macro may have an aspect like this:

:NAME [OPTIONAL CHANNEL]

10 INSTRUCTION ...

20 INSTRUCTION ....

...

.... ETC.

TSTHOST commands not active in the macro

----------------------------------------

This commands are not available in the macro:

DIR,  ULIST LIST, because open a window  in tsthost, and require an

active presence on the tsthost keyboard.

SOURCE,  because  i  cannot  do  a  preliminary  control  over  the

contents of a command file.

TSTHOST commands that require an detailed arguments

---------------------------------------------------

This  commands, may be used in a  macro structure, but require that

the arguments is specified in the command line.

SEND,  YPUT, because if you do not  do the arguments open a windows

that permits the directory selection.

RUN,  because if executed  without parameters shell  to dos without

reenter  in tsthost  until the  operator do not  type EXIT  to exit

from the shell.

TstHost version 1.43B                                      30

Macro example

-------------

:PIPPO 2

10 CONNECT IK1MSL-8

20 DELAY 2

25 +RECORD POSTA

30 +SAY RM

35 +SAY KM

40 +SAY B

This  is a  really simple macro.  It connect  my bbs  on channel 2,

note  the delay instruction  please. Next  opne a  record file, and

queue in the  bbs the needed command to read and kill my mail, next

disconnect.  All the macro, are executed in a unique time. When the

macro  stop, probably the connection  is again active,  but all the

needed  bbs  command was  done.  When the  bbs  disconnect, tsthost

automatically  close the POSTA file, and nexty  i can examine it to

read  my mail. Note again  that line 25..40 are  executed only if a

connection was done, due to the + sign before the commands.

Here  is a  more complex macro.  In this  macro, i  assume that the

prompt  of my  bbs is  "IK1MSL BBS  =>". Again,  my  bbs ask  for a

password with the password prompt "PASSWORD?".

>>For the password please read the relative section in the doc<<

:MSL 3

5  WAITDISC

10 CONNECT IK1MSL-8

12 DELAY 2

14 +WAITSTRING 30 (1000) PASSWORD?

16 +PASSWORD

18 +WAITSTRING 30 (1000) IK1MSL BBS =>

#

40 +YD VARIE\PIPPO.ARJ

42 +WAITSTRING 30 (100) IK1MSL BBS =>

44 +IFYAPPERROR 40

46 +GOTO 1000

#

100 +IFYAPPACTIVE 42

110 +SAY

120 +WAITSTRING 30 (1000) IK1MSL BBS =>

130 +GOTO 44

#

1000 DISC

TstHost version 1.43B                                      31

005: wait until the channel is free.

010:  connect my bbs using channel number  3. Again note the little

delay  to recognize the connection. All the other line will be

executed  only if connected,  due the +  sign on  the start of

the command.

014:  wait for not more of 30  seconds the password request. If not

received,  sonthing is wrong. Bbs may be crashed or may be out

of  program due to  dos operation. In  this case i  will go to

line 100, where force a disconnection and stop the macro.

016: send my password.

018:  and  wait for  the bbs  prompt. Again,  max  time wait  is 30

seconds,  if  in  this  time  the  prompt  was  not  received,

assuming  that something  is wrong,  and go  to line  1000 for

disconnect and stop.

040:  ask to download  the file PIPPO.ARJ,  that are  in the VARIE\

directory, using yapp protocol.

042:  wait for  not more of  30 seconds  to obtain  the bbs prompt,

that  signal the  and of  the yapp  transfer.  30 is  a little

number but do not have now a great importance.

044: when the  prompt is received, if i have an error ask again the

file going to the line 40.

046: no error, so go to line 1000, disconnect and stop macro.

100: due to the  statement in line 42, if after 30 seconds i do not

receive  the bbs prompt, tstshost macro  will jump here. First

test  if the yapp transfer is again  active. If is active, ask

for more time jumping again in line 42.

110: if  the transfer is stopped, but i have not received a prompt,

is possible that  i have an abirt yapp. fbb 515c have a ad bug

in  the abotrt yapp routine:  it stop to send  data, but do ot

confirm  the  abort  to the  corrispondance  with  the correct

sequence.  Due to this,  90% of  case the  remote station wait

for  an abort  ack from  fbb while fbb  have already  stop the

yapp   transfer.   To  avoid   this,   and   resyncronize  the

communication  with the  bbs, i  will send  an empty  line, to

force the bbs to emit its prompt.

120:  wait here the prompt.  Again, if not  received, disconnect on

1000.  May be  that there  was not  an error  in the  yapp, is

possible that the bbs is crash.

130:  if  receive the  prompt, jump  on line  44  to test  the yapp

status.

This macro  is not yet perfect. The problem is that if you continue

to  receive  error  on  the  yapp  transfer,  the  macro  will

continue  to ask for  the file. But  this is  only an example,

you may surely study a macro without this problem.

Obviusly,  macro, prompt,  timing, password  are only  example, you

must insert the appropriate value for you needed.

TstHost version 1.43B                                      32

AUTOMATIC PASSWORD

------------------

The  TSTHOST.PSW  file  contains  the  passwords to use with those

systems where it is requested for the access. The file format is:

CALL STRING_TO_SEARCH PASSWORD

CALL is the callsign for the referred password. WHITOUT SSID

STRING_TO_SEARCH   is   the   system  prompt  when the  password is

being  requested, max 50  character long   with no spaces, PASSWORD

is  your  password,  up  to  255  alphanumeric  characters  without

spaces.

Example:

IK1MSL IK1MSL-8> abcdef

IK1MSL Password? 123456

IK1GKJ IK1GKJ-8> ABCEDF

Warning:  Upper and  lower case are ignored  in  the  string being

searched, but are important in the password string.

Here  are  some example  for the  BBS  prompt, that  can  solve any

doubt,  and make you write the correct tsthost.psw. In the example,

my demo password is ABCDEF

IK1MSL-8> 11 22 33 44 55 [0123....           IK1MSL-8> ABCDEF

Password? IK1MSL-8> 11 22 33 ....            IK1MSL-8> ABCDEF

IK1MSL-8> Password? 11 22 33 .....           Password? ABCDEF

IK1MSL-8> Password ? 11 22 33 ....           ? ABCDEF

As  you have  seen, the  string being searched  is ALWAYS  the LAST

word or characters that came before the password code.

If the BBS prompt is:

Insert your password 11 22 33 44 55

and you insert in your tsthost.psw file:

Insert your password ABCDEF

TSTHost  will interpret  "Insert" as the string to be matched, and

"your"  as your password.  This is an   error! The  correct form in

tsthost.psw must be:

password ABCDEF

TstHost version 1.43B                                      33

NOTE:  if you insert  like call an  '*', the program  will not test

the  equivalence between the line and the  connected call, but only

the  equivalence between the prompt received and  the prompt in the

line  of the file. So if you use  this capability, make sure to put

the * lines at teh end of the file, to avoid confusion.

IMPORTANT, PLEASE READ

----------------------

This  information are  not valid  for the automatic  connection for

forward  and unproto  mail request,  since they have  a particoular

protocol.

When  you do the PASsword command, or press the ALT-F1 key, tsthost

will  start the serching  of the  password with  the comparation of

the task callsign and the callsign declared in tsthost.psw

If  you do direct  connection, this  work without  any problem, but

look  for this example.  I connect a  node IK1GKJ-7.  Ok, IK1GKJ is

the  task callsign. Now i send to  the node, the command to connect

my  bbs, IK1MSL-8, and the  node connect my  station with IK1MSL-8.

Now  IK1MSL-8 will  ask for  the password, i  press ALT-F1  or type

PASsword command, and tsthost emit an error message!

Why?  Because  for  the tnc  you  are  connected  to  IK1GKJ-7, not

IK1MSL-8,  since the link to  ik1msl was done in  the NODE, and NOT

in your TNC link.

So  tsthost will search an abbination for  the password prompt with

IK1GKJ  and  not IK1MSL,  and since  do  not found  it,  declare an

error.  To avoid this, PASSWORD command accept a parameters; if you

do the command:

PASSWORD IK1MSL

Tsthost  will ignore  the connected  callsign and execute  a search

for  IK1MSL  callsign,  ignoring the  task  call,  IK1GKJ  in thist

example.  For frequently connection with password trought node, you

may  find useful to program the related PASSWORD CALLSIGN in one of

the 20 programmable function key, to emulate the ALT-F1 short cut.

TstHost version 1.43B                                      34

FORWARD,  UNPROTO  list,  CHANNEL  8  and automatic password

------------------------------------------------------------

FIRST  READ THIS: In this  section, channel 8 is  to be intended if

the  program is  setup for all  8 channel.  If you  have change the

number  of  available channel  with  the /K  command,  the forward-

unproto  request will be  done on  the last  available channel. For

example, if  you ha start tsth with the command /K4, channel 8 here

is to be intended like channel 4.

TSTHost  connects  the  HomeBBS  for  forward  or  unproto list  on

channel  8 only.  If this  channel is BUSY,  the program  waits and

retries   to connect the station when this channel is free. For the

forward,  the  program always  calls the  master  BBS to  allow the

reverse  forwarding  but  the unproto  list  is  requested  only if

active and there are some selected messages.

NOTE:  in a  multiport system  (multiport  TFPCX), you  MUST ensure

that the channel 8 is associated to the same port of the HomeBBS.

Again,  since it is possible  that the program   calls  the station

while you are already connected to the HomeBBS on another channel,

it is suggested to set for channel8 a  unique  SSID (AX25 PORTCALL

command).

???????.CON - HomeBbs connection file

-------------------------------------

This  file permit to define  exactly how to connect  you homebbs in

order  to ask  for unproto  messages or  do forward  procedure. You

need  a file  for any  homebbs used, the  name of  the file  is the

callsign  of the bbs  without ssid,  the exetnsion  is always .CON,

and  the file must  reside in  the directory  where is tsthost.exe.

For  example,  my homebbs  is IK1MSL-8,  so  i have  a  file called

IK1MSL.CON

The file have this rules:

- Any line must be at most 80 characters lenght.

- Any empty line, and any line that start with # is ignored.

- Any other line must start with a valid command.

Actually, four are the supported command:

TstHost version 1.43B                                      35

-  C, tell to  tsthost that this  is a connection  line. The format

is:  C (CALL-SSID) CONNECTION COMMAND.  The () are  needed, and the

callsign  inside  is  the callsign  of  the  connection. CONNECTION

COMMAND  is  the  command  to  perform  in  order  to connect  that

station.

-  S, change the  answer return  prompt on  the connection command.

This is  normally used only for that node that do not have standard

return  messages. Normally  all node  return  CONNECTED in  case of

right   connection,  or  FAILURE  or  BUSY  in  case  of  a  failed

connection.   This  messages  are  already  defined  internally  to

tsthost,  and  do not  need  to  be redefined.  But  some  node may

return,  for example, CONNE instead of  CONNECTED, ECHEC instead of

FAILURE, OCCUP instead of BUSY. The command:

S CONNE ECHEC OCCUP

advise  tsthost that  from this  point all the  connection messages

will  have that messages. The redefinition will remain active until

a new redefinition will be made.

ALL 3 PARAMETERS MUST BE SPECIFIED.

-  L  COMMAND VALUE,  This  command is  used  to  setup particoular

parameters in the  TNC. All data after the L command is sent to the

tnc,  like command. The command  must have the  WA8DED host sintax,

parameters  modified have  effect only  on the  connection channel,

and  will  be restored  to  the orginal  value  at the  end  of the

connection.

-  $ PASSWORD PROMPT. If  you bbs require a  password to access it,

you  may  define here  the  password request  prompt.  This  is the

oldest   HPASPROMPT  command.  For  more  details  about  automatic

password, and configuration file, see the related section.

Here  is some example, referred  to my homebbs IK1MSL-8,  so to the

file IK1MSL.CON:

#bbs may be reached directy, so

C (IK1MSL-8) C IK1MSL-8

|    |       |

|    |       |--from here to end the connection command.

|    |----------callsign of the connection

|---------------tsthost command

TstHost version 1.43B                                      36

#bbs is reached directly trough digipeaters

C (IK1MSL-8) C IK1MSL-8 VIA IK1ABC IK1QWE

#More complex, bbs need node and digipeaters to be reached.

#First, i need to connect the node IK1AAA-7

#

C (IK1AAA-7) C IK1AAA-7

#

#now, i need to connect the node IK1PIP-2, but here i have some

#problem. First, the node AAA-7 do not use the C command to

#connect, but use X. Next, the return message for busy is not BUSY

#but OCCUP, the other messages are standard.

#

S CONNECTED FAILURE OCCUP

C (IK1PIP-2) X IK1PIP-2

#

#Ok, now i may connect my homebbs IK1MSL-8, but warning. PIP use

#standard connection messages, so i must first redefine the

#previously altered messages. PIP do not heard directly ik1msl-8

#so i need to use the digipeaters IK2DED

#

S CONNECTED FAILURE BUSY

C (IK1MSL-8) C IK1MSL-8 VIA IK2DED

#This bbs may be reached directly, but require a password to

#access it. Password prompt is PASSWORD?

#I will use maxframe 3 for this connections.

#

L O 3

$ PASSWORD?

C (IK1MSL-8) C IK1MSL-8

TstHost version 1.43B                                      37

FORWARD protocol

----------------

The   PMS can forward  with a  master BBS according to the standard

MBL/RLI  or FBB COMPRESSED  protocol. Any  BBS may  connect the PMS

and  forward data, but the  PMS   will  send  the   message only to

the BBS defined with the HOMEBBS command.

The   PMS  will  send  to  the HomeBBS   all the messages that have

a  specified  route set  ( @Field not blank),  and locally created.

If  a received   message   has  been forwarded  by another  BBS, it

will  be forwarded to the   connected  BBS  only  if  the  callsign

of   the connected  BBS is  not the same of  the callsign stored in

the RCVBBS field.

For  personal  messages,  they  will  be  forwarded  only  if  the

addressee of the message has not read it.

In  the message  list, the letter 'L'  near the type/status of  the

message  shows that   this  message was written  locally, otherwise

the message was received by the forwarding activity.

You  may always connect  Home Bbs and  send your mail,  but to have

forward, you  must be enabled from the sysop of the bbs. So contact

the sysop and ask for this feature, if you are interesting.

To do this, the sysop of Home Bbs must:

1) Create a forward file like this:

#

A IK1GKJ

P B

R

B IK1GKJ

F IK1GKJ

C C IK1GKJ

----------

#

If  you also need  the  @AMSAT  messages, the  sysop  must  add

the G AMSAT statement after the F IK1GKJ line

2) Edit the BBS.SYS file, and add your CALLSIGN to the list

3)  Edit your record and change  the bbs field, now  it must be set

not to bbs callsign, but to YOUR callsign. (NH command.)

This  info is for fbb  type bbs, i do  not know how  work the other

sotware.

TstHost version 1.43B                                      38

Problem to connect for UNPROTO and FORWARD

------------------------------------------

TstHost  use to call  homebbs for fwd  or unproto  message a simple

protocol.  First, will be connected the bbs,  always on the channel

8,  using  HNode  if set,  or  directly.  Next  TstHost  wait until

receive  the from  the BBS,  the SID  and/or PASSWORD  request, the

order  is not rilevant. But  the SID must be  received, (sid is the

string  [FBB-5.15-ABFHM$]  or similar).  The  program  MUST receive

this  string, because from the sid the  program can detect the type

of protocol to use.

Now,  tsthost  wait  for  a bbs  prompt.  When  the  bbs  prompt is

received,  the sequence is  completed, and forward  or unproto list

request may started.

The  bbs prompt, is a  char > at the  end of a line.  This may do a

problem  on some bbs,  since many sysops  have set the  char > into

the  welcome text on  their bbs. In  the normal  connection this is

not  a problem, but with a remote  protocol yes, because if tsthost

receive  this character, it  think that it  is the  bbs prompt, and

the sequence was not corrected.

For example, if the bbs welcome text have a line line this:

HALLO MARIO, FOR HELP TYPE THE COMMAND <?>

surely  you will not be  able to start fwd  or unproto, because the

character > at the end of the line confuse tsthost.

But,  this line, HALLO MARIO, TYPE <?> TO  HELP, do not do any type

of problem.

If  you have this problem, you may ask to  the sysop bbs to set you

like  a real bbs.  The bbs never  send text  to real  bbs, but when

connected send only the prompt (or password request).

NOTE: If  you cannot start fwd o unproto list, and cannot solve the

problem,  to  obtain  help,  please  switch  on  the  fwd  channel,

manually  start a fwd or unproto request, and when disconnected, do

the  command WRITEBUF ABC. Next send me a message whit the contents

of the ABC file, so i can examine it.

Another  common problem, using FBB compressed mode forward only, is

that when your  homebbs must send mail to you, its send FF and mark

the  message like forwarded. This is a  bug into fbb, to solve, you

must  connect  your  homebbs, and  set  your  default  bbs  to your

callsign, with the command NH YOURCALL

TstHost version 1.43B                                      39

PMS INTERFACE

-------------

Every  channel of  the software is a PMS and  a  normal connection

channel at the same time. All non-connected  channels  are  set by

default in PMS mode. This means that a remote  user  that connects

your station will operate in PMS mode.

But,  if  YOU issue  the  Connect command,  the  channel  becomes a

normal conversation channel.

There  is  an exception  to this. When the  software starts, if one

or  more   channel in   your  TNC   are  already connected  for any

reasons,  they are  conversation channels,  not PMS. The  reason is

that    it   is  impossible to  know  if those  channels  have been

connected by  us  or by a remote user.

You    can switch    from PMS    or NORMAL  channel  mode  with the

commands  TALK  (from PMS to NORMAL,  for example after a chat with

the  remote user),   and   PMS   (from NORMAL  to PMS).  You cannot

switch  a session if  the task is  not stand-by. The  PMS cannot be

executed  if there is an active file transfer on the channel (YAPP,

send or capture). A  TALK  command   has  no  effect  if the PMS on

that channel is processing a user command.

Moreover,  you cannot switch a window to conversation mode  that is

already  active in  PMS mode.  This is  correct because  you cannot

chat  with  a  user,  or your  text  will  interfere  with  his PMS

activity. You have to switch to TALK mode before chatting.

MAIL WORD FLASH IN SCREEN

-------------------------

When a new  mail for you arrive on the pms interface, the word MAIL

will flash in the screen.

To  stop flashing, you MUST press ALT-F10  key or do the LM command

from KEYBOARD; i have used this soluction for two reasons:

1)  TSTHOST.DMS is not in memory,  and scan it to  see if your mail

is already read cost some time.

2)  a PIRATE  user may  connect your pms  with your  callsign, read

your  mail, and  next kill this  message. I  case 1)  the line will

flash off, and you do not see that new mail was arrived. With this

system,  you may note that  mail is arrived, even  if that mail was

killed or read.

TstHost version 1.43B                                      40

PRIVATE DIRECTORY FOR PMS USERS

-------------------------------

For  any user connected to  the PMS interface is  possible define a

private  directiry with the utility program MAINTUSR, provided with

TstHost.  The user  that connect  your pms if  have defined  a PRIV

directory  will may access it with the  virtual drive prefix P: The

user  will access to  any subdirectory of  the priv, but  not to an

eventually upper directory. For example, assuming exist this tree:

C:\TSTDATA\PRIV\LEVEL1\LEVEL2

Is   defined  like  priv   dir  C:\TSTDATA\PRIV   for  IK1GKJ,  and

C:\TSTDATA\PRIV\LEVEL1 for IK1MSL.

With   the  prefix  P:  (es   YD  P:xxx)  IK1GKJ   will  access  to

C:\TSTDATA\PRIV, LEVEL1 and LEVEL2.

while IK1MSL will access only to C:\TSTDATA\PRIV\LEVEL1, LEVEL2.

NOTE:  Even is possible (BUT  NOT RECOMENDED) to  use MAINTUSR with

tsthost  active, if you change a path for an user already connected

to  the pms,  the new path  will not  became active  until the user

disconnect and reconnect.

THE SERVER

----------

What  is the server? The  server are programs com  or exe that must

be  exist in  the tsthost.exe  directory.  When a  personal message

arrive  to your  pms interface,  and tsthost  detect a  server (see

SERVER  command), tsthost copy  the contents  of this  message to a

file,  next  execute the  related com  or  exe file  and  pass like

argument  the name of this  file. This is a  standard forward file,

so  the first line will  contain SP SERVERNAME <  IK1AAA, next line

will be the  title of the messages, next to end the contents of the

messages.  Server may do anything, but any  return message from the

server  to sender must go into the tsthost.in file. When the server

stop  the  execution, control  return to  tsthost,  messaged copied

will  be  deleted,  original message  will  be  put  into  K state.

Remember that will server are running, tsthost is stopped.

TstHost version 1.43B                                      41

PG SERVER

---------

The  PG servers are  programs written  to expand  the capability of

the  software. They MUST be  placed in the directory  PG, one level

below the  location   of  tsthost.exe. As  the  system  is inactive

while  executing  the  PG  servers, they  must be small and use the

minimum machine time. The  displayed  image  is  saved  during  the

program  execution.  The  server  must  NOT  access  the video  RAM

directly.

The server receives the following command-line:

argv[0]     Drive\path\servername;

argv[1]     Callsign of the user, with ssid

argv[2]     Server level, 0 the first call, up to 255 the next.

argv[3...]  User-specified parameters, each word  is  a  separated

argument.

The  server data output  (with printf() and  putf()) is transmitted

to the user.

The server may return:

0 - The output of the server is  sent  to  the  user  and  returns

to the PMS prompt

1 - The output of the server is sent to the user and the server is

called again. Level incremented

2 - The output of the server is executed like a PMS  command,  and

the user is reconnected to the PMS interface.

3 - The output of the server is executed like a PMS  command,  and

the server is called again. Level incremented.

4 - The output of the server is sent to the user and the server is

called again. Level NOT incremented.

128..255 - The output of the server is sent to the user and the

server is called again. The level will be set equal to the PG

return code

NOTE   for 2   and 3:   The output  must be maximum  80 chars long,

and  must start  at the line  beginning;   the line  must NOT start

with    a carriage   return  or line  feed.  The server  must NEVER

return another PG command.

TstHost version 1.43B                                      42

For  instance, this  is a very simple ECHO server  that  returns to

the user all he have typed, until the user types BYE:

#include <stdio.h>

#include <string.h>

main(int argc, char **argv)

{

int i,mode;

sscanf(argv[2],"%d",&mode);

if(!stricmp(argv[3],"BYE") && mode)

{

printf("\nBest 73\n");

return 0;

}

printf("\nHallo %s, you have send:\n",argv[1]);

for(i=3;i<argc;i++) printf("%s ",argv[i]);

printf("\nType BYE to end.\n");

return 1;

}

The user IK1AAA calls the server with PG ECHO AA BB.

The server returns HALLO IK1AAA, YOU HAVE SEND AA BB,

TYPE BYE TO END

the user types I AM MARIO

The server returns HALLO IK1AAA, YOU HAVE SEND I AM MARIO,

TYPE BYE TO END

the user types BYE

The  server returns BEST  73,  and the  user will return to the PMS

prompt.

TstHost version 1.43B                                      43

CON_FILT.EXE, connection filter

--------------------------------

CON_FILT.EXE  is a  program that  may be  exist in  the tsthost.exe

directory. TstHost test if this program exist only when statup.

If  TstHost have found at the startup  time this program, for every

incoming  connections  on  the  pms  interface,  con_filt  will  be

called.  The  purpose  of  this  program  is  to  provide a  filter

connections,  but it  may do  anything you  need,  you may  write a

con_filt.exe program for any your need.

When TstHost call con_filt.exe, pass those arguments:

CALL-SSID of the user

Level, 0 the first time, up to 255 the other time

0 dummy, always 0

not 0 if is a new user, not registered in tsthost.usr

0 dummy, always 0

channel number where is connected the user

anything received from the user, each word a new argument.

Con_filt.exe must be return to TstHost:

0   to accept the connection

1   con_filt will be called again with the data received from the

user

2   connection refuse, the user will be disconnected.

Anything   write  from  con_filt  on  the  standard  ouput  channel

(printf(), puts()), will be send to the connected user.

MES_FILT.EXE, message filter

----------------------------

This  program, if exist, must reside  in the tsthost.exe directory.

Tsthost,  when  start, check  the  existance of  this  program : if

found,  it will be called every time  a message are inserted on the

pms. If  at the start time tsthost do not found the program, simple

it  will be  ignored and  not called.....  Note  that if  you start

tsthost  without mes_filter, and next add the  program, you need to

restart tsthost in order to recognize mes_filt.exe.

The purpose  of this program is to provide an user interface to all

incoming  message. Since it was  called evry time a  mail arrive on

the  pms (keyboard, forward, tsthost.in,  unproto....), is possible

TstHost version 1.43B                                      44

examine  it and  do something  that you  need  and is  not provided

directly by tsthost.

MES_FILT.EXE  receive an  unique arguments,  a  number that  is the

pointer  to the file TSTHOST.DMS where the  record for that message

is  stored : in this record you may found any information about the

message.  To access the message text, simple  use the number in the

record and search it in the MAIL directory.

Is  safe add or modify the text  message in the MAIL directory, but

about  the TSTHOST.DMS, remeber  that you  may ONLY  ONLY ONLY READ

the  file, do not attempt any write operation, or serius damage may

be  done inside the database, because tsthost  maintain in memory a

copy  of the  record, that  will be again  written in  the database

when MES_FILT ends.

If  you use  mes_filt with  automatic reply,  please do  not insert

directly  the return mail inside the MAIL directory and TSTHOST.DMS

directly..... use the TSTHOST.IN tecnique, for the above reasons.

Mes_filt.exe may return 3 code to tsthost :

0    means that the message is accepted (N)

1    means that the message is put in H state, HELD

2    means that the message is put in K state, KILLED.

Any other value will be considered an execution error, and the

message will be accepted.

Since  MES_FILT is executed  prior a server  execution, if mes_filt

kill  or held an incoming message addressed  to a server, the sever

do  not start. Note that simple removing the  H or K state from the

pms  list will not start  the server, because  server execution may

be  possible only at  receiveing message time.  You need  to send a

copy  of  the message  to the  server  to start  it,  naturally the

server must support this.

PMS MESSAGE INSERTION SEQUENCE

------------------------------

This   is  a  simple  description  of  the  sequence  of  operation

performed  by  tshost when  insert  a message  into  the  pms : the

purpose  of this note is for  server and similar programmation.....

for this  note, please assume that the AX MYCALL is IK1GKJ, ssid is

not relevant.

1)  will be detected  the last number  of the  messages.... This is

done  reading the last valid record inside  the dms file and adding

1.  Note that a comparison is made with the number memorized inside

tsthost.

TstHost version 1.43B                                      45

2) If a  message is P, and addressed to my callsign, the route will

be deleted. (stay here... SP IK1GKJ @ XXXXX became P IK1GKJ @)

3)  if the  message is  received in  forward, and  the route  is my

callsign, the route is removed (SP XXXX @ IK1GKJ became SP XXXX @)

4)  if the message is  not received in forward,  i.e. is local, and

do  not have  a route  field, will be  scanned the  HBB file  for a

corrispondance.

5)  If a  message  do not  have a  bid, a  bid  will be  added from

tsthost

6) The bid will be registered inside the BID file.

7)  The record  will be  written inside  tsthost.dms file,  but not

discarded,  and the text  part of the  mail is written  in the MAIL

directory.

8)  A test is  performed to see  if a copy  of the  message must be

saved according the SAVEMSG command.

9)  If exist, MES_FILT is  started, the status of  the message will

be changed according to the mes_filt return code.

10)  A search in the SERVER list was made,  to see if a server need

to  be  started. The  server  do not  start  if the  status  of the

message is not N.

11) If the message was not for a server (started or not):

A) if contain 7plus code, is not local, and command 7PCOPY is

ON, the message is copied inside the 7PDATA.FWD file.

B) A test to see if a mail for my callsign is performed, (MAIL

flash in status line.

12)  If the mail is not for server, is  type P, do not have routing

(stay  here),  is not  local, and  have the  /ACK  line, an  ack is

generated according to the ACK command.

TstHost version 1.43B                                      46

MESSAGGE FLAGS

--------------

This  is a little description of the  letters that specify the type

and the status of the messages:

1 LPND IK1GKJ .......

||||

||||--Message contain 7plus code

|||---Messagge status (NYKFH)

||----Messagge type (BPT)

|-----Local

LOCAL:

The  L  letter,  if present,  means  that  the  message  is locally

created  on this pms. If absent, means that the message was receved

in forward from another bbs.

TYPE:

P  means that the message is personal:  this type of message may be

accessed  only from the  sender and  the addressee  of the message,

and only them may kill the message.

B means  that the message is a bulletin: Any callsign may read this

message,  but since  this message  is public,  only the  sender may

cancel it.

T  means Traffic, i do not  have any info about  this type of mail,

so it will be treated like the B type

STATUS:

N  means new mail. The message status may have some alterations: fr

example,  a personal mail,  if readed  from the  addressee, go into

the  Y status. Bulletin messages instead, do nt change to status Y,

because do not have a really addressee, but if for ALL.

X means that the message was rejectd from HomeBbs while forward.

F means that the message was forwarded to homebbs.

K means that the message was killed. A message killed is not

accessible from the user.

H means that the message is locked. A locked message is not

accessible from the user and will not forwarded until the sysop

will unlock it.

NOTE: The sysop may access to all messages.

TstHost version 1.43B                                      47

PMS REMOTE MESSAGE MANAGEMENT

-----------------------------

Any  user,  after  a succesfully  SYS  command,  have  the complete

access  to all message in the pms  if his sys flag  have a value of

16 in the sum of his permission.

This  permission,  enable  every  operation  on  the pms  messages,

exactly like the sysop on the keyboard.

FA NSGNBR   Add the MSGNBR to the forward queue.

FR MSGNBR   Remove MSGNBR from the forward queue.

EM MSGNBR   Edit the header of MSGNBR.

THE TZ ENVIROMENT VARIABLE

--------------------------

Some  pms function, like rline, must convert your local time to the

GMT  Universal  Time.  The  program  provide  automatically to  the

conversion,  but for a corrected conversion, your  PC must have set

the TZ enviroment variable.

This  variable,  TZ (timezone),  is  a standard  name  used  by all

program  that make a conversion timing. The variable must be set in

your AUTOEXEC.BAT file, with the command:

SET TZ=XXXY

Where  XXX is a 3 code  letters, your time zone  name, this data is

not  important  for  the  soft, and  Y  is  a  number,  positive or

negative,  indicating the  diference, in  hours, from UTC  time and

YOUR local time.

For  example, for Italy, when are the  12.00 (solar time), UTC time

is 11.00. So i have put into my autoexec.bat file:

SET TZ=EST-1

TstHost version 1.43B                                      48

PMS MESSAGE MANAGEMENT

----------------------

All  the manipolation of the pms messages  is done via a menu style

selection  in a window. The activation of  this window is done with

the L command, for LIST.

The messages list may be global or partial under filtering fields.

Those are the available commands for list the messages:

L              List all messages  (only LIST, not READ/KILL)

LL 50          List last 50 messages (only LIST, not READ/KILL)

L 100          List message number 100

L 100 102 150  List messages 100 102 and 150 (max 10 numbers)

L 100-150      List message from 100 to 150

L -150         List message from begin to 150

L 150-         List message from 150 to end

L< CALL        List all messages sent by CALL

L> CALL        List all messages for CALL

L@ ROUTE       List all messages for that route.

LS STRING      List all messages that have STRING in the title.

LM             List all my messages

LN (FKHYX)     List all messages in status N,F,K,H,Y,X

LQ             List all messages not yet forwarded.

LP             List all P messages

LB             List all B messages

LT             List all T messages

To  activate the message  window, do the  appropriate list command.

The  window will display the related messages, next you may move in

the  window withthe classic  cursor movement keys:  UP, DOWN, HOME,

END, PAGE UP, PAGE DOWN. The ESCAPE key will end the session.

In  list mode, you can  edit, change status or  type, read or reply

to   a  messages......  Simple  move  the   highlited  bar  on  the

intersting messages and press one of thos combination of keys:

ALT-I   The  more   complex,  is  used  to  change  status,  title,

destination  etc... To move around the  fields, use the TAB

or  RETURN keys.  To end  the operation,  move to  the <OK>

field and press RETURN. ESCAPE will abort the operation.

ALT-H   If  the message is in H state,  will became N, otherwise it

will be change in H state.

ALT-K   Kill the message. No other operation is available.

TstHost version 1.43B                                      49

ALT-F   All  the messages in  N state that  do not  have a routing,

will  be add to forward list queue.  For any other case, if

the status  of the message is N it will became F, otherwise

change to F state.

RETURN              Read the message

CTRL-RETURN    Read the message, but with the RLINE expanded.

NOTE:  while  reading,  you can  move  into  the  message  with the

classic  direction keys.  Press ESCAPE  to end the  reading session

and return into the list window.

ALT-E   Edit the message.

ALT-R   SEND  REPLY, make  a return  message to  the sender  of the

selected message.

CTRL-R  Like ALT-R,  but will put the contents of this message into

the text of the reply message.

ALT-C   SEND COPY, make a copy of the message.

ALT-M  Mailcopy, to copy/append the the highlited mail to a file.

NOTE:  ALT-R,  CTRL-R,  ALT-C, ALT-M  open  another  window  in the

screen.  In this window  you may specify  a filename to  add at the

end  of the new message. If  you do not need  to add external file,

simple press  enter on the empty field. Instead, if you need to add

file,  type here the name. If the  filename do not have a specified

path,  this file will be searched in  the WORKDIR directory. If you

do  not remember were is the  file, you may press  the F2 key, that

open another window for a selection from a list.

The  ALT-C command also have to specify  the address and optionally

the  destination bbs for the message. Use  tha TAB or RETURN key to

move in the field.

ESCAPE always abort the operation.

While  editing  the message,  the internal  editor  do any  type of

movement  with  the direction  keys  and ist  combination  with the

CONTROL KEY.  CTRL-Y delete a line. Press F2 to end the editing, or

F8  to abort. At the end of the  edit session, you will enter again

in the list window.

TstHost version 1.43B                                      50

* K

For  more simplex  operation, killing  of the  message may  be done

also  directli from  the keyboard.  K command accept  all estension

provided for the L command.

* S  CALL [@ ROUTE] [+FILENAME]

* SB CALL [@ ROUTE] [+FILENAME]

* SP CALL [@ ROUTE] [+FILENAME]

* SR[R] MESSAGE_NUMBER [+FILENAME]

* SC    MESSAGE_NUMBER CALL [@ROUTE] [+FILENAME]

The 'SP' sets the message to Personal,  that  means  that only the

addressee can read the  message.  The  'SB'  sets  the message  to

Bulletin,  that means  that anybody  can read  the message.  If the

'SB'  or 'SP' are not  expressly specified, (the  'S' command), the

program  assigns the 'P' mode for any valid CALL, otherwise the 'B'

mode.

SR  automatically replies with  a P-type  message to  the sender of

the  specified  message number.  The title   of the message will be

"Re:" and the title of the original message.

SRR  work like SR, but insert in the  editor a copy of the contents

of message_number

SC send a copy of the specified message number to CALL.

During   those   command,  you   cannot   switch the  task, but you

must  terminate the message  editing session.  If +FILENAME exists,

the  content  of the specified file is  appended to the end of your

message.  If  FILENAME has a drive/path  specification, the file is

read  from the selected location, otherwise it  will be searched in

the WORKDIR path.

TstHost version 1.43B                                      51

PMS COMMAND FROM KEYBOARD

-------------------------

* ACK [H | Y | N]

This  command enable the ack  of any message  received that contain

the  string /ACK  in the  first column  of oe  of  the line  of the

received  messages. The  ack is  like a  return answer  confim that

your  pms  have  received  the  message.  The  confirmation of  the

reception  of the message  is automatically returned  to the sender

of the message.

Y  enable the ack function, N disable,  H make the ack message, but

put  it in held state,  this means that the  ack message is locked,

and will not be returned until you unlock it.

* BMail [ON | OFF]

Enable  disable the MAIL FOR section in the beacon. If enable, will

be shown a list of callsign for which exist mail in the pms.

* BReak [callsign | ALL] text

Send  text to the callsign  connected on PMS. Using  ALL instead of

callsign  will send text on all PMS connected users. The break will

be send only for the pms station that are in idle state.

* CH8RESERVED ON | OFF

If on,  the channel 8, (or the latest channel available if you have

start  tsthost with channel limitation parameter), may be connected

only by homebbs or halias

* ENdmessage filename | OFF

This  command defines a  file that  will be  automatically added at

the  end  of  any message written  with the S?/SR  functions of the

PMS.  If the  S?/SR  command already have  the +FILENAME extension,

first  will be  added the  FILENAME  contents, then  the ENDMESSAGE

contents.

OFF Disables this function.

If ENDMESSAGE does not have a specified path, it will  be searched

in the program directory.

TstHost version 1.43B                                      52

* EXPORT N | F

This  command export to a disk file  called EXPORT.FWD the messages

that  normally will be forwarded to homebbs. The file format is the

standard MBL-RLI forward type.

N means that the messages are exported and not marked F

F means that the messages are exported and marked F

* FBbmode ON OFF

Enable  (default) or disable the fbb  compressed forward mode. This

type   of  forward  require  about  32  kbyte  of  memory  for  the

compression  routine, if the program do not  found enough memory to

start  the compressed forward, automatically  switch to the MBL/RLI

forward type.

* FREverse ON OFF

Enable  (default) or disable the reverse forwarding. If freverse is

OFF,  tsthost will call homebbs  ONLY if there are  mail to send in

the   pms.  With  freverse  ON,  tsthost   will  call  homebbs  for

forwarding even if there are not mail to send.

* FTIME 0..60 minutes

Every  FTIME minutes, the  PMS will  connect the  master BBS asking

for  the  forward. Any value  below   5 disables the function,  and

the forward is performed only if the HomeBBS calls your station.

* FWD

Manual start of the forward with the HomeBBS.

* HAlias CALL-SSID

To  define an extra callsign that will be considered like a homebbs

when connected.

* HIeraddr TEXT | OFF

Is  a field of max 23 characters,  used to define your hierarchical

address, for example .IPIE.ITA.EU

OFF  disable the  function. This  field, if  present, is  used with

RLine command.

TstHost version 1.43B                                      53

* HOmebbs CALL-SSID

Defines  the BBS  where the  PMS messages  have to  be transmitted,

those  with the routing field  (@) set. SSID must  be specified, as

IK1MSL is different from IK1MSL-8. For example:

HOMEBBS IK1MSL-8

NOTE:  if  you change  HomeBbs callsign,  and  the unproto  list is

active,  the list for the previsuly homebbs will be closed and will

be open a list for the new homebbs.

* HRoute CALL [/V | /P ROUTE]

This  command set  or display  the hroute  address for  a callsign.

Given  in  the form  HROUTE CALL,  display  the h-address  for that

call, if known, reading the file tsthost.hrt.

Given in the  form CALL /V (or /P) ROUTE set or change the haddress

for  that  callsign.  /V means  volatile,  tsthost  may  change the

hroute if  detect a new hroute for that callsign. /P means that the

route is fixed, and cannot be changed automatically from tsthost.

If  the ROUTE paameters  is not provided,  the call will  be set or

changed  to an empty haddress, if the  ROUTE parameter is provided,

the haddress are set for that call with the specified route.

* LBHeld ON | OFF

Any  bulletins locally generated will  be put in  status HELD. This

meand that  only the sysop and the sender of the message may accest

to this mail.

* LPHeld ON | OFF

Like LBHELD, but works on personal mail

* MAIlcopy [number... <IK1GKJ >IK1GKJ @IK1GKJ] [filename]

Append   to  filename  any  valid   combination  of  the  specified

arguments.  You may specify a list of  number, a sender callsign if

preceeded  by <, a destination  callsign (>), a routing  (@) or any

comabination  of those parameters. The latest arguments must always

be  the fil were append the messages. If  file name does not have a

drive/path  specification, the file will be  written in the workdir

directory.

TstHost version 1.43B                                      54

* MSGHEADER ON | OFF

Enable/disable  the pms message  header, (TST HOST  .... LOCAL TIME

.....)

* NBID bidnumer (1...65535)

To assign a  new bid start sequence. DO NOT USE THIS COMMAND IF YOU

DO NOT KNOW WHAT YOU ARE DOING!!. Read TSTHOST.BID section before.

If   you  do  this  command  without  parameters,  simple  will  be

displayed the next internal bid number.

* PMS

The current task is switched in PMS mode, for instance after a QSO

with the remote user that called you. The user returns to  the PMS

prompt. You cannot switch if you have any active file  transfer in

the channel (YAPP, record, send). See the TALK command.

* PYAPP [ON | OFF]

Enable disable yapp function of pms.

* RIng [ON | OFF]

Ring  work with  the PMS  command T, used  by an  user to  call the

sysop. If ON (default), calling sysop is enabled, if OFF no.

* RLine ON | OFF

If ON, when  your PMS forward a message, your R: line will be added

to  the message,  if OFF no.  WARNING: with  RLine ON,  you will be

considered exactly  like a regular bbs, so, if a user use a command

SR  on  your mail,  the  return address  will  be YOURCALL@YOURCALL

insted  of YOURCALL@YOURHOMEBBS. If @YOURCALL is not defined in the

homebbs  and/or adiacent bbs inhis  forward file, mail  sent to you

vill  never arrive to you,  this because you  are not a  bbs in the

regular  path. Whit off  this is not  a problem,  because your mail

will be returned to your homebbs, and homebbs is know in the path.

But,  some server have the capability to communicate one to another

trougth  the RLine, and  in this case,  if you do  not enable RLine

this type of server cannot work properly. So see you need.....

TstHost version 1.43B                                      55

* SAvemsg +/-TOFIELD +/-@ROUTE +/-<FROMCALL .... up to 20

To  define up to 20  address field. Whe a  message will be received

on  the pms interface, if its TO , @  or FROM field match one field

in the list,  the message will be copied into a file in the WORKDIR

path,  with .MSG extension and TO name  if match to, @ROUTE name if

match a route, #FROM name if match a from field.

* SERver +/-SERVERNAME +/-SERVERNAME .... up to 20

To  define  up to  20 server  program  that will  be  executed when

arrive a personal message addressed to SERVERNAME@YOURPMS.

+  ADD server, - REMOVE server from  list. Do not specify extension

or  path  to  the server  statement,  only  the  filename,  and the

executable file must be located into the TSTHOST.EXE directory.

* TALK

The  current task is switched  to  the  normal  conversation mode,

for  example when  the remote  user TALKs the sysop (see PMS). You

cannot switch if the PMS is not in stand-by status.

* TImeout minutes

Specify  the time after, without activity on  the channel,  i will

be automatically disconnected. Time is between 0 and 60 minutes; 0

(default) disables the timeout function.

* UPdatedms pday bday pfday

When  a message  is killed, the file is not  deleted from the disk

but the status flag becomes 'K'. This command allows  the deletion

from  the system of those  messages that are killed,  and to define

how  time a message may  reside on your pms  become of an automatic

deletion.  The  parameters are  the number  of  day from  the today

date, to the date of the creation of the messages.

The  first parameters is  referred to a  personal message  not in F

state.  The second are for bulletins, any  state. Next the personal

mail in F state.

Messages in K state are always immediately deleted.

Messages in H state is never deleted.

Please  note that  you cannot run  this  command  if  any  user is

connected to the PMS interface.

TstHost version 1.43B                                      56

For   example,  to   erase  all   private  messages after  10 days,

bulletin messages after 4 days, and PF after 3 day:

UP 10 4 3

* USerdir [path] [path] [path] [path] [path]

Sets the directory for the users connected to the PMS interface.

The default path is a subdirectory within  the  one  that contains

tsthost.exe, but you can change to a new path with this command.

Note that TSTHOST does not CREATE  the  directory,  the  path MUST

EXIST.

Is  possible to define up to 5 path,  that a user connected to your

pms,  may access via the virtual drive  specification letters A: B:

C:  D: E:. The first  path is identified  like drive C:,  ma is the

defualt,  and drive letter may be omitted. For example, if the user

do  the command  W E:, this  users will  access to  the 3 specified

path.

NOTE:   An  user,  after  a  succesfully  sys  command,  that  have

permission  to read  or write  on all your  system, see  the really

drive, not this virtual drives.

TstHost version 1.43B                                      57

PMS commands for remote users

-----------------------------

NOTE:  ANY COMMAND, BEFORE TO BE DECLARED UNKNOWN, WELL BE SEARCHED

LIKE  A PG, THIS PERMIT TO EXPAND  THE PMS COMMAND WITH PG PROGRAMS

WRITTEN FOR PARTICULAR FUNCTION.

* R [XR]  to read messages

* K       to kill messages

* L       to list messages

The valid parameters for the R, L, K commands are listed in the

following table (the examples are for the List command) :

L              List all messages (only LIST, not READ/KILL)

Will be displayed only the message received after

your last list command.

LL 50          List last 50 message (only LIST, not READ/KILL)

L 100          List message number 100

L 100 102 150  List messages 100 102 and 150 (max 10 number)

L 100-150      List message from 100 to 150

L -150         List message up to number 150

L 150-         List message from number 150 on

L< CALL        List all messages sent by CALL

L> CALL        List all messages for CALL

L@ ROUTE       List all messages for that route.

LS STRING      List all messages that have STRING in the title.

LM             List all my messages

NOTE:    Any  users  have access  to  all  bulletin    and personal

messages addressed to  his callsign or that  he  have  sent. Killed

messages will never be listed.

XR  work exactly like R, but while R  compress the R line like fbb,

XR display those line completely.

* B

Disconnects the system.

* DOS DOS_COMMAND

If enabled, TSTHost executes the  specified DOS command.

TstHost version 1.43B                                      58

* I

List the users connected to the PMS interface.

NOTE:  Only the stations connected to the PMS are listed. Channels

not  in PMS-mode are not listed. Display also the available virtual

drive.

* N NAME

Register your name.

* PG [[NAMESERVER] SERVER PARAMETERS]

If called without  arguments, the  program sends  to the  user the

contents   of the   file TSTHOST.PGS, if exist,  otherwise sends NO

PG  AVAILABLE. In  this way it  is possible  for the  sysop to have

some  PG servers not declared. A user  will not execute the servers

but  the  sysop,    or a  restricted  group  of  users,  can easily

activate these extra services.

If  the server  name parameter  is specified, the  program executes

the specified  server. The   output of  the server  is sent back to

the user.

* SYS

When  the  user   performs  the SYS command,  his callsign  will be

searched  in   the  TSTHOST.SYS  file, and,  if  found,  the system

requires  a password, codified  with the standard  five letters and

also  with the MD2 system. The user may  answer with one of the two

modes.  If the user enters the corrected password, to the user will

be  assigned some privileges, specified in  tsthost.sys file by the

flag parameters. See tsthost.sys section for info about flags

The  password prompt is the PMS callsign without SSID,  with _PMS>

and the password request, like:

IK1GKJ_PMS> 5 1 3 7 6 [0123456789]

To  cancel previusly authorization, you may  repeat the sys command

and  do  a  bad  or  null  password.  If  you  change  the flag  in

tsthost.sys  while a user with a  valid authorization is connected,

the  new flag will not have effect  until the current authorization

will not be cleared.

TstHost version 1.43B                                      59

* S  CALL [@ROUTE]   * SB CALL [@ROUTE]           * SR MSG_NBR

* SP CALL [@ROUTE]   * SC MSG_NBR CALL [@ROUTE]

Stores  a message into  the  PMS.  SB is for  bulletin messages, SP

for personal   mail. The  field @ROUTE  is optional.  If specified,

the  message will be forwarded to the  specified HomeBBS. If  the P

or  B types are not expressly specified,  the program automatically

assignes the P type for  any  valid  CALL, otherwise the B type.

SR replies a P-type message to the sender of a specific message.

SC send a copy of message number # to call.

To terminate the message editing, send CTRL-Z  or /EX in the first

text column.

* T

Calls  the sysop of the connected PMS.  You are always connected to

the PMS interface until the sysop do not switch it in TALK mode.

* V FILENAME

Display  to the users the  contents of an ascii  file. File must be

in USERSDIR or its subdirectory.

* YD FILENAME

Sends  to the   user  the specified file  using YAPP  protocol. The

file must  be in  the  USERS  (USERDIR  command) directory or  its

subdirectories.

* YU FILENAME

TSTHost receives a file from the user, with the YAPP protocol.

The Resume option is always enabled, so if the transfer  stops for

any reason, the user can reconnect the system and restart the file

transfer from the  point where  he was  arrived. The file will  be

put in the USERS\UPLOAD directory.

After  any succesfull or interrupt transfer, the system will advise

the sysop with a mail on the pms that a new upload was done.

* W [PATH]

Display the contents of the USERDIR directory.

* ?

Sends to the  user the  contents of  the TSTHOST.HLP file. If this

file   do  not  exist,  the  user  receives  the  message  NO  HELP

AVAILABLE.

TstHost version 1.43B                                      60

* @ [CALL | ALL] TEXT

Interrupts  the  user  CALL,  that  must  be  connected to the PMS

interface,  by  notifying the  TEXT message.  If  you use  ALL like

callsign, the break will be sent to all pms channel.

* /EW /ER /ED /E@

The  command permits  a remote  shutdown,  and the  WRD@ characters

work  like in the keyboard EXIT command.  To do /E? remote command,

the users  require a SYS validation with the value of 32 present in

the TSTHOST.SYS file.

TstHost version 1.43B                                      61

UNPROTO LIST

------------

When a FBB BBS receives a new  message,  emits  a  special Unproto

frame  with the message  description. Also,  it is  possible to get

the BBS message list without connecting the system.

With  those  features,  a  user with the PC active all the day can

receive and maintain the list of the messages present on the BBS.

It is not needed to connect the BBS to get the message list, too.

To  use this feature  you must be enabled on the BBS system so, if

you need the Unproto list, contact the Sysop.

Even  if you are not enabled, the program  can  create  a  list by

intercepting  the Unproto  messages,  but cannot  resyncronize with

the BBS system if a packet is lost.

NOTE: to use the unproto list, monitor channel  must  be  open for

the  Unproto frames,  letter U.  AX MON  U,  AX MON US, AX MON UC,

AX MON USC are correct, some other modes without the 'U' aren't !

The Unproto  list can save in memory up to 800 message headers, but

to  avoid memory consumation, you may specifi  the unproto size via

the /U command line parameters.

The  HOmebbs  command  sets the BBS from which the Unproto list is

accepted.  You must  specify  the callsign SSID, because IK1GKJ is

different from IK1GKJ-8.

You can set up  to 20 fields that are automatically  selected when

received, like is  possible to set up to 20 fields that will not be

inserted  in  the  list. The  list  can  be  obtained  without user

operations,  or  by issuing   a  special  command (please  read the

section  about channel 8 on  the manual), even if  the BBS requires

an   access  password.  Unproto   list  is   transferred  with  the

compressed  FBB protocol. This  protocol requires about  19K of RAM

for the message decoding.

The  received files  are then  put into the  PMS interface,  and me

next  view with the L  command, that access to  the pms function of

tsthost.  The  received  file  have  the  same  proprierty  of  the

standard  file  received in  forward,  so may  have  /ACK, SAVEMAIL

function etc...

TstHost version 1.43B                                      62

Unproto  List  is  directly connected  with  the  HomeBbs callsign,

since  is  this  callsign  that  generate  the  unproto frame.  The

information  received by  tsthost, is  stored in  a file  that have

".ULS"  like extension, and the  call without ssid  of your homebbs

like  name.  File is  stored in  the  tsthost.exe directory.  So is

simple  have multiple unproto list. If you receive for example, two

list,  from two  bbs, when  you change  homebbs tsthost  change the

list information, and are ready to work with the other bbs.

IMPORTANT NOTE

--------------

Some  programs allow the use of unproto frames with  the  poll bit

set, that is not a  problem  for  the  Unproto  list.  However, in

certain conditions, this can become a problem.

For  instance,  you are  connected to  your  HomeBBS on  channel 1.

TSTHost  emits  a  Sync  Unproto  frame  towards  your HomeBBS, for

the Unproto   list.  The  HomeBBS, when receives the Unproto  frame

with  the poll bit set, returns you  a   DM  frame. This is correct

since your monitor channel is not connected.

But  your AX25 interface receives the DM frame ,  and  then  it is

redirected to the channel 1 where you are connected, to disconnect

the   link. In  this case you  are disconnected by  your TNC, while

for the BBS you are still connected.

To  avoid this problem, TSTHost  starts by sending  the @U0 command

to  disable the poll  bit in unproto  frames. Maybe  some Eproms or

Driver do not  recognise the command (as far as I know, only  TFPCR

v2.10)  and if the  Eprom/Driver starts with  the poll  bit set you

cannot  fix the problem so  you'll be randomly  disconnected by the

system.

TstHost version 1.43B                                      63

UList Subcommands

-----------------

Use these commands after the ULIST prefix, for instance:

ULIST QUERY 122

* Autoselect +/-TO_Field +/-<FROM_field +/-@ROUTE_field

Automatic selection of the messages in the  Unproto list according

to  the desired TO_Fields (up  to 20). For instance,  to select the

ALL, DX and NEWS fields the command is:

UL A +ALL +DX +NEWS

Then, to add the KWD field and remove the ALL field:

UL A +KWD -ALL

With  the same format of  above, you may select  the FROM field; to

do this, you  must add the prefix '<' to the sender of the message.

For example, UL A +ALL +<IK1GKJ

UL A  without parameters displays the autoselect list.

* Disable

Disables the  Unproto list. The  Unproto frames are not decoded to

the list. The list is saved in the TSTHOST.ULS file in the TSTHost

default  directory. The memory used for the list is released. NOTE:

the list will be automatically saved every hour.

* Enable

Activates the Unproto list.  The previous list is loaded from disk

(TSTHOST.ULS), then any new message is added to the list.

Being  enabled by the HomeBBS, a missing  message activates a synch

request so to keep the list always updated.

* Get

Connects the HomeBBS to receive the selected messages.

* Killmail ON | OFF

If  on, after any connection  to homebbs to  download selected file

in  unproto list, tsthost send to the  bbs the command KM, in order

to  delete any message for you already received from the bbs queue.

This is  useful to avoid the needed to manually connect the bbs and

send the KM command to delete your mail.

TstHost version 1.43B                                      64

* List

Displays  in a  window the  message list.  Use the  standard cursor

keys  to  scroll the  list,  INSert to  select/deselect  a message,

ESCape or  RETURN to close the window. A "-" displayed in the first

column,  means that the message was already  present at the time of

the  last list command, an "R" means that the message was requested

and   received  from  HomeBbs.  "X"  means  that  the  message  was

requestes to HomeBbs, but was not available.

Whitout  parameters, all the  list will  be displayed.  You may use

some exstension command to display a selective list:

UL L           display all

UL L> CALL     all addressed to call

UL L< CALL     all send from call

UL L@ ROUTE    all with the specified route

UL LS AABB     all that have AABB in the title

UL LX          all the messages in X state

UL LQ          all the messages market to be received (*)

UL LR          all the message received

UL LN          all the new arrived messages

Note:  The commands LX LQ  LR LN are  static, so do  not follow the

really  list status, but reflect the status of the message when the

UL L command was done

In  the unproto list window,  the key + select  all message present

in  the list  for reception,  the key  -  deselct all  the messages

selected for the reception, useful for selective list.

* Merge filename

The purpose  of the command is to merge the contents of the unproto

list  in memory with an unproto list on  a file. In this manner, is

possible  make  server,  program,  or  other,  that  transform  the

standard bbs connection list, to an unproto list compatible.

The  union work in conjunction with all the filtering selection for

the  active unproto list, autoselect, notselect, pmsg..... Filename

will  be searched  for default  in the  workdir, if  a path  is not

directly specified.

Order,  or not order, for  the message   in the merge  file are not

important,  they will be sorted with the unproto in memory. But the

file  must have a  unproto format  compatible, data  must be stored

exatcly like if they was received via radio.

TstHost version 1.43B                                      65

Here  is the format  for the unproto  list, and the  format for our

file.  Lines cannot have more of 80 characters, any line invalid is

discarded, any valid line have the form:

NUMBER TYPE SIZE TO [@ROUTE] FROM DATE TITLE

|      |    |    |  |        |    |    |--title of the message

|      |    |    |  |        |    |--date of the msg, es:DDMMYY

|      |    |    |  |        |--call of the sender without ssid

|      |    |    |  |--routing, optional, no ssid

|      |    |    |--addressee of the message, without ssid

|      |    |-- a number, is the size of the message.

|      |-- type, a characters, may be P,A,B,T

|-- number of the message.

Right example:

123    P    3214 IK1GKJ@IK1MSL IK1VCQ 110293 TEST

124    B    1233 TUTTI @ITA    IK1MSL 110293 TEST 1

125    P   12345 IK1MSL        IK1GKJ 110293 TEST 2

You must do the right number of element and correct spaces

* Not +/-TO_Field +-<FROM_field +/-@ROUTE_field

Work  exactly like Autoselect,  but the  fields set  into this list

will not be inserted in your unproto list.

* Pmsg ON | OFF

If  OFF,  only personal  mail addresses  to  your callsign  will be

added  to the unproto list,  other P mail will  be discarded. If ON

all P mail will be added to the unproto list.

* Query [msg_nbr]

Manual  request  to the  BBS  to  send a  list  from  the specified

message  number.  This  command should  not  be  needed,  since the

program  is always  automatically synchronised.  If msg_nbr  is not

specified,  automatically  the program  ask for  the  last received

message in  the list. NOTE: if a manual or automatic QUERY will not

receive an answer, after 1,5 minuts the program will ask again.

TstHost version 1.43B                                      66

* Rpath [OFF | digipeater list]

Normally  you do  not need to  use this  command, may  be some case

where  the  connection bbs  have a  digi  list, and  resync request

another  path. If you define  a digipeater here,  the connection to

homebbs  will be executed with  the path defined in  HBBS and HNODE

command,  but  the  RESYNC request  will  be  addressed  to homebbs

trought this path.

* Time 0..60 minutes

This  command    sets  the  time  interval,  in  minutes,  for  the

automatic  connection. If there  is  any  message selected, TSTHost

connects  the HomeBBS for  the message  request. Any  value below 5

min.  disables this feature.

* Waitresync 0..600

Define  the time after which,  if a pending unproto  list resync is

not   satisfied,  the  request  will  be  repeated.  Accept  timing

between 15 and 600 seconds, default 60.

If  set to 0, switch   the unproto ist in  passive mode. This means

that tsthost  continue  to insert the received  data in the unproto

list,  but, if  a packet  was loss, the   resync will  never asked.

This  may be useful for who update  the unproto list converting the

bbs  list  and  merging  in the  unproto  list  with  the  UL MERGE

command.

TstHost version 1.43B                                      67

TstHost commands description

----------------------------

The uppercase letters are the minimum command abbreviation for the

command recognition. Facultative parameters are  shown  in  square

brackets, while mutually exclusive options are separated by |.

Send,Dir,Yput parameter window

------------------------------------------

If  the parameters for the SEND,  YPUT and DIR commands   have  not

been  specified,  the  program  activates  a request window on the

screen showing the files contained in the WORKDIR directory.

Select  the  desired  file  with  the  ENTER  key,  by  moving  the

selection   bar with  the   cursor  keys. To  log to another drive,

press the TAB  key and type the corresponding  letter.  The  ESCape

key  aborts the  request. The ENTER  key has  no effect  in the DIR

window.

Option that include a directory display

---------------------------------------

All  the option that  display a  directory contents  on the screen,

have  the  capability to  sort  the directory  contents  in several

mode.  Simple with the directory displayed press  CTRL-N to list on

filename+extension  order,  CTRL-E  for  extension+filename, CTRL-D

for  date,  CTRL-S  for file  size,  CTRL-U  unsorted.  The program

remember  the latest option  selected, and use  it again  for a new

directory  list.  NOTE:  the command  COPY  F5,  work  only  in the

current  directory. To copy a  file in another  directory, you must

use the command COPY from the tsthost command line.

* 7PCopy ON | OFF

If ON,  any message arrived in your pms, or with unproto list, that

have  any valid 7plus code, will be copied into the file 7PDATA.FWD

in  the  WORKDIR path.  This file  is  in the  standard MBL-RLI-FBB

forward-export  mode, and may  be processed from  server or prigram

that recognize this format, or directly from 7plus etc......

NOTE:   to  avoid  duplicated  reconstruction  of  programs,  7plus

messages  imported via the tsthost.in file will  not be copied into

the 7pdata.fwd file.

* AUtoyapp [on|off]

When  activated,  TSTHost starts  automatically  the  YAPP protocol

when  the init protocol code   is  received   from   the  connected

station,  otherwise you have  to start  the file  download with the

YREC command. The file is saved in the WORKDIR directory.

TstHost version 1.43B                                      68

* AX25 subcommand command

TNC setup command, read the description further on.

* CLS

Clears the receive screen for the active channel.

* CLRbuf

Clears the receive buffer for the active channel.

* COLor process_number color_number

Selects the color for the screen display. Read  the  help  text by

issuing this command without parameters.

* Connect callsign [v digipeater digipeater]

Connect the specified callsign on the current channel.

* COPy source destination

Copies the source file in the  destination name. If a path name is

not specified, the default directory is the WORKDIR. Wildcards are

not allowed, and you cannot  overwrite an existing file.

* Cronoffset minuts /I

Define  the  hour  offset to  execute  the  tsthost.cron  file. For

default,  the cron is executed when the  hour change, at 00 minuts,

for  example 12.00, 13.00.... if  you define 8,  the execution will

be shifted on 12.08, 13.08.....

/I,  if present,  instruct tsthost  to  execute   immediately   the

cron   file  after   the  cronoffset  statement.  If  the /I switch

is  not  present, the  cron  file will  be   executed  like  in the

previusly   release,   when   the   minuts   reach   the     offset

specification.  If the  /I is  present, the  cron will  be executed

immediately, and next at the offset specification.

TstHost version 1.43B                                      69

* DElete [filename]

Deletes  the file.  If  no drive/path  is  specified,  the default

directory is the WORKDIR path. Wildcards are not allowed.

* DIR

Directory   display   in the screen  window. The first  time you do

the  dir commands, the path is put  equal to the workdir. The other

dir  command, will rememeber the latest  path selected. DIR command

permits  the more common  file operation, like  delete, edit, view,

rename,  create/remove directory. Use the cursor key to move around

the  list; If  you press  ENTER on  a directory  name, you  will be

entered  in that directory. If you press  ENTER on a file name, you

may  automatically start  an external  dos program selected  by the

file extension. See the documentation about the TSTHOST.EXT file.

All  the function key for the command  is referred to the highlited

file.

* DISconnect

Disconnects the station.

* EBell [bit field value]

This  command set all the acoustic segnalation  of the software. Is

a bit  field value: this means that you must add the specific sound

value to obtain the desidered configuration.

1=Bell active on rx data (ex BELL)

2=Bell active on connect/disconnect (ex CBELL)

4=Bell active on end of yapp activity

8=Bell every 10 seconds if exist unread data for our connection

16=Bell every 10 seconds if new mail foor you are on the system

Default  value is 6, bell active for  connection and yapp activity:

this because 6=4+2.

* ECho [on|off]

If enable,  the  text entered from the keyboard is also written in

the receive window,  in the receive buffer and (if active)  in the

capture file.

TstHost version 1.43B                                      70

* EXIT [parameters]

EXIT      Whit no parameters, stop the program only if there are

not connected channels.                 EXIT CODE 0

EXIT D    Disconnect all channels, empty buffers, and next quit

from the program.                       EXIT CODE 1

EXIT W    Wait for all channels disconnected, disable new users

connection, next exit.                  EXIT CODE 2

After 30 minuts from the activation of the command,

if there are again users connected channels, the

program will force the disconnection.

EXIT X    Immediately stop. No action.            EXIT CODE 3

EXIT R    In case of wait to exit, (EXIT W), you may cancel the

exit request withthe EXIT R command.

EXIT @    Close all connection, close tsthost, and reboot the

PC. WARNING: this may be dangerous if you use disk

writing cache programs.

NOTE: While wait to exit, the program name in status line blink.

* FUnction function_key(1-20) [command.....]

It   is   possible   to define  ten commands  (up to  80 characters

long) that  will be executed by depressing the keys  CTRL + F1/F10.

Issue  the  command  with  the   function key number to display the

current  assignment. For instance,  FU 1  CON IK1MSL-8  assignes to

CTRL-F1   the connect command to IK1MSL-8.  Value from 1 to 10 will

be  associated  to CTRL-F1..F10,  value  from 11  to  20  to SHIFT-

F1..F10

* Help or ?

Displays a command list.

* LOg [on | off]

Enables/disables the connection  record  log,  saved  in  the file

TSTHOST.LOG  in  the  program directory.  Every  line  in  the file

contains  the callsign,  time and  date   for  any connection   and

disconnection.  To avoid that this   file  grows  too much,  it  is

suggested   to  move   and  rename   the file  in another directory

for further storage every month.

NOTE:   the   file  data   is   valid  only   if   connections  and

disconnections  happen  while   the programs  are active (including

the  shell command  RUN). If  the program starts  while the  TNC is

already  connected,  the missing  data are  logged  in the  file as

question marks (??).

TstHost version 1.43B                                      71

* MHeard [on | off | clr]

Similar  to the AX MHEARD  command, this command is  used to enable

(ON)  or disable (OFF) or clear (CLR) the internal mheard list. The

list have  a fixed size of 15 callsign, and naturally may work ONLY

if the  monitor channel is OPEN. If you for example set the monitor

for  only the U frame, the list is update  only when any U frame is

received,  because the tnc send only this  type of frames. Internal

MHeard  list is supplied only for that tnc  that do not have a list

inside.   For  tnc  capable  to  manage  the  list  internally,  is

preferable  the use  of the  AX MHEARD,  maintaining MHEARD  OFF in

order to save cpu time.

* MPerm [0...15]

When   you  activate  this  command  (ON),   a  little  windows  is

maintained  in the channel  1..8. In  this window  the program will

write  the monitor packets. Pratically, this is  the same data that

normally  you  see on  the channel  9,  monitor channel.  The value

0..15  set the  size of  the window.  Any value  under 3  close the

window and disable the function.

NOTE:  Monitor  data  type and  activation  are  done  with  the AX

MONITOR command.

* PARam channel text

Use   this   command  with care,  only if  you  know very  well the

WA8DED  host mode software. This command set a specific value for a

channel  or    an    extended  command  not  included  in the  AX25

subcommands  using the   host  mode commands, and  the data  is not

checked  by TSTHost.  It is  used for  particular settings  or non-

standard  Eproms. The user is responsible  for   any  data  sent to

the  host interface. The global communication   channel  is  number

0,   the  others   from  1   to  8  correspond  to   the eight task

windows.  Notice that data  written on channel  0 is  stored as the

default,   while  those  written   on  the  other channels are used

until  the  next disconnection  on the  channel  and return  to the

channel 0 default value.

* PASsword [CALLSIGN]

Automatic transmission  of  the  requested  password.  Support the

standard  request and  the MD2  algorithm. It  is also  possible to

send  the password with the  ALT_F1 key, that is  the equivalent of

PASSWORD command.

TstHost version 1.43B                                      72

If  CALLSIGN is not  specified, the  password will  be searched for

the  callsign  known  in  the  actual  tnc  link,  if  CALLSIGN  is

specified,  the password  will be  searched  for the  provided call

insted of the tnc link call.

* RECord filename | off

Captures to a disk file the task activity. The file  logging stops

with  the OFF filename or with the  task  disconnection. The status

line  displays  the  disk file  size  and  not  the  received bytes

number  because  logging  to an existing  file appends  data to the

end of the file without deleting the previous text.

* REName old_name new_name

Change  the name for the specified file. Wildcards are not allowed.

The   default directory is  WORKDIR. It is possible  to move a file

in  the same disk  to a different  directory by  specifying the new

path. For instance:

REN  pippo.txt pluto.txt

The file pippo.txt becomes pluto.txt

The file remains in the default directory.

REN  e:\bin\utils\vp.exe e:\bin\vp.exe

moves the file vp.exe from the path e:\bin\utils into

e:\bin

* RUn [filename]

Executes  the  Filename  DOS  program. If  the program name is not

specified, the command interpreter (COMMAND.COM) is run  until the

EXIT   command,  that   returns  to   TSTHost. The TSTHost activity

is suspended while executing DOS programs.

* SCReensave [Minuts 1..60 | TEST | OFF]

OFF disable screen saver

TEST activate it immediately

Any  numeric value tell tsthost to activate  the screen saver after

the specified time with no activity on the keyboard.

* SENd [filename | OFF]

Sends the specified file to the connected station. OFF  closes the

file, or  it  is closed at the end of the file or when the task is

TstHost version 1.43B                                      73

disconnected.  The  file must  contain  ASCII text  and  not binary

data, and the   SEnd command cannot be issued in a PMS session. The

status  line   counter  displays  the number  of bytes  sent to the

TNC,  so the file  can   be  closed when the TNC still have data in

the  buffer  to  be  transmitted.  If  the    filename    is    not

specified,  a  selection window is displayed on the screen.

* SOurce filename

Executes  a command  file. The  file contains  the commands  in the

same  form    as   for    the   keyboard.    Each  line  (up  to 80

characters)  must contain  a command.  The source file  is searched

ONLY in the  program directory.

* TNCDelay X Y

This   command  set  some program timings, that  you shouldn't need

to  change. The  values are  expressed in  18th  of second  and the

range is from 3 to 255, that correspond to 2 tenth and 14 seconds.

The  X parameter is   the maximum delay  for the TNC  reply. If the

TNC  does  not   reply to TSTHost   in  the   specific   time,  the

program  retries to synchronise the serial port. The Y parameter is

the speed for the serial port synchronisation.

If   your  Y parameter  is too high,  you can  go to have  a coffee

while  the  program  resynchronises! You  must  increase  X  to fix

frequent  timeouts, and Y if  the program cannot  resync the serial

port.

IF  YOU ISSUE THE COMMAND WITHOUT PARAMETERS, WILL BE DISPLAYED THE

CURRENT TIMING VALUE AND A STATISTIC OF THE PROGRAM ERROR.

* TRaceconn [ON | OFF]

When  you do a connection via  a node, for your  tnc you are always

connected  to the node, and this is  the callsign that is displayed

on  the tsthost status  line. This  command, if  active, attempt to

maintain  a trace of the node connection,  and report on the status

line  the  really connected  callsign.  The analisis  will  be done

reading   the  incoming  frame,  and  searching  for  the  standard

connection  response  issued by  the node  when  it receive  your C

command.  This message is  normally "CONNECTED TO"  followed by the

callsign   of  the  connected   station.  If   this  condition  are

satisfied,   tsthost  report  on  the   status  line  the  callsign

received.

TstHost version 1.43B                                      74

Unfortunately,  a  perfect  100%  analisis  is  not possible,  this

because  a node may not use standard  messages, or the message-call

combination  may  enter  at  your  system  from  other sources,  or

other....  In this case tsthost  may report an invalid  call on the

status  line.  For this  reasons  this function  may  be  enable or

disable,  use it for your exigence... if  you note invalid callsign

displayed when connect trought a node, leave this function OFF.

The command also update the log file if enabled.

* UList subcommand command

Command  for FBB Unproto  list, read   the   specific   section  in

this manual.

* View filename

Simple  activate the viewer  and display the  contents of filename.

If  a specified drive or directory is present in filename, the file

will  be  searched in  the  specified path,  otherwise  it  will be

serached in the path specified with the workdir command.

* WOrkdir [path]

Set the default work path for TSTHost.  Usually, it corresponds to

the  directory where the  program is  stored. When  a filename does

not  have a drive/directory  path, it will  be   searched   in  the

WOrkdir directory.

* WRitebuf filename

Writes  the contents of the  receive buffer in an  ASCII disk file.

If  the filename does not have a drive/directory path specified, it

is saved in the WOrkdir directory.

* YABort

Starts the procedure to interrupt the YAPP transfer protocol.

The  interruption  is not  immediate because  the  terminal station

must  empty   his   buffer   before   accepting   the   command. To

perform  an immediate abort, type this command TWICE.

* YAPpc [on|off]

Enables/disables  the  YAPPC protocol  version.  You  cannot change

this option if there is any YAPP file transfer active.

TstHost version 1.43B                                      75

* YRec filename

YAPP  receive  protocol  manual  start.  If  Autoyapp  is on,  this

command  is not  requested. If  Autoyapp is  off, you  must execute

this  command just after  having received by  the connected station

the   Init  YAPP transmit  code. If  the filename  does not  have a

drive/directory  path specified,  it will  be saved in  the WOrkdir

directory.  This command is not active in a PMS session.

* YPut [filename]

Start  the  YAPP  transmit protocol.  If a  drive/directory path is

not  specified, the file is  taken  from   the  WOrkdir  directory.

This  command is   not active   in a  PMS session.  If the filename

parameter is not specified, the program activates a select window.

* YTimeout [seconds]

To   specify  the  time  to  wait  without  receiving  data  before

declaring timeout in a yapp session. Default 150.

TstHost version 1.43B                                      76

AX25 Subcommands

----------------

These   parameters   must   follow the  AX25 command,  for instance

AX25  MONITOR N. The TNC manual reports   more  informations  about

these commands.

* AX25l2v2 [On|Off]

Enables the level 2 of the Ax25 protocol.

* Beacon [0-60] [Beacon Text | <Beacon file]

Set   the  beacon   text. The  time value is  expressed in minutes,

and  any  value below  5  disables   the   beacon.    If  the first

character  of the  beacon   text  is  '<', then the  others are the

text  filename. If no  drive/directory is specified,  then the file

will be  searched in the TSTHOST.EXE directory.

Example for text beacon:   AX BE 30 ST. IK1GKJ * IJ45IK

Example for file beacon:   AX BE 30 <F:\UTILS\BEACON.TXT

* Check [0..65535]

After  this time,  if was  not detect traffic  on the  channel, the

program test the integrity of the link. In 10ms.

* CLock ON | OFF | SET

The SET parameter sets the current time/date on the TNC.

ON enables the time/date display in the monitor window and  in the

status  messages, OFF  disables this  time/date display.  NOTE: Not

all  TNC EPROMs  accept this  command; it  works  fine on  TF24 and

TF26.

* CText [Text | OFF]

To  define a text  that will be  automatically sent to  the user at

the connection time.

* Digipeat [On|Off]

Enables/disables the digipeating capability of the TNC.

TstHost version 1.43B                                      77

* FRack [1..65535]

Pause  (if ACK has  not been received)  for the  TNC before sending

the  data packet  again. This value  can be  from 1  to 15 seconds.

Some  TNC EPROMs accept  valid data from  16 to   65535,  expressed

in 1/33  of second.

* FUllduplex [On|Off]

Enables/disables the full duplex transmission of the TNC.

* MAxframe [1..7]

Max number of data packets to transmit before waiting for ACK.

* MHeard [ON | OFF | CLR 3..MAX]

Without parameters, displays the list of the received stations.

CLR clears the list.

ON  enables the list automatic update.

OFF disables the list automatic update.

The  list size, set by CLR, must  be a value between  3 and the max

supported  by the tnc,  but warning, since  the mheard  list is not

maintained  in memory, but only displayed, you you ask for a number

greater  than the screen dimension, due to  the scroll you may lose

the  first  callsign  displayed.  Warning:  This  command has  been

tested  with EPROMs TF24  and TF26, but  not any  EPROM accept this

command.

* MYcall callsign

Set my callsign.

* MOnitor [NIUSC] [+- callsign callsign...] [/F] [/F]

N disable

I display I  Frames

U display UI frames

S display Supervisory frames

C display Connections frames

+ Include max. 8 callsign

- Exclude max. 8 callsign

If  the /F switch is set, the  program will continue to process ani

type  of data received, but display only  the header information of

a  pcket, without the data. To restore the original monitor display

option,  you may  do again  the command  AX MON  /F,  or set  a new

monitor mode.

TstHost version 1.43B                                      78

The  extension /T in the command will  activate the tracing for any

monitored  data packet....  data will  be displayed  in hexadecimal

format.  WARNING: tracing will  use some  cpu-time, so  use it only

if needed on slow machine.

* PAclen [40..250]

Byte size of the transmitted packet.

* PErsist [0..255]

Transmit  chance. When  the  TNC   must transmit data, it generates

a  random  number between 0 and 255.   If the number is higher than

the PErsist value it  can transmit, otherwise  the  TNC  must  wait

for SLottime time before generating a new number.

* POrtcall [channel 1..8] [call|OFF]

This command  assigns  a   different  SSID  or  callsign  for every

channel.    The command  without  parameters displays  the  list of

those  channels  with a  callsign  different from  the  AX25 mycall

value.  OFF  sets  the  channel  to   use  the callsign-ssid set by

the AX25 mycall command.

To   activate  the  default  callsign, use the OFF parameter. Note:

a  channel  with a  particular  callsign will  not  change  even by

changing  the global callsign. To disable the extended callsign use

OFF.  This  command has  effect,  for a  connected  channel,   only

after  the disconnection

Example: AX25 MYCALL IK1GKJ

AX25 PORTCALL 2 IK1GKJ-2

* PTt [ON | OFF]

Enables/disables the TNC PTT line. The  software  works correctly,

but, while in the OFF state, the TNC  does  not  activate  the PTT

line.

* RESptime [0..65535]

Wait time before ACKing an incoming packet, in 10ms.

* RETry [0..127]

Retry  number before  failure and  station disconnection.  0 stands

for forever.

TstHost version 1.43B                                      79

* Slottime [0..127]

See PErsist, in 10ms.

* Txdelay [0..127]

Delay   before  data   transmission after the  PTT line activation,

in 10ms.

* UNproto [text]

Unproto text and route where the Unproto messages are addressed.

* USers [0..8]

Max number of incoming connections.

* Validcall [On|Off]

Enables/disables the valid callsign control.

TstHost version 1.43B                                      80

THE STATUS LINE

---------------

From the bottom to the top of the screen:

The  last screen line  displays the  channel status,  the number of

bytes  transmitted for send file,  the disk file  size for incoming

text  capture. Only the TNC  status and the number  of free buffers

are displayed for Monitor and Unconnected channels.

* FR is the number of free buffers in the TNC (or TFPCX/R driver).

Every buffer contains 32 bytes. This values helps detecting if the

TNC   is  collapsing,   as  the Host  software disables the monitor

if  this  value is  under 256.  The TNC  collapses  when the  FR is

around  128. Among   the   various   reasons, check  the high speed

traffic  and slow   serial   connection  baud  rate,   the  monitor

active   with  multiconnection, etc.  The free buffers   value   is

global and not referred to a single channel.

*  BF is the number  of packet in  the TNC queue  to be transmitted

for  the channel. TSTHost has a ten  buffers queue. If the FR value

drops  below 300, the queue is  reduced to 5, and  if FR goes below

210 then the queue is reduced to 2.

This  function does  not  interfere with the MAXFRAME 7 parameter,

since the correct number of packets is BUF+UNACK.

* UN is the number of packets transmitted still to be confirmed.

This value depends on AX25 maxframe.

* RE is the time of the TNC attempts to  transmit  the  unack data

without   receiving   ACK   from   the   other station.  After AX25

retry value, the station will be disconnected.

* US is the amount of memory used by the system

* OK is the free memory in the system

* TX is displayed only by sending a file. This value is the number

of bytes read from the file and  transferred intothe TNC. When the

file transfer terminates on the PC side,  it  can  still  continue

because of the TNC buffer.

* RX is displayed only when the RECORD command is active, and this

value corresponds to the disk file size.

Next line up on the screen  displays the TSTHost status:  CONV for

conversation  mode   (typed data  is sent   to the  other station),

CMD   for  command   mode  (enter   commands  for  TSTHost   to  be

interpreted).  On  the  right side  there  is  the  current channel

number and the callsign and duration of the active connection.

TstHost version 1.43B                                      81

Then  there is a number that display the number of resynconization,

and  the  program  version.   REVIEW  indicates   that the revision

buffer  is activated (PageUp  key);  PMS  for the channel  set in a

PMS  session (default) or active in a  PMS session, MAIL means that

you  have received  new mail  for you. A  single M  blinking, means

that there is a macro active on that channel.

Above  these lines,  there is  a line  containing the  callsign for

each connected channel,  from 1 to 8. If there is new data that you

have not yet read from a channel, TSTHost will flash the callsign.

Another line is for the YAPP transfer status line, active  only if

the  channel is running  a YAPP file  transfer. The  target on this

line means:

Hder Header, the filename and file size

Stat Status of the yapp transfer

Xfer How many byte transferred

Bt/s Transfer speed, in byte for second

Tle  Time left to end, in the form HH:MM, Hours:Minuts, or

MM.SS, Minuts.Seconds if HH is 0.

Note: since  the Tle are valuated in "real time", is normally see a

great  variation of the value in the  first second of the transfer.

This is  due to the great variation of the time with the relatively

little  number of bytes  transferred. The value  will be stabilized

after about 30-40 seconds.

JOLLY CHARACTERS * AND ?

------------------------

The system  support the jolly characters ? and * in all the command

where  a selection may be made, like unproto list display, pms list

display, savemail, mailcopy... ect command.

?  ignore the characters in  that position, * ignore  the rest. For

example:

IK1GKJ, IK1GLJ are the same if searched with IK1G?J

IK1ABC, IK1GEF are the same if searched with IK1*

TstHost version 1.43B                                      82

EXIT CODE

---------

TstHost, at the end of execution, return this code:

0, if stop for EXIT command.

1, if stop for EXIT D command.

2, if stop for EXIT W command.

3, if stop for EXIT X command.

4, if stop for any unrecoverable error.

HOW PRINT A FILE

----------------

Any  command related to  a registration  of a  file, like MAILCOPY,

RECORD,  WRITEBUF.... may be redirected to the printer. Simple, use

like  filename the name of a standard output DOS device, such LPT1,

LPT2, PRN..... Example MAILCOPY 22 LPT1

Remember that you print is slow, it may be slow down tsthost.

SMARTDRV, DISK CACHE

--------------------

Even  is not indispensable, disk caching programs like smartdrv may

increase  the performance of  the program. Since  the tsthost files

is  not maintained in memory, but is  always read birectly from the

disk, a  cache program may speed up the program activity. If you do

not  have other reasons, the use of smartdrv or other cache program

is highly recomended.

TstHost version 1.43B                                      83

FILE AND MESSAGES VIEWER

------------------------

The same viewer  used to display the contents of a file, is used to

display  the  contents of  a PMS  messages.  The viewer  may handle

unlimited  size file and messages, but the  max line size is set to

255 characters:  if a line excedeed this size, the rest of the line

will be displayed on the next row of the screen.

Viewer available commands:

CURSOR UP      One line up

CURSOR DWN     One line down

HOME           Go to top of file

END            Go to bottom of file

CTRL-HOME      Go to top of file

CTRL-END       Go to bottom of file

CTRL-PAGE UP   Go to top of file

CTRL-PAGE DWN  Go to bottom of file

CURSOR LEFT    Move the video one column left

CURSOR RIGHT   Move the video one column right

CTRL-LEFT      Move the video on the next tabulation point

CTRL-RIGHT     Move the video on previusly tabulation point.

CTRL-W         Toggle Word-Wrap

F1             Display the available commands.

NOTE: tabulation is assumed of 8 characters.

The  << simbol on  the left upper  corner of the  viewer screen, if

present, tell   you that the screen is shifted right, is not on the

first column of the line.

The viewer riquire about 9k bytes of free memory.

TstHost version 1.43B                                      84

FILE AND MESSAGES EDITOR

------------------------

The  same  editor used  to edit  a  file is  used to  edit  the PMS

messages.  The editor  is structured  to edit  only TEXT  file, not

binary.

Editor available commands:

CURSOR UP/DWN       move to previusly/next line

PAGE UP-DOWN        move to previusly/next page

CTRL-HOME/END       move to top/bottom of editor

CTRL-PG UP/DWN      move to top/bottom of editor

CURSOR RIGHT/LEFT   move to next/previusly character

HOME/END            move to start/end of line

CTRL-RIGHT/LEFT     move to start of next/previusly word

F1   display a little help for the available commands

F2   save the contents of the editor and exit

F5   insert a file on current cursor position. Will be displayed

a list of files starting from the workdir directory: select

the desidered file with the enter key or abort the operation

with the escape key.

F8   Abort editing and exit. Before abort, will be asl to enter

Yes or No to confirm the abort.

BACKSPACE      Delete character left of the cursor

DELETE         Delete character under the cursor

CTRL-BS        Delete from cursor position to the start of the

left word.

CTRL-Y         Delete the current line

CTRL-W         Toggle on off word wrap mode

CTRL-J         Toggle on off justfication mode

INSERT         Toggle insert overwrite mode

RETURN         Go to end of line and eventually trunch the current

line if insert is active and the cursor is not on

the end of the line.

TAB            insert space to the next tabulation point if insert

is active, or move the cursor to te next tabulation

point if the overwrite mode is selected.

The status line diasplay this information:

I    Insert active, little cursor

W    Word wrap active

Fxx  Free memory left

Cxx Current cursor column

Lxx Current cursor line

TstHost version 1.43B                                      85

WORD  WRAP: Word wrap  is active on  column 78, adn  is referred to

the  cursor postion, not the effective line lenght: in thi mode the

text to  right of the cursor is maintained to the original form and

you  do not destroy ist formattation while  correct the text to the

left.  For the  same reason,  word wrap  is  active ONLY  in insert

mode: if  you are in overwrite mode, the editor assume that you are

corretting a line, and do not touch its original format.

Again,  the word warp is available only  while insert a characters:

if  for example you are on  a line that is  more than 78 characters

size,  and you move the cursor without insert charaters, the editor

do  not split the line,  so you may  examine a longh  line with the

wrap active without destroy its format.

File  reading and import do  not have word wrap:  file while readed

will be maintained to its original formattation.

If  word wrap is deactivated, you may  edit line to unlimited size,

or  limited only by  the free memory:  you must press  enter key to

insert a new line.

In  overwrite mode, all  typed will  overwrite the  contents of the

previusly  data, with an  excpetion: if you  are at the  end of the

line,  and  insert  new  data,  the  data  will  not overwrite  the

contents  of the  next line  but will be  added to  the end  of the

current line.

Justification,  if active, will be done  by the wraparound routine,

so follow the same rules.

NOTE:  The editor may  handle up  to 24576  characters, and require

about 26K of memory to run.

TstHost version 1.43B                                      86

MAINTUSR - program to manage user database

------------------------------------------

MAINTUSR  program  is supplied  to  modify the  user  database file

TSTHOST.USR:  any user  that connect  your pms  have a  record here

defined in this file.

Running  MAINTUSR you  will see  the list  of  all users  that have

connected  your  pms.  In this  list  will  be  presented  the main

information  about the user: the name,  the privated directory, the

date  of last connection on  your system, the  number of connection

made to your pms from the user.

Pressing  F1  key  the  programm  will  display  a  little help  to

remember ypu the available options:

- Using CURSOR  KEY, or home, end, pg up, pg down, you may navigate

trought  the  list. The  actual selected  record  is marked  by the

highlighted line.

-  Pressing ENTER you may be  able to modify the  user name and his

private  directory. In the high part of the screen the program will

ask  to you for enter the  user data: end the  input with the ENTER

key  or abort it with ESCAPE key.  If you press enter without enter

any  data, the  field will  be empty,  this is  useful to  remove a

previusly  set private directory. The max  dimension for name field

is  12  characters,  while the  size  of  private  directory  is 30

characters,  that  will  be  reduce  to  29  because  the  programs

automatically add the '\' terminator.

-  ESCAPE permits to abort the database upadating. Maintusr program

will ask for a confirm: Y for yes or N for no.

- With  F10 key you will save the modified record and terminate the

program. Again, maintusr will aska confirm to you. (Y/N).

-  With  ALT-D combination  key  you may  marked  as  "deleted" the

current  record, or, if the record is  already deleted, restore it.

A  deleted record will  be displayed with  a 'D'  characters in the

first  column. Warning:  once saved  database, you  cannot restrore

deleted  record, because thay  will be phisically  removed from the

database.

-  With ALT-K you may  automatically delete all the  user that does

not connect you  pms from more than X days: the program will ask to

you  for the  specified number  of day;  pressing ENTER  to confirm

input data or ESCAPE to abort.

TstHost version 1.43B                                      87

- ALT-F is  used to search a particoular callsign, or a part of it.

Maintuser  will ask to input  the callsign to  search for. Pressing

ENTER  will confirm the input  data, while ESCAPE is  used to abort

the  input  callsign.  If found,  the  desidered  callsign  will be

placed  under the highlited  bar. The  search field  will be always

start from the beginning of the file.

The  same  command ALT-F,  if any  record  will be  found,  will be

provided,  after operator  confirmation, to  the creation of  a new

record for the specified callsign.

- ALT-N work  similar to the ALT-F key, but is used to continue the

previusly search comparison from the currently selected record.

PROBLEMS

--------

Please   report any   kind of  problem  or  defect with   a message

to IK1GKJ@IK1MSL, reporting:

- Defect type.

- Which operation you have done when you have noticed the defect.

- PC model and configuration

- Memory driver (EMM, QEMM etc.)

- PC Operating system version.

- (If possible) the TSR programs loaded in memory.

- TNC model and driver type

Thanks.

TstHost version 1.43B                                      88

Features .........................................................1

Tsthost with TNC in KISS or BAYCOM modem  (/K) ...................3

The sound and BAYCOM modem  (/Sx) ................................3

Tsthost with the standard HOST mode  (/H) ........................4

Tsthost with DRSI driver  (/D) ...................................4

Tsthost with 2 or more tnc .......................................5

TstHost and diskette usage, parameter /ND ........................6

Memory usage .....................................................6

Expanded memory and /NOEMS command ...........................6

Extended memory and /NOXMS command ...........................7

Reduction of RX buffers, /Rsize command ......................7

Number of channels, /K parameters ............................7

Unproto list size, /U parameters .............................7

VGA mode .........................................................8

Extended interface for data request, switch /V ...................8

TSR interface for monitor data, TSRMON command ..................10

Short cut keys ..................................................12

Path of the files ...............................................13

PG - Server directory .......................................13

MAIL - Messagges directory ..................................14

USERS - Users directory .....................................14

UPLOAD - Users upload directory .............................14

WORKDIR - Standard work directory ...........................14

TSTHOST.IRQ - Internal interrupt detection ..................14

TSTHOST.USR - Users database ................................15

TSTHOST.HBB - Automatic routing file ........................15

TSTHOST.HRT - Hierarchical routing database. ................15

TSTHOST.PSW - Automatic password file .......................16

TSTHOST.SYS - Password file for PMS SYS command .............17

TSTHOST.CFG - Automatic configuration file ..................17

TSTHOST.SHD - Shutdown configuration file ...................17

???????.ULS - Unproto list file .............................18

TSTHOST.ENT - PMS connection text ...........................18

???????.ENT - Selective PMS connection text .................18

TSTHOST.HLP - PMS help file .................................18

TSTHOST.PGS - PMS PG server list ............................18

TSTHOST.LOG - Connections log ...............................19

SOURCE files ................................................19

TSTHOST.CRN - Command file executed every hours .............19

TSTHOST.BID - Bid file ......................................20

TSTHOST.IN - Mail import file ...............................21

TSTHOST.DMS - PMS messages database .........................22

TSTHOST.EXT - File extension for automatic execution ........23

THE MACRO COMMANDS ..............................................24

What is a macro? ............................................24

Macro specified command .....................................25

* DELAY XX ..............................................25

* DISPLAY TEXT ..........................................25

TstHost version 1.43B                                      89

* IFYAPPACTIVE XX .......................................25

* IFNOTYAPPACTIVE XX ....................................26

* IFYAPPERROR XX ........................................26

* IFNOTYAPPERROR XX .....................................26

* IFFILESEND XX .........................................26

* IFNOTFILESEND XX ......................................26

* IFUNDEFPAR PARNUMBER LINENUMBER .......................26

* IFULSELECTED LINENUMEBER ..............................26

* GOTO XX ...............................................26

* SAY DATA DATA DATA...... [CTRL-?] .....................27

* WAITSTRING TIME (LINENUMBER) STRING TO WAIT ...........27

* WAITDISC [XX] .........................................27

* WAITCONN ..............................................27

TstHost.mac macro file format ...............................28

TSTHOST commands not active in the macro ....................29

TSTHOST commands that require an detailed arguments .........29

Macro example ...............................................30

AUTOMATIC PASSWORD ..............................................32

IMPORTANT, PLEASE READ ......................................33

FORWARD,  UNPROTO  list,  CHANNEL  8  and automatic password ....34

???????.CON - HomeBbs connection file .......................34

FORWARD protocol ............................................37

Problem to connect for UNPROTO and FORWARD ..................38

PMS INTERFACE ...................................................39

MAIL WORD FLASH IN SCREEN ...................................39

PRIVATE DIRECTORY FOR PMS USERS .............................40

THE SERVER ..................................................40

PG SERVER ...................................................41

CON_FILT.EXE, connection filter .............................43

MES_FILT.EXE, message filter ................................43

PMS MESSAGE INSERTION SEQUENCE ..............................44

MESSAGGE FLAGS ..............................................46

PMS REMOTE MESSAGE MANAGEMENT ...............................47

THE TZ ENVIROMENT VARIABLE ..................................47

PMS MESSAGE MANAGEMENT ......................................48

PMS COMMAND FROM KEYBOARD ...................................51

* ACK [H | Y | N] .......................................51

* BMail [ON | OFF] ......................................51

* BReak [callsign | ALL] text ...........................51

* CH8RESERVED ON | OFF ..................................51

* ENdmessage filename | OFF .............................51

* EXPORT N | F ..........................................52

* FBbmode ON OFF ........................................52

* FREverse ON OFF .......................................52

* FTIME 0..60 minutes ...................................52

* FWD ...................................................52

* HAlias CALL-SSID ......................................52

* HIeraddr TEXT | OFF ...................................52

* HOmebbs CALL-SSID .....................................53

TstHost version 1.43B                                      90

* HRoute CALL [/V | /P ROUTE] ...........................53

* LBHeld ON | OFF .......................................53

* LPHeld ON | OFF .......................................53

* MAIlcopy [number... <IK1GKJ >IK1GKJ @IK1GKJ] [filename]53

* MSGHEADER ON | OFF ....................................54

* NBID bidnumer (1...65535) .............................54

* PMS ...................................................54

* PYAPP [ON | OFF] ......................................54

* RIng [ON | OFF] .......................................54

* RLine ON | OFF ........................................54

* SAvemsg +/-TOFIELD +/-@ROUTE +/-<FROMCALL .... up to 2055

* SERver +/-SERVERNAME +/-SERVERNAME .... up to 20 ......55

* TALK ..................................................55

* TImeout minutes .......................................55

* UPdatedms pday bday pfday .............................55

* USerdir [path] [path] [path] [path] [path] ............56

PMS commands for remote users ...............................57

UNPROTO LIST ....................................................61

IMPORTANT NOTE ..............................................62

UList Subcommands ...........................................63

* Autoselect +/-TO_Field +/-<FROM_field +/-@ROUTE_field .63

* Disable ...............................................63

* Enable ................................................63

* Get ...................................................63

* Killmail ON | OFF .....................................63

* List ..................................................64

* Merge filename ........................................64

* Not +/-TO_Field +-<FROM_field +/-@ROUTE_field .........65

* Pmsg ON | OFF .........................................65

* Query [msg_nbr] .......................................65

* Rpath [OFF | digipeater list] .........................66

* Time 0..60 minutes ....................................66

* Waitresync 0..600 .....................................66

TstHost commands description ....................................67

Send,Dir,Yput parameter window ..............................67

Option that include a directory display .....................67

* 7PCopy ON | OFF .......................................67

* AUtoyapp [on|off] .....................................67

* AX25 subcommand command ...............................68

* CLS ...................................................68

* CLRbuf ................................................68

* COLor process_number color_number .....................68

* Connect callsign [v digipeater digipeater] ............68

* COPy source destination ...............................68

* Cronoffset minuts /I ..................................68

* DElete [filename] .....................................69

* DIR ...................................................69

* DISconnect ............................................69

* EBell [bit field value] ...............................69

TstHost version 1.43B                                      91

* ECho [on|off] .........................................69

* EXIT [parameters] .....................................70

* Help or ? .............................................70

* LOg [on | off] ........................................70

* MHeard [on | off | clr] ...............................71

* MPerm [0...15] ........................................71

* PARam channel text ....................................71

* PASsword [CALLSIGN] ...................................71

* RECord filename | off .................................72

* REName old_name new_name ..............................72

* RUn [filename] ........................................72

* SCReensave [Minuts 1..60 | TEST | OFF] ................72

* SENd [filename | OFF] .................................72

* SOurce filename .......................................73

* TNCDelay X Y ..........................................73

* TRaceconn [ON | OFF] ..................................73

* UList subcommand command ..............................74

* View filename .........................................74

* WOrkdir [path] ........................................74

* WRitebuf filename .....................................74

* YABort ................................................74

* YAPpc [on|off] ........................................74

* YRec filename .........................................75

* YPut [filename] .......................................75

* YTimeout [seconds] ....................................75

AX25 Subcommands ............................................76

* AX25l2v2 [On|Off] .....................................76

* Beacon [0-60] [Beacon Text | <Beacon file] ............76

* Check [0..65535] ......................................76

* CLock ON | OFF | SET ..................................76

* CText [Text | OFF] ....................................76

* Digipeat [On|Off] .....................................76

* FRack [1..65535] ......................................77

* FUllduplex [On|Off] ...................................77

* MAxframe [1..7] .......................................77

* MHeard [ON | OFF | CLR 3..MAX] ........................77

* MYcall callsign .......................................77

* MOnitor [NIUSC] [+- callsign callsign...] [/F] [/F] ...77

* PAclen [40..250] ......................................78

* PErsist [0..255] ......................................78

* POrtcall [channel 1..8] [call|OFF] ....................78

* PTt [ON | OFF] ........................................78

* RESptime [0..65535] ...................................78

* RETry [0..127] ........................................78

* Slottime [0..127] .....................................79

* Txdelay [0..127] ......................................79

* UNproto [text] ........................................79

* USers [0..8] ..........................................79

* Validcall [On|Off] ....................................79

TstHost version 1.43B                                      92

THE STATUS LINE .................................................80

JOLLY CHARACTERS * AND ? ........................................81

EXIT CODE .......................................................82

HOW PRINT A FILE ................................................82

SMARTDRV, DISK CACHE ............................................82

FILE AND MESSAGES VIEWER ........................................83

FILE AND MESSAGES EDITOR ........................................84

MAINTUSR - program to manage user database ......................86

PROBLEMS ........................................................87