CPUID 4.30 (C)Copyright 2000 William H. Decorie.  All rights reserved.

The CPUID program is very simple to run.  It takes no parameters and runs

on any MS-DOS compatible operating system.  Note that any multitasking

operating systems may slightly affect reported internal speed and may

also throw off the cache size detection.

During development, different versions of this program have been tested

under MS-DOS 5.0/6.0/6.20/6.22, DR-DOS 6, PC-DOS 6.1/6.3, Windows 3.1,

Windows 3.11, Windows for Workgroups 3.11 and Windows 95.  It *should*

work just fine under OS/2, but I haven't had a chance to try it.  My

machine currently has Windows 95 on it, and for the most part most of the

machines that I run it on at work have some form of 32-bit Windows.

Although this program will run just fine in V86 mode, for best results

I recommend running it in real mode.  Some tests are not performed if

running in V86 mode, such as the more accurate Time Stamp Counter speed

measurement.

This program reports at least three pieces of information about the target

machine:  CPU type, FPU type, and internal CPU speed.  The prefetch

queue size is also always reported on all CPUs below Pentiums.

If the cpu is a 486 or better, then the size of the internal data cache

is also displayed.  If the external cache is large enough and good

enough, it will mask the size of the internal cache.  This will be

reported as a cache size followed by a question mark.  This reported

cache size will usually be one increment larger than the real size

(e.g. if the cache is really 8KB, it might be reported as '16KB?').

If the cpu supports the internal CPUID instruction, then it is executed

and some of its information is displayed: family, model and stepping

values, and the vendor ID string.

If the cpu is a Pentium or equivalent or better, the Time Stamp Counter is

used to more accurately measure the clock frequency.  However, this can only

be done in real mode (NOT V86 mode), so if you are running an EMM or are

in Windows, the older algorithm will still be used.

If the cpu supports MMX instructions, this will be indicated after the

cpu name by showing "(MMX)".  Support for 3DNow! and KNI are shown in

a similar fashion.

For AMD and Intel Pentium class and higher CPUs, if the CPUID command

supports it, the data about the cache is retrieved using CPUID instead

of measuring it.

For all Cyrix parts, the contents of the Cyrix DIR registers is displayed.

I use several tricks to differentiate between the various cpus, some

of them officially approved and some not.  This program is by no

means perfect, but it will at a minimum tell you what family your CPU is

in and what speed it is running at accurately.

On Cyrix 5x86/6x86 parts, this program enables the CPUID instruction and

leaves it enabled upon exit.  What this means is that after running,

a 6x86 will appear to some programs as a Pentium (whereas before running

it would appear as a 486).  One example is the cpu detection code in the

original version of Windows 95.  If you have a Cyrix, look in System in

Control Panel; on the first tab (General) it will say 486.  Stick my

program in your AUTOEXEC.BAT file and reboot.  Now looking in System*General

will show Pentium(r).  The algorithm used by Win95 apparently just uses

the CPUID instruction to get the family (which is 5 for the 6x86).  If it

detects that the cpu doesn't support CPUID (which the 5x86/6x86 don't

unless you turn it on), then it assumes the cpu is a 386 or 486.  It

decides between the two the same way I do, by checking for the AC bit

in EFLAGS.

So far the only vendors supported are Intel, AMD and Cyrix (and NEC for

their V20 and V30).  The newer IBM parts are really Cyrix parts and will

be reported as Cyrix.  The older Cyrix parts were also manufactured by TI

and SGS and will also be identified as Cyrix.  The older IBM parts (e.g.

Blue Lightning) will probably be reported as Intel 386 or 486s.  I have

no idea what UMC or NexGen parts will be identified as.  (I've never seen

a NexGen, and I've only seen UMC parts on motherboards outside of cases

at computer shows.)

In many cases, the only differentiation possible between Intel 486 parts

and AMD 486 parts is clock speed (e.g. only Intel made 486DX4/75, only

AMD made 486DX2/80).  Where only one vendor made a part at the detected

speed, that vendor is shown.  Otherwise, Intel is the assumed vendor.

(NOTE: All pre-486 parts are reported as being Intel, because internally

they are all identical.)

If you find that my program incorrectly reports any information,

please let me know 1) what my program reported and 2) what you

actually have.  Also, if you know of a way to detect a certain type

of CPU or FPU, then please drop me a line and let me know.  I will

include your name (if you want) in future releases if I use your

code.

I wrote this program to assist me in my work (I am a computer

technician) and that is it's main purpose.  Any other purpose is

entirely secondary and I cannot be held liable for any use or misuse

of this program.  I am giving it away for free for the good of all

mankind (yes, even Intel employees).  If you find my program in error,

please tell me and I will fix it.

This is a complete list of all of the CPU types that the program

currently detects:

Intel 8088(1), Intel 8086(1), NEC V20, NEC V30, Intel 80188(1),

Intel 80186(1), Intel 80286(1), Intel 80386SX(1), Intel 80386DX(1),

Cx486SLC, Cx486DLC, Cx486SX/SX2, Cx486DX/DX2/DX4, i486SX(3), i486SX2(3),

i486DX(3), IntelDX2(3,4), IntelDX4(4,5), Am486DX(3), Am486DX2(3,4),

Am486DX4(3,4,5), Am5x86(3), Cyrix 5x86(6), AMD K5, AMD K6/-2/-3,

AMD K7(6), Cyrix 6x86, Cyrix 6x86MX/M-II, Pentium (buggy FPU)(7),

Pentium(7), Pentium Pro, Pentium II, Celeron, Pentium III.

1) All CPUs are reported as Intel, regardless of actual vendor

2) --deleted--

3) Internal frequency is used to determine vendor

4) CPUID vendor string is used to determine vendor

5) Cache size is used to determine vendor

6) Internal frequency is a complete guess at this point

7) FPU status means presence of divide flaw

This is a complete list of all of the FPU types that the program

currently detects:

8087(1), 80287(1,2), 80287XL(2), 80387SX(3,4), 80387DX(2,3,4),

Cx387SLC(3,4), Cx387DLC(3,4), internal FPU.

1) FPU type determined by CPU type

2) FPU type determined by infinity control

3) 16- or 32-bit model is determined by CPU type

4) FPU vendor is determined as Cyrix/not Cyrix

Note that except for the Cyrix FPUs, the vendor is NOT identified.

The Cyrix 5x86, NEC V30, and 80186 are the only reported CPUs that I

have not actually been able to run this program on.  All of the other

CPUs have had some version of this program run on them and had the correct

name returned.  Of course, as this program is developed the older machines

are harder to come by to test on, so some new feature or other might break

this program on older CPUs.  If you discover this, please let me know.

The prefetch queue size is the most unreliable of all the tests.  Mostly

on 386s, it will sometimes report a too small size.  I remember reading

somewhere about low-water and high-water marks affecting how the queue

works (and therefore it's apparent size), but I can't remember where I

read that information.  Also, some very early 386s had a bug which caused

their queues to be only 12 bytes instead of 16.  Unfortunately, 12 is

also the misreported size on 386s, so getting this value DOES NOT MEAN that

you have a very early 386.  I added the prefetch queue size routine long

after I had access to anything short of a 386, so I don't know how accurate

(if at all) it is on pre-386 machines.

If you have any suggestions, comments or other useful information, please

let me know!  I can be reached at:  wdecorie@yahoo.com

UPDATED NOTE (06/25/2000): I haven't touched this program in a long time,

and I've found it locks up quite often on the newer CPUs.  I will (RSN) do

a complete rewrite and call it version 5.00; until then, please report any

problems so I know where to look to fix it.  Also, I will be adding support

for the new CPUs (Athlon, Thunderbird, Duron, Celeron II, Cyrix III,

Willamette (Pentium IV?)).